FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: IMS+MSM+MPR 2+10+30 OD Product name(s): CERENET Chemical active substances: Iodosulfuron-methyl-sodium, 2 g/L Mesosulfuron-methyl, 10 g/L Mefenpyr-diethyl (safener), 30 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Certiplant BV Submission date: June 2024 Finalisation date: 01/2025; 04/2025

Version history

| When | What |
| --- | --- |
| January 2025 | ZRMs evaluated dRR submitted by Applicant. |
| April 2025 | The Final Registration Report |

Table of Contents

5	Analytical methods	5

5.1	Conclusion and summary of assessment	5

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	5

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	5

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	5

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	8

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	8

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	8

5.2.2	Methods for the determination of residues (KCP 5.1.2)	8

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	13

5.3.1	Analysis of the plant protection product (KCP 5.2)	13

5.3.2	Description of analytical methods for the determination of residues of iodosulfuron-methyl-sodium (KCP 5.2)	13

5.3.2.1	Overview of residue definitions and levels for which compliance is required	13

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	14

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	15

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	16

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	17

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	17

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	18

5.3.2.8	Other studies/ information	18

5.3.3	Description of analytical methods for the determination of residues of mesosulfuron-methyl (KCP 5.2)	18

5.3.3.1	Overview of residue definitions and levels for which compliance is required	18

5.3.3.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	19

5.3.3.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	20

5.3.3.4	Description of methods for the analysis of soil (KCP 5.2)	21

5.3.3.5	Description of methods for the analysis of water (KCP 5.2)	21

5.3.3.6	Description of methods for the analysis of air (KCP 5.2)	22

5.3.3.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	22

5.3.3.8	Other studies/ information	23

Appendix 1	Lists of data considered in support of the evaluation	24

Appendix 2	Detailed evaluation of submitted analytical methods	44

A 2.1	Analytical methods for Iodosulfuron-methyl-sodium	44

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	44

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	54

A 2.2	Analytical methods for Mesosulfuron-methyl	54

# Analytical methods

This document is a summary of the data submitted to support the registration of the plant protection product IMS+MSM+MPR 2+10+30 OD in the central zone.

The product contains 2 active substances (iodosulfuron-methyl-sodium and mesosulfuron-methyl) and 1 safener (mefenpyr-diethyl). Since no EU agreed regulation for the evaluation of safeners exists, review may differ amongst countries. As peer recommendation of the central zone Steering Committee (czSC), the full risk assessment for the safener is not addressed in the core dossier but left to the review by Member States as part of their national evaluation.

## Conclusion and summary of assessment

State whether submitted data are sufficient for evaluation. Data gaps and conditions for authorization should be listed, if appropriate.

Sufficiently sensitive and selective analytical methods are available for the active substance(s) and relevant impurities in the plant protection product.

Noticed data gaps are:

- None

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are:

- none

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Cereals | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

Insert information on analytical methods and their validation (include information as to whether the methods were already evaluated during the EU approval process of the active substance).

If study has not been previously evaluated, provide study summary.

When more than one active substance is present in the plant protection product, a method capable of determining each, in the presence of the other(s), shall be provided.

When an applicable CIPAC method is recommended for the determination of the active substance in the plant protection product, it is sufficient to refer to section 5.2.1.4.

An overview on the acceptable methods and possible data gaps for analysis of iodosulfuron-methyl-sodium and mesosulfuron-methyl in plant protection product is provided as follows:

Report only those studies that have not been previously evaluated within a peer reviewed process at EU level.

| Comments of zRMS: | The analytical method (HPLC-UV/Vis) for the determination of Mesosulfuron-methyl and Iodosulfuron-methyl-sodium in the formulation CERENET has been submitted. The analytical method is validated and meets the criteria of specificity, linearity and precision according to the requirements of SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1/01 |
| --- | --- |
| Report | IMS+MSM+MPR 2+10+30 OD: Validation of the Analytical Method for the Determination of the Active Ingredient Content, Mattioli B, 2024, CH-0227-2024 |
| Guideline(s): | Yes EEC guideline SANCO/3030/99 rev. 5 dated 22/03/2019 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Chromatographic conditions

| HPLC column | PHENOMENEX or equivalent |
| --- | --- |
|  | Luna C18 100 Å, 5 µm, 250 x 4.6 mm i.d. |
| Detector | UV/Vis operating at 244 nm until 10 minutes then change signal at 320 nm |
| Column temperature | 25°C |
| Eluent A | Water |
| Eluent B | Acetonitrile |
| Eluent D | Phosphoric acid 1% v/v |
| Eluent (gradient) | From A:B:D 40:50:10 to A:B:D 5:85:10 in 10 minutes A:B:D 5:85:10 for 5 minutes From A:B:D 5:85:10 to A:B:D 0:90:10 in 1 minute A:B:D 0:90:10 for 10 minutes |
| Eluent flow | 1.0 mL/min |
| Volume of injection | 10 µL |
| Mesosulfuron-methyl ret. time | about 5.3 minutes |
| Iodosulfuron-methyl sodium ret. time | about 6.8 minutes |
| Mefenpyr-diethyl ret. time | about 12.3 minutes |
| Post time | 5 minutes |
| Total analysis time | 31 minutes |

Results

Specificity

The analytical method, using the HPLC/UV instrument with quantification by external standard, was shown to be specific for Mesosulfuron-methyl and Iodosulfuron-methyl-sodium active ingredients content in test item formulation samples.

Linearity

Five working standard solutions were prepared and each solution was analysed by HPLC/UV.

The injected range and the relevant linearity range for the active ingredients are detailed in the table here below.

| Active ingredient | No. of WSS | Linear calibration: | Injected range (µg/mL) | Linearity Range (g/L) |
| --- | --- | --- | --- | --- |
| Mesosulfuron-methyl | 5 | y=0.360x-0.758 r=0.9995 | 48.92 – 146.76 | 4.84 – 14.53 |
| Iodosulfuron-methyl-sodium | 5 | y=0.303x-0.195 r=0.9998 | 9.39 – 28.17 | 0.93 – 2.79 |

(1) Calculated with respect to the nominal test item weight and preparative in repeatability.

No significant memory signal was detected in the washing injected after the highest working standard solution and the range tested for the active ingredients was found to be linear (correlation coefficient r > 0.99).

Repeatability (Precision)

The precision test was performed by five determinations of the test item (labelled A to E).

| Active ingredient | Test No. | Mean value (% w/w) (1) | Standard deviation (S.D.) (% w/w) | Relative Standard Deviation (RSD%) | Horwitz RSDr (2) |
| --- | --- | --- | --- | --- | --- |
| Mesosulfuron-methyl | 5 | 1.07 | 0.02 | 2.11 | 2.65 |
| Iodosulfuron-methyl-sodium | 5 | 0.22 | 0.005 | 2.11 | 3.36 |

(1) Calculated with respect to the weighed test item.

(2) % RSDr = % RSDR x 0.67; % RSDR = 2(1-0.5 log C), based on the Horwitz equation.

From data obtained, the Horrat value (RSD% / RSDr) resulted to be lower than 1 for the active ingredients and therefore the precision of the analytical method is considered acceptable.

Recovery (Trueness)

The test was performed by addition of the reference material aliquots to two weights of the Placebo (active ingredient concentration less than 10%) at one fortification level corresponding to the nominal concentration of the active ingredient content in the test item solution.

For the recovery, the mean recovery values obtained comply with the SANCO/3030/99 rev. 5 guideline’s requirement, as below:

- in the range 90 to 110 % for active ingredient content ≥ 1 % w/w and < 10 % w/w.

- in the range 80 to 120 % for active ingredient content ≥ 0.1 % w/w and < 1 % w/w.

Since all recovery values were in the correct range, these criteria were fulfilled and therefore recovery of the analytical method is considered acceptable.

| Active ingredient | Level | Tests No. | Recovery value (%) |
| --- | --- | --- | --- |
| Mesosulfuron-methyl | Spike A Added 10.82 g/kg | 1 | 97.28 |
|  | Spike B Added 10.88 g/kg | 1 | 101.14 |
|  | Mean value: |  | 99.2 |
| Iodosulfuron-methyl-sodium | Spike A Added 2.00 g/kg | 1 | 105.27 |
|  | Spike B Added 2.01 g/kg | 1 | 110.03 |
|  | Mean value: |  | 107.6 |

Conclusion

Since all the experimental results followed requirements, this analytical method can be considered validated according to EEC SANCO/3030/99 rev. 5 guideline.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

Not applicable.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

With respect to toxicological, eco-toxicological or environmental aspects the product IMS+MSM+MPR 2+10+30 OD does not contain any relevant formulants. Therefore, a special analytical method and validation is not needed.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

There is no CIPAC method available for the determination of iodosulfuron-methyl-sodium or mesosulfuron-methyl.

A CIPAC method exists for the analysis of the safener mefenpyr-diethyl in formulations (CIPAC 651.229).

### Methods for the determination of residues (KCP 5.1.2)

Iodosulfuron-methyl-sodium

An overview on the acceptable methods and possible data gaps for analysis of residues of iodosulfuron-methyl-sodium for the generation of pre-authorization data is given in the following table.

Table 5.2-1:	Validated methods for the generation of pre-authorization data

| Component of residue definition: iodosulfuron-methyl and its salts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Wheat grain (Residues) | Primary Method AL008/96-0 | 0.01 mg/kg | HPLC-UV | Wrede A., 1997 DAR, Germany, 2000; EU agreed |
|  | Amendment Method AL008/96-0 | 0.01 mg/kg | HPLC-UV | Wrede A., 1998 DAR, Germany, 2000; EU agreed |
|  | Confirmatory Method AL008/96-0 | 0.01 mg/kg | HPLC-UV | Wrede A., 1998 DAR, Germany, 2000; EU agreed |
|  | ILV Method AL 008/96-0 | 0.01 mg/kg | HPLC-UV | Taylor N., 1998 DAR, Germany, 2000; EU agreed |
| Wheat straw (Residues) | Primary Method AL121/96-0 | 0.05 mg/kg | HPLC-UV | Wrede A., 1999 DAR, Germany, 2000; EU agreed |
| Wheat shoot (Residues) | Primary Method AL120/96-0 | 0.05 mg/kg | HPLC-UV | Wrede A., 1997 DAR, Germany, 2000; EU agreed |
| Wheat shoot and straw (Residues) | Radiovalidation of methods AL 120/96-0 (shoot) and AL121/96-0 (straw) | 0.05 mg/kg | HPLC-UV | Wrede A., 1998 DAR, Germany, 2000; EU agreed |
| Cereal grain Cereal shoot Cereal straw | Primary Method EM F08/99 renamed 00815 | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg | LC-MS/MS | Wrede A., 2000 DAR, Germany, 2000; EU agreed |
| Cereal grain Cereal shoot Cereal straw | Validation Method EM F08/99-0 | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg | LC-MS/MS | Wrede A., 2000 DAR, Germany, 2000; EU agreed |
| Lemon Tomato Maize kernel | Validation of EM Method F08/99-0 | 0.01 mg/kg 0.01 mg/kg 0.01 mg/kg | LC-MS/MS | Wrede A., 2002 RAR, Sweden, 2015; EU agreed |
| Wheat grain | ILV Method EM F08/99-0 | 0.01 mg/kg | LC-MS/MS | Ansprach T., 2001 RAR, Sweden, 2015; EU agreed |
| Tomato Citrus | ILV Method EM F08/99-0 | 0.01 mg/kg | LC-MS/MS | Reichert N., Klimmek S., 2002 RAR, Sweden, 2015; EU agreed |
| Cereal grain Cereal shoot Cereal straw Flax (grain, pomace, wet, oil) Flax (plant without root) | Primary Method 00815/M001 (modification of EM F08/99-0) | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Heinemann, O., 2004 RAR Dec. 2015; EU agreed |
| Cereal grain Cereal shoot Cereal straw | Primary Method 00815/M001 Additional validation in the residue study 13-2127 | 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Stuke, S., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
|  | Primary Method 00815/M001 Additional validation in the residue study 13-2129 | 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Stuke, S., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Cereal green material, Cereal grain, Cereal straw | Primary Method 00815/M001 Additional validation in the residue study 16-2030 (barley only) | 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Mahlo, C.; Gabriel, E.; Vagt, I.; Meyer, M., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Wheat grain Wheat green material Wheat straw Rape seed | Primary method 01376/M002 | 0.01 mg/kg | LC-MS/MS (2 MRM transitions validated) | Kaussmann M., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Wheat grain Wheat green material Wheat straw Barley grain Barley green material Barley straw | Primary method 01514 | 0.01 mg/kg (iodosulfuron-methyl-sodium metabolites: AE F059411 and AE 0031838) | LC-MS/MS (2MRM transitions validated) | Kaussmann M., 2017 Not peer reviewed Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Soil (Environmental fate) | Primary Method 01115 | 0.01 µg/kg | HPLC-MS/MS | Freitag T., Wolters A., 2013 RAR, Sweden, 2015 EU agreed |
|  | Primary Report BY98R004 | 0.0005 ppm | HPLC-MSD GC-NPD | Norris F., 2000 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Water (Environmental fate) | Primary Method 01387 | Iodosulfuron-methyl-sodium: 0.05 µg/L Metsulfuron-methyl: 0.05 µg/L | HPLC-MS/MS | Krebber R., Braune M., 2013 RAR, Sweden, 2015 EU agreed |
|  | ILV Method 01387 | Iodosulfuron-methyl-sodium: 0.05 µg/L Metsulfuron-methyl: 0.05 µg/L | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, Sweden, 2015 EU agreed |
| Sediment (Environmental fate) | Primary Method 01115 | 0.01 mg/kg | HPLC-MS/MS | Freitag T., Wolters A., 2013 RAR, Sweden, 2015 EU agreed |
|  | Primary Report BY98R004 | 0.0005 ppm | HPLC-MSD GC-NPD | Norris F., 2000 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Soil, water, … (Efficacy) | Not relevant |  |  |  |
| Air (Exposure) | Primary Report RESID/98/27 | 1.05 µg/m³ | GC-ECD | Everitt, S. L., 1998 Monograph (22 May 2000); EU agreed |
|  | Primary Report IF-100/21283-00 | 1.6 µg/m³ | LC-UV | Reichter, N., 2000, amended by Zietz, E., 2009 Addendum to Monograph (18 June 2001); EU agreed |
| Soil (Ecotoxicology) | Primary Method 01115 | 0.01 mg/kg | HPLC-MS/MS | Freitag T., Wolters A., 2013 RAR, Sweden, 2015 EU agreed |
| Water (Ecotoxicology) | Primary Method 01387 | Iodosulfuron-methyl-sodium: 0.05 µg/L Metsulfuron-methyl: 0.05 µg/L | HPLC-MS/MS | Krebber R., Braune M., 2013 RAR, Sweden, 2015 EU agreed |
|  | ILV Method 01387 | Iodosulfuron-methyl-sodium: 0.05 µg/L Metsulfuron-methyl: 0.05 µg/L | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, Sweden, 2015 EU agreed |
|  | Primary | Iodosulfuron-methyl-sodium: 0.00184 µg/L | HPLC-MS/MS | Schuler, L., 2022, study no. S22-01264 See Appendix 2 |
| 50% (w/v) aqueous sucrose solution (Ecotoxicology) | Primary | Iodosulfuron-methyl-sodium: 0.0510 mg/kg | HPLC-MS/MS | Vergé, E., 2022, study no. S22-01252 See Appendix 2 |
| Feeding solution (Ecotoxicology) | Primary | Iodosulfuron-methyl-sodium: 0.0102 mg/kg | HPLC-MS/MS | Vergé, E., 2023, study no. S22-01253 See Appendix 2 |
| Tap water (Ecotoxicology) | Primary | Iodosulfuron-methyl-sodium: 0.6 mg/L | HPLC-MS/MS | Ripperger, D., 2022, study no. S22-01280 See Appendix 2 |
| Water, buffer solutions, … (Properties) | Not relevant |  |  |  |

Mesosulfuron-methyl

An overview on the acceptable methods and possible data gaps for analysis of residues of mesosulfuron-methyl for the generation of pre-authorization data is given in the following table.

Table 5.2-2:	Validated methods for the generation of pre-authorization data

| Component of residue definition: mesosulfuron-methyl |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Wheat grain Wheat straw Wheat shoot (Residues) | Primary DGM F02/99-0 | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg | LC-MS/MS | Wrede, A., 1999 RAR, France, 2016; EU agreed |
| Cereal grain Cereal shoot Cereal straw | Primary EM F08/99 renamed 00815 | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg | LC-MS/MS | Wrede, A., 2000 RAR, France, 2016 ; EU agreed |
| Cereal grain Cereal shoot Cereal straw | Validation of EM F08/99-0 renamed 00815 | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg | LC-MS/MS | Wrede, A., 2000 RAR, France, 2016 ; EU agreed |
| Lemon Tomato Maize kernel | Validation of EM F08/99-0 renamed 00815 | 0.01 mg/kg | LC-MS/MS | Wrede, A., 2002 RAR, France, 2016; EU agreed |
| Wheat grain | ILV of EM F08/99-0 renamed 00815 | 0.01 mg/kg | LC-MS/MS | Reichert, N., 2000 RAR, France, 2016; EU agreed |
| Tomato Citrus | ILV of EM F08/99-0 renamed 00815 | 0.01 mg/kg | LC-MS/MS | Reichert, N., Klimmek, S., 2002 RAR, France, 2016; EU agreed |
| Cereal grain Cereal shoot Cereal straw Flax (grain, pomace, wet, oil) Flax (plant without root) | Primary Method 00815/M001 (modification of EM F08/99-0) | 0.01 mg/kg 0.05 mg/kg 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Heinemann, O., 2004 RAR, France, 2016; EU agreed |
| Cereal green material Cereal grain Cereal straw | Primary Method 00815/M001 Additional validation in the residue study 13-2127 | 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Stuke, S., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
|  | Primary Method 00815/M001 Additional validation in the residue study 13-2129 | 0.05 mg/kg 0.01 mg/kg 0.05 mg/kg | LC-MS/MS | Stuke, S., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Soil (Environmental fate) | Primary Method 01115 | 0.1 µg/kg | HPLC-MS/MS | Freitag T., Wolters A., 2013 RAR, France, 2016; EU agreed |
| Soil, water,… (Efficacy) | Not relevant |  |  |  |
| Air (Exposure) | Primary | 12 µg/m³ | LC-UV | Reichter, N., 2000, amended by Zietz, E., 2009 Addendum to Monograph (18 June 2001) EU agreed |
| Water (Ecotoxicology) | Primary Method 01542 | BCS-CV14885 0.05 µg/L | HPLC-MS/MS | Krebber, R., Ruttmann, F., 2017 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Water (Ecotoxicology) | Primary Method 01387 | 0.05 µg/L | HPLC-MS/MS | Krebber, R., Braune, M., 2013 RAR, France, 2016; EU agreed |
|  | ILV Method 01387 | 0.05 µg/L | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, France, 2016; EU agreed |
|  | Primary | 0.0102 µg/L | HPLC-MS/MS | Schuler, L., 2022, study no. S22-01264 See Appendix 2 |
| 50% (w/v) aqueous sucrose solution (Ecotoxicology) | Primary | 0.268 mg/kg | HPLC-MS/MS | Vergé, E., 2022, study no. S22-01252 See Appendix 2 |
| Feeding solution (Ecotoxicology) | Primary | 0.0535 mg/kg | HPLC-MS/MS | Vergé, E., 2023, study no. S22-01253 See Appendix 2 |
| Tap water (Ecotoxicology) | Primary | 3.2 mg/L | HPLC-MS/MS | Ripperger, D., 2022, study no. S22-01280 See Appendix 2 |

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of iodosulfuron-methyl-sodium (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft (Renewal) Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-3:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl | 0.01* mg/kg / LOQ | Reg. (EU) 289/2014 Reg. (EU) 2024/1077 |
| Plant, high acid content |  | 0.01* mg/kg / LOQ | Reg. (EU) 289/2014 Reg. (EU) 2024/1077 |
| Plant, high protein/high starch content (dry commodities) |  | 0.01* mg/kg / LOQ | EFSA Journal, 2012; 10(11):2974 Reg. (EU) 289/2014 Reg. (EU) 2024/1077 |
| Plant, high oil content |  | 0.02* mg/kg / LOQ | Reg. (EU) 289/2014 Reg. (EU) 2024/1077 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.05* mg/kg / LOQ | Reg. (EU) 289/2014 Reg. (EU) 2024/1077 |
| Muscle | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl | 0.02* mg/kg / LOQ | Reg. (EU) 289/2014 Reg. (EU) 2024/1077 |
| Milk |  |  |  |
| Eggs |  |  |  |
| Fat |  |  |  |
| Liver, kidney |  |  |  |
| Soil (Ecotoxicology) | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl-sodium | 0.01 µg/kg / LOQ | EFSA Conclusion, EFSA Journal 2016;14(4):4453 |
| Drinking water (Human toxicology) | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl-sodium and metsulfuron-methyl (AE F075736) | 0.05 µg/L / LOQ (both analytes) | general limit for drinking water: 0.1 µg/L |
| Surface water (Ecotoxicology) | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl-sodium and metsulfuron-methyl (AE F075736) | 0.05 µg/L / LOQ (both analytes) | EFSA Conclusion, EFSA Journal 2016;14(4):4453 |
| Air | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl-sodium | 1.05 µg/m3 / LOQ 1.6 µg/m3 / LOQ | EFSA Conclusion, EFSA Journal 2016;14(4):4453 |
| Body fluids | Sum of iodosulfuron-methyl and its salts, expressed as iodosulfuron-methyl | 0.05 mg/L | EFSA Conclusion, EFSA Journal 2016;14(4):4453 |
| Tissue (meat or liver) |  | 0.1 mg/kg | EFSA Conclusion, EFSA Journal 2016;14(4):4453 |

* indicates lower limit of analytical quantification

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of iodosulfuron-methyl-sodium in plant matrices is given in the following tables.

Table 5.3-4:	Validated methods for food and feed of plant origin

| Component of residue definition: iodosulfuron-methyl-sodium |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content (sugar beet leaf) High acid content (lemon fruit) High oil content (oilseed rape) High protein/high starch content (dry) (wheat straw/sugar beet body) | Primary Method 01360 | 0.01 mg/kg | LC-MS/MS (2 MRM validated) | Stuke, S., Ballmann, C., 2013; EU agreed, EFSA, 2016 |
|  | ILV Method 01360 | 0.01 mg/kg | LC-MS/MS (2 MRM validated) | Konrad, S., 2013; EU agreed, EFSA, 2016 |
|  | Confirmatory is not required. The primary method was confirmed by validation of two different mass transitions. |  |  |  |
| High starch content (dry) (wheat grain) | Primary Method 01360/M001 | 0.01 mg/kg | LC-MS/MS (2 MRM validated) | Stuke, S., 2015; Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
|  | ILV – not required as available for the 4 other matrix types |  |  |  |
|  | Confirmatory method is not required. The primary method was confirmed by validation of two different mass transitions. |  |  |  |

For any special comments or remarkable points concerning the analytical methods for the determination of residues in plant matrices, please refer to Appendix 2.

Table 5.3-5:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Available from: | Stuke, S., 2015; RAR, Sweden, 2015; EU agreed |
| Not required, because: | As residues are not expected to be ≥ LOQ in cereal grain, extraction efficiency is not required. |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods for analysis of iodosulfuron-methyl-sodium in animal matrices is given in the following tables. In matrices of animal origin, no residue definition was proposed during the Annex I Renewal since no residues are anticipated. Although not required for iodosulfuron-methyl-sodium by the current EU regulations, some analytical methods for matrices of animal origin were developed for sulfonylureas.

Table 5.3-6:	Validated methods for food and feed of animal origin (if appropriate)

| Component of residue definition: iodosulfuron-methyl-sodium* |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk, eggs, muscle, fat, kidney, liver | Primary Method 01208/M001 | 0.01 mg/kg | LC-MS/MS (2 MRM validated) | Schmeer, K.; Philipowski, C., 2011; Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
|  | ILV Method 01208/M001 | 0.01 mg/kg | LC-MS/MS (2 MRM validated) | Netzband, D., 2015; Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |

* refers to the analyte relevant to the method as no residue definition is proposed for matrices of animal origin.

For any special comments or remarkable points concerning the analytical methods for the determination of residues in animal matrices, please refer to Appendix 2.

Table 5.3-7:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Methods for producs of animal origin are not required (no residue definition). Residues are not expected to be ≥ LOQ in animal matrices, so extraction efficiency is not required. |

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of iodosulfuron-methyl-sodium in soil is given in the following tables.

Table 5.3-8:	Validated methods for soil

| Component of residue definition: iodosulfuron-methyl-sodium |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.01 µg/kg | HPLC-MS/MS | Freitag, T., Wolters, A., 2013 RAR, Sweden, 2015; EU agreed |
| Confirmatory | 0.01 µg/kg | HPLC-MS/MS |  |

For any special comments or remarkable points concerning the analytical methods for soil please refer to Appendix 2.

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of iodosulfuron-methyl-sodium and its metabolite metsulfuron-methyl (AE F075736) in surface and drinking water is given in the following tables.

Table 5.3-9:	Validated methods for water

| Component of residue definition: iodosulfuron-methyl-sodium (and metabolite metsulfuron-methyl (AE F075736) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary Method 01387 | 0.05 μg/L (for both substances) | HPLC-MS/MS | Krebber, R., Braune, M., 2013 RAR, Sweden, 2015; EU agreed |
|  | ILV Method 01387 | 0.05 μg/L (for both substances) | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, Sweden, 2015; EU agreed |
|  | Confirmatory | Provided by a fully validated second ion transition in the study by Krebber & Braune, 2013. |  |  |
| Surface water | Primary Method 01387 | 0.05 μg/L (for both substances) | HPLC-MS/MS | Krebber, R., Braune, M., 2013 RAR, Sweden, 2015; EU agreed |
|  | ILV Method 01387 | 0.05 μg/L (for both substances) | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, Sweden, 2015; EU agreed |
|  | Confirmatory | Provided by a fully validated second ion transition in the study by Krebber & Braune, 2013. |  |  |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of iodosulfuron-methyl-sodium in air is given in the following tables.

Table 5.3-10:	Validated methods for air

| Component of residue definition: iodosulfuron-methyl-sodium |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 1.05 μg/m3 | GC-ECD | Everitt, S.L., 1998 Monograph (22 May 2000); EU agreed |
| Confirmatory | 1.6 μg/m3 | LC-UV | Reichert, N., 2000 Addendum to Monograph (18 June 2001); EU agreed |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of iodosulfuron-methyl-sodium in body fluids and tissues is given in the following table.

Table 5.3-11:	Methods for body fluids and tissues

| Component of residue definition: iodosulfuron-methyl |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Body tissues (liver, kidney, meat) |  |  |  |
| Primary Method 01208/M001 | 0.01 mg/kg | LC-MS/MS | Schmeer, K.; Philipowski, C., 2011 Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Confirmatory | Provided by a fully validated second ion transition in the study by Schmeer, K.; Philipowski, C.,2011. |  |  |
| Body fluids (plasma) |  |  |  |
| Primary Method 01478 | 50 µg/L | HPLC-MS/MS (2 transitions) | Kaussmann, M.; 2016; Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Confirmatory | Provided by a fully validated second ion transition in the study by Kaussmann, M., 2016. |  |  |

#### Other studies/ information

None.

### Description of analytical methods for the determination of residues of mesosulfuron-methyl (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft (Renewal) Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-12:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Mesosulfuron-methyl | 0.01* mg/kg / LOQ | Reg. (EU) 289/2014 |
| Plant, high acid content |  | 0.01* mg/kg / LOQ | Reg. (EU) 289/2014 |
| Plant, high protein/high starch content (dry commodities) |  | 0.01* mg/kg / LOQ | EFSA Journal, 2012; 10(11):2974 Reg. (EU) 289/2014 |
| Plant, high oil content |  | 0.02* mg/kg / LOQ | Reg. (EU) 289/2014 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.05* mg/kg / LOQ | Reg. (EU) 289/2014 |
| Muscle | Mesosulfuron-methyl | 0.02* mg/kg / LOQ | Reg. (EU) 289/2014 |
| Milk |  |  |  |
| Eggs |  |  |  |
| Fat |  |  |  |
| Liver, kidney |  |  |  |
| Soil (Ecotoxicology) | Mesosulfuron-methyl | 0.1 µg/kg / LOQ | EFSA Conclusion, EFSA Journal 2016;14(10):4584 |
| Drinking water (Human toxicology) | Mesosulfuron-methyl | 0.05 µg/L / LOQ | EFSA Conclusion, EFSA Journal 2016;14(10):4584 |
| Surface water (Ecotoxicology) | Mesosulfuron-methyl | 0.05 µg/L / LOQ | EFSA Conclusion, EFSA Journal 2016;14(10):4584 |
| Air | Mesosulfuron-methyl | 12 µg/m3 / LOQ | EFSA Conclusion, EFSA Journal 2016;14(10):4584 |
| Body fluids | Mesosulfuron-methyl | 0.05 mg/L | EFSA Conclusion, EFSA Journal 2016;14(10):4584 |
| Tissue (meat or liver) |  | 0.1 mg/kg | EFSA Conclusion, EFSA Journal 2016;14(10):4584 |

* indicates lower limit of analytical quantification

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of mesosulfuron-methyl in plant matrices is given in the following tables.

Table 5.3-13:	Validated methods for food and feed of plant origin

| Component of residue definition: mesosulfuron-methyl |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content High acid content High oil content High protein/high starch content (dry): straw | Primary EM 01360 | 0.01 mg/kg | LC-MS/MS | Stuke, S., Ballmann, C., 2013; RAR, France, 2016; EU agreed |
|  | ILV Method 01360 | 0.01 mg/kg | LC-MS/MS | Konrad, S., 2013; RAR, France, 2016; EU agreed |
| Dry commodities: cereal grain | Primary EM 01360/M001 (MR-15/090) | 0.01 mg/kg | LC-MS/MS | Stuke, S., 2015; Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |

Table 5.3-14:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Available from: | Stuke, S., 2015, RAR, France, 2016, EU agreed |
| Not required, because: | As residues are not expected to be ≥ LOQ in cereal grain, extraction efficiency is not required. |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods for analysis of mesosulfuron-methyl in animal matrices is given in the following tables. In matrices of animal origin, no residue definition was proposed during the Annex I Renewal since no residues are anticipated. Although not required for mesosulfuron-methyl by the current EU regulations, some analytical methods for matrices of animal origin were developed for sulfonylureas.

Table 5.3-15:	Validated methods for food and feed of animal origin

| Component of residue definition: mesosulfuron-methyl |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk, eggs, muscle, fat, kidney, liver | Primary Method 01208/M001 | 0.01 mg/kg | LC-MS/MS | Schmeer, K.; Philipowski, C., 2011 RAR, France, 2016; EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Netzband, D., 2015; RAR, France, 2016; EU agreed |
|  | Confirmatory (if required) | Provided by a fully validated second ion transition in the study by Schmeer, K.; Philipowski, C., 2011. |  |  |

Table 5.3-16:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Methods for products of animal origin are not required (no residue definition). Residues are not expected to be ≥ LOQ in animal matrices, so extraction efficiency is not required. |

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of mesosulfuron-methyl in soil is given in the following tables.

Table 5.3-17:	Validated methods for soil

| Component of residue definition: mesosulfuron-methyl |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary Method 01115 | 0.01 µg/kg | HPLC-MS/MS | Freitag, T., Wolters, A., 2013 RAR, France, 2016; EU agreed |
| Confirmatory |  | HPLC-MS/MS |  |

For any special comments or remarkable points concerning the analytical methods for soil please refer to Appendix 2.

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of mesosulfuron-methyl in surface and drinking water is given in the following tables.

Table 5.3-18:	Validated methods for water

| Component of residue definition: mesosulfuron-methyl |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary Method 01387 | 0.05 μg/L | HPLC-MS/MS | Krebber, R., Braune, M., 2013 RAR, France, 2016; EU agreed |
|  | ILV | 0.05 μg/L | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, France, 2016; EU agreed |
|  | Confirmatory | Provided by a fully validated second ion transition in the study by Krebber & Braune, 2013. |  |  |
| Surface water | Primary | 0.05 μg/L | HPLC-MS/MS | Krebber, R., Braune, M., 2013 RAR, France, 2016; EU agreed |
|  | ILV | 0.05 μg/L | DI-HPLC-MS/MS | Stanislowski, 2013 RAR, France, 2016; EU agreed |
|  | Confirmatory | Provided by a fully validated second ion transition in the study by Krebber & Braune, 2013. |  |  |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of mesosulfuron-methyl in air is given in the following tables.

Table 5.3-19:	Validated methods for air

| Component of residue definition: mesosulfuron-methyl |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 12 μg/m3 | HPLC-UV | Reichert, N., 2000 RAR, France, 2016; EU agreed |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of mesosulfuron-methyl in body fluids and tissues is given in the following table.

Table 5.3-20:	Methods for body fluids and tissues

| Component of residue definition: mesosulfuron-methyl |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Body tissues (liver, kidney, meat) |  |  |  |
| Primary Method 01208/M001 | 0.01 mg/kg | LC-MS/MS | Schmeer, K.; Philipowski, C., 2011 RAR, France, 2016; EU agreed |
| Confirmatory | Provided by a fully validated second ion transition in the study by Schmeer, K.; Philipowski, C., 2011. |  |  |
| Body fluids (plasma) |  |  |  |
| Primary Method 01478 | 50 µg/L | HPLC-MS/MS (2 transitions) | Kaussmann, M.; 2016; Submitted and accepted in support of the renewal of Atlantis 12 OD. Reference is made to the Registration Report of Atlantis 12 OD. |
| Confirmatory | Provided by a fully validated second ion transition in the study by Kaussmann, 2016. |  |  |

#### Other studies/ information

None.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1/01 | Mattioli B. | 2024 | IMS+MSM+MPR 2+10+30 OD: Validation of the analytical method for the determination of the active ingredient content CH-0227-2024 LabAnalysis Life Science Srl GLP Not published | N | Certiplant BV |
| KCP 5.1.2/01 (also submitted as KCP 10.2.1/01) | Schuler, L. | 2022 | IMS+MSM+MPR 2+10+30 OD: Toxicity to the Duckweed Lemna gibba under laboratory conditions (semi-static test design). S22-01264 Eurofins Agroscience services GLP Not published | N | Certiplant BV |
| KCP 5.1.2/02 (also submitted as KCP 10.3.1.2/01) | Vergé, E. | 2022 | IMS+MSM+MPR 2+10+30 OD: Honey bee Apis mellifera L. Chronic oral toxicity test 10 day feeding in the laboratory S22-01252 Eurofins Agroscience services GLP Not published | N | Certiplant BV |
| KCP 5.1.2/03 (also submitted as KCP 10.3.1.3/01) | Vergé, E. | 2023 | IMS+MSM+MPR 2+10+30 OD: Honey bee Apis mellifera 22 day larval toxicity test (repeated exposure) S22-01253 GLP Not published | N | Certiplant BV |
| KCP 5.1.2/04 (also submitted as KCP 10.6.2/01) | Ripperger, D. | 2022 | IMS+MSM+MPR 2+10+30 OD: Effects on the vegetative vigour of non-target terrestrial plant species under greenhouse conditions. S22-01280 Eurofins Agroscience services GLP Not published | N | Certiplant BV |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| Mesosulfuron-methyl |  |  |  |  |  |
| KCA 4.1.2 /01 | Wrede, A. | 1999 | Data generation method and validation for cereal by LC-MS/MS Code: AE F130060 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C005129, Report includes Trial Nos.: CR99 /003 Edition Number: M-191437-01-1 Date: 1999-10- 20 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /02 | Wrede, A. | 2000 | Data generation method for soil by LC-MS/MS Amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, AE F130060, AE F130360 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C008681, Edition Number: M-197688-01-1 Date: 2000-06- 28 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /03 | Wrede, A. | 2000 | Validation of the method DGM F04/99-0 in soil by LC-MS/MS - Amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, AE F130060, AE F130360 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C008682, Edition Number: M-197689-01-1 Date: 2000-06-28 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /04 | Reichert, N. | 2000 | Development and validation of an analytical method for the determination of AE F130060 in air Institut Fresenius Chem.und Biolog. Lab. AG, Taunusstein, Germany Bayer CropScience, Report No.: C009587, Edition Number: M-198860-01-1 Date: 2000-10- 02 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /05 | Lecointre, B. | 2016 | Mesosulfuron-methyl - Request for additional information - Response document to answer 12 (analytical methods) Bayer CropScience Bayer CropScience, Report No.: M-549830-01-1, Edition Number: M-549830-01-1 Date: 2016-03- 15 GLP/GEP: n.a., unpublished | N | Bayer CropScience |
| KCA 4.2 /01 | Wrede, A. | 2000 | Enforcement Method for Cereal Grain, Straw and Shoot by LC-MS/MS Amidosulfuron (AE F075032) Metsulfuron-methyl (AE F075736) Iodosulfuron-methyl-sodium (AE F115008) AE F130060 AE F130360 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006734, Edition Number: M-194528-01-1 Date: 2000-03- 02 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /02 | Wrede, A. | 2000 | Validation of the Enforcement Method EM F08/99-0 of cereal grain, straw and shoot by LC-MS/MS - Amidosulfuron (AE F075032) – Metsulfuron-methyl (AE F075736) Iodosulfuron-methyl-sodium (AE F115008) - AE F130060 - AE F 130360 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006735, Edition Number: M-194531-01-1 Date: 2000-03- 02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /03 | Wrede, A. | 2000 | Validation of the enforcement method EM F08/99-0 in cereal grain, straw and shoot by LC-MS/MS - Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C008827, Edition Number: M-197830-01-1 Date: 2000-07- 05 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /04 | Reichert, N. | 2000 | Independent laboratory validation of the method of analysis EM F08/99-0 for the determination of AE F130060 in cereal (grain) Institut Fresenius Chem.und Biolog. Lab. AG, Taunusstein, Germany Bayer CropScience, Report No.: C009586, Edition Number: M-198857-01-1 Date: 2000-10- 12 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /05 | Wrede, A. | 2000 | Enforcement method and validation for cereal grain by HPLC-UV Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C009496, Edition Number: M-198685-01-1 Date: 2000-09- 08 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /06 | Wrede, A. | 2000 | Multi-residue method for the determination of AE F130060 in cereal grain (statement) Mesosulfuron Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C009649, Edition Number: M-198985-01-1 Date: 2000-09- 14 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /07 | Wrede, A. | 2000 | Enforcement method and validation of soil by HPLC-UV Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C009151, Edition Number: M-198143-01-1 Date: 2000-09- 08 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /08 | Wrede, A. | 2000 | Enforcement Method for Soil by LC-MS/MS Metsulfuron-methyl (AE F075736) Iodosulfuron-methyl-sodium (AE F115008) Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006394, Edition Number: M-193807-01-1 EPA MRID No.: 45108502 Date: 2000-02- 04 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /09 | Wrede, A. | 2000 | Validation of the enforcement method EM F13/99-0 of soil by LC-MS/MS - Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C009563, Edition Number: M-198810-01-1 Date: 2000-10- 06 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /10 | Wrede, A. | 2000 | Enforcement method for surface and drinking water by HPLC-UV Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C008689, Edition Number: M-197696-01-1 Date: 2000-06- 16 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /11 | Wrede, A. | 2000 | Validation of the enforcement method EM F15/99-0 for surface and drinking water by HPLC-UV - Code: AE F130060 Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C008686, Edition Number: M-197693-01-1 Date: 2000-06- 16 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /12 | Wrede, A. | 2000 | Validation of the enforcement method EM F13/99-0 in soil by LC-MS/MS – Metsulfuron-methyl – Iodosulfuron-methyl-sodium -Code: AE F075736, AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006396, Edition Number: M-193814-01-1 EPA MRID No.: 45108720 Date: 2000-02- 04 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /13 | Wrede, A. | 2001 | Enforcement method for surface and drinking water by LC-MS/MS Amidosulfuron (AE F075032) Metsulfuron-methyl (AE F075736) lodosulfuron-methyl-sodium (AE F115008) Mesosulfuron-methyl (AE F130060) Foramsulfuron (AE F130360) Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C011206, Edition Number: M-200404-01-1 Date: 2001-02- 14 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /14 | Wrede, A.; Neuss, B. | 2001 | Validation of the enforcement method EM F04/00-0 for surface and drinking water by LC-MS/MS-Amidosulfuron (AE 075032) – Metsulfuron-methyl (AE F075736)- lodosulfuron-methyl-sodium (AE F115008)- Mesosulfuron-methyl (AE F130060)- Foramsulfuron (AE F130360) Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C011207, Edition Number: M-200406-01-1 Date: 2001-02- 14 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /15 | Wrede, A. | 2002 | Validation of the enforcement method EM F08/99-0 for lemon, tomato and maize kernel by LC-MS/MS - Amidosulfuron (AE F075032) – Iodosulfuron-methyl-sodium (AE F115008) – Mesosulfuron-methyl (AE F130060) - Foramsulfuron (AE F130360) Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C022220, Edition Number: M-212674-01-1 Date: 2002-04- 30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /16 | Heinemann, O. | 2004 | Modification M001 to method 00815 for the determination of residues of amidosulfuron, iodosulfuron-methyl-sodium including metabolite metsulfuron-methyl, foramsulfuron and mesosulfuron-methyl in/on flax and wheat matrices by HPLC-MS/MS Bayer CropScience, Report No.: 00815/M001, Edition Number: M-226888-01-1 Date: 2004-01- 30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /17 | Reichert, N. | 2001 | Independent laboratory validation of the method of analysis EM F08/99-0 for the determination of AE F130060 in cereal (plant and straw) Institut Fresenius Chem.und Biolog. Lab. AG, Taunusstein, Germany Bayer CropScience, Report No.: C011938, Edition Number: M-201813-01-1 Date: 2001-03- 13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /18 | Reichert, N.; Klimmek, S. | 2002 | Independent laboratory validation of the analytical method EM F08/99-0 for the residue analysis of Amidosulfuron (AE F075032), Iodosulfuron-methyl-sodium (AE F115008), Mesosulfuron-methyl (AE F130060), Foramsulfuron (AE F130360) in tomato a Institut Fresenius Chem.und Biolog. Lab. AG, Taunusstein, Germany Bayer CropScience, Report No.: C023679, Edition Number: M-215456-01-1 Date: 2002-06- 06 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /19 | Stuke, S.; Ballmann, C. | 2013 | Analytical method 01360 for the determination of amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, mesosulfuron-methyl, and foramsulfuron in samples from plant origin by HPLC-MS/MS Bayer CropScience, Report No.: MR-13/007, Edition Number: M-455564-01-1 Date: 2013-05- 28 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /20 | Konrad, S. | 2013 | Independent lab validation of BCS method 01360 for the determination of residues of amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, mesosulfuron-methyl and foramsulfuron in samples from plant origin by HPLC-MS/MS Currenta GmbH & Co. OHG, Leverkusen, Germany Bayer CropScience, Report No.: 2013/0060/01, Edition Number: M-470160-01-1 Date: 2013-10- 18 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /21 | Freitag, T. | 2008 | Amendment no. 0001 to report no.: MR-08/138 - Analytical Method 01115 for the determination of residues of amidosulfuron, iodosulfuron-methyl-sodium, metsulfuron-methyl, mesosulfuron-methyl and foramsulfuron in soil by HPLC-MS/MS Bayer CropScience, Report No.: M-310074-03-1, Edition Number: M-310074-03-1 Date: 2008-10- 27 ...Amended: 2013-08-08 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /22 | Krebber, R.; Braune, M. | 2013 | Analytical method 01387 for the determination of various pesticides in drinking and surface water by HPLC-MS/MS Bayer CropScience, Report No.: MR-13/085, Edition Number: M-466732-01-1 Date: 2013-10- 09 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /23 | Stanislowski, T. | 2013 | Independent laboratory validation of BCS analytical methods 01333 and 01387 for determination of various pesticides in surface water by Di-HPLC-MS/MS PTRL Europe, Ulm, Germany Bayer CropScience, Report No.: P3117 G, Edition Number: M-470714-02-1 Date: 2013-12- 13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /24 | Wrede, A | 2001 | Aged residue in wheat straw and shoot - Radio validation of the residue analytical method EM F 08/99-0 – Mesosulfuron-methyl-Code: AE F130060 Report C011980 Eidition number : M-201883-01 | N | Bayer CropScience |
| KCA 4.2 /25 | Stuke, S. | 2015 | Cross validation of enforcement method 01360 for the determination of sulfonylureas vs. extraction procedure applied in 14C-metabolism studies using incurred residues in plant matrices analysed by HPLC-MS/MS Bayer CropScience, Report No.: MR-15/036, Edition Number: M-525863-01-1 Date: 2015-06- 15 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /26 | Stuke, S. | 2015 | Modification 001 of analytical method 01360 for the determination of amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, mesosulfuron-methyl, and foramsulfuron in samples from plant origin by HPLC-MS/MS Bayer CropScience, Report No.: MR-15/090, Edition Number: M-537921-01-1 Date: 2015-10- 30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /27 | Netzband, D. | 2010 | Independent laboratory validation of an analytical method 01208/M001 for the determination of amidosulfuron (AE F075032), metsulfuron-methyl (AE F075736), iodosulfuron-methyl-sodium (AE F115008), mesosulfuron-methyl (AE F130060), foramsulfuron (AE F130360) in animal tissues (meat, fat, liver, kidney), egg, and milk by HPLC-MS/MS Bayer CropScience LP, Stilwell, KS, USA Bayer CropScience, Report No.: RAMML014-01, Edition Number: M-398300-02-1 Date: 2010-12- 22 ...Amended: 2015-11-03 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /28 | Schmeer, K.; Philipowski, C. | 2010 | Modification M001 of the residue analytical method 01208 for the determination of amidosulfuron (AE F075032), metsulfuron-methyl (AE F075736), iodosulfuron-methyl-sodium (AE F115008), mesosulfuron-methyl (AE F130060), foramsulfuron (AE F130360) in animal tissues (meat, fat, liver, kidney), egg, and milk by HPLC-MS/MS Bayer CropScience, Report No.: 01208/M001, Edition Number: M-389788-03-1 Date: 2010-09- 06 ...Amended: 2011-01-03 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 8.2.7 | E. Bruns | 2009 | Lemna gibba G3 – Prolonged growth inhibition test with mesosulfuron-methyl (AE F130060) with stepwise decreasing concentrations over an 8 week test duration Bayer CropScience, Report No.: EBMML017, Edition Number: M-445139-01-1 Date: 2013-01- 09 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| / | / | / | Analytical method for the determination of AE F130060 in dog diet by High Performance liquid chromatography (HPLC) Report 98.0289 Document M-147837-01-1 GLP | N | Bayer CropScience |
| KCA 5.3_01 | B A Mallyon | 2000 | Analytical method for the determination of test material in diet by HPLC RESID/93/34 M-198511-01-1 GLP | N | Bayer CropScience |
| / | / | / | Hoe 130060 Determination of concentrations in dietary formulations by high performance liquid chromatography Report 96.0191 M-143016-01-1 GLP | N | Bayer CropScience |
| KCA 5.6.2_04 | Hofmann Th | 2000 | AE F130060; substance technical Code: AE F130060 00 1C95 0001 Rabbit oral developmental toxicity (teratogenicity) study Report 98.0254 M-181336-02-1 GLP | N | Bayer CropScience |
| Iodosulfuron-methyl-sodium |  |  |  |  |  |
| KCA 4.1.2 /01 | Wrede, A. | 1997 | Analytical method, validation Determination of residues in straw by HPLC Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: A59678, Report includes Trial Nos.: CR96/027 Edition Number: M-143342- 01-1 Date: 1997-12-02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /02 | Wrede, A. | 1997 | Analytical method and validation Determination of residues in shoot by HPLC Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: A59290, Report includes Trial Nos.: CR96/028 Edition Number: M-142974- 01-1 Date: 1997-09-04 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /03 | Wrede, A. | 1997 | Aged residue in wheat straw and shoot. Radio validation of the residue analytical method AL121/96-0 (straw) and AL120/96-0 (shoot) Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C001460, Edition Number: M-182685- 01-1 Date: 1998-11-09 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.1.2 /04 | Wrede, A. | 1997 | Aged residue of AE F115008 and its metabolite AE F075736 in soil. - Radio validation of the residue analytical method DGM F 06/97 - 0 - Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C001581, Edition Number: M-182982- 01-1 EPA MRID No.: 45108718 Date: 1998-11-26 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /01 | Wrede, A. | 1997 | Analytical method, validation Determination of residues in grain by HPLC Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: A58043, Report includes Trial Nos.: CR96/019 Edition Number: M-141767- 02-1 Date: 1997-04-28 ...Amended: 1998-06-29 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /02 | Wrede, A. | 1998 | Validation of the analytical method AL - 008/96 -0 for the determination of residues of AE F115008 in grain by HPLC with two different HPLC columns Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C001452, Edition Number: M-182662- 01-1 Date: 1998-11-05 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /03 | Taylor, N. W. | 1998 | Independent laboratory validation (ILV) of the Analytical Method (AL-008/96-0) for the determination of AE F115008 residues in cereal grain - Final Report AE F115008 analytical grade Code: AE F115008 AgrEvo UK Crop Protection Ltd., Chesterford Park, United Kingdom Bayer CropScience, Report No.: C000836, Report includes Trial Nos.: 200/07/001 Edition Number: M-181323- 01-1 Date: 1998-09-28 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /04 | Stan, H. J.; Schwarzer, F.; Brockmeyer, R. | 1992 | Practical application of methods for the analysis of sulfonylurea herbicides and their metabolites in foodstuffs Techn. Universitaet Berlin, DEU;Institut fuer Lebensmittelchemie Report No.: C005929, Edition Number: M-131964- 01-2 Date: 1992-04-30 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /05 | Wrede, A. | 1998 | Multi-residue method for the determination of AE F115008 in grain (statement) Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C001451, Edition Number: M-182660- 01-1 Date: 1998-11-06 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2 /06 | Wrede, A. | 1998 | Analytical method and validation for the determination of residues of AE F115008 and its metabolite AE F075736 in soil using HPLC (method and validation) Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: A67461, Report includes Trial Nos.: CR97/032 Edition Number: M-147842- 01-1 EPA MRID No.: 45108717 Date: 1998-07-14 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /06 | Wrede, A. | 2000 | Enforcement Method for Soil by LC-MS/MS Metsulfuron-methyl (AE F075736) Iodosulfuron-methyl-sodium (AE F115008) Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006394, Edition Number: M-193807- 01-1 EPA MRID No.: 45108502 Date: 2000-02-04 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2/08 | Wrede, A. | 2000 | Validation of the enforcement method EM F13/99-0 in soil by LC-MS/MS – Metsulfuron-methyl - Iodosulfuron-methyl-sodium - Code: AE F075736, AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006396, Edition Number: M-193814- 01-1 EPA MRID No.: 45108720 Date: 2000-02-04 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2/09 | Wrede, A. | 1998 | Analytical method and validation for the determination of residues of AE F115008 and its metabolite AE F075736 in water using HPLC Code: AE F115008 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C000710, Report includes Trial Nos.: CR98/002 Edition Number: M-181134- 01-1 EPA MRID No.: 45108721 Date: 1998-09-22 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2/10 | Wrede, A. | 2000 | Enforcement method of iodosulfuron-methyl-sodium and its metabolite metsulfuron-methyl in surface water by HPLC incl. validation. Extension of the enforcement method EM F 01/98-0 for iodosulfuron-methyl-sodium in drinking water to its metabolite metsulfuron-methyl incl. validation Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006395, Report includes Trial Nos.: CR99/029 Edition Number: M-193810- 01-1 EPA MRID No.: 45108722 Date: 2000-02-08 G LP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2/12 | Reichert, N. | 2000 | Development and validation of an analytical method for the determination of iodosulfuron methyl sodium in air Institut Fresenius Chem.und Biolog. Lab. AG, Taunusstein, Germany Bayer CropScience, Report No.: IF-100/21283-00, Edition Number: M-199299- 02-1 Date: 2000-11-22 ...Amended: 2009-06-19 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2/13 | Wrede, A. | 2000 | Enforcement Method for Cereal Grain, Straw and Shoot by LC-MS/MS Amidosulfuron (AE F075032) Metsulfuron-methyl (AE F075736) Iodosulfuron-methyl-sodium (AE F115008) AE F130060 AE F130360 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006734, Edition Number: M-194528- 01-1 Date: 2000-03-02 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 4.2/14 | Wrede, A. | 2000 | Validation of the Enforcement Method EM F08/99-0 of cereal grain, straw and shoot by LC-MS/MS - Amidosulfuron (AE F075032) – Metsulfuron-methyl (AE F075736) Iodosulfuron-methyl-sodium (AE F115008) - AE F130060 - AE F 130360 Hoechst Schering AgrEvo GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C006735, Edition Number: M-194531- 01-1 Date: 2000-03-02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2/15 | Wrede, A. | 2002 | Validation of the enforcement method EM F08/99-0 for lemon, tomato and maize kernel by LC-MS/MS - Amidosulfuron (AE F075032) - Iodosulfuron-methyl-sodium (AE F115008) – Mesosulfuron-methyl (AE F130060) - Foramsulfuron (AE F130360) Aventis CropScience GmbH, Frankfurt am Main, Germany Bayer CropScience, Report No.: C022220, Edition Number: M-212674- 01-1 Date: 2002-04-30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /16 | Heinemann, O. | 2004 | Modification M001 to method 00815 for the determination of residues of amidosulfuron, iodosulfuron-methyl-sodium including metabolite metsulfuron-methyl, foramsulfuron and mesosulfuron-methyl in/on flax and wheat matrices by HPLC-MS/MS Bayer CropScience, Report No.: 00815/M001, Report includes Trial Nos.: P602033000 Edition Number: M-226888- 01-1 Date: 2004-01-30 GLP/GEP: yes, unpublished ...also filed: KCA 6.1 /06 | N | Bayer CropScience |
| KCA 4.2/17 | Anspach, T. | 2001 | Independent laboratory validation of the enforcement method for the determination of residues of iodosulfuron-methyl-sodium (AE F115008) and amidosulfuron (AE F075032) in wheat (grain) Dr. Specht & Partner, Chemische Laboratorien GmbH, Germany Bayer CropScience, Report No.: C015636, Edition Number: M-200884- 01-1 Date: 2001-08-28 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /18 | Reichert, N.; Klimmek, S. | 2002 | Independent laboratory validation of the analytical method EM F08/99-0 for the residue analysis of Amidosulfuron (AE F075032), Iodosulfuron-methyl-sodium (AE F115008), Mesosulfuron-methyl (AE F130060), Foramsulfuron (AE F130360) in tomato and citrus Institut Fresenius Chem.und Biolog. Lab. AG, Taunusstein, Germany Bayer CropScience, Report No.: C023679, Edition Number: M-215456- 01-1 Date: 2002-06-06 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2 /19 | Stuke, S.; Ballmann, C. | 2013 | Analytical method 01360 for the determination of amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, mesosulfuron-methyl, and foramsulfuron in samples from plant origin by HPLC-MS/MS Bayer CropScience, Report No.: MR-13/007, Edition Number: M-455564- 01-1 Method Report No.: MR-13/007 Date: 2013-05-28 GLP/GEP: yes, unpublished ...also filed: KCA 6.1 /07 | N | Bayer CropScience |
| KCA 4.2 | Stuke, S. | 2015 | Cross validation of enforcement method 01360 for the determination of sulfonylureas vs. extraction procedure applied in 14C-metabolism studies using incurred residues in plant matrices analysed by HPLC-MS/MS | N | Bayer CropScience |
| KCA 4.2/20 | Konrad, S. | 2013 | Independent lab validation of BCS method 01360 for the determination of residues of amidosulfuron, metsulfuron-methyl, iodosulfuron-methyl-sodium, mesosulfuron-methyl and foramsulfuron in samples from plant origin by HPLC-MS/MS Currenta GmbH & Co. OHG, Leverkusen, Germany BCS, Report No.: 2013/0060/01, Edition Number: M-470160- 01-1 Date: 2013-10-18 GLP/GEP: yes, unpublished ...also filed: KCA 6.1 /08 | N | Bayer CropScience |
| KCA 4.2/22 | Krebber, R.; Braune, M. | 2013 | Analytical method 01387 for the determination of various pesticides in drinking and surface water by HPLC-MS/MS Bayer CropScience, Report No.: MR-13/085, Edition Number: M-466732- 01-1 Method Report No.: MR-13/085 Date: 2013-10-09 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2/23 | Stanislowski, T. | 2013 | Independent laboratory validation of BCS analytical methods 01333 and 01387 for determination of various pesticides in surface water by Di-HPLC-MS/MS PTRL Europe, Ulm, Germany Bayer CropScience, Report No.: P3117 G, Edition Number: M-470714- 02-1 Date: 2013-12-13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 4.2/24 | Freitag, T. | 2008 | Amendment no. 0001 to report no.: MR-08/138 - Analytical Method 01115 for the determination of residues of amidosulfuron,iodosulfuron-methyl-sodium, metsulfuron-methyl, mesosulfuron-methyl and foramsulfuron in soil by HPLC-MS/MS Bayer CropScience, Report No.: M-310074-03-1, Edition Number: M-310074- 03-1 Method Report No.: MR-08/138 Date: 2008-10-27 ...Amended: 2013-08-08 GLP/GEP: yes, unpublished | N | Bayer CropScience |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for Iodosulfuron-methyl-sodium

    - Methods used for the generation of pre-authorization data (KCP 5.1)

      - Validation data of Analytical method in water (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/01, also submitted as KCP 10.2.1/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Toxicity to the Duckweed Lemna gibba under laboratory conditions (semi-static test design). Schuler, L., 2022, study no. S22-01264 |
| Guideline(s): | OECD 221 (2006) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). - Quantifier mass transition m/z 508 → 167 (evaluated and used for quantification) - Qualifier mass transition m/z 508 → 325 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.995 |
| Calibration range: | 0.000450 – 0.0450 ng/mL Iodosulfuron-methyl-sodium with at least five (5) data points (corresponding to 0.000000540 – 0.0000540 mg Iodosulfuron-methyl-sodium/L) |
| Limit of Quantification (LOQ): | 0.000954 mg test item /L (0.00000184 mg Iodosulfuron-methyl-sodium/L) |
| Limit of Detection (LOD): | 0.000000540 mg Iodosulfuron-methyl-sodium/L (defined as lowest calibration standard) |
| Accuracy/precision: | Residues in control samples < LOD. Fortification level (mg/L); Iodosulfuron-methyl-sodium Nominal (mg/L); Mean recovery (%); RSD (%); (n). 0.000954; 0.00000184; 94; 5; 5. 1.30; 0.00251; 96; 6; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of working solution (ng/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1035; 85; 0.8. |

Conclusion

An analytical method for the determination of Iodosulfuron-methyl-sodium in test medium was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

      - Validation data of Analytical method in 50% (w/v) aqueous sucrose solution (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/02, also submitted as KCP 10.3.1.2/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Honey bee Apis mellifera L. Chronic oral toxicity test 10 day feeding in the laboratory. Vergé, E., 2022, study no. S22-01252 |
| Guideline(s): | OECD 245, 2017 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). - Quantifier mass transition m/z 508 → 167 (evaluated and used for quantification) - Qualifier mass transition m/z 508 → 141 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.99, residuals randomly distributed |
| Calibration range: | 0.400 – 7.50 ng/mL with at least five (5) data points (corresponding to 0.0135í 0.252 mg a.i./kg)* |
| Limit of Quantification (LOQ): | 0.0510 mg iodosulfuron-methyl-sodium/kg |
| Limit of Detection (LOD): | 0.0135 mg iodosulfuron-methyl-sodium/kg* (defined as lowest calibration standard and equivalent to ≤ 30 % of the LOQ) |
| Accuracy/precision: | Residues in control samples < LOD. Iodosulfuron-methyl-sodium Nominal (mg/L); Mean recovery (%); RSD (%); (n). 0.0510; 92.9; 2; 5. 11.4; 92.4; 2; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of stored working solution (µg/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1035; 63; 3. |
| Storage stability | Matrix; Storage timing; Test item fortification level (mg/kg); Active ingredient fortification level (mg/kg); Recovery (%); Mean recovery (%); % of freshly fortified samples. 50% aqueous sucrose solution containing 0.1% xanthan; 0 d; 5600; 11.4; 94.5, 90.9, 92.8, 92.6, 91.5; 92.4; -. 39 d; 5600; 11.4; 96.6, 96.0; 96.3; 104. |

*Corresponding concentrations are calculated considering the same dilution factors used during the preparation of LOQ recovery samples.

Conclusion

An analytical method for the determination of iodosulfuron-methyl-sodium in 50 % (w/v) aqueous sucrose solution containing 0.1 % xanthan was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, storage stability, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

      - Validation data of Analytical method in feeding solution (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/03, also submitted as KCP 10.3.1.3/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Honey bee Apis mellifera 22 day larval toxicity test (repeated exposure). Vergé, E., 2023, study no. S22-01253 |
| Guideline(s): | OECD 239, 2021 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). Analysed by LC-MS/MS as iodosulfuron-methyl - Quantifier mass transition m/z 508 → 167 (evaluated and used for quantification) - Qualifier mass transition m/z 508 → 141 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.99, residuals randomly distributed |
| Calibration range: | 0.14 – 6.5 ng/mL with at least five (5) data points (corresponding to 0.00280 - 0.130 mg a.i./kg) |
| Limit of Quantification (LOQ): | 0.0102 mg iodosulfuron-methyl-sodium/kg |
| Limit of Detection (LOD): | 0.00280 mg iodosulfuron-methyl-sodium/kg (defined as lowest calibration standard and equivalent to ≤ 30 % of the LOQ) |
| Accuracy/precision: | Residues in control samples < LOD. Iodosulfuron-methyl-sodium Nominal (mg/L); Mean recovery (%); RSD (%); (n). 0.0102; 101; 5.38; 5. 1.94; 100; 1.17; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of stored working solution (µg/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1035; 63; 3. |

Conclusion

An analytical method for the determination of iodosulfuron-methyl-sodium in larval diet was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

      - Validation data of Analytical method in tap water (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/04, also submitted as KCP 10.6.2/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Effects on the vegetative vigour of non-target terrestrial plant species under greenhouse conditions. Ripperger, D., 2022, S22-01280 |
| Guideline(s): | OECD 227, 2006 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). - Quantifier mass transition m/z 508 → 167 (evaluated and used for quantification) - Qualifier mass transition m/z 508 → 141 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.995 |
| Calibration range: | 0.5 - 10 ng/mL iodosulfuron-methyl-sodium with at least five (5) data points (corresponding to 0.1 - 2 mg/L, calculated using the dilution factor of the LOQ recovery level) |
| Limit of Quantification (LOQ): | 300.0 mg test item /L (0.6 mg iodosulfuron-methyl-sodium/L) |
| Limit of Detection (LOD): | 0.100 mg iodosulfuron-methyl-sodium/L (defined as lowest calibration standard and equivalent to ≤ 30 % of the LOQ) |
| Accuracy/precision: | Residues in control samples < LOD. Fortification level (mg/L); Iodosulfuron-methyl-sodium Nominal (mg/L); Mean recovery (%); RSD (%); (n). 300.0; 0.6; 100; 5; 5. 5096 (mean); 10 (mean); 94; 4; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of stored working solution (µg/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1035; 63; 3. |

Conclusion

An analytical method for the determination of iodosulfuron-methyl-sodium in tap water was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted.

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted.

  - Analytical methods for Mesosulfuron-methyl

      - Validation data of Analytical method in water (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/01, also submitted as KCP 10.2.1/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Toxicity to the Duckweed Lemna gibba under laboratory conditions (semi-static test design). Schuler, L., 2022, study no. S22-01264 |
| Guideline(s): | OECD 221 (2006) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). - Quantifier mass transition m/z 504 →182 (evaluated and used for quantification) - Qualifier mass transition m/z 504 → 306 - (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.995 |
| Calibration range: | 0.00225 – 0.225 ng/mL Mesosulfuron-methyl with at least five (5) data points (corresponding to 0.00000270 – 0.000270 mg Mesosulfuron-methyl/L) |
| Limit of Quantification (LOQ): | 0.000954 mg test item /L (0.0000102 mg Mesosulfuron-methyl/L) |
| Limit of Detection (LOD): | 0.00000270 mg Mesosulfuron-methyl /L (defined as lowest calibration standard) |
| Accuracy/precision: | Residues in control samples < LOD. Fortification level (mg/L); Mesosulfuron-methyl Nominal (mg/L); Mean recovery (%); RSD (%); (n). 0.000954; 0.0000102; 94; 2; 5. 1.30; 0.0139; 95; 4; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of working solution (ng/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1091; 63; 5. |

Conclusion

An analytical method for the determination of Mesosulfuron-methyl in test medium was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

      - Validation data of Analytical method in 50% (w/v) aqueous sucrose solution (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/02, also submitted as KCP 10.3.1.2/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Honey bee Apis mellifera L. Chronic oral toxicity test 10 day feeding in the laboratory. Vergé, E., 2022, study no. S22-01252 |
| Guideline(s): | OECD 245, 2017 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). - Quantifier mass transition m/z 504 → 182 (evaluated and used for quantification) - Qualifier mass transition m/z 504 → 83 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.99, residuals randomly distributed |
| Calibration range: | 0.400 – 7.50 ng/mL with at least five (5) data points (corresponding to 0.0538 - 1.01 mg a.i./kg)* |
| Limit of Quantification (LOQ): | 0.268 mg mesosulfuron-methyl/kg |
| Limit of Detection (LOD): | 0.0538 mg mesosulfuron-methyl/kg* (defined as lowest calibration standard and equivalent to ≤ 30 % of the LOQ) |
| Accuracy/precision: | Residues in control samples < LOD. Mesosulfuron-methyl Nominal (mg/L); Mean recovery (%); RSD (%); (n). 0.268; 98.7; 1; 5. 59.9; 105; 2; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of stored working solution (µg/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1091; 63; 5. |
| Storage stability | Matrix; Storage timing; Test item fortification level (mg/kg); Active ingredient fortification level (mg/kg); Recovery (%); Mean recovery (%); % of freshly fortified samples. 50% aqueous sucrose solution containing 0.1% xanthan; 0 d; 5600; 59.9; 106, 105, 101, 107, 104; 105; -. 39 d; 5600; 59.9; 115, 115; 115; 110. |

*Corresponding concentrations are calculated considering the same dilution factors used during the preparation of LOQ recovery samples.

Conclusion

An analytical method for the determination of mesosulfuron-methyl in 50 % (w/v) aqueous sucrose solution containing 0.1 % xanthan was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, storage stability, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

      - Validation data of Analytical method in feeding solution (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/03, also submitted as KCP 10.3.1.3/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Honey bee Apis mellifera 22 day larval toxicity test (repeated exposure). Vergé, E., 2022, study no. S22-01253 |
| Guideline(s): | OECD 239, 2021 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). Analysed by LC-MS/MS as iodosulfuron-methyl - Quantifier mass transition m/z 504 → 182 (evaluated and used for quantification) - Qualifier mass transition m/z 504 → 83 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.99, residuals randomly distributed |
| Calibration range: | 0.14 – 6.5 ng/mL with at least five (5) data points (corresponding to 0.0140 – 0.65 mg a.i./kg) |
| Limit of Quantification (LOQ): | 0.0535 mg mesosulfuron-methyl/kg |
| Limit of Detection (LOD): | 0.0140 mg mesosulfuron-methyl/kg (defined as lowest calibration standard and equivalent to ≤ 30 % of the LOQ) |
| Accuracy/precision: | Residues in control samples < LOD. Mesosulfuron-methyl Nominal (mg/L); Mean recovery (%); RSD (%); (n). 0.0535; 106; 5.42; 5. 10.2; 106; 2.37; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of stored working solution (µg/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1091; 63; 5. |

Conclusion

An analytical method for the determination of mesosulfuron-methyl in larval diet was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).

      - Validation data of Analytical method in tap water (Ecotoxicology)

zRMS: method is accepted

| Reference: | KCP 5.1.2/04, also submitted as KCP 10.6.2/01 |
| --- | --- |
| Report: | IMS+MSM+MPR 2+10+30 OD: Effects on the vegetative vigour of non-target terrestrial plant species under greenhouse conditions. Ripperger, D., 2022, S22-01280 |
| Guideline(s): | OECD 227, 2006 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | yes |

Material and methods

Method for determination:	HPLC-MS/MS

Method flow chart:

Chromatographic conditions:

Mass spectrometric conditions:

Results and discussion

| Specificity: | No significant interferences at the retention time of analyte in any of the blank matrix tested (< LOD). - Quantifier mass transition m/z 504 → 182 (evaluated and used for quantification) - Qualifier mass transition m/z 504 → 83 (monitored for confirmation of peak identity but was not used for quantification) |
| --- | --- |
| Linearity/Quantification: | Linear regression analysis with 1/x weighting, r ≥ 0.995 |
| Calibration range: | 2.5 - 50 ng/mL mesosulfuron-methyl with at least five (5) data points (corresponding to 0.5 - 10 mg/L, calculated using the dilution factor of the LOQ recovery level) |
| Limit of Quantification (LOQ): | 300.0 mg test item /L (3.2 mg mesosulfuron-methyl/L) |
| Limit of Detection (LOD): | 0.500 mg mesosulfuron-methyl/L (defined as lowest calibration standard and equivalent to ≤ 30 % of the LOQ) |
| Accuracy/precision: | Residues in control samples < LOD. Fortification level (mg/L); Iodosulfuron-methyl-sodium Nominal (mg/L); Mean recovery (%); RSD (%); (n). 300.0; 3.2; 108; 3; 5. 5096 (mean); 54 (mean); 98; 3; 5. |
| Stability of working solutions: | Solvent of working solution; Type of solution; Concentration of stored working solution (µg/mL); Storage period (days); Mean Difference (in %) of stored Working Solution compared to freshly prepared Solution. Acetonitrile; Stock solution; 1091; 63; 5. |

Conclusion

An analytical method for the determination of mesosulfuron-methyl in tap water was validated with regard to recovery, linearity of detector response, repeatability, specificity, matrix effect, stability of working solutions, limit of quantification and limit of detection. The analytical method fulfils the requirements of guideline SANTE/2020/12830 rev. 1/ 24/02/2021 (risk assessment method).