FINAL REGISTRATION REPORT Part B Section 8 Environmental Fate Detailed summary of the risk assessment

Product code: IMS+MSM+MPR 2+10+30 OD Product name(s): CERENET Chemical active substance(s): Iodosulfuron-methyl-sodium, 2 g/L Mesosulfuron-methyl, 10 g/L Mefenpyr-diethyl (safener), 30 g/L

Central Zone Zonal Rapporteur Member State: Poland

NATIONAL ASSESSMENT - Poland (new authorization)

Applicant: Certiplant B.V. Submission date: January 2025 MS Finalisation date: 01.2025; 04.2025

Version history

| When | What |
| --- | --- |
| January 2025 | Section 8.9.2 updated by applicant |
| January 2025 | Assessment by zRMS |
| April 2025 | The Final Registration Report |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	5

8.1	Critical GAP and overall conclusions	6

8.2	Metabolites considered in the assessment	8

8.3	Rate of degradation in soil (KCP 9.1.1)	8

8.3.1	Aerobic degradation in soil (KCP 9.1.1.1)	8

8.3.1.1	Mefenpyr-diethyl (safener) and its metabolites	8

8.3.2	Anaerobic degradation in soil (KCP 9.1.1.1)	11

8.3.2.1	Mefenpyr-diethyl (safener) and its metabolites	11

8.4	Field studies (KCP 9.1.1.2)	12

8.4.1	Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)	12

8.4.1.1	Mefenpyr-diethyl (safener) and its metabolites	12

8.4.2	Soil accumulation testing (KCP 9.1.1.2.2)	14

8.4.2.1	Mefenpyr-diethyl (safener) and its metabolites	14

8.5	Mobility in soil (KCP 9.1.2)	14

8.5.1	Laboratory studies (KCP 9.1.2.1)	14

8.5.1.1	Mefenpyr-diethyl (safener) and its metabolites	14

8.5.2	Lysimeter studies (KCP 9.1.2.2)	16

8.5.2.1	Mefenpyr-diethyl (safener) and its metabolites	16

8.5.3	Field leaching studies (KCP 9.1.2.3)	16

8.6	Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)	16

8.6.1	Mefenpyr-diethyl (safener) and its metabolites	16

8.7	Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)	17

8.7.1	Justification for new endpoints	17

8.7.2	Active substance(s) and relevant metabolite(s)	17

8.7.2.1	Mefenpyr-diethyl (safener) and its metabolites	18

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	20

8.8.1	Justification for new endpoints	20

8.8.2	Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)	20

8.8.2.1	Mefenpyr-diethyl (safener) and its metabolites	21

8.9	Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)	23

8.9.1	Justification for new endpoints	23

8.9.2	Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)	23

8.9.2.1	Spray drift exposure calculation for the formulated product- IMS+MSM+MPR 2+10+30 OD	25

8.9.2.2	Iodosulfuron-methyl-sodium and its metabolites	25

8.9.2.3	Mesosulfuron-methyl and its metabolites	27

8.9.3	Mefenpyr-diethyl and its metabolites	29

8.10	Fate and behaviour in air (KCP 9.3, KCP 9.3.1)	33

8.10.1	Fate and behaviour of iodosulfuron-methyl-sodium in air	34

Appendix 1	Lists of data considered in support of the evaluation	35

Appendix 2	Detailed evaluation of the new Annex II studies	45

# Fate and behaviour in the environment (KCP 9)

Following  recommendation by the central zone Steering Committee (czSC) as well as by the RMS of mesosulfuron-methyl, France, exposure and risk assessments for the application of mefenpyr-diethyl with the present formulation are not presented in the core assessment, but are addressed in the National Addenda of this submission, for evaluation on national level.

The fate and behaviour of the safener mefenpyr-diethyl have been reviewed in a work-sharing project of European competent authorities, in analogy to the procedures foreseen for active substances in Directive 91/414/EEC: France and Austria prepared an assessment report for this substance in the format of a Draft Assessment Report which was “peer-reviewed” (in an unscheduled procedure on voluntary basis) by all Member States in September 2011. The revised assessment report can be found on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)).

For all other information related to the active substances iodosulfuron-methyl-sodium and mesosulfuron-methyl, reference is made to the core assessment.

## Critical GAP and overall conclusions

Table 8.1-1:	Critical use pattern of the formulated product

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha | Conclusion |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | kg or L product/ha a) max. rate per appl. b) max. total rate per crop/season | g or kg as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  | Groundwater |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | EU | Winter wheat | F | ALOMY APESV VIOAR | Spraying | BBCH 21-31 | 1 | - | 1.5 | 3 (1) 15 (2) 45 (3) | 200-400 | - | Spring use | A |
| 2 | PL | Winter triticale Winter wheat | F | ALOMY APESV VIOAR | Spraying | BBCH 21-31 | 1 | - | 1.2 | 2.4 (1) 12 (2) 36 (3) | 200-400 | - | Spring use | A |
| Interzonal uses (use as seed treatment, in greenhouses (or other closed places of plant production), as post-harvest treatment or for treatment of empty storage rooms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor uses according to Article 51 (zonal uses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor uses according to Article 51 (interzonal uses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

(1) Iodosulfuron-methyl-sodium		(2) Mesosulfuron-methyl		(3) Mefenpyr-diethyl

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

Explanation for column 15 “Conclusion”

| A | Safe use |
| --- | --- |
| R | Further refinement and/or risk mitigation measures required |
| C | To be confirmed by cMS |
| N | No safe use |

Table 8.1-2:	Assessed (critical) uses during approval of mefenpyr-diethyl (safener) concerning the Section Environmental Fate

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I * | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | kg or L product/ha a) max. rate per appl. b) max. total rate per crop/season | g as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  |
| 1 | EU | Cereals | F | Weeds | Spraying broadcast | BBCH 10-39 | a) 1 b) 1 | - | a) N/A b) N/A | a) 100 b) 100 | 100-600 | - |  |
| 2 | EU | Turf | F | Weeds | Hand held sprayer | Young turf (after third mowing) or established turf | a) 1 b) 1 | - | a) N/A b) N/A | a) 100 b) 100 | 300-800 | - |  |

* 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

## Metabolites considered in the assessment

For the metabolites of the active substances iodosulfuron-methyl-sodium and mesosulfuron-methyl, please refer to the core assessment.

Table 8.2-3:	Metabolites of mefenpyr-diethyl (safener)

| Metabolite | Molar mass | Chemical structure | Maximum observed occurrence in compartments | Exposure assessment required due to |
| --- | --- | --- | --- | --- |
| AE F113225 | 345.2 |  | Soil: 44.1% Water/sediment: 82.8% | PECsoil PECgw PECsw/sed |
| AE F094270 | 271.11 |  | Soil: 72.2% Water/sediment: 62.4% | PECsoil PECgw PECsw/sed |
| AE F109453 | 317.13 |  | Water/sediment: 46.5% | PECsw/sed |
| AE F114952 | 345.18 |  | Soil: 11.5% Water/sediment: 18.6% | PECsoil PECgw PECsw/sed |
| AE 2211046 (M8) | 391.26 |  | Soil: 11% | PECsoil PECgw PECsw/sed |

## Rate of degradation in soil (KCP 9.1.1)

Studies on degradation in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Aerobic degradation in soil (KCP 9.1.1.1)

#### Mefenpyr-diethyl (safener) and its metabolites

The fate and behaviour of the safener mefenpyr-diethyl in the environment has been reviewed in a work-sharing project of European competent authorities, in analogy to the procedures foreseen for active substances in Directive 91/414/EEC: France and Austria prepared an assessment report for this substance in the format of a Draft Assessment Report which was “peer-reviewed” (in an unscheduled procedure on voluntary basis) by all MS in September 2011. The revised assessment report can be found on CIRCA. All exposure and risk assessments presented here are based on these agreed endpoints, if not otherwise stated. No additional studies are considered for this assessment.

Under aerobic conditions, degradation of the diester parent molecule is initiated by hydrolysis at one ester group to form the monocarboxylic acid AE F113225 (max. 44%) as a major metabolite. Hydrolysis may also occur at the alternative ester group of the parent molecule to form the isomeric monocarboxylic acid AE F114952 (max. 11.5%) as an additional, but minor initial step in the total degradation pathway. Resulting from a multi-step transformation process, the pyrazole carboxylic acid AE F094270 (max. 72%) is formed as another major soil metabolite. Starting from one of the monocarboxylic acids AE F113225 or AE F114952, the multi-step conversion to AE F094270 includes hydrolysis of the corresponding remaining ester group followed by a combined decarboxylation / elimination of carbonic acid in the heterocycle. All degradates are transient intermediates, which are either transformed to their respective metabolic downstream products, mineralized to carbon dioxide (up to 8.0%), or integrated into the soil matrix as non-extractable residues (up to 61.7%). The biotic nature of transformation of mefenpyr-diethyl under aerobic conditions was confirmed by investigations in sterilised soil.

Under irradiated conditions in the laboratory degradation of mefenpyr-diethyl AE 2211046 was identified as a major metabolite, specific to this route of degradation (up to 11%).

The degradation of mefenpyr-diethyl under aerobic conditions in soil including photolysis is presented in Figure 8.3-1 below.

Figure 8.3-1:	Proposed pathway of mefenpyr-diethyl in soil

Table 8.3-4:	Summary of aerobic degradation rates for mefenpyr-diethyl (safener) – laboratory studies

| Mefenpyr-diethyl (safener), Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (USDA) | pH (CaCl2) | t. °C | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20 °C1 pF2/10kPa | Chi² (%) | Kinetic model | Evaluated on EU level y/n/ Reference |
| LS 2.2 | Sandy loam | 5.0 | 20-25 ² | 40 | 2.90 | 9.6 | 4.1 | 11.0 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLV | Loamy sand | 6.1 | 20-25 ² | 40 | 1.54 | 5.12 | 2.4 | 15.1 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLH | Silt loam | 6.1 | 20 | 40 | 2.32 | 7.69 | 1.7 ³ | 10.9 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLH | Silt loam | 6.1 | 20 | 60 | 1.06 | 3.52 | 1.1 ³ | 17.2 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| LSN | Loamy sand | 7.1 | 20 | 60 | 2.44 | 8.11 | 2.3 | 18.4 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean/Median (n=4) ³ |  |  |  |  |  |  | 2.4/2.3 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | n |  |  |  |

1 normalised to a 20 °C and pF 2 moisture, using an activation energy of 65.4 kJ/mol and a Walker coefficient of 0.7

² The test temperature was reported between 20-25 °C. RMS considered conduct at 25 °C as worst-case.

³ Two DT50 values from the same soil, thus geometric mean calculated first (1.4 days), to contribute as single value to the calculation of the overall geometric mean.

Table 8.3-5:	Summary of aerobic degradation rates for AE F113225 - laboratory studies

| AE F113225, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (USDA) | pH (CaCl2) | t. °C | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20 °C 1 pF2/10kPa | Chi² (%) | Kinetic model | Evaluated on EU level y/n/ Reference |
| LS 2.2 | Sandy loam | 5.0 | 20-25 ² | 40 | 5.68 | 18.9 | 7.9 | 20.4 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLV | Loamy sand | 6.1 | 20-25 ² | 40 | 6.77 | 22.5 | 10.6 | 11.7 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLH | Silt loam | 6.1 | 20 | 40 | 3.47 | 11.5 | 2.61 ³ | 7.4 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLH | Silt loam | 6.1 | 20 | 60 | 4.56 | 15.2 | 4.55 ³ | 2.50 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| LSN | Loamy sand | 7.1 | 20 | 60 | 4.89 | 16.2 | 4.59 | 16.8 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean/Median (n=4) ³ |  |  |  |  |  |  | 6.1/6.2 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | n |  |  |  |

1 normalised to a 20 °C and pF 2 moisture, using an activation energy of 65.4 kJ/mol and a Walker coefficient of 0.7

² The test temperature was reported between 20-25 °C. RMS considered conduct at 25 °C as worst-case.

³ Two DT50 values from the same soil, thus geometric mean calculated first (3.4 days), to contribute as single value to the calculation of the overall geometric mean.

Table 8.3-6:	Summary of aerobic degradation rates for AE F094270 - laboratory studies

| AE F094270, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (USDA) | pH (CaCl2) | t. °C | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20 °C 1 pF2/10kPa | Chi² (%) | Kinetic model | Evaluated on EU level y/n/ Reference |
| LS 2.2 | Sandy loam | 5.0 | 20-25 ² | 40 | 271 | 900 | 379 | 6.66 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLV | Loamy sand | 6.1 | 20-25 ² | 40 | 259 | 859 | 406 | 4.55 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLH | Silt loam | 6.1 | 20 | 40 | 91.2 | 303 | 68 ³ | 8.21 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLH | Silt loam | 6.1 | 20 | 60 | 53.1 | 176 | 53 ³ | 8.36 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| LSN | Loamy sand | 7.1 | 20 | 60 | 266 | 884 | 250 | 5.68 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Höfchen, 03/12 | Silt loam | 6.84 | 20 | 50 | 126 | 418 | 126 | 3.97 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Laacher Hof, 03/13 | Sandy loam | 6.19 | 20 | 45 | 314 | > 1000 | 314 | 1.63 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Flint Hall, 03/14 | Clay loam | 7.39 | 20 | 50 | 152 | 505 | 152 | 3.63 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean/Median (n=7) ³ |  |  |  |  |  |  | 202/250 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | n |  |  |  |

1 normalised to a 20 °C and pF 2 moisture, using an activation energy of 65.4 kJ/mol and a Walker coefficient of 0.7

² The test temperature was reported between 20-25 °C. RMS considered conduct at 25 °C as worst-case.

³ Two DT50 values from the same soil, thus geometric mean calculated first (60.0 days), to contribute as single value to the calculation of the overall geometric mean.

### Anaerobic degradation in soil (KCP 9.1.1.1)

#### Mefenpyr-diethyl (safener) and its metabolites

The anaerobic degradation of mefenpyr-diethyl has been evaluated in the work sharing project, full details of the studies can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

Degradation of mefenpyr-diethyl in soil under anaerobic conditions follows basically the same pathways as under aerobic conditions, with metabolites AE F113225 and AE F094270 being formed in major amounts of 46.7% AR and 34.9% AR, respectively.

Table 8.3-7:	Summary of anaerobic degradation rates for mefenpyr-diethyl – laboratory studies

| Mefenpyr-diethyl (safener), Laboratory studies, anaerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (USDA) | pH (CaCl2) | t. °C | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20 °C pF2/10kPa | Chi² (%) | Kinetic model | Evaluated on EU level y/n/ Reference |
| SL | Sandy loam | 5.8 | 20 | N/A (flooded) | 67 1 | 223 1 | - | - | First order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean/Median (n=x) |  |  |  |  |  |  | -1 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | - |  |  |  |

1 Degradation rates for the anaerobic phase could be considered as less reliable since four data points are available to result in a non-optimal visual fit. However and for the intended use, anaerobic conditions are not expected to last long term in field and thus the result is not considered relevant for the risk assessment.

## Field studies (KCP 9.1.1.2)

### Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)

#### Mefenpyr-diethyl (safener) and its metabolites

The field dissipation of mefenpyr-diethyl (safener) has been evaluated in the work sharing project; full details of the studies can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

The degradation of mefenpyr-diethyl under field conditions was investigated at four German sites at a nominal application rate of 90 g/ha. Mefenpyr-diethyl was found to dissipate very rapidly at all sites investigated. After application, significant concentrations of the parent compound were found only at site “Stelle”.

Table 8.4-8:	Summary of aerobic degradation rates for mefenpyr-diethyl - field studies: Triggering endpoints

| Mefenpyr-diethyl (safener), Field studies – Triggering endpoints |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type 1 | Location | pH (KCl) | Depth (cm) | DissT50 (d) actual | DT90 (d) actual | Kinetic parameters | St. (r²) | Method of calc. | Evaluated on EU level y/n/ Reference |
| Loamy sand, Maize | Stelle, Germany | 5.6 | 10 | - ² | - | - | - | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, Phacelia | Gersthofen, Germany | 6.3 | 10 | - ² | - | - | - | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, spring barley | Bornheim, Germany | 7.1 | 10 | - ² | - | - | - | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy silty loam, spring barley | Schwanheim, Germany | 5.7 | 10 | - ² | - | - | - | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Maximum (n=4) |  |  |  | - ² |  |  |  |  |  |

1 Classification system not reported.

² Too few number of data points with significant concentrations exist to estimate reliable degradation parameters of mefenpyr-diethyl. Hence, no reliable field DT50 values of mefenpyr-diethyl are available.

Table 8.4-9:	Summary of aerobic degradation rates for AE F094270 - field studies: Triggering endpoints

| AE F094270, Field studies – Triggering endpoints |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type 1 | Location | pH (KCl) | Depth (cm) | DissT50 (d) actual | DT90 (d) actual | f.f. | Kinetic parameters | St. (r²) | Method of calc. | Evaluated on EU level y/n/ Reference |
| Loamy sand, Maize | Stelle, Germany | 5.6 | 10 | - ² | - | - | - | - | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, Phacelia | Gersthofen, Germany | 6.3 | 10 | 23 | 76 | N/A | N/A | 0.971 | 1st order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, spring barley | Bornheim, Germany | 7.1 | 10 | - ³ | - |  | - | 0.913 | 1st order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy silty loam, spring barley | Schwanheim, Germany | 5.7 | 10 | 79 | 263 | N/A | N/A | 0.869 | 1st order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Maximum (n=2) |  |  |  | 79 | 263 | N/A |  |  |  |  |

N/A: not available

1 classification system not reported

² no AE F094270 residues > LOQ detected at this site at any time point

³ non-normalised DT50 of 44 d. However, quality of fit not considered acceptable by RMS.

Modelling endpoints

Table 8.4-10:	Summary of aerobic degradation rates for mefenpyr-diethyl - field studies: Modelling endpoints

| Mefenpyr-diethyl (safener), Field studies – Triggering endpoints |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Soil type 1 | Location | pH (KCl) | Depth (cm) | DT50 (d) 20 °C, pF2 | Fit, kinetic | Evaluated on EU level y/n/ Reference |
| Loamy sand, maize | Stelle, Germany | 5.6 | 10 | - ² | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, Phacelia | Gersthofen, Germany | 6.3 | 10 | - ² | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, spring barley | Bornheim, Germany | 7.1 | 10 | - ² | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy silty loam, spring barley | Schwanheim, Germany | 5.7 | 10 | - ² | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean (n=x) |  |  |  | - ² |  |  |
| pH-dependency y/n |  |  |  | - |  |  |

Table 8.4-11:	Summary of aerobic degradation rates for relevant metabolite(s) - field studies: Modelling endpoints

| AE F094270, Field studies – Triggering endpoints |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type 1 | Location | pH (KCl) | Depth (cm) | DT50/DT90 (d) 20 °C, pF2 | f.f. | Fit (Chi²), kinetic | Evaluated on EU level y/n/ Reference |
| Loamy sand, maize | Stelle, Germany | 5.6 | 10 | - ² | - | - | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, Phacelia | Gersthofen, Germany | 6.3 | 10 | 12.8/42.5 | N/A | 9.0, simple first order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy loam, spring barley | Bornheim, Germany | 7.1 | 10 | 13.4/44.4 | N/A | 5.8, simple first order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Sandy silty loam, spring barley | Schwanheim, Germany | 5.7 | 10 | 43.8/- | N/A | 9.0, simple first order | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |

N/A: not available

1 classification system not reported

² residues of AE F094270 were below the LOQ at this site at all sampling intervals.

### Soil accumulation testing (KCP 9.1.1.2.2)

#### Mefenpyr-diethyl (safener) and its metabolites

The accumulation of mefenpyr-diethyl (safener) has been evaluated in the work sharing project; full details can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

No soil accumulation studies were performed or are required as the times for a 90% degradation of all mefenpyr-diethyl residues under conditions of the field were well below 1 year. No potential for accumulation of residues in soil is therefore indicated for the parent compound and/or its major metabolites.

## Mobility in soil (KCP 9.1.2)

Studies on mobility in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Laboratory studies (KCP 9.1.2.1)

#### Mefenpyr-diethyl (safener) and its metabolites

The soil mobility of mefenpyr-diethyl (safener) has been evaluated in the work sharing project; full details of the studies can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

Table 8.5-12:	Summary of soil adsorption/desorption for mefenpyr-diethyl

| Mefenpyr-diethyl (safener) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | OC (%) | pH (CaCl2) | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| SLH | Silt loam 1 | 1.10 | 6.2 | 7.08 | 644 | 0.99 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SLV | Sandy loam 1 | 1.13 | 5.8 | 6.71 | 593 | 1.20 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| S2.1 | Sand 1 | 1.17 | 5.0 | 5.68 | 486 | 1.20 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| LS2.2 | Loamy sand 1 | 2.91 | 5.0 | 23.96 | 823 ³ | 1.23 ³ | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| LSN | Loamy sand 1 | 0.89 | 7.1 | 5.16 | 580 | 1.20 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Environmental fate soil #8 (EFS-8) | Sand ² | 0.49 | 4.98 | 3.16 | 648 | 0.96 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Environmental fate soil #15 (EFS-15) | Sandy clay loam ² | 2.70 | 7.9 | 19.9 | 738 | 0.96 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean (n=6) |  |  |  |  | 610 |  |  |
| Arithmetic mean (n=6) |  |  |  |  | 614 | 1.09 |  |
| pH-dependency y/n |  |  |  |  | n |  |  |

1 classification system not reported

² soil classification according to USDA

³ recovery below 50%. Value therefore not considered for risk assessment.

Table 8.5-13:	Summary of soil adsorption/desorption for AE F113225

| AE F113225 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | OC (%) | pH (CaCl2) | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| Shelley Field | Loam | 1.9 | 7.1 | 2.73 | 144 | 0.90 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Manningtree | Loam | 3.0 | 5.2 | 3.6 | 120 | 0.93 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean (n=3) |  |  |  |  | 110 |  |  |
| Arithmetic mean (n=3) |  |  |  |  | 113 | 0.92 |  |
| pH-dependency y/n |  |  |  |  | n |  |  |

Table 8.5-14:	Summary of soil adsorption/desorption for AE F094270

| AE F094270 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type 1 | OC (%) | pH ² | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| LS 2.2 | Loamy sand | 2.91 | 5.0 | 7.49 | 257 | 0.96 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| LS 2.3 | Sandy loam | 1.15 | 5.0 | 4.57 | 397 | 0.80 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| SL H | Silt loam | 1.33 | 6.8 | 1.57 | 118 | 1.02 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| F 821 | Sandy loam | 2.28 | 7.1 | 1.49 | 65 | 1.00 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| S 2.1 | Loamy sand | 1.17 | 5.0 | 2.58 | 221 | 0.86 | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean (n=5) |  |  |  |  | 177 |  |  |
| Arithmetic mean (n=5) |  |  |  |  | 212 | 0.93 |  |
| pH-dependency y/n |  |  |  |  | n |  |  |

1 classification system not reported

² medium not reported

AE 2211046: For the adsorption of metabolite AE 2211046 to soil, the RMS derived a Koc value of 1320 L/kg by a QSAR-type approach using KOCWIN v.2.00. The Freundlich coefficient 1/n was set to 1.

Column leaching studies with non-aged residues of the parent compound did not indicate a risk for mobility into deeper layers of soil. For aged residues, a moderate mobility was found under the conditions of the laboratory. The mobility of residues of mefenpyr-diethyl was therefore further investigated under outdoor conditions in a lysimeter test.

### Lysimeter studies (KCP 9.1.2.2)

#### Mefenpyr-diethyl (safener) and its metabolites

An outdoor lysimeter study performed following application of mefenpyr-diethyl (safener) has been evaluated in the work sharing project; full details of the studies can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

The study showed a rapid overall degradation of 14C-labelled mefenpyr-diethyl and its residues accompanied by the formation of bound residues and 14C-carbon dioxide. There were no indications for translocation of the parent substance or its residues into deeper layers of soil. Analysis of percolates showed that no individual component was showed up above a mean annual concentration of 0.1 µg parent-equiv./L.

### Field leaching studies (KCP 9.1.2.3)

Field leaching studies have not been conducted for mefenpyr-diethyl (safener) as sufficient information can be derived from the existing studies

## Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)

Studies on degradation in water/sediment systems with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Mefenpyr-diethyl (safener) and its metabolites

The degradation of mefenpyr-diethyl (safener) in water/sediment has been evaluated in the work sharing project; full details of the studies can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

Table 8.6-15:	Summary of degradation in water/sediment of mefenpyr-dietyl

| Mefenpyr-diethyl (safener) Distribution: (max. water < LOQ from DAT-30 onwards, max. sediment 34.3% after 0 days) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water/ sediment system | pH water/ sed. (CaCl2) | DT50 whole syst. (d) deg=diss | DegT90 whole syst. (d) | Kinetic, Fit | DissT50 water (d) | DissT90 water (d) | Kinetic, Fit | DissT50 sed. (d) | Kinetic, Fit | Evaluated on EU level y/n/ Reference |
| Gravel Pit | 9.0/7.1 | 1.0 | 3.5 | SFO | 0.65 | 2.2 | SFO | 0.95 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Nidda | 7.7/6.9 | 1.1 | 3.6 | SFO | 0.92 | 3.0 | SFO | 1.5 | SFO | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Geometric mean (n=2) |  | 1.1 |  |  | 0.8 |  |  | 1.2 |  |  |

Table 8.6-16:	Summary of observed metabolites

| AE F113225 Water/sediment system | Max. in water: 74.9% after 7 d (Gravel Pit, chlorophenyl-U-14C) Max. in sediment: 18.0% after 14 d (Nidda, chlorophenyl-U-14C) | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| --- | --- | --- |
| AE F114952 Water/sediment system | Max. in water: 17.3% after 7 d (Gravel Pit, chlorophenyl-U-14C) Max. in sediment: 3.8% after 101 d (Gravel Pit, chlorophenyl-U-14C) | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| AE F109453 Water/sediment system | Max. in water: 42.0% after 101 d (Gravel Pit, chlorophenyl-U-14C) Max. in sediment: 5.6% after 36 d (Nidda, chlorophenyl-U-14C) | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| AE F094270 Water/sediment system | Max. in water: 28.5% after 101 d (Nidda, chlorophenyl-U-14C) Max. in sediment: 33.9% after 101 d (Nidda, chlorophenyl-U-14C) | y/ DAR of mefenpyr-diethyl, VOL 3 – B.8; rev 1 of 06/2011; available on CIRCA |

## Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)

For calculations of predicted environmental concentrations in soil (PECsoil) for the product, the active substances and their metabolites, reference is made to the core assessment. In the core dossier, a dose rate of 1.5 L/ha is proposed whereas in Poland, a maximum application rate of 1.2 L/ha will apply. No separate PECsoil calculations are necessary in this national addendum for Poland for IMS and MSM. The dose rate of 1.5 L/ha presented in the core dossier covers the dose rate for Poland.

PECsoil calculations for the safener are provided below.

### Justification for new endpoints

No new endpoints were used.

### Active substance(s) and relevant metabolite(s)

Input parameters related to application:

Mefenpyr-diethyl (safener)

For the safener mefenpyr-diethyl (and metabolites) risk assessments based on PECsoil calculations are passed without any refinement, even if worst case PECsoil values are considered. Therefore, to simplify the assessment, PECsoil is calculated in a “risk envelope approach”, addressing the maximum proposed application rate in the central zone and overall worst case exposure situation (no crop interception, no tillage).

This dose rate also covers the lower proposed dose rate in Poland.

Table 8.7-17:	Input parameters related to application for PECsoil calculations – Iodosulfuron-methyl-sodium

| Use No. | Risk envelope covering all uses of mefenpyr-diethyl |
| --- | --- |
| Crop | Cereals |
| Application rate (g as/ha) | 45 g a.s./ha (risk envelope) |
| Number of applications/interval | 1 |
| Crop interception (%) | 0 |
| Depth of soil layer (relevant for plateau concentration) (cm) | 10 cm (no tillage) |

Substance parameters for active substances and metabolites:

Table 8.7-18:	Input parameters for mefenpyr-diethyl and relevant metabolites for PECsoil calculations

| Compound | Molecular weight (g/mol) | Max. occurrence (%) | DT50 (days) | Value in accordance to EU endpoint y/n Reference |
| --- | --- | --- | --- | --- |
| Mefenpyr-diethyl | 373.26 | 100 | 4.6 | Y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| AE F113225 (AE F114952**) | 345.2 | 44.1 | 10.6 | Y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| AE F094270 | 271.11 | 72.2* | 425 | Y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| AE 2211046 | 391.26 | 11 | 35.5 | Y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |

* 100% for accumulation calculations

** Metabolites AE F113225, AE F094270, and AE 2211046 were assessed by detailed PEC calculations. AE F114952 was observed as a transient metabolite (DT50 ≤1.3 days) in a single test soil. Within the review it was concluded that risk assessment of AE F114952 for soil and groundwater is conservatively covered by its closely related isomer AE F113225. The latter is characterized by markedly higher occurrence in tests and higher values of half-life in soil (cf. DAR Vol. 3 B.8., page 12)

#### Mefenpyr-diethyl (safener) and its metabolites

Table 8.7-19:	PECsoil for mefenpyr-diethyl

| PECsoil (mg/kg) |  | Cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.060 | - | - | - |
| Short term | 24h | 0.052 | 0.056 | - | - |
|  | 2d | 0.044 | 0.052 | - | - |
|  | 4d | 0.033 | 0.045 | - | - |
| Long term | 7d | 0.021 | 0.037 | - | - |
|  | 14d | 0.007 | 0.025 | - | - |
|  | 21d | 0.003 | 0.018 |  |  |
|  | 28d | 0.001 | 0.014 | - | - |
|  | 50d | < 0.001 | 0.008 | - | - |
|  | 100d | < 0.001 | 0.004 | - | - |

PECsoil of metabolites

Table 8.7-20:	PECsoil for AE F113225*

| PECsoil (mg/kg) |  | Cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.024 | - | - | - |
| Short term | 24h | 0.023 | 0.024 | - | - |
|  | 2d | 0.021 | 0.023 | - | - |
|  | 4d | 0.019 | 0.022 | - | - |
| Long term | 7d | 0.015 | 0.020 | - | - |
|  | 14d | 0.010 | 0.016 | - | - |
|  | 28d | 0.004 | 0.011 | - | - |
|  | 50d | 0.001 | 0.007 | - | - |
|  | 100d | < 0.001 | 0.004 | - | - |

* covers also AE F114952

Table 8.7-21: 	PECsoil for AE F094270

| PECsoil (mg/kg) |  | Cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.031 | - | - | - |
| Short term | 24h | 0.031 | 0.031 | - | - |
|  | 2d | 0.031 | 0.031 | - | - |
|  | 4d | 0.031 | 0.031 | - | - |
| Long term | 7d | 0.031 | 0.031 | - | - |
|  | 14d | 0.031 | 0.031 | - | - |
|  | 28d | 0.030 | 0.031 | - | - |
|  | 50d | 0.029 | 0.030 | - | - |
|  | 100d | 0.027 | 0.029 | - | - |
| Plateau concentration (10 cm) |  | 0.027 | - | - | - |
| PECaccumulation (PECact + PECsoil plateau) (10 cm) |  | 0.058 | - | - | - |

Table 8.7-22: 	PECsoil for AE 2211046

| PECsoil (mg/kg) |  | Cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.007 | - | - | - |
| Short term | 24h | 0.007 | 0.007 | - | - |
|  | 2d | 0.007 | 0.007 | - | - |
|  | 4d | 0.006 | 0.007 | - | - |
| Long term | 7d | 0.006 | 0.006 | - | - |
|  | 14d | 0.005 | 0.006 | - | - |
|  | 28d | 0.004 | 0.005 | - | - |
|  | 50d | 0.003 | 0.004 | - | - |
|  | 100d | 0.001 | 0.003 | - | - |

- ZRMS comments:

- zRMS agree with calculation PECs performed for the safener mefenpyr diethyl and its metabolites

- AE F113225, AE F094270, and AE 2211046.

## Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)

For calculations of predicted environmental concentrations in groundwater (PECgw) for the product, the active substances and their metabolites, reference is made to the core assessment. In the core dossier, a dose rate of 1.5 L/ha is proposed whereas in Poland, a maximum application rate of 1.2 L/ha will apply. No separate PEC calculations are necessary in this national addendum for Poland for IMS and MSM. The dose rate of 1.5 L/ha presented in the core dossier covers the dose rate for Poland.

PECgw calculations for the safener are provided below.

### Justification for new endpoints

No new endpoints were used.

### Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)

Mefenpyr-diethyl (safener)

For the safener mefenpyr-diethyl (and metabolites) PECgw is calculated in a “risk envelope approach”, addressing the maximum proposed application rate in the central zone and overall worst case exposure situation (no crop interception, no tillage).

This dose rate also covers the lower proposed dose rate in Poland.

Table 8.8-23:	Input parameters related to application for PECgw calculations

| Use No. | 1 |
| --- | --- |
| Crop | Winter cereals |
| Application rate (g as/ha) | Mefenpyr-diethyl: 45 g a.s./ha |
| Number of applications/interval (d) | 1 / - |
| Absolute application date | Using AppDate tool (application dates defined for BBCH 21, i.e. tillering) |
| Crop interception (%) | 0% |
| Frequency of application | Annual |
| Models used for calculation | FOCUS PEARL v5.5.5, FOCUS PELMO v6.6.4, FOCUS MACRO v5.5.4 |

Explanatory note on definition of application dates

- End-of-winter to spring use in winter cereals

The application in winter cereals according to GAP is intended at the onset of the spring vegetation period, when climate conditions allow for resumption of crop and weed growth after winter dormancy and soil moisture level allows again for field trafficability by the farmer’s equipment. Treatment is made to well-established crop at the growth stage reached at that time, within the BBCH boundaries specified in the GAP.

No pre-defined event dates are however implemented in the FOCUS model that would directly translate this cropping situation into discrete PAT windows for each surface water scenario setting. To generate an adequate scenario-adapted representation with relative date setting, the following approach was therefore used: the simulated treatment was referenced relative to the tabulated crop emergence date of the earliest emerging spring crop (i.e. not necessarily cereals) that was defined by FOCUS for the respective scenario. The application is then set 14 days before this respective date as a conservative estimate of the beginning of the vegetation period, at the same time avoiding unrealistically early applications which would never occur under normal conditions (e.g. farmer spraying snow-covered fields or frozen soil).

An overview of the date selection per scenario is presented in the table below; for technical reason, such application dates must be entered to the simulation model formally as ‘absolute’ dates, even though referencing was in fact of relative type.

Table 8.8.2-24:	Application dates used for groundwater exposure assessment  - End-of-winter use in winter cereals

| Crop | Scenario | Earliest emerging crop (to define begin of vegetation period) | Tabulated emergence date | Application date (absolute) |
| --- | --- | --- | --- | --- |
| Winter cereals | Châteaudun | Spring cereals | 10-Mar | 24-Feb |
|  | Hamburg | Carrots | 10-Mar | 24-Feb |
|  | Kremsmünster | Carrots | 10-Mar | 24-Feb |
|  | Okehampton | Field beans | 15-Mar | 01-Mar |
|  | Piacenza | Sugar beet | 20-Mar | 06-Mar |
|  | Porto | Carrots | 28-Feb | 14-Feb |

#### Mefenpyr-diethyl (safener) and its metabolites

Table 8.8-25:	Input parameters related to safener mefenpyr-diethyl and metabolites for PECgw calculations (Tier 1)

| Compound | Mefenpyr-diethyl | AE F113225 (AE F114952*) | AE F094270 | AE 2211046 | Value in accordance with EU endpoint y/n/ Reference* |
| --- | --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 373.26 | 345.2 | 271.11 | 391.26 | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Water solubility (g/mol): | 20 (20 °C) | 5.826 (20 °C) | 50 (20 °C) | 1000a | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Saturated vapour pressure (Pa): | 6.3x10-6 # (20 °C) | 1x10-10 Pab | 1x10-10 Pab | 1x10-10 Pab | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA b) Generic worst case assumption |
| DT50 in soil (d) | 2.4 | 6.1 | 19.6 | 35.5 in first horizon, 1000 in deeper horizons. | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Kfoc (mL/g) / Kfom | 614 / 356 (geomean) | 113.3 / 65.7 (geomean) | 212 / 123 | 1320 / 765.7 (QSAR) | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| 1/n | 1.085 (arithmetic mean) | 0.92 (arithmetic mean) | 0.928 (arithmetic mean) | 1.0 (default) | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Plant uptake factor | 0 | 0 | 0 | 0 | Generic worst case assumption |
| Formation fraction | - | 0.76 (from mefenpyr-diethyl) | 1.0 (from AE F113225) | - (Photoproduct. PECgw are therefore calculated using a pseudo-application rate) | y/ DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |

a Not measured. Default value used.

* Metabolites AE F113225, AE F094270 and AE 2211046 were assessed by detailed PEC calculations. AE F114952 was observed as a transient metabolite (DT50 ≤ 1.3 days) in a single test soil. Within the review it was concluded that risk assessment of AE F114952 for soil and groundwater is conservatively covered by its closely related isomer AE F113225. The latter is characterized by markedly higher occurrence in tests and higher values of half-life in soil (cf. DAR Vol. 3 B.8, page 12).

PECgw FOCUS PEARL and PELMO – parent substance and all metabolites

Table 8.8-26:	PECgw for mefenpyr-diethyl and metabolites

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mefenpyr-diethyl |  | AE F113225 |  | AE F094270 |  | AE 2211046 |  |
|  |  | PEARL | PELMO | PEARL | PELMO | PEARL | PELMO | PEARL | PELMO |
| Winter cereals End-of-winter – spring application 45 g/ha = 1.5 L product/ha | Châteaudun | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |
|  | Hamburg | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |
|  | Kremsmünster | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |
|  | Okehampton | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |
|  | Piacenza | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |
|  | Porto | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |

* Photoproduct. PECgw are therefore calculated using a pseudo-application rate of 5.19 g/ha (calculated considering the molecular ratio (391.26/373.3) and the maximal occurrence in soil (11%))

Groundwater exposure assessment mefenpyr-diethyl – Overall conclusion:

The safener mefenpyr-diethyl and its metabolites do not breach the EU threshold value of 0.1 µg/L for the intended uses of the present formulation. The risk for groundwater is acceptable and no relevance assessment in Part B.10 for any of the assessed metabolites is required.

Since all PECgw calculated with FOCUS PEARL and PELMO are < 0.001 µg/L, no calculations with FOCUS MACRO are necessary.

## ZRMS comments:

## zRMS agree with calculation PECgw performed for the safener mefenpyr diethyl and its metabolites

## AE F113225, AE F094270, and AE 2211046.

## Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)

In the core dossier, calculations of predicted environmental concentrations in surface water (PECsw) for the product, the active substances and their metabolites, have been presented for a maximum application rate of 1.5 L/ha. However, in Poland, the maximum application rate is only 1.2 L/ha. Therefore, additional PECsw calculations are provided below for the active substances iodosulfuron-methyl-sodium and mesosulfuron-methyl and their metabolites for an application rate of 1.2 L/ha following the same approach as in the core dossier.

In addition, PECsw calculations for the safener mefenpyr-diethyl are provided below for both a dose rate of 1.5 L/ha (central zone) as for a dose rate of 1.2 L/ha (PL).

### Justification for new endpoints

No new endpoints were used.

### Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)

Structure of PECsw presentation

To enable a stepwise ecotoxicological risk assessment according the tiered approaches of the EFSA Aquatic Guidance Document (AGD), a comprehensive set of exposure calculations and supportive information for exposure description is presented here in dRR Part B.8, in an order consistent with the later data used for risk assessment in dRR Part B Section 9:

As a first step, a spray-drift exposure calculation for the formulated product is made, based on Ganzelmeier tabulated standard drift values.

Thereafter, Exposure calculations for the individual components (active substances and metabolites) via the FOCUSsw approach are made, in a structure as follows:

(a) FOCUS Steps 1-2 – PECsw/sed for a generic risk envelope use pattern covering all uses

to enable a simplified screening level assessment for components and organisms characterised by a wide margin of safety even when based on highly conservative exposure assumptions.

Reference is made to the core dossier. FOCUS Steps 1-2 were calculated for a dose rate of 1.5 L/ha.

(b) FOCUS Step 3 – PECsw/sed (maximum and TWA) for the critical GAPs

to enable Tier 1 risk assessment based on the accurate GAP and standard FOCUS Step 3 exposure description, where assessment was not resolved at the before screening level. For the present product and uses, this applies only for the iodosulfuron-methyl-sodium metabolite AE F075736, and the active substance mesosulfuron-methyl on which all further risk assessments will concentrate.

(c) FOCUS Step 4 – PECsw/sed (maximum and TWA) for the critical GAPs

to enable consideration of exposure mitigating measures, where required.

The same headline structure (a) to (c) is followed for each substance in the subsequent sections.

Methodology information

(a) FOCUS Step 1-2

Reference is made to the core dossier.

(b) FOCUS Step 3 – PECsw/sed (maximum and TWA) [for Tier 1 assessment]

Table 8.9.2-27:	Input parameters related to application for PECSW/SED calculations – iodosulfuron-methyl-sodium, mesosulfuron-methyl

| Plant protection product | IMS+MSM+MPR 2+10+30 OD |
| --- | --- |
| Use no. | 2 |
| Crop | Winter cereals (arable crops) |
| Application rate | Iodosulfuron-methyl-sodium: 2.4 g a.s./ha Mesosulfuron-methyl: 12 g a.s./ha |
| Number of applications/interval (d) | 1 / - |
| Application window | End of winter to spring (start of vegetation period): see specific information provided below. Application window used for modelling: see Table 8.9.2-4 |
| Application method | Spray application |
| CAM (Chemical application method) | 2 (application foliar linear) |
| Soil depth (cm) | 4 (default) |
| Models used for calculation | FOCUS SWASH v5.3 FOCUS PRZM v4.3.1 FOCUS MACRO v5.5.4 FOCUS TOXWA v4.4.3 |

Explanatory note on definition of application dates

- End-of-winter to spring use in winter cereals

The application in winter cereals according to GAP is intended at the onset of the spring vegetation period, when climate conditions allow for resumption of crop and weed growth after winter dormancy and soil moisture level allows again for field trafficability by the farmer’s equipment. Treatment is made to well-established crop at the growth stage reached at that time, within the BBCH boundaries specified in the GAP. At FOCUS Step 3, actual application dates are generally determined by the PAT (Pesticide Application Timer) included within SWASH, which within a defined time window selects an appropriate actual application date to ensure at least 10 mm of rainfall in the first 10 days after application, and at the same time less than 2 mm of rain per day in a five day period around the date of application.

No pre-defined event dates are however implemented in the FOCUS model that would directly translate this cropping situation into discrete PAT windows for each surface water scenario setting. To generate an adequate scenario-adapted representation with relative date setting, the following approach was therefore used: the simulated treatment was referenced relative to the tabulated crop emergence date of the earliest emerging spring crop (i.e. not necessarily cereals) that was defined by FOCUS for the respective scenario. Start of the PAT window was then set to 14 days before that date, as an adequate representation of the start of the vegetation period in the respective scenario environment.

An overview of the date selection per scenario is presented in the table below; for technical reason, such application dates must be entered to the simulation model formally as ‘absolute’ dates, even though referencing was in fact of relative type.

The approach and resulting model application date settings were discussed and agreed suitable during the EU review of active substance mesosulfuron-methyl (cf. DAR Vol. 3 – B.8 (PPP) – Atlantis OD).

Since this concerns a national addendum for Poland, only following scenarios are relevant and taken into account:

- D3 Vredepeel

- D4 Skousbo

- R1 Weiherbach

Table 8.9.2-28:	Application date definition used for surface water exposure assessment – end-of-winter use in winter cereals

| Scenario | Location | Crop | Emergence date | Start of application window Julian date |
| --- | --- | --- | --- | --- |
| D3 D4 R1 | Vredepeel Skousbo Weiherbach | Spring cereals Field beans Field beans | 01-Apr 15-Apr 10-Apr | 18-Mar 01-Apr 27-Mar |

(c) FOCUS Step 4 – PECsw/sed (maximum and TWA) [for Tier 1 assessment considering mitigation options]

Exposure simulations considering options for exposure mitigation according FOCUS Step 4 methodology were conducted for all components of biological relevance, based on the same substance and timing parameters previously used at Step 3.

#### Spray drift exposure calculation for the formulated product- IMS+MSM+MPR 2+10+30 OD

Reference is made to the core assessment.

#### Iodosulfuron-methyl-sodium and its metabolites

For iodosulfuron-methyl-sodium, agreed endpoints were used as input to exposure modelling as follows :

Table 8.9.2-29:	Input parameters related to active substance iodosulfuron-methyl-sodium and metabolite AE F075736 for PECsw/sed calculations STEP 3(/4)

| Compound | Iodosulfuron-methyl-sodium | AE F075736 | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- | --- |
| Molecular weight (g/mol) | 529.3 | 381.4 | Y / EFSA 2016;14(4) |
| Saturated vapour pressure (Pa) | 2.6 x 10-9 # (20 °C) | 1 x 10-10 a | # Y / EFSA 2016;14(4) |
| Water solubility (mg/L) | 25000 (20 °C) | 2790 (20 °C) | Y / EFSA 2016;14(4) |
| Diffusion coefficient in water (m²/d) | 4.3 x 10-5 | 4.3 x 10-5 | default |
| Diffusion coefficient in air (m²/d) | 0.43 | 0.43 | default |
| Kfoc (mL/g) | 33.45 (geomean, n = 9) | 14.0 (geomean, n = 22) | Y / EFSA 2016;14(4) |
| Freundlich Exponent 1/n | 0.87 (arithmetic mean, n = 9) | 1.0 (arithmetic mean, n = 22) | Y / EFSA 2016;14(4) |
| Plant Uptake | 0 | 0 | N / Worst case assumption |
| Wash-Off factor from Crop (1/mm) | 0.05 (MACRO) 0.50 (PRZM) | 0.05 (MACRO) 0.50 (PRZM) | Default |
| DT50,soil (d) | 2.7 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n = 11) | 24.9 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n = 19) | Y / EFSA 2016;14(4) |
| DT50,water (d) | 19.8 | 131 | Y / EFSA 2016;14(4) |
| DT50,sed (d) | 1000) | 1000 | Y / EFSA 2016;14(4) |
| DT50,whole system (d) | 19.8 (geomean, n = 3) | 131 (geomean, n = 3) | Y / EFSA 2016;14(4) |
| Maximum occurrence observed (% molar basis with respect to the parent) | - | Soil: 88.5 Water/sediment: 67.8 | Y / EFSA 2016;14(4) |
| Formation fraction in soil | - | 0.86 (from IMS) | Y / EFSA 2016;14(4) |

a not measured. Default value used

- FOCUS Steps 1-2 – Risk envelope PECsw/sed of iodosulfuron-methyl-sodium and all metabolites [for screening level assessment]

Reference is made to the core dossier.

- FOCUS Step 3 – PECsw/sed (maximum and TWA) of iodosulfuron-methyl-sodium and metabolite AE F075736 [for Tier 1 assessment]

Table 8.9.2-30:	FOCUS STEP 3 PECsw and PECsed for iodosulfuron-methyl-sodium following application of IMS+MSM+MPR 2+10+30 OD – use: winter cereals, 1 x 2.4 g iodosulfuron-methyl-sodium/ha, end of winter to spring application

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- |
| Iodosulfuron-methyl-sodium Step 3: winter cereals, 1 x 2.4 g iodosulfuron-methyl-sodium/ha, end of winter to spring application |  |  |  |  |
| D3 | Ditch | 0.01519 | Spray drift | 0.002928 |
| D4 | Pond | 0.000525 | Spray drift | 0.000740 |
| D4 | Stream | 0.01161 | Spray drift | 0.000349 |
| R1 | Pond | 0.000525 | Runoff | 0.000878 |
| R1 | Stream | 0.01002 | Runoff | 0.001688 |

Table 8.9.2-31:	FOCUS STEP 3 PECsw and PECsed for AE F075736 following application of IMS+MSM+MPR 2+10+30 OD – use: winter cereals, 1 x 2.4 g iodosulfuron-methyl-sodium/ha, end of winter to spring application

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- |
| Metabolite AE F075736 Step 3: winter cereals, 1 x 2.4 g iodosulfuron-methyl-sodium/ha, end of winter to spring application |  |  |  |  |
| D3 | Ditch | 0.002081 | Spray drift | 0.002451 |
| D4 | Pond | 0.005825 | Spray drift | 0.005588 |
| D4 | Stream | 0.003330 | Spray drift | 0.002204 |
| R1 | Pond | 0.000143 | Runoff | 0.000081 |
| R1 | Stream | 0.005842 | Runoff | 0.000400 |

- FOCUS Step 4 – PECsw/sed (maximum and TWA) of iodosulfuron-methyl-sodium and metabolite AE F075736 [for Tier 1 assessment considering mitigation options]

For the use of iodosulfuron-methyl-sodium with the present formulation, an exposure calculation at FOCUS Step 4 was not deemed relevant.

#### Mesosulfuron-methyl and its metabolites

For mesosulfuron-methyl, agreed endpoints were used as input to exposure modelling as follows:

Table 8.9.2-32	Input parameters related to active substance mesosulfuron-methyl for PECsw/sed calculations STEP 3/4

| Compound | Mesosulfuron-methyl | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- |
| Molecular weight (g/mol) | 503.5 | Y / EFSA 2016;14(10):4 584 |
| Saturated vapour pressure (Pa) | 3.5 x 10-12 (20 °C) | Y / EFSA 2016;14(10):4 584 |
| Water solubility (mg/L) | 483 (20 °C, pH 7)* | * Y / EFSA 2016;14(10):4 584 # Y / KCA 2.14/05, 07, 15, 17; values listed in RAR Vol 3 – B.8 (PPP) – Atlantis OD (07/2016) |
| Diffusion coefficient in water (m²/d) | 4.3 x 10-5 | default |
| Diffusion coefficient in air (m²/d) | 0.43 | default |
| Kfoc (mL/g) | 64 (geomean, n = 9)1 | Y / EFSA 2016;14(10):4 584 |
| Freundlich Exponent 1/n | 0.92 (arithmetic mean, n = 9) | Y / EFSA 2016;14(10):4 584 |
| Plant Uptake | 0 | Y / EFSA 2016;14(10):4 584 |
| Wash-Off factor from Crop (1/mm) | 50 | Default |
| DT50,soil (d) | Tier 1 – SFO: 49.72 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n = 9) | Y / EFSA 2016;14(10):4 584 |
| DT50,water (d) | 43 (geomean, total system, n = 4) | Y / EFSA 2016;14(10):4 584 |
| DT50,sed (d) | 1000 (default) | Y / EFSA 2016;14(10):4 584 |
| DT50,whole system (d) | 43 (geomean, total system, n = 4) | Y / EFSA 2016;14(10):4 584 |
| Maximum occurrence observed (% molar basis with respect to the parent) | - | Y / EFSA 2016;14(10):4 584 |

1 Kom = 37.1 ml/g

- FOCUS Steps 1-2 – Risk envelope PECsw/sed of mesosulfuron-methyl and all metabolites [for screening level assessment]

Reference is made to the core assessment.

- FOCUS Step 3 – PECsw/sed (maximum and TWA) of mesosulfuron-methyl

Table 8.9.2-33: 	FOCUS STEP 3 PECsw and PECsed for mesosulfuron-methyl following single application of IMS+MSM+MPR 2+10+30 OD – use: winter cereals, 1 x 12 g mesosulfuron-methyl/ha, end of winter to spring application

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- |
| Mesosulfuron-methyl Step 3: winter cereals, 1 x 12 g mesosulfuron-methyl/ha, end of winter to spring application |  |  |  |  |
| D3 | Ditch | 0.07828 | Spray drift | 0.02052 |
| D4 | Pond | 0.03217 | Spray drift | 0.07722 |
| D4 | Stream | 0.06148 | Spray drift | 0.03063 |
| R1 | Pond | 0.005071 | Runoff | 0.008030 |
| R1 | Stream | 0.08001 | Runoff | 0.01396 |

- FOCUS Step 4 – PECsw/sed (maximum and TWA) of mesosulfuron-methyl [for Tier 1 assessment considering mitigation options]

For the use of mesosulfuron-methyl with the present formulation, an exposure calculation at FOCUS Step 4 was not deemed relevant.

### Mefenpyr-diethyl and its metabolites

Table 8.9.3-34:	Input parameters related to application for PECSW/SED calculations – mefenpyr-diethyl

| Plant protection product | IMS+MSM+MPR 2+10+30 OD |
| --- | --- |
| Use no. | 1 |
| Crop | Winter cereals (arable crops) |
| Application rate | Central zone: Mefenpyr-diethyl: 45 g/ha Poland: Mefenpyr-diethyl: 36 g/ha |
| Number of applications/interval (d) | 1 / - |
| Crop interception | No crop interception |
| Region & season of application | North Europe – Mar-May |
| Models used for calculation | FOCUS Steps 1-2 v3.2, FOCUS SWASH v5.3 |

Table 8.9.3-35:	Input parameters related to safener mefenpyr-diethyl and metabolites for PECsw/sed calculations STEP 1/2 and 3(/4)

| Compound | Mefenpyr-diethyl | AE F113225 | AE F094270 | AE F109453 | AE F114952 | AE 2211046 | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 373.26 | 345.2 | 271.11 | 317.13 | 345.18 | 391.26 | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Saturated vapour pressure (Pa) | 6.3x10-6 (20 °C) | 1x10-10 Paa | 1x10-10 Paa | 1x10-10 Paa | 1x10-10 Paa | 6.3 x 10-6 Pa | N / worst case assumption |
| Water solubility (mg/L) | 20 (20 °C) | 5.826 (20 °C) | 50 (20 °C) | 1173 (20 °C) | 563 (20 °C) | 1000 (conservative value) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Diffusion coefficient in water (m²/d) | - a | - a | - a | - a | - a | - a | - |
| Diffusion coefficient in air (m²/d) | - a | - a | - a | - a | - a | - a | - |
| Kfoc / Kom(mL/g) | 614 / 356 (geomean) | 113.3 / 65.7 (geomean) | 212 / 123 (geomean) | 20e / 11.6 (geomean) | 10 (conservative) | 1320 (QSAR) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Freundlich Exponent 1/n | 1.09 (arithmetic mean) | 0.92 (arithmetic mean) | 0.928 (arithmetic mean) | 1.0a | 1.0a | 1.09 | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Plant Uptake | - a | - a | - a | - a | - a | - a | - |
| Wash-Off factor from Crop (1/mm) | - a | - a | - a | - a | - a | - a | - |
| DT50,soil (d) | 2.4 (lab) | 6.1 (lab) | 19.6 (field) | 1000d | 1000d | 35.5 (photolytic DT50) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| DT50,water (d) | 1.1 | 42.5 | 109.2 | 30.2 | 19.9 | 1000 (conservative value) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| DT50,sed (d) | 1.1 | 42.5 | 109.2 | 30.2 | 19.9 | 1000 (conservative value) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| DT50,whole system (d) | 1.1 | 42.5 | 109.2 | 23 | 19.9 | 1000 (conservative value) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Maximum occurrence in water/sediment (%) | 100 | 82.8 | 62.4 | 46.5 | 18.6 | 0 (photolysis in water is not a significant pathway and therefore formation of AE 2211046 is not expected) | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |
| Max occurrence in soil (%) | 100 | 44.1 | 72.2 | 0 | 11.5 | 11% | DAR of mefenpyr-diethyl, Vol 3 – B.8; rev 1 of 06/2011; available on CIRCA |

a not required for Steps 1-2 simulations

d conservative default value

e Conservative value (consistent with the low transfer to sediment found during the water/sediment studies)

f Assumed to be the same value as for the isomeric compound AE F113225

- FOCUS Steps 1-2 –PECsw/sed of mefenpyr-diethyl and all metabolites [for screening level assessment]

Central zone GAP (1.5 L/ha)

Table 8.9.3-36:	FOCUS Step 1,2 PECsw and PECsed for Mefenpyr-diethyl and metabolites AE F113225 and AE F094270 following single application of IMS+MSM+MPR 2+10+30 OD in cereals at an application rate of 1.5 L/ha

| Scenario FOCUS | Waterbody | Mefenpyr-diethyl |  |  | AE F113225 |  | AE F094270 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max PECsw (μg/L) | 21d twa PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) |
| Step 1 | --- | 8.66 | 0.65 | 50.64 | 15.61 | 17.35 | 11.62 | 24.24 |
| Step 2 |  |  |  |  |  |  |  |  |
| Northern Europe | Spring | 0.54 | 0.04 | 3.28 | 1.57 | 1.74 | 1.55 | 3.25 |

Table 8.9.3-37:	FOCUS Step 1,2 PECsw and PECsed for AE F109453, AE F114952 and AE 2211046 following single application of IMS+MSM+MPR 2+10+30 OD in cereals at an application rate of 1.5 L/ha

| Scenario FOCUS | Waterbody | AE F109453 |  | AE F114952 |  | AE 2211046 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) |
| Step 1 | --- | 5.94 | 1.15 | 4.19 | 0.41 | 0.63 | 8.27 |
| Step 2 |  |  |  |  |  |  |  |
| Northern Europe | Spring | 0.51 | 0.10 | 0.54 | 0.05 | 0.12 | 1.53 |

Poland GAP (1.2 L/ha)

Table 8.9.3-38:	FOCUS Step 1,2 PECsw and PECsed for Mefenpyr-diethyl and metabolites AE F113225 and AE F094270 following single application of IMS+MSM+MPR 2+10+30 OD in cereals at an application rate of 1.2 L/ha

| Scenario FOCUS | Waterbody | Mefenpyr-diethyl |  |  | AE F113225 |  | AE F094270 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max PECsw (μg/L) | 21d twa PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) |
| Step 1 | --- | 6.93 | 0.52 | 40.51 | 12.49 | 13.88 | 9.30 | 19.51 |
| Step 2 |  |  |  |  |  |  |  |  |
| Northern Europe | Spring | 0.43 | 0.03 | 2.62 | 1.26 | 1.39 | 1.24 | 2.60 |

Table 8.9.3-39:	FOCUS Step 1,2 PECsw and PECsed for AE F109453, AE F114952 and AE 2211046 following single application of IMS+MSM+MPR 2+10+30 OD in cereals at an application rate of 1.2 L/ha

| Scenario FOCUS | Waterbody | AE F109453 |  | AE F114952 |  | AE 2211046 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) | Max PECsw (μg/L) | Max PECsed (μg/kg) |
| Step 1 | --- | 4.75 | 0.92 | 3.35 | 0.33 | 0.50 | 6.62 |
| Step 2 |  |  |  |  |  |  |  |
| Northern Europe | Spring | 0.41 | 0.08 | 0.43 | 0.04 | 0.09 | 1.22 |

- FOCUS Step 3 – PECsw/sed (maximum and TWA) of mefenpyr-diethyl

Central zone GAP (1.5 L/ha)

Table 8.9.3-40: 	FOCUS STEP 3 PECsw and PECsed for mefenpyr-diethyl following single application of IMS+MSM+MPR 2+10+30 OD – use: winter cereals, 1 x 45 g mefenpyr-diethyl/ha, end of winter to spring application

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- |
| Mesosulfuron-methyl Step 3: winter cereals, 1 x 36 g mefenpyr-diethyl/ha, end of winter to spring application |  |  |  |  |
| D3 | Ditch | 0.2848 | Spray drift | 0.04650 |
| D4 | Pond | 0.009840 | Spray drift | 0.01171 |
| D4 | Stream | 0.2178 | Spray drift | 0.006048 |
| R1 | Pond | 0.009840 | Runoff | 0.01436 |
| R1 | Stream | 0.1878 | Runoff | 0.01684 |

Poland GAP (1.2 L/ha)

Table 8.9.3-41: 	FOCUS STEP 3 PECsw and PECsed for mefenpyr-diethyl following single application of IMS+MSM+MPR 2+10+30 OD – use: winter cereals, 1 x 36 g mefenpyr-diethyl/ha, end of winter to spring application

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- |
| Mesosulfuron-methyl Step 3: winter cereals, 1 x 36 g mefenpyr-diethyl/ha, end of winter to spring application |  |  |  |  |
| D3 | Ditch | 0.2278 | Spray drift | 0.03767 |
| D4 | Pond | 0.007872 | Spray drift | 0.009490 |
| D4 | Stream | 0.1742 | Spray drift | 0.004872 |
| R1 | Pond | 0.007872 | Runoff | 0.01166 |
| R1 | Stream | 0.1503 | Runoff | 0.01368 |

- ZRMS comments:

- zRMS agree with calculation PECsw/sed performed for the mefenpyr-diethyl its metabolites.

- metabolites.

## Fate and behaviour in air (KCP 9.3, KCP 9.3.1)

For the fate and behaviour in air of the product, the active substances and their metabolites, reference is made to the core assessment.

Fate and behaviour in air of the safener is discussed below.

### Fate and behaviour of iodosulfuron-methyl-sodium in air

The fate of mefenpyr-diethyl (safener) in air has been evaluated in the work sharing project; full details of the studies can be found in the revised assessment report on CIRCA (Archive individual substances – Mefenpyr-diethyl (safener)). No additional studies are considered for this assessment.

Table 8.10-42:	Summary of atmospheric degradation and behaviour: mefenpyr-diethyl

| Compound | Mefenpyr-diethyl |
| --- | --- |
| Direct photolysis in air | No studied, no data required |
| Quantum yield of direct phototransformation | No studies, no data required |
| Photochemical oxidative degradation in air | DT50 (d): 1.96 derived by the Atkinson model OH (12h) concentration assumed = 1.5 x 106 radicals/cm³ |
| Volatilisation | Vapour pressure (Pa): 6.3 x 10-6 Pa at 20 °C Henry’s law constant (Pa m³/mol) : - |
| Metabolites | - |

The vapour pressure of the safener mefenpyr-diethyl at 20 °C is 6.3 x 10-6 Pa. Hence the safener mefenpyr-diethyl is regarded as non-volatile. Therefore exposure of adjacent surface waters and terrestrial ecosystems from volatilization followed by subsequent deposition is not expected to occur.

- ZRMS comments: Accepted

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 9.1.3/02 | Anonymous | 2023 | PECsoil MPR and metabolites calculations - Not GLP/GEP Not published | N | Certiplant BV |
| KCP 9.2.4.1/04 | Anonymous | 2023 | PECgw MPR and metabolites - Not GLP/GEP Not published | N | Certiplant BV |
| KCP 9.2.4.1/06 | Anonymous | 2023 | PECgw MPR and metabolites – FOCUS input and output files - Not GLP/GEP Not published | N | Certiplant BV |
| KCP 9.2.5/02 | Anonymous | 2023 | PECsw and PECsed MPR and metabolites – FOCUS input and output files Certiplant BV Not GLP/GEP Not published | N | Certiplant BV |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KIIA, 7.1.1/01 | Buerkle W.L. | 1993f | Hoe 107892-14C, Aerobic Degradation in Two Soils - Laboratory Study – Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 1 Document No: A51665 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.1.1/02 | Buerkle W.L. | 1994d | Code: Hoe 107892 00 ZE99 0001 Hoe 107892 14C, Aerobic Degradation in Two Soils at Different Conditions in the Laboratory Generated by: Hoechst Schering AgrEvo GmbH; Environmental Chemistry Frankfurt Document No: A52824 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.1.2/01 | Knoch E., Knoell H.-E. | 1993a | Degradation of 14C-labelled Hoe 107892 in one soil incubated under anaerobic conditions after a preliminary aerobic incubation phase Generated by: Res.Consult.Comp., DEU; Document No: A50265 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.1.3/01 | Tarara G. | 1995a | Code: Hoe 107892 00 ZE99 0001 Hoe 107892-14C, Photolytic Degradation on Soil Surfaces Generated by: Hoechst Schering AgrEvo GmbH; Environmental Chemistry Frankfurt Document No: A53645 GLP / GEP Yes unpublished | N | BCS |
| KIIA 7.1.3/02 | Stupp, H., Bornatsch, W. | 2005 | Mefenpyr-diethyl: Identification of photometabolite M8. Report No.: MEF-05/175 Edition number: M-257584-01-1 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.2.1/01 | Buerkle W.L. | 1993f | Hoe 107892-14C, Aerobic Degradation in Two Soils - Laboratory Study – Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 1 Document No: A51665 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.2.1/02 | Buerkle W.L. | 1994d | Code: Hoe 107892 00 ZE99 0001 Hoe 107892 14C, Aerobic Degradation in Two Soils at Different Conditions in the Laboratory Generated by: Hoechst Schering AgrEvo GmbH; Environmental Chemistry Frankfurt Document No: A52824 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.2.1/04 | Kahl, G. | 2010a | Kinetic evaluation of the aerobic metabolism of mefenpyr-diethyl (AE F107892), AE F113225, and AE F094270 in soil for modelling and trigger purposes Generated by BayerCropScience Report No.: MEF-10/261 Edition number: M-369635-02-1 Date: 2010-04-29, amended 2010-10-20 GLP / GEP No unpublished | N | BCS |
| KIIA, 7.2.2/01 | Buerkle W.L. | 1994d | Code: Hoe 107892 00 ZE99 0001 Hoe 107892 14C, Aerobic Degradation in Two Soils at Different Conditions in the Laboratory Generated by: Hoechst Schering AgrEvo GmbH; Environmental Chemistry Frankfurt Document No: A52824 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.2.3/01 | Oddy A., Doble M. | 2004a | (14C)-AE F094270: Rate of degradation in three soils at 20 degrees C Generated by: Battelle AgriFood Ltd, Ongar, UK; Document No: C039761 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.2.4/01 | Knoch E., Knoell H.-E. | 1993a | Degradation of 14C-labelled Hoe 107892 in one soil incubated under anaerobic conditions after a preliminary aerobic incubation phase Generated by: Res.Consult.Comp., DEU; Document No: A50265 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.3.1/00 | Tillkes, M. | 1994 | Storage stability of Hoe 107892, Hoe 113225 and Hoe 094270 in soil. Generated by Dr Specht & Partner, Chemische Laboratorien GmbH, Germany Report No.: A53230 Edition number: M-133973-01-1 Date: 1994-10-31 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.3.1/01 | Baedelt H., Idstein H., Junker H. | 1994a | Fenoxaprop-P-ethyl + Hoe 107892 oil in water emulsion (69+75 g/l) (Code: Hoe 046360 24 EW14 A202) Investigation of the degradation behaviour of Hoe 107892 in soil under field conditions (Stufe 2 in accordance with the BBA Guideline, part IV, 4-1) Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 2 Document No: A53053 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.3.1/02 | Baedelt H., Idstein H., Junker H. | 1994b | Fenoxaprop-P-ethyl + Hoe 107892 oil in water emulsion (69+75 g/l) (Code: Hoe 046360 24 EW1l4 A201) Investigation of the degradation behaviour of Hoe 107892 in soil under field conditions (Stufe 2 in accordance with Richtlinie BBA, Teil IV, 4-1) Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 2 Document No: A53311 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.3.1/04 | Hardy, I., Patterson, D. | 2010 | Mefenpyr-diethyl: kinetic modelling analysis of data from field soil dissipation studies conducted in Germany to derive normalised degradation rates for AE F094270 Battelle UK Ltd., Ongar, Essex, United Kingdom BCS, Report No.: VC/08/040, Edition Number: M-351396-01-1 Date: 2009-07-07 Non GLP, unpublished | N | BCS |
| KIIA, 7.4.1/01 | Mueller J. | 1992a | Determination of the Adsorption/Desorption Behavior for Hoe 107892 in the System Soil/Water Generated by: Fraunhofer-Institut fuer Umweltch. und Oekotox.; Document No: A49291 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.1/02 | Judge D.N. | 1997a | The Adsorption/Desorption of AE F107892 in Two Soil Types Generated by: AgrEvo USA Company; AgrEvo Pikeville Document No: A57708 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.2/01 | Jacob O., Oddy A. | 2004a | (14C)-AE F113225: Adsorption to and desorption from three soils Generated by: Battelle AgriFood Ltd, UK; Document No: C046040 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.2/02 | Oddy A. | 2005a | [ 14C]-AE F113225: Adsorption to and Desorption from Three Soils, Addendum to Report CX/04/035 (Doc.No. C046040) Generated by: Battelle AgriFood Ltd, UK; Document No: C046793 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.2/03 | Schollmeier M., Eyrich U. | 1992j | Hoe 094270 00 ZC99 0001 Adsorption/Desorption in the System Soil/Water (Hoe 094270) Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 1 Document No: A47382 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.3/01 | Schwab W., Buerkle W.L., Mueller A. | 1993b | Hoe 107892-14C Leaching behaviour of the formulated non-aged active ingredient in the LUFA standard soils 2.1, 2.2 and 2.3 in the presence of Hoe 046360 (Hoe 046360 24 EW14 A2) Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 1 Document No: A51617 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.5/01 | Schwab W., Buerkle W.L., Mueller A. | 1993a | Hoe 107892-14C Leaching behaviour of the aged active ingredient in the LUFA standard soil 2.1 Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 1 Document No: A51623 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.7/01 | Kubiak R. | 1995a | Degradation and leaching of 14C-diethyl-1-(2,4- dichlorophenyl)-5-methyl-2-pyrazoline-3,5- dicarboxylate (Hoe 107892) in two sand lysimeters under outdoor conditions Generated by: SLFA, Neustadt/Weinstrasse; Fachbereich Phytomedizin Document No: A55271 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.4.7/02 | Kubiak R. | 2001a | Degradation and leaching of 14C-diethyl-1-(2,4- dichlorophenyl)-5-methyl-2-pyrazoline-3,5- dicarboxylate (Hoe 107892) in two sand lysimeters under outdoor conditions – Final report amendment Generated by: SLFA, Neustadt/Weinstrasse; Fachbereich Phytomedizin Document No: C014368 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.5/01 | Schollmeier M., Eyrich U. | 1993i | Hoe 107892 Determination of the Abiotic Hydrolysis as a Function of pH according to EPA Pesticide Assessment Guideline, Subdivision N, § 161-1 Hoe 107892 00 ZB99 0001 Generated by: Hoechst AG; GB C / Produktentwicklung Oekologie 1 Document No: A50834 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.5/02 | Mamouni A. | 1998b | Hydrolysis of AE F113225 at different pH values Generated by: RCC Umweltchemie AG, Itingen, CH; Document No: C000767 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.6/01 | Buerkle W.L., Sadowsky-Dunkmann I. | 1994a | Code: Hoe 107892 00 ZE99 0001 14C - Hoe 107892 Direct Photolysis in Aqueous Buffered Solution, Degradation Kinetics and Quantum Yield Generated by: Hoechst Schering AgrEvo GmbH; Environmental Chemistry Frankfurt Document No: A53195 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.7/01 | Voelskow H. | 1994h | Testing for ready biodegradability of Hoe 107892 Generated by: Hoechst AG; Abteilung Umweltschutz Document No: A58716 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.8.3/01 | Buerkle W.L. | 1994b | Code: Hoe 107892 00 ZE99 0002 Distribution and Degradation of the 14C-Labelled Test Substance in Two Water/Sediment Systems Generated by: Hoechst Schering AgrEvo GmbH; Environmental Biology Frankfurt Document No: A53414 GLP / GEP Yes unpublished | N | BCS |
| KIIA, 7.8.3/02 | Buerkle L.W. | 2000f | Identification of an intermediate degradation product that appeared in the water-sediment study CB92/137 (Addendum to report CB92/137) Mefenpyr-diethyl Code: AE F107892 Generated by: Aventis CropScience GmbH, DEU; Oekochemie, Frankfurt Document No: C008769 GLP / GEP No unpublished | N | BCS |
| KIIA, 7.8.3/03 | Sur, R. | 2007a | Kinetic evaluation of the aerobic aquatic metabolism of mefenpyr-diethyl and its monocarboxylic acid metabolites AE F113225 and AE F114952 in water-sediment systems Bayer CropScience, Report No.: MEF-06/506, Edition Number: M-289041-01-1 Date: 2007-06-05 Non GLP, unpublished | N | BCS |
| KIIA, 7.8.3/04 | Oddy, A., Doble, M.L. | 2005 | (14C)-AE F109453: Degradation and retention in two water / sediment systems Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Report No.: C048573, Edition Number: M-251518-01-1 Date: 2005-05-06 GLP, unpublished | N | BCS |
| KIIA, 7.8.3/05 | Sur, R. | 2007b | Kinetic evaluation of the aerobic aquatic metabolism of the phenylpyrazole carboxylic acid metabolite AE F094270 of mefenpyr-diethyl in a water-sediment system Bayer CropScience, Report No.: MEF-06/540, Edition Number: M-289400-01-1 Date: 2007-06-18 Non GLP, unpublished | N | BCS |
| KIIA, 7.8.3/06 | Doble, M.L, Oddy, A | 2005 | [14C]AE F094270: Degradation and retention in one water/sediment system Battelle AgriFood Ltd., Ongar, Essex, United Kingdom BCS, Report No.: CX/05/002, Edition Number: M-255572-01-1 Date: 2005-07-27 GLP, unpublished | N | BCS |
| KIIA, 7.8.3/07 | Sur, R. | 2007c | Kinetic evaluation of the aerobic aquatic metabolism of the dicarboxylic acid metabolite AE F109453 of mefenpyr-diethyl in water-sediment systems Bayer CropScience, Report No.: MEF-06/537, Edition Number: M-289415-01-1 Date: 2007-06-18 Non GLP, unpublished | N | BCS |
| KIIA, 7.10/01 | Buerkle L.W. | 1999j | Estimation of the reaction with photochemically produced hydroxyl radicals in the atmosphere Code: AE F107892 Generated by: Hoechst Schering AgrEvo GmbH; Entwicklung Umweltforschung, Frankfurt Document No: C003563 GLP / GEP No unpublished | N | BCS |
| KIIA, 7.10/02 | Deas A.H.B. | 1994a | The Volatility Of [14C]-Hoe 107892 (formulated as Hoe 046360 24 EW14 A7) from plant and soil surfaces Generated by: AgrEvo UK Ltd.; Document No: A53472 GLP / GEP Yes unpublished | N | BCS |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Detailed evaluation of the new Annex II studies

No new studies submitted.

“Guidance document on tiered risk assessment for plant protection products for aquatic organisms in edge-of-field surface waters in the context of Regulation (EC) No 1107/2009“, as provided by the Commission Services (SANTE-2015-00080, 15 January 2015). (Cited as « EFSA Aquatic Guidance Document » or « AGD » in the following pages)

FOCUS (2007). “Landscape and mitigation factors in aquatic risk assessment. Volume 2. Detailed Technical Reviews”. Report of the FOCUS Working Group on Landscape and Mitigation Factors in Ecological Risk Assessment, EC Document Reference SANCO/10422/2005 v2.0. 436pp.