REGISTRATION REPORT Part B Section 8 Environmental Fate Detailed summary of the risk assessment

Product code: IN233C1560 Product name(s): AVTAR Chemical active substance(s): Prothioconazole, 250 g/L Difenoconazole, 130 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (zonal authorization)

Applicant: XXXX Submission date: March 2023 Evaluation date: June 2023 MS Finalisation date: February 2024

Version history

| When | What |
| --- | --- |
| January 2022 | Original application |
| March 2023 | Update of the dRR part B8 according to Poland’s comments |
| June 2023 | Version evaluated by zRMS PL |
| November 2023 | Version updated following zRMS reply of the commenting table |
| February 2024 | Final dRR version |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	5

8.1	Critical GAP and overall conclusions	6

8.2	Metabolites considered in the assessment	11

8.3	Rate of degradation in soil (KCP 9.1.1)	12

8.3.1	Aerobic degradation in soil (KCP 9.1.1.1)	12

8.3.1.1	Prothioconazole and its metabolites	12

8.3.1.2	Difenoconazole and its metabolites	14

8.3.2	Anaerobic degradation in soil (KCP 9.1.1.1)	15

8.4	Field studies (KCP 9.1.1.2)	15

8.4.1	Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)	15

8.4.1.1	Prothioconazole and its metabolites	16

8.4.1.2	Difenoconazole and its metabolites	19

8.4.2	Soil accumulation testing (KCP 9.1.1.2.2)	20

8.5	Mobility in soil (KCP 9.1.2)	21

8.5.1	Prothioconazole and its metabolites	21

8.5.2	Difenoconazole and its metabolites	22

8.5.3	Column leaching (KCP 9.1.2.1)	23

8.5.4	Lysimeter studies (KCP 9.1.2.2)	23

8.5.5	Field leaching studies (KCP 9.1.2.3)	24

8.6	Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)	24

8.6.1	Prothioconazole and its metabolites	24

8.6.2	Difenoconazole and its metabolites	26

8.7	Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)	27

8.7.1	Justification for new endpoints	30

8.7.2	Active substance(s) and relevant metabolite(s)	30

8.7.2.1	Prothioconazole and its metabolites	32

8.7.2.2	Difenoconazole and its metabolites	35

8.7.2.3	PECsoil of prothioconazole, M04 and difenoconazole - refined application scheme on cereals BBCH 30+	38

8.7.2.4	PECsoil of prothioconazole and difenoconazole - refined application scheme on oilseed rape - one split application at BBCH 14+	40

8.7.2.5	PECsoil of prothioconazole and difenoconazole - refined application scheme on oilseed rape - 2 applications at BBCH 20+	42

8.7.2.6	PECsoil of formulation	43

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	43

8.8.1	Justification for new endpoints	44

8.8.2	Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)	44

8.8.2.1	Prothioconazole and its metabolites	47

8.8.2.2	Difenoconazole and its metabolites	49

8.8.2.3	Conclusion for PECGW	53

8.9	Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)	54

8.9.1	Justification for new endpoints	59

8.9.2	Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)	59

8.9.2.1	Prothioconazole and its metabolites	62

8.9.2.2	Difenoconazole and its metabolites	63

8.9.2.3	Additionnal STEP 4 calculation with VFSmod	77

8.9.2.4	PECsw of formulation	79

8.10	Fate and behaviour in air (KCP 9.3, KCP 9.3.1)	79

Appendix 1	Lists of data considered in support of the evaluation	81

Appendix 2	Detailed evaluation of the new Annex II studies	84

Appendix 3	Additional information provided by the applicant (e.g. detailed modelling data)	84

List of electronic input files	84

Winter cereals, FOCUS-PEARL 4.4.4	85

Spring cereals, FOCUS-PEARL 4.4.4	86

Winter oilseed rape, FOCUS-PEARL 4.4.4	87

Spring oilseed rape, FOCUS-PEARL 4.4.4	88

# Fate and behaviour in the environment (KCP 9)

This document is an update of the dRR submitted to Poland in January 2022.

The Zonal Rapporteur Member State, Poland, asked to repeat the calculation of the PEC ground water and surface water using the geometric mean Kfoc for Difenoconazole and its metabolites for the modeling of PECgw and PECsw and the following endpoints for Prothioconazole and its metabolites for the estimation of the PECsw:

| Compound | Prothioconazole | Prothioconazole-desthio | 1,2,4-triazole |
| --- | --- | --- | --- |
| Water solubility (mg/L) | 2000 as a worse case | 2000 as a worse case | 730000 (in accordance with agreed endpoints) |
| DT50,water (d) | 1 | 1000 | 1000 |
| DT50,sed (d) | 2.8 | 1000 | 1000 |
| DT50,whole system (d) | 2.8 | 1000 | 1000 |

The updated endpoints and results are highlighted in yellow in this document.

Detailed protocol and results are described in study KCP 9.2.4.1/01, KCP 9.2.4.1/02, KCP 9.2.5/01 and KCP 9.2.5/02.

In March 2023, an updated Core dRR was submitted to zRMS Poland including new modelling.

In the context of evaluation, Poland has requested an addendum including modelling for the metabolite of the Prothiconazole, Prothioconazole-S-methyl (M01). The calculation for the PECsw and PECsed for M01 was not included in the original application because the M01 is not in the definition of the Residue (Annex IIA, point 7.3 in the EFSA conclusions of 2007, page 77 of 98). However, Prothioconazole-S-methyl (M01) is found at over 14.6% in soil. Consequently, this report presents the PECSW and PECSED determination for the metabolite, Prothioconazole-S-methyl for the uses of AVTAR containing 250 g/L of Prothioconazole and 130 g/L of Difenoconazole on Cereals and Oilseed rape.

## Critical GAP and overall conclusions

Table 8.1-1:	Critical use pattern of the formulated product

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha | Conclusion |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | L product/ha a) max. rate per appl. b) max. total rate per crop/season | kg g as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  | Groundwater |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1, 2,3,4 and 5 | Central Zone EU | Wheat, Barley, Triticale, Rye and Oats | F | Not relevant for this chapter | Foliar spray | BBCH 25(30)-69 (1) | a) 2 b) 2 | 14 | a) 0.7 b) 1.4 | a) Prothioconazole 175 + Difenoconazole 91 b) Prothioconazole 350 + Difenoconazole 182 | 100-400 | NA | Risk envelope for cereals 0.6 – 0.7 L/ha 0.6 L/ha = Prothioconazole 150 g as/ha + Difenoconazole 78 g as/ha 0.7 L/ha = Prothioconazole 175 g as/ha + Difenoconazole 91 g as/ha | A |
| 6-7 | Central Zone EU | Oilseed Rape | F | Not relevant for this chapter | Foliar spray | BBCH 14-69 (2) | a) 2 b) 2 | 14 | a) 0.6 b) 1.2 | a) Prothioconazole 150 + Difenoconazole 78 b) Prothioconazole 300 + Difenoconazole 156 | 100-500 | NA | Risk envelope for OSR (PECsoil, PECgw, PECsw) Maximum 1 application in autumn/winter, and maximum 1 application in spring/summer | A |
| 6 | Central Zone EU | Oilseed Rape | F | Not relevant for this chapter | Foliar spray | BBCH 14(20)-69 (2) | a) 2 b) 2 | 14 | a) 0.6 b) 1.2 | a) Prothioconazole 150 + Difenoconazole 78 b) Prothioconazole 300 + Difenoconazole 156 | 100-500 | NA | OSR detailed scenario #1 (refined PECsoil) Maximum 1 application in autumn/winter, and maximum 1 application in spring/summer | A |
| 7 | Central Zone EU | Oilseed Rape | F | Not relevant for this chapter | Foliar spray | BBCH 14-69 (2) | a) 1, split b) 1, split | 14 | a) 0.3 + 0.3 b) 0.6 | a) Prothioconazole 75+75 + Difenoconazole 39+39 b) Prothioconazole 150 + Difenoconazole 78 | 100-500 | NA | OSR detailed scenario #2 (refined PECsoil) If the product is applied before BBCH 20, only one split application per crop is allowed. | A |

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

(1) The PEC were calculated initially for BBCH 25-69, and then restricted to BBCH 30-69 due to a necessary refinement for PECSOIL. The restriction may not be required in all cMS (see national addendum B9, if relevant).

(2) The PEC were calculated initially for BBCH 14-69 with a risk envelope of two applications at 0.6 L/ha, and then restricted to two different detailed scenarios due to a necessary refinement for PECSOIL. The restrictions may not be required in all cMS (see national addendum B9, if relevant).

Explanation for column 15 “Conclusion”

| A | Safe use |
| --- | --- |
| R | Further refinement and/or risk mitigation measures required |
| C | To be confirmed by cMS |
| N | No safe use |

Table 8.1-2:	Assessed uses during approval of Prothioconazole (EFSA, 2007)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season (2) | Min. interval between applications (days) | L product/ha a) max. rate per appl. b) max. total rate per crop/season | kg as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n.c. | EU North South | wheat, rye, triticale | F | Rusts, Eyespot, Fusarium spp., Powd. Mildew, Rhynchospor.Septoria | overall spray | start 26-29 up to BBCH69 # | 3# | 14 (up to 21) | 0.8 (3) | 0.2 | 200/400 | 35 | Product Proline (EC, 250 g/L) # timing , no. of applic. depends on national conditions |
| n.c. | EU North South | barley, oat | F | Rusts, Eyespot, Pyren. teres, Powd. Mildew, Fusarium spp., Rhynchospor. | overall spray | start 30 up to BBCH 61 # | 2# | 14 (up to 21) | 0.8 (3) | 0.2 | 200/400 | 35 | Product Proline (EC, 250 g/L) # timing , no. of applic. depends on national conditions |
| n.c. | EU North | rape | F | Sclerotinia, Botrytis, Alternaria, Leptosphaeria | overall spray | start BBCH 53# | 2# | 14 (up to 28) | 0.7 (3) | 0.175 | 200/400 | 56 | Product Proline (EC, 250 g/L) # timing , no. of applic. depends on national conditions |
| n.c. | EU North South | wheat, rye, triticale, oat, barley | F | Fusarium spp., Bunt, Smut | seed treatment | pre sowing | 1 | n.a. | n.a. | *approx. 9-18g as/ha (180 kg seed/ha) | 200-400 ml water/dt | n.a. | Product Redigo (EC, 100 g/L) *5 – 10 g as/dt seed [1] |

[1] A high first tier high risk to birds and mammals was identified

(2) not stated in EFSA (2007) if per use or per crop/season

(3) not stated in EFSA (2007), calculated from a.s. content in product

Table 8.1-3:	Assessed uses during approval of Difenoconazole (EFSA, 2011)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season (1) | Min. interval between applications (days) | L product/ha a) max. rate per appl. b) max. total rate per crop/season | kg as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n.c. | EU (N/S) | Pome fruit | F | Podosphaera leucotricha Venturia inaequalis | High vol spray or mist blower | Spray programme beginning at flowering (BBCH 61) | 4 | 10 (up to 14) | 0.225 (2) | 0.05625 (min. 0.01875) | 500/1500 | 28 | Product: SCORE A7402T, EC 250 g/l |
|  |  |  |  |  |  |  |  |  | 0.3 (2) | 0.0750 (min. 0.0375) | 500/1000 | 14 |  |
| n.c. | EU (N/S) | Carrot | F | Alternaria dauci Erysiphe heraclei | High vol. spray | First application at BBCH 42/43 | 3 | 14 | 0.5 (2) | 0.125 | 100/500 | 14 | Product: SCORE A7402T, EC 250 g/l |
| n.c. | EU (N/S) | Wheat | F | Fusarium spp. Tilletia spp. | Seed treatment | BBCH 00 | 1 | - |  | 0.012 (min; 0.005) | n.a. | - | Product DIVIDEND FS A9142G, FS 30 g/l |
| n.c. | EU (N/S) | Barley | F | Pyrenophorma granimea | Seed treatment | BBCH 00 | 1 | - |  | 0.012 (min; 0.005) | n.a. | - | Product DIVIDEND FS A9142G, FS 30 g/l |
| n.c. | EU (N/S) | Triticale | F | Fusarium spp. Tilletia spp | Seed treatment | BBCH 00 | 1 | - |  | 0.012 (min; 0.005) | n.a. | - | Product DIVIDEND FS A9142G, FS 30 g/l |
| n.c. | EU (N/S) | Rye | F | Fusarium spp. Tilletia spp | Seed treatment | BBCH 00 | 1 | - |  | 0.012 (min; 0.005) | n.a. | - | Product DIVIDEND FS A9142G, FS 30 g/l |
| n.c. | EU (N/S) | Oats | F | Ustilago avenae Pyrenphora avenae Cochliobolus sativum Fusarium culmorum Gibberella avenacea Pythium ultimum | Seed treatment | BBCH 00 | 1 | - |  | 0.012 (min; 0.005) | n.a. | - | Product DIVIDEND FS A9142G, FS 30 g/l |

(1) not stated in EFSA (2011) if per use or per crop/season

(2) not stated in the original table in EFSA (2007), calculated from a.s. content in product

[I] The groundwater exposure assessment for metabolites that may be formed in soil was not finalised.

[II] The risk assessment to soil dwelling organisms from exposure to difenoconazole under southern European conditions was not finalised.

[III] The chronic risk assessment to sediment dwelling organisms from the exposure to difenoconazole could not be finalised for the representative uses on cereals.

[IV] The chronic risk of difenoconazole to earthworms could not be finalised.

[V] The consumer risk assessment has to be considered provisional since the possible contribution of the TDM metabolites to the overall consumer exposure was not considered.

[VI] Possible impact on the toxicity, the consumer risk assessment, worker exposure and the environment of the variable isomer ratio in the technical and of the potential enantio-selective biologically mediated metabolism/degradation/transformation in plants, animals, and the environment needs to be addressed.

| *For uses where the column "Remarks" is marked in grey further consideration is necessary. Uses should be crossed out when the notifier no longer supports this use(s). (a)For crops, the EU and Codex classifications (both) should be taken into account; where relevant, the use situation should be described (e.g. fumigation of a structure) (b) Outdoor or field use (F), greenhouse application (G) or indoor application (I) (c) e.g. biting and suckling insects, soil born insects, foliar fungi, weeds (d) e.g. wettable powder (WP), emulsifiable concentrate (EC), granule (GR) (e) GCPF Codes - GIFAP Technical Monograph No 2, 1989 (f) All abbreviations used must be explained (g) Method, e.g. high volume spraying, low volume spraying, spreading, dusting, drench (h) Kind, e.g. overall, broadcast, aerial spraying, row, individual plant, between the plant- type of equipment used must be indicated | (i) g/kg or g/L. Normally the rate should be given for the active substance (according to ISO) and not for the variant in order to compare the rate for same active substances used in different variants (e.g. fluoroxypyr). In certain cases, where only one variant is synthesised, it is more appropriate to give the rate for the variant (e.g. benthiavalicarb-isopropyl). (j) Growth stage at last treatment (BBCH Monograph, Growth Stages of Plants, 1997, Blackwell, ISBN 3-8263-3152-4), including where relevant, information on season at time of application (k) Indicate the minimum and maximum number of application possible under practical conditions of use (l) The values should be given in g or kg whatever gives the more manageable number (e.g. 200 kg/ha instead of 200 000 g/ha or 12.5 g/ha instead of 0.0125 kg/ha (m) PHI - minimum pre-harvest interval |
| --- | --- |

## Metabolites considered in the assessment

Table 8.2-3:	Metabolites of Prothioconazole potentially relevant for exposure assessment

| Metabolite | Molar mass | Chemical structure | Maximum observed occurence in compartements | Exposure assessment required due to |
| --- | --- | --- | --- | --- |
| Prothioconazole-desthio (M04) | 312.2 | (1-chlorocycloproyl)1-(2- chlorophenyl)-3-(1,2,4- triazol-1-yl)-propan-2-ol | Water/Sediment: 32.3 % Sediment: 26.9% Soil: 49.4 57.1 % | Occurrence observed in soil and occurrence observed in water/sediment system (according to EFSA conclusion |
| 1.2.4-triazole | 69.065 | 1H-1,2,4-triazole | Water/Sediment: 37.2 % Soil: < 2 % | Occurrence observed in water/sediment system (according to EFSA conclusion |
| Prothioconazole-S-methyl (M01) | 358.3 | 2-(1-chlorocyclopropyl)- 1-(2-chlorophenyl)-3- (4,5-dihydro-5- methylthio-1,2,4- triazolyl-1)-propan-2-ol | Soil: 14.6% | Occurrence observed in soil (a ccording to EFSA conclusion |

Table 8.2-2:	Metabolites of Difenoconazole potentially relevant for exposure assessment

| Metabolite | Molar mass | Chemical structure | Maximum observed occurence in compartements | Exposure assessment required due to |
| --- | --- | --- | --- | --- |
| CGA 205375 | 350 | 1-[2-[2-chloro-4-(4-chloro-phenoxy)-phenyl]-2-1H- [1,2,4]triazol-yl]-ethanol | Soil: 9.7 Water/Sediment: 11.6 | Occurrence observed in soil and occurrence observed in water/sediment system (according to EFSA conclusion |
| 1,2,4-triazole (CGA 71019) | 69.065 | 1H-1,2,4-triazole | Soil: 23.4 Water/Sediment: 9.6 max. 3.2% in pond system (day 148), max. 14.1% in river system (day 148) | Occurrence observed in soil and occurrence observed in water/sediment system (according to EFSA conclusion |

## Rate of degradation in soil (KCP 9.1.1)

Studies on degradation in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Aerobic degradation in soil (KCP 9.1.1.1)

Relevant endpoints are cited in the EFSA conclusions for, prothioconazole (2007) and difenoconazole (2011).

#### Prothioconazole and its metabolites

Table 8.3-4:	Summary of aerobic degradation rates for Prothioconazole - laboratory studies

| Prothioconazole, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH (water) | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | R2 | Kinetic model | Evaluated on EU level y/n/ Reference |
| Laacher Hof AXXXa, Rhineland, Germany | Sandy loam | 7.2 | 20 | 48 | 0.07 | 5.3 | (1) | 1.000 | FOMC | Y/ EFSA 2007 |
| Stanley, Kansas, USA | Silty clay loam | 5.9 | 20 | 48 | 0.7 | 78.2 | (1) | 0.989 | FOMC | Y/ EFSA 2007 |
| Höfchen, Rhineland, Germany | Silt | 7.1 | 20 | 50 | 0.30 | 0.99 | (1) | 0.990 | 1st SFO | Y/ EFSA 2007 |
| Byromville, Georgia, USA | Loamy sand | 6.8 | 20 | 50 | 1.27 | 4.22 | (1) | 0.987 | 1st SFO | Y/ EFSA 2007 |
| Median (n=4) |  |  |  |  | 0.5 |  |  |  |  |  |
| pH-dependency: y/n |  |  |  |  | N |  |  |  |  |  |

(1) Considered identical to DT50. No normalization was deemed necessary in the DAR (2004) and EFSA conclusion (2007)

Table 8.3-5:	Summary of aerobic degradation rates for M01 (prothioconazole-S-methyl) - laboratory studies

| Prothioconazole-S-methyl (M01), Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH (water) | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | R2 | Kinetic model | Evaluated on EU level y/n/ Reference |
| Höfchen, Rhineland, Germany | Loamy silt (HF) | 7.3 | 20 | 40 | 5.9 | 19.6 | (1) | 0.97 | SFO | Y/ EFSA 2007 |
| Laacher Hof AIII, Rhineland, Germany | Loamy silt (LH) | 7.9 |  |  | 27.2 | 90.2 | (1) | 0.955 | SFO | Y/ EFSA 2007 |
| Laacher Hof AXXa, Rhineland, Germany | Sandy loam (LH) | 7.2 |  |  | 8.2 | 27.2 | (1) | 0.959 | SFO | Y/ EFSA 2007 |
| Stanley, Kansas, USA | Silty Clay (ST) | 6.3 |  |  | 46 | 153 | (1) | 0.965 | SFO | Y/ EFSA 2007 |
| Median (n=4) Geometric mean (n=4) |  |  |  |  | 17.7 15.7 |  |  |  |  |  |
| pH-dependency: y/n |  |  |  |  | N |  |  |  |  |  |

(1) Considered identical to DT50. No normalization was deemed necessary in the DAR (2004) and EFSA conclusion (2007)

Table 8.3-3:	Summary of aerobic degradation rates for Prothioconazole-desthio (M04)- laboratory studies

| Prothioconazole-desthio (M04), Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH (water) | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | R2 | Kinetic model | Evaluated on EU level y/n/ Reference |
| Höfchen, Rhineland, Germany | Loamy silt (HF) | 7.3 | 20 | 40 | 34.0 | 113.0 | (1) | 0.820 | SFO | Y/ EFSA 2007 |
| Laacher Hof AXXXa, Rhineland, Germany | Loamy silt (LH) | 7.9 |  |  | 29.6 | 59.2 | (1) | 0.987 | SFO | Y/ EFSA 2007 |
| Laacher Hof AXXXa, Rhineland, Germany | Sandy loam (LH) | 7.2 |  |  | 7.0 | 23.2 | (1) | 0.985 | SFO | Y/ EFSA 2007 |
| Stanley, Kansas, USA | Silty Clay (ST) | 6.3 |  |  | 18.6 | 61.9 | (1) | 0.979 | SFO | Y/ EFSA 2007 |
| Median (n=4) |  |  |  |  | 24.1 |  |  |  |  |  |
| pH-dependency: y/n |  |  |  |  | N |  |  |  |  |  |

(1) Considered identical to DT50. No normalization was deemed necessary in the DAR (2004) and EFSA conclusion (2007)

#### Difenoconazole and its metabolites

Table 8.3-4:	Summary of aerobic degradation rates for difenoconazole - laboratory studies

| Difenoconazole, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | r² | Kinetic model | Evaluated on EU level y/n/ Reference |
| Les Barges (Switzerland) | Loam | 7.2 | 20 | 40 | n=2 geomean = 111 | 345 392 Mean= 368 | 111 | 0.998 to 0.999 | SFO | Y/ EFSA 2011 |
| Neuhofen (SP 229) (Germany) | Sandy loam (1) | 5.0 | 20 | 40 | 123 | 409 | 123 | 0.913 | SFO | Y/ EFSA 2011 |
| Les Evouettes (Switzerland) | Silt loam | 7.2 | 20 | 48 | n=3 geomean = 235 | >277 | 235 | 0.892 to 0.973 | SFO | Y/ EFSA 2011 |
| Les Evouettes | Loam | 7.2 | 20 | 43 | 53 | 175 | 53 | 0.995 | SFO | Y/ EFSA 2011 |
| Schanz (Switzerland) | Sandy loam | 7.4 | 20 | 40 | 149 | 496 | 136 | 0.977 | SFO | Y/ EFSA 2011 |
| Pappelacker (Switzerland) | Sandy loam/Loamy sand | 7.5 | 20 | 40 | 186 | 617 | 177 | 0.939 | SFO | Y/ EFSA 2011 |
| Senozan (France) | Silty clay loam | 6.7 | 20 | 40 | 187 | 620 | 151 | 0.972 | SFO | Y/ EFSA 2011 |
| Geometric mean (n=7) |  |  |  |  |  |  | 130 |  |  |  |
| Median (n=7) |  |  |  |  |  |  | 136 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | N |  |  |  |

(1) “Sand” in the DAR, “Sandy loam” in EFSA (2011)

Table 8.3-5:	Summary of aerobic degradation rates for 1,2,4-triazole (CGA 71019)- laboratory studies

| 1,2,4-triazole (CGA 71019), Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | r2 | Kinetic model | Evaluated on EU level y/n/ Reference |
| Laacherhof AXXa (Germany) | Sandy loam | 6.4 | 20 | 40 | 6.3 | 21 | 5.0 | 0.75 | SFO | Y/ EFSA 2011 |
| Hanhofen (BBA 2.2) (Germany) | Loamy sand | 5.8 | 20 | 40 | 9.9 | 33 | 9.9 | 0.81 | SFO | Y/ EFSA 2011 |
| Laacherhof A III (Germany) | Silt loam | 6.7 | 20 | 40 | 12 | 41 | 8.2 | 0.95 | SFO | Y/ EFSA 2011 |
| Geometric mean (n=3) |  |  |  |  |  |  | 7.4 |  |  |  |
| Median (n=3) |  |  |  |  |  |  | 7.7 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | N |  |  |  |

Table 8.3-6:	Summary of aerobic degradation rates for CGA 205375- laboratory studies

| CGA 205375, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | Chi2 (%) | Kinetic model | Evaluated on EU level y/n/ Reference |
| Schanz (Switzerland) | Sandy loam | 7.4 | 20° | 40 | 93 | 309 | 85 | 0.980 | SFO | Y/ EFSA 2011 |
| Pappelacker (Switzerland) | Sandy loam/ Loamy sand | 7.5 | 20° | 40 | 83 | 275 | 79 | 0.995 | SFO | Y/ EFSA 2011 |
| Senozan (France) | Silt loam | 5.8 | 20° | 40 | 152 | 504 | 123 | 0.996 | SFO | Y/ EFSA 2011 |
| Geometric mean (n=3) |  |  |  |  |  |  | 94 |  |  |  |
| Median (n=3) |  |  |  |  |  |  | 85 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  |  | N |  |  |  |

### Anaerobic degradation in soil (KCP 9.1.1.1)

Not applicable. Due to the proposed use patterns as a foliar fungicide on cereals and oilseed rape, prothioconazole and difenoconazole will not, in general, be exposed to anaerobic conditions.

## Field studies (KCP 9.1.1.2)

### Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)

Relevant endpoints are cited in the EFSA conclusions for, prothioconazole (2007) and difenoconazole (2011) with the following exceptions:

- Endpoints for 1,2,4-triazole in soil were revised by EFSA (revision following confirmatory data: 2011-2013);

- Formation fraction in soil for M04 (prothioconazole-desthio) and the associated degradation of its parent prothioconazole were described only in the RAR (2015:) in a new non-GLP kinetic study based on unprotected legacy data from the DAR (2004). They are used as a refinement of default values in the PECSW (STEP 3) section.

#### Prothioconazole and its metabolites

Triggering endpoints

Table 8.4-6:	Summary of aerobic degradation rates for Prothioconazole - field studies: Triggering endpoints

| Prothioconazole, Field studies – Triggering endpoints |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type (DIN 19682 / USDA) | Location | pH (CaCl2) | Depth (cm) | DissT50 (d) actual | DT90 (d) actual | Kinetic parameters | r2 | Method of calculation | Evaluated on EU level y/n/ Reference |
| Loamy silt / Silt loam | 51399 Burscheid, Höfchen station, Germany | 6.25 | 50 cm (10 cm layers) | 1.9 | 6.4 | SFO: k = ln(2)/DT50) | 1.00 | SFO | Y/ EFSA 2007 |
| Sandy clay loam / Sandy clay loam | Thurston, GB | 7.56 |  | 1.6 2.8 n = 2 | 5.5 9.3 n = 2 |  | 1.00 0.99 | SFO | Y/ EFSA 2007 |
| Weak loamy silt / Silt | Fresnes l’Archevêque, FR | 6.42 |  | 1.3 1.4 n = 2 | 4.4 4.5 n = 2 |  | 1.00 1.00 | SFO | Y/ EFSA 2007 |
| Sandy loamy silt / Silt loam | St Etienne du Gres, FR | 7.61 |  | 1.7 | 5.6 |  | 0.99 | SFO | Y/ EFSA 2007 |
| Weak loamy sand / Sandy loam | Pradelle Di Nogarole Rocca, IT | 7.56 |  | 1.6 | 5.4 |  | 0.99 | SFO | Y/ EFSA 2007 |
| Loamy sand / Sandy loam | Monheim, Germany, Laacherof St. | 6.32 |  | 1.5 | 5.1 |  | 1.00 | SFO | Y/ EFSA 2007 |
| Maximum (n=6/8 (1)) |  |  |  | 2.8 | 9.3 |  |  |  | Y/ EFSA 2007 |

(1) 6 locations, 8 trials

Table 8.4-7:	Summary of aerobic degradation rates for M04 - field studies: Triggering endpoints

| M04, Field studies – Triggering endpoints |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type (DIN 19682 / USDA) | Location | pH (CaCl2) | Depth (cm) | DissT50 (d) actual | DT90 (d) actual | f.f. (1) | Kinetic parameters | r2 | Method of calc. | Evaluated on EU level y/n/ Reference |
| Loamy silt / Silt loam | 51399 Burscheid, Höfchen station, Germany | 6.25 | 50 cm (10 cm layers) | 16.3 | 54.1 | n.c. | SFO: k = ln(2)/DT50) | 0.98 | SFO | Y/ EFSA 2007 |
| Sandy clay loam / Sandy clay loam | Thurston, GB | 7.56 |  | 54.7 50.2 n=2 | 182 167 n=2 | n.c. |  | 0.96 0.91 | SFO | Y/ EFSA 2007 |
| Weak loamy silt / Silt | Fresnes l’Archevêque, FR | 6.42 |  | 47.6 36.8 n=2 | 158 122 n=2 | n.c. |  | 0.94 0.93 | SFO | Y/ EFSA 2007 |
| Sandy loamy silt / Silt loam | St Etienne du Gres, FR | 7.61 |  | 72.3 | 240 | n.c. |  | 0.91 | SFO | Y/ EFSA 2007 |
| Weak loamy sand / Sandy loam | Pradelle Di Nogarole Rocca, IT | 7.56 |  | 30.5 | 101 | n.c. |  | 0.98 | SFO | Y/ EFSA 2007 |
| Loamy sand / Sandy loam | Monheim, Germany, Laacherof St. | 6.32 |  | 27.9 | 92.6 | n.c. |  | 0.98 | SFO | Y/ EFSA 2007 |
| Maximum (n=6) |  |  |  | 72.3 | 240 | n.c. (1) |  |  |  | Y/ EFSA 2007 |

(1) A "mean conversion rate" of 57.1 % is cited in the EU review.

Modelling endpoints

Table 8.4-8:	Summary of aerobic degradation rates for Prothioconazole - field studies: Modelling endpoints

| Prothioconazole, Field studies – Modelling endpoints |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Soil type (DIN 19682 / USDA) | Location | pH (CaCl2) | Depth (cm) | DT50 (d) 20°C, pF2 | Fit, Kinetic | Evaluated on EU level y/n/ Reference |
| Loamy silt / Silt loam | 51399 Burscheid, Höfchen station, Germany | 6.25 | 50 cm (10 cm layers) | 1.2 | SFO | Y/ EFSA 2007 |
| Sandy clay loam / Sandy clay loam | Thurston, GB | 7.56 |  | 0.8 1.4 n=2 | SFO | Y/ EFSA 2007 |
| Weak loamy silt / Silt | Fresnes l’Archevêque, FR | 6.42 |  | 1.6 1.6 n=2 | SFO | Y/ EFSA 2007 |
| Sandy loamy silt / Silt loam | St Etienne du Gres, FR | 7.61 |  | 1.1 | SFO | Y/ EFSA 2007 |
| Weak loamy sand / Sandy loam | Pradelle Di Nogarole Rocca, IT | 7.56 |  | 1.5 | SFO | Y/ EFSA 2007 |
| Loamy sand / Sandy loam | Monheim, Germany, Laacherof St. | 6.32 |  | 0.6 | SFO | Y/ EFSA 2007 |
| Geometric mean (n=8 (1)) |  |  |  | 1.2 | SFO | Y/ EFSA 2007 |
| pH-dependency y/n |  |  |  | N |  |  |

(1) 6 locations, 8 trials

Table 8.4-9:	Summary of aerobic degradation rates for relevant metabolite(s) - field studies: Modelling endpoints

| M04 (prothioconazole-desthio), Field studies – Modelling endpoints |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type (DIN 19682 / USDA) | Location | pH (CaCl2) | Depth (cm) | DT50 (d) 20°C, pF2 | f.f. (1) | Fit, Kinetic | Evaluated on EU level y/n/ Reference |
| Loamy silt / Silt loam | 51399 Burscheid, Höfchen station, Germany | 6.25 | 50 cm (10 cm layers) | 10.3 | n.c. | SFO | Y/ EFSA 2007 |
| Sandy clay loam / Sandy clay loam | Thurston, GB | 7.56 |  | 27.0 23.4 n=2 | n.c. | SFO | Y/ EFSA 2007 |
| Weak loamy silt / Silt | Fresnes l’Archevêque, FR | 6.42 |  | 27.5 20.1 n=2 | n.c. | SFO | Y/ EFSA 2007 |
| Sandy loamy silt / Silt loam | St Etienne du Gres, FR | 7.61 |  | 61.9 | n.c. | SFO | Y/ EFSA 2007 |
| Weak loamy sand / Sandy loam | Pradelle Di Nogarole Rocca, IT | 7.56 |  | 20.7 | n.c. | SFO | Y/ EFSA 2007 |
| Loamy sand / Sandy loam | Monheim, Germany, Laacherof St. | 6.32 |  | 15.2 | n.c. | SFO | Y/ EFSA 2007 |
| Geometric mean (n=8 (2)) |  |  |  | 22.7 | (1) | SFO | Y/ EFSA 2007 |
| pH-dependency y/n |  |  |  | N |  |  |  |

(1) A "mean conversion rate" of 57.1 % is cited in the EU review. It probably can be used as a formation fraction but a value of 1 was used instead at Tier 1 conservatively for PECgw and PECsw.

(2) 6 locations, 8 trials

Table 8.4-10b: Refined modelling endpoints: Summary of laboratory and field normalised DT50 datasets (prothioconazole and M04) in the RAR (2018) (1).

| Study | Soil / trial number and site | DT50 value (days), normalized |  |
| --- | --- | --- | --- |
|  |  | Prothioconazole (2) | JAU 6476-desthio |
| Laboratory studies |  |  |  |
| Gilges, M., Baczinski, P. (2000, rev. 2001) | Laacher Hof | 0.23 | 1000 |
|  | Stanley | 0.53 | 1000 |
| Hellpointer, E. (2001) | Höfchen | 0.24 | 104 |
|  | Byromville | 0.39 | 138 |
| Gilges, M. (2001) | Höfchen | - | 83.9 |
|  | Laacher Hof A III | - | 17.8 |
|  | Laacher Hof A XXa | - | 9.0 |
|  | Stanley | - | 17.6 |
| Field studies |  |  |  |
| Schramel, O. (2001) | R812587 (Germany) | 1.3 | - |
|  | R812595 (UK) | 0.49 | 24.7 |
|  | R812609 (N. France) | 0.72 | 23.8 |
|  | R812617 (UK) | 0.61 | 23.3 |
|  | R812625 (N. France) | 0.72 | 23.8 |
|  | R812633 (S. France) | 0.68 | 32.2 |
|  | R815667 (Italy) | 0.92 | 28.7 |
|  | R815675 (Germany) | 0.76 | 18.8 |
|  | Hypothesis test | Datasets not significantly different | Datasets significantly different |
|  | Geomean | 0.57 | Not combined (Field: geomean = 24.7) |

(1) Table B.8-22 in RAR Volume 3 - B.8 (AS)

(2) DegT50 of prothioconazole must be used in combination with DegT50 of M04 when modelling the formation of the metabolite from its parent substance (both must originate from the same kinetic analysis). The corresponding formation fraction is 0.57 (p. 25 in RAR Volume 3 – B.8 (AS)).

| zRMS Comments: | The submitted refined modelling endpoints were not accepted for further exposure assessment. The new data (RAR, 2018) have not been agreed yet and published in EFSA Journal. This approach could be decided at the Member State level. For prothioconazole and its metabolite M04 the agreed endpoint DT50 (EFSA, 2007) should be used. For consistency the metabolite name JAU 6476-desthio is equivalent to M04. |
| --- | --- |

#### Difenoconazole and its metabolites

Triggering endpoints

Table 8.4-5:	Summary of aerobic degradation rates for difenoconazole - field studies: Triggering endpoints

| Difenoconazolee, Field studies – Triggering endpoints |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil type | Location | pH | Depth (cm) | DissT50 (d) actual | DT90 (d) actual | Kinetic parameters | r2 | Method of calculation | Evaluated on EU level y/n/ Reference |
| silt loam | Germany | 7.4 | 0-20 | 160 | 532 | SFO: k = ln(2)/DT50) | 18.6 | SFO | Y/ EFSA 2007 |
| silt loam | Germany | 6.6 | 0-10 | 20 | 68 |  | 13.0 | SFO | Y/ EFSA 2007 |
| loamy sand | Germany | 6.2 | 0-10 | 59 | 195 |  | 18.3 | SFO | Y/ EFSA 2007 |
| silt loam | Germany | 6.8 | 0-20 | 64 | 211 |  | 14.1 | SFO | Y/ EFSA 2007 |
| loamy sand | Germany | 5.6 | 0-10 | 61 | 202 |  | 14.8 | SFO | Y/ EFSA 2007 |
| sandy loam | Germany | 6.0 | 0-20 | 265 | 879 |  | 18.6 | SFO | Y/ EFSA 2007 |
| silt loam | Germany | 6.0 | 0-20 | 242 | 802 |  | 20.9 | SFO | Y/ EFSA 2007 |
| silt loam | Germany | 5.7 | 0-20 | 118 | 397 |  | 21.8 | SFO | Y/ EFSA 2007 |
| clay loam | Switzerland | 7.3 | 0-10 | 83 | 277 |  | - | SFO | Y/ EFSA 2007 |
| Maximum (n=9) |  |  |  | 265 | 879 |  |  |  | Y/ EFSA 2007 |

1,2,4-triazole (confirmatory data) - modelling endpoints

| Kinetic parameters | Method of calculation | Evaluated on EU level y/n/ Reference |
| --- | --- | --- |
| DegT50 - fast = 1.68 d Proportion - fast = 0.489 | DFOP | Yes: EFSA (2011-2013) |
| DegT50 - slow = 60.5 d Proportion - slow = 0.511 |  |  |

| zRMS Comments: | In accordance with Triazole Derived Metabolite: 1,2,4-Triazole. Proposed revision to DT50 Summary, Scientific Evaluation and Assessment July 2011, revised September 2011 (after comments from MS and EFSA) and further revised January 2013, the relevant parameters for DFOP kinetics were accepted for further assessment. |
| --- | --- |

### Soil accumulation testing (KCP 9.1.1.2.2)

Prothioconazole and metabolites: Not applicable (Soil accumulation testing is not necessary since DT90f values of prothioconazole and prothioconazole-desthio (M04) are less than one year.)

Difenoconazole and metabolites: Four studies were conducted in the EU review. No accumulation was observed after up to 10 years.

## Mobility in soil (KCP 9.1.2)

Studies on mobility in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

All relevant endpoints are described in the EFSA conclusions for prothioconazole (2007) and difenoconazole (2011).

### Prothioconazole and its metabolites

Due to its rapid degradation in soil; no batch equilibrium test could be performed on prothioconazole.

Table 8.5-1:	Summary of soil adsorption/desorption for M01

| Prothioconazole-S-methyl (M01) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH (H2O) | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| Laacher Hof AXXa, Germany | sandy loam | 2.02 | 7.2 | 56.0 | 2772.4 | 0.87 | Y/ EFSA 2007 |
| Höfchen, Germany | silt | 2.14 | 7.1 | 64.1 | 2995.0 | 0.88 | Y/ EFSA 2007 |
| Stanley, USA | silty clay loam | 1.66 | 5.9 | 41.2 | 2484.0 | 0.91 | Y/ EFSA 2007 |
| Byromville, USA | loamy sand | 0.79 | 6.8 | 15.6 | 1973.6 | 0.85 | Y/ EFSA 2007 |
| Arithmetic mean (n=4) |  |  |  |  | 2556.3 | 0.88 | Y/ EFSA 2007 |
| Geometric mean (n=4) |  |  |  |  | 2525.9 |  |  |
| pH-dependency y/n |  |  |  |  | N |  |  |

Table 8.5-11:	Summary of soil adsorption/desorption for M04

| Prothioconazole-desthio (M04) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH (-) | Kd (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| Laacher Hof AXXa, Germany | sandy loam | 2.02 | 7.2 | 12.46 | 616.8 | 0.79 | Y/ EFSA 2007 |
| Höfchen, Germany | silt | 2.14 | 7.1 | 13.38 | 625.3 | 0.83 | Y/ EFSA 2007 |
| Stanley, USA | silty clay loam | 1.66 | 5.9 | 8.90 | 536.4 | 0.83 | Y/ EFSA 2007 |
| Byromville, USA | loamy sand | 0.79 | 6.8 | 4.13 | 523.0 | 0.80 | Y/ EFSA 2007 |
| Arithmetic mean (n=4) |  |  |  |  | 575.4 | 0.81 | Y/ EFSA 2007 |
| Geometric mean (n=4) |  |  |  |  | 573.5 |  |  |
| pH-dependency y/n |  |  |  |  | N |  |  |

### Difenoconazole and its metabolites

Table 8.5-3:	Summary of soil adsorption/desorption for Difenoconazole

| Difenoconazole |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| USA | sand | 0.36 | 7.9 | 12.8 | 3870 | 0.74 | Y/ EFSA 2007 |
| USA | sandy loam | 1.98 | 7.8 | 63.0 | 3520 | 0.76 | Y/ EFSA 2007 |
| USA | silt loam | 1.74 | 6.5 | 54.8 | 3470 | 0.85 | Y/ EFSA 2007 |
| USA | silty clay loam | 0.67 | 6.9 | 47.2 | 7730 | 0.91 | Y/ EFSA 2007 |
| Maryland I, USA | clay | 2.79 | 5.9 | 97.8 | 3470 | 0.89 | Y/ EFSA 2007 |
| Maryland II, USA | sand | 0.52 | 6.5 | 2.1 | 400 | 0.80 | Y/ EFSA 2007 |
| Mississipi, USA | silt loam | 0.58 | 7.5 | 35.0 | 5660 | 0.88 | Y/ EFSA 2007 |
| California, USA | sandy loam | 0.58 | 8.5 | 11.5 | 1960 | 0.94 | Y/ EFSA 2007 |
| Arithmetic mean (n=8) |  |  |  |  | 3760 | 0.85 | Y/ EFSA 2007 |
| Geometric mean (n=8) |  |  |  |  | 2943 |  |  |
| pH-dependency y/n |  |  |  |  | N |  |  |

Table 8.5-4:	Summary of soil adsorption/desorption for CGA 205375

| CGA 205375 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| Speyer 2.2 (DE) | loamy sand | 2.17 | 5.7 | 118 | 5440 | 0.81 | Y/ EFSA 2007 |
| Senozan (FR) | silty clay loam | 1.16 | 6.6 | 45.5 | 3920 | 0.76 | Y/ EFSA 2007 |
| Itingen (CH) | clay | 2.63 | 6.7 | 44.1 | 1680 | 0.76 | Y/ EFSA 2007 |
| Schanz (CH) | sandy loam | 1.17 | 6.8 | 22.6 | 1930 | 0.72 | Y/ EFSA 2007 |
| Mechthildshausen (DE) | loam | 1.22 | 7.6 | 23.6 | 1930 | 0.77 | Y/EFSA 2007 |
| Arithmetic mean (n=5) |  |  |  |  | 2980 | 0.76 | Y/ EFSA 2007 |
| Geometric mean (n=8) |  |  |  |  | 2661 |  |  |
| pH-dependency y/n |  |  |  |  | N |  |  |

Table 8.5-5:	Summary of soil adsorption/desorption for 1,2,4-triazole

| 1,2,4-triazole (CGA 71019) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH (H2O) | Kf (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| Alpaugh, USA | silty clay | 0.7 | 8.8 | 0.83 | 120 | 0.90 | Y/ EFSA 2007 |
| Hollister, US | clay loam | 1.74 | 6.9 | 0.75 | 43 | 0.83 | Y/ EFSA 2007 |
| Lawrenceville, USA | silty clay loam | 0.7 | 7.0 | 0.72 | 104 | 0.92 | Y/ EFSA 2007 |
| Paschappa, USA | sandy loam | 0.81 | 6.9 | 0.72 | 89 | 1.02 | Y/ EFSA 2007 |
| Arithmetic mean (n=) |  |  |  |  | 89 | 0.91 | Y/ EFSA 2007 |
| Geometric mean (n=4) |  |  |  |  | 96.5 |  |  |
| pH-dependency y/n |  |  |  |  | N |  |  |

| zRMS Comments: | The geometric mean of Kfoc was added for both active substances and their metabolites. In accordance with EFSA guidance (2014) the geometric mean of Kfoc is recommended to use in PECgw and PECsw assessment. As the arithmetic mean was agreed at the EU level these values also could be used in further modelling. |
| --- | --- |

### Column leaching (KCP 9.1.2.1)

Summary of soil adsorption/desorption for Prothioconazole

| Prothioconazole |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | OC (%) | pH (H2O) | Kd (mL/g) | Kfoc (mL/g) | 1/n (-) | Evaluated on EU level y/n/ Reference |
| Byromville, USA | Loamy sand | 0.86 | 6.7 | 15.2 | 1765 | 0.9/1 (1) | Y / EFSA (2007) |
| Arithmetic mean (n=1) |  |  |  |  | 1765 |  |  |
| pH-dependency y/n |  |  |  |  | n.a. |  |  |

(1) 0.9 was the default value during the EU review. It was replaced by 1 (the current FOCUS default value).

Values were also determined for M01 (Kfoc = 678 mL/g) and M04 (Kfoc = 678 mL/g) but the agreed EU-endpoints for these metabolites are those from batch equilibrium tests.

### Lysimeter studies (KCP 9.1.2.2)

Prothioconazole and metabolites: No data submitted, none required (EFSA conclusion, 2007)

Difenoconazole and metabolites: No data submitted in the EU review, none required (EFSA conclusion,  2011)

### Field leaching studies (KCP 9.1.2.3)

No data submitted, none required

## Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)

Studies on degradation in water/sediment systems with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

Relevant endpoints are from EFSA conclusions of prothioconazole (2007) and difenoconazole (2011).

Refined formation fraction endpoints (and the associated DegT50 values) were extracted from the new kinetic evaluation in RAR of prothioconazole (2015). They are cited in a new non-GLP kinetic study based on unprotected data from legacy studies in the DAR.

### Prothioconazole and its metabolites

Table 8.6-12:	Summary of degradation in water/sediment of Prothioconazole (EFSA, 2007)

| Prothioconazole Distribution (max.in sediment 23.4 % after 1 day) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water/ sediment system | pH water/ sed. (water/ CaCl2) | DegT50 whole syst. (d) | DegT90 whole syst. (d) | Kinetic, Fit | DissT50 water (d) | DissT90 water (d) | Kinetic, Fit | DissT50 sed. (d) | Kinetic, Fit | Evaluated on EU level y/n/ Reference |
| Hönniger Weiher | 6.6/5.8 | 2.8 k1=0.759 d-1 (1) k2=0.022 d-1 | 76.4 | Hockey Stick | 0.8 | 2.7 | SFO | n.c. | b.c. | y (DAR and EFSA 2007) (2) |
| Anglerweiher | 8.5/7.4 | 1.6 k1=0.424 d-1 k2=0.014 d-1 (3) | 23.6 | Hockey Stick | 1 | 3.4 | SFO | n.c. | n.c. | y (DAR and EFSA 2007) (2) |
| Geometric mean (n=2) |  | n.c. | n.c. |  | n.c. | n.c. |  | n.c. |  | - |

(1) Worst case for the formation of the metabolite M04 and used as modelling endpoint at Tier 1 - STEP 3 for M04 formation

(2) Worst-case endpoints for modelling at Tier 1 were interpreted from the DAR because no FOCUS modelling was presented in the EFSA conclusion

(3) Worst case used for FOCUS modelling of prothioconazole at STEPs 1-2

Table 8.6-13:	Summary of observed metabolites

| M04 Hönniger Weiher & Anglerweiher | Water layer: phenyl-label: max 13.9 – 32.3 %AR, 0- 7 d (n=2) triazole-label: max 9.2 – 31.9 %AR, 1 - d (n=2) Sediment: phenyl-label: max 21.9 – 26.9 %AR, 14 – 59 d (n=2) triazole-label: max 17.7 – 26.9 %AR, 14 – 59 d (n=2) | Evaluated on EU level: y (EFSA 2007) |
| --- | --- | --- |
| 1,2,4-triazole Hönniger Weiher & Anglerweiher | Water layer: triazole-label: max 0.8 – 37.2 %AR, 59 – 121 d (n=2) | Evaluated on EU level: y (EFSA 2007) |

Table 8.6-3:	Summary of degradation in water/sediment for M04: Refined modelling endpoints (from Table B.8-18 in RAR - 2018) (3)

| Water/sediment system | Prothioconazole |  | M04 (JAU 6476-desthio) |  |
| --- | --- | --- | --- | --- |
|  | Kinetic model | DT50 (days) | Kinetic model | DT50 (days) |
| Water phase (dissipation) |  |  |  |  |
| Hönniger Weiher | DFOP | 0.16 (fast) 4.0 (slow) | DFOP | 7.4 |
| Anglerweiher | SFO | 0.71 | SFO | 30.7 |
| Geometric mean | - | 0.34 (fast) 1.7 (slow) | - | 15.1 |
| Sediment phase (dissipation) |  |  |  |  |
| Hönniger Weiher | SFO | 74.6 | - | 1000 (default) |
| Anglerweiher | DFOP | 3.4 (fast) 93.5 (slow) | SFO | 57.0 |
| Geometric mean | - | 15.9 (fast) 83.5 (slow) | - | 239 |
| Whole system (degradation) - parent and metabolite fitting |  |  |  |  |
| Hönniger Weiher | DFOP | 0.39 (fast) 44.3 (slow) | SFO | 87.0 |
| Anglerweiher | DFOP | 1.1 (fast) 45.5 (slow) | SFO | 40.6 |
| Geometric mean | - | 0.63 (fast) (1) 44.9 (slow) | - | 59.4 (2) |
| Whole system (degradation) – metabolite decline curves |  |  |  |  |
| Hönniger Weiher | - | - | - | 1000 (default) |
| Anglerweiher | - | - | SFO | 44.1 |
| Geometric mean | - | - | - | 210 |

(1) 0.63 d is worst case for the formation of the metabolite M04 and used at STEP 3. Corresponding formation fractions are 0.5 in water and sediment.

(2) used at STEP 3

(3) New, non-GLP kinetic analysis based on legacy unprotected data in the DAR

| zRMS Comments: | The submitted refined modelling endpoints were not accepted for further exposure assessment. The new data (RAR, 2018) have not been agreed yet and published in EFSA Journal. This approach could be decided at the Member State level. For prothioconazole and its metabolite M04 the agreed endpoint DT50 (EFSA, 2007) should be used. |
| --- | --- |

### Difenoconazole and its metabolites

Table 8.6-4:	Summary of degradation in water/sediment of Difenoconazole (EFSA, 2011)

| Difenoconazole Distribution (max. water 88/80 % after 0 days) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water/sediment system | pH water /sed. | DegT50 whole syst. (d) | DegT90 whole syst. (d) | Kinetic, Fit (r2) | DissT50 water (d) | DissT90 water (d) | Kinetic, Fit (r2) | DissT50 sed. (d) | Kinetic, Fit | Evaluated on EU level y/n/ Reference |
| Pond | n.c./6.9 | ca 324 | >1000 | SFO -0.998 | 1.0 | 3.3 | SFO - 0.987 | n.c. | n.a. | y EFSA, 2011 |
| River | n.c./7.2 | ca 307 | >1000 | SFO - 0.999 | 2.0 | 6.6 | SFO -0.968 | n.c. | n.a. | y EFSA, 2011 |
| Geometric mean (n=2) |  | 315 | >1000 |  | 1.1 | 4.6 |  |  |  | y EFSA, 2011 |

Table 8.6-5:	Summary of observed metabolites

| CGA 205375 Water/sediment system | Max. in water 96/97 % at day 0 (River/pond) Max in sediment 87-91 % after 28-62 d (River/pond) | Evaluated on EU level y EFSA, 2011 |
| --- | --- | --- |
| 1,2,4-triazole Water/sediment system | Max. in water/sediment 14.1/3.2 % after 148 d (River/pond) | Evaluated on EU level y EFSA, 2011 |

Table 8.6-6:	Summary of degradation in water/sediment of CGA 205375 (EFSA, 2011)

| Difenoconazole Distribution (max. water 96/97 % after 0 days, sediment 91/87 % after 62/28 d) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water/sediment system | pH water /sed. | DegT50 whole syst. (d) | DegT90 whole syst. (d) | Kinetic, Fit (r2) | DissT50 water (d) | DissT90 water (d) | Kinetic, Fit (r2) | DissT50 sed. (d) | Kinetic, Fit | Evaluated on EU level y/n/ Reference |
| Pond | 7.97/7.09 | ca 630 | >1000 | SFO -0.765 | 1.4 | 4.7 | SFO - 0.958 | n.c. | n.a. | y EFSA, 2011 |
| River | 8.1/7.46 | ca 301 | >1000 | SFO - 0.932 | 3.1 | 10.2 | SFO -0.985 | n.c. | n.a. | y EFSA, 2011 |
| Geometric mean (n=2) |  | ca 435 | >1000 |  | 2.1 | 6.9 |  |  |  | y EFSA, 2011 |

## Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)

| zRMS Comments: | Calculations of PECS for active substances, their metabolites and formulation were submitted. The endpoints used for PECs assessment were agreed at the EU level. The risk envelope approach was accepted. The refined application scheme was not considered as it is covered by application scheme presented in Table 8.7-1. This approach could be considered at cMS level. Prothioconazole. The PECs assessment for active substance considering the single and multiple application and input parameters proposed by the Applicant was corrected by evaluator. The PECs accum was not taken into consideration (DT90 below 1 year). The tiered approach presented: Tier 1 – BBCH 25 for cereals and BBCH 14 for oilseed rape; Tier 2 – BBCH 30 for cereals. The following PECs values were obtained in Tier 1 Cereals; BBCH 25; application rate 175 g a.s./ha PECsoil (mg/kg); Cereals (risk envelop). Single application; Multiple applications. Actual; TWA; Actual; TWA. Initial; 0.1867; -; 0.0525. Short term; 24h; 0.1457; 0.1654; 0.0410; 0.0465. 2d; 0.1138; 0.1472; 0.0320; 0.0414. 4d; 0.0693; 0.1185; 0.0195; 0.0333. Long term; 7d; 0.0330; 0.0887; 0.0093; 0.0249. 14d; 0.0058; 0.0522; 0.0016; 0.0147. 21d; 0.0010; 0.0357; 0.0003; 0.0100. 28d; 0.0002; 0.0269; 0.0001; 0.0076. 50d; 0.0000; 0.0157; 0.0000; 0.0044. 100d; 0.0000; 0.0075; 0.0000; 0.0021. Oilseed rape; application rate 150 g a.s./ha PECsoil (mg/kg); Oilseed rape (risk envelop). Single application; Multiple applications. Actual; TWA; Actual; TWA. Initial; 0.1200; -; 0.0438. Short term; 24h; 0.0937; 0.1063; 0.0342; 0.0388. 2d; 0.0731; 0.0946; 0.0267; 0.0345. 4d; 0.0446; 0.0762; 0.0163; 0.0278. Long term; 7d; 0.0212; 0.0570; 0.0077; 0.0208. 14d; 0.0038; 0.0335; 0.0014; 0.0122. 21d; 0.0007; 0.0230; 0.0002; 0.0084. 28d; 0.0001; 0.0173; 0.0000; 0.0063. 50d; 0.0000; 0.0101; 0.0000; 0.0037. 100d; 0.0000; 0.0048; 0.0000; 0.0018. The difference between PECs for single application calculated by the Applicant and evaluator does not effect on final conclusion. The PECs ini values for metabolites of prothioconazole: prothioconazole-desthio (M04) and prothioconazole-S-methyl (M01) were accepted. Compound; Cereals; Oilseed rape. PECs ini mg/kg soil. M01; 0.0301; 0.0209. M04; 0.1088; 0.0751. The following PECs values were obtained in Tier 2 Cereals; BBCH 30; application rate 175 g a.s./ha PECsoil (mg/kg); Cereals (risk envelop). Single application; Multiple applications. Actual; TWA; Actual; TWA. Initial; 0.0467; -; 0.0248. Short term; 24h; 0.0364; 0.0413; 0.0194; 0.0220. 2d; 0.0284; 0.0368; 0.0151; 0.0196. 4d; 0.0173; 0.0296; 0.0092; 0.0157. Long term; 7d; 0.0082; 0.0222; 0.0044; 0.0118. 14d; 0.0015; 0.0130; 0.0008; 0.0069. 21d; 0.0003; 0.0089; 0.0001; 0.0047. 28d; 0.0000; 0.0067; 0.0000; 0.0036. 50d; 0.0000; 0.0039; 0.0000; 0.0021. 100d; 0.0000; 0.0019; 0.0000; 0.0010. For both metabolites the following PECs value were calculated: Compound; Cereals. PECs ini mg/kg soil. M01; nr. M04; 0.0332. For further risk assessment the PECs values for single application will be used as a worse case. Difenoconazole The used endpoints were agreed at the EU level: DT50 for difenoconazole and its metabolite CGA205375 (EFSA, 2011) and for CGA71019/1,2,4-triazole (EFSA, 2013). The submitted PECs assessment for active substance and its metabolites was accepted. The tiered approach presented: Tier 1 – BBCH 25 for cereals and BBCH 14 for oilseed rape; Tier 2 – BBCH 30 for cereals The maximum PECS values for active substance and its metabolites at Tier 1 are presented in following table: Compound; Cereals BBCH 25; Oilseed rape BBCH 14. Multiple application. PECs ini mg/kg soil. Difenoconazole; 0.1178 0.1915*; 0.0810 0.1317*. CGA205375; 0.0118 0.0135*; 0.0080 0.0092*. CGA71019; 0.0048 0.0213*; 0.0033 0.0147*. * - PECs accum In Tier 2 assessment the following PECs values for active substance used in cereals were obtained Compound; Cereals BBCH 30. Multiple application. PECs ini mg/kg soil. Difenoconazole; 0.0355 0.0411*. - PECs accum The risk envelope (Tier 1) approach for metabolites was accepted. Split application in oilseed rape. The proposed split application of formulation was also taken into consideration and was accepted. In Tier 1 the BBCH 14 and in Tier 2 BBCH 20 were considered. The following PECs ini values for both active substances were obtained. Compound; Oilseed rape BBCH 14; Oilseed rape BBCH 20. PECs ini mg/kg soil. Prothioconazole; 0.0600; 0.0412. Difenoconazole; 0.0405 0.0468*; 0.0409 0.0472*. - PECs accum The risk envelope (Tier 1) approach for metabolites was accepted. Formulation. The PECs for formulation was corrected. Crop; Application rate L/ha; Interception %; PECact mg/kg. Cereals winter and spring; 0.70; 20; 0.8624. Rapeseed oil winter and spring; 0.60; 40; 0.5544. These values will be used in further risk assessment. |
| --- | --- |

### Justification for new endpoints

There is no deviation from EU-agreed endpoints.

### Active substance(s) and relevant metabolite(s)

Table 8.7-14:	Input parameters related to application for PECsoil calculations (Tier I)

| Use No. | 1-5 | 6-7 |
| --- | --- | --- |
| Crop | Cereals (risk envelope) | Oilseed rape (risk envelope) |
| Application rate (g as/ha) | Prothioconazole: 175 Difenoconazole: 91 | Prothioconazole: 150 Difenoconazole: 78 |
| Number of applications/interval | 2 / 14 d | 2 / 14 d (4) |
| Crop interception (%) (1) | 20 / 80 (2) | 40 / 80 (3) |
| Depth of soil layer (relevant for plateau concentration) (cm) | 5 cm (no tillage) (5) and 20 cm | 5 cm (no tillage) (5) and 20 cm |

(1) Crop interception according to EFSA Journal;12(5):3662 (2014):Appendix C.

(2) BBCH 25 at first application and BBCH > 30 at second application according to calendars in AppDate 3.06

(3) BBCH 14 at first application and BBCH > 20 at second application according to calendars in AppDate 3.06

(4) An extreme worst case: according to the GAP 1application is allowed in autumn/winter and one application in spring. The worst interpretation is application on 15 Feb followed by application on 1st March.

(5) 5 cm to be considered as screening step, 20 cm is the most relevant value for cereals and OSR

Table 8.7-15:	Input parameter for active substance(s) and relevant metabolite(s) for PECsoil calculation

| Compound | Molecular weight (g/mol) | Max. occurrence (%) | DT50 (days) | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- |
| Prothioconazole | 344.26 | - | Longest 1st order DT50 (field): 2.8 | Y (EFSA, 2007) |
| M01 (prothioconazole-S-methyl) | 358.3 | 14.6 | Longest 1st order DT50 (lab): 46 | Y (EFSA, 2007) |
| M04 (prothioconazole-desthio) | 312.2 | 57.1 (3) | Longest 1st order DT50 (field): 72.3 | Y (EFSA, 2007) |
| Difenoconazole | 406.3 | - | 265, SFO longest DT50 from field studies | Y (EFSA, 2011) |
| CGA 205375 | 350.2 | 11.9 | 123 (max. from lab. studies) (2) | Y (EFSA, 2011) (2) |
| 1,2,4-triazole (1) | 69.1 | 23 | 1000 (worst case) | Y (EFSA, 2011) (except DegT50, see note (1)) |

- 23 % is the value for occurrence in soil in EFSA (2011). In the meantime, the Triazole Derivative Metabolite Group (TDMG) has submitted additional field dissipation data for 1,2,4-triazole as part of confirmatory data requirements, which resulted in a revision of the DT50 values to be used in FOCUS modelling (proposed revision to DT50 for 1,2,4-triazole (July 2011, revised September 2011 and January 2013)), but the max. observed field DT50 value could not be found (only instructions for PECGW modelling ). Furthermore, the DegT50 from confirmatory data could probably not be combined with the value of 23% of occurrence in soil. Thus, a worst-case value of 1000 d was used for PECSOIL in this dossier. In the EFSA conclusions of difenoconazole (2011) no DegT50 was considered (only initial PEC values were calculated).

- Only initial PECSOIL were calculated in the EFSA conclusions (no DegT50 was used)

- "Mean conversion rate" as used in the EU review. The maximum occurrence in soil is 49.7% AR, thus 57.1 is a conservative value

The assumptions taken for difenoconazole, CGA 205375 and 1,2,4-triazole trigger the calculation of PECPLATEAU and PECACCUMULATION, but there is no evidence of accumulation in soil for these substances in the EFSA conclusions of difenoconazole. Therefore, the PECACCU. are very conservative values for ecotox. risk assessment.

The software Escape 2.0 (M. Klein) was used to cope with the different crop interception values at 1st and second applications, but with the following parameters:

Table 8.7-3:	Model parameters (Escape 2)

| Parameter | Value | Comment |
| --- | --- | --- |
| Soil density (g cm-3) | 1.5 | Standard assumption |
| Soil depth for calculation (cm) | 5 | Standard assumption |
| Tillage depth (cm) | 5 and 20 | 20 cm is relevant for cereals and oilseed rape (tilled crops). Values at 5 cm are worst case and given as a screening step. |
| Degradation model | SFO | Single First Order |
| Number of substances in model | Parent only | Only one substance is modelled at a time. Each metabolite is applied as a parent substance using MM ratio and occurrence in soil. |
| Crop interception | Entered by user | Applied by the software to the application rate. |
| Temperature | 20 °C constant | The DegT50 is not modified by the software. |
| Q10 | n.a. | No impact on results |
| Season of application | n.a. | No impact on results |

Values for a single application were interpreted from the graph and are given indicatively for PECinitial only. They are not required for the ecotoxicological risk assessment.

#### Prothioconazole and its metabolites

Table 8.7-4:	PECsoil for prothioconazole on cereals

| PECsoil (mg/kg) |  | Cereals (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.1867 | - | 0.1867 | - |
| Short term | 24h | n.c. | n.c. | 0.1457 | 0.1162 |
|  | 2d | n.c. | n.c. | 0.1138 | 0.1480 |
|  | 4d | n.c. | n.c. | 0.0693 | 0.1191 |
| Long term | 7d | n.c. | n.c. | 0.0330 | 0.0891 |
|  | 14d | n.c. | n.c. | 0.0525 | 0.0541 |
|  | 21d | n.c. | n.c. | 0.0093 | 0.0444 |
|  | 28d | n.c. | n.c. | 0.0016 | 0.0344 |
|  | 50d | n.c. | n.c. | <0.0001 | 0.0194 |
|  | 100d | n.c. | n.c. | <0.0001 | 0.0097 |
| Plateau concentration (x cm) after year x |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

A refinement is necessary, because the regulatory acceptable concentration of Prothioconazole at Tier is 0.104 mg/kg (see section B9). Refinement for cereals is performed in section 8.7.2.3.

Table 8.7-5:	PECsoil for prothioconazole on oilseed rape

| PECsoil (mg/kg) |  | Oilseed rape (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.1200 | - | 0.1200 | - |
| Short term | 24h | n.c. | n.c. | 0.0937 | 0.1068 |
|  | 2d | n.c. | n.c. | 0.0731 | 0.0951 |
|  | 4d | n.c. | n.c. | 0.0446 | 0.0766 |
| Long term | 7d | n.c. | n.c. | 0.0212 | 0.0573 |
|  | 14d | n.c. | n.c. | 0.0437 | 0.0351 |
|  | 21d | n.c. | n.c. | 0.0077 | 0.0304 |
|  | 28d | n.c. | n.c. | 0.0014 | 0.0237 |
|  | 50d | n.c. | n.c. | <0.0001 | 0.0134 |
|  | 100d | n.c. | n.c. | <0.0001 | 0.0067 |
| Plateau concentration (x cm) after year x |  | n.a. | - | n.a. |  |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

A refinement is necessary, because the regulatory acceptable concentration of Prothioconazole is 0.104 mg/kg (see section B9). Refinement for OSR is performed in sections 8.7.2.4 and 8.7.2.5.

PECsoil of metabolites

Table 8.7-6:	PECsoil for prothioconazole-S-methyl (M01) on cereals

| PECsoil (mg/kg) |  | Cereals (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0284 | - | 0.0301 | - |
| Short term | 24h | n.c. | n.c. | 0.0296 | 0.0298 |
|  | 2d | n.c. | n.c. | 0.0292 | 0.0296 |
|  | 4d | n.c. | n.c. | 0.0283 | 0.0292 |
| Long term | 7d | n.c. | n.c. | 0.0271 | 0.0285 |
|  | 14d | n.c. | n.c. | 0.0244 | 0.0273 |
|  | 21d | n.c. | n.c. | 0.0219 | 0.0267 |
|  | 28d | n.c. | n.c. | 0.0197 | 0.0265 |
|  | 50d | n.c. | n.c. | 0.0142 | 0.0239 |
|  | 100d | n.c. | n.c. | 0.0067 | 0.0181 |
| Plateau concentration (x cm) after year x |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

Table 8.7-7:	PECsoil for prothioconazole-S-methyl (M01) on oilseed rape

| PECsoil (mg/kg) |  | Oilseed rape (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0182 | - | 0.0209 | - |
| Short term | 24h | n.c. | n.c. | 0.0205 | 0.0207 |
|  | 2d | n.c. | n.c. | 0.0202 | 0.0205 |
|  | 4d | n.c. | n.c. | 0.0196 | 0.0202 |
| Long term | 7d | n.c. | n.c. | 0.0188 | 0.0198 |
|  | 14d | n.c. | n.c. | 0.0169 | 0.0189 |
|  | 21d | n.c. | n.c. | 0.0152 | 0.0180 |
|  | 28d | n.c. | n.c. | 0.0137 | 0.0177 |
|  | 50d | n.c. | n.c. | 0.0098 | 0.0163 |
|  | 100d | n.c. | n.c. | 0.0046 | 0.0124 |
| Plateau concentration (x cm) after year x |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

Table 8.7-8:	PECsoil for prothioconazole-desthio (M04) on cereals

| PECsoil (mg/kg) |  | Cereals (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.097 | - | 0.1088 | - |
| Short term | 24h | n.c. | n.c. | 0.1077 | 0.1083 |
|  | 2d | n.c. | n.c. | 0.1067 | 0.1077 |
|  | 4d | n.c. | n.c. | 0.1047 | 0.1067 |
| Long term | 7d | n.c. | n.c. | 0.1017 | 0.1052 |
|  | 14d | n.c. | n.c. | 0.0951 | 0.1019 |
|  | 21d | n.c. | n.c. | 0.0889 | 0.0989 |
|  | 28d | n.c. | n.c. | 0.0832 | 0.0966 |
|  | 50d | n.c. | n.c. | 0.0674 | 0.0918 |
|  | 100d | n.c. | n.c. | 0.0417 | 0.0765 |
| Plateau concentration (x cm) after year x |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

A refinement is necessary, because the regulatory acceptable concentration of M04 at Tier is 0.1 mg/kg (see section B9). Refinement is performed in section 8.7.2.3.

Table 8.7-9:	PECsoil for prothioconazole-desthio (M04) on oilseed rape

| PECsoil (mg/kg) |  | Oilseed rape (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0621 | - | 0.0751 | - |
| Short term | 24h | n.c. | n.c. | 0.0744 | 0.0747 |
|  | 2d | n.c. | n.c. | 0.0736 | 0.0744 |
|  | 4d | n.c. | n.c. | 0.0722 | 0.0737 |
| Long term | 7d | n.c. | n.c. | 0.0702 | 0.0726 |
|  | 14d | n.c. | n.c. | 0.0656 | 0.0703 |
|  | 21d | n.c. | n.c. | 0.0614 | 0.0682 |
|  | 28d | n.c. | n.c. | 0.0574 | 0.0661 |
|  | 50d | n.c. | n.c. | 0.0465 | 0.0622 |
|  | 100d | n.c. | n.c. | 0.0288 | 0.0522 |
| Plateau concentration (x cm) after year x |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

#### Difenoconazole and its metabolites

Table 8.7-10:	PECsoil for difenoconazole on cereals

| PECsoil (mg/kg) |  | Cereals (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.097 | - | 0.1178 | - |
| Short term | 24h | n.c. | n.c. | 0.1175 | 0.1177 |
|  | 2d | n.c. | n.c. | 0.1172 | 0.1175 |
|  | 4d | n.c. | n.c. | 0.1166 | 0.0072 |
| Long term | 7d | n.c. | n.c. | 0.1157 | 0.0068 |
|  | 14d | n.c. | n.c. | 0.1136 | 0.1157 |
|  | 21d | n.c. | n.c. | 0.1115 | 0.1147 |
|  | 28d | n.c. | n.c. | 0.1095 | 0.1136 |
|  | 50d | n.c. | n.c. | 0.1034 | 0.1105 |
|  | 100d | n.c. | n.c. | 0.0907 | 0.1042 |
| Plateau concentration (5 cm) after year 10 |  | n.c. | - | 0.0737 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.1916 0.1915 | TWA 21 d: 0.1884 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0184 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.1363 | TWA 21 d: 0.1331 |

A refinement is necessary, because the regulatory acceptable concentration of difenoconazole in soil is 0.05 mg/kg (see section B9). See section 8.7.2.3.

Table 8.7-11:	PECsoil for difenoconazole on oilseed rape

| PECsoil (mg/kg) |  | Oilseed rape (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0625 | - | 0.0810 | - |
| Short term | 24h | n.c. | n.c. | 0.0807 | 0.0809 |
|  | 2d | n.c. | n.c. | 0.0805 | 0.0807 |
|  | 4d | n.c. | n.c. | 0.0801 | 0.0805 |
| Long term | 7d | n.c. | n.c. | 0.0795 | 0.0802 |
|  | 14d | n.c. | n.c. | 0.0780 | 0.0795 |
|  | 21d | n.c. | n.c. | 0.0766 | 0.0788 |
|  | 28d | n.c. | n.c. | 0.0752 | 0.0781 |
|  | 50d | n.c. | n.c. | 0.0710 | 0.0759 |
|  | 100d | n.c. | n.c. | 0.0623 | 0.0713 |
| Plateau concentration (5 cm) after year 10 |  | n.c. | - | 0.0507 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.1316 0.1317 | TWA 21 d: 0.1294 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0127 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0936 | TWA 21 d: 0.0914 |

A refinement is necessary because the regulatory acceptable concentration of difenoconazole is 0.05 mg/kg dw (see section B9). See sections 8.7.2.4 and 8.7.2.5.

PECsoil of metabolites

Table 8.7-12:	PECsoil for CGA 205375 on cereals

| PECsoil (mg/kg) |  | Cereals (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0101 | - | 0.0118 | - |
| Short term | 24h | n.c. | n.c. | 0.0117 | 0.0117 |
|  | 2d | n.c. | n.c. | 0.0116 | 0.0117 |
|  | 4d | n.c. | n.c. | 0.0115 | 0.0116 |
| Long term | 7d | n.c. | n.c. | 0.0113 | 0.0115 |
|  | 14d | n.c. | n.c. | 0.0109 | 0.0113 |
|  | 21d | n.c. | n.c. | 0.0105 | 0.0111 |
|  | 28d | n.c. | n.c. | 0.0101 | 0.0109 |
|  | 50d | n.c. | n.c. | 0.0089 | 0.0104 |
|  | 100d | n.c. | n.c. | 0.0067 | 0.0094 |
| Plateau concentration (5 cm) after year 10 |  | n.c. | - | 0.0017 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0135 | TWA 21 d: 0.0128 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0004 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0122 | TWA 21 d: 0.0115 |

Table 8.7-13:	PECsoil for CGA 205375 on oilseed rape

| PECsoil (mg/kg) |  | Oilseed rape (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0064 | - | 0.0080 | - |
| Short term | 24h | n.c. | n.c. | 0.0080 | 0.0080 |
|  | 2d | n.c. | n.c. | 0.0079 | 0.0080 |
|  | 4d | n.c. | n.c. | 0.0077 | 0.0080 |
| Long term | 7d | n.c. | n.c. | 0.0074 | 0.0079 |
|  | 14d | n.c. | n.c. | 0.0071 | 0.0077 |
|  | 21d | n.c. | n.c. | 0.0069 | 0.0076 |
|  | 28d | n.c. | n.c. | 0.0064 | 0.0074 |
|  | 50d | n.c. | n.c. | 0.0061 | 0.0070 |
|  | 100d | n.c. | n.c. | 0.0046 | 0.0064 |
| Plateau concentration (5/20 cm) after year x |  | n.c. | - | 0.0012 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0092 | TWA 21 d: 0.0088 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0003 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0083 | TWA 21 d: 0.0079 |

Table 8.7-14:	PECsoil for 1,2,4-triazole on cereals

| PECsoil (mg/kg) |  | Cereals (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0204 | - | 0.0048 | 0.0048 |
| Short term | 24h | n.c. | n.c. | 0.0048 | 0.0048 |
|  | 2d | n.c. | n.c. | 0.0048 | 0.0048 |
|  | 4d | n.c. | n.c. | 0.0047 | 0.0048 |
| Long term | 7d | n.c. | n.c. | 0.0047 | 0.0047 |
|  | 14d | n.c. | n.c. | 0.0047 | 0.0047 |
|  | 21d | n.c. | n.c. | 0.0047 | 0.0047 |
|  | 28d | n.c. | n.c. | 0.0047 | 0.0047 |
|  | 50d | n.c. | n.c. | 0.0046 | 0.0047 |
|  | 100d | n.c. | n.c. | 0.0044 | 0.0046 |
| Plateau concentration (5 cm) after year 10 |  | n.c. | - | 0.0165 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0213 | TWA 21 d: 0.0213 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0041 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0089 | TWA 21 d: 0.0089 |

Table 8.7-15:	PECsoil for 1,2,4-triazole on oilseed rape

| PECsoil (mg/kg) |  | Oilseed rape (risk envelop) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0025 | - | 0.0033 | - |
| Short term | 24h | n.c. | n.c. | 0.0033 | 0.0033 |
|  | 2d | n.c. | n.c. | 0.0033 | 0.0033 |
|  | 4d | n.c. | n.c. | 0.0033 | 0.0033 |
| Long term | 7d | n.c. | n.c. | 0.0033 | 0.0033 |
|  | 14d | n.c. | n.c. | 0.0033 | 0.0033 |
|  | 21d | n.c. | n.c. | 0.0032 | 0.0033 |
|  | 28d | n.c. | n.c. | 0.0032 | 0.0033 |
|  | 50d | n.c. | n.c. | 0.0032 | 0.0032 |
|  | 100d | n.c. | n.c. | 0.0031 | 0.0032 |
| Plateau concentration (5 cm) after year 10 |  | n.c. | - | 0.0114 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0147 | TWA 21 d: 0.0147 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0029 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0061 | TWA 21 d: 0.0061 |

#### PECsoil of prothioconazole, M04 and difenoconazole - refined application scheme on cereals BBCH 30+

A restriction of the application pattern on cereals is applied, with application at or after BBCH 30 (instead of BBCH 25 in the risk envelope). According to AppDate software (M. Klein), in the FOCUS crop calendars for Châteaudun, Hamburg, Kremsmünster and Okehampton, the mean interval between BBCH 30 and BBCH 41 (BBCH 40 is not listed) is 13.75 d. Therefore, the condition to make the second application at or after BBCH 40 is implicit due to the minimum spray interval of 14 d.

The input is modified as follows:

| Use No. | 1-5 |
| --- | --- |
| Crop | Cereals (application at or after BBCH 30) |
| Application rate (g as/ha) | Prothioconazole: 175 Difenoconazole: 91 |
| Number of applications/interval | 2 / 14 d |
| Crop interception (%) (1) | 80 / 90 (1) |
| Depth of soil layer (relevant for plateau concentration) (cm) | 20 |

(1) Crop interception according to EFSA Journal;12(5):3662 (2014):Appendix C.

(2) BBCH 30 at first application and BBCH ≥ 40 at second application, see justification above

Table 8.7-16:	Refined PECsoil for prothioconazole on cereals (applic. at or after BBCH 30)

| PECsoil (mg/kg) |  | Cereals (at and after BBCH30) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.0467 | - | 0.0467 | - |
| Short term | 24h | n.c. | n.c. | 0.0364 | 0.0415 |
|  | 2d | n.c. | n.c. | 0.0284 | 0.0370 |
|  | 4d | n.c. | n.c. | 0.0173 | 0.0293 |
| Long term | 7d | n.c. | n.c. | 0.0082 | 0.0223 |
|  | 14d | n.c. | n.c. | 0.0248 | 0.0139 |
|  | 21d | n.c. | n.c. | 0.0044 | 0.0132 |
|  | 28d | n.c. | n.c. | 0.0008 | 0.0105 |
|  | 50d | n.c. | n.c. | <0.0001 | 0.0059 |
|  | 100d | n.c. | n.c. | <0.0001 | 0.0030 |
| Plateau concentration (20 cm) after year 10 |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

Table 8.7-17:	Refined PECsoil for M04 on cereals (applic. at or after BBCH 30)

| PECsoil (mg/kg) |  | Cereals (at and after BBCH30) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0242 | - | 0.0332 | - |
| Short term | 24h | n.c. | n.c. | 0.0329 | 0.0331 |
|  | 2d | n.c. | n.c. | 0.0326 | 0.0329 |
|  | 4d | n.c. | n.c. | 0.0320 | 0.0326 |
| Long term | 7d | n.c. | n.c. | 0.0311 | 0.0322 |
|  | 14d | n.c. | n.c. | 0.0291 | 0.0311 |
|  | 21d | n.c. | n.c. | 0.0272 | 0.0301 |
|  | 28d | n.c. | n.c. | 0.0254 | 0.0292 |
|  | 50d | n.c. | n.c. | 0.0206 | 0.0267 |
|  | 100d | n.c. | n.c. | 0.0127 | 0.0227 |
| Plateau concentration (20 cm) after year 10 |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

Table 8.7-18:	Refined PECsoil for difenoconazole on cereals (applic. at or after BBCH 30)

| PECsoil (mg/kg) |  | Cereals (at and after BBCH30) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0242 | - | 0.0355 | - |
| Short term | 24h | n.c. | n.c. | 0.0354 | 0.0355 |
|  | 2d | n.c. | n.c. | 0.0353 | 0.0354 |
|  | 4d | n.c. | n.c. | 0.0352 | 0.0353 |
| Long term | 7d | n.c. | n.c. | 0.0349 | 0.0352 |
|  | 14d | n.c. | n.c. | 0.0343 | 0.0349 |
|  | 21d | n.c. | n.c. | 0.0336 | 0.0346 |
|  | 28d | n.c. | n.c. | 0.0330 | 0.0343 |
|  | 50d | n.c. | n.c. | 0.0312 | 0.0333 |
|  | 100d | n.c. | n.c. | 0.0274 | 0.0313 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0056 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0411 | TWA 21 d: 0.0401 |

The PECSOIL of M01, CGA 205375 and 1,2,4-triazole were calculated for a risk envelope pattern and do not require refinement.

#### PECsoil of prothioconazole and difenoconazole - refined application scheme on oilseed rape - one split application at BBCH 14+

A restriction of the application pattern on oilseed rape is applied down to a single split application at or after BBCH 14. This is the first out of two refined application schemes which are considered for oilseed rape (the second is detailed in section 8.7.2.5). According to AppDate software (M. Klein), in the FOCUS crop calendars for Châteaudun, Hamburg, Kremsmünster and Okehampton, the mean interval between BBCH 14 and BBCH 20 for oilseed rape is 7.6 d. Therefore, the condition to make the second application at or after BBCH 20 is implicit due to the minimum spray interval of 14 d.

The input is modified as follows:

| Use No. | 6 |
| --- | --- |
| Crop | Oilseed rape (1 split application at after BBCH 14) |
| Application rate (g as/ha) | Prothioconazole: 75+75 Difenoconazole: 39+39 |
| Number of applications/interval | 1 split in 2 / 14 d |
| Crop interception (%) (1) | 40 / 80 (1) |
| Depth of soil layer (relevant for plateau concentration) (cm) | 20 |

(1) Crop interception according to EFSA Journal;12(5):3662 (2014):Appendix C.

(2) BBCH 14 at first application and BBCH ≥ 20 at second application, see justification above

Table 8.7-19:	Refined PECsoil for prothioconazole on OSR (1 split applic. at or after BBCH 14)

| PECsoil (mg/kg) |  | OSR (one split application) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.0600 (1) | - | 0.0600 (1) | - |
| Short term | 24h | n.c. | n.c. | 0.0468 | 0.0534 |
|  | 2d | n.c. | n.c. | 0.0366 | 0.0476 |
|  | 4d | n.c. | n.c. | 0.0223 | 0.0383 |
| Long term | 7d | n.c. | n.c. | 0.0106 | 0.0286 |
|  | 14d | n.c. | n.c. | 0.0219 (2) | 0.0176 |
|  | 21d | n.c. | n.c. | 0.0039 | 0.0152 |
|  | 28d | n.c. | n.c. | 0.0007 | 0.0119 |
|  | 50d | n.c. | n.c. | <0.0001 | 0.0067 |
|  | 100d | n.c. | n.c. | <0.0001 | 0.0033 |
| Plateau concentration (20 cm) after year 10 |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

(1) The max. PEC is reached at first application, due to a combination of a short DT50 and a higher crop interception on second application.

(2) Lower peak at second application

Table 8.7-20:	Refined PECsoil for difenoconazole on OSR (1 split applic. at or after BBCH 14)

| PECsoil (mg/kg) |  | OSR (one split application) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0312 | - | 0.0405 | - |
| Short term | 24h | n.c. | n.c. | 0.0404 | 0.0404 |
|  | 2d | n.c. | n.c. | 0.0403 | 0.0404 |
|  | 4d | n.c. | n.c. | 0.0401 | 0.0403 |
| Long term | 7d | n.c. | n.c. | 0.0397 | 0.0401 |
|  | 14d | n.c. | n.c. | 0.0390 | 0.0397 |
|  | 21d | n.c. | n.c. | 0.0383 | 0.0394 |
|  | 28d | n.c. | n.c. | 0.0376 | 0.0390 |
|  | 50d | n.c. | n.c. | 0.0355 | 0.0379 |
|  | 100d | n.c. | n.c. | 0.0312 | 0.0357 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0063 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0468 | TWA 21 d: 0.0457 |

The PECSOIL of M01, M04, CGA205375 and 1,2,4-triazole were calculated for a risk envelope pattern on OSR and do not require refinement for this crop.

#### PECsoil of prothioconazole and difenoconazole - refined application scheme on oilseed rape - 2 applications at BBCH 20+

A restriction of the application pattern on oilseed rape is applied down to 2 applications at or after BBCH 20 (instead of BBCH 14). This is the second out of two refined application schemes which are considered for oilseed rape (the first is detailed in section 8.7.2.4).

The input is modified as follows:

| Use No. | 6 |
| --- | --- |
| Crop | Oilseed rape (application at or after BBCH 20) |
| Application rate (g as/ha) | Prothioconazole: 150 Difenoconazole: 78 |
| Number of applications/interval | 2 / 14 d |
| Crop interception (%) (1) | 80 / 80 (1) |
| Depth of soil layer (relevant for plateau concentration) (cm) | 20 |

(1) Crop interception according to EFSA Journal;12(5):3662 (2014):Appendix C.

Table 8.7-21:	Refined PECsoil for prothioconazole on OSR (2 applications at BBCH 20+)

| PECsoil (mg/kg) |  | OSR (one split application) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0400 | - | 0.0412 | - |
| Short term | 24h | n.c. | n.c. | 0.0322 | 0.0367 |
|  | 2d | n.c. | n.c. | 0.0251 | 0.0327 |
|  | 4d | n.c. | n.c. | 0.0153 | 0.0273 |
| Long term | 7d | n.c. | n.c. | 0.0073 | 0.0216 |
|  | 14d | n.c. | n.c. | 0.0013 | 0.0130 |
|  | 21d | n.c. | n.c. | 0.0002 | 0.0150 |
|  | 28d | n.c. | n.c. | <0.0001 | 0.0121 |
|  | 50d | n.c. | n.c. | <0.0001 | 0.0069 |
|  | 100d | n.c. | n.c. | <0.0001 | 0.0034 |
| Plateau concentration (20 cm) after year 10 |  | n.a. | - | n.a. | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.a. | n.a. | n.a. | n.a. |

Table 8.7-22:	Refined PECsoil for difenoconazole on OSR (2 applications at BBCH 20+)

| PECsoil (mg/kg) |  | OSR (one split application) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | ≈ 0.0210 | - | 0.0409 | - |
| Short term | n.c. | n.c. | n.c. | 0.0407 | 0.0408 |
|  | n.c. | n.c. | n.c. | 0.0406 | 0.0407 |
|  | n.c. | n.c. | n.c. | 0.0404 | 0.0406 |
| Long term | n.c. | n.c. | n.c. | 0.0401 | 0.0405 |
|  | n.c. | n.c. | n.c. | 0.0394 | 0.0401 |
|  | n.c. | n.c. | n.c. | 0.0387 | 0.0398 |
|  | n.c. | n.c. | n.c. | 0.0380 | 0.0394 |
|  | n.c. | n.c. | n.c. | 0.0358 | 0.0383 |
|  | n.c. | n.c. | n.c. | 0.0314 | 0.0359 |
| Plateau concentration (20 cm) after year 10 |  | n.c. | - | 0.0064 | - |
| PECaccumulation (PECact +PECsoil plateau) |  | n.c. | n.c. | 0.0472 | TWA 21 d: 0.0461 |

The PECSOIL of M01, M04, CGA205375 and 1,2,4-triazole were calculated for a risk envelope pattern on OSR and do not require refinement for this crop.

#### PECsoil of formulation

Table 8.7-23:	PECsoil of formulation on cereals and oilseed rape

| Product density (g/L) | Crop | Application rate (g/ha) | Crop interception (%) | App. Rate on soil (g/ha) | Cumulated app. rate on soil (g/ha) | PECact (mg product /kg) | Equivalent a.s. (mg/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1155 | Cereals (risk envelope) | 808.5 | 20/80 | 646.8 / 161.7 | 808.5 | 1.078 | Prot. 0.233 Dif. 0.121 |
|  | Oilseed rape (risk envelope) | 693.0 | 40/80 | 415.8 / 138.6 | 554.4 | 0.7392 | Prot. 0.160 Dif. 0.083 |

The above values cumulate the two applications without considering any degradation. The long-term PECSOIL of the formulation are not relevant mainly because prothioconazole degrades quickly in soil.

## Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)

| zRMS Comments: | The PECgw assessment for both active substances and their metabolites was submitted. The recommended FOCUS models were used: FOCUS PELMO, FOCUS PEARL and FOCUS MACRO. In case of scenarios not defined for spring OSR, the winter OSR was taken into consideration. The proposed surrogate crops (cabbage, sugar beet) for spring OSR will be decided at national level. The application dates were accepted. Prothioconazole. Calculations of PECGW for active substance and its relevant metabolite were provided in Tier 1 with PUF = 0. The used endpoints were agreed at the EU level. The highest water solubility was taken for parent and its metabolites, this approach was accepted as represents the worst case. The recommended geometric mean of Kfoc should be used in accordance with FOCUS guidance, 2014. As the arithmetic and geometric means are of similar values – the former were accepted. The maximum PECGW values for active substance and its metabolites M01 and M04 were below the trigger value of 0.1 µg/L. Difenoconazole. In PECGW calculations the factor PUF = 0 for active substance and its relevant metabolites were taken into consideration. The recommended geometric mean of Kfoc was used in modelling. The maximum PECgw values for active substance and metabolite CGA 205375 were below the trigger value of 0.1 µg/L in all considered scenarios. CGA 71019. The fast and slow degradation rates were taken into consideration. In accordance with EFSA conclusion considering the PECgw for 1,2,4-triazole is to be below the 0.1 µg/L. For proposed use pattern the PECgw values for metabolite CGA 71019 is below the trigger value. |
| --- | --- |

### Justification for new endpoints

There is no deviation from EU-agreed endpoints. However, formation fractions were set to 1 when no EU-agreed endpoint was available. Endpoints for 1,2,4-triazole were revised by EFSA in 2011-2013 following the submission of confirmatory data.

### Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)

Detailed protocol and results are described in study KCP 9.2.4.1/01 and KCP 9.2.4.1/02.

The product can be applied twice on cereals from BBCH 25 to BBCH 69 in spring and summer at 0.7 L/ha. Four cases were set up to represent the uses on cereals (Table 8.8.1: a, b, c, d).

The product can be applied on oilseed rape once in autumn/winter (up to 0.6 L/ha) and once in spring/summer (up to 0.6 L/ha). Additional cases of split applications are less critical and not considered for modelling. Five cases were set up to represent the uses on oilseed rape (Table 8.8.1: e, f, g, h, i).

Table 8.8-16:	Input parameters related to application for PECgw calculations.

| Use No. | 1-5 | 1-5 |  | 1-5 | 1-5 | 6 |  | 6 |  | 6 |  | 6 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case | a | b |  | c | d | e |  | f |  | g |  | h | i |
| Representative Crop | Winter cereals | Spring Cereals |  | Winter cereals | Spring Cereals | Winter OSR |  | Winter OSR |  | Winter OSR |  | Spring OSR | Spring OSR |
| Surrogate crops1 | n.a. | Winter Cereals |  | n.a. | Winter Cereals | n.a. |  | n.a. |  | n.a. |  | Cabbage, Sugar beet | Cabbage, Sugar beet |
| Application rate (g as/ha) | prothioconazole: 175 g/ha difenoconazole: 91 g/ha |  |  |  |  | prothioconazole: 150 g/ha difenoconazole: 78 g/ha |  |  |  |  |  |  |  |
| Number of applications/interval (d) | 2 / 14 | 2 / 14 |  | 2 / 14 | 2 / 14 | 2 / 14 |  | 2 / variable |  | 2 / variable |  | 1 | 1 |
| Application stage | BBCH 25 = 1st app BBCH 25 + 14 d = 2nd app. |  |  | BBCH 69 – 14 d = 1st app. BBCH 69 = 2nd app. |  | 15 Feb / 1st Mar3 |  | BBCH 14 / BBCH 69 |  | BBCH 14 / 1st March |  | BBCH 14 | BBCH 69 |
| Crop interception (%) | 20 / 80 | 20 / 80 |  | 90 / 90 | 90 / 90 | 80 / 80 |  | 40 / 80 |  | 40 / 80 |  | 40 | 80 |
| Frequency of application | annual |  |  |  |  |  |  |  |  |  |  |  |  |
| Models used for calculation | FOCUS PEARL v4.4.4, FOCUS PELMO v5.5.3, FOCUS MACRO v5.5.4 |  | FOCUS PEARL v4.4.4 |  |  |  | FOCUS PEARL v4.4.4., FOCUS PELMO v5.5.3, FOCUS MACRO v5.5.4 |  | FOCUS PEARL v4.4.4 |  | FOCUS PEARL v4.4.4, FOCUS PELMO v5.5.3, FOCUS MACRO v5.5.4 |  | FOCUS PEARL v4.4.4 |
| Critical use y/n 2 | Yes |  | No |  |  |  | Yes |  | No |  | Yes |  | No |

1 when the crop is not defined for a scenario required in the Central zone. Justification is given in the report.

2 Results from PEARL were used to narrow the range of critical uses for the other models

3 Extreme theoretical worst case of the succession of a winter application with an early spring application

Table 8.8-17:	Application dates used for groundwater risk assessment (source: AppDate 3.06)

| Crop | Scenario | Application dates (absolute) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | a: Winter cereals, BBCH 25 | b: Spring cereals, BBCH 25 | c: Winter cereals, BBCH 69 | d: Spring cereals, BBCH 69 |
| Cereals | Châteaudun | 10 Apr / 24 Apr | 07 Apr / 21 Apr | 31 May / 14 Jun | 08 Jun / 22 Jun |
|  | Hamburg | 29 Apr / 13 May | 21 Apr / 05 May | 08 Jun / 22 Jun | 14 Jun / 28 Jun |
|  | Kremsmünster | 19 Apr / 03 May | 21 Apr / 05 May | 11 Jun / 25 Jun | 14 Jun / 28 Jun |
|  | Jokioinen | 09 May / 23 May | 01 Jun / 15 Jun | 26 Jun / 10 Jul | 03 Jul / 17 Jul |
|  | Okehampton | 16 Apr/ 30 Apr | 17 Apr/ 01 May | 24 May / 07 Jun | 04 Jun / 18 Jun |
|  | Piacenza | 15 Mar / 29 Mar | n.d. 1 | 12 May / 26 May | n.d. 1 |
|  | Porto (AppDate) | 15 Jan / 29 Jan | 07 Apr / 21 Apr | 04 May / 18 May | 08 Jun / 22 Jun |
|  | Porto (moved to March) | 01 Mar / 15 Mar | n.a. | n.a. | n.a. |
|  | Sevilla (AppDate) | 01 Jan / 15 Jan | n.d. | 14 Mar / 28 Mar | n.d. |
|  | Sevilla (moved to March) | 01 Mar / 15 Mar | n.a. | n.a. | n.a. |
|  | Thiva (AppDate) | 06 Jan / 20 Jan | n.d. | 13 Apr / 27 Apr | n.d. |
|  | Thiva (moved to March) | 01 Mar / 15 Mar | n.a. | n.a. | n.a. |

1 Crop not defined for this scenario. Winter cereals is the surrogate crop.

Table 8.8-3:	Application dates used for groundwater risk assessment (source: AppDate 3.06)

| Crop | Scenario | Application dates (absolute) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | e: Winter OSR Feb & Mar | f: Winter OSR, BBCH 14 & 69 | g: Winter OSR, BBCH 14 & 1st March | h: Spring OSR, BBCH 14 | i: Spring OSR, BBCH 69 |
| Oilseed rape | Châteaudun | 15 Feb / 1st Mar | 14 Sep / 14 May | 14 Sep / 01 Mar | 07 May 1 | n.d. 3 |
|  | Hamburg |  | 09 Sep / 30 May | 09 Sep / 01 Mar | 07 May 1 | n.d. 3 |
|  | Kremsmünster |  | 09 Sep / 30 May | 09 Sep / 01 Mar | 07 May 1 | n.d. 3 |
|  | Jokioinen |  | n.d. | n.d. | 22 May | 17 Jul |
|  | Okehampton |  | 21 Aug / 25 May | 21 Aug / 01 Mar | 05 Apr | 13 Jun |
|  | Piacenza |  | 12 Oct / 15 May | 12 Oct / 01 Mar | 14 Apr 2 | n.d. 3 |
|  | Porto |  | 30 Sep / 14 May | 30 Sep / 01 Mar | 30 Mar | 26 Jun |
|  | Sevilla |  | n.d. | n.d. | 26 Mar1 | n.d. |
|  | Thiva |  | n.d. | n.d. | 14 May² | n.d. |

1 Crop not defined for this scenario. Cabbage is the surrogate crop.

2 Crop not defined for this scenario. Sugar Beet is the surrogate crop.

3 Crop not defined for this scenario. Winter OSR is the surrogate crop: see column f

#### Prothioconazole and its metabolites

Table 8.8-4:	Input parameters related to active substance Prothioconazole and metabolites for PECgw calculations

| Compound/ parameter | Prothioconazole | Prothioconazole-S-methyl (M01) | Prothioconazole-desthio (M04) | Value in accordance with EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 344.26 | 358.3 | 312.2 | Y (EFSA, 2007) |
| Water solubility (mg/L) | 2000 (pH 9 and 20°C: highest measured value) | 2000 (parent value) | 2000 (parent value) | Y (worst case in DAR) |
| Saturated vapour pressure (Pa) (20°C) | 0 (value cited is “<< 4 x 10-7 Pa” in EFSA 2007) | 0 (Default) | 0 (Default) | Y (EFSA, 2007) |
| DT50 in soil (d) | 1.2 (geomean) Molar activation energy of 54 kJ/mol | 15.7 (mean lab studies n=4) Molar activation energy of 54 kJ/mol | 22.7 (geomean field, normalized) Molar activation energy of 54 kJ/mol | Y (EFSA, 2007) |
| Kfoc (mL/g)/Kfom | 1765 / 1023.8 | 2556.3 (mean, n = 4 ) / 1482.8 | 575.4 (mean, n = 4) / 333.8 | Y (EFSA, 2007) |
| 1/n | 1 (current FOCUS default) | 0.88 (arithmetic mean, n = 4) | 0.81 (arithmetic mean, n = 4) | Y (EFSA, 2007), but defaultvalue switched to 1 for prothioconazole. |
| Plant uptake factor | 0 (current FOCUS default) | 0 (current FOCUS default) | 0 (current FOCUS default) | default |
| Formation fraction | - | 1 from parent (worst case) | 1 from parent (worst case) (1) | Conservative default value |

(1) A "mean conversion rate" is cited from field studies in the EU review and could be used alternately.

Table 8.8-5:	PECgw for Prothioconazole and metabolite(s) on Winter Cereals (BBCH 25, case a)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Prothioconazole | M01 | M04 |
| Winter Cereals, BBCH 25 and BBCH 25 + 14 d | PEARL 5.5.5 |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 | <0.001 |
|  | Jokioinen | <0.001 | <0.001 | <0.001 |
|  | Okehampton | <0.001 | <0.001 | <0.001 |
|  | Piacenza | <0.001 | <0.001 | <0.001 |
|  | Porto (January) | <0.001 | <0.001 | <0.001 |
|  | Porto (March) | <0.001 | <0.001 | <0.001 |
|  | Sevilla (January) | <0.001 | <0.001 | <0.001 |
|  | Sevilla (March) | <0.001 | <0.001 | <0.001 |
|  | Thiva (Dec/Jan) | <0.001 | <0.001 | <0.001 |
|  | Thiva (March) | <0.001 | <0.001 | <0.001 |
|  | PELMO 5.5.3 |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 | <0.001 |
|  | Jokioinen | <0.001 | <0.001 | <0.001 |
|  | Okehampton | <0.001 | <0.001 | <0.001 |
|  | Piacenza | <0.001 | <0.001 | <0.001 |
|  | Porto (January) | <0.001 | <0.001 | <0.001 |
|  | Porto (March) | <0.001 | <0.001 | <0.001 |
|  | Sevilla (January) | <0.001 | <0.001 | <0.001 |
|  | Sevilla (March) | <0.001 | <0.001 | <0.001 |
|  | Thiva (January) | <0.001 | <0.001 | <0.001 |
|  | Thiva (March) | <0.001 | <0.001 | <0.001 |
|  | MACRO 5.2 (in FOCUS MACRO 5.5.4) |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | <0.001 |

Results for Spring Cereals at BBCH 25 (case b) are identical to those obtained for Winter cereals for all 3 models.

Results for Winter or Spring cereals at BBCH 69 (cases c and d) are identical to those obtained for Winter cereals, with PEARL.

Table 8.8-6:	PECgw for Prothioconazole and metabolite(s) on Winter oilseed Rape (BBCH 14 + BBCH 69, case f)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Prothioconazole | M01 | M04 |
| Winter Oilseed Rape, BBCH 14 + BBCH 69 | PEARL 5.5.5 |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 | <0.001 |
|  | Jokioinen | n.d. Spring OSR: < 0.001 | n.d. Spring OSR: < 0.001 | n.d. Spring OSR: < 0.001 |
|  | Okehampton | <0.001 | <0.001 | <0.001 |
|  | Piacenza | <0.001 | <0.001 | <0.001 |
|  | Porto | <0.001 | <0.001 | <0.001 |
|  | Sevilla1 | < 0.0011 | < 0.0011 | < 0.0011 |
|  | Thiva² | < 0.001² | < 0.001² | < 0.001² |
|  | PELMO 5.5.3 |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 | <0.001 |
|  | Jokioinen | n.d. Spring OSR: < 0.001 | n.d. Spring OSR: < 0.001 | n.d. Spring OSR: < 0.001 |
|  | Okehampton | <0.001 | <0.001 | <0.001 |
|  | Piacenza | <0.001 | <0.001 | <0.001 |
|  | Porto | <0.001 | <0.001 | <0.001 |
|  | Sevilla1 | < 0.0011 | < 0.0011 | < 0.0011 |
|  | Thiva² | < 0.001² | < 0.001² | < 0.001² |
|  | MACRO 5.2 (in FOCUS MACRO 5.5.4) |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | <0.001 |

1 Surrogate crop = cabbage/leafy vegetable

² Surrogate crop = sugarbeet

Results for Spring OSR at BBCH 14 (case h) are identical to those obtained for Winter OSR at BBCH 14+69 for all 3 models.

Results with PEARL for the remaining cases related to oilseed rape (cases e, g, i) are identical to those obtained with PEARL for Winter OSR at BBCH 14+69.

#### Difenoconazole and its metabolites

Reference is EFSA conclusions (2011) except cited otherwise and for 1,2,4-triazole: revision following confirmatory data (2011-2013) as described in the CTGB guidance document.

Table 8.8-7:	Input parameters related to active substance Difenoconazole and metabolite(s) for PECgw calculations

| Compound/ parameter | Difenoconazole | CGA 205375 | 1,2,4-Triazole (CGA71019) formed from CGA 205375 | 1,2,4-Triazole (CGA71019) applied as parent |
| --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 406.3 | 350.2 | 69.1 | 69.1 (MM ratio = 69.1 / 406.3 applied to applic. rate) |
| Water solubility (mg/L) | 15 at 25 °C | 12 at 25 °C (from DAR Vol. B.2.) | 700 000 at 20 °C (from DAR Vol. B.2.) | 700 000 at 20 °C (from DAR Vol. B.2.) |
| Saturated vapour pressure (Pa) (20°C) | 3.32 x 10-8 Pa at 25°C | 0 (default) | 0 (default) | 0 (default) |
| DT50 in soil (d) – | 130 d (geomean normalised) Molar activation energy of 54 kJ/mol2 | 94 d (geomean normalized) Molar activation energy of 54 kJ/mol2 | Revised endpoint for triazole metabolite (DFOP) DT50 fast = 1,68 d DT50 slow = 60.5 d Molar activation energy of 65.4 kJ/mol | Revised endpoint for triazole metabolite (DFOP) DT50 fast = 1,68 d DT50 slow = 60.5 d Molar activation energy of 65.4 kJ/mol |
| Kfoc (mL/g)/Kfom – Geometric mean | 2942.90 / 1707.02 | 2661.10 / 1543.56 | 83.13 / 48.22 | 83.13 / 48.22 |
| 1/n | 0.85 (mean) | 0.8 (mean) | 0.916 (CTGB guidance doc.) 4 | 0.916 (CTGB guidance doc.) 4 |
| Plant uptake factor | 0 (current FOCUS default) | 0 (current FOCUS default) | 0 (current FOCUS default) | 0 (current FOCUS default) |
| Formation fraction | - | 1 from difenoconazole (worst case in the absence of data) | 0.489 from CGA 205375 3 (fast compartment) + 0.511 from CGA 205375 3 (slow compartment) | 0.489 (applied to applic. rate) for fast comp. 0.511 (applied to applic. rate) for slow comp. |

Results are shown below only for crop and application scenarios deemed as critical in Table 8.8.1. Results for other non-critical cases are available in the report and either equivalent or lower.

Table 8.8-8:	PECgw for Difenoconazole and metabolite(s) on Winter Cereals (BBCH 25, case a)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA205375 | 1,2,4-triazole formed from CGA 205375 | 1,2,4-triazole applied as a parent substance |
| Winter Cereals, BBCH 25 and BBCH 25 + 14 d | PEARL 4.4.4 |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | 0.0166 | 0.0111 |
|  | Hamburg | <0.001 | <0.001 | 0.0768 | 0.0653 |
|  | Kremsmünster | <0.001 | <0.001 | 0.0511 | 0.0470 |
|  | Jokioinen | <0.001 | <0.001 | 0.0589 | 0.0235 |
|  | Okehampton | <0.001 | <0.001 | 0.0730 | 0.0656 |
|  | Piacenza | <0.001 | <0.001 | 0.0424 | 0.0300 |
|  | Porto (January) | <0.001 | <0.001 | 0.0426 | 0.0396 |
|  | Porto (March) | <0.001 | <0.001 | 0.0426 | 0.0217 |
|  | Sevilla (January) | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Sevilla (March) | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Thiva (January) | <0.001 | <0.001 | 0.0227 | 0.0071 |
|  | Thiva (March) | <0.001 | <0.001 | 0.0229 | 0.0048 |
|  | PELMO 5.5.3 |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | 0.0120 | 0.008 |
|  | Hamburg | <0.001 | <0.001 | 0.0875 | 0.07 |
|  | Kremsmünster | <0.001 | <0.001 | 0.0565 | 0.051 |
|  | Jokioinen | <0.001 | <0.001 | 0.038 | 0.03 |
|  | Okehampton | <0.001 | <0.001 | 0.0780 | 0.066 |
|  | Piacenza | <0.001 | <0.001 | 0.0525 | 0.0365 |
|  | Porto (January) | <0.001 | <0.001 | 0.0700 | 0.067 |
|  | Porto (March) | <0.001 | <0.001 | 0.0705 | 0.04 |
|  | Sevilla (January) | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Sevilla (March) | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Thiva (January) | <0.001 | <0.001 | 0.006 | 0.0035 |
|  | Thiva (March) | <0.001 | <0.001 | 0.006 | 0.0015 |
|  | MACRO 5.2 (in FOCUS MACRO 5.5.4) |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | n.c. | n.c. |

Table 8.8-9:	PECgw for Difenoconazole and metabolite(s) on Spring Cereals (BBCH 25, case b)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA205375 | 1,2,4-triazole formed from CGA 205375 | 1,2,4-triazole applied as a parent substance |
| Spring Cereals, BBCH 25 and BBCH 25 + 14 d | PEARL 4.4.4 |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | 0.0138 | 0.0090 |
|  | Hamburg | <0.001 | <0.001 | 0.0865 | 0.0756 |
|  | Kremsmünster | <0.001 | <0.001 | 0.0531 | 0.0490 |
|  | Jokioinen | <0.001 | <0.001 | 0.0568 | 0.0250 |
|  | Okehampton | <0.001 | <0.001 | 0.0696 | 0.0599 |
|  | Piacenza | Crop not defined. Winter cereals is the surrogate crop, see previous Table. |  |  |  |
|  | Porto | <0.001 | <0.001 | 0.0452 | 0.0236 |
|  | Sevilla | n.d. | n.d. | n.d. | n.d. |
|  | Thiva | n.d. | n.d. | n.d. | n.d. |
|  | PELMO 5.5.3 |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | 0.0205 | 0.0055 |
|  | Hamburg | <0.001 | <0.001 | 0.0825 | 0.0635 |
|  | Kremsmünster | <0.001 | <0.001 | 0.0545 | 0.048 |
|  | Jokioinen | <0.001 | <0.001 | 0.027 | 0.0250 |
|  | Okehampton | <0.001 | <0.001 | 0.0855 | 0.0575 |
|  | Piacenza | Crop not defined. Winter cereals is the surrogate crop, see previous Table. |  |  |  |
|  | Porto | <0.001 | <0.001 | 0.0795 | 0.033 |
|  | Sevilla | n.d. | n.d. | n.d. | n.d. |
|  | Thiva | n.d. | n.d. | n.d. | n.d. |
|  | MACRO 5.2 (in FOCUS MACRO 5.5.4) |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | n.c. | n.c. |

Table 8.8-10:	PECgw for Difenoconazole and metabolite(s) on Winter Oilseed Rape (BBCH 14 + BBCH 69, case f)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA205375 | 1,2,4-triazole formed from CGA 205375 | 1,2,4-triazole applied as a parent substance |
| Winter OSR BBCH 14 & BBCH 69 | PEARL 5.5.5 |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | 0.0131 | 0.0156 |
|  | Hamburg | <0.001 | <0.001 | 0.0489 | 0.0575 |
|  | Kremsmünster | <0.001 | <0.001 | 0.0322 | 0.0339 |
|  | Jokioinen | n.d. | n.d. | n.d. | n.d. |
|  | Okehampton | <0.001 | <0.001 | 0.0448 | 0.0459 |
|  | Piacenza | <0.001 | <0.001 | 0.0218 | 0.0284 |
|  | Porto | <0.001 | <0.001 | 0.0291 | 0.0403 |
|  | Sevilla | n.d. | n.d. | n.d. | n.d. |
|  | Thiva | n.d. | n.d. | n.d. | n.d. |
|  | PELMO 5.5.3 |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | 0.012 | 0.0145 |
|  | Hamburg | <0.001 | <0.001 | 0.051 | 0.0635 |
|  | Kremsmünster | <0.001 | <0.001 | 0.034 | 0.0375 |
|  | Jokioinen | n.d. | n.d. | n.d. | n.d. |
|  | Okehampton | <0.001 | <0.001 | 0.054 | 0.0565 |
|  | Piacenza | <0.001 | <0.001 | 0.026 | 0.0325 |
|  | Porto | <0.001 | <0.001 | 0.0495 | 0.0585 |
|  | Sevilla | n.d. | n.d. | n.d. | n.d. |
|  | Thiva | n.d. | n.d. | n.d. | n.d. |
|  | MACRO 5.2 (in FOCUS MACRO 5.5.4) |  |  |  |  |
|  | Châteaudun | <0.001 | <0.001 | n.c. | n.c. |

Table 8.8-11:	PECgw for Difenoconazole and metabolite(s) on Spring Oilseed Rape (BBCH 14, case h)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA205375 | 1,2,4-triazole formed from CGA 205375 | 1,2,4-triazole applied as a parent substance |
| Spring OSR BBCH 14 | PEARL 5.5.5 |  |  |  |  |
|  | Châteaudun1 | <0.001 | <0.001 | 0.0046 | 0.0092 |
|  | Hamburg1 | <0.001 | <0.001 | 0.0115 | 0.0228 |
|  | Kremsmünster1 | <0.001 | <0.001 | 0.0074 | 0.0154 |
|  | Jokioinen | <0.001 | <0.001 | 0.0079 | 0.0088 |
|  | Okehampton | <0.001 | <0.001 | 0.0120 | 0.0232 |
|  | Piacenza2 | <0.001 | <0.001 | 0.0099 | 0.0182 |
|  | Porto | <0.001 | <0.001 | 0.0072 | 0.0098 |
|  | Sevilla1 | <0.001 | <0.001 | 0.0009 | 0.0008 |
|  | Thiva² | <0.001 | <0.001 | 0.0072 | 0.0093 |
|  | PELMO 5.5.3 |  |  |  |  |
|  | Châteaudun1 | <0.001 | <0.001 | 0.0035 | 0.0007 |
|  | Hamburg1 | <0.001 | <0.001 | 0.0115 | 0.024 |
|  | Kremsmünster1 | <0.001 | <0.001 | 0.0075 | 0.0165 |
|  | Jokioinen | <0.001 | <0.001 | 0.004 | 0.009 |
|  | Okehampton | <0.001 | <0.001 | 0.0125 | 0.025 |
|  | Piacenza2 | <0.001 | <0.001 | 0.0110 | 0.0255 |
|  | Porto | <0.001 | <0.001 | 0.0115 | 0.0165 |
|  | Sevilla1 | <0.001 | <0.001 | 0.0005 | 0.0005 |
|  | Thiva² | <0.001 | <0.001 | 0.002 | 0.006 |
|  | MACRO 5.2 (in FOCUS MACRO 5.5.4) |  |  |  |  |
|  | Châteaudun1 | <0.001 | <0.001 | n.c. | n.c. |

1 Surrogate crop cabbage / leafy vegetable

2 Surrogate crop sugar beet

#### Conclusion for PECGW

The 80th percentile of predicted concentration in groundwater at 1 m depth does not exceed 0.1 µg/L for the uses of product AVTAR on cereals and oilseed rape in every FOCUS scenario relevant to the Central zone for the following substances: Prothioconazole, prothioconazole-S-methyl (M01), prothioconazole-desthio (M04), difenoconazole, CGA 205375 and 1,2,4-triazole.

## Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)

| zRMS Comments: | The submitted PECsw and PECsed calculations/results were accepted. In case of scenarios not defined for spring cereals and OSR, the winter cereals and winter OSR, respectively, were taken into consideration. The use of proposed other surrogate crops will be decided at national level. The recommended FOCUS models were used: FOCUS Step 1 & 2 and Step 3; mitigation measures were proposed in Step 4 (SWAN model and VFSmod were used). The proposed application dates were accepted. Prothioconazole and its metabolite. The endpoints agreed at the EU level (EFSA, 2007) were used in PECsw and PECsed assessment. For prothioconazole the PECsw assessment in Step 2 was sufficient. For prothioconazole metabolite M04 the PECsw and PECsed assessment was provided in Step 1 & 2 and Step 3 and Step 4 (SWAN model and VFSmod). The mitigation measures were proposed. In accordance with FOCUS Landscape guidance, the only 10 m and 20 m vegetative buffer strip were accepted. The other width of vegetative buffer strips will be decided at the national level. The proposed surrogate crop for spring cereals was accepted. The following mitigation measures were proposed: Prothioconazole-desthio (M04). Crop; Application rate g a.s./ha; Vegetative strip (m); No spray buffer (m); Central Zone Max PECsw (μg/L) SWAN model; Central Zone Max PECsw (μg/L) VFSmod. Winter cereals BBCH 25; 2 x 175; 20; 20; 0.2489 R4 stream. 10; -; 0.2612 R3 stream. Winter cereals BBCH 25; 1 x 175; 10; 10; 0.1957 R4 stream. 1; -; 0.2417 R3 stream. Winter cereals BBCH < 69; 2 x 175; 10; 10; 0.2798 R4 stream. 2*; -; < 0.3123 R3 stream. Winter cereals BBCH < 69; 1 x 175; 10; 10; 0.2798 R4 stream. 1; -; 0.2712 R4 stream. Spring cereals BBCH 25; 2 x 175; 20; 20; 0.2489 R4 stream. 10; -; 0.2612** R3 stream. Spring cereals BBCH 25; 1 x 175; 10; 10; 0.2950 R4 stream. 1; -; 0.2631** R4 stream. Winter OSR BBCH 14; 2 x 150; 20; 20; 0.1940 R4 stream. 5; -; 0.2298 R3 stream. Winter OSR BBCH 14; 1 x 150; 10; 10; 0.2923 R3 stream. 5; -; 0.2298 R3 stream. Spring OSR BBCH 14; 1 x 150; 20; 20; 0.1843 R4 stream. 1; -; 0.2955 R4 stream. * as 2 m VFS is not accepted, the 5m VFS is recommended, to be considered at cMS level ** winter cereals as a surrogate crop for spring one was usd. For the other prothioconazole metabolite M13 the following PECsw and PECsed values were assessed in Step 2 (North EU as representative for Central Zone): Compound; Cereals; Oilseed rape. PECsw μg/L. M13 1,2,4-Triazole; 0.32; 0.36. PECsed μg/kg. 0.28; 0.32. In accordance with the comment, the PECsw assessment (Step 2) for metabolite M01 was added. Compound; Cereals; Oilseed rape. PECsw μg/L. M01; 1.48; 1.53. PECsed μg/kg. 28.01; 35.19. Difenoconazole. All used endpoints for active substance and its metabolites were agreed at the EU level (EFSA, 2011 and EFSA, 2013 for 1,2,4-triazole) and accepted. The mitigation measures were proposed. SWAN model Crop; Application rate g a.s./ha; Vegetative strip (m); No spray buffer (m); Central Zone Max PECsw (μg/L); Central Zone Max PECsed (μg/kg). Winter cereals BBCH 25; 2 x 91; nr; 0.5011 D3 ditch; nr. Winter cereals BBCH 25; 1 x 91; -; 5; 0.1549 D3 ditch; nr. Winter cereals BBCH < 69; 2 x 91; 10; -; nr; 1.192 R4 stream. Winter cereals BBCH < 69; 1 x 91; -; 5; 0.1554 D3 ditch; nr. Spring cereals BBCH 25; 2 x 91; 10; -; 0.3274 R4 stream; 1.183 R4 stream. Spring cereals BBCH 25; 1 x 91; -; 5; 0.1550 D3 ditch; nr. Winter OSR BBCH 14; 2 x 78; 10; -; nr; 0.9482 R3 stream. Winter OSR BBCH 14; 1 x 78; nr; nr; nr. Spring OSR BBCH 14; 1 x 78; nr; nr; nr. nr – not relevant VFSmod model (relevant for R scenarios) Crop; Application rate g a.s./ha; Vegetative strip (m); No spray buffer (m); Central Zone Max PECsw (μg/L). Winter cereals BBCH < 69; 2 x 91; 1; -; 4.138 R4 stream. Spring cereals BBCH 25; 2 x 91; 1; -; 3.611 R4 stream. Spring cereals BBCH 25; 1 x 91; 2 (5); -; 4.338 R3 stream. Winter OSR BBCH 14; 2 x 78; 1; -; 3.132 R3 stream. For the difenoconazole metabolites the following PECsw and PECsed values were assessed in Step 2 (North EU as representative for Central Zone): Compound; Cereals; Oilseed rape. PECsw μg/L. CGA 205375; 0.41; 0.63. 1,2,4-Triazole; 0.46; 0.73. PECsed μg/kg. CGA 205375; 10.58; 16.37. 1,2,4-Triazole; 0.39; 0.61. 1,2,4-Triazole. This metabolite is a common one for both active substances. In this case the concentration in surface water of 1,2,4-Triazole is a sum of concentration of 1.2.4-Triazole and CGA 71019. Compound; Cereals; Oilseed rape. PECsw μg/L. 1,2,4-Triazole; 0.78; 1.09. PECsed μg/kg. 1,2,4-Triazole; 0.67; 0.93. These PEC values of summed up concentration in surface water and sediments will be used in further risk assessment. National assessment for Poland. All relevant scenarios were taken into consideration. According to national requirements only scenarios D3, D4 and R1 were considered. The proposed surrogate crop was accepted. SWAN model. Prothioconazole-desthio (M04) Crop; Application rate g a.s./ha; Vegetative strip (m); No spray buffer (m); Poland. Winter cereals BBCH 25; 2 x 175; 20; 20; 0.2245 R1 stream. Winter cereals BBCH 25; 1 x 175; nr*; 0.3049 R1 stream. Winter cereals BBCH < 69; 2 x 175; 10; 10; 0.1653 R1 stream. Winter cereals BBCH < 69; 1 x 175; 10; 10; 0.1653 R1 stream. Spring cereals BBCH 25; 2 x 175; 20; 20; 0.2245 R1 stream. Spring cereals BBCH 25; 1 x 175; nr; 0.3049 R1 stream. Winter OSR BBCH 14; 2 x 150; nr; 0.2904 R1 stream. Winter OSR BBCH 14; 1 x 150; nr; 0.2257 R1 stream. Spring OSR BBCH 14; 1 x 150; 10; 10; 0.1590 R1 stream. * - nr means that PECsw was assessed in Step 3 SWAN model. Difenoconazole Crop; Application rate g a.s./ha; Vegetative strip (m); No spray buffer (m); Poland Max PECsw (μg/L); Poland Max PECsed (μg/kg). Winter cereals BBCH 25; 2 x 91; nr; 0.5011 D3 ditch; nr. Winter cereals BBCH 25; 1 x 91; -; 5; 0.1549 D3 ditch; nr. Winter cereals BBCH < 69; 2 x 91; nr; 0.5027 D3 ditch; nr. Winter cereals BBCH < 69; 1 x 91; -; 5; 0.1554 D3 ditch; nr. Spring cereals BBCH 25; 2 x 91; nr; 0.5015 D3 ditch; nr. Spring cereals BBCH 25; 1 x 91; -; 5; 0.1550 D3 ditch; nr. Winter OSR BBCH 14; 2 x 78; nr; 0.4307 D3 ditch. Winter OSR BBCH 14; 1 x 78; nr; 0.4929 D3 ditch; nr. Spring OSR BBCH 14; 1 x 78; nr; 0.4910 D3 ditch; nr. nr means that PECsw was assessed in Step 3 VFSmod. Crop; Appl. rate g a.s./ha; Veget. strip (m); No spray buffer (m); Poland; Appl. rate g a.s./ha; Veget. strip (m); No spray buffer (m); Poland Max PECsw (μg/L). Prothioconazole-desthio; Difenoconazole. Winter cereals BBCH 25; 2 x 175; 1; 0.1801 R1 stream; 2 x 91; nr; nr; 0.5011 D3 ditch. Winter cereals BBCH 25; 1 x 175; nr*; 0.3049 R1 stream; 1 x 91; 5; 0.1549 D3 ditch. Winter cereals BBCH < 69; 2 x 175; 1; -; 0.2436 R1 stream; 2 x 91; nr; 0.5027 D3 ditch. Winter cereals BBCH < 69; 1 x 175; 1; -; 0.1313 R1 stream; 1 x 91; 5; 0.1554 D3 ditch. Spring cereals BBCH 25; 2 x 175; 1; -; 0.1801 R1 stream; 2 x 91; nr; 0.5015 D3 ditch. Spring cereals BBCH 25; 1 x 175; nr; 0.3049 R1 stream; 1 x 91; 5; 0.1550 D3 ditch. Winter OSR BBCH 14; 2 x 150; nr; 0.2904 R1 stream; 2 x 78; nr; 0.4307 D3 ditch. Winter OSR BBCH 14; 1 x 150; nr; 0.2612 D3 ditch; 1 x 78; nr; 0.4929 D3 ditch. Spring OSR BBCH 14; 1 x 150; 1; -; 0.1126 R1 stream; 1 x 78; nr; 0.4910 D3 ditch. * nr means that PECsw was assessed in Step 3 Formulation. The drift exposure was corrected by evaluator using the Drift Calculator in SWASH model and single application of formulation. Crop; - Application rate - g formulation/ha; - Buffer strip - m; - PECsw - µg/L. Winter/Spring cereals; - 808.5; - 1; - 5.194. - 5; - 1.408. - 10; - 0.747. Winter/Spring OSR; - 693.0; - 1; - 4.452. - 5; - 1.207. - 10; - 0.640. The relevant PECsw and PECsed values will be used in further risk assessment. The relevant mitigation measure will be recommended in ecotoxicological section. |
| --- | --- |

Detailed protocol and results are described in study KCP 9.2.5/01 and KCP 9.2.5/02.

### Justification for new endpoints

There is no deviation from EU-agreed endpoints

### Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)

Table 8.9-18:	Input parameters related to application for PECSW/SED calculations

| Plant protection product | AVTAR |  |
| --- | --- | --- |
| Use No. | 1, 3, 5, 6 and 7 | 8 |
| Crop | Wheat (covering barley, triticale, rye and oat) | OSR |
| Application rate (g as/ha) | Prothioconazole: 175 Difenoconazole: 91 | Prothioconazole: 150 Difenoconazole: 78 |
| Number of applications/interval (d) | 2/14 | 2/14 |
| Application window | Northern EU, Mar-May and Southern EU, Mar-May (relevant for STEP 1 and 2 only) | Northern EU, Oct – Feb and Southern EU, Oct - Feb Following Appdate , the application period is between September and May (BBCH 14-69). At step 2, the Oct-Feb senario in Northen EU is the worst case with 5% of soil residue (runoff/drainage input). Therefore, only the results for the period of Oct-Feb are presented since they are covering the whole period. (relevant for STEP 1 and 2 only) |
| Application method | Foliar spray |  |
| CAM (Chemical application method) | CAM 2 (foliar spray) |  |
| Soil depth (cm) | 4 = default value |  |
| Models used for calculation | FOCUS (v. 3.2.) STEPs 1-2, FOCUS SWASH v3.1, FOCUS PRZM v3.3.1, FOCUS MACRO v5.5.3, FOCUS TOXWA v3.3.1 |  |

Table 8.9-19:	FOCUS Step 3 Scenario related input parameters for PECsw/sed calculations for the application of the product AVTAR (source: AppDate 3.06)

| Crop | Application scenario | Scenario | Application window used in modelling (Julian day) Length = 30 d + (number of app. -1) x interval | Min. interval between applications(day) |
| --- | --- | --- | --- | --- |
| Winter Cereals | 2 app. BBCH25 at 1st app. | D3 | 101-145 | 14 |
|  |  | D4 | 72-116 |  |
|  |  | D5 | 69-113 |  |
|  |  | R1 | 109-153 |  |
|  |  | R3 | 73-117 |  |
|  |  | R4 | 6-50 |  |
|  | 1 app. BBCH25 at 1st app. | D3 | 101-131 | / |
|  |  | D4 | 72-102 |  |
|  |  | D5 | 69-99 |  |
|  |  | R1 | 109-139 |  |
|  |  | R3 | 73-103 |  |
|  |  | R4 | 6-36 |  |
|  | 2 app. BBCH69 at last app. | D3 | 168-212 | 14 |
|  |  | D4 | 146-190 |  |
|  |  | D5 | 109-153 |  |
|  |  | R1 | 132-176 |  |
|  |  | R3 | 102-146 |  |
|  |  | R4 | 109-153 |  |
|  | 1 app. BBCH69 at last app. | D3 | 182-212 | / |
|  |  | D4 | 160-190 |  |
|  |  | D5 | 123-153 |  |
|  |  | R1 | 146-176 |  |
|  |  | R3 | 116-146 |  |
|  |  | R4 | 123-153 |  |
| Spring cereals | 2 app. BBCH25 at 1st app. | D3 | 111-155 | 14 |
|  |  | D4 | 133-177 |  |
|  |  | D5 | 93-137 |  |
|  |  | R1 | SC: Winter cereals |  |
|  |  | R3 | SC: Winter cereals |  |
|  |  | R4 | 93-137 |  |
|  | 1 app. BBCH25 at 1st app. | D3 | 111-141 | / |
|  |  | D4 | 133-163 |  |
|  |  | D5 | 93-123 |  |
|  |  | R1 | SC: Winter cereals |  |
|  |  | R3 | SC: Winter cereals |  |
|  |  | R4 | 93-123 |  |
| Winter OSR | 2 app. BBCH14 at 1st app. 30 March at 2nd app. | D3 | 252-90 | 173 |
|  |  | D4 | 253-90 | 172 |
|  |  | D5 | 270-90 | 155 |
|  |  | R1 | 254-90 | 171 |
|  |  | R3 | 285-90 | 140 |
|  |  | R4 | SC: Winter cereals (304-90) | 121 SC: Winter cereals |
|  | 1 app. BBCH14 at 1st app. | D3 | 252-282 | / |
|  |  | D4 | 253-283 |  |
|  |  | D5 | 270-300 |  |
|  |  | R1 | 254-284 |  |
|  |  | R3 | 285-315 |  |
|  |  | R4 | SC: Winter cereals (304-334) |  |
| Spring OSR | 1 app. BBCH14 at 1st app. | D3 | 108-138 | / |
|  |  | D4 | 127-157 |  |
|  |  | D5 | 82-112 |  |
|  |  | R1 | 107-137 |  |
|  |  | R3 | SC: Leafy vegetable (85-115) |  |
|  |  | R4 | SC: Leafy vegetable (85-115) |  |

#### Prothioconazole and its metabolites

Table 8.9-20:	Input parameters related to active substance Prothioconazole and metabolite(s) for PECsw/sed calculations STEP 1/2 and 3(/4) (if necessary)

| Compound | Prothioconazole JAU 6476 | JAU 6476-desthio (M04) | 1,2,4-triazole (M13) | Prothioconazole-S-methyl (M01) | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 344.26 | 312.2 | 69.065 | 358.3 | EFSA Conclusion (2007) |
| Saturated vapour pressure (Pa) | 0 (20°C) | 0 (20°C) |  | 0 (20°C) |  |
| Water solubility (mg/L) | 2000 (pH 9 and 20°C) | 2000 | 730000 | 2000 |  |
| Diffusion coefficient in water (m²/d) (STEP 3 only) | 4.3 x 10-5 |  |  |  | default |
| Diffusion coefficient in air (m²/d) (STEP 3 only) | 0.43 |  |  |  |  |
| Kfoc (mL/g) | 1765 | Mean = 575.4 (n = 4) | Mean = 89 (n = 4) | Mean = 2556.3 (n = 4 ) | EFSA Conclusion (2007) |
| Freundlich Exponent 1/n | 1 (default) | 0.81 | 0.9155 | 0.88 (arithmetic mean, n = 4) | EFSA Conclusion (2007) |
| Plant Uptake | 0 |  |  |  | default |
| Wash-Off factor from Crop (1/mm) (STEP 3 only) | 0.05 (MACRO) 0.50 (PRZM) |  |  |  |  |
| DT50,soil (d) | 1.2 geometric mean (EFSA2007) | median: 22.7 (EFSA2007) | Fast phase: 1.68 Slow phase: 60.5 g-value: 0.489 New endpoints (EFSA 2014) Modelled as a SFO with DT50 = 60.5 d | Mean = 15.7 (lab studies n=4) | EFSA Conclusion (2007) , Revised endpoint for triazole metabolite (2014). |
| DT50,water (d) | 1 | 1000 (default) | 1000 (default) | 1000 (default) | EFSA (2007) |
| DT50,sed (d) | 2.8 | 1000 (default) | 1000 (default) | 1000 (default) | EFSA (2007) RAR (2018) |
| DT50,whole system (d) (STEP 1-2 only) | 2.8 | 1000 (default) | 1000 (default) | 1000 (default) | EFSA (2007) |
| Maximum occurrence observed (% molar basis with respect to the parent) (STEP 1-2 only) | - | Water/Sediment: 32.3 Soil: 49.4 | Water/Sediment: 37.2 Soil: < 2 (modelled: 2) | Water/Sediment: 77 Soil: 14.6 | EFSA (2007) |
| Formation fraction in soil (STEP 3 only) | - | 0.57 from active substance | Not required | Not required | EFSA (2007) DAR (2005) |

#### Difenoconazole and its metabolites

Table 8.9-4:	Input parameters related to active substance Difenoconazole and metabolite(s) for PECsw/sed calculations STEP 1/2 and 3(/4) (if necessary)

| Compound | Difenoconazole CGA 71019 | CGA 205375 | 1,2,4-triazole (CGA 71019) | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 406 | 350 | 69.065 | EFSA Conclusion (2011) |
| Saturated vapour pressure (Pa) | 3.32 x 10-8 Pa at 25°C | 0 (Default) | 0 (default) |  |
| Water solubility (mg/L) | 15 | 12 | 730 000 |  |
| Diffusion coefficient in water (m2/d) | 4.3 x 10-5 | n.a. | n.a. | default |
| Diffusion coefficient in air (m2/d) | 0.43 | n.a. | n.a. | default |
| Kfoc (mL/g) | 2942.9 (geometric mean) | 2661.1 (geometric mean) | 83.132 (geometric mean) | EFSA Conclusion (2011) |
| Kfom (mL/g) | 1707.018 | 1543.561 | 48.220 |  |
| Freundlich Exponent 1/n | 0.85 | 0.8 | 0.9 |  |
| Plant Uptake | not required for Step 1+2 / 0 |  |  | default |
| Wash-Off factor from Crop (1/m) | not required for Step 1+2/ 0.05 (MACRO) 0.50 (PRZM) |  |  | default |
| DT50,soil (d) | geomean normalised DT50 soil 130 d | geomean normalised DT50 soil 94 d | Revised endpoint for triazole metabolite (DFOP) DT50 1 = 1,68 d DT50 2 = 60.5 d Modelled as a SFO with DT50 = 60.5 d | EFSA Conclusion (2011) and Revised endpoint for triazole metabolite (2014) |
| DT50,water (d) | 315.5 | 465.5 (mean value, degradation whole system) | 1000 (worst case) |  |
| DT50,sed (d) | 315.5 | 465.5 (mean value, degradation whole system) | 1000 (worst case) |  |
| DT50,whole system (d) | 315.5 | 465.5 (arithmetic mean) | 1000 (worst case) |  |
| Maximum occurrence observed (% molar basis with respect to the parent) | - | Soil: 9.7 Water/Sediment: 11.6 | Soil: 23.4 Water/Sediment: 9.6 |  |
| Formation fraction in soil (STEP 3): | - | n.a. | n.a. |  |

PECsw/sed for Prothioconazole

The RAC for the most sensitive aquatic organisms for the prothioconazole is 4.7 µg/L. The PECsw exceeding this value are highlighted in bold.

Table 8.9-5:	FOCUS Step 1 and 2 PECsw and PECsed for Prothioconazole following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 19.01 |  | 3.45 | 307.03 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 1.61 |  | 0.12 | 6.60 |
| Southern Europe | March-May | 1.61 |  | 0.16 | 11.47 |

*	twa-time as required by ecotox

Bold: exceeding 4.7 µg/L

Table 8.9-6:	FOCUS Step 1,2 PECsw and PECsed for Prothioconazole following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 16.29 |  | 2.96 | 263.17 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 1.38 |  | 0.13 | 9.31 |
| Southern Europe | Oct - Feb | 1.38 |  | 0.12 | 7.74 |

*	twa-time as required by ecotox

Bold: exceeding 4.7 µg/L

PECsw/sed for Prothioconazole-desthio (M04)

The RAC for the most sensitive aquatic organisms for the prothioconazole-desthio is 0.334 µg/L. The PECsw exceeding this value are highlighted in bold.

Table 8.9-7:	FOCUS Step 1,2 PECsw and PECsed for Prothioconazole-desthio following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 49.85 |  | 49.10 | 284.32 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 4.16 |  | 4.05 | 23.47 |
| Southern Europe | March-May | 7.77 |  | 7.64 | 44.24 |

*	twa-time as required by ecotox

Bold: exceeding 0.334 µg/L

Table 8.9-8:	FOCUS Step 1,2 PECsw and PECsed for Prothioconazole-desthio following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 42.73 |  | 42.08 | 243.68 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 6.28 |  | 6.16 | 35.69 |
| Southern Europe | Oct - Feb | 5.11 |  | 5.01 | 29.01 |

*	twa-time as required by ecotox

Bold: exceeding 0.334 µg/L

FOCUS Step 3

The prothioconazole-desthio (M04) required further modelling to reduce the PECsw below the RAC of 0.334 µg/L.

Table 8.9-9:	FOCUS Step 3 PECsw and PECsed for Prothioconazole-desthio following the application of the product AVTAR.

| Scenario FOCUS Waterbody |  | Max PECsw (μg/L) (bold: exceeding 0.334 µg/L) | Dominant entry route ( = causing highest PECSW) | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Winter cereals, 2 app., BBCH25 and onwards, 175 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.1929 |  | 0.01432 | 0.3700 |
| D4 | pond | 0.03511 |  | 0.03397 | 0.4326 |
| D4 | stream | 0.08751 |  | 0.000272 | 0.009598 |
| D5 | pond | 0.04178 |  | 0.04018 | 0.5106 |
| D5 | stream | 0.1259 |  | 0.000816 | 0.02159 |
| R1 | pond | 0.1228 |  | 0.1075 | 1.204 |
| R1 | stream | 0.9441 | runoff | 0.06499 | 1.036 |
| R3 | stream | 1.030 | runoff | 0.05139 | 1.229 |
| R4 | stream | 1.045 | runoff | 0.05090 | 0.6223 |
| Winter cereals, 1 app., BBCH25 and onwards, 175 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.1208 |  | 0.007731 | 0.1639 |
| D4 | pond | 0.02353 |  | 0.02248 | 0.2619 |
| D4 | stream | 0.09779 |  | 0.000182 | 0.005120 |
| D5 | pond | 0.02619 |  | 0.02507 | 0.3170 |
| D5 | stream | 0.1257 |  | 0.000233 | 0.006912 |
| R1 | pond | 0.04720 |  | 0.04137 | 0.5175 |
| R1 | stream | 0.3049 |  | 0.02105 | 0.3468 |
| R3 | stream | 0.3960 | runoff | 0.01852 | 0.5365 |
| R4 | stream | 0.4303 | runoff | 0.02111 | 0.2844 |
| Winter cereals, 2 app., Late application before BBCH69, 175 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.2800 |  | 0.04447 | 0.5955 |
| D4 | pond | 0.04068 |  | 0.03832 | 0.4753 |
| D4 | stream | 0.1454 |  | 0.002309 | 0.07665 |
| D5 | pond | 0.04303 |  | 0.04101 | 0.5204 |
| D5 | stream | 0.1760 |  | 0.004267 | 0.09715 |
| R1 | pond | 0.1393 |  | 0.1235 | 1.294 |
| R1 | stream | 0.6485 | runoff | 0.04228 | 1.547 |
| R3 | stream | 0.7071 | runoff | 0.04993 | 1.130 |
| R4 | stream | 0.6149 | runoff | 0.08495 | 1.077 |
| Winter cereals, 1 app., Late application before BBCH69, 175 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.3195 |  | 0.03047 | 0.4608 |
| D4 | pond | 0.02879 |  | 0.02668 | 0.01915 |
| D4 | stream | 0.1681 |  | 0.002667 | 0.07162 |
| D5 | pond | 0.02829 |  | 0.02671 | 0.3290 |
| D5 | stream | 0.2035 |  | 0.004338 | 0.09532 |
| R1 | pond | 0.06217 |  | 0.05747 | 0.6315 |
| R1 | stream | 0.3634 |  | 0.01567 | 1.042 |
| R3 | stream | 0.3817 | runoff | 0.02783 | 0.2959 |
| R4 | stream | 0.6149 | runoff | 0.08385 | 1.080 |
| Spring cereals, 2 app., BBCH25 and onwards, 175 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app., BBCH25 |  |  |  |  |  |
| D3 | ditch | 0.1956 |  | 0.02211 | 0.4043 |
| D4 | pond | 0.04260 |  | 0.04026 | 0.4544 |
| D4 | stream | 0.1136 |  | 0.001386 | 0.04222 |
| D5 | pond | 0.04151 |  | 0.03975 | 0.5002 |
| D5 | stream | 0.1189 |  | 0.000685 | 0.01848 |
| R1 | pond | 0.1228 |  | 0.1075 | 1.204 |
| R1 | stream | 0.9441 | runoff | 0.06499 | 1.036 |
| R3 | stream | 1.030 | runoff | 0.05139 | 1.229 |
| R4 | stream | 1.842 | runoff | 0.2460 | 2.707 |
| Spring cereals, 1 app., BBCH25 and onwards, 175 g a.s./ha In italics: Surrogate Crop: Winter cereals, 1 app., BBCH25 |  |  |  |  |  |
| D3 | ditch | 0.1300 |  | 0.009147 | 0.1827 |
| D4 | pond | 0.02780 |  | 0.02597 | 0.2873 |
| D4 | stream | 0.1160 |  | 0.000534 | 0.01857 |
| D5 | pond | 0.02615 |  | 0.02498 | 0.3107 |
| D5 | stream | 0.1251 |  | 0.000225 | 0.006702 |
| R1 | pond | 0.04720 |  | 0.04137 | 0.5175 |
| R1 | stream | 0.3049 |  | 0.02105 | 0.3468 |
| R3 | stream | 0.3960 | runoff | 0.01852 | 0.5365 |
| R4 | stream | 0.6535 | runoff | 0.08059 | 0.9610 |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app., BBCH00 |  |  |  |  |  |
| D3 | ditch | 0.2282 |  | 0.02745 | 0.3901 |
| D4 | pond | 0.02301 |  | 0.02243 | 0.3721 |
| D4 | stream | 0.1123 |  | 0.001796 | 0.04580 |
| D5 | pond | 0.02280 |  | 0.02179 | 0.3300 |
| D5 | stream | 0.1469 |  | 0.003123 | 0.06926 |
| R1 | pond | 0.04750 |  | 0.04171 | 0.6681 |
| R1 | stream | 0.2904 |  | 0.008619 | 0.2381 |
| R3 | stream | 0.6423 | runoff | 0.03912 | 0.8776 |
| R4 | stream | 0.8144 | runoff | 0.04138 | 0.6924 |
| Winter OSR, 1 app., one app. BBCH14 and onwards, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 1 app., BBCH00 |  |  |  |  |  |
| D3 | ditch | 0.2612 |  | 0.03144 | 0.4430 |
| D4 | pond | 0.02375 |  | 0.02238 | 0.2652 |
| D4 | stream | 0.1298 |  | 0.002043 | 0.05251 |
| D5 | pond | 0.02457 |  | 0.02323 | 0.2724 |
| D5 | stream | 0.1861 |  | 0.003968 | 0.09061 |
| R1 | pond | 0.02830 |  | 0.02432 | 0.3882 |
| R1 | stream | 0.2257 |  | 0.006895 | 0.1430 |
| R3 | stream | 0.6423 | runoff | 0.03945 | 0.8814 |
| R4 | stream | 0.5639 | runoff | 0.02388 | 0.3623 |
| Spring OSR, 1 app., BBCH14 and onwards, 150 g a.s./ha In italics: Surrogate Crop: Leafy vegetables, 1 app., BBCH14 |  |  |  |  |  |
| D3 | ditch | 0.1174 |  | 0.008719 | 0.1699 |
| D4 | pond | 0.02380 |  | 0.02222 | 0.2546 |
| D4 | stream | 0.09978 |  | 0.000467 | 0.01633 |
| D5 | pond | 0.02237 |  | 0.02136 | 0.2688 |
| D5 | stream | 0.1072 |  | 0.000192 | 0.005763 |
| R1 | pond | 0.04416 |  | 0.03847 | 0.4833 |
| R1 | stream | 0.3418 | runoff | 0.02103 | 0.3128 |
| R3 | stream | 0.3806 | runoff | 0.01838 | 0.4445 |
| R4 | stream | 0.7735 | runoff | 0.06673 | 0.7201 |
| Bold: exceeding 0.334 µg/L |  |  |  |  |  |

FOCUS Step 4

Table 8.9-10:	Global maximum PECsw values for Prothioconazole-desthio, following single/multiple application(s) of the product AVTAR according to the central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | STEP 4 |  |  |
| --- | --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | None | 10 | 20 |
|  | No spray buffer (m) | 5 | 10 | 20 |
| Winter cereals, 2 app., BBCH25 and onwards, 175 g a.s./ha |  |  |  |  |
| None | R1 stream | 0.9441 | 0.4288 | 0.2245 |
| None | R3 stream | 1.030 | 0.4698 | 0.2465 |
| None | R4 stream | 1.045 | 0.4751 | 0.2489 |
| Winter cereals, 1 app., BBCH25 and onwards, 175 g a.s./ha |  |  |  |  |
| None | R3 stream | 0.3960 | 0.1807 | - |
| None | R4 stream | 0.4303 | 0.1957 | - |
| Winter cereals, 2 app., Late application before BBCH69, 175 g a.s./ha |  |  |  |  |
| None | R1 stream | 0.6485 | 0.1653 | - |
| None | R3 stream | 0.7071 | 0.1906 | - |
| None | R4 stream | 0.6149 | 0.2798 | - |
| Winter cereals, 1 app., Late application before BBCH69, 175 g a.s./ha |  |  |  |  |
| None | R1 stream | 0.3634 | 0.1653 | - |
| None | R3 stream | 0.3817 | 0.1906 | - |
| None | R4 stream | 0.6149 | 0.2798 | - |
| Spring cereals, 2 app., BBCH25 and onwards, 175 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app., BBCH25 |  |  |  |  |
| None | R1 stream | 0.9441 | 0.4288 | 0.2245 |
| None | R3 stream | 1.030 | 0.4698 | 0.2465 |
| None | R4 stream | 1.045 | 0.8380 | 0.2489 |
| Spring cereals, 1 app., BBCH25 and onwards, 175 g a.s./ha In italics: Surrogate Crop: Winter cereals, 1 app., BBCH25 |  |  |  |  |
| None | R3 stream | 0.3960 | 0.1807 | - |
| None | R4 stream | 0.6535 | 0.2950 | - |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app., BBCH00 |  |  |  |  |
| None | R3 stream | 0.6423 | 0.2923 | - |
| None | R4 stream | 0.8144 | 0.3704 | 0.1940 |
| Winter OSR, 1 app., one app. BBCH14 and onwards, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 1 app., BBCH00 |  |  |  |  |
| None | R3 stream | 0.6423 | 0.2923 | - |
| None | R4 stream | 0.5639 | 0.2544 | - |
| Spring OSR, 1 app., BBCH14 and onwards, 150 g a.s./ha In italics: Surrogate Crop: Leafy vegetables, 1 app., BBCH14 |  |  |  |  |
| None | R1 stream | 0.3418 | 0.1552 | - |
| None | R3 stream | 0.3806 | 0.1736 | - |
| None | R4 stream | 0.7735 | 0.3518 | 0.1843 |
| Bold: exceeding 0.334 µg/L |  |  |  |  |

PECsw/sed for Prothioconazole-S-methyl (M01)

The RAC for the most sensitive aquatic organisms for the prothioconazole-S-methyl (M01) is 18 µg/L. The PECsw exceeding this value are highlighted in bold. In addition, output modelling files are included in the doc KCP 9.2.5/03.

Table 8.9-11:	FOCUS Step 1 and 2 PECsw and PECsed for M01 following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 27.81 |  | 25.68 | 659.46 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 1.48 |  | 1.06 | 28.01 |
| Southern Europe | March-May | 1.86 |  | 1.67 | 42.91 |

*	twa-time as required by ecotox

Bold: exceeding 18 µg/L

Table 8.9-12:	FOCUS Step 1 and 2 PECsw and PECsed for Prothioconazole-S-methyl (M01) following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 23.84 |  | 22.01 | 565.25 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 1.53 |  | 1.37 | 35.19 |
| Southern Europe | Oct - Feb | 1.35 |  | 1.18 | 30.40 |

*	twa-time as required by ecotox

Bold: exceeding 18 µg/L

PECsw/sed for 1.2.4-Triazole (M13)

The RAC for the most sensitive aquatic organisms for the 1,2,4-Triazole (M13) is 320 µg/L.

Table 8.9-13:	FOCUS Step 1,2 PECsw and PECsed for 1,2,4-triazole (M13) following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 8.44 |  | 8.36 | 7.49 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 0.32 |  | 0.31 | 0.28 |
| Southern Europe | March-May | 0.44 |  | 0.43 | 0.38 |

*	twa-time as required by ecotox

Table 8.9-14:	FOCUS Step 1,2 PECsw and PECsed for 1.2.4-Triazole (M13) following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 7.24 |  | 7.16 | 6.42 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 0.36 |  | 0.35 | 0.32 |
| Southern Europe | Oct - Feb | 0.32 |  | 0.31 | 0.28 |

*	twa-time as required by ecotox

PECsw/sed for Difenoconazole

The RAC for the most sensitive aquatic organisms for the Difenoconazole is 0.56 µg/L. The PECsw exceeding this value are highlighted in bold. For the sediment organisms, the RAC is 5.25 µg/kg. The PECsed exceeding this value are also highlighted in bold.

Table 8.9-15:	FOCUS Step 1,2 PECsw and PECsed for difenoconazole following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 13.99 |  | 12.41 | 371.78 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 2.26 |  | 2.11 | 63.24 |
| Southern Europe | March-May | 4.12 |  | 3.93 | 117.87 |

*	twa-time as required by ecotox

Bold: exceeding 0.56 µg/ L for PECsw and 5.25µg/kg for PECsed

Table 8.9-16	FOCUS Step 1,2 PECsw and PECsed for difenoconazole following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 12.00 |  | 10.63 | 318.67 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 3.33 |  | 3.17 | 95.18 |
| Southern Europe | Oct - Feb | 2.74 |  | 2.59 | 77.62 |

*	twa-time as required by ecotox

Bold: exceeding 0.56 µg/ L for PECsw and 5.25µg/kg for PECsed

FOCUS Step 3

The Difenoconazole require further modelling to reduce the PECsw below the RAC of 0.56 µg/L for the surface water and 5.25 µg/kg for the sediment.  The PEC exceeding these values are highlighted in bold.

Table 8.9-17:	FOCUS Step 3 PECsw and PECsed for difenoconazole following the application of the product AVTAR.

| Scenario FOCUS Waterbody |  | Max PECsw (μg/L) (bold: exceeding 0.56 µg/L) | Dominant entry route ( = causing highest PECSW) | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) (bold: exceeding 5.25 µg/kg) |
| --- | --- | --- | --- | --- | --- |
| Winter cereals, 2 app., BBCH25 and onwards, 91 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.5011 | Drift | 0.02677 | 0.5365 |
| D4 | pond | 0.02355 | Drift | 0.01856 | 0.3341 |
| D4 | stream | 0.3780 | Drift | 0.001889 | 0.03138 |
| D5 | pond | 0.02698 | Drift | 0.02148 | 0.3672 |
| D5 | stream | 0.4365 | Drift | 0.002715 | 0.04128 |
| R1 | pond | 0.05721 | Runoff | 0.04726 | 1.361 |
| R1 | stream | 0.3260 | Runoff | 0.02662 | 3.492 |
| R3 | stream | 0.4609 | Runoff | 0.02097 | 2.470 |
| R4 | stream | 0.4839 | Runoff | 0.02755 | 3.224 |
| Winter cereals, 1 app., BBCH25 and onwards, 91 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.5722 | Drift | 0.02559 | 0.3743 |
| D4 | pond | 0.01967 |  | 0.01427 | 0.2049 |
| D4 | stream | 0.4231 |  | 0.000765 | 0.01255 |
| D5 | pond | 0.01972 |  | 0.01159 | 0.2265 |
| D5 | stream | 0.4570 |  | 0.000812 | 0.01352 |
| R1 | pond | 0.02305 |  | 0.01916 | 0.5883 |
| R1 | stream | 0.3771 |  | 0.009447 | 1.468 |
| R3 | stream | 0.5299 |  | 0.009167 | 1.399 |
| R4 | stream | 0.3735 |  | 0.01292 | 1.709 |
| Winter cereals, 2 app., Late application before BBCH69, 91 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.5027 |  | 0.06407 | 0.7169 |
| D4 | pond | 0.02378 |  | 0.01903 | 0.3573 |
| D4 | stream | 0.4283 |  | 0.005876 | 0.1104 |
| D5 | pond | 0.02615 |  | 0.02106 | 0.3671 |
| D5 | stream | 0.4621 |  | 0.01034 | 0.1514 |
| R1 | pond | 0.05902 |  | 0.05084 | 1.456 |
| R1 | stream | 0.3250 |  | 0.02278 | 4.711 |
| R3 | stream | 0.4612 |  | 0.02340 | 2.495 |
| R4 | stream | 0.3279 | Run-off for PECSED | 0.03471 | 7.153 |
| Winter cereals, 1 app., Late application before BBCH69, 91 g a.s./ha |  |  |  |  |  |
| D3 | ditch | 0.5743 | drift | 0.03969 | 0.5470 |
| D4 | pond | 0.01968 |  | 0.01474 | 0.2209 |
| D4 | stream | 0.4953 |  | 0.006791 | 0.1081 |
| D5 | pond | 0.01971 |  | 0.01498 | 0.2280 |
| D5 | stream | 0.5344 |  | 0.009719 | 0.1526 |
| R1 | pond | 0.02531 |  | 0.02100 | 0.6329 |
| R1 | stream | 0.3782 |  | 0.009970 | 2.627 |
| R3 | stream | 0.5330 |  | 0.01442 | 1.396 |
| R4 | stream | 0.3787 |  | 0.03108 | 2.777 |
| Spring cereals, 2 app., BBCH25 and onwards, 91 g a.s./ha In italics = Surrogate crop: Winter cereals, 2 app., BBCH25 |  |  |  |  |  |
| D3 | ditch | 0.5015 |  | 0.05089 | 0.5995 |
| D4 | pond | 0.02616 |  | 0.02071 | 0.3559 |
| D4 | stream | 0.4181 |  | 0.004785 | 0.06692 |
| D5 | pond | 0.02694 |  | 0.02131 | 0.3606 |
| D5 | stream | 0.4306 |  | 0.002316 | 0.03494 |
| R1 | pond | 0.05721 |  | 0.04726 | 1.361 |
| R1 | stream | 0.3260 |  | 0.02662 | 3.492 |
| R3 | stream | 0.4609 |  | 0.02097 | 2.470 |
| R4 | stream | 0.5823 | Run-off for PECSED | 0.09171 | 6.503 |
| Spring cereals, 1 app., BBCH25 and onwards, 91 g a.s./ha In italics = Surrogate crop: Winter cereals, 1 app., BBCH25 |  |  |  |  |  |
| D3 | ditch | 0.5726 | drift | 0.02796 | 0.4049 |
| D4 | pond | 0.01968 |  | 0.01467 | 0.2154 |
| D4 | stream | 0.4684 |  | 0.002016 | 0.03280 |
| D5 | pond | 0.01972 |  | 0.01469 | 0.2232 |
| D5 | stream | 0.4549 |  | 0.000786 | 0.01310 |
| R1 | pond | 0.02305 |  | 0.01916 | 0.5883 |
| R1 | stream | 0.3771 |  | 0.009447 | 1.468 |
| R3 | stream | 0.5299 |  | 0.009167 | 1.399 |
| R4 | stream | 0.3781 |  | 0.03266 | 3.169 |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 78 g a.s./ha In italics = Surrogate crop: Winter cereals, 2 app., BBCH00 |  |  |  |  |  |
| D3 | ditch | 0.4307 |  | 0.03683 | 0.4951 |
| D4 | pond | 0.01624 |  | 0.01279 | 0.3092 |
| D4 | stream | 0.3670 |  | 0.005031 | 0.08022 |
| D5 | pond | 0.01577 |  | 0.01182 | 0.2636 |
| D5 | stream | 0.3960 |  | 0.007200 | 0.1133 |
| R1 | pond | 0.03517 |  | 0.02980 | 1.082 |
| R1 | stream | 0.2805 |  | 0.01001 | 1.235 |
| R3 | stream | 0.3943 | Run-off for PECSED | 0.01679 | 5.510 |
| R4 | stream | 0.4033 |  | 0.02379 | 3.313 |
| Winter OSR, 1 app., one app. BBCH14 and onwards, 78 g a.s./ha In italics = Surrogate crop: Winter cereals, 2 app., BBCH00 |  |  |  |  |  |
| D3 | ditch | 0.4929 |  | 0.04219 | 0.5653 |
| D4 | pond | 0.01687 |  | 0.01268 | 0.2172 |
| D4 | stream | 0.4245 |  | 0.005819 | 0.09274 |
| D5 | pond | 0.01690 |  | 0.01283 | 0.2094 |
| D5 | stream | 0.4580 |  | 0.008327 | 0.1309 |
| R1 | pond | 0.02723 |  | 0.02181 | 0.6509 |
| R1 | stream | 0.3245 |  | 0.007571 | 0.6300 |
| R3 | stream | 0.4538 |  | 0.01585 | 2.433 |
| R4 | stream | 0.3246 |  | 0.01052 | 0.7855 |
| Spring OSR, 1 app., BBCH14 and onwards, 78 g a.s./ha In italics = Surrogate crop: Leafy vegetables, 2 app., BBCH14 |  |  |  |  |  |
| D3 | ditch | 0.4910 |  | 0.02539 | 0.3661 |
| D4 | pond | 0.01686 |  | 0.01256 | 0.1862 |
| D4 | stream | 0.4021 |  | 0.001771 | 0.02881 |
| D5 | pond | 0.01690 |  | 0.01257 | 0.1925 |
| D5 | stream | 0.3896 |  | 0.000670 | 0.01117 |
| R1 | pond | 0.02502 |  | 0.02212 | 0.8815 |
| R1 | stream | 0.3232 |  | 0.01045 | 1.835 |
| R3 | stream | 0.4547 | Run-off for PECSED | 0.009859 | 9.002 |
| R4 | stream | 0.3240 |  | 0.02692 | 2.372 |

Bold: exceeding 0.56 µg/L for PECsw and 5.25µg/kg for PECsed

FOCUS Step 4

Table 8.9-18:	Global maximum PECsw values for Difenoconazole, following single/multiple application(s) of the product AVTAR according to the central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | STEP 4 |  |  |
| --- | --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | None | 10 | 20 |
|  | No spray buffer (m) | 5 | None | None |
| Winter cereals, 1 app., BBCH25 and onwards, 91 g a.s./ha |  |  |  |  |
| None | D3 ditch | 0.1549 | - | - |
| Winter cereals, 1 app., Late application before BBCH69, 91 g a.s./ha |  |  |  |  |
| None | D3 ditch | 0.1554 | - | - |
| Spring cereals, 2 app., BBCH25 and onwards, 91 g a.s./ha |  |  |  |  |
| None | R4 stream | 0.5823 | 0.3274 |  |
| Spring cereals, 1 app., BBCH25 and onwards, 91 g a.s./ha |  |  |  |  |
| None | D3 ditch | 0.1550 | - | - |

Table 8.9-19:	Global maximum PECsed values for Difenoconazole, following single/multiple application(s) of the product AVTAR according to the central EU zone GAP according to surface water Step 4

| PECsed (µg/kg) | Scenario | STEP 4 |  |  |
| --- | --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | None | 10 | 20 |
|  | No spray buffer (m) | 5 | None | None |
| Winter cereals, 2 app., Late application before BBCH69, 91 g a.s./ha |  |  |  |  |
| None | R4 stream | 7.147 | 1.192 | - |
| Spring cereals, 2 app., BBCH25 and onwards, 91 g a.s./ha |  |  |  |  |
| None | R4 stream | 6.498 | 1.183 | - |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 78 g a.s./ha |  |  |  |  |
| None | R3 stream | 5.495 | 0.9482 | - |
| Spring OSR, 1 app., BBCH14 and onwards, 78 g a.s./ha In italics = SC: Leafy vegetables, 2 app., BBCH14 |  |  |  |  |
| None | R3 stream | 8.994 | 1.443 | - |
| Bold: exceeding 5.25 µg/L |  |  |  |  |

PECsw/sed for CGA 205375

The RAC for the most sensitive aquatic organisms for the CGA 205375 is 7.4 µg/L. The PECsw exceeding this value are highlighted in bold.

Table 8.9-20:	FOCUS Step 1,2 PECsw and PECsed for CGA 205375 following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 2.62 |  | 2.45 | 66.06 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 0.41 |  | 0.39 | 10.58 |
| Southern Europe | March-May | 0.78 |  | 0.75 | 20.31 |

*	twa-time as required by ecotox

Table 8.9-21:	FOCUS Step 1,2 PECsw and PECsed for CGA 205375 following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 2.24 |  | 2.10 | 56.62 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 0.63 |  | 0.61 | 16.37 |
| Southern Europe | Oct - Feb | 0.51 |  | 0.49 | 13.24 |

*	twa-time as required by ecotox

PECsw/sed for 1.2.4-Triazole (CGA 71019)

The RAC for the most sensitive aquatic organisms for the 1,2,4-Triazole (CGA 71019) is 320 µg/L. The PECsw exceeding this value are highlighted in bold.

Table 8.9-22:	FOCUS Step 1,2 PECsw and PECsed for 1,2,4-Triazole (CGA 71019) following multiple application(s) of the product AVTAR to Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 3.09 |  | 3.07 | 2.57 |
| Step 2 |  |  |  |  |  |
| Northern Europe | March-May | 0.46 |  | 0.46 | 0.39 |
| Southern Europe | March-May | 0.91 |  | 0.90 | 0.75 |

*	twa-time as required by ecotox

Table 8.9-23:	FOCUS Step 1,2 PECsw and PECsed for 1,2,4-Triazole (CGA 71019) following multiple application(s) of the product AVTAR to Oilseed rape

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L)* | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 2.65 |  | 2.63 | 2.20 |
| Step 2 |  |  |  |  |  |
| Northern Europe | Oct - Feb | 0.73 |  | 0.72 | 0.61 |
| Southern Europe | Oct - Feb | 0.59 |  | 0.58 | 0.49 |

*	twa-time as required by ecotox

#### Additionnal STEP 4 calculation with VFSmod

In some member states, VFSmod is accepted as a mitigation for the PECsw. Here is an overview of the impact of mitigations with VFSmod on PECSW.

Table 8.9-24:	Global maximum PECsw values for Prothioconazole-desthio, following single/multiple application(s) of the product AVTAR according to the central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | STEP 4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | 1 | 2 | 5 | 10 |
| Winter cereals, 2 app., BBCH25 and onwards, 175 g a.s./ha |  |  |  |  |  |
| None | R1 stream | 0.1801 | - | - | - |
| None | R3 stream | 0.6216 | 0.5640 | 0.4216 | 0.2612 |
| None | R4 stream | 0.4592 | 0.3060 | - | - |
| Winter cereals, 1 app., BBCH25 and onwards, 175 g a.s./ha |  |  |  |  |  |
| None | R3 stream | 0.2417 | - | - | - |
| None | R4 stream | 0.1893 | - | - | - |
| Winter cereals, 2 app., Late application before BBCH69, 175 g a.s./ha |  |  |  |  |  |
| None | R1 stream | 0.2436 | - | - | - |
| None | R3 stream | 0.3447 | 0.3123 | - | - |
| None | R4 stream | 0.2712 | - | - | - |
| Winter cereals, 1 app., Late application before BBCH69, 175 g a.s./ha |  |  |  |  |  |
| None | R1 stream | 0.1313 | - | - | - |
| None | R3 stream | 0.1906 | - | - | - |
| None | R4 stream | 0.2712 | - | - | - |
| Spring cereals, 2 app., BBCH25 and onwards, 175 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app., BBCH25 |  |  |  |  |  |
| None | R1 stream | 0.1801 | - | - | - |
| None | R3 stream | 0.6216 | 0.5640 | 0.4216 | 0.2612 |
| None | R4 stream | 0.8073 | 0.6927 | 0.4508 | 0.1540 |
| Spring cereals, 1 app., BBCH25 and onwards, 175 g a.s./ha In italics: Surrogate Crop: Winter cereals, 1 app., BBCH25 |  |  |  |  |  |
| None | R3 stream | 0.2417 | - | - | - |
| None | R4 stream | 0.2631 | - | - | - |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app. on bare soil |  |  |  |  |  |
| None | R3 stream | 0.3810 | 0.3359 | 0.2298 | - |
| None | R4 stream | 0.3533 | 0.2336 | - | - |
| Winter OSR, 1 app., one app. after BBCH14, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 1 app. on bare soil |  |  |  |  |  |
| None | R3 stream | 0.3810 | 0.3359 | 0.2298 | - |
| None | R4 stream | 0.1461 | - | - | - |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 150 g a.s./ha In italics: Surrogate Crop: Winter cereals, 2 app. on bare soil |  |  |  |  |  |
| None | R1 stream | 0.1126 | - | - | - |
| None | R3 stream | 0.2268 | - | - | - |
| None | R4 stream | 0.2955 | - | - | - |
| Bold: exceeding 0.334 µg/L |  |  |  |  |  |

Table 8.9-25:	Global maximum PECsed values for Difenoconazole, following single/multiple application(s) of the product AVTAR according to the central EU zone GAP according to surface water Step 4

| PECsed (µg/kg) | Scenario | STEP 4 |  |
| --- | --- | --- | --- |
| Nozzle reduction | Vegetative filter strip (m) | 1 | 2 |
| Winter cereals, 2 app., Late application before BBCH69, 91 g a.s./ha |  |  |  |
| None | R4 stream | 4.138 | - |
| Spring cereals, 2 app., at BBCH25, 91 g a.s./ha |  |  |  |
| None | R4 stream | 3.611 | - |
| Winter OSR, 2 app., one app. after BBCH14 and one in March, 78 g a.s./ha |  |  |  |
| None | R3 stream | 3.132 | - |
| Spring OSR, 2 app., one app. after BBCH14, 78 g a.s./ha |  |  |  |
| None | R3 stream | 5.396 | 4.338 |
| Bold: exceeding 5.25 µg/kg |  |  |  |

#### PECsw of formulation

The PECsw are based on the Ganzelmeier/Rautmann drift values, no degradation and the cumulated application rate of 2 applications.

Table 8.9-26:	PECsw for formulation on cereals and oilseed rape

| Crop | # App. | App. rate (L/ha) | Density (g/L) | App. rate (g/ha) | App. rate cumulated (g/ha) | Portion reaching the target (%) | Application rate reaching water surface (µg/m2) | Depth (cm) | PECsw (µg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cereals | 2 | 0.7 | 1155 | 808.5 | 1617 | 2.38 | 3848.46 | 30 | 12.83 |
| Oilseed rape | 2 | 0.6 |  | 693 | 1386 |  | 3298.68 |  | 11.00 |

The calculation with a 5m buffer zone is realised according to the formula Rautmann, 1999  on field crops with 2 applications. This formula is giving the % of the application rate that is reaching the soil or the water expressed in % of a distance x in m. Therefore, by applying a 5 m buffer zone, the new drift value is 0.47 % at the place of 2.38% without buffer zone.

Table 8.9-27:	PECsw for formulation on cereals and oilseed rape with a drift mitigation of 5m buffer zone

| Crop | # App. | App. rate (L/ha) | Density (g/L) | App. rate (g/ha) | App. rate cumulated (g/ha) | Portion reaching the target (%) | Application rate reaching water surface (µg/m2) | Depth (cm) | PECsw (µg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cereals | 2 | 0.7 | 1155 | 808.5 | 1617 | 0.47 | 759.99 | 30 | 2.53 |
| Oilseed rape | 2 | 0.6 |  | 693 | 1386 |  | 651.42 |  | 2.17 |

The long-term PECSW of the formulation are not relevant, mainly because prothioconazole degrades quickly in water (DT50, WATER < 1 day).

## Fate and behaviour in air (KCP 9.3, KCP 9.3.1)

Table 8.10-21 Summary of atmospheric degradation and behaviour: prothioconazole

| Compound | Prothioconazole, M04 |
| --- | --- |
| Direct photolysis in air | Not studied – no data requested |
| Quantum yield of direct phototransformation | Not measured – no data requested |
| Photochemical oxidative degradation in air | Prothioconazole: DT50: 1.1 hours Chemical lifetime: 1.6 hours 12 hour day, 1.5x106 OH radicals/cm3 Prothioconazole-desthio (M04): DT50: 14.2 hours Chemical lifetime: 20.5 hours 12 hour day, 1.5x106 OH radicals/cm3 |
| Volatilisation | Prothioconazole: Vapour pressure (Pa): << 4 x 10-7 Henry's Law Constant (Pa.m3/mol): << 3 x 10-5 |
| Metabolites | see above |

The vapour pressure at 20 °C of the active substance Prothioconazole is < 10-5 Pa. Hence the active substance Prothioconazole is regarded as non-volatile.

Table 8.10-2 Summary of atmospheric degradation and behaviour: difenoconazole

| Compound | Difenoconazole |
| --- | --- |
| Direct photolysis in air | Not submitted, not required |
| Quantum yield of direct phototransformation | Difenoconazole: 0.0155 (in water) CGA 205375: 0.0266 (in water) |
| Photochemical oxidative degradation in air | Difenoconazole: DT50 = 5 hours derived by the Atkinson method (AOP 1.85). OH (12 h) concentration assumed: 1.5 x 106 radicals/cm3 . |
| Volatilisation | Volatilisation from soil: <0.05% after 24 h (measured as % 14C in absorption trap) Volatilization from plants and soil: <9% after 24 h (measured as %loss) Vapour pressure (Pa): 3.32 x 10-8 (25 °C) Henry's Law Constant (Pa.m3/mol): 9.07 x 10-7 (25 °C) |
| Metabolites | None identified |

The vapour pressure at 20 °C of the active substance Difenoconazole is < 10-5 Pa. Hence the active substance Difenoconazole is regarded as non-volatile

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 9.2.4.1/01 | Ooms D. | 2021 | Predicted environmental concentrations in groundwater of prothioconazole, difenoconazole and metabolites, following application of product IN233C1560 on cereals and oilseed rape in Central EU using FOCUS PEARL 4.4.4, FOCUS PELMO 5.5.3 and FOCUS MACRO 5.5.4. Company Report No 2021-IND-PECGW01 Eric Neumans Consulting s.r.l. non GLP Unpublished | N | Indofil Industries (Netherlands) B.V. |
| KCP 9.2.5/01 | Rifflart M. and Ooms D. | 2021 | Predicted environmental concentrations in Surface water of prothioconazole, difenoconazole and metabolites, following application of product IN233C1560 on cereals and oilseed rape in Central EU using STEPs 1-2 in FOCUS, SPIN 2.2, SWASH v5.3, MACRO v5.5.4, PRZM v4.3.1, TOXSWA v5.5.3 and SWAN v5.0.0. Company Report No R2021-IND-PECSW01 Eric Neumans Consulting s.r.l. non GLP Unpublished | N | Indofil Industries (Netherlands) B.V. |
| KCP 9.2.4.1/02 | Rifflart M. | 2023 | Predicted environmental concentrations in groundwater of prothioconazole, difenoconazole and metabolites, following application of product AVTAR on cereals and oilseed rape in Central EU using FOCUS PEARL 4.4.4, FOCUS PELMO 5.5.3 and FOCUS MACRO 5.5.4. Company Report No 2023-IND-PECGW01 Eric Neumans Consulting s.r.l. non GLP Unpublished | N | Indofil Industries (Netherlands) B.V. |
| KCP 9.2.5/02 | Rifflart M. | 2023 | Predicted environmental concentrations in Surface water of prothioconazole, difenoconazole and metabolites, following application of product AVTAR on cereals and oilseed rape in Central EU using STEPs 1-2 in FOCUS, SPIN 2.2, SWASH v5.3, MACRO v5.5.4, PRZM v4.3.1, TOXSWA v5.5.3 and SWAN v5.0.0. Company Report No R2023-IND-PECSW01 Eric Neumans Consulting s.r.l. non GLP Unpublished | N | Indofil Industries (Netherlands) B.V. |
| KCP 9.2.5/03 | Anonymous | 2023 | Predicted environmental concentrations in Surface water of metabolite prothioconazole-S-methyl (M01), following application of product AVTAR on cereals and oilseed rape in using STEPs 1-2 in FOCUS. Modelling output files Eric Neumans Consulting s.r.l. non GLP Unpublished | N | Indofil Industries (Netherlands) B.V. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Detailed evaluation of the new Annex II studies

No data submitted.

- Additional information provided by the applicant (e.g. detailed modelling data)

## List of electronic input files

PECgw for Prothioconazole have not been recalculated. See report 2023-IND-PECgw for files numbers.

### Winter cereals, FOCUS-PEARL 4.4.4

Table AII-1:	File identification for Prothioconazole, difenoconazole and metabolites on Winter Cereals (with FOCUS PEARL 4.4.4):

| Winter cereals | Scenario | File numbering |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA 205375 | 1,2,4-triazole formed from CGA 205375* | 1,2,4-triazole applied as parent |
|  | 2 applications starting at BBCH 25, interval = 14 d |  |  |  |  |
|  | Châteaudun | 2 | 2 | 2 | 51+58 |
|  | Hamburg | 3 | 3 | 3 | 52+59 |
|  | Kremsmünster | 4 | 4 | 4 | 53+60 |
|  | Jokioinen | 149 | 149 | 149 | 163+164 |
|  | Okehampton | 5 | 5 | 5 | 54+61 |
|  | Piacenza | 6 | 6 | 6 | 55+62 |
|  | Porto – January | 7 | 7 | 7 | 56+63 |
|  | Porto – March | 8 | 8 | 8 | 57+64 |
|  | Sevilla – January | 150 | 150 | 150 | 165+166 |
|  | Sevilla – March | 151 | 151 | 151 | 167+168 |
|  | Thiva – January | 152 | 152 | 152 | 169+170 |
|  | Thiva – March | 153 | 153 | 153 | 171+172 |
|  | 2 applications ending at BBCH 69, interval = 14 d |  |  |  |  |
|  | Châteaudun | 9 | 9 | 9 | 65+71 |
|  | Hamburg | 10 | 10 | 10 | 66+72 |
|  | Kremsmünster | 11 | 11 | 11 | 67+73 |
|  | Jokioinen | 154 | 154 | 154 | 173+174 |
|  | Okehampton | 12 | 12 | 12 | 68+74 |
|  | Piacenza | 13 | 13 | 13 | 69+75 |
|  | Porto | 14 | 14 | 14 | 70+76 |
|  | Sevilla | 155 | 155 | 155 | 175+176 |
|  | Thiva | 156 | 156 | 156 | 177+178 |

### Spring cereals, FOCUS-PEARL 4.4.4

Table AII-2:	File identification for Prothioconazole, difenoconazole and metabolites on Spring Cereals (with FOCUS PEARL 4.4.4):

| Spring cereals | Scenario | File numbering |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA 250375 | 1,2,4-triazole formed from CGA205375 | 1,2,4-triazole applied as parent |
|  | 2 applications starting at BBCH 25, interval = 14 d |  |  |  |  |
|  | Châteaudun | 15 | 15 | 15 | 77+82 |
|  | Hamburg | 16 | 16 | 16 | 78+83 |
|  | Kremsmünster | 17 | 17 | 17 | 79+84 |
|  | Jokioinen | 157 | 157 | 157 | 179+180 |
|  | Okehampton | 18 | 18 | 18 | 80+85 |
|  | Piacenza | Crop not defined. Winter cereals is the surrogate crop, see Table AII-1 |  |  |  |
|  | Porto | 19 | 19 | 19 | 81+86 |
|  | Sevilla | n.d | n.d | n.d | n.d |
|  | Thiva | n.d | n.d | n.d | n.d |
|  | 2 applications ending at BBCH 69, interval = 14 d |  |  |  |  |
|  | Châteaudun | 20 | 20 | 20 | 87+92 |
|  | Hamburg | 21 | 21 | 21 | 88+93 |
|  | Kremsmünster | 22 | 22 | 22 | 89+94 |
|  | Jokioinen | 158 | 158 | 158 | 181+182 |
|  | Okehampton | 23 | 23 | 23 | 90+95 |
|  | Piacenza | Crop not defined. Winter cereals is the surrogate crop, see Table AII-1 |  |  |  |
|  | Porto | 24 | 24 | 24 | 91+96 |
|  | Sevilla | n.d | n.d | n.d | n.d |
|  | Thiva | n.d | n.d | n.d | n.d |

### Winter oilseed rape, FOCUS-PEARL 4.4.4

Table AII-3:	File identification for Prothioconazole, difenoconazole and metabolites on Winter Oilseed Rape (with FOCUS PEARL 4.4.4):

| PEARL - Winter oilseed rape | Scenario | 80th Percentile PECGW at 1 m Soil Depth (g/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA 205375 | 1,2,4-triazole formed from CGA 205375 | 1,2,4-triazole applied as parent |
|  | 2 applications: 1st on 15th February and 2nd on 1st March |  |  |  |  |
|  | Châteaudun | 25 | 25 | 25 | 97+103 |
|  | Hamburg | 26 | 26 | 26 | 98+104 |
|  | Kremsmünster | 27 | 27 | 27 | 99+105 |
|  | Jokioinen | n.d | n.d | n.d | n.d |
|  | Okehampton | 28 | 28 | 28 | 100+106 |
|  | Piacenza | 29 | 29 | 29 | 101+107 |
|  | Porto | 30 | 30 | 30 | 102+108 |
|  | Sevilla | n.d | n.d | n.d | n.d |
|  | Thiva | n.d | n.d | n.d | n.d |
|  | 2 applications: 1st at BBCH 14 in late summer/early autumn and 2nd at BBCH 69 in spring |  |  |  |  |
|  | Châteaudun | 31 | 31 | 31 | 109+115 |
|  | Hamburg | 32 | 32 | 32 | 110+116 |
|  | Kremsmünster | 33 | 33 | 33 | 111+117 |
|  | Jokioinen | n.d | n.d | n.d | n.d |
|  | Okehampton | 34 | 34 | 34 | 112+118 |
|  | Piacenza | 35 | 35 | 35 | 113+119 |
|  | Porto | 36 | 36 | 36 | 114+120 |
|  | Sevilla | n.d | n.d | n.d | n.d |
|  | Thiva | n.d | n.d | n.d | n.d |
|  | 2 applications: 1st at BBCH 14 in late summer/early autumn and 2nd on 1st March |  |  |  |  |
|  | Châteaudun | 37 | 37 | 37 | 121+127 |
|  | Hamburg | 38 | 38 | 38 | 122+128 |
|  | Kremsmünster | 39 | 39 | 39 | 123+129 |
|  | Jokioinen | n.d | n.d | n.d | n.d |
|  | Okehampton | 40 | 40 | 40 | 124+130 |
|  | Piacenza | 41 | 41 | 41 | 125+131 |
|  | Porto | 42 | 42 | 42 | 126+132 |
|  | Sevilla | n.d | n.d | n.d | n.d |
|  | Thiva | n.d | n.d | n.d | n.d |

### Spring oilseed rape, FOCUS-PEARL 4.4.4

Table AII-4:	File identification for Prothioconazole, difenoconazole and metabolites on spring oilseed rape (with FOCUS PEARL 4.4.4):

| PEARL - Spring oilseed rape (SB = surrog. crop sugar beet, CA = cabbage) | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Difenoconazole | CGA 205375 | 1,2,4-triazole formed from CGA 205375 | 1,2,4-triazole applied as parent |
|  | 1 application at BBCH 14 |  |  |  |  |
|  | Châteaudun - CA | 43 | 43 | 43 | 133+139 |
|  | Hamburg - CA | 44 | 44 | 44 | 134+140 |
|  | Kremsmün. - CA | 45 | 45 | 45 | 135+141 |
|  | Jokioinen | 159 | 159 | 159 | 183+186 |
|  | Okehampton | 46 | 46 | 46 | 136+142 |
|  | Piacenza - SB | 47 | 47 | 47 | 137+143 |
|  | Porto | 48 | 48 | 48 | 138+144 |
|  | Sevilla - CA | 160 | 160 | 160 | 187+188 |
|  | Thiva - SB | 161 | 161 | 161 | 189+190 |
|  | 1 application at BBCH 69 |  |  |  |  |
|  | Châteaudun | Crop not defined. Winter oilseed rape is the surrogate crop, see Table AII-3 |  |  |  |
|  | Hamburg | Crop not defined. Winter oilseed rape is the surrogate crop, see Table AII-3 |  |  |  |
|  | Kremsmünster | Crop not defined. Winter oilseed rape is the surrogate crop, see Table AII-3 |  |  |  |
|  | Jokioinen | 162 | 162 | 162 | 191+192 |
|  | Okehampton | 52 | 52 | 52 | 145+147 |
|  | Piacenza | Crop not defined. Winter oilseed rape is the surrogate crop, see Table AII-3 |  |  |  |
|  | Porto | 54 | 54 | 54 | 146+148 |
|  | Sevilla | n.d | n.d | n.d | n.d |
|  | Thiva | n.d | n.d | n.d | n.d |

CTGB work instruction: Implementation of bi-phasic kinetics for 1,2,4-triazole in PEARL/GeoPEARL: "The Triazole Derivative Metabolite Group (TDMG) has submitted additional field dissipation data for 1,2,4-triazole as part of confirmatory data requirements, which resulted in a revision of the DT50 values to be used in FOCUS modelling (proposed revision to DT50 for 1,2,4-triazole (July 2011, revised September 2011 and January 2013)" https://english.ctgb.nl/binaries/ctgb-en/documents/assessment-framework-ppp/2016/10/26/work-instruction-bi-phasic-kinetics-124-triazole-in-pearl/ctgb_work-instruction-implementation-of-bi-phasic-kinetics-for-1-2-4-triazole-in-pearl_geopearl.pdf

Table B.8.5-3, p. 22 in Volume 3 – B.8 (PPP) – Aviator EC 225

Chapple, A.,  Hoerold, C. 2015 Kinetic evaluation (trigger and modelling endpoints)  of the soil degradation of prothioconazole and its soil  metabolites desthio and S-methyl under laboratory conditions. In: RAR - Volume 3 – B.8 (AS)

In: CTGB work instruction: Implementation of bi-phasic kinetics for 1,2,4-triazole in PEARL/GeoPEARL

Table B.8.5-3, p. 22 in Volume 3 – B.8 (PPP) – Aviator EC 225

Chapple, A., Hoerold, C. 2015 Kinetic evaluation (trigger and modelling endpoints) of the soil degradation of prothioconazole and its soil metabolites desthio and S-methyl under laboratory  conditions and Chapple, A.C. and Hoerold, C., 2015g. Kinetic evaluation of the aerobic aquatic metabolism of prothioconazole in two laboratory water/sediment systems. In: RAR - Volume 3 – B.8 (AS)

https://www.ctgb.nl/binaries/ctgb/documenten/instructies-gewasbeschermingsmiddelen/2016/10/28/bi-phasic-kinetics-for-124-triazole-in-pearl_geopearl/ctgb_work-instruction-implementation-of-bi-phasic-kinetics-for-1-2-4-triazole-in-pearl_geopearl.pdf

Rautmann, D., Streloke, M., & Winkler, R. (1999, September). New basic drift values in the authorization procedure for plant protection products. In Workshop on risk management and risk mitigation measures in the context of authorization of plant protection products (pp. 133-141).