FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: NIC-HER 060 OD Product name(s): - Chemical active substance: nicosulfuron, 60 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Pestila Sp. z o.o. Submission date: 07/2022 MS Finalisation date: 09/2023; 12/2023

Version history

| When | What |
| --- | --- |
| 09/2023 | ZRMs evaluated dRR submitted by Applicant |
| 12/2023 | The final Registration Report |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	6

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	6

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	6

5.2.2	Methods for the determination of residues (KCP 5.1.2)	6

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	7

5.3.1	Analysis of the plant protection product (KCP 5.2)	7

5.3.2	Description of analytical methods for the determination of residues nicosulfuron (KCP 5.2)	7

5.3.2.1	Overview of residue definitions and levels for which compliance is required	7

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	9

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	9

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	9

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	10

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	10

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	10

5.3.2.8	Other studies/ information	10

Appendix 1	Lists of data considered in support of the evaluation	12

Appendix 2	Detailed evaluation of submitted analytical methods	18

A 2.1	Analytical methods for nicosulfuron	18

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	18

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	18

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for the active substance and relevant impurities in the plant protection product.

Noticed data gaps:

- none

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Data gaps:

- none

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Maize | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of nicosulfuron in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical method is suitable for the determination of the active substance nicosulfuron in the plant protection product Nicorn Grande 060 OD. The proposed analytical method has been fully validated in terms of specificity, linearity, precision, and recovery. The proposed methods fulfil the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical methods has been accepted. |
| --- | --- |

Materials and methods

Determination of nicosulfuron was performed with LC Liquid chromatograph equipped with DAD detector wavelength 245 nm and external standard.

Equipment and chromatographic conditions for nicosulfuron analysis

- Agilent Technologies 1220 Infinity II LC Liquid chromatograph equipped with DAD detector, AKP/15; Analytical balance, Sartorius, AKP/05; Ultrasonic bath, Bandelin Sonorex Digiplus, WP/78; Basic laboratory glassware and equipment.

- Test system: Agilent Technologies 1220 Infinity II LC (AKP/15)

- Detector wavelength: 245 nm (bandwidth 4 nm)

- Reference wavelength: 360 nm (bandwidth 100 nm)

- Column: Zorbax Eclipse Plus C18, 4.6 mm x 100 mm x 3.5 µm (042/HPLC)

- Eluent: 1) ACN/H2O/5% H3PO4 (100/900/10) ml – 75%

- 2) ACN/H2O/5% H3PO4 (900/100/10) ml – 25%

- Column temperature: 40°C

- Flow rate: 1.5 ml/min.

- Injection volume: 2 μl

- Run time: 7 min.

- Retention time of nicosulfuron: about 2.4 min.

- Retention time of DPMU, internal standard: about 4.3 min.

The preparation of test solution for nicosulfuron analysis

Weigh 0.040±0.008 g of the nicosulfuron standard was weighed with accuracy of ± 0.0002 g into the flask of 100 ml, 10 ml of the tetrahydrofuran was added, and the content was mixed. Then 10 ml of the internal standard solution was added with a one-mark pipette and the flask was made up to ¾ of the volume of the acetonitrile. It was left in an ultrasonic bath for 10 minutes and the solution was cooled to ambient temperature (18÷25°C) in a water bath for 30 min. The flask was made to the volume of the acetonitrile and the content was mixed.

The preparation of samples for nicosulfuron analysis

Weigh 0.64 g ± 0.13 g of the preparation was weighed with accuracy of ± 0.0002 g into the flask of 100 ml, 10 ml of the tetrahydrofuran was added and mixed until the preparation was completely dispersed. Then 10 ml of the internal standard solution was added with a one-mark pipette and the flask was made up to ¾ of the volume of the acetonitrile. It was left in an ultrasonic bath for 10 minutes and the solution was cooled to ambient temperature (18÷25°C) in a water bath for 30 min. The flask was made to the volume of the acetonitrile, the content was mixed and filtered through a 0.22 μm PTFE syringe filter prior to injection.

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of active substances nicosulfuron in plant protection product NIC-HER 060 OD

|  | nicosulfuron |
| --- | --- |
| Author(s), year | Zając S, 2021 |
| Principle of method | SANCO/3030/99 rev.5, 22 March 2019 |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | The linearity of the analytical method was assessed using seven nicosulfuron standard solutions in the concentration range from 15.509 mg/L (2.401 g/kg) to 775.428 mg/L (120.035 g/kg) with 10 ml addition of DPMU standard solution about concentration 3.29 g/L. Correlation coefficient: R2 = 0.99994 Required: R2 ≥ 0.99 |
| Precision – Repeatability Mean n = 7 (%RSD) | Hr = 0.10 Required: Hr ≤ 1 RSD = 0.20% Required: RSD ≤ 2.04% |
| Accuracy n = 4 (% Recovery) | Total recovery: 100.54% (range: 100.1% - 101.15%) Required: 90% ÷ 110% |
| Interference/ Specificity | There are no any interferences coming from impurities for the peak of the nicosulfuron and DPMU standard. |
| Comment | No comments. |

Conclusion

The LC method with DAD detector, used to quantify nicosulfuron was fully validated. Method validation included linearity, non-analyte interference, precision, accuracy and specificity. All measured parameters meet the criteria given in SANCO/3030/99 rev.5, 22 March 2019.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

Not relevant. The product NIC-HER 060 OD does not contain relevant impurities.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant. The product NIC-HER 060 OD does not contain materials of toxicological, ecotoxicological or environmental concern.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

No CIPAC method for determination of nicosulfuron in OD formulation exists.

### Methods for the determination of residues (KCP 5.1.2)

An overview on the acceptable methods and possible data gaps for analysis of residues of nicosulfuron for the generation of pre-authorization data is given in the following table. All studies have been previously evaluated at EU level and are described in detail in the DAR for nicosulfuron (UK, 2006). New studies were not submitted.

Table 5.2-2:	Validated methods for the generation of pre-authorization data

| Matrix type | Method type | Method LOQ | Principle of method (i.e., GC-MS or HPLC-UV) | Author(s), year, study no. |
| --- | --- | --- | --- | --- |
| Water Non-target plants – Seedling Emergence and Seedling Growth (Ecoloxicology) | Primary & confirmatory | 0.00005 g/L | UHPLC-TOF-MS/MS | Mautino G., 2023, Study no: 1192.1H.SAG22 |
| Water Terrestrial Plant Vegetative Vigour (Ecoloxicology) | Primary & confirmatory | 0.00005 g/L | UHPLC-TOF-MS/MS | Mautino G., 2023, Study no: 1193.1H.SAG22 |
| Soil Eisenia fetida (Ecoloxicology) | Primary & confirmatory | 0.05 mg/kg (referred to dry soil) | UHPLC-TOF-MS/MS | Mautino G., 2023, Study no: 1191.1H.SAG22 |
| Sucrose solution Bumblebees (Bombus spp.), Acute Oral Toxicity Test (Ecoloxicology) | Primary | 50 mg/kg | HPLC-DAD | Knapik M., 2021, Study no: B-43-21 |
| Water Bumblebees (Bombus spp.), Acute Contact Toxicity (Ecoloxicology) | Primary | 10 mg/kg | HPLC-DAD | Knapik M., 2021, Study no: B-44-21 |
| Aqueous solutions Lemna sp. (Ecoloxicology) | Primary & confirmatory | 0.15 µg/L | HPLC-MS/MS | Tediosi E., 2023, Study no: CH – 0043-2023 |

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues nicosulfuron (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-3:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | nicosulfuron | 0.01 mg/kg | Regulation (EU) No 617/2014 |
| Plant, high acid content |  | 0.01 mg/kg |  |
| Plant, high protein/high starch content (dry commodities) |  | 0.01 mg/kg |  |
| Plant, high oil content |  | 0.01 mg/kg |  |
| Muscle | nicosulfuron | 0.02 mg/kg | Regulation (EU) No 617/2014 |
| Milk |  | 0.02 mg/kg |  |
| Eggs |  | 0.02 mg/kg |  |
| Fat |  | 0.02 mg/kg |  |
| Liver, kidney |  | 0.02 mg/kg |  |
| Soil (Ecotoxicology) | nicosulfuron | 0.05 mg/kg | common limit |
| Drinking water (Human toxicology) | nicosulfuron | 0.1 µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | nicosulfuron | 0.0017 mg a.s./L (EC50 for Lemna gibba) | EFSA Scientific Report (2007) 120, 1-91 |
| Air | nicosulfuron | 240 µg/m3 (AOEL sys = 0.8 mg/kg bw/day) | Calculated according to SANTE/2020/12830, Rev.1 24. February 2021 |
| Tissue (meat or liver) | not required | not required | not classified as T / T+ |
| Body fluids |  | not required | not classified as T / T+ |

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of nicosulfuron in plant matrices is given in the following tables. No new studies have been submitted with this application.

Table 5.3-4:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: nicosulfuron |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e., GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High protein/high starch content (dry) - maize | Primary | LOQ=0.01 mg/kg | LC-MS/MS | Wolf, 2000/DAR |
|  | ILV | LOQ=0.01 mg/kg | LC-MS/MS | Ginzburg, 2000/DAR |
|  | Confirmatory (if required) | Provided by validation of a second transition in the study by Wolf, 2008/DAR |  |  |

Table 5.3-5:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | Not required |
| Not required, because: | No residues of nicosulfuron >LOQ are expected in products of plant origin. |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No significant intake of residues by livestock, no residue definition and no MRLs are proposed for commodities of animal origin. An analytical method for food of animal origin is not required.

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of nicosulfuron in soil is given in the following table. No new studies have been submitted with this application.

Table 5.3-6:	Validated methods for soil (if appropriate)

| Component of residue definition: nicosulfuron |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e., GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.05 µg/kg | LC-MS/MS | Wais, 2000/DAR |
| Primary | 0.05 µg/kg (nicosulfuron) | LC-MS/MS |  |
| Primary | 0.005 mg/kg (nicosulfuron) 0.02 mg/kg (ADMP and ASDM) | HPLC/UV | Huber, 1996/DAR |
| Primary | 0.01 mg/kg (AUSN and UCSN) | LC-MS/MS | Wolf, 2003/DAR |

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of nicosulfuron in surface and drinking water is given in the following tables. No new studies have been submitted with this application.

Table 5.3-7:	Validated methods for water (if appropriate)

| Component of residue definition: nicosulfuron |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e., GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | 0.05 μg/L | HPLC/UV | Schulz and Ullrich-Mitzel, 1995a/DAR |
|  | ILV | 0.05 μg/L | LC-DAD | Wais, 2000/DAR |
|  | Confirmatory | 0.05 μg/L | LC-MS/MS | Wolf, 2007/DAR |
| Surface water | Primary | 0.05 μg/L | LC-DAD | Wais, 2000/DAR |
|  | Confirmatory | - | - | - |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of nicosulfuron in air is given in the following tables. No new studies have been submitted with this application.

Table 5.3-8:	Validated methods for air (if appropriate)

| Component of residue definition: nicosulfuron |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e., GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 1.2 μg/m3 | LC/UV | Schulz and Ullrich-Mitzel, 1995/DAR |
| Primary | 1.2 μg/m3 | LC/UV | Wais, 2000/DAR |
| Confirmatory | - | - | - |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

Methods for body fluids and tissues are not required, because nicosulfuron is not considered to be toxic or very toxic (T / T+) nor is it classified according to CLP regulatory as follows: Acute toxicity (cat. 1 - 3), CMR (cat. 1) or STOT (cat. 1).

#### Other studies/ information

All analytical methods for the determination of residues in crops were provided in the EU review of nicosulfuron and were considered adequate.

According to evaluation report (EFSA Scientific Report (2007) 120, 1-91, Conclusion on the peer review of nicosulfuron) as well as Draft Assessment Report and its addendums, during the evaluation analytical methods for determination of nicosulfuron in corn of maize were submitted. Methods are based on extraction, partition, clean-up and then analyse with HPLC and GC chromatography with detector LC/MS and GC/MS. Acceptable validation data were submitted for analysis of active substance, the validation data submitted for metabolite analysis were less satisfactory, how-ever the residue definition for plant and products is ‘parent nicosulfuron’, therefore the lack of validation data in these cases is not a critical issue. For food and feed of plant origin Limit of Quantification of HPLC-MS/MS method is 0.01 mg/kg. Independent laboratory validation method con-firmed this value (LOQ=0.01 mg/kg).

The LOQ for the analysis of soil using LC/MS method is 0.05 µg/kg. For the analysis of drinking water LOQ = 0.05 μg/L using HPLC/UV method. Independent laboratory validation method was not required. Confirmatory method was LC-DAD and LOQ was 0.05 μg/L. For surface water method used was LC-DAD and LOQ = 0.05 μg/L. Confirmatory method was not required Primary method for analysis of nicosulfuron in air was HPLC/UV. The LOQ determined by this method is 1.2 μg/m3. Confirmatory method is not required.

Summary of validation of analytical methods generated by the applicant and used in dRR sections are provided in Appendix 2.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1/01 | Zając S. | 2021 | NIC-HER 060 OD. Determination of an active substance content in a preparation in an HDPE bottle. Stage 1: Determination of an active substance content in an initial preparation Report No 003/DPL/2021 Pestila Spółka z ograniczoną odpowiedzialnością GLP Yes Unpublished | N | Pestila Sp. z o.o. |
| KCP 5.2/01 | Mautino G. | 2023 | Analytical Phase Report. Effects of NIC-HER 060 OD (nicosulfuron 60 g/L) on terrestrial Non-target plants – Seedling Emergence and Seedling Growth Study code: 1192.1H.SAG22; Test Site code: 22442-02R Renolab S.r.l., Italy GLP Unpublished | N | Pestila Sp. z o.o. |
| KCP 5.2/02 | Mautino G. | 2023 | Analytical Phase Report. Effects of NIC-HER 060 OD (nicosulfuron 60 g/L) on Terrestrial Plant Vegetative Vigour - OECD 227 Study code: 1193.1H.SAG22; Test Site code: 22442-03R Renolab S.r.l., Italy GLP Unpublished | N | Pestila Sp. z o.o. |
| KCP 5.2/03 | Mautino G. | 2023 | Analytical Phase Report. Earthworm Reproduction Test (Eisenia fetida) with NIC-HER 060 OD (nicosulfuron 60 g/L) Study code: 1191.1H.SAG22; Test Site code: 22442-01R Renolab S.r.l., Italy GLP Unpublished | N | Pestila Sp. z o.o. |
| KCP 5.2/04 (filed as KCP 10.3.1.1.1/02) | Knapik M. | 2021 | NIC-HER 060 OD Bumblebees (Bombus spp.), Acute Oral Toxicity Test Study code: B-43-21 Łukasiewicz Research Network – Institute of Industrial Organic Chemistry, Branch Pszczyna GLP Unpublished | N | Pestila Sp. z o.o. |
| KCP 5.2/05 (filed as KCP 10.3.1.1.2/02) | Knapik M. | 2021 | NIC-HER 060 OD Bumblebees (Bombus spp.), Acute Contact Toxicity Test Study code: B-44-21 Łukasiewicz Research Network – Institute of Industrial Organic Chemistry, Branch Pszczyna GLP Unpublished | N | Pestila Sp. z o.o. |
| KCP 5.2/06 | Tediosi E. | 2023 | Validation of the analytical method for the determination of Nicosulfuron content in aqueous solutions coming from ecotoxicological tests with NIC-HER 060 OD Study No. CH – 0043-2023 ChemService S.r.l. Controlli e Ricerche, Italy GLP Unpublished | N | Pestila Sp. z o.o. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.2/01 | Kanza T. and al. | 1991 | Analytical Methods for SL-950 and its metabolites (ADMP, ASDM and 5-HDUD) in crops – Third edition ISK ISK, LTD. Central Research Institute, report no. H-641, 1991-05-08 no GLP unpublished | N | ISK |
| KCP 5.1.2/02 | Ganse, Y. et al. | 1990 | Analytical Methods for SL-950 and its metabolites (ADMP, ASDM and 5-HDUD) in crops – Revised Method ISK ISK, LTD. Central Research Institute, report no. H-623, 1990-05-25 no GLP unpublished | N | ISK |
| KCP 5.1.2/03 | Huber H.P. | 1996 | Compilation of analytical methods for SL-950 and its metabolites ADMP and ASDM in maize (various types) from existing validated residue analyses. ISK RCC Umweltchemie AG, report no. 614351, 1996-06-13 no GLP unpublished | N | ISK |
| KCP 5.1.2/04 | Mirbah M.J. | 1998 | Compilation of confirmatory analytical methods of SL-950 in crop, soil, water and air. ISK RCC Ltd, report no. 699873, 1998-08-15 no GLP unpublished | N | ISK |
| KCP 5.1.2/05 | Wolf St. | 2000 | Development and validation of the residue analytical method for SL-950 (nicosulfuron) in maize (corn and straw). ISK RCC Ltd, report no. 793596, 2000-12-04 GLP unpublished | N | ISK |
| KCP 5.1.2/06 | Ginzburg N. | 2000 | Independent laboratory validation (ILV) of the residue analytical method for SL-950 (nicosulfuron) in maize (corn and straw). ISK Battelle, report no. A-22-00-04, 2000-12-20 GLP unpublished | N | ISK |
| KCP 5.1.2/07 | Wais A. | 2000 | Validation of the residue analytical method for SL-950 (nicosulfuron) in soil. ISK RCC Ltd, report no. 770117, 2000-05-03 GLP unpublished | N | ISK |
| KCP 5.1.2/08 | Huber H.P. | 1996 | Compilation of analytical methods for SL-950 and its metabolites ADMP and ASDM in soil (various types) from existing validated residue analyses. ISK RCC Umweltchemie AG, report no. 614340, 1996-06-13 no GLP unpublished | N | ISK |
| KCP 5.1.2/09 | Wolf St. | 2003 | Development and validation of an analytical method for the determination of AUSN and UCSN in soil. ISK RCC Ltd, report no. 849365, 2003-08-13 GLP unpublished | N | ISK |
| KCP 5.1.2/10 | Schulz, M. and Ullrich-Mitzel, A. | 1995 | SL-950: Validation of analytical method for determination of residues in drinking water. ISK RCC Umweltchemie AG, report no. 604383, 1995-12-21 GLP unpublished | N | ISK |
| KCP 5.1.2/11 | Wais, A. and Ullrich-Mitzel, A. | 1997 | Development and validation of analytical methods for determination of residues of ADMP, ASDM and AUSN in drinking water. ISK RCC Umweltchemie AG, report no. 604574, 1997-02-19 GLP unpublished | N | ISK |
| KCP 5.1.2/12 | Wais A. | 2000 | Development and validation of the residue analytical method for SL-950 (nicosulfuron) in surface water. ISK RCC Ltd, report no. 770128, 2000-07-19 GLP unpublished | N | ISK |
| KCP 5.1.2/13 | Wolf St. | 2007 | Development and validation of a residue analytical method for nicosulfuron in drinking water and surface water ISK RCC Study No B25773, GLP unpublished | N | ISK |
| KCP 5.1.2/14 | Wais A. | 2000 | Validation of the residue analytical method for SL-950 (nicosulfuron) in air. ISK RCC Ltd, report no. 765358, 2000-05-17 GLP unpublished | N | ISK |
| KCP 5.1.2/15 | Schulz, M. and Ullrich-Mitzel, A. | 1995 | Analytical method for the determination of SL-950 in air. ISK RCC Umweltchemie AG, report no. 385470, 1995-12-13 GLP unpublished | N | ISK |
| KCP 5.1.2/16 | Ducat, N. Pigeon, O. | 2004 | Independent Laboratory validation of DuPont-11776, “Analytical Enforcement Method for the Determination of Nicosulfuron in Corn Matrices using HPLC/ESI-MS/MS Centre wallon de Recherches Agronomiques (CRA-W) Report: DuPont-12347 GLP: yes Published: No | N | E.I. du Pont de Nemours and Company |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for nicosulfuron

    - Methods used for the generation of pre-authorization data (KCP 5.1)

Please refer to the points 5.2.1.1 and 5.2.1.2.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted.

      - A.2.A.9	Other Studies/ Information

        - UHPLC-TOF-MS/MS (in water)

          - Method validation

|  | Comments of Evaluator: | Method is accepted. |  |  |
| --- | --- | --- | --- | --- |
| Reference: |  |  | KCP 5.2/01 |  |
| Report |  |  | Analytical Phase Report. Effects of NIC-HER 060 OD (nicosulfuron 60 g/L) on terrestrial Non-target plants – Seedling Emergence and Seedling Growth, Mautino G., 2023, Study code: 1192.1H.SAG22; Test Site code: 22442-02R |  |
| Guidelines: |  |  | SANTE/2020/12830, rev.2. |  |
| Deviations: |  |  | No |  |
| GLP: |  |  | Yes |  |
| Acceptability: |  |  | Yes |  |

Principle of the method

The content of nicosulfuron active ingredient was determined in the highest concentration of the water stock solutions prepared in the biological phase of the study.

The analytical method for nicosulfuron in water was fully validated according to the guideline SANTE/2020/12830 rev.2, of 14 February 2023, by calibration (linearity), selectivity, specificity, blank samples analysis, recoveries, accuracy, repeatability, matrix effect, limit of quantification (LOQ) and limit of detection (LOD).

Nicosulfuron was determined in water stock solutions after sample dilution with acetonitrile and the final analysis was performed in positive ionisation mode by Ultra High-Performance Liquid Chromatography, tandem High Resolution Mass Spectrometry (UHPLC-TOF-MS/MS).

Three ions were acquired, the Parent Ion 411.11 (target) for quantification purpose and the fragment Ions 182.05 and 213.03 (qualifiers) for confirmation.

Materials and methods

Equipment

- Standard laboratory glassware and equipment

- Water purification system Direct-Q 3UV, MILLIPORE

- Analytical electronical balance with 0.1 mg readability AT 261, METTLER TOLEDO

- Automatic pipette 2-20 µL, Pipetman, GILSON

- Automatic pipette 20-200 µL, Finpipette F2, THERMO SCIENTIFIC

- Automatic pipette 100-1000 µL, Finpipette F2, THERMO SCIENTIFIC

- UHPLC column Zorbax Eclipse Plus C18, 2.1 x 50 mm, 1.8 µm, AGILENT

- LC-TOF-MS/MS System HPLC Series 1260 AGILENT and Triple TOF Spectrometer 4600 AB SCIEX

Reagents and materials

- Acetonitrile for HPLC gradient grade, CARLO ERBA ≥ 99.9 %

- MilliQ Water produced in lab by Direct-Q 3UV, MILLIPORE

- Acetic acid glacial ≥ 99.9%, CARLO ERBA

Instrumental parameters LC-TOF-MS/MS

| System | UHPLC Series 1290 Infinity AGILENT |  |  |  |
| --- | --- | --- | --- | --- |
|  | Triple TOF 4600 AB SCIEX (ESI) |  |  |  |
| Analytical Column | ZORBAZ Eclipse Plus C18, 2.1 x 50 mm, 1.8 µm, AGILENT |  |  |  |
| Retention time | ~2.7 min |  |  |  |
| UHPLC | Mobile phases: Solvent A: LC-MS Water 1 % acetic acid Solvent B: Acetonitrile 1 % acetic acid |  |  |  |
|  | Pump Gradient: from 0 to 1 min: A 90 % - B 10 % 5 min: A 0 % - B 100 % from 5 to 8 min: A 0 % - B 100 % 8.5 min: A 90 % - B 10 % to 10 min: A 90 % - B 10 % |  |  |  |
|  | Flow rate: 0.5 mL/min |  |  |  |
|  | Column Temperature: 35 °C |  |  |  |
|  | Injection volume: 5 μL |  |  |  |
| TOF – MS/MS | Duration: | 10 min |  |  |
|  | Delay: | 0 sec |  |  |
|  |  | Experiment 1 | Experiment 2 |  |
|  | Scan type: | Positive TOF MS | Positive Product Ion (of 411.11 Da) |  |
|  | Temperature (TEM): | 500 °C | 500 °C |  |
|  | Curtain gas (CUR): | 25 psi | 25 psi |  |
|  | Ion Spray Voltage (ISVF): | 5500 V | 5500 V |  |
|  | Gas 1: | 45 psi | 45 psi |  |
|  | Gas 2: | 45 psi | 45 psi |  |
|  | TOF Masses range: | 50 - 700 Da | 50 - 700 Da |  |
|  | Accumulation time: | 0.2 sec | 0.2 sec |  |
|  | DP: | 30 | 30 |  |
|  | CE: | 10 | 25 |  |
|  | CES: | - | 0 |  |
| Ions | Nicosulfuron (primary): m/z (411.1061 – 411.1108), from Experiment 1 Nicosulfuron-1 (confirmatory): m/z (182.0544 – 182.0562), from Experiment 2 Nicosulfuron-2 (confirmatory): m/z (213.0317 – 213.0339), from Experiment 2 |  |  |  |

Stock and working reference item solutions

10.1 mg of the reference item were accurately weighed into a 20 mL volumetric flask and brought to volume with acetonitrile (stock solution at 486.8 μg/mL, taking into account 96.4 % purity; stored in freezer at a nominal temperature of - 18 °C).

By dilution of the reference item stock solution described above in acetonitrile reference item working solutions were prepared, as reported in the table below.

| Nicosulfuron reference item parent solution concentration (μg/mL) | Taken Volume (mL) | Final Volume (mL) | Nicosulfuron reference item working solution concentration (μg/mL) |
| --- | --- | --- | --- |
| 486.8 | 0.2 | 10 | 9.74 |
| 9.74 | 0.05 | 1 | 0.487 |

The nicosulfuron standard solutions were freshly prepared on the same analysis day.

Stock test item solution and fortification procedure

1502 mg of the test item were transferred into a 10 mL volumetric flask and diluted to volume with pure water. This test item solution (containing 9393 ug/mL of nicosulfuron active ingredient) was used as spiking solution for recovery at the second fortification level: 0.43 mL of this solution were transferred into a 10-mL volumetric flask and brought to volume with water (II level recovery at 0.404 g/L nicosulfuron concentration.

By dilution of this test item solution (0.1 mL in 200 mL pure water) a solution at nicosulfuron concentration of 4.697 µg/mL was obtained: this solution was used to prepare the water test item fortification solution for LOQ recovery trials: 0.11 mL of this solution were transferred into a 10 mL volumetric flask and brought to volume with water (LOQ recovery at 0.00005 g/L nicosulfuron concentration).

Reference item calibration solutions

Reference item calibration solutions were prepared in acetonitrile according to the following table.

| Spiking standard solution concentration (µg/mL) | Taken Volume (mL) | Total Final Volume (mL) | Calibration solution concentration (ng/mL) | Concentration in sample unit matrix water (g/L) |
| --- | --- | --- | --- | --- |
| 486.8 | 0.2 | 1 | 97.4 | 0.001 |
| 486.8 | 0.1 | 1 | 48.7 | 0.0005 |
| 486.8 | 0.05 | 1 | 24.3 | 0.00025 |
| 97.4 | 0.1 | 1 | 9.74 | 0.00010 |
| 97.4 | 0.05 | 1 | 4.87 (a) | 0.00005 |
| 9.74 | 0.1 | 1 | 0.974 | 0.00001 |

(a) Nominal for LOQ

Analytical procedure for sample extracts preparation

The sample was let to warm up to room temperature, then vigorously shaken by hand for a minute and immediately 1 mL is taken and brought to 10 mL with acetonitrile.

Sample extracts with analyte concentration exceeding 800 ng/mL were opportunely diluted with acetonitrile in order to fall within the ± 20% of the calibration range.

Recovery samples at LOQ were prepared by taking 1 mL of the solution at LOQ level and reaching 10 mL with acetonitrile.

Recovery samples at second level were prepared by taking 1 mL of the solution at second level and reaching 10 mL with acetonitrile. This recovery samples are opportunely diluted with ACN in order to fall within the ± 20% of the calibration range.

The extract was finally analysed by HPLC-TOF-MS/MS.

Quantification was performed using solvent calibration standards in the concentration range 1 - 100 ng/mL.

Validation of method

Linearity and instrumental precision

The linearity range for nicosulfuron was found between 1 - 100 ng/mL corresponding to the range from 0.00001 to 0.001 g/L of nicosulfuron in water samples. The correlation coefficient of the weighed linear (1/x) multipoint external standard solvent calibration curve was found ≥ 0.999 in the analytical sequence performed.

The linearity range comprised the concentration range from 20% of the LOQ to at least 20% above the highest measured concentration.

The suitability of the calibration line was assessed using the residuals di that describes the vertical distance of measured values from the regression curve according to:

di = yi - yyi

where:

yi is the measured value i;

yyi is the estimated value which corresponds to yi and is derived from the calibration function.

The regression residuals were presented in residual plots and visual inspections were applied to decide if di were randomly distributed and hence linear calibration is demonstrated: no trend was visible by plotting the residuals vs the concentration.

The linearity range comprised the concentration range of the samples ± 20%.

Blank and selectivity

Two independent analyses of the blank sample were performed: no significant interference exceeding 20% of the limit of quantification were found at the retention time of nicosulfuron for both the monitored transitions.

Therefore, nicosulfuron can be regarded as not detectable in untreated water sample used in fortification trials (< 20% of LOQ).

The retention time of the reference item matched the retention time of the analyte in extracts from fortified samples.

Based on the analysis of the blank matrix, the method was confirmed to be selective for the analysis of nicosulfuron in water solution matrix, without significant interferences above 20% of LOQ.

Specificity

Nicosulfuron was analysed by UHPLC-TOF-MS/MS highly specific detection system; three ions were simultaneously acquired: the Parent Ion 411.11 (target) for quantification purpose and the fragment Ions 182.05 and 213.03 (qualifiers) for confirmation.

The mass spectrum of the analyte was acquired in the range 50-700 m/z.

Recoveries

The analytical method was validated by recovery trials: a known quantity of the test item was added to the control sample and the percentage recovery was calculated.

The recoveries were performed by fortifying the untreated blank at two levels.

The LOQ level was set at nicosulfuron concentration of 0.00005 g/L (0.0008 g/L as test item), lower than the minimum found nicosulfuron content in samples, while the second level was at 0.4 g/L (6.4 g/L as test item), higher than the maximum expected concentration in the samples, in order to cover with the method validation all the range of nicosulfuron concentrations in the analytical samples; five replicated analyses were carried out for each fortification level.

The background content in the control sample used for dilution of test item in fortification experiments was not detectable.

In these recovery samples the nicosulfuron content was determined as reported in table below.

| Fortification level (g/L) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.00005 (a) (LOQ) | 95.6 | 112.7 | 9.7 | 93.0 ± 24.1 |
|  | 116.5 |  |  |  |
|  | 111.3 |  |  |  |
|  | 114.8 |  |  |  |
|  | 125.5 |  |  |  |
| 0.404 (b) (II level) | 74.7 | 73.4 | 8.8 |  |
|  | 74.8 |  |  |  |
|  | 64.0 |  |  |  |
|  | 81.8 |  |  |  |
|  | 71.5 |  |  |  |

(a) 0.0008 g/L as test item

(b) 6.4 g/L as test item

For each fortification level the mean recovery and the precision (RSD, relative standard deviation) are in compliance with the requirements of guideline SANTE/2020/12830 rev.2.

Accuracy

The accuracy of the analysis method for nicosulfuron in water, defined as mean recovery ± relative standard deviation, is 93.0 ± 24.1.

Repeatability

The repeatability, defined as the % RSD (Relative Standard Deviation) at each fortification level, and the overall RSD is reported in the table below.

| Fortification level (g/L) | RSD % (n = 5) | Overall RSD % (n = 10) |
| --- | --- | --- |
| 0.00005 (a) (LOQ) | 9.7 | 24.1 |
| 0.4 (b) (II level) | 8.8 |  |

(a) 0.0008 g/L as test item

(b) 6.4 g/L as test item

Matrix effects

Matrix effects were not assessed: considering the dilution solvent of sample matrix (acetonitrile) and the use of the same solvent for standards and samples dilution (acetonitrile) the nicosulfuron analyte in the solutions used for calibration and quantification is in the same medium as the samples.

Confirmation

The confirmation of the analyte identity is simultaneous to the primary detection by the acquisition of the additional transition.

The recovery data and the precision data for the additional transition are reported in the tables below.

Validation data for nicosulfuron (confirmatory ion 182.05)

| Fortification level (g/L) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.00005 (a) (LOQ) | 116.3 | 109.0 | 4.7 | 93.5 ± 18.9 |
|  | 106.9 |  |  |  |
|  | 103.6 |  |  |  |
|  | 106.2 |  |  |  |
|  | 112.1 |  |  |  |
| 0.404 (b) (II level) | 77.6 | 77.9 | 11.0 |  |
|  | 79.1 |  |  |  |
|  | 64.5 |  |  |  |
|  | 88.3 |  |  |  |
|  | 80.0 |  |  |  |

(a) 0.0008 g/L as test item

(b) 6.4 g/L as test item

Validation data for nicosulfuron (confirmatory ion 213.03)

| Fortification level (g/L) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.00005 (a) (LOQ) | 96.3 | 104.3 | 11.9 | 88.9 ± 21.3 |
|  | 87.7 |  |  |  |
|  | 107.8 |  |  |  |
|  | 110.9 |  |  |  |
|  | 119.0 |  |  |  |
| 0.404 (b) (II level) | 70.9 | 73.4 | 9.9 |  |
|  | 73.4 |  |  |  |
|  | 63.8 |  |  |  |
|  | 84.0 |  |  |  |
|  | 75.1 |  |  |  |

(a) 0.0008 g/L as test item

(b) 6.4 g/L as test item

Also, for the confirmatory transition, the mean recovery and the precision (RSD, relative standard deviation) at each fortification level are in compliance with the requirement of guideline SANTE/2020/12830, rev.2.

Stability of final extracts and reference item solutions

The final extracts were analysed within 24 hours form extraction; moreover, the stability of the extracts during the analysis was proven by the acceptability of recoveries performed concurrently with the samples analysis.

The stability of nicosulfuron reference item stock solution was verified in the GLP study 1191.1H.SAG22 (Renolab code 22442-01R) at ≤ - 18 °C in the dark: the means from at least 5 replicate measurements for a fresh solution compared to a stored one (at ≤ - 18 °C in the dark) should not differ by more than 10%.

Limits of quantification and detection

The limit of quantification (LOQ) is defined as the lowest concentration tested at which an unambiguous identification of the analyte can be proven and at which an acceptable mean recovery with an acceptable relative standard deviation (RSD) is obtained.

The LOQ for nicosulfuron in water was assessed in this study at 0.00005 g/L.

The limit of detection is the lowest amount that can be detected but not necessarily quantitated as an exact value.

For the analysis of nicosulfuron in water the LOD is 0.00001 g/L. This value was calculated from the nicosulfuron concentration corresponding to the lowest calibration point.

Conclusion

The data presented above confirm that the validated analytical method provides a specific, reliable, accurate and precise procedure for the determination of nicosulfuron active ingredient in water samples in the range 0.00005 – 0.4 g/L (corresponding to 0.0008 – 6.4 g/L as test item and corresponding to 0.33 – 2640 mL f.p./ha).

As the analysis of the samples was performed during the recovery tests, the reliability of the found values is demonstrated.

        - UHPLC-TOF-MS/MS (in water)

          - Method validation

|  | Comments of Evaluator: | Method is accepted. |  |  |
| --- | --- | --- | --- | --- |
| Reference: |  |  | KCP 5.2/02 |  |
| Report |  |  | Analytical Phase Report. Effects of NIC-HER 060 OD (nicosulfuron 60 g/L) on Terrestrial Plant Vegetative Vigour - OECD 227, Mautino G., 2023, Study code: 1193.1H.SAG22; Test Site code: 22442-03R |  |
| Guidelines: |  |  | SANTE/2020/12830, rev.2. |  |
| Deviations: |  |  | No |  |
| GLP: |  |  | Yes |  |
| Acceptability: |  |  | Yes |  |

Principle of the method

The content of nicosulfuron active ingredient was determined in the highest concentration of the water stock solutions prepared in the biological phase of the study.

The analytical method for nicosulfuron in water was fully validated according to the guideline SANTE/2020/12830 rev.2, of 14 February 2023, by calibration (linearity), selectivity, specificity, blank samples analysis, recoveries, accuracy, repeatability, matrix effect, limit of quantification (LOQ) and limit of detection (LOD).

Nicosulfuron was determined in water stock solutions after sample dilution with acetonitrile and the final analysis was performed in positive ionisation mode by Ultra High-Performance Liquid Chromatography, tandem High Resolution Mass Spectrometry (UHPLC-TOF-MS/MS).

Three ions were acquired, the Parent Ion 411.11 (target) for quantification purpose and the fragment Ions 182.05 and 213.03 (qualifiers) for confirmation.

Materials and methods

Equipment

- Standard laboratory glassware and equipment

- Water purification system Direct-Q 3UV, MILLIPORE

- Analytical electronical balance with 0.1 mg readability AT 261, METTLER TOLEDO

- Automatic pipette 2-20 µL, Pipetman, GILSON

- Automatic pipette 20-200 µL, Finpipette F2, THERMO SCIENTIFIC

- Automatic pipette 100-1000 µL, Finpipette F2, THERMO SCIENTIFIC

- UHPLC column Zorbax Eclipse Plus C18, 2.1 x 50 mm, 1.8 µm, AGILENT

- LC-TOF-MS/MS System HPLC Series 1260 AGILENT and Triple TOF Spectrometer 4600 AB SCIEX

Reagents and materials

- Acetonitrile for HPLC gradient grade, CARLO ERBA ≥ 99.9 %

- MilliQ Water produced in lab by Direct-Q 3UV, MILLIPORE

- Acetic acid glacial ≥ 99.9%, CARLO ERBA

Instrumental parameters LC-TOF-MS/MS

| System | UHPLC Series 1290 Infinity AGILENT |  |  |  |
| --- | --- | --- | --- | --- |
|  | Triple TOF 4600 AB SCIEX (ESI) |  |  |  |
| Analytical Column | ZORBAZ Eclipse Plus C18, 2.1 x 50 mm, 1.8 µm, AGILENT |  |  |  |
| Retention time | ~2.7 min |  |  |  |
| UHPLC | Mobile phases: Solvent A: LC-MS Water 1 % acetic acid Solvent B: Acetonitrile 1 % acetic acid |  |  |  |
|  | Pump Gradient: from 0 to 1 min: A 90 % - B 10 % 5 min: A 0 % - B 100 % from 5 to 8 min: A 0 % - B 100 % 8.5 min: A 90 % - B 10 % to 10 min: A 90 % - B 10 % |  |  |  |
|  | Flow rate: 0.5 mL/min |  |  |  |
|  | Column Temperature: 35 °C |  |  |  |
|  | Injection volume: 5 μL |  |  |  |
| TOF – MS/MS | Duration: | 10 min |  |  |
|  | Delay: | 0 sec |  |  |
|  |  | Experiment 1 | Experiment 2 |  |
|  | Scan type: | Positive TOF MS | Positive Product Ion (of 411.11 Da) |  |
|  | Temperature (TEM): | 500 °C | 500 °C |  |
|  | Curtain gas (CUR): | 25 psi | 25 psi |  |
|  | Ion Spray Voltage (ISVF): | 5500 V | 5500 V |  |
|  | Gas 1: | 45 psi | 45 psi |  |
|  | Gas 2: | 45 psi | 45 psi |  |
|  | TOF Masses range: | 50 - 700 Da | 50 - 700 Da |  |
|  | Accumulation time: | 0.2 sec | 0.2 sec |  |
|  | DP: | 30 | 30 |  |
|  | CE: | 10 | 25 |  |
|  | CES: | - | 0 |  |
| Ions | Nicosulfuron (primary): m/z (411.1061 – 411.1108), from Experiment 1 Nicosulfuron-1 (confirmatory): m/z (182.0544 – 182.0562), from Experiment 2 Nicosulfuron-2 (confirmatory): m/z (213.0317 – 213.0339), from Experiment 2 |  |  |  |

Stock and working reference item solutions

10.1 mg of the reference item were accurately weighed into a 20 mL volumetric flask and brought to volume with acetonitrile (stock solution at 486.8 μg/mL, taking into account 96.4 % purity; stored in freezer at a nominal temperature of - 18 °C).

By dilution of the reference item stock solution described above in acetonitrile reference item working solutions were prepared, as reported in the table below.

| Nicosulfuron reference item parent solution concentration (μg/mL) | Taken Volume (mL) | Final Volume (mL) | Nicosulfuron reference item working solution concentration (μg/mL) |
| --- | --- | --- | --- |
| 486.8 | 0.2 | 10 | 9.74 |
| 9.74 | 0.05 | 1 | 0.487 |

The nicosulfuron standard solutions were freshly prepared on the same analysis day.

Stock test item solution and fortification procedure

1502 mg of the test item were transferred into a 10 mL volumetric flask and diluted to volume with pure water. This test item solution (containing 9393 ug/mL of nicosulfuron active ingredient) was used as spiking solution for recovery at the second fortification level: 0.43 mL of this solution were transferred into a 10-mL volumetric flask and brought to volume with water (II level recovery at 0.404 g/L nicosulfuron concentration.

By dilution of this test item solution (0.1 mL in 200 mL pure water) a solution at nicosulfuron concentration of 4.697 µg/mL was obtained: this solution was used to prepare the water test item fortification solution for LOQ recovery trials: 0.11 mL of this solution were transferred into a 10 mL volumetric flask and brought to volume with water (LOQ recovery at 0.00005 g/L nicosulfuron concentration).

Reference item calibration solutions

Reference item calibration solutions were prepared in acetonitrile according to the table below.

| Spiking standard solution concentration (µg/mL) | Taken Volume (mL) | Total Final Volume (mL) | Calibration solution concentration (ng/mL) | Concentration in sample unit matrix water (g/L) |
| --- | --- | --- | --- | --- |
| 486.8 | 0.2 | 1 | 97.4 | 0.001 |
| 486.8 | 0.1 | 1 | 48.7 | 0.0005 |
| 486.8 | 0.05 | 1 | 24.3 | 0.00025 |
| 97.4 | 0.1 | 1 | 9.74 | 0.00010 |
| 97.4 | 0.05 | 1 | 4.87 (a) | 0.00005 |
| 9.74 | 0.1 | 1 | 0.974 | 0.00001 |

(a) Nominal for LOQ

Analytical procedure for sample extracts preparation

The sample was let to warm up to room temperature, then vigorously shaken by hand for a minute and immediately 1 mL is taken and brought to 10 mL with acetonitrile.

Sample extracts with analyte concentration exceeding 800 ng/mL were opportunely diluted with acetonitrile in order to fall within the ± 20% of the calibration range.

Recovery samples at LOQ were prepared by taking 1 mL of the solution at LOQ level and reaching 10 mL with acetonitrile.

Recovery samples at second level were prepared by taking 1 mL of the solution at second level and reaching 10 mL with acetonitrile. This recovery samples are opportunely diluted with ACN in order to fall within the ± 20% of the calibration range.

The extract was finally analysed by HPLC-TOF-MS/MS.

Quantification was performed using solvent calibration standards in the concentration range 1 - 100 ng/mL.

Validation of method

Linearity and instrumental precision

The linearity range for nicosulfuron was found between 1 - 100 ng/mL corresponding to the range from 0.00001 to 0.001 g/L of nicosulfuron in water samples. The correlation coefficient of the weighed linear (1/x) multipoint external standard solvent calibration curve was found ≥ 0.999 in the analytical sequence performed.

The linearity range comprised the concentration range from 20% of the LOQ to at least 20% above the highest measured concentration.

The suitability of the calibration line was assessed using the residuals di that describes the vertical distance of measured values from the regression curve according to:

di = yi - yyi

where:

yi is the measured value i;

yyi is the estimated value which corresponds to yi and is derived from the calibration function.

The regression residuals were presented in residual plots and visual inspections were applied to decide if di were randomly distributed and hence linear calibration is demonstrated: no trend was visible by plotting the residuals vs the concentration.

The linearity range comprised the concentration range of the samples ± 20%.

Blank and selectivity

Two independent analyses of the blank sample were performed: no significant interference exceeding 20% of the limit of quantification were found at the retention time of nicosulfuron for both the monitored transitions.

Therefore, nicosulfuron can be regarded as not detectable in untreated water sample used in fortification trials (< 20% of LOQ).

The retention time of the reference item matched the retention time of the analyte in extracts from fortified samples.

Based on the analysis of the blank matrix, the method was confirmed to be selective for the analysis of nicosulfuron in water solution matrix, without significant interferences above 20% of LOQ.

Specificity

Nicosulfuron was analysed by UHPLC-TOF-MS/MS highly specific detection system; three ions were simultaneously acquired: the Parent Ion 411.11 (target) for quantification purpose and the fragment Ions 182.05 and 213.03 (qualifiers) for confirmation.

The mass spectrum of the analyte was acquired in the range 50-700 m/z .

Recoveries

The analytical method was validated by recovery trials: a known quantity of the test item was added to the control sample and the percentage recovery was calculated.

The recoveries were performed by fortifying the untreated blank at two levels.

The LOQ level was set at nicosulfuron concentration of 0.00005 g/L (0.0008 g/L as test item), lower than the minimum found nicosulfuron content in samples, while the second level was at 0.4 g/L (6.4 g/L as test item), higher than the maximum expected concentration in the samples, in order to cover with the method validation all the range of nicosulfuron concentrations in the analytical samples; five replicated analyses were carried out for each fortification level.

The background content in the control sample used for dilution of test item in fortification experiments was not detectable.

In these recovery samples the nicosulfuron content was determined as reported in table below.

| Fortification level (g/L) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.00005 (a) (LOQ) | 95.6 | 112.7 | 9.7 | 93.0 ± 24.1 |
|  | 116.5 |  |  |  |
|  | 111.3 |  |  |  |
|  | 114.8 |  |  |  |
|  | 125.5 |  |  |  |
| 0.404 (b) (II level) | 74.7 | 73.4 | 8.8 |  |
|  | 74.8 |  |  |  |
|  | 64.0 |  |  |  |
|  | 81.8 |  |  |  |
|  | 71.5 |  |  |  |

(a) 0.0008 g/L as test item
(b) 6.4 g/L as test item

For each fortification level the mean recovery and the precision (RSD, relative standard deviation) are in compliance with the requirements of guideline SANTE/2020/12830 rev.2.

Accuracy

The accuracy of the analysis method for nicosulfuron in water, defined as mean recovery ± relative standard deviation, is 93.0 ± 24.1.

Repeatability

The repeatability, defined as the % RSD (Relative Standard Deviation) at each fortification level, and the overall RSD is reported in the table below.

| Fortification level (g/L) | RSD % (n = 5) | Overall RSD % (n = 10) |
| --- | --- | --- |
| 0.00005 (a) (LOQ) | 9.7 | 24.1 |
| 0.4 (b) (II level) | 8.8 |  |

Matrix effects

Matrix effects were not assessed: considering the dilution solvent of sample matrix (acetonitrile) and the use of the same solvent for standards and samples dilution (acetonitrile) the nicosulfuron analyte in the solutions used for calibration and quantification is in the same medium as the samples.

Confirmation

The confirmation of the analyte identity is simultaneous to the primary detection by the acquisition of the additional transition.

The recovery data and the precision data for the additional transition are reported in the tables below.

Validation data for nicosulfuron (confirmatory ion 182.05)

| Fortification level (g/L) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.00005 (a) (LOQ) | 116.3 | 109.0 | 4.7 | 93.5 ± 18.9 |
|  | 106.9 |  |  |  |
|  | 103.6 |  |  |  |
|  | 106.2 |  |  |  |
|  | 112.1 |  |  |  |
| 0.404 (b) (II level) | 77.6 | 77.9 | 11.0 |  |
|  | 79.1 |  |  |  |
|  | 64.5 |  |  |  |
|  | 88.3 |  |  |  |
|  | 80.0 |  |  |  |

(a) 0.0008 g/L as test item

(b) 6.4 g/L as test item

Validation data for nicosulfuron (confirmatory ion 213.03)

| Fortification level (g/L) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.00005 (a) (LOQ) | 96.3 | 104.3 | 11.9 | 88.9 ± 21.3 |
|  | 87.7 |  |  |  |
|  | 107.8 |  |  |  |
|  | 110.9 |  |  |  |
|  | 119.0 |  |  |  |
| 0.404 (b) (II level) | 70.9 | 73.4 | 9.9 |  |
|  | 73.4 |  |  |  |
|  | 63.8 |  |  |  |
|  | 84.0 |  |  |  |
|  | 75.1 |  |  |  |

(a) 0.0008 g/L as test item

(b) 6.4 g/L as test item

Also, for the confirmatory transition, the mean recovery and the precision (RSD, relative standard deviation) at each fortification level are in compliance with the requirement of guideline SANTE/2020/12830, rev.2.

Stability of final extracts and reference item solutions

The final extracts were analysed within 24 hours form extraction; moreover, the stability of the extracts during the analysis was proven by the acceptability of recoveries performed concurrently with the samples analysis.

The stability of nicosulfuron reference item stock solution will be verified in the GLP study 1191.1H.SAG22 (Renolab code 22442-01R) at ≤ - 18 °C in the dark: the means from at least 5 replicate measurements for a fresh solution compared to a stored one (at ≤ - 18 °C in the dark) should not differ by more than 10%.

Limit of quantification (LOQ) and limit of Detection (LOD)

The limit of quantification (LOQ) is defined as the lowest concentration tested at which an unambiguous identification of the analyte can be proven and at which an acceptable mean recovery with an acceptable relative standard deviation (RSD) is obtained.

The LOQ for nicosulfuron in water was assessed in this study at 0.00005 g/L.

The limit of detection is the lowest amount that can be detected but not necessarily quantitated as an exact value.

For the analysis of nicosulfuron in water the LOD is 0.00001 g/L. This value was calculated from the nicosulfuron concentration corresponding to the lowest calibration point.

Conclusion

The data presented above confirm that the validated analytical method provides a specific, reliable, accurate and precise procedure for the determination of nicosulfuron active ingredient in water samples in the range 0.00005 – 0.4 g/L (corresponding to 0.0008 – 6.4 g/L as test item and corresponding to 0.33 – 2640 mL f.p./ha).

As the analysis of the samples was performed during the recovery tests, the reliability of the found values is demonstrated.

        - UHPLC-TOF-MS/MS (in soil)

          - Method validation

|  | Comments of Evaluator: | Method is accepted. |  |  |
| --- | --- | --- | --- | --- |
| Reference: |  |  | KCP 5.2/03 |  |
| Report |  |  | Analytical Phase Report. Earthworm Reproduction Test (Eisenia fetida) with NIC-HER 060 OD (nicosulfuron 60 g/L), Mautino G., 2023, Study code: 1191.1H.SAG22; Test Site code: 22442-01R |  |
| Guidelines: |  |  | SANTE/2020/12830, rev.1. |  |
| Deviations: |  |  | The analysis of soil samples was performed by UHPLC-TOF/MS, instead of UHPLC-MS/MS instrument. No impact on the study. |  |
| GLP: |  |  | Yes |  |
| Acceptability: |  |  | Yes |  |

Principle of the method

The content of nicosulfuron active ingredient was determined in soil samples collected during the biological phase of the study.

The analytical method for nicosulfuron in soil was fully validated according to the guideline SANTE/2020/12830 rev.1, of 24 February 2021, by calibration (linearity), selectivity, specificity, blank samples analysis, recoveries, accuracy, repeatability, matrix effects, limit of quantification (LOQ) and limit of detection (LOD).

Nicosulfuron was extracted from soil samples with acetonitrile; the final analysis was performed in positive ionisation mode by Ultra-High Performance Liquid Chromatography, tandem High Resolution Mass Spectrometry (UHPLC-TOF-MS/MS).

Three ions were acquired, the Parent Ion 411.11 (target) for quantification purpose and the fragment Ions 182.05 and 213.03 (qualifiers) for confirmation.

Materials and methods

Equipment

- Standard laboratory glassware and equipment

- Analytical electronical balance with 0.1 mg accuracy, AT 261, METTLER TOLEDO

- Technical electronic balance with 0.01 g accuracy PS 2100.R2 , RADWAG

- Automatic pipette 2-20 µL, P20, GILSON

- Automatic pipette 20-200 µL, Finnpipette F2, THERMO SCIENTIFIC

- Automatic pipette 100-1000 µL, DV1000, DISCOVERY COMFORT

- Thermostatic Oven, model 11120-VF, MPM Instruments

- Centrifuge Megafuge 16, HERAEUS

- Water purification system Direct-Q 3UV, MILLIPORE

- UHPLC column Zorbax Eclipse Plus C18, 2.1 x 50 mm, 1.8 µm, AGILENT

- LC-TOF-MS/MS System HPLC Series 1260 AGILENT and Triple TOF Spectrometer 4600 AB SCIEX

Reagents and materials

- MilliQ Water from Direct-Q 3UV MILLIPORE

- Acetonitrile for UHPLC-MS, CARLO ERBA

- Formic acid ≥ 95 %, SIGMA ALDRICH

- Acetic acid glacial ≥ 99.9 %, CARLO ERBA

- Ammonium carbonate, VWR

- Quechers extractive kit EN method, (batch 225015), SHARLAU

- Syringe PTFE filters 0.22 µm, 13 mm diameter, J.T. Baker

- Single use syringes

Instrumental parameters LC-TOF-MS/MS

| System | UHPLC Series 1290 Infinity AGILENT |  |  |  |
| --- | --- | --- | --- | --- |
|  | Triple TOF 4600 AB SCIEX (ESI) |  |  |  |
| Analytical Column | ZORBAX Eclipse Plus C18, 2.1 x 50 mm, 1.8 µm, AGILENT |  |  |  |
| Retention time | ~2.7 min |  |  |  |
| UHPLC | Mobile phases: Solvent A: LC-MS Water 1 % acetic acid Solvent B: Acetonitrile 1 % acetic acid |  |  |  |
|  | Pump Gradient: from 0 to 1 min: A 90 % - B 10 % 5 min: A 0 % - B 100 % from 5 to 8 min: A 0 % - B 100 % 8.5 min: A 90 % - B 10 % to 10 min: A 90 % - B 10 % |  |  |  |
|  | Flow rate: 0.5 mL/min |  |  |  |
|  | Column Temperature: 35 °C |  |  |  |
|  | Injection volume: 5 μL |  |  |  |
| TOF – MS/MS | Duration: | 10 min |  |  |
|  | Delay: | 0 sec |  |  |
|  |  | Experiment 1 | Experiment 2 |  |
|  | Scan type: | Positive TOF MS | Positive Product Ion (of 411.11 Da) |  |
|  | Temperature (TEM): | 500 °C | 500 °C |  |
|  | Curtain gas (CUR): | 25 psi | 25 psi |  |
|  | Ion Spray Voltage (ISVF): | 5500 V | 5500 V |  |
|  | Gas 1: | 45 psi | 45 psi |  |
|  | Gas 2: | 45 psi | 45 psi |  |
|  | TOF Masses range: | 50 - 700 Da | 50 - 700 Da |  |
|  | Accumulation time: | 0.2 sec | 0.2 sec |  |
|  | DP: | 30 | 30 |  |
|  | CE: | 10 | 25 |  |
|  | CES: | - | 0 |  |
| Ions | Nicosulfuron (primary): m/z (411.1061 – 411.1108), from Experiment 1 Nicosulfuron-1 (confirmatory): m/z (182.0544 – 182.0562), from Experiment 2 Nicosulfuron-2 (confirmatory): m/z (213.0317 – 213.0339), from Experiment 2 |  |  |  |

Stock and working reference item solutions

10.1 mg of the reference item were weighed into a 20 mL volumetric flask and brought to volume with acetonitrile (stock solution at Nicosulfuron concentration 486.8 μg/mL; stored in freezer at temperature ≤ - 18 °C).

By dilution of the reference item stock solution described above with acetonitrile, intermediate reference item working solutions were prepared, as reported in the table below (nominal values).

| Nicosulfuron reference item parent solution concentration (μg/mL) | Taken Volume (mL) | Final Volume (mL) | Nicosulfuron reference item working solution concentration (μg/mL) | Solution type |
| --- | --- | --- | --- | --- |
| 486.8 | 0.2 | 10 | 9.74 | STM preparation |
| 9.74 | 0.1 | 10 | 0.0974 | STD preparation |
| 0.0974 | 0.05 | 1 | 0.00487 | Check of matrix effect |

The nicosulfuron standard solutions were freshly prepared on the analysis day.

A second stock solution was prepared by dissolving 10.1 mg of nicosulfuron reference item in 20 mL of acetonitrile (standard solution 486.9 µg/mL; 96.4 % purity taken into account). This stock solution was diluted with acetonitrile to prepare a working solution at 0.0974 µg/mL used to verify the stability of standard solutions after 88 days of storage at ≤ - 18 °C in the dark.

Test item solutions for fortification

1502 mg of the test item were weighed into a 10 mL volumetric flask and diluted to volume with demineralised water obtaining a test item stock solution containing 9393 µg/mL of nicosulfuron used to spike the untreated matrix at the second fortification level.

By dilution of the test item stock solution (0.1 mL in 200 mL of demineralised water) a solution containing 4.7 µg/mL of nicosulfuron active ingredient was obtained: this test item working solution was used to spike the untreated matrix at LOQ fortification level.

Fortification procedure

The fortification procedure is described in the table below.

| Matrix | Matrix weight(*) (g) | Fortification level (mg/kg)(*) | Spiking solution concentration (µg/mL) | Spiking solution volume (mL) |
| --- | --- | --- | --- | --- |
| soil | 4.5 g | 0.05 mg/kg (LOQ) | 4.7 | 0.05 |
|  |  | 2090 mg/kg (II Level) | 9393 | 1 |

(*) dry soil weight

The fortification levels of recovery samples cover all the range of Nicosulfuron concentrations in the analytical samples.

Reference item matrix matched solutions

Reference item matrix matched solutions were prepared by spiking aliquot of blank extract with small amount of reference item, according to the table below.

| Spiking standard solution concentration (ng/mL) | Taken Volume (µL) | Blank extract Volume (µL) | Total Final Volume (µL) | Calibration solution concentration (ng/mL) | Concentration in sample unit (mg/kgdry weight) | Solution type |
| --- | --- | --- | --- | --- | --- | --- |
| 9740 | 50 | 950 | 1000 | 487 | - | Intermediate |
| 487 | 200 | 800 | 1000 | 97.4 | 1.08 | Calibration |
| 487 | 100 | 900 | 1000 | 48.7 | 0.54 | Calibration |
| 487 | 50 | 950 | 1000 | 24.34 | 0.27 | Calibration |
| 97.4 | 100 | 900 | 1000 | 9.74 | 0.11 | Calibration |
| 97.4 | 50 | 950 | 1000 | 4.87 | 0.054 | Calibration |
| 9.74 | 100 | 900 | 1000 | 0.974 | 0.011 | Calibration |

The nicosulfuron matrix-matched standard solutions were freshly prepared on the analysis day.

Analytical procedure for soil samples analysis

The sample was let to warm up to room temperature.

The homogenised sample (6 ± 0.5 g) was weighted into a 50 mL centrifuge tube, recovery samples were fortified at this point.

Then 8.5 mL of demineralised water were added and the sample was manually shaken for a few seconds to homogenise.

Then 10 mL of acetonitrile were added, the sample was manually shaken for a minute. In recovery trials, the 10 mL volume comprises the volume of the fortification solution added. After this step, the content of a sachet of Quechers was added to the sample.

The tube was shaken vigorously by hand for 1 minute and then centrifuged at 4000 rpm for five minutes.

1 mL of the surnatant layer was diluted with a solution of ammonium carbonate 0.1M in water in a 5 mL flask (final volume 50 mL).

The final extract was filtered with PTFE filter, porosity 0.20 µm and analysed by HPLC-TOF/MS/MS.

Quantification was conducted using an external standard calibration curve obtained by linear regression of matrix matched calibration standards injected throughout the run in the range 1 – 100 ng/mL.

Sample extracts with analyte concentration exceeding 80 ng/mL were opportunely diluted with blank extract in order to fall within the ± 20% of the calibration range.

Validation of method

Linearity and instrumental precision

The linearity range for nicosulfuron was found between 1 - 100 ng/mL corresponding to 0.011 to 1.11 mg/kg of nicosulfuron in dry soil sample. The correlation coefficient of the weighed linear (1/x) multipoint external matrix matched standard calibration curves was found ≥ 0.996 for both ion transitions in all the analytical sequences performed.

The linearity range comprised the concentration range from 30% of the LOQ to at least 20% above the highest measured concentration.

The suitability of the calibration lines was assessed using the residuals di that describes the vertical distance of measured values from the regression curve according to:

di = yi - yyi

where:

yi is the measured value i;

yyi is the estimated value which corresponds to yi and is derived from the calibration function.

The regression residuals were presented in residual plots and visual inspections were applied to decide if di were randomly distributed and hence linear calibration is demonstrated: no trend was visible by plotting the residuals vs the concentration.

Blank and selectivity

Three independent analyses of the blank sample were performed: no significant interference exceeding 30% of the limit of quantification were found at the retention time of nicosulfuron for both the monitored ions.

Therefore, Nicosulfuron can be regarded as not detectable in untreated soil sample used in fortification trials (< 30% of LOQ).

The retention time of the reference item matched the retention time of the analyte in extracts from fortified samples.

Based on the analysis of the blank matrix, the method was confirmed to be selective for the analysis of Nicosulfuron in soil matrix, without significant interferences above 30% of LOQ.

Specificity

Nicosulfuron was analysed by UHPLC-TOF-MS/MS highly specific detection system; three ions were simultaneously acquired: the Parent Ion 411.11 (target) for quantification purpose and the fragment Ions 182.05 and 213.03 (qualifiers) for confirmation.

The mass spectrum of the analyte was acquired in the range 50-700 m/z.

Recoveries

The analytical method was validated by recovery trials: a known quantity of the test item was added to the control sample and the percentage recovery was calculated.

The recoveries were performed by fortifying the untreated blank at two levels.

The LOQ level was set at nicosulfuron concentration of 0.05 mg/kg dry weight (lower than the minimum found nicosulfuron content in samples, while the second level was at 2090 mg/kg dry weight (higher than the maximum expected concentration in the samples), in order to cover with the method validation all the range of nicosulfuron concentrations in the analytical samples; five replicated analyses were carried out for each fortification level.

The background content in the control sample used for fortification experiments was not detectable. In these recovery samples the nicosulfuron content was determined as reported in table below.

| Fortification level (mg/kg DW) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.05 (a) (LOQ) | 104.9 | 106.0 | 6.2 | 96.3 ± 16.5 |
|  | 116.7 |  |  |  |
|  | 107.2 |  |  |  |
|  | 99.9 |  |  |  |
|  | 101.4 |  |  |  |
| 2090 (b) (II level) | 76.4 | 86.6 | 19.7 |  |
|  | 61.9 |  |  |  |
|  | 103.3 |  |  |  |
|  | 94.6 |  |  |  |
|  | 96.9 |  |  |  |

DW = Dry Weight

For each fortification level the mean recovery was in the range 70 – 120 % and the precision (RSD, relative standard deviation) ≤ 20 %, in compliance with the requirements of guideline SANTE/2020/12830 rev.1.

Accuracy

The accuracy of the analysis method for Nicosulfuron in soil, defined as mean recovery ± relative standard deviation, is 96.3 ± 16.5.

Repeatability

The repeatability, defined as the % RSD (Relative Standard Deviation) at each fortification level, and the overall RSD is reported in the table below.

| Fortification level (mg/kg DW) | RSD % (n = 5) | Overall RSD % (n = 10) |
| --- | --- | --- |
| 0.05 LOQ | 6.2 | 16.5 |
| 2090 (II level) | 19.7 |  |

DW = Dry Weight

Matrix effects

To check possible signal enhancement or suppression effect in the LC-TOF-MS/MS analysis, the control sample extract fortified to achieve the nominal concentration of nicosulfuron at 5 ng/mL (nearest to the nominal concentration for the LOQ level) was compared to nicosulfuron in solvent at the same concentration. The results are summarized in the table below.

| Ions | Matrix response over solvent response % | Matrix effects % |
| --- | --- | --- |
| 411.1 (Target) | 126 | +26 |
| 182.05 (Qualifier) | 119 | +19 |
| 213.03 (Qualifier) | 120 | +20 |

Matrix effects for nicosulfuron in soil matrix were found significant (≥ 20%) for both target an qualifier ions, therefore matrix matched calibration standards were used in the quantification of samples.

Confirmation

The confirmation of the analyte identity is simultaneous to the primary detection by the acquisition of the additional transition.

The recovery data and the precision data for the additional transition are reported in tables below.

Validation data for Nicosulfuron (confirmatory ion 185.05)

| Fortification level (mg/kg DW) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.05 (LOQ) | 117.4 | 110.3 | 11.7 | 97.9 ± 17.5 |
|  | 123.7 |  |  |  |
|  | 116.3 |  |  |  |
|  | 92.5 |  |  |  |
|  | 101.5 |  |  |  |
| 2090 (II level) | 82.4 | 85.4 | 12.1 |  |
|  | 75.4 |  |  |  |
|  | 102.7 |  |  |  |
|  | 85.7 |  |  |  |
|  | 81.0 |  |  |  |

DW = Dry Weight

Validation data for Nicosulfuron (confirmatory ion 213.03)

| Fortification level (mg/kg DW) | Recoveries (%) | Mean Recovery (%) | RSD (%) | Overall mean recovery ± RSD % |
| --- | --- | --- | --- | --- |
| 0.05 (LOQ) | 119.6 | 106.9 | 13.7 | 96.9 ± 16.4 |
|  | 121.6 |  |  |  |
|  | 106.2 |  |  |  |
|  | 101.2 |  |  |  |
|  | 85.7 |  |  |  |
| 2090 (II level) | 74.3 | 86.9 | 11.9 |  |
|  | 78.6 |  |  |  |
|  | 99.1 |  |  |  |
|  | 93.6 |  |  |  |
|  | 89.0 |  |  |  |

DW = Dry Weight

Also, for the confirmatory ions, the mean recovery was in the range 70 – 120 % and the precision (RSD, relative standard deviation) ≤ 20 %, in compliance with the requirements of guideline SANTE/2020/12830 rev.1.

Stability of final extracts and reference item solutions

During the analytical sequences the injection of intermediate standard solutions and QC samples (recoveries) were done to check the calibration, the accuracy of the method and the samples stability during the course of the analysis.

The final extracts were analysed within 24 hours form extraction. Moreover, the stability of the extracts during the analysis was proven by the acceptability of recoveries performed concurrently with the samples analysis.

The nicosulfuron reference item stock solution was proven to be stable for 88 days after preparation at ≤ - 18 °C: the means from at least 5 replicate measurements for a fresh solution compared to a stored one (at ≤ - 18 °C in the dark) did not differ by more than 10%, according to SANTE/2020/12830, Rev.1.

Limit of quantification (LOQ) and limit of Detection (LOD)

The limit of quantification (LOQ) is defined as the lowest concentration tested at which an unambiguous identification of the analyte can be proven and at which an acceptable mean recovery with an acceptable relative standard deviation (RSD) is obtained.

The LOQ for nicosulfuron in soil was assessed in this study at 0.05 mg/kg (referred to dry soil).

The limit of detection is the lowest amount that can be detected but not necessarily quantitated as an exact value.

For the analysis of nicosulfuron in soil the LOD is 0.011 mg/kg (referred to dry soil). This value, calculated from the nicosulfuron concentration corresponding to the lowest calibration point, is below 30% of LOQ.

Conclusion

The data presented above confirm that the validated analytical method provides a specific, reliable, accurate and precise procedure for the determination of nicosulfuron active ingredient in soil samples in the range 0.05 – 2090 mg/kg DW.

As the analysis of the samples was performed concurrently with the recovery tests, the reliability of the found values was demonstrated, according to guideline SANTE/2020/12830 rev. 1 of 24 February 2021. Additional requirements as stated in the superseding guidance document SANTE/2020/12830 rev. 2 of 14 February 2023 were considered and fulfilled.

        - HPLC - DAD (in sucrose solution)

          - Method validation

|  | Comments of Evaluator: | Method is accepted. |  |  |
| --- | --- | --- | --- | --- |
| Reference: |  |  | KCP 5.2/04 (filled as KCP 10.3.1.1.1/02) |  |
| Report |  |  | NIC-HER 060 OD Bumblebees (Bombus spp.), Acute Oral Toxicity Test, Knapik M., 2021, Study code: B-43-21 |  |
| Guideline(s): |  |  | SANTE/2020/12830, Rev. 1 and Standard Operating Procedure SOP/C/9. |  |
| Deviations: |  |  | No |  |
| GLP: |  |  | Yes |  |
| Acceptability: |  |  | Yes |  |

Principle of the method

The analytical method was developed for the determination of nicosulfuron in sucrose solution. The range of linearity of the analytical graphs, the regression residual (di), selectivity and specificity, precision, matrix effect, accuracy, stability stock solution, limit of quantification and detection were determined.

Materials and methods

Chromatography system:	High Performance Liquid Chromatography (HPLC) Chromatograph Shimadzu, Prominence-i (Shimadzu Corporation Japan)

Conditions

Column:			Luna 5μm C18(2) 100Å, l = 250 mm, ɸ = 4,6 mm

Wave lenght:			240 nm

Injection volume:		10 μL

Oven temperature: 		35°C

Mobile phase:			acetonitrile for HPLC : 0.05% ortho-phosphoric acid (50:50, v/v)

Flow rate:			1 mL/min

Detection system:		Diode Array Detector

Validation

Selectivity and specificity

The analytical method specificity was estimated on the basic of the analysis of the chromatograms obtained for the control matrix, and fortified samples of matrix. Considering the results of the analysis, no signal of detected substance was overlapping with matrix signal of the control samples in the experimental conditions. Therefore, the specificity of the method was demonstrated. Furthermore, any interference is directly apparent and would be observed in the chromatogram of the matrix control sample.

Linearity

Working solutions of nicosulfuron at the concentrations of 1, 2, 5, 10, 20, 50 and 100 μg/mL were injected successively to the chromatographic column and the chromatograms were recorded. The range of linearity of the analytical graph is from 1 μg/mL to 100 μg/mL. The range of calibration curve of nicosulfuron is equivalent to range from 10 mg nicosulfuron/kg to 1000 mg nicosulfuron/kg in sucrose solution.

The equations of the calibration line were presented as the linear equation:

y = ax + b

where:

a - slope

b - intercept

The linear coefficient r2 must be higher than 0.99. Range of the linear was given in μg/mL equivalent to mg/L.

| Analyte | Slope | Intercept | Coefficient |
| --- | --- | --- | --- |
| nicosulfuron | 4.82459e-005 | -0.171464 | 0.9986175 |

Linear weighted calibration (1/x weighting) is used. The suitability of the chosen function was demonstrated as the regression residual (di).

Criterion of acceptance r2 ≥ 0.99 was fulfilled.

Accuracy

The accuracy of the method is reported as mean recovery ± relative standard deviation. Recovery data was reported for 2 fortification levels of nicosulfuron appropriate to level corresponding with LOQ and 10 x LOQ. Mean recoveries ± relative standard deviation (RSD) for each level is in the range 70-120%.

| Matrix | Detected substance | Fortification level [mg/kg] | Number of replicates | Mean recovery [%] | RSD [%] |
| --- | --- | --- | --- | --- | --- |
| Sucrose solution | nicosulfuron | 50 | 5 | 94.7 | 0.1 |
|  |  | 500 | 5 | 97.3 | 0.2 |

In order to study the recovery level, the solution of the detected substance was added to non-treated water and then analyzed.

Sample preparation for the chromatographic analysis

First, 1 g sucrose sample was weighted into a volumetric flask with a capacity of 10 mL and added 6 mL of deionized water (v/v). Next, the volume was made up to 10 mL with acetonitrile for HPLC. The eluate was diluted with mixture acetonitrile for HPLC and deionized water (50:50; v/v), if necessary. An aliquot of the final volume was transferred into a HPLC vial for further quantification using HPLC-DAD.

Matrix Effect

Matrix effect for each method was checked during the method validation. Assessment of matrix effects was performed by comparing the standard preparing in solution to standard preparing in control matrix at appropriate concentration. The matrix effects and concentrations are presented in table below.

| Matrix | Detected substance | Concentration [mg/l] | Matrix effect [%] |
| --- | --- | --- | --- |
| Sucrose solution | nicosulfuron | 5 | 4.0 |

Matrix effect, expressed in % enhancement or suppression was evaluated according to the following equation:

Matrix effects [%] = 100 × peak area (matrix) / peak area (solvent) – 100

The matrix effect is not exceed ± 20 % in of the method.

Precision

Precision of a particular method is defined as its repeatability (RSD – relative standard deviation [%]). The repeatability for detected substance analysed in the sucrose solution is from 0.1% to 0.2%.

The precision is 0.1% at level 50 mg nicosulfuron/kg sucrose solution, 0.2% at level 500 mg nicosulfuron/kg sucrose solution.

The RSD for both method is ≤ 20% per each level.

| Nominal concentration [mg/kg] | Determined concentration of nicosulfuron in replicates [mg/kg] |  |  |  |  | Average [mg/kg] | Recovery [%] | SD [mg/kg] | RDS [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |  |  |  |  |
| Control | 0.000 | 0.000 | - | - | - | 0.000 | - | 0.000 | - |
| 50 | 47.358 | 47.297 | 47.429 | 47.278 | 47.364 | 47.345 | 94.7 | 0.060 | 0.1 |
| 500 | 484.937 | 486.896 | 487.269 | 487.532 | 485.513 | 486.429 | 97.3 | 1.141 | 0.2 |

LOQ = 50 mg/kg

LOD = 10 mg/kg

Limit of quantification (LOQ) and limit of detection (LOD)

Limit of Quantification was estimated as the lowest concentration of a detected substance at which an acceptable mean recovery is obtained (normally 70 – 120% with a relative standard deviation of preferably ≤ 20%).

The LOQ is 50 mg nicosulfuron/kg sucrose solution and equivalent to the calibration level at concentration 5 μg nicosulfuron/mL.

The limit of detection (LOD) is defined as the lowest detectable concentration or amount of an analyte in a sample. The LOD is 10 mg nicosulfuron/kg sucrose solution and equivalent to the lowest calibration standard i.e., 1 μg nicosulfuron/mL.

| Detected substance | LOQ (limit of quantification) | LOD (limit of detection) |
| --- | --- | --- |
| nicosulfuron | 50 mg/kg | 10 mg/kg |

Stock solution stability

The stability of stock solution was tested at concentrations 1000 mg nicosulfuron/L. The results for stability were obtained after 0, 5 and 21 days of storage at cool temperature i.e., from 2°C to 8°C. Compared to the mean recoveries measured at 0-day, significant decline was not observed after 21 days. The mean recovery for each of the solutions do not differ by more than 10%. The results are presented in table below.

| Days of storage | Average concentration [mg/l] | RSD [%] | Recovery [%] | Decrease in relation to the initial concentration [%] |
| --- | --- | --- | --- | --- |
| 0 | 949.0 | 0.1 | 94.9 | - |
| 5 | 948.2 | 0.1 | 94.8 | 0.1 |
| 21 | 948.5 | 0.1 | 94.9 | 0.1 |

Conclusion

The method was fully validated. Results of the validation of analytical method was confirmed that this method is suitable for analysis the content of the test item in sucrose solution.

        - HPLC - DAD (in water)

          - Method validation

|  | Comments of Evaluator: | Method is accepted. |  |  |
| --- | --- | --- | --- | --- |
| Reference: |  |  | KCP 5.2/05 (filled as KCP 10.3.1.1.2/02) |  |
| Report |  |  | NIC-HER 060 OD Bumblebees (Bombus spp.), Acute Contact Toxicity Test, Knapik M., 2021, Study code: B-44-21 |  |
| Guideline(s): |  |  | SANTE/2020/12830, Rev. 1 and Standard Operating Procedure SOP/C/9. |  |
| Deviations: |  |  | No |  |
| GLP: |  |  | Yes |  |
| Acceptability: |  |  | Yes |  |

Principle of the method

The analytical method was developed for the determination of nicosulfuron in water. The range of linearity of the analytical graphs, the regression residual (di), selectivity and specificity, precision, matrix effect, accuracy, stability stock solution, limit of quantification and detection were determined.

Materials and methods

Chromatography system:	High Performance Liquid Chromatography (HPLC) Chromatograph Shimadzu, Prominence-i (Shimadzu Corporation Japan)

Conditions

Column:			Luna 5μm C18(2) 100Å, l = 250 mm, ɸ = 4,6 mm

Wave lenght:			240 nm

Injection volume:		10 μL

Oven temperature: 		35°C

Mobile phase:			acetonitrile for HPLC : 0.05% ortho-phosphoric acid (50:50, v/v)

Flow rate:			1 mL/min

Detection system:		Diode Array Detector

Validation

Selectivity and specificity

The analytical method specificity was estimated on the basic of the analysis of the chromatograms obtained for the control matrix, and fortified samples of matrix. Considering the results of the analysis, no signal of detected substance was overlapping with matrix signal of the control samples in the experimental conditions. Therefore, the specificity of the method was demonstrated. Furthermore, any interference is directly apparent and would be observed in the chromatogram of the matrix control sample.

Linearity

Working solutions of nicosulfuron at the concentrations of 1, 2, 5, 10, 20, 50 and 100 μg/mL were injected successively to the chromatographic column and the chromatograms were recorded. The range of linearity of the analytical graph is from 1 μg/mL to 100 μg/mL. The range of calibration curve of nicosulfuron is equivalent to range from 2 mg nicosulfuron/kg to 200 mg nicosulfuron/kg in water.

The equations of the calibration line were presented as the linear equation:

y = ax + b

where:

a - slope

b - intercept

The linear coefficient r2 must be higher than 0.99. Range of the linear was given in μg/mL equivalent to mg/L.

| Analyte | Slope | Intercept | Coefficient |
| --- | --- | --- | --- |
| nicosulfuron | 4.82459e-005 | -0.171464 | 0.9986175 |

Linear weighted calibration (1/x weighting) is used. The suitability of the chosen function was demonstrated as the regression residual (di).

Criterion of acceptance r2 ≥ 0.99 was fulfilled.

Accuracy

The accuracy of the method is reported as mean recovery ± relative standard deviation. Recovery data was reported for 2 fortification levels of nicosulfuron appropriate to level corresponding with LOQ and 10 x LOQ. Mean recoveries ± relative standard deviation (RSD) for each level is in the range 70-120%.

| Matrix | Detected substance | Fortification level [mg/kg] | Number of replicates | Mean recovery [%] | RSD [%] |
| --- | --- | --- | --- | --- | --- |
| Water | nicosulfuron | 10 | 5 | 95.9 | 0.4 |
|  |  | 100 | 5 | 100.8 | 0.8 |

In order to study the recovery level, the solution of the detected substance was added to non-treated water and then analyzed.

Sample preparation for the chromatographic analysis

Acetonitrile for HPLC a volume of 5 mL of was added to sample a volume of 5 mL and mixed. The sample were diluted mixture of acetonitrile for HPLC and deionized water (50:50; v/v) if necessary. An aliquot of the final volume was transferred into a HPLC vial for further quantification using HPLC-DAD.

Matrix Effect

Matrix effect for each method was checked during the method validation. Assessment of matrix effects was performed by comparing the standard preparing in solution to standard preparing in control matrix at appropriate concentration. The matrix effects and concentrations are presented in table below.

| Matrix | Detected substance | Concentration [mg/l] | Matrix effect [%] |
| --- | --- | --- | --- |
| Water | nicosulfuron | 5 | 5.4 |

Matrix effect, expressed in % enhancement or suppression was evaluated according to the following equation:

Matrix effects [%] = 100 × peak area (matrix) / peak area (solvent) – 100

The matrix effect is not exceed ± 20 % in of the method.

Precision

Precision of a particular method is defined as its repeatability (RSD – relative standard deviation [%]). The repeatability for detected substance analysed in the sucrose solution is from 0.4% to 0.8%.

The precision is 0.4% at level 10 mg nicosulfuron/kg sucrose solution, 0.8% at level 100 mg nicosulfuron/kg water.

The RSD for both method is ≤ 20% per each level.

| Nominal concentration [mg/kg] | Determined concentration of nicosulfuron in replicates [mg/kg] |  |  |  |  | Average [mg/kg] | Recovery [%] | SD [mg/kg] | RDS [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |  |  |  |  |
| Control | 0.000 | 0.000 | - | - | - | 0.000 | - | 0.000 | - |
| 10 | 9.624 | 9.602 | 9.616 | 9.575 | 9.522 | 9.588 | 95.9 | 0.041 | 0.4 |
| 100 | 102.231 | 100.646 | 100.088 | 100.549 | 100.693 | 100.841 | 100.8 | 0.813 | 0.8 |

LOQ = 10 mg/kg

LOD = 2 mg/kg

Limit of quantification (LOQ) and limit of detection (LOD)

Limit of Quantification was estimated as the lowest concentration of a detected substance at which an acceptable mean recovery is obtained (normally 70 – 120% with a relative standard deviation of preferably ≤ 20%).

The LOQ is 10 mg nicosulfuron/kg sucrose solution and equivalent to the calibration level at concentration 5 μg nicosulfuron/mL.

The limit of detection (LOD) is defined as the lowest detectable concentration or amount of an analyte in a sample. The LOD is 2 mg nicosulfuron/kg sucrose solution and equivalent to the lowest calibration standard i.e., 1 μg nicosulfuron/mL.

| Detected substance | LOQ (limit of quantification) | LOD (limit of detection) |
| --- | --- | --- |
| nicosulfuron | 10 mg/kg | 2 mg/kg |

Stock solution stability

The stability of stock solution was tested at concentrations 1000 mg nicosulfuron/L. The results for stability were obtained after 0, 5 and 21 days of storage at cool temperature i.e., from 2°C to 8°C. Compared to the mean recoveries measured at 0-day, significant decline was not observed after 21 days. The mean recovery for each of the solutions do not differ by more than 10%. The results are presented in Table below.

| Days of storage | Average concentration [mg/l] | RSD [%] | Recovery [%] | Decrease in relation to the initial concentration [%] |
| --- | --- | --- | --- | --- |
| 0 | 949.0 | 0.1 | 94.9 | - |
| 5 | 948.2 | 0.1 | 94.8 | 0.1 |
| 21 | 948.5 | 0.1 | 94.9 | 0.1 |

Conclusion

The method was fully validated. Results of the validation of analytical method was confirmed that this method is suitable for analysis the content of the test item in water.

        - HPLC-MS/MS (in aqueous solutions)

          - Method validation

|  | Comments of Evaluator: | Method is accepted. |  |  |
| --- | --- | --- | --- | --- |
| Reference: |  |  | KCP 5.2/06 |  |
| Report |  |  | Validation of the analytical method for the determination of Nicosulfuron content in aqueous solutions coming from ecotoxicological tests with NIC-HER 060 OD, Tediosi E., 2023, Study code: CH – 0043-2023 |  |
| Guidelines: |  |  | SANTE/2020/12830, rev.1. |  |
| Deviations: |  |  | No |  |
| GLP: |  |  | Yes |  |
| Acceptability: |  |  | Yes |  |

Principle of the method

The aim of this study was to validate the analytical method for the determination of nicosulfuron content in aqueous solutions coming from Lemna sp. Growth inhibition test.

The analytical method for nicosulfuron in water was fully validated according to the guideline SANTE/2020/12830 rev.1, of 24 February 2021, by calibration (linearity), selectivity, specificity, blank samples analysis, recoveries, accuracy, repeatability, matrix effect, stability, limit of quantification (LOQ) and limit of detection (LOD).

The method has been validated by the analysis of control and fortified samples.

The determination of nicosulfuron residues is performed by HPLC using an external standard and MS triple quadrupole detector.

Transitions (MS/MS):

- 411.1 → 182.1 (quantifier)

- 411.1 → 213.0 (qualifier 1)

- 411.1 → 106.0 (qualifier 2)

Its quantification is achieved using the calibration curve obtained by plotting standard solutions concentrations and the corresponding peak area.

Materials and methods

Equipment

- HPLC/MS-QQQ, Agilent mod. 1260, equipped with quaternary pump, autosampler coupled with an ESI interfaced Triple Quadrupole Mass Detector, mod. 6420, managed by Mass Hunter software.

- Analytical balance

- Refrigerator

- Freezer

- Aspirated Safety Storage Cabinet

- Water purification system

- Usual and volumetric laboratory glassware

- Volumetric glassware class A

- HPLC column, Luna Omega Polar C18, 3 μm, 100 x 3 mm i.d.

Reagents and materials

- Acetonitrile, HPLC grade

- Water, HPLC grade

- Formic Acid

- Ammonium formate

Instrumental parameters

| System | HPLC/MS-QQQ, Agilent mod. 1260, equipped with quaternary pump, autosampler coupled with an ESI interfaced Triple Quadrupole Mass Detector, mod. 6420, managed by Mass Hunter software |  |  |
| --- | --- | --- | --- |
| Column | HPLC column, Luna Omega Polar C18, 3 µm, 100 x 3 mm i.d.; Internal code LCN 406 (or equivalent) |  |  |
| Detector | MS Triple quadrupole (Scan in MRM mode) |  |  |
| Column temperature | Not Controlled |  |  |
| Eluent A | Water / formic acid 0.1% / ammonium formate 10 mM |  |  |
| Eluent B | Acetonitrile |  |  |
| Eluent flow | 0.7 mL/min |  |  |
| Elution mode | Gradient condition |  |  |
| Mixture | % A | % B | Time (min.) |
|  | 80 | 20 | 0 |
|  | 20 | 80 | 8 |
|  | 20 | 80 | 9 |
|  | 80 | 20 | 16 |
| Volume of injection | 10 μL |  |  |
| Retention time | Approximately 4.3 minutes |  |  |
| Total analysis time | 16 minutes + 3 minutes post time |  |  |
| Mass scan parameters |  |  |  |
| Ion mode | ESI, positive polarity |  |  |
| Scan type | MRM |  |  |
| Electro multiplier voltage (V) | 200 |  |  |
| Dry gas temperature (°C) | 350 |  |  |
| Dry gas flow (L/min) | 10 |  |  |
| Nebuliser (psi) | 40 |  |  |
| Capillary current (V) | 4000 |  |  |
| Precursor ion (m/z) - (fragmentor, V) | 411.1 - (100) |  |  |
| Product ions (m/z) - (Collision Energy, V) | quantifier 182.1 – (15) qualifiers 213.0 – (10) 106.0 – (35) |  |  |
| Dwell time (msec) | 200 |  |  |

Validation of method

Linearity

The ability of a detection system, within a defined range, to produce an acceptable linear correlation between the test results and the concentration of analyte in the sample.

The analytical calibration was performed by injecting, in duplicate determination, five matrix matched standard (MSS) solutions prepared in blank matrix (Lemna growth medium).

Linear regression analysis was performed using the least squares method; the correlation coefficient was calculated using regression analysis.

| Linearity / Calibration | r ≥ 0.99 |
| --- | --- |
| result | Quantifier transition m/z 411.1 → m/z 182.1 |
|  | Range 0.05 – 4.85 µg/L, r = 0.99987 |
|  | Qualifier transition 1 m/z 411.1 → m/z 213.0 |
|  | Range 0.05 – 4.85 µg/L, r = 0.99974 |
|  | Qualifier transition 2 m/z 411.1 → m/z 106.0 |
|  | Range 0.05 – 4.85 µg/L, r = 0.99972 |

Selectivity and specificity

The ability of a method to distinguish between the analyte being measured and other substances, based upon sufficient characteristics of the analyte to make the results completely specific to the analyte, irrespective of the characteristics of other materials present.

The nicosulfuron determination was conducted by HPLC-MS/MS in MRM mode, monitoring three MS/MS ion mass transitions 411.1 → 182.1 for quantification, 411.1 → 213.0 and 411.1 → 106.0 for qualitative purpose (confirmation).

The SANTE/2020/12830, Rev.1 guideline requires that the specificity of instrumental response to the substance presence is demonstrated.

The blank values (untreated samples) should not exceed 30% of the LOQ.

The specificity test was conducted injecting the samples (solvent wash (acetonitrile), blank matrix sample (Lemna growth medium), standard at lowest calibration level in matrix (MSS1), a fortified Lemna growth medium sample at lowest level (LOQ)) and comparing the signals to check for the possible interference by other analytes or by the matrix.

A comparison of the signal obtained for the different solutions showed that, following the operating conditions recommended in the analytical method, no interfering signals were detected in the untreated matrix samples exceeding 30% of the LOQ, suggested by the guideline requirement.

Blank matrix sample response and LOQ response were compared and the Ratio % (Blank / LOQ) is reported in the table below:

| Injected solutions | Area | % Blank vs LOQ |
| --- | --- | --- |
| Blank sample (Lemna growth medium) | 0 | 0% |
| A fortified Lemna growth medium sample at lowest level (LOQ) | 259 |  |

The method was found to be selective to determinate nicosulfuron in the tested matrix, as the ratio between blank matrix sample intensity and LOQ intensity (BLK / LOQ %) did not exceed 30%.

Recovery (Accuracy) and Repeatability (Precision)

Recovery is the amount measured as a percentage of the amount of active substance or relevant metabolite originally added to a sample of the appropriate matrix which contains either no detectable level of the analyte or a known detectable level (not higher than 30% of the amount added).

Repeatability is defined as the closeness of agreement between mutually independent test results obtained with the same method on identical test material in the same laboratory by the same operator using the same equipment within short intervals of time.

The recovery and the repeatability (as precision, % RSD) of the analytical method were determined using freshly fortified control samples of Lemna growth medium.

Fortification levels were chosen at the LOQ and at the higher concentration tested in ecotoxicological studies.

The samples were fortified at following nominal concentrations:

- Low level: 2.4 μg test item/L (0.15 μg nicosulfuron/L)

- High level: 0.5 mg test item/L (31 μg nicosulfuron/L)

| Repeatability (Precision) Recovery (Accuracy) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Fortification level | Low (µg/L) |  | High (µg/L) |
|  | Nominal | 0.15 |  | 31 |
|  | Corrected* | 0.15 |  | 31.56 |
|  | Mean found | 0.16 |  | 31.97 |
|  | Recovery | Mean between 70% – 120% |  |  |
|  | result | 108.8 |  | 101.3 |
|  | Repeatability | as precision RSD % ≤ 20% |  |  |
|  | result | 2 | 2 |  |

(*) corrected for exact test item weighed amount during fortification solution preparation.

Matrix effects

Is the capacity of the matrix to modify the intensity of the instrumental response of the analyte, is expressed as % of enhancement or suppression of the signal.

The assessment of matrix effects was performed by comparing the slope of the curve obtained with three working standard solutions (WSS1 – WSS3 – WSS5), prepared in solvent (acetonitrile), to the slope obtained with three matrix-matched standard solutions (MSS1 – MSS3 – MSS5), prepared in blank matrix (Lemna growth medium).

Matrix effects, expressed in % enhancement or suppression of signal, are considered significant if they exceed ±20%.

The matrix effects were calculated comparing the slope of the curves obtained in matrix and the slope of the curve obtained in solvent.

The result obtained in Lemna growth medium expressed in % enhancement of signal is reported in table below:

| Slope of matrix (Lemna growth medium) | 1064 |
| --- | --- |
| Slope of solvent (acetonitrile) | 988 |
| Matrix Effects % (100 * Slope (matrix) / Slope (solvent) – 100) | 7.6% |

Matrix effects in Lemna growth medium, expressed in % enhancement of signal, resulted not significant (lower than 20%), in any case matrix matched standard solutions were used to quantify samples, in order to fully compensate the small difference observed.

Confirmation

The methods required to demonstrate the selectivity of the primary method for all representative sample matrices.

The nicosulfuron determination was conducted by HPLC-MS/MS in MRM mode, monitoring three MS/MS ion mass transitions 411.1 → 182.1 for quantification, 411.1 → 213.0 and 411.1 → 106.0 for qualitative purpose (confirmation).

Final extract and standard stability

Extract stability is the stability of the analyte in the final extracts is proven if the recoveries in the fortified samples are within the acceptable range of 70 - 120%, measured against freshly prepared standards.

Standard stability is the stability of an existing standard (old) was checked by preparing a new standard solution (fresh) and comparing the responses. The means from 5 replicate measurements for each of the two solutions should not differ by more than 10%.

All samples were analysed within 24 hours from preparation; therefore, no experimental demonstration of stability was required.

The analytical standards were always freshly prepared.

Limit of quantification (LOQ) and Limit of detection (LOD)

The limit of quantification (LOQ) is defined as the lowest concentration tested, at which an acceptable mean recovery (70 - 120%), with an acceptable RSD (≤ 20 %), is obtained.

The limit of quantification (LOQ) was prepared at 0.15 μg/L in Lemna growth medium (2.4 μg test item/L).

The limit of detection (LOD) is defined as the minimum concentration which can be detected, but not quantified with sufficient precision. It will be the lowest concentration standard solution.

The limit of detection (LOD) was a final injected solution of nicosulfuron equal to 0.05 μg/L.

| LOD | Lowest calibration level |
| --- | --- |
| result | 0.05 µg/L |
| LOQ | Lowest fortified level |
| result | 0.15 µg/L |

Conclusion

Regarding obtained results, the analytical method described above can be considered fully validated according to the SANTE/2020/12830 rev. 1 of 24 February 2021 and it can be considered suitable for the determination of nicosulfuron residues in the ecotoxicological matrix investigated (Lemna growth medium).