FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: 054-01-05 Product name(s): Meso-Iodo OD-Life Chemical active substances: Mesosulfuron-methyl, 10 g/L Iodosulfuron-methyl-sodium, 2 g/L Safener: mefenpyr-diethyl, 30 g/L

Central Zone Zonal Rapporteur Member State: Poland Concerned Member State: Germany

CORE ASSESSMENT (Authorisation)

Applicant: Life Scientific Ltd Submission date: Q1 2023 MS Finalisation date: 10.2023; 04.2024

Version history

| When | What |
| --- | --- |
| Q1 2023 | Submission date |
| 10.2023 | zRMS assessment |
| 04.2024 | FRR after commenting period |

Table of Contents

1	Section 1: Identity of the plant protection product	5

1.1	Applicant (KCP 1.1)	5

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	5

1.2.1	Producer(s) of the preparation	5

1.2.2	Producer(s) of the active substance(s)	5

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	Iodosulfuron-methyl-sodium	6

1.2.3.2	Mesosulfuron-methyl	6

1.2.3.3	Mefenpyr-diethyl (safener)	6

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	6

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	6

1.4.1	Composition of the plant protection product (KCP 1.4.1)	6

1.4.2	Information on the active substance(s) (KCP 1.4.2)	7

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	8

1.5	Type and code of the plant protection product (KCP 1.5)	8

1.6	Function (KCP 1.6)	8

2	Section 2: Physical, chemical and technical properties of the plant protection product	9

3	Section 3 is presented as a separate document	23

4	Section 4: Further information on the plant protection product	23

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	23

Appendix 1	Lists of data considered in support of the evaluation	26

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	30

A 2.1	Mesosulfuron-methyl	30

A 2.2	Iodosulfuron-methyl-sodium	30

- zRMS comments:

- Submitted data are sufficient for evaluation. Based on the accelerated storage stability results, expiration date of 2 years is acceptable and proposed.

Packaging: proposed packaging type HDPE/PA and HDPE/EVOH– accepted. See section 4.1 for details.

Introduction

This application is being submitted to support the registration of Meso-Iodo OD-Life, an oil dispersion (OD) formulation containing 10 g/L mesosulfuron-methyl, 2 g/L iodosulfuron-methyl-sodium and 30 g/L mefenpyr-diethyl in Poland under Regulation (EC) 1107/2009. As the applicant also intends to register the product in Germany, they have been listed as a ‘concerned’ Member State (cMS). This evaluation is required subsequent to the inclusion of mesosulfuron-methyl on Annex I of Directive 91/414/EEC under Commission Directive 2003/119/EC on 1st April 2004. Mesosulfuron-methyl was renewed under Implementing Regulation (EU) 2017/755 on 1st July 2017. Iodosulfuron-methyl-sodium was included on Annex I of Directive 91/414/EEC under Commission Directive 2003/84/EC on 1st January 2004. Iodosulfuron-methyl-sodium was renewed under Implementing Regulation (EU) 2017/407 on 1st April 2017. Meso-Iodo OD-Life will be referred to as product 054-01-05 for the remainder of this document.

Product 054-01-05 is a professional use herbicide formulated as an oil dispersion containing 10 g/L mesosulfuron-methyl, 2 g/L iodosulfuron-methyl-sodium and 30 g/L mefenpyr-diethyl. The product has not previously been evaluated in Poland according to Uniform Principles.

The sources of mesosulfuron-methyl (source 1 (20151195 PWSG), source 2 (20190977 PWSG)) and iodosulfuron-methyl-sodium (20150953 PWSG) have previously been assessed by the CTGB in the Netherlands and deemed technically equivalent to the Annex I reference source. The source of iodosulfuron-methyl-sodium was later assessed by the Central Institute for Supervising and Testing in Agriculture (UKZUZ 038555/2022) in the Czech Republic following Annex I renewal, where it was concluded that the source still met the specification listed in the renewal regulation. The results of each of these assessments were sent to Member States for commenting. Details of the evaluations are available on CIRCA BC.

zRMS comment: according to the Applicant’s statement, source 1 (20151195 PWSG) of mesosulfuron-methyl has not been evaluated post-renewal and therefore this source cannot be included in the authorisation – the source has been crossed out. The second source (20190977 PWSG) is still valid. Please refer to Part C for details.

In support of the proposed formulation, the applicant is providing data showing satisfactory chemical and physical properties for the product and their retention after accelerated storage (at 35 °C) for twelve weeks under Section 2 of this document.

As part of this application, Life Scientific Ltd. wishes to have the proposed formulation assessed for comparability to the Polish reference product Atlantis 12 OD (10 g/L mesosulfuron-methyl, 2 g/L iodosulfuron-methyl-sodium and 30 g/L mefenpyr-diethyl, OD, authorisation number R-98/2009) of Bayer AG. The applicant considers product 054-01-05 to be comparable, if not identical to Atlantis 12 OD: details provided in Table 1.2-1 in the confidential dossier of this submission (Draft Registration Report – Part C). The uses and claims for which approval is being sought are the same as those already approved for Atlantis 12 OD in Poland.

Atlantis 12 OD (authorisation number R-98/2009) was first authorised on 14th August 2009 and re-registered on 24th August 2020. Given the 30-month data protection period for Atlantis 12 OD and the associated active substances, namely mesosulfuron-methyl and iodosulfuron-methyl-sodium, expired in February 2023, a new application is being submitted to apply for the authorisation of product 054-01-05 in Poland, whereby the applicant submits that it is scientifically valid to extrapolate data and information submitted by Bayer AG on Atlantis 12 OD and use it to evaluate product 054-01-05. This includes the data supporting uses that were applied for after the introduction of Regulation 1107/2009 on 14th of June 2011. According to Paragraph 22 of Commission Notice - Technical Guidelines on Data Protection according to Regulation (EC) No. 1107/2009, 2019/C 229/01, new use data attracts 10 years protection from the date of first authorisation of that product in each Member State (not the date of authorisation of the new crop). Therefore, under Regulation 1107/2009, new use data attracts zero data protection when the original 10-year data protection of the product has expired.

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name:	Life Scientific Limited

Address:	Block 4

Belfield Office Park

Beech Hill Road

Dublin 4

Ireland

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information is provided separately (Part C).

Name:    	Life Scientific Ltd.

Address: 	Block 4

Belfield Office Park

Beech Hill Road

Dublin 4

Ireland

Person to contact:

Name: 		Nicola Mitchell

Phone:     	+353 1 2832024

FAX: 		+353 1 2832026

e-mail:		nicola.mitchell@lifescientific.com

Location of the production site	
CONFIDENTIAL information - data provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information is provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

Confidential information is provided separately (Part C).

#### Iodosulfuron-methyl-sodium

| Iodosulfuron-methyl-sodium | min. 910 g/kg (SANTE/2016/11167 Rev 3 and Reg (EU) 2017/407) |
| --- | --- |
| No relevant impurities |  |

#### Mesosulfuron-methyl

| Mesosulfuron-methyl No relevant impurities | min. 930 g/kg (SANTE/11827/2016 Rev 2 and Reg (EU) 2017/755) min. 950 g/kg (Life Scientific Ltd sources 1&2) |
| --- | --- |

#### Mefenpyr-diethyl (safener)

| Mefenpyr-diethyl | min. 960 g/kg |
| --- | --- |
| No relevant impurities | min. 980 g/kg (Life Scientific Ltd source) |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Meso-Iodo OD-Life |
| --- | --- |
| Company code number: | 054-01-05 |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Product 054-01-05 is a professional use herbicide formulated as an oil dispersion (OD) containing 10 g/L mesosulfuron-methyl, 2 g/L iodosulfuron-methyl-sodium and 30 g/L mefenpyr-diethyl. The product is considered to be identical to the Polish product Atlantis 12 OD (10 g/L mesosulfuron-methyl, 2 g/L iodosulfuron-methyl-sodium and 30 g/L mefenpyr-diethyl, OD; Authorisation number R-98/2009) of Bayer AG.

Details of the formulation composition are provided in the confidential dossier of this submission (Draft Registration Report - Part C).

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Mesosulfuron-methyl | 10.0 g/L | 8.5 – 11.5 g/L (10.0 g/L ± 15%) | Please refer to the confidential dossier (Draft Registration Report Part C) 10.53 | Please refer to the confidential dossier (Draft Registration Report Part C) 1.05 |
| Iodosulfuron-methyl-sodium | 2.0 g/L | 1.7 – 2.3 g/L1 (2.0 g/L ± 15%) | Please refer to the confidential dossier (Draft Registration Report Part C) 2.2 | Please refer to the confidential dossier (Draft Registration Report Part C) 0.22 |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

** 	Based on the density of the formulation = 0.99963

Table 1.4-2:	Safener and synergists

| Safener / synergist | Declared content of the safener / synergist (g/L or g/kg) | Limits (min – max) | Technical content (g/L or g/kg) | Technical content (%w/w) |
| --- | --- | --- | --- | --- |
| Mefenpyr-diethyl | 30 g/L | 27 – 33 g/L1 (30 g/L ± 10%) | Please refer to the confidential dossier (Draft Registration Report Part C) 30.61 | Please refer to the confidential dossier (Draft Registration Report Part C) 3.06 |

* 	Based on the minimum purity of the safener declared for registration: 98%

** 	Based on the density of the formulation = 0.99963

Table 1.4-3:	Relevant impurities

| Relevant impurity | Maximum content (g/L or g/kg) |
| --- | --- |
| N/A |  |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-4:	Information on active substance Mesosulfuron-methyl

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Mesosulfuron-methyl |
| CAS No. | 208465-21-8 |
| EC No. | - |
| CIPAC No. | 663.201 |

Table 1.4-5:	Information on Iodosulfuron-methyl-sodium

| Type | Name/Code Number |
| --- | --- |
| ISO com | Iodosulfuron-methyl-sodium |
| CAS No. | 144550-36-7 |
| EC No. | - |
| CIPAC No. | 634.501 |

CONFIDENTIAL information is provided separately (Part C).

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

Table 1.4-6:	Information on Mefenpyr-diethyl (safener)

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Mefenpyr-diethyl |
| CAS No. | 135590-91-9 (racemate) |
| EC No. | - |
| CIPAC No. | 651.229 |

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Oil Dispersion | [Code: OD] |
| --- | --- |

## Function (KCP 1.6)

Herbicide

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable. The appearance of the product is a brown opaque uniform liquid, with a solvent odour. It is not explosive and has no oxidising properties. The product is not flammable/has a flash point of 88 °C. In aqueous solution, it has a pH value around 9.12. There is no effect of low temperature on the stability of the formulation, since after 7 days at 0 °C neither the active ingredient content nor the technical properties were changed. After 12 weeks at 35 °C, neither the active ingredient content nor the technical properties were changed, with the exception of the iodosulfuron-methyl-sodium content which decreased by 5.96 %. However, after 18 weeks at 30 °C iodosulfuron-methyl-sodium was found to be stable and therefore all criteria are met. The stability data indicate a shelf life of at least 2 years at ambient temperature when stored in packaging material proposed. Its technical characteristics are acceptable for an OD formulation.

The maximum application rate is 1.5 L/ha, equivalent to 15 g a.s./ha and 3 g a.s./ha of mesosulfuron-methyl and iodosulfuron-methyl-sodium, respectively. The concentration range of mesosulfuron-methyl in the diluted spray solution is 0.0375 to 0.075 g/L, and the concentration range of iodosulfuron-methyl-sodium in the diluted spray solution is 0.0075 to 0.015 g/L, when used in a water volume of 200 to 400 L/ha.

The minimum application rate is 0.5 L/ha, equivalent to 5 g a.s.ha and 1.0 g a.s/ha of mesosulfuron-methyl and iodosulfuron-methyl-sodium, respectively. The concentration range of mesosulfuron-methyl in the diluted spray solution is 0.0125 to 0.025 g/L, and the concentration range of iodosulfuron-methyl-sodium in the diluted spray solution is 0.0025 to 0.005 g/L, when used in a water volume of 200 to 400 L/ha.

The intended concentration of use is 0.125 % to 0.75 % v/v.

No tank mixes recommended.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

The product is not classified on the basis of its physical and chemical properties.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

Not applicable

Compliance with FAO specifications:

There is no FAO specification for mesosulfuron-methyl. The content of mesosulfuron-methyl in 054-01-05 is within the permitted tolerance of ± 15% based on the limits take from the CRD “Guidance Document for the Generation of Data on the Physical, Chemical and Technical Properties of Plant Protection Products under Regulation (EC) No. 1107/2009”.

There is no FAO specification for iodosulfuron-methyl-sodium. The content of iodosulfuron-methyl-sodium in 054-01-05 is within the permitted tolerance of ± 15% based on the limits take from the CRD “Guidance Document for the Generation of Data on the Physical, Chemical and Technical Properties of Plant Protection Products under Regulation (EC) No. 1107/2009”.

There is no FAO specification for mefenpyr-diethyl oil dispersion. The content of iodosulfuron-methyl-sodium mefenpyr-diethyl in 054-01-05 is within the permitted tolerance of ± 10% based on the limits take from the CRD “Guidance Document for the Generation of Data on the Physical, Chemical and Technical Properties of Plant Protection Products under Regulation (EC) No. 1107/2009”.

Formulation used for tests

The formulation used in the tests was product 054-01-05, which has the same composition as the one cited in Part C of the dRR.

The following batches have been used in the physico-chemical studies:

- Product 054-01-05, batch E670; Mesosulfuron-methyl-10.7 g/L, Iodosulfuron-methylsodium-2.1 g/L, Mefenpyr-diethyl-29.8 g/L

The information from accelerated storage stability studies (Mc Donald, S. 2020 and Harding, L. 2020) are presented in Table 2-1 for information purposes.

For the storage stability assessment (Harding, L. 2020), one aliquot of test item was stored at 35 °C for 12 weeks. Another aliquot was stored at ambient temperature. The aliquots were tested as “After Storage” and “Before Storage” samples respectively at the end of the 12 weeks period. The iodosulfuron-methyl-sodium content in the “After Storage” sample showed a loss of 5.96% in comparison to the iodosulfuron-methyl-sodium content in the “Before Storage” sample.

Therefore, a second accelerated storage study was commissioned (Mc Donald, S. 2020) in which the iodosulfuron-methyl-sodium content was measured following storage of an aliquot of the test item for 18 weeks at 30 ˚C. As all of the other criteria in the 12-week storage study met the required specification, only the iodosulfuron-methyl-sodium content was measured in the Mc Donald study. After 18 weeks storage the iodosulfuron-methyl-sodium content had decreased by 4.48% relative to the before sample. As there was ≤ 5% loss of active substance, the formulation was considered to be stable. See section KCP 2.7.2 of Table 2-1 for full results.

Table 2-5:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | Visual assessment SOP CEM-3249 | Product 054-01-05 Lot no°: E670 | Before storage; After 12 weeks at 35 °C. Appearance; Opaque, uniform liquid; Opaque, uniform liquid. Colour; Brown; Brown. Odour; Solvent smell; Solvent smell. | Y | Harding, L., 2020 | Accepted |
| Explosive properties (KCP 2.2.1) | A14 Theoretical assessment DSC | Product 054-01-05 Lot no°: E670 | Product 054-01-05 does not present evidence of possessing explosive properties Differential scanning calorimetry results: The test item is not a candidate for classification as a UN Class 1 explosive substance as the total decomposition energy was < 500 J.g-1. The test item is not a candidate for classification as a UN Class 4, Division 4.1 self-reactive substance as the total decomposition energy was < 300 J.g-1. | Y | Siusiene, E., 2021 | Accepted Based on the theoretical analysis and DSC test, results explosive properties are not expected. |
| Oxidizing properties (KCP 2.2.2) | A21 Theoretical assessment Recommendations on the Transport of Dangerous Goods, Manual of Tests and Criteria, 7 th revised edition, UN; App. 6 | Product 054-01-05 Lot no°: E670 | Product 054-01-05 does not present evidence of possessing oxidizing properties | Y | Siusiene, E., 2021 | Accepted Based on the theoretical assessment and using UN screening procedures, oxidising properties are not expected. |
| Flash point (KCP 2.3.1) | A9 | Product 054-01-05 Lot no°: E670 | Product 054-01-05 does not present evidence of being flammable. Flash point 88 °C. | Y | Siusiene, E., 2021 | Accepted Performed with the Pensky-Martens closed cup apparatus. Not flammable. |
| Flammability (KCP 2.3.2) | - | - | See section KCP 2.3.1 | - | - | See KCP 2.3.1 |
| Self-heating (KCP 2.3.3) | A15 | Product 054-01-05 Lot no°: E670 | Self-heating temperature of 389 °C. | Y | Harding, L., 2020 | Accepted Not auto-flammable. |
| Acidity or alkalinity and pH (KCP 2.4.1) | - | - | As the pH value was found to be within the range > 4 and < 10 pH, acidity/alkalinity was not determined. See section KCP 2.4.2 | - | - | Not required (see KCP 2.4.2) |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | Product 054-01-05 Lot no°: E670 | Storage condition; Before storage; After 12 weeks at 35 °C. pH (1% dispersion in DI water); 9.12 at 20.4 °C; 9.35 at 22.2°C. pH (Neat formulation); 7.93 at 20.7 °C; 8.79 at 20.3 °C. | Y | Harding, L., 2020 | Accepted |
| Viscosity (KCP 2.5.1) | OECD 114 | Product 054-01-05 Lot no°: E670 | The sample is a non-Newtonian liquid. Dynamic viscosity: 20ºC: from 49.2 to 17.8 mPa.s (50.0 – 300 rpm) 40ºC: from 27 to 17.8 mPa.s (50.0 – 300 rpm) Before storage. Viscosity at 20 °C at 100 s-1 (mPa.s); 41.4. Viscosity at 40 °C at 100 s-1 (mPa.s); 21.9. | Y | Harding, L., 2020 | Accepted The formulation is non-Newtonian liquid. The formulation does not pose an aspiration hazard. |
| Surface tension (KCP 2.5.2) | OECD 115 | Product 054-01-05 Lot no°: E670 | Before storage. Surface tension at 1.5%v/v (mN/m) at 20 °C; 31.14. | Y | Harding, L., 2020 | Accepted Determined at the concentration higher than the highest in-use spray concentration (0.75%). The formulation is surface active. |
| Relative density (KCP 2.6.1) | OECD 109 | Product 054-01-05 Lot no°: E670 | Before storage. Relative density 20 °C; 0.99963. Relative density 40 °C; 0.9840. | Y Y | Harding, L., 2020 Seward, E., 2020 | Accepted |
|  |  |  | Before storage. Relative density 40 °C; 0.9840. | Y | Seward, E., 2020 | Accepted |
| Bulk density (KCP 2.6.2) | - | - | Not required for OD formulations. | - | - | Not required |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) |  |  | Please refer to section KCP 2.7.2 |  |  | See KCP 2.7.2 |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | Method VAL-18-035-Rev1 HPLC Method VAL-18-035-Rev1 HPLC Visual assessment CIPAC MT 75.3 CIPAC MT 180 CIPAC MT 187 CIPAC MT 148 CIPAC MT 185 _____________ - | Product 054-01-05 Lot no°: E670 Product 054-01-05 Lot no°: E670 Product 054-01-05 Lot no°: E670 | 12 weeks at 35 ºC Before storage; 12 weeks at 35°C. Mesosulfuron-methyl; 10.84 g/L; 10.75 g/L (0.83% change). Iodosulfuron-methyl sodium; 2.18 g/L; 2.05 g/L (5.96% change). Mefenpyr-diethyl; 29.43 g/L; 29.40 g/L (0.10% change). The iodosulfuron-methyl-sodium content in the “After Storage” sample showed a loss of 5.96% in comparison to the iodosulfuron-methyl-sodium content in the “Before Storage” sample. Therefore, the accelerated storage study was repeated (report LS-20-027) by placing an aliquot of the test item on storage at 30 ˚C for 18 weeks. 18 weeks at 30 ºC Before storage; 18 weeks at 30°C. Iodosulfuron-methyl sodium; 2.23 g/L; 2.13 g/L (4.48% change). Post 18 weeks storage the iodosulfuron-methyl-sodium content had decreased to 4.48%. Therefore, the active content met the specification of less than 5% loss of active substance on storage. 12 weeks at 35 ºC Before storage; After 12 weeks at 35 °C. Appearance; Opaque, uniform liquid; Opaque, uniform liquid. Colour; Brown; Brown. Odour; Solvent smell; Solvent smell. pH (1% dispersion in DI water); 9.12; 9.35. pH (Neat formulation); 7.93; 8.79. Dispersion stability. CIPAC water A; Before storage; After 12 weeks at 35 °C. Initial determination; Uniform formulation; Uniform formulation. Determination of sediment volume; None observed; None observed. Determination of top cream (or oil) volume; None observed; None observed. Determination of re-dispersion. Bottom sediment re-dispersed?; Yes; Yes. Sediment present; None observed; None observed. Cream/oil present; None observed; None observed. Dispersion stability. CIPAC water D; Before storage; After 12 weeks at 35 °C. Initial determination; Uniform formulation; Uniform formulation. Determination of sediment volume; None observed; None observed. Determination of top cream (or oil) volume; None observed; None observed. Determination of re-dispersion. Bottom sediment re-dispersed?; Yes; Yes. Sediment present; None observed; None observed. Cream/oil present; None observed; None observed. Particle size distribution. Storage condition; Before storage; After 12 weeks at 35 °C. d(0.1) μm; 0.067; 0.070. d(0.5) μm; 0.139; 0.155. d(0.9) μm; 1.331; 2.051. Pourability; Before storage; After 12 weeks at 35 °C. Residue (R) (%); 1.22; 1.13. Rinsed Residue (R’) (%); 0.23; 0.23. Wet sieve test; Before storage; After 12 weeks at 35 °C. % retained on 75 μm test sieve; 0.0100; 0.0058. The weights of the container, with lids, were recorded before and after storage. The container and lid were examined after storage. No signs of warping, deterioration or leakage were found Before storage; After 12 weeks at 35 °C. Stability of packaging; No pack product interaction observed; No pack product interaction observed. | Y N Y | Harding, L., 2020 Mc Donald, S., 2020 Harding, L., 2020 | Accepted The tested material was stored in the glass bottle at 35°C for 12 weeks (Harding, L., 2020). No leaking etc. was observed. No significant changes to physical and technical characteristics of the formulation. Dispersion characteristics: conc. tested 5g in 250 mL – higher than the highest in-use spray concentration (0.75%). The lowest conc. was not tested. As no separation was observed before and after storage, the test is considered acceptable. Changes in a.s. content were <5%, except for iodosulfuron-methyl sodium – 5.96% decrease. Therefore the formulation was again stored, at 30 ˚C for 18 weeks (Mc Donald, S., 2020). The result (4.48 % decrease) under the alternative time/temperature regime is acceptable. Information on the storage conditions below 30°C has already been included in the label. |
| Minimum content after heat stability testing (KCP 2.7.3) | - | - | Please refer to section KCP 2.7.2 | - | - | See KCP 2.7.2 |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 CIPAC MT 180 CIPAC MT 185 | Product 054-01-05 Lot no°: E670 | Low temperature stability Observation; Volume (mL); Description (oil/cream etc). Separated material present after 7 days at 0 °C ± 2 °C?; 0; Uniform formulation, no separation observed. Separated material present following 24 hr to reach room temp and a single inversion?; 0; Uniform formulation, no separation observed. Dispersion stability. CIPAC water A; Before storage; Low temperature. Initial determination; Uniform formulation; Uniform formulation. Determination of sediment volume; None observed; None observed. Determination of top cream (or oil) volume; None observed; None observed. Determination of re-dispersion. Bottom sediment re-dispersed?; Yes; Yes. Sediment present; None observed; None observed. Cream/oil present; None observed; None observed. Dispersion stability. CIPAC water D; Before storage; Low temperature. Initial determination; Uniform formulation; Uniform formulation. Determination of sediment volume; None observed; None observed. Determination of top cream (or oil) volume; None observed; None observed. Determination of re-dispersion. Bottom sediment re-dispersed?; Yes; Yes. Sediment present; None observed; None observed. Cream/oil present; None observed; None observed. Wet sieve test; Before storage; Low temperature. % retained on 75 μm test sieve; 0.0100; 0.005. | Y | Harding, L., 2020 | Accepted Not affected by low temperature |
| Ambient temperature shelf life (KCP 2.7.5) | - | - | Two years storage can be provided under request. | - | - | Not required |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Not applicable. | - | - | Not applicable |
| Wettability (KCP 2.8.1) | - | - | Not required for OD formulations. | - | - | Not required |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.3 | Product 054-01-05 Lot no°: E670 | Persistent foaming of the formulation has been measured at the highest and lowest in-use concentrations. Highest concentration. Foam after 1 min (mL); 26. Foam after 12 min (mL); 12. Lowest concentration. Foam after 1 min (mL); 14. Foam after 12 min (mL); 7. | Y N | Harding, L., 2020 Mc Donald, S., 2021 | Accepted The ‘Highest concantration’ was higher than the highest in-use spray conc. (0.75%). The ‘Lowest concentration’ was the lowest in-use spray conc. (0.125%). |
| Suspensibility (KCP 2.8.3.1) | - | - | Not required for OD formulations. | - | - | Not required |
| Spontaneity of dispersion (KCP 2.8.3.2) | - | - | Not required for OD formulations. | - | - | Not required |
| Dispersion stability (KCP 2.8.3.3) | CIPAC MT 180 | Product 054-01-05 Lot no°: E670 | CIPAC water A Dispersion stability. Before storage; 12 weeks at 35 °C; Low temperature. Initial determination; Uniform formulation; Uniform formulation; Uniform formulation. Determination of sediment volume; None observed; None observed; None observed. Determination of top cream (or oil) volume; None observed; None observed; None observed. Determination of re-dispersion. Bottom sediment re-dispersed?; Yes; Yes; Yes. Sediment present; None observed; None observed; None observed. Cream/oil present; None observed; None observed; None observed. CIPAC water D Dispersion stability. Before storage; 12 weeks at 35 °C; Low temperature. Initial determination; Uniform formulation; Uniform formulation; Uniform formulation. Determination of sediment volume; None observed; None observed; None observed. Determination of top cream (or oil) volume; None observed; None observed; None observed. Determination of re-dispersion. Bottom sediment re-dispersed?; Yes; Yes; Yes. Sediment present; None observed; None observed; None observed. Cream/oil present; None observed; None observed; None observed. | Y | Harding, L., 2020 | Accepted Conc. tested 5g in 250 mL – higher than the highest in-use spray concentration (0.75%). The lowest conc. was not tested. As no separation was observed before and after storage, the test is considered acceptable. |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not required for OD formulations. | - | - | Not required |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 187 | Product 054-01-05 Lot no°: E670 | Storage condition; Before storage; 12 weeks at 35 °C. d(0.1) μm; 0.067; 0.070. d(0.5) μm; 0.139; 0.155. d(0.9) μm; 1.331; 2.051. | Y | Harding, L., 2020 | Accepted Additional study |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | Product 054-01-05 Lot no°: E670 | Wet sieve test; Before storage; 12 weeks at 35 °C; Low temperature. % retained on 75 μm test sieve; 0.0100; 0.0058; 0.005. | Y | Harding, L., 2020 | Accepted |
| Dust content (KCP 2.8.5.2.1) | - | - | Not required for OD formulations. | - | - | Not required |
| Particle size of dust (KCP 2.8.5.2.2) | - | - | Not required for OD formulations. | - | - | Not required |
| Attrition (KCP 2.8.5.3) | - | - | Not required for OD formulations. | - | - | Not required |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not required for OD formulations. | - | - | Not required |
| Emulsifiability (KCP 2.8.6.1) | - | - | Not required for OD formulations. | - | - | Not required |
| Emulsion stability (KCP 2.8.6.2) | - | - | Not required for OD formulations. | - | - | Not required |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Not required for OD formulations. | - | - | Not required |
| Flowability (KCP 2.8.7.1) | - | - | Not required for OD formulations. | - | - | Not required |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148 | Product 054-01-05 Lot no°: E670 | Pourability; Before storage; After 12 weeks at 35 °C. Residue (R) (%); 1.22; 1.13. Rinsed Residue (R’) (%); 0.23; 0.23. | Y | Harding, L., 2020 | Accepted |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not required for OD formulations. | - | - | Not required |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | Nor applicable | - | - | Not required |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | Not applicable | - | - | Not required |
| Adhesion to seeds (KCP 2.10.1) | - | - | Not required as the preparation is not used for seed treatment | - | - | Not required |
| Distribution to seed (KCP 2.10.2) | - | - | Not required as the preparation is not used for seed treatment | - | - | Not required |
| Other/special studies (KCP 2.11) | - CRD Efficacy guideline 305 EPPO PP1/292(1) - Cleaning pesticide application equipment (PAE) - efficacy aspects, 2016, Revision 1 | Product 054-01-05 Lot no°: E670 | The weights of the container, with lids, were recorded before and after storage. The container and lid were examined after storage. No signs of warping, deterioration or leakage were found. Before storage; After 12 weeks at 35 °C. Stability of packaging; No pack product interaction observed; No pack product interaction observed. A dilution of test item in water was added to polyethylene bottles and these subjected to a three-stage rinsing procedure. The contents of the active substances, mesosulfuron-methyl and iodosulfuron-methyl-sodium, in the final acetonitrile rinse were then quantified using a high performance liquid chromatography (HPLC) method with ultraviolet (UV) detection. The analytes, mesosulfuron-methyl and iodosulfuron-methyl-sodium, were not detected in the final acetonitrile rinse. | Y Y | Harding, L., 2020 Dunning, J., 2020 | Accepted |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

RMS comment: The accelerated test for the product 054-01-05 (Meso-Iodo OD-Life) was performed in a glass bottle, which is an acceptable packaging in this study. However, the Applicant requests the same packaging as registered for the reference product Atlantis 12 OD – HDPE/PA and HDPE/EVOH. As Meso-Iodo OD-Life is considered comparable to Atlantis 12 OD and as the data protection for Atlantis 12 OD has expired, it is considered appropriate to use the Atlantis 12 OD evaluation to support the intended packaging types for Meso-Iodo OD-Life.

Conclusion: proposed packaging type HDPE/PA and HDPE/EVOH– accepted.

The applicant requests the same packaging for product 054-01-05 as registered for the reference product Atlantis 12 OD (authorisation number R-98/2009) of Bayer AG. The applicant also requests the use of both HDPE/PA and HDPE/EVOH for all packaging volumes authorised for Atlantis 12 OD (authorisation number R-98/2009).

The packaging requested for product 054-01-05 is as follows:

- 50 mL, 100 mL, 250 mL, 500 mL, 1 L HDPE/PA bottles

3 L, 5 L, 10 L, 15 L HDPE/PA jerry cans

- 50 mL, 100 mL, 250 mL, 500 mL, 1 L HDPE/EVOH bottles

3 L, 5 L, 10 L, 15 L HDPE/EVOH jerry cans

Table 4.1-6:	Packaging information for 50 mL bottle

| Volume | 50 mL |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Cylindrical / 44 x 82.6 mm |
| Opening: | 27.8 mm |
| Closure: | Screw cap |
| Seal: | HF seal, Foam Disc |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-2:	Packaging information for 100 mL bottle

| Volume | 100 mL |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Cylindrical / 44 x 114.5 mm |
| Opening: | 27.8 mm |
| Closure: | Screw cap |
| Seal: | HF seal, Foam Disc |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-3:	Packaging information for 250 mL bottle

| Volume | 250 mL |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Cylindrical / 62.5 x 137 mm |
| Opening: | 45 mm |
| Closure: | Screw cap |
| Seal: | HF seal, Foam Disc |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-4:	Packaging information for 500 mL bottle

| Volume | 500 mL |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Cylindrical / 69 x 187 mm |
| Opening: | 45 mm |
| Closure: | Screw cap |
| Seal: | Induction heat sealing liner |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-5:	Packaging information for 1 L bottle

| Volume | 1 L |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Cylindrical / 88.5 x 238 mm |
| Opening: | 45 mm |
| Closure: | Screw cap |
| Seal: | Induction heat sealing liner |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-6:	Packaging information for 3 L jerry can

| Volume | 3 L |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Rectangular / 190 x 140 x 236 mm |
| Opening: | 57.8 mm |
| Closure: | Screw cap |
| Seal: | HF seal, Foam Disc |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-7:	Packaging information for 5 L jerry can

| Volume | 5L |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Rectangular / 190 x 140 x 305 mm |
| Opening: | 57.8 mm |
| Closure: | Screw cap |
| Seal: | Induction heat sealing liner |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-8:	Packaging information for 10 L jerry can

| Volume | 10 L |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Rectangular / 226 x 186 x 370 mm |
| Opening: | 57.8 mm |
| Closure: | Screw cap |
| Seal: | HF seal, Foam Disc |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

Table 4.1-9:	Packaging information for 15 L jerry can

| Volume | 15 L |
| --- | --- |
| Material 1: | Coex HDPE/EVOH |
| Material 2: | Coex HDPE/PA |
| Shape/size: | Rectangular / 245 x 225 x 404 mm |
| Opening: | 57.8 mm |
| Closure: | Screw cap |
| Seal: | HF seal |
| Manner of construction | Extruded |
| UN/ADR | Compliant |

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1 KCP 2.3.3 KCP 2.4.2 KCP 2.5.1 KCP 2.5.2 KCP 2.6.1 KCP 2.7.2 KCP 2.7.4 KCP 2.8.2 KCP 2.8.3.3 KCP 2.8.5.1.1 KCP 2.8.5.1.2 KCP 2.8.7.2 KCP 2.11 | Harding, L. | 2020 | Storage Stability study of 2 OD formulations. Mesosulfuron-methyl/Iodosulfuron-methyl-sodium/Mefenpyr-diethyl 7.5/7.5/22.5 g/L OD and Mesosulfuron-methyl/Iodosulfuron-methyl-sodium/Mefenpyr-diethyl 10/2/30 g/L OD Report No: CEMS-9287 CEMAS GLP Unpublished | N | Life Scientific Ltd |
| KCP 2.2.1 KCP 2.2.2 KCP 2.3.1 | Siusiene, E. | 2021 | Phys/ Chem on a Test Item of Mesosulfuron-methyl/ Iodosulfuron-methyl-sodium/ Mefenpyr-diethyl 10/2/30 g/L OD Report No: GLP3016008680BR1/2020 DEKRA GLP Unpublished | N | Life Scientific Ltd |
| KCP 2.6.1 | Seward, E. | 2020 | Density measurement of 2 OD formulations. Mesosulfuron-methyl/Iodosulfuron-methyl-sodium/Mefenpyr-diethyl 7.5/7.5/22.5 g/L OD and Mesosulfuron-methyl/Iodosulfuron-methyl-sodium/Mefenpyr-diethyl 10/2/30 g/L OD Report No: CEMS-9750 CEMAS GLP Unpublished | N | Life Scientific Ltd |
| KCP 2.7.2 | Mc Donald, S. | 2020 | Accelerated storage stability study of mesosulfuron-methyl/iodosulfuron-methyl-sodium/mefenpyr-diethyl 10/2/30 g/L OD Report No: LS-20-027 Non GLP Unpublished | N | Life Scientific Ltd |
| KCP 2.8.2 | Mc Donald, S. | 2021 | Persistent foam analysis of mesosulfuron-methyl/iodosulfuron-methyl-sodium/mefenpyr-diethyl 10/2/30 g/L OD Report No: LS-21-003 Non GLP Unpublished | N | Life Scientific Ltd |
| KCP 2.11 | Dunning, J., | 2020 | Mesosulfuron/iodosulfuron OD formulations: cleaning application equipment - small scale jar test protocol Report No: ENV-19-258 Agrochemex GLP Unpublished | N | Life Scientific Ltd |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Additional data on the physical, chemical and technical properties of the active substance

  - Mesosulfuron-methyl

N/A

  - Iodosulfuron-methyl-sodium

N/A

The purity of mefenpyr-diethyl indicated in DAR, September 2011, was min. 940 g/kg. It has been increased to 960 g/kg in 2018

Limits taken from Guidance Document for the Generation and Evaluation of Data on the Physical, Chemical and Technical Properties of Plant Protection Products under Reg (EC) No. 1107/2009 https://www.hse.gov.uk/pesticides/pesticides-registration/data-requirements-handbook/crd-guidance-document.pdf