FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: RNB 012 A Product name: FLENID Chemical active substance: Mesotrione, 100 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Shandong Weifang Rainbow Chemical Co., Ltd. Submission date: September 2024 MS Finalisation date: 03/2025; 06/2025

Version history

| When | What |
| --- | --- |
| September 2024 | Submsission date |
| March 2025 | zRMS first evaluation |
| June 2025 | The final Registration Report |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	4

1.2.3.1	Mesotrione	4

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	4

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	4

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	5

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	5

1.5	Type and code of the plant protection product (KCP 1.5)	5

1.6	Function (KCP 1.6)	5

2	Section 2: Physical, chemical and technical properties of the plant protection product	6

3	Section 3 is presented as a separate document	15

4	Section 4: Further information on the plant protection product	15

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	15

Appendix 1	Lists of data considered in support of the evaluation	26

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	28

A 2.1	Mesotrione	28

zRMS comment: Sufficient data on identity, physical and chemical properties and other information are available for the plant protection product and the contained technical active substance. No data gaps.

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name:	Shandong Weifang Rainbow Chemical Co., Ltd.

Address:	Binhai Economic Development Area, Weifang, Shandong, 262737, China

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Mesotrione

| Mesotrione Mesotrione | min. 920 g/kg (SANTE/11654/2016) min. 980 g/kg (Rainbow source) |
| --- | --- |

Relevant impurities:

| R287431 R287432 1,2-dichloroethane | max. 2 mg/kg (SANTE/11654/2016) max. 2 g/kg (SANTE/11654/2016) max. 1 g/kg (SANTE/11654/2016) |
| --- | --- |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Please refer to Registration Report Part A for the relevant country (or) |
| --- | --- |
| Trade name: | FLENID |
| Company code number: | RNB 012 A |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Mesotrione | 100 g/L | 90 – 100 110 g/L (±10%) | 102.04 g/L | 9.72% |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers by the Applicant

** 	Based on the density of the formulation = 1.0502 g/mL

Table 1.4-2:	Relevant impurities

| Relevant impurity | Maximum content (mg/L) |
| --- | --- |
| 1,2-dichloroethane | 102.04 |
| 1-cyano-6-(methylsulfonyl)-7-nitro-9H-xanthen-9-one (R287431) | 0.20408 |
| 6-(methylsulfonyl)-9-oxo-9H-xanthene-1-carbonitrile (R287432) | 204.08 |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-3:	Information on Mesotrione

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Mesotrione |
| CAS No. | 104206-82-8 |
| EC No. | 609-064-00 |
| CIPAC No. | 625 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Suspension concentrate | [Code: SC] |
| --- | --- |

## Function (KCP 1.6)

Mesotrione 100 g/L SC is intended to be used as herbicide.

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.  The appearance of the product is that of off white liquid, with a characteristic odour. It is not explosive, has no oxidising properties. The product has a flash point above 135 °C. It has a self-ignition temperature of 530 °C. In aqueous solution, it has a pH value around 3.5 at 20 °C. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0 °C and 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. Its technical characteristics are acceptable for a suspension concentrate formulation.

The intended concentration of use is 0.33% to 0.50%.

No tank mixes recommended

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

Neither classification or labelling are relevant for this section.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No risk and safety phrases are relevant for this section.

Compliance with FAO specifications:

The product RNB 012 A complies with FAO specifications.

At the time of the evaluation, there is no FAO specification for the formulation with mesotrione.

Formulation used for tests

The test item used for tests is the one cited in dRR Part C.

Table 2-4:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Off white liquid with characteristic odour | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Explosive properties (KCP 2.2.1) | Manual of Tests and Criteria ST/SG/AC.10/11/Rev. 7 (2019)– Appendix 6, Section 3 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | The test item has no explosive properties. | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted Determined by DSC analysis. The test item formulation sample present an exothermic event of low intensity and an endothermic event. The total area for the exothermic events is lower than 500 J/g. The formulation is not explosive. |
| Oxidizing properties (KCP 2.2.2) | EC A. 21 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | The test item has no oxidizing properties. | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted No oxidising properties under the conditions of the test – the formulation is not oxidising. |
| Flash point (KCP 2.3.1) | EC A. 9 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Flash point > 135°C | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted Performed with the Pensky-Martens closed cup apparatus. The formulation is not flammable. |
| Flammability (KCP 2.3.2) | - | - | Not relevant | - | - | - |
| Self-heating (KCP 2.3.3) | EC A. 15 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | T = 530 °C | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted Performed with a Koehler instrument. |
| Acidity or alkalinity and pH (KCP 2.4.1) | CIPAC MT 75.3 CIPAC MT 31 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Ph pH neat = 4.7 Acidity: 1.45% w/w as H2SO4 | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | pH 1% aqueous dispersion: 3.5 | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Viscosity (KCP 2.5.1) | CIPAC MT 192 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Shear rate (s-1); Dynamic Viscosiy (cP). T = 20°C; T = 40°C. 2.04; 1163.33; 1235.00. 3.40; 806.67; 826.00. 4.08; 702.67; 719.67. 6.80; 475.33; 496.67. 10.20; 353.67; 362.00. 17.00; 243.30; 249.00. 20.40; 212.60; 217.17. 34.00; 149.03; 159.20. | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted Considered non Newtonian liquid. The formulation does not contain at least 10% of a substance which has been classified as H304. No aspiration hazard expected. |
| Surface tension (KCP 2.5.2) | OECD 115 EC A. 5 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Neat: 31.6 mN/m 0.33 % v/v = 38.4 mN/m 0.75% v/v: 34.2 mN/m | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted The highest conc. tested was higher than the max use rate recommended (0.5%). A surface-active formulation |
| Relative density (KCP 2.6.1) | CIPAC MT 3.3.1 OECD No 109 EC A. 3 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Density 1.0502 g/mL at 20 °C Relative density: 1.0502 | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Bulk density (KCP 2.6.2) | - | - | Not relevant | - | - | - |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 SANCO/3030/99 rev. 5 CIPAC MT 75.3 CIPAC MT 31 CIPAC MT 148 CIPAC MT 184.1 CIPAC MT 160 CIPAC MT 185 CIPAC MT 187 CIPAC MT 46.4 HPLC-UV GC-MS HPLC-MS/MS | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Test; Initial; After. Appearance; Off white liquid with characteristic odour; Off white liquid with characteristic odour. Mesotrione content; 9.13 % w/w 95.9 g/L; 9.18 % w/w 96.4 g/L. 1,2-dichloroethane relevant impurity content; <5 µg/kg µg/g; <5 µg/kg µg/g. R287431 relevant impurity content; <0.01 µg/kg µg/g (< LOD); <0.01 µg/kg µg/g (< LOD). R287432 relevant impurity content; <5 µg/kg µg/g (< LOD); <5 µg/kg µg/g (< LOD). Suspensibility; 0.25% v/v in Standard Water D: 95.2% 0.33% v/v in Standard Water D: 98.1% 0.75% v/v in Standard Water D: 98.1%; 0.25% v/v in Standard Water D: 97.9% 0.33% v/v in Standard Water D: 99.1% 0.75% v/v in Standard Water D: 96.8%. Spontaneity of dispersion (5 % v/v); 93.1%; 93.3%. Wet sieve test (10 % v/v); No residue on the 75 µm sieve.; No residue on the 75 µm sieve.. Particle size distribution; Dv 10: 0.0285 μm Dv 50: 0.158 μm Dv 90: 5.75 μm % under 45 μm: 100.00 % above 75 μm: 0.00; Dv 10: 0.0280 μm Dv 50: 0.143 μm Dv 90: 2.39 μm % under 45 μm: 100.00 % above 75 μm: 0.00. pH neat; 4.7; 4.6. pH 1% dispersion; 3.5; 3.6. Acidity; 1.45% w/w as H2SO4; 1.44% w/w as H2SO4. Pourability; R = 2.48% R’ = 0.24%; R = 2.93% R’ = 0.14%. Stability of the packaging material; -; The container (HDPE bottle) didn’t present any deformation in both bottom and lateral layers, or loss of sample and evident corrosion phenomena. Weight variation: -0.02% and -0.03 %. | Y | E. Rigamonti, 2023 Report No. CH-1209-2022 | Accepted Stored in HDPE bottle, packaging proved to be resistant. All tests required after storage were performed, no significant changes in the physical, chemical, and technical properties of the formulation were observed. For the determination of the a.s. and relevant impurities content, the methods validated according to SANCO/3030/99 rev.5 were used (see Part B5 of the dRR). No a.s. decrase. For the relevant impurity 1,2-DCE, results were < LOQ (5.0 μg/g), higher than LOD (2.0 μg/g). |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | - | - | Not relevant | - | - | - |
| Minimum content after heat stability testing (KCP 2.7.3) | - | - | Not relevant | - | - | - |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 CIPAC MT 185 CIPAC MT 184.1 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Test; Initial; After 7 days at 0°C. Wet sieve test; No residue on the 75 µm sieve.; No residue on the 75 µm sieve.. Suspensibility; 0.25% v/v in Standard Water D: 95.2% 0.33% v/v in Standard Water D: 98.1% 0.75% v/v in Standard Water D: 98.1%; 0.25% v/v in Standard Water D: 98.1% 0.33% v/v in Standard Water D: 97.1% 0.75% v/v in Standard Water D: 96.7%. | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted No significant changes in formulation properties prior to storage. Not sensitive to low temperatures. |
| Ambient temperature shelf life (KCP 2.7.5) | - | - | Not relevant | - | - | Not required, see KCP 2.7.1 |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Not relevant | - | - | - |
| Wettability (KCP 2.8.1) | - | - | Not relevant | - | - | - |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.3 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | 0.25% v/v in Standard Water D: 0 mL after 1 minute 0.33% v/v in Standard Water D: 0 mL after 1 minute 0.75% v/v in Standard Water D: 0 mL after 1 minute | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted The highest conc. tested was higher than the max use rate recommended (0.5%). |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184.1 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | 0.25% v/v in Standard Water D: 95.2% 0.33% v/v in Standard Water D: 98.1% 0.75% v/v in Standard Water D: 98.1% | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted The highest conc. tested was higher than the max use rate recommended (0.5%). |
| Spontaneity of dispersion (KCP 2.8.3.2) | CIPAC MT 160 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | 5 % v/v: 93.1% | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Dispersion stability (KCP 2.8.3.3) | - | - | Not relevant | - | - | - |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not relevant | - | - | - |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 187 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Dv 10: 0.0285 μm Dv 50: 0.158 μm Dv 90: 5.75 μm % under 45 μm: 0.00 % above 75 μm: 100.00 | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | 10 % v/v: no residue on the 75 µm sieve. | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Dust content (KCP 2.8.5.2.1) | - | - | Not relevant | - | - | - |
| Particle size of dust (KCP 2.8.5.2.2) | - | - | Not relevant | - | - | - |
| Attrition (KCP 2.8.5.3) | - | - | Not relevant | - | - | - |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not relevant | - | - | - |
| Emulsifiability (KCP 2.8.6.1) | - | - | Not relevant | - | - | - |
| Emulsion stability (KCP 2.8.6.2) | - | - | Not relevant | - | - | - |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Not relevant | - | - | - |
| Flowability (KCP 2.8.7.1) | - | - | Not relevant | - | - | - |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | R = 2.48% R’ = 0.24% | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not relevant | - | - | - |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | Not relevant | - | - | - |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | Not relevant | - | - | - |
| Adhesion to seeds (KCP 2.10.1) | - | - | Not relevant | - | - | - |
| Distribution to seed (KCP 2.10.2) | - | - | Not relevant | - | - | - |
| Other/special studies (KCP 2.11) | PSD 302 and 305 | Mesotrione 100 g/L SC (Batch No. RFDOT0701) | Washing efficacy after triple rinsing: 100% removed | Y | E. Rigamonti, 2023 Report No. CH-1205-2022 | Accepted |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

zRMS comment: Based on the accelerated storage results, HDPE packaging is an appropriate and acceptable packaging type for RNB 012 A/ FLENID. According to the current guidelines, extrapolation to HDPE/PA, HDPE/EVOH and HDPE/F packaging is acceptable. All packaging types proposed are acceptable.

## Packaging and Compatibility with the Preparation (KCP 4.4)

Table 4.1-1:	Packaging information for 100 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 45.0 mm diameter x 115.0 mm |
| Opening: | 26.8 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 250 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 58.0 mm diameter x 149.0 mm |
| Opening: | 31.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 500 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 72.0 mm diameter x 192.0 mm |
| Opening: | 38.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 1.0 L bottle

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 250.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 4.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Square barrel / approx. 190.0 mm x 250.0 mm x 282.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-6:	Packaging information for 5.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Square barrel / approx. 183.0 mm x 133.0 mm x 300.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-7:	Packaging information for 10.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Square barrel / approx. 260.0 mm x 185.0 mm x 305.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-8:	Packaging information for 20.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Square barrel / approx. 285.00 mm x 225.0 mm x 474.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-9:	Packaging information for 200.0 L drum

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Drum / approx. 585.0 mm diameter x 940.0 mm |
| Opening: | 64 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-10:	Packaging information for 100 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Round bottle / approx. 45.0 mm diameter x 115.0 mm |
| Opening: | 26.8 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-11:	Packaging information for 250 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Round bottle / approx. 58.0 mm diameter x 149.0 mm |
| Opening: | 31.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-12:	Packaging information for 500 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Round bottle / approx. 72.0 mm diameter x 192.0 mm |
| Opening: | 38.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-13:	Packaging information for 1.0 L bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 250.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-14:	Packaging information for 4.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Square barrel / approx. 190.0 mm x 250.0 mm x 282.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-15:	Packaging information for 5.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Square barrel / approx. 183.0 mm x 133.0 mm x 300.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-16:	Packaging information for 10.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Square barrel / approx. 260.0 mm x 185.0 mm x 305.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-17:	Packaging information for 20.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Square barrel / approx. 285.00 mm x 225.0 mm x 474.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-18:	Packaging information for 200.0 L drum

| Type | Description |
| --- | --- |
| Material: | HDPE/PA |
| Shape/size: | Drum / approx. 585.0 mm diameter x 940.0 mm |
| Opening: | 64 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-19:	Packaging information for 100 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Round bottle / approx. 45.0 mm diameter x 115.0 mm |
| Opening: | 26.8 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-20:	Packaging information for 250 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Round bottle / approx. 58.0 mm diameter x 149.0 mm |
| Opening: | 31.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-21:	Packaging information for 500 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Round bottle / approx. 72.0 mm diameter x 192.0 mm |
| Opening: | 38.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-22:	Packaging information for 1.0 L bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 250.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-23:	Packaging information for 4.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Square barrel / approx. 190.0 mm x 250.0 mm x 282.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-24:	Packaging information for 5.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Square barrel / approx. 183.0 mm x 133.0 mm x 300.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-25:	Packaging information for 10.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Square barrel / approx. 260.0 mm x 185.0 mm x 305.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-26:	Packaging information for 20.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Square barrel / approx. 285.00 mm x 225.0 mm x 474.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-27:	Packaging information for 200.0 L drum

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH |
| Shape/size: | Drum / approx. 585.0 mm diameter x 940.0 mm |
| Opening: | 64 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-28:	Packaging information for 100 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Round bottle / approx. 45.0 mm diameter x 115.0 mm |
| Opening: | 26.8 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-29:	Packaging information for 250 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Round bottle / approx. 58.0 mm diameter x 149.0 mm |
| Opening: | 31.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-30:	Packaging information for 500 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Round bottle / approx. 72.0 mm diameter x 192.0 mm |
| Opening: | 38.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-31:	Packaging information for 1.0 L bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 250.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-32:	Packaging information for 4.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Square barrel / approx. 190.0 mm x 250.0 mm x 282.0 mm |
| Opening: | 50 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-33:	Packaging information for 5.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Square barrel / approx. 183.0 mm x 133.0 mm x 300.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-34:	Packaging information for 10.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Square barrel / approx. 260.0 mm x 185.0 mm x 305.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-35:	Packaging information for 20.0 L square barrel

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Square barrel / approx. 285.00 mm x 225.0 mm x 474.0 mm |
| Opening: | 63 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-36:	Packaging information for 200.0 L drum

| Type | Description |
| --- | --- |
| Material: | HDPE/F |
| Shape/size: | Drum / approx. 585.0 mm diameter x 940.0 mm |
| Opening: | 64 mm inner diameter |
| Closure: | HDPE screw cap with PE gasket |
| Seal: | Pressure seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1-2.11 | E. Rigamonti | 2023 | Mesotrione 100 g/L SC: Determination of the physico-chemical properties. Chemservice S.r.l. Report No. CH-1205-2022 GLP Unpublished | N | Shandong Weifang Rainbow Chemical Co., Ltd. |
| KCP 2.7.1 | E. Rigamonti | 2023 | Mesotrione 100 g/L SC: Determination of the accelerated storage syability and corrosion characteristics. Chemservice S.r.l. Report No. CH-1209-2022 GLP Unpublished | N | Shandong Weifang Rainbow Chemical Co., Ltd. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Additional data on the physical, chemical and technical properties of the active substance

  - Mesotrione

No new data submitted.