FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: RNB 012 A Product name(s): FLENID Chemical active substance: Mesotrione, 100 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Shandong Weifang Rainbow Chemical Co., Ltd. Submission date: September 2024 Finalisation date: 02/2025; 06/2025

Version history

| When | What |
| --- | --- |
| 02/2025 | zRMS Assesment |
| 06/2025 | The final Registration Report |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	6

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	10

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	10

5.2.2	Methods for the determination of residues (KCP 5.1.2)	10

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	10

5.3.1	Analysis of the plant protection product (KCP 5.2)	10

5.3.2	Description of analytical methods for the determination of residues of Mesotrione (KCP 5.2)	11

5.3.2.1	Overview of residue definitions and levels for which compliance is required	11

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	12

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	13

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	14

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	14

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	15

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	15

5.3.2.8	Other studies/ information	16

Appendix 1	Lists of data considered in support of the evaluation	17

Appendix 2	Detailed evaluation of submitted analytical methods	22

A 2.1	Analytical methods for Mesotrione	22

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for the active substance(s) and relevant impurities in the plant protection product.

Noticed data gaps are: none

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are: none

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Maize | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in plant protection product is provided as follows:

| Comments of zRMS: | The analytical method (HPLC-UV) for determination of active substance Mesotrione in the formulation Flenid is validated and meets criteria of specificity, linearity and precision according to the requirements SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1-1 |
| --- | --- |
| Report | Mesotrione 100G/L SC: Validation of the Analytical Method for the Determination of Active Ingredient Content, Rigamonti E., 2023, Report No. CH-1206-2022 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Test item:			Mesotrione 100 g/L SC, Batch No. RFDOT0701

Reference material: 		Mesotrione, CAS No. 104206-82-8, Batch No. BCCD6166,

Purity 99.6 %

Internal standard: 		Ethyl benzoate, CAS No. 93-89-0, Batch No. STBJ7313, Purity > 99.9 %

Chromatographic conditions

Instrument: 			HPLC Agilent mod. 1200 equipped with UV detector, autosampler,

managed by Chromeleon software

Column: 			Luna C18, 5 μm, 250 x 4.6 mm i.d., Phenomenex

Column temperature: 		25 ºC

Mobile phase A: 		Water

Mobile phase B: 		Acetonitrile

Mobile phase D: 		Phosphoric acid at 10 % v/v

Gradient: 			from A:B:D 70:20:10 to A:B:D 10:80:10 in 25 min

A:B:D 10:80:10 for 5 min

Flow rate: 			1.0 mL/min

Wavelength: 			λ = 223 nm

Injection volume:		10 μL

Retention time: 		about 15.2 min for Mesotrione, about 20.5 for Ethyl benzoate

Standard solutions

Using the analytical balance and volumetric flasks, the standard solutions were prepared in acetonitrile.

Test item solution

Using the analytical balance, about 500 mg of the test item and about 50 mg of the internal standard were weighed into a 100 mL conical flask and 50 mL of acetonitrile was added. The solution was placed into ultrasonic bath for about 10 minutes, then filtered using plastic syringe and PTFE filter 0.45 μm. After diluting 1:20 with acetonitrile, an aliquot was finally transferred into a vial for the HPLC/UV analysis.

Validation - Results and discussions

Table 5.2-1:	Method suitable for the determination of active substance Mesotrione  in plant protection product (PPP) FLENID/RNB 012 A

|  | Mesotrione |
| --- | --- |
| Author(s), year | Rigamonti E., 2023 |
| Principle of method | HPLC-UV |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | 32.27 – 75.30 μg/mL (5.74 – 13.39 % w/w calculated considering the test item preparation; 60.2 – 140.6 g/L calculated considering the test item preparation and the density value 1.0502 g/mL) n = 5 y = 0.84274x – 0.01512 r = 0.99992 |
| Precision – Repeatability Mean n = 5 (% RSD) | Mean conc.: 9.13 % w/w RSD = 0.41 % RSDr = 1.92 % Hr = 0.21, Hr ≤ 1 |
| Accuracy n = 2 (% Recovery) | Total mean recovery: 97.2 % |
| Interference/ Specificity | No interferences between the analyte and other components (interferences do not contribute more than 3% of total peak area measured for the target analyte), the method is specific |
| Comment | - |

Conclusion

According to SANCO/3030/99 rev. 5 the presented method was sufficiently validated and is suitable for the determination of Mesotrione content in the test item Mesotrione 100 g/L SC.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The analytical method (GC-MS) for determination of relevant impurity 1,2-Dichloroethane in the formulation Flenid is validated and meets criteria of specificity, linearity and precision according to the requirements SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1-2 |
| --- | --- |
| Report | Mesotrione 100G/L SC: Validation of the Analytical Method for the Determination of the 1,2-Dichloroethane (1,2-DCE) as relevant impurity Content, Rigamonti E., 2023, Report No. CH-1207-2022 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Test item:			Mesotrione 100 g/L SC, Batch No. RFDOT0701

Reference material: 		1,2-dichlorethane (1,2-DCE), CAS No. 107-06-2, Batch No. BCCD1179,

Purity 100 %

Chromatographic conditions

Instrument: 	GC Agilent mod. 7890A equipped with a Single Quadrupole mass detector, split/splitless injector, autosampler, managed by Masshunter software

Column: 			ZB-624 Plus, 60 m x 0.32 mm i.d., Phenomenex

Oven program: 			40 °C for 15 min

5°C/min from 40°C to 115°C

30°C/min from 115°C to 220°C

220°C for 2 min

Injector temperature: 		270°C

Injection volume: 		1 μL

Flow rate: 			1.0 mL/min

Front inlet mode: 		Splitless

Purge Flow to Split Vent: 	70 mL/min at 0.7 min

Pressure: 			11.921 psi (at 40°C)

Retention time: 		about 28.2 min for 1,2-DCE

EI MSD conditions

Solvent delay: 			15.00 min

MS source: 			230°C

MS Quadrupole: 		150°C

SIM parameter:			62 m/z (quantifier)

49 m/z (qualifier)

64 m/z (qualifier)

98 m/z (qualifier)

Standard solutions

Using the analytical balance and volumetric flasks, the standard solutions were prepared in acetone.

Test item solution

Using the analytical balance, about 100 mg of the test item was weighed into a 50.00 mL volumetric flask and made to volume with acetone. The solution was placed into ultrasonic bath for 10 minutes and finally filtered using plastic syringe and filter 0.45 μm.

Validation - Results and discussions

Table 5.2-2:	Method suitable for the determination of the relevant impurity 1,2-DCE in plant protection product (PPP) FLENID/RNB 012 A

|  | 1,2-dichloroethane (1,2-DCE) max. 0.1 g/kg in PPP |
| --- | --- |
| Author(s), year | Rigamonti E., 2023 |
| Principle of method | GC-MS |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | Nominal values 8.00 – 500.00 ng/mL (4.00 – 250.00 μg/g calculated considering the test item preparation) 1,2-DCE 8.67 – 542.00 ng/mL (4.34 – 271.00 μg/g calculated considering the test item preparation) n = 5 y = 26x – 31 r = 0.99998 |
| Precision – Repeatability Mean n = 5 (% RSD) | Mean conc.: < LOQ Since the 1,2-DCE impurity content was not quantifiable in repeatability test, the precision was determined via the accuracy test with the lowest fortification level and was fixed at ±10 % of the lowest fortification level Mean conc.: 5.38 μg/g RSD = 4.97 % RSDr = 8.32 % Hr = 0.60, Hr ≤ 1 |
| Accuracy n = 2 (% Recovery) | Level I (5 det.): 5.42 μg/g: 105.4 % Level II (2 det.): 54.20 μg/g: 100.5 % Mean recovery: 104.0 % |
| Interference/ Specificity | No interferences between the analyte and other components (interferences do not contribute more than 3% of total peak area measured for the target analyte), the method is specific |
| LOQ | Nominal value 10.00 ng/mL (5.00 μg/g) 1,2-DCE 10.84 ng/mL (5.42 μg/g) |
| Comment | - |

Conclusion

According to SANCO/3030/99 rev. 5 the presented method was sufficiently validated and is suitable for the determination of 1,2-dichloroethane (1,2-DCE) content in the test item Mesotrione 100 g/L SC.

| Comments of zRMS: | The analytical method (HPLC-MS/MS) for determination of relevant impurities 1-cyano-6-(methylsulfonyl)-7-nitro-9H-xanthen-9-one and 6-(methylsulfonyl)-9-oxo-9H-xanthene-1-carbonitrile in the formulation Flenid are validated and meets criteria of specificity, linearity and precision according to the requirements SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1-3 |
| --- | --- |
| Report | Mesotrione 100G/L SC: Validation of the Analytical Method for the Determination of R287431 and R287432 as Relevant Impurities Content., Rigamonti E., 2023, Report No. CH-1208-2022 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Test item:			Mesotrione 100 g/L SC, Batch No. RFDOT0701

Reference materials: 	1-cyano-6-(methylsulfonyl)-7-nitro-9H-xanthen-9-one (R287431), Batch No. 13-GUY-162-1, Purity 95.74 %

6-(methylsulfonyl)-9-oxo-9H-xanthene-1-carbonitrile (R287432), Batch No. 4116-084A7, Purity 98.7 %

Chromatographic conditions

Instrument: 			UHPLC/MS-QQQ Agilent mod. 1290 equipped with binary pump,

autosampler coupled with an Agilent Jet Stream (AJS) ESI interfaced Triple Quadrupole Mass Detector, Agilent mod. 6470, managed by Mass Hunter software

Column: 			InfinityLab Poroshell 120 EC-C18, 2.7 μm, 150 x 3 mm i.d., Agilent

Column temperature: 		30 ºC

Mobile phase A: 		Water with 0.1 % formic acid + 10 mM ammonium

Mobile phase B: 		Acetonitrile

Gradient: 			from A:B: 90:10 to A:B: 40:60 in 20 min

from A:B: 40:60 to A:B: 10:9 in 5 min

A:B: 10:90 for 5 min

Flow rate: 			0.7 mL/min

Injection volume:		5 μL

Retention time: 		about 11.1 min for R287432, about 13.4 for R287431

Mass scan parameters

Ion mode: 			AJS ESI +

Gas temperature: 		350°C

Gas flow: 			10 L/min

Nebulizer: 			40 psi

Capillary current: 		3500 V

Sheath gas flow: 		12 L/min

Capillary: 			3500 V

Charging: 			1500 V

Scan type: 			Dynamic MRM

Precursor ion: 			362 m/z [M-NH4]+ for R287431

300 m/z [M]+ for R287432

Product ion: 			345 m/z with collision energy of 0 V (quantifier) for R287431

235 m/z with collision energy of 30 V (qualifier) for R287431

221.1 m/z with collision energy of 25 V (quantifier) for R287432

209.1 m/z with collision energy of 25 V (qualifier) for R287432

Standard solutions

Using the analytical balance and volumetric flasks, the standard solutions were prepared in acetonitrile.

Test item solution

Using the analytical balance, about 500 mg of the test item was weighted into a 20.00 mL volumetric flask and made up to volume with acetonitrile.The solution was shaken for some minutes and then filtered using plastic syringe and PTFE filter 0.45 μm.

An aliquot was finally transferred into a vial for analysis of impurity R287431.

The solution was diluted 1:100 (by transferring 1.00 mL into a 100.00 mL volumetric flask and making up to volume with acetonitrile) to analyze the impurity R287432.

Validation - Results and discussions

Table 5.2-3:	Methods suitable for the determination of the relevant impurities R287431 and R287432 in plant protection product (PPP) FLENID/RNB 012 A

|  | 1-cyano-6-(methylsulfonyl)-7-nitro-9H-xanthen-9-one (R287431) max. 0.0002 g/kg in PPP | 6-(methylsulfonyl)-9-oxo-9H-xanthene-1-carbonitrile (R287432) max. 0.2 g/kg in PPP |
| --- | --- | --- |
| Author(s), year | Rigamonti E., 2023 |  |
| Principle of method | HPLC-MS/MS | HPLC-MS/MS |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | Nominal values 0.25 – 12.50 ng/mL (0.01 – 0.50 μg/g calculated considering the test item preparation) R287431 0.26 – 12.92 ng/mL (0.01 – 0.52 μg/g calculated considering the test item preparation) n = 5 y = 23x – 1 r = 0.99980 | Nominal values 2.50 – 125.00 ng/mL (10.00 – 500.00 μg/g calculated considering the test item preparation) R287432 2.57 – 128.31 ng/mL (10.26 – 513.24 μg/g calculated considering the test item preparation) n = 5 y = 139x – 272 r = 0.99953 |
| Precision – Repeatability Mean n = 5 (% RSD) | Mean conc.: < LOD 0.13 ng/mL (0.01 μg/g) Since the R287431 content was not detectable in repeatability test, the precision was determined via the trueness test with the lowest fortification level and was fixed at ±10 % of the lowest fortification level Mean conc.: 0.02 μg/g RSD = 4.23 % RSDr = 19.47 % Hr = 0.22, Hr ≤ 1 | Mean conc.: < LOD 1.25 ng/mL (5.00 μg/g) Since the R287431 content was not detectable in repeatability test, the precision was determined via the trueness test with the lowest fortification level and was fixed at ±10 % of the lowest fortification level Mean conc.: 24.90 μg/g RSD = 0.85 % RSDr = 6.61 % Hr = 0.13, Hr ≤ 1 |
| Accuracy n = 2 (% Recovery) | Level I (5 det.): 0.02 μg/g: 91.9 % Level II (2 det.): 0.21 μg/g: 103.4 % Mean recovery: 95.22 % | Level I (5 det.): 20.53 μg/g: 121.4 % Level II (2 det.): 205.30 μg/g: 92.7 % Mean recovery: 113.20 % |
| Interference/ Specificity | No interferences between the analyte and other components (interferences do not contribute more than 3% of total peak area measured for the target analyte), the method is specific | No interferences between the analyte and other components (interferences do not contribute more than 3% of total peak area measured for the target analyte), the method is specific |
| LOQ | Nominal values 0.50 ng/mL (0.02 μg/g) R287431 0.52 ng/mL (0.02 μg/g) | Nominal values 5.00 ng/mL (20.00 μg/g) R287432 5.13 ng/mL (20.53 μg/g) |
| Comment | - | - |

Conclusion

According to SANCO/3030/99 rev. 5 the presented methods were sufficiently validated and are suitable for the determination of 1-cyano-6-(methylsulfonyl)-7-nitro-9H-xanthen-9-one (R287431) and 6-(methylsulfonyl)-9-oxo-9H-xanthene-1-carbonitrile (R287432) content in the test item Mesotrione 100 g/L SC.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

The CIPAC method No. 625 is available for Mesotrione.

### Methods for the determination of residues (KCP 5.1.2)

Please refer to the post-registration method.

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of Mesotrione (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-4:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Mesotrione | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Plant, high acid content |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Plant, high protein/high starch content (dry commodities) |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Plant, high oil content |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.05 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Muscle | Mesotrione | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Milk |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Eggs |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Fat |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Liver, kidney |  | 0.01 mg/kg | Reg. (EU) 2017/626 Reg. (EU) 2024/1077 |
| Soil (Ecotoxicology) | Mesotrione and metabolite A (open) Mesotrione MNBA AMBA | 0.05 mg/kg 0.002 mg/kg 0.002 mg/kg 0.002 mg/kg | Common limit SANTE/2020/12830 rev. 2 EFSA Journal 2016;14(3):4419 |
| Drinking water (Human toxicology) | Mesotrione and metabolite A (open) MNBA AMBA | 0.1 0.05 µg/L 0.05 mg/L 0.05 mg/L | Common limit SANTE/2020/12830 rev. 2 EFSA Journal 2016;14(3):4419 |
| Surface water (Ecotoxicology) | Mesotrione | 7.7 µg/L | Lowest EC50 from aquatic toxicity study (Lemna gibba) EU agreed, EFSA Journal 2016;14(3):4419 |
| Air | Mesotrione | 1.5 µg/m3 | AOEL: 0.005 mg/kg bw/d Reg. (EU) 2017/725 |
| Tissue (meat or liver) | Mesotrione | 0.01 mg/kg | SANTE/2020/12830 rev. 2 |
| Body fluids |  | 0.01 mg/kg | SANTE/2020/12830 rev. 2 |

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in plant matrices is given in the following tables.

Table 5.3-5:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: Mesotrione |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013a EU agreed, EFSA Journal 2016;14(3):4419 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Tessier V., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| High acid content | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013a EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| High oil content | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013a EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| High protein/high starch content (dry) | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013a EU agreed, EFSA Journal 2016;14(3):4419 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Tessier V., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |

Table 5.3-6:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Not required, residues ≥ LOQ are not expected |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in animal matrices is given in the following tables.

Table 5.3-7:	Validated methods for food and feed of animal origin (if appropriate)

| Component of residue definition: Mesotrione(a) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Bernal J., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| Eggs | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Bernal J., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| Muscle | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| Fat | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |
| Kidney, liver | Primary | 0.01 mg/kg | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Bernal J., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory (if required) | - | - | Not required, the method is highly specific |

(a)No residue definition has been set for food and feed of animal origin.

Table 5.3-8:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Not required, residues ≥ LOQ are not expected |

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in soil is given in the following table.

Table 5.3-9:	Validated methods for soil (if appropriate)

| Component of residue definition: Mesotrione (and metabolite MNBA, AMBA) |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.002 mg/kg | LC-MS/MS | Jutsum L., Williams R.W., 2013 Jutsum L., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
| Confirmatory | - | - | Not required, the method is highly specific |

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in surface and drinking water is given in the following table.

Table 5.3-10:	Validated methods for water (if appropriate)

| Component of residue definition: Mesotrione (and metabolite MNBA, AMBA) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | 0.05 μg/L | LC-MS/MS | Jutsum L., Chamkesam N., 2013 Jutsum L., 2013a EU agreed, EFSA Journal 2016;14(3):4419 |
|  | ILV | 0.05 μg/L | LC-MS/MS | Wiesner F., Breyer N., 2013 EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory | - | - | Not required, the method is highly specific |
| Surface water | Primary | 0.05 μg/L | LC-MS/MS | Jutsum L., Chamkesam N., 2013 Jutsum L., 2013a EU agreed, EFSA Journal 2016;14(3):4419 |
|  | Confirmatory | - | - | Not required, the method is highly specific |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in air is given in the following table.

Table 5.3-11:	Validated methods for air (if appropriate)

| Component of residue definition: Mesotrione |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.45 μg/m3 | LC-MS/MS | Jutsum L., 2013b Jutsum L., 2013c EU agreed, EFSA Journal 2016;14(3):4419 |
| Confirmatory | - | - | Not required, the method is highly specific |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Mesotrione in body fluids and tissues is given in the following table.

Table 5.3-12:	Methods for body fluids and tissues (if appropriate)

| Component of residue definition: Mesotrione |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.01 mg/kg (in blood) | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
| Confirmatory | - | - | Not required, the method is highly specific |
| Primary | 0.01 mg/kg (in muscle) | LC-MS/MS | Watson G., 2013b EU agreed, EFSA Journal 2016;14(3):4419 |
| Confirmatory (if required) | - | - | Not required, the method is highly specific |

#### Other studies/ information

No other studies/ information.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1-1 | Rigamonti E. | 2023 | Mesotrione 100G/L SC: Validation of the Analytical Method for the Determination of Active Ingredient Content. Report No. CH-1206-2022 ChemService S.r.l. Controlli e Ricerche GLP Unpublished | N | Shandong Weifang Rainbow Chemical Co., Ltd. |
| KCP 5.1.1-2 | Rigamonti E. | 2023 | Mesotrione 100G/L SC: Validation of the Analytical Method for the Determination of the 1,2-Dichloroethane (1,2-DCE) as relevant impurity Content. Report No. CH-1207-2022 ChemService S.r.l. Controlli e Ricerche GLP Unpublished | N | Shandong Weifang Rainbow Chemical Co., Ltd. |
| KCP 5.1.1-2 | Rigamonti E. | 2023 | Mesotrione 100G/L SC: Validation of the Analytical Method for the Determination of R287431 and R287432 as Relevant Impurities Content. Report No. CH-1208-2022 ChemService S.r.l. Controlli e Ricerche GLP Unpublished | N | Shandong Weifang Rainbow Chemical Co., Ltd. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.2 | Watson, G. | 2013a | Mesotrione - Validation of the QuEChERS Method for the Determination of Residues of mesotrione in Crop Matrices by LC-MS/MS. Report No. S12-03251 Syngenta File No. ZA1296_10090 Eurofins Agroscience Services Ltd, Wilson, UK GLP Published | N | Syngenta |
| KCP 5.2 | Tessier, V. | 2013 | Mesotrione - Independent Laboratory Validation of the QuEChERS Method for the Determination of Residues of Mesotrione in Crop Matrices by LC-MS/MS. Report No. S12-04607 Syngenta File No. ZA1296_10129 Eurofins Agroscience Services Chem SAS, Vergèze, France GLP Published | N | Syngenta |
| KCP 5.2 | Watson, G. | 2013b | Mesotrione - Validation of the QuEChERS Method for the Determination of Residues of mesotrione in Animal Matrices by LC-MS/MS. Report No. S12-03250 Syngenta File No. ZA1296_10093 Eurofins Agroscience Services Ltd, Wilson, UK GLP Published | N | Syngenta |
| KCP 5.2 | Bernal, J. | 2013 | Bernal J., 2013 Mesotrione - Independent Laboratory Validation of the QuEChERS Method for the Determination of Residues of Mesotrione in Animal matrices by LC-MS/MS. Report No. S12-04608 Syngenta File No. ZA1296_10130 Eurofins Agroscience Services Chem SAS, Vergèze, France GLP Published | N | Syngenta |
| KCP 5.2 | Jutsum L., Williams R. | 2013 | Analytical Method GRM007.10A for the Determination of Mesotrione and its Metabolites AMBA and MNBA in Soil. Report No. GRM007.10A Syngenta File No. ZA1296_10092 Syngenta CEMAS, North Ascot, United Kingdom, GRM007.10A Not GLP Published | N | Syngenta |
| KCP 5.2 | Jutsum, L. | 2013 | Mesotrione – Validation of Draft Residue Method GRM007.10A for the Determination of Mesotrione and its Metabolites AMBA and MNBA in Soil. Report No. CEMR-5657-REG Syngenta File No. ZA1296_10088 CEMAS, North Ascot, United Kingdom GLP Published | N | Syngenta |
| KCP 5.2 | Jutsum L., Chamkesam N. | 2013 | Analytical Method GRM007.09A for the Determination of Mesotrione and its Metabolites AMBA and MNBA in Water. Report No. GRM007.09A Syngenta File No. ZA1296_10091 CEMAS, North Ascot, United Kingdom Not GLP Published | N | Syngenta |
| KCP 5.2 | Jutsum L. | 2013a | Validation of Draft Residue Method GRM007.09A for the Determination of Mesotrione and its metabolites AMBA and MNBA in Water. Report No. CEMR-5658-REG Syngenta File No. ZA1296_10087 CEMAS, North Ascot, United Kingdom GLP Published | N | Syngenta |
| KCP 5.2 | Wiesner F., Breyer N. | 2013 | Mesotrione - Independent Laboratory Validation of Analytical Method GRM007.09A for the Determination of Residues of Mesotrione and its Metabolites in AMBA and MNBA Water. Report No. S13-04185 Syngenta File No. ZA1296_10174 Eurofins Agroscience Services Chem GmbH, Hamburg, Germany GLP Published | N | Syngenta |
| KCP 5.2 | Jutsum L. | 2013b | Mesotrione - Residue Method GRM007.08B for the Determination of Mesotrione in Air. Report No. GRM007.08B Syngenta File No. ZA1296_10089 Syngenta CEMAS, North Ascot, United Kingdom Not GLP Published | N | Syngenta |
| KCP 5.2 | Jutsum L. | 2013c | Mesotrione - Validation of Residue Method GRM007.08A for the Determination of Mesotrione in Air. Report No. CEMR-5403-REG Syngenta File No. ZA1296_10084 Syngenta CEMAS, North Ascot, United Kingdom GLP Published | N | Syngenta |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for Mesotrione

    - Methods used for the generation of pre-authorization data (KCP 5.1)

No new or additional studies have been submitted.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted.

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted.