FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: SHA 5148 A Product name(s): PLANET Chemical active substance: 2,4-D, 275 g/L (in form of DMA salt), MCPA, 275 g/L (in form of DMA salt)

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Sharda Cropchem Limited Submission date: October 2022 Update date: January 2023; May 2023 MS Finalisation date: June 2023; May 2024

Version history

| When | What |
| --- | --- |
| October 2022 | Submission date |
| January 2023 | Applicant update |
| May 2023 | Applicant update |
| June 2023 | ZRMs evaluated dRR submitted by Applicant. |
| May 2024 | ZRMs prepared fRR after Applicant’s RT comments |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	2,4-D	5

1.2.3.2	MCPA	5

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	6

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	16

4	Section 4: Further information on the plant protection product	17

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	17

Appendix 1	Lists of data considered in support of the evaluation	20

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	23

A 2.1	2,4-D	23

A 2.2	MCPA	23

zRMS comments:

Submitted data are sufficient for evaluation. Shelf life – 2 years.

Packaging: preparation is to be packed in COEX PA/PE containers, which were used in storage studies and remained unaffected. Proposed packaging type is considered acceptable.

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name: 	Sharda Cropchem Ltd.

Address:		2nd floor, Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	Shardaint@vsnl.com

regn@shardaintl.com

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Name: 	Sharda Cropchem Ltd.

Address:		2nd floor, Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	Shardaint@vsnl.com

regn@shardaintl.com

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Name: 	Sharda Cropchem Ltd.

Address:		2nd floor, Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	Shardaint@vsnl.com

regn@shardaintl.com

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### 2,4-D

| 2,4-D | min. 960 g/kg (SANCO/11961/2014 Rev 5 and Regulation (EU) No. 540/2011) min. 985 g/kg (Sharda source) |
| --- | --- |

| Free phenols (expressed as 2,4-DCP) | max. 3 g/kg |
| --- | --- |
| Sum of dioxins and furans (WHO-TCDD TEQ) | max. 0.01 mg/kg |

#### MCPA

| MCPA | min. 960 930 g/kg (SANCO/4062/2001-final and Regulation (EU) No. 540/2011) min. 985 g/kg (Sharda source) |
| --- | --- |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | PLANET |
| --- | --- |
| Company code number: | SHA 5148 A 2,4-D 27.5% + MCPA 27.5% SL |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| 2,4-D | 275.0 g/L | 261.3 – 288.8 g/l (± 5%) | 279.2 g/L | 23.79 % w/w |
| MCPA | 275.0 g/L | 261.3 – 288.8 g/l (± 5%) | 279.2 g/L | 23.79 % w/w |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

** 	Based on the density of the formulation = 1.1738 g/ml

Table 1.4-2:	Relevant impurities

| Relevant impurity | Maximum content (g/L or g/kg) |
| --- | --- |
| Free phenols (expressed as 2,4-DCP) | 3 g/kg in the technical material 0.84 g/L in the formulation |
| Sum of dioxins and furans (WHO-TCDD TEQ) | 0.00001 g/kg in the technical material 0.0000027 g/L in the formulation |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-23:	Information on 2,4-D

| Type | Name/Code Number |
| --- | --- |
| ISO common name | 2,4-D |
| CAS No. | 94-75-7 |
| EC No. | 202-361-1 |
| CIPAC No. | 1 |

Table 1.4-34:	Information on MCPA

| Type | Name/Code Number |
| --- | --- |
| ISO common name | MCPA |
| CAS No. | 94-74-6 |
| EC No. | 202-360-6 |
| CIPAC No. | 2 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Soluble concentrate | [Code: SL] |
| --- | --- |

## Function (KCP 1.6)

2,4-D 27.5% + MCPA 27.5% SL (PLANET) is intended to be used as an herbicide.

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.  The appearance of the product is that of clear light yellow liquid, with a characteristic odour. The product is not flammable. In aqueous solution, it has a pH value around 6.8 at 20 °C. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0 °C and 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The results of the two years storage stability is being performed indicate the shelf life of 2 years. Its technical characteristics are acceptable for a SL formulation.

The intended concentration of use is 0.325% to 0.75% v/v

No tank mixes recommended

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

Neither classification nor labelling is relevant for this section.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No risk and safety phrases are relevant for this section.

Compliance with FAO specifications:

The product 2,4-D 27.5% + MCPA 27.5% SL complies with FAO specifications.

At the time of the evaluation, there is no FAO specification for the formulation with 2,4-D and MCPA.

Formulation used for tests

The formulation used for the tests has the same composition as the one cited in Part C, 2,4-D 27.5% + MCPA 27.5% SL (batch: SWEPL-42803, SCL-38251, SCL-46522).

Table 2-3:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | Clear light yellow liquid with characteristic odour. | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted |
| Explosive properties (KCP 2.2.1) | CHETAH software | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | The product may not be expected to have an explosive behaviour. | Y | Nichetti S., ChemService, Report CH-279/2016 | Accepted Explosive behaviour is not expected |
| Oxidizing properties (KCP 2.2.2) | EC A.21 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | The product does not show oxidizing properties. | Y | Mazzei, A., INNOVHUB, Report 1603399 | Accepted No spontaneous ignition was reported; the mean pressure rise time was higher than for the reference substance No oxidising properties |
| Flash point (KCP 2.3.1) | EC A.9 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | The product didn’t show the typical flammability phenomena until 130°C. It is not flammable. | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted Determined using a Setaflash closed cup apparatus according to ISO 3680 method Not flammable |
| Flammability (KCP 2.3.2) |  |  | Not applicable. |  |  | Not required |
| Self-heating (KCP 2.3.3) | EC A.15 | 2,4-D 275 g/L + MCPA 275 g/L SL (Batch No. SCL-38251) | Not required T=420℃ | Y | M. Michalec-Minch, 2019 Report No. 16/2019 | Accepted Not auto-flammable |
| Acidity or alkalinity and pH (KCP 2.4.1) | CIPAC MT 75.3 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | pH value (neat test item): 7.7 (20 ºC) Since the pH value ranged from 4 to 10, the acidity or alkalinity test was not performed. | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | pH value (1% aqueous dilution): 6.8 (20 ºC) | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted |
| Viscosity (KCP 2.5.1) | CIPAC MT 22.1 OECD 114 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | Kinematic viscosity: 16.31 cSt (mm2/s) at 20 ºC 8.08 cSt (mm2/s) at 40 ºC Dynamic viscosity: 19.14 cP (mPa*s) at 20 ºC 9.48 cP (mPa*s) at 40 ºC | Y | Nichetti S., ChemService, Report CH-278/2016 | Based on the composition, not considered an aspiration hazard. Accepted |
| Surface tension (KCP 2.5.2) | OECD 115 EC A.5 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | Undiluted product: 41.6 mN/m (20 ºC) 0.325 % v/v: 68.8 mN/m (20 ºC) 0.725 % v/v: 56.0 mN/m (20 ºC) The product is a surface-active material. | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted Determined at the highest in-use spray conc (0.75%). Considered surface-active |
| Relative density (KCP 2.6.1) | CIPAC MT 3.2 OECD 109 EC A.3 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | Density: 1.1738 g/mL at 20°C Relative density: 1.1738 | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted |
| Bulk density (KCP 2.6.2) |  |  | Not required. |  |  | Not required |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46 OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 CIPAC MT 75.3 CIPAC MT 47.2 CIPAC MT 18 CIPAC MT 41 HPLC-UV UV-Vis spectrophotometry GC/MS | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) 2,4-D 27.5% + MCPA 27.5% SL (SCL-46522) (Report No. HAL19-281 and 392/2019) | Appearance: Before storage: clear light yellow liquid with characteristic odour After storage: clear light yellow liquid with characteristic odour Stability of packaging: Before storage: COEX bottle with a screw cap. After storage: The container didn’t present any deformation in both bottom and lateral layers, or loss of sample and evident corrosion phenomena. pH (neat formulation): Before storage: 7.7 After storage: 7.8 pH (1% aqueous dilution): Before storage: 6.8 After storage: 6.9 Persistent foaming: Before storage: 0.325% v/v = 0 mL; 0.725% v/v = 0 mL. After storage: 0.325% v/v = 0 mL; 0.725% v/v = 0 mL. Dilution stability: Before storage: After 18 hours at 20 ± 2 ºC: no visual separation of solid or liquid material. After storage: After 18 hours at 20 ± 2 ºC: no visual separation of solid or liquid material. Determine active ingredient MCPA: Before storage: 23.3 % w/w; 273 g/L After storage: 23.1 % w/w; 272 g/L Determine active ingredient 2,4-D: Before storage: 23.5 % w/w; 275 g/L After storage: 23.4 % w/w; 274 g/L Free phenol relevant impurity content (as 2,4-DCP) Before storage: 2.099 mg/g After storage: 2.15 mg/g Dioxins and furans relevant impurities (as total TEQ equivalents): Before storage: 0.0133 μg/kg After storage: 0.0105 μg/kg The product is stable in its commercial packaging (COEX) under the tested accelerated storage conditions. | Y | Nichetti S., ChemService, Report CH-280/2016 (initial a.s. content) Nichetti S., ChemService, Report CH-281/2016 (results after accel. storage) N. Ordsmith, 2019 Report No. HAL19-281 (dioxins content, initial and after accel. testing) M. Pardo Martinez, 2019 Report No. 392/2019 (free phenols content, initial and after accel. testing) Nichetti S., ChemService, Report CH-278/2016 (initial appearance, pH and dilution stability) | Accepted Results demonstrate the stability of a.s. and formulation on storage. The change in a.s. contents: - MCPA: -0.86 % - 2,4-D: -0.43% A.s. contents were determined by HPLC/UV and impurities contents by GC/MS (for PCDD/F) and UV-Vis (for 2,4-DCP). The methods were validated according to SANCO/3030/99 rev. 5 (see Part B5 for details). Commercial packaging was used (COEX PA/PE bottle) and it remained unaltered. |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) |  |  | Not required. |  |  | Not required |
| Minimum content after heat stability testing (KCP 2.7.3) |  |  | Not required. |  |  | See KCP 2.7.1 |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | After 7 days at 0 ± 2 ºC: no visual separation of solid or liquid material. | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted Not affected by low temperature |
| Ambient temperature shelf life (KCP 2.7.5) | OPPTS 830.6302; OPPTS 830.6303; OPPTS 830.6304 CIPAC MT 75.3 CIPAC MT 18 CIPAC MT 41 HPLC-UV UV-Vis spectrophotometry GC/MS | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | On-going Test; Initial; After 6 months storage; After 12 months storage; After 24 months storage. MCPA content (active substance); 23.3 ± 0.2 % w/w 273 ± 2 g/L; 23.4 ± 0.2 % w/w 274 ± 2 g/L; 23.4 ± 0.2 % w/w 274 ± 2 g/L; 23.3 ± 0.2 % w/w 274 ± 2 g/L. 2,4-D content (active substance); 23.5 ± 0.2 % w/w 275 ± 2 g/L; 23.6 ± 0.2 % w/w 277 ± 2 g/L; 23.5 ± 0.2 % w/w 276 ± 2 g/L; 23.5 ± 0.2 % w/w 276 ± 2 g/L. Free Phenols content – as 2.4-DCP impurity (relevant impurity); 2099 ± 58 µg/g (<3042 µg/g); 2344 ± 55 µg/g (<3042 µg/g); 2097 ± 55 µg/g (<3042 µg/g; 1968 ± 21 µg/g (<3042 µg/g). Appearance (Colour, odour and physical state); Clear light yellow liquid with characteristic odour; Clear light yellow liquid with characteristic odour; Clear light yellow liquid with characteristic odour; Clear light yellow liquid with characteristic odour. pH value (neat test item); 7.7; 7.7; 8.0; 8.0. pH value (1% aqueous dilution); 6.8; 6.8; 6.8; 6.8. Dilution stability (Starndard water C & Standard water D); After 18 hours at 20 ± 2°C: no visual separation of solid or liquid material; After 18 hours at 20 ± 2°C: no visual separation of solid or liquid material; After 18 hours at 20 ± 2°C: no visual separation of solid or liquid material; After 18 hours at 20 ± 2°C: no visual separation of solid or liquid material. Packaging; COEX bottle – commercial packaging; COEX bottle – commercial packaging; COEX bottle– commercial packaging; COEX bottles (two bottles) – commercial packaging. Compability (resistance) of packaging material; -; The container didn’t present any deformation in both bottom and lateral layers, or loss of sample and evident corrosion phenomena; The container didn’t present any deformation in both bottom and lateral layers, or loss of sample and evident corrosion phenomena; The container didn’t present any deformation in both bottom and lateral layers, or loss of sample and evident corrosion phenomena. Weight variation (%); -; -0.03%; -0.06%; -0.13% -0.14% (2 bottles). From the obtained results it can be concluded that no significant change was found in the MCPA, 2,4-D (active ingredient contents) and free phenols (relevant impurity content) for the sample stored in the COEX bottle for 24 months of storage at ambient warehouse temperature, comparing the obtained results at the beginning of the storage stability, that it complies with the tolerance and it is in accordance with the declared value. From the above reported data, it can be concluded that the sample of 2,4-D Dimethylamine 275 g/L (as Acid) +MCPA Dimethylamine 275 g/L (as Acid) SL formulation sample is stable in its commercial packaging during 24 months storage at ambient warehouse temperature. | Y | Nichetti S., ChemService Report CH-282/2016 M. Pardo Martinez, ChemService Report CH-393/2019 | Accepted A.s. contents were determined by HPLC/UV and impurity (2,4-DCP) content by UV-Vis. The methods were validated according to SANCO/3030/99 rev. 5 (see Part B5 for details). The content of dioxins and furans was not determined after storage. The content of the a.s. was stable during the shelf-life storage and the content of dioxins and furans after accelerated storage was below the limit value (see KCP 2.7.1). Results demonstrate the stability of a.s. and formulation on storage. No decrease in the a.s. content after storage. Commercial packaging was used (COEX PA/PE bottle) and it remained unaltered, weight variation was negligible. |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) |  |  | Not required. |  |  | Not required |
| Wettability (KCP 2.8.1) |  |  | Not applicable. |  |  | Not required |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.2 and MT 18 (using Standard Water D) | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | Foam after 1 minute: 0.325 % v/v = 0 mL 0.725 % v/v = 0 mL | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted Determined at the min and max recommended concentrations |
| Suspensibility (KCP 2.8.3.1) |  |  | Not applicable. |  |  | Not required |
| Spontaneity of dispersion (KCP 2.8.3.2) |  |  | Not applicable. |  |  | Not required |
| Dispersion stability (KCP 2.8.3.3) |  |  | Not applicable. |  |  | Not required |
| Degree of dissolution and dilution stability (KCP 2.8.4) | CIPAC MT 41 and MT 18 (using Standard Water C and Standard Water D) | 2,4-D 27.5% + MCPA 27.5% SL (SWEPL-42803) | After 18 hours at 20 ± 2°C: no visual separation of solid or liquid material. | Y | Nichetti S., ChemService, Report CH-278/2016 | Accepted Conc tested: 5.26% v/v |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) |  |  | Not applicable. |  |  | Not required |
| Wet sieve test (KCP 2.8.5.1.2) |  |  | Not applicable. |  |  | Not required |
| Dust content (KCP 2.8.5.2.1) |  |  | Not applicable. |  |  | Not required |
| Particle size of dust (KCP 2.8.5.2.2) |  |  | Not applicable. |  |  | Not required |
| Attrition (KCP 2.8.5.3) |  |  | Not applicable. |  |  | Not required |
| Hardness and integrity (KCP 2.8.5.4) |  |  | Not applicable. |  |  | Not required |
| Emulsifiability (KCP 2.8.6.1) |  |  | Not applicable. |  |  | Not required |
| Emulsion stability (KCP 2.8.6.2) |  |  | Not applicable. |  |  | Not required |
| Re-emulsifiability (KCP 2.8.6.3) |  |  | Not applicable. |  |  | Not required |
| Flowability (KCP 2.8.7.1) |  |  | Not applicable. |  |  | Not required |
| Pourability (KCP 2.8.7.2) |  |  | Not applicable. |  |  | Not required |
| Dustability following accelerated storage (KCP 2.8.7.3) |  |  | Not applicable. |  |  | Not required |
| Physical compatibi-lity of tank mixes (KCP 2.9.1) |  |  | Not required. |  |  | Not required |
| Chemical compatibi-lity of tank mixes (KCP 2.9.2) |  |  | Not required. |  |  | Not required |
| Adhesion to seeds (KCP 2.10.1) |  |  | Not applicable. |  |  | Not required |
| Distribution to seed (KCP 2.10.2) |  |  | Not applicable. |  |  | Not required |
| Other/special studies: (KCP 2.11) |  |  | Not relevant. |  |  | Not required |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

Table 4.1-4:	Packaging information for 0.250 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (PE/PA) |
| Shape/size: | Round bottle / approx. 61 mm diameter x 138.8 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 0.500 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (PE/PA) |
| Shape/size: | Round bottle / approx. 69 mm diameter x 199.8 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 1 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (PE/PA) |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 239.5 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 5 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (PE/PA) |
| Shape/size: | jerrycan / approx. 136 mm x 192 mm x 285 mm |
| Opening: | 54.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 10 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (PE/PA) |
| Shape/size: | jerrycan / approx. 174 mm x 226 mm x 368 mm |
| Opening: | 54.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-6:	Packaging information for 20 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (PE/PA) |
| Shape/size: | jerrycan / approx. 245 mm x 294 mm x 400 mm |
| Opening: | 55.8 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

## Effectiveness of cleaning procedures

Experience in use of plant protection products based on 2,4-D and MCPA has not indicated any particular problems. Low levels of residues of PLANET (2,4-D 275 g/L + MCPA 275 g/L, SL) in the equipment are not expected to present any particular risk to crops to be treated from a tank that has previously been used for the product.

The efficacy of cleaning of the application equipment with regard to impacts on “other” crops can be estimated on the basis of the PSD Efficacy Guideline 302 (December 2001). As worst case, the following prerequisites were considered:

Application rate:		 1.5 L product/ha, (0.412 kg 2,4-D /ha + 0.412 kg MCPA/ha)

Tank volume:			 2000 L

Volume remaining in spray

lines and pump after spraying:	 20 L

Spray volume:	200 L/ha (lowest spray volume corresponding to the maximum concentration of PLANET in diluted spray)

Based on these prerequisites and in consideration of 3 rinses with each 200-400 L of water based on good agricultural cleaning procedures, 2,4-D and MCPA residues remaining in the tank after spraying will be diluted to the following levels:

| Cleaning step | Water volume [L] | Concentration of residues |  |  |
| --- | --- | --- | --- | --- |
|  |  | product [L PPP/L water] | 2,4-D [kg as/L] | MCPA [kg as/L] |
| Tank filling: Residues after spraying: | 2000 20 | 7.5 x 10-3 | 2.06 x 10-3 | 2.06 x 10-3 |
| 1st step: 1/10 dilution of residual spray volume: Residues after spraying: | 200 20 | 7.5 x 10-4 | 2.06 x 10-4 | 2.06 x 10-4 |
| 2nd step: 20% of tank volume added: Residues after spraying: | 400 20 | 3.75 x 10-5 | 1.03 x 10-5 | 1.03 x 10-5 |
| 3rd step: 20% of tank volume added: Residues after spraying: | 400 20 | 1.875 x 10-6 | 5.156 x 10-7 | 5.156 x 10-7 |
| Addition of fresh spray solution: Residues in the tank filling: | 2000 20 | 1.875 x 10-8 | 5.156 x 10-9 | 5.156 x 10-9 |

PPP = PLANET

Residues remaining in the last cleaning solution were calculated to be 0.5156 mg/L of 2,4-D resulting in residue concentration of 5.156 µg/L 2,4-D after refilling the tank with 2000 L of water for another spray work. Assuming a range of spray volumes of 200 – 400 L/ha applied to succeeding crops, residues of 1.031 – 2.062 mg of 2,4-D will be applied per ha. These residue levels should not present any effect on non-target plants and may be considered as acceptable.

Residues remaining in the last cleaning solution were calculated to be 0.5156mg/L of MCPA resulting in residue concentration of 5.156 µg/L MCPA after refilling the tank with 2000 L of water for another spray work. Assuming a range of spray volumes of 200 – 400 L/ha applied to succeeding crops, residues of 1.031 – 2.062 mg of MCPA will be applied per ha. These residue levels should not present any effect on non-target plants and may be considered as acceptable

zRMS comment: accepted

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1 KCP 2.3.1 KCP 2.4.1 KCP 2.4.2 KCP 2.5.1 KCP 2.5.2 KCP 2.6.1 KCP 2.7.4 KCP 2.8.2 KCP 2.8.4 | Nichetti, S. | 2016 | 2,4-D DIMETHYLAMINE 275 g/L (AS ACID) + MCPA DIMETHYLAMINE 275 g/L (AS ACID) SL: DETERMINATION OF THE PHYSICO-CHEMICAL PROPERTIES. ChemService S.r.l., CH-278/2016 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.2.1 | Nichetti, S. | 2016 | 2,4-D DIMETHYLAMINE 275 g/L (AS ACID) + MCPA DIMETHYLAMINE 275 g/L (AS ACID) SL: DETERMINATION OF THE OXIDIZING PROPERTIES AND EXPLOSIVE PROPERTIES. ChemService S.r.l., CH-279/2016 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.7 | Nichetti, S. | 2016 | 2,4-D DIMETHYLAMINE 275 g/L (AS ACID) + MCPA DIMETHYLAMINE 275 g/L (AS ACID) SL: VALIDATION OF THE ANALYTICAL METHOD FOR THE DETERMINATION OF THE ACTIVE INGREDIENTS CONTENT. ChemService S.r.l., CH-280/2016 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.7.1 | M. Pardo Martinez | 2019 | 2,4 D 275 g/L + MCPA 275 g/L SL: Determination of the Active Ingredient Content and the Accelerated Storage Stability (Free Phenols Relevant Impurity Content) ChemService S.r.l., CH-392/2019 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.7.1 | N. Ordsmith | 2019 | A stufy for the Analysis of 2,4-D (27.5%) MCPA (27.5%) for the presence of targeted polychlorinated Dibenzodioxins and Dibenzofurans with accelerated storage Hallanalytical, Report No. HAL19-281 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.7.1 | Nichetti, S. | 2016 | 2,4-D DIMETHYLAMINE 275 g/L (AS ACID) + MCPA DIMETHYLAMINE 275 g/L (AS ACID) SL: DETERMINATION OF THE ACCELERATED STORAGE STABILITY AND CORROSION CHARACTERISTICS. ChemService S.r.l., CH-281/2016 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.2.2 | Mazzei, A. | 2017 | Determination of Oxidizing properties (liquids) on the Sample 2,4-D Dimethylamine 275 g/L (as Acid) + MCPA Dimethylamine 275 g/L (as Acid) SL INNOVHUB, 1603399 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.3.3 | M. Michalec-Minch | 2019 | 2,4-D 275 g/L + MCPA 275 g/L SL Determination of flammability (self-heating) (EEC A.15 Auto-ignition temperature (liquids and gases) Report No. 16/2019 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.7.5 | Nichetti, S. | 2016 | 2,4-D Dimethylamine 275 g/L (as Acid) + MCPA Dimethylamine 275 g/L (as Acid) SL: Two Years Storage Stability and Corrosion Characteristics ChemService S.r.l., CH-282/2016 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 2.7.5 | M. Pardo Martinez | 2021 | 2,4 D 275 g/L + MCPA 275 g/L SL: Two Years Storage Stability (Free Phenols Relevant Impurity Content) ChemService S.r.l., CH-393/2019 GLP, Unpublished | N | Sharda Cropchem Ltd. |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Additional data on the physical, chemical and technical properties of the active substance

  - 2,4-D

No additional data on 2,4-D is provided.

  - MCPA

No additional data on MCPA is provided.