FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: SHA 5148 A Product name(s): PLANET (2,4-D 27.5% +MCPA 27.5% SL) Chemical active substances: 2,4-D, 275 g/L (in form of DMA salt), MCPA, 275 g/L (in form of DMA salt)

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Sharda Cropchem Ltd. Submission date: October 2022 Finalisation date: May 2023; May 2024

Version history

| When | What |
| --- | --- |
| 10.2022 | Submition date |
| 05.2023 | ZRMs evaluated dRR updated by Applicant |
| 05.2024 | ZRMs prepared fRR after Applicant’s RT comments |

Table of Contents

5	Analytical methods	5

5.1	Conclusion and summary of assessment	5

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	5

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	5

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	5

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	9

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	17

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	17

5.2.2	Methods for the determination of residues (KCP 5.1.2)	17

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	17

5.3.1	Analysis of the plant protection product (KCP 5.2)	17

5.3.2	Description of analytical methods for the determination of residues 2,4-D (KCP 5.2)	17

5.3.2.1	Overview of residue definitions and levels for which compliance is required	17

5.3.2.2	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	20

5.3.2.3	Description of methods for the analysis of soil (KCP 5.2)	22

5.3.2.4	Description of methods for the analysis of water (KCP 5.2)	22

5.3.2.5	Description of methods for the analysis of air (KCP 5.2)	23

5.3.2.6	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	23

5.3.2.7	Other studies/ information	24

5.3.3	Description of analytical methods for the determination of residues MCPA (KCP 5.2)	24

5.3.4	Description of analytical methods for the determination of residues MCPA (KCP 5.2)	24

5.3.4.1	Overview of residue definitions and levels for which compliance is required	24

5.3.4.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	25

5.3.4.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	26

5.3.4.4	Description of methods for the analysis of soil (KCP 5.2)	27

5.3.4.5	Description of methods for the analysis of water (KCP 5.2)	27

5.3.4.6	Description of methods for the analysis of air (KCP 5.2)	28

5.3.4.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	28

5.3.4.8	Other studies/ information	29

Appendix 1	Lists of data considered in support of the evaluation	30

Appendix 2	Detailed evaluation of submitted analytical methods	35

A 2.1	Analytical methods for 2,4-D	35

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	35

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	35

A 2.2	Analytical methods for MCPA	71

A 2.2.1	Methods used for the generation of pre-authorization data (KCP 5.1)	71

A 2.2.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	71

# Analytical methods

## Conclusion and summary of assessment

- The analytical methods (HPLC-UV, UV/Vis and GC/MS) for determination of active substances (2,4-D and MCPA) and relevant impurities (2,4-DCP and PCDD/F) in plant protection product (PLANET (2,4-D 27.5% +MCPA 27.5% SL)) has been submitted and validated. Submitted data are sufficient for evaluation and there are no data gaps.

Sufficiently sensitive and selective analytical methods are not available for all analytes included in the residue definitions.

Noticed data gaps are (MCPA):

- ILV for the determination of MCPA, MCPB including their salts, esters and conjugates expressed as MCPA in high protein/high starch content matrices

- ILV for the determination of MCPA, MCPB including their salts, esters and conjugates expressed as MCPA in animal matrices

- ILV for the determination of MCPA, MCPB including their salts, esters and conjugates expressed as MCPA in drinking water

- Analytical methods for the determination of residues in body fluids and tissues

These data gaps can be covered after renewal of MCPA.

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Dry commodities / Spring barley, Winter wheat, Winter barley | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of 2,4-D and MCPA in plant protection product is provided as follows:

| Comments of zRMS: | The analytical method (HPLC-UV) for determination of 2,4-D and MCPA in PLANET (2,4-D 27.5% +MCPA 27.5% SL) has been submitted. The method was validated and meets criteria of specificity, linearity and precision according to the requirements of SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1.1 |
| --- | --- |
| Report | 2,4-D DIMETHYLAMINE 275 g/L (AS ACID) + MCPA DIMETHYLAMINE 275 g/L (AS ACID) SL: VALIDATION OF THE ANALYTICAL METHOD FOR THE DETERMINATION OF THE ACTIVE INGREDIENTS CONTENT, Simona Nichetti, 2016, Report No. CH-280/2016 |
| Guideline(s): | SANCO/3030/99 rev. 4-5 Working document “Technical Material and Preparations: Guidance for generating and reporting methods of analysis in support of pre and post registration data requirements for Annex II (part A, Section 4) and Annex III (part A, Section 5) of Directive 91/414 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The determination of the active substances was performed by HPLC, using an external standard and UV detector at 228 nm.

A sample of 2,4-D Dimethylamine 275 g/L (as Acid) + MCPA Dimethylamine 275 g/L (as Acid) SL, contained in a COEX bottle with screw cap, was received by the Test Facility on July 20, 2016. Information on the test item is as follows:

Test item identification	            :	2,4-D Dimethylamine 275 g/L (as Acid) +

MCPA Dimethylamine 275 g/L (as Acid) SL

Nominal active ingredient	:	2,4-D 276 g/L + MCPA 276 g/L

Active ingredients content	:	2,4-D Dimethylamine 276 g/L (as Acid) +

MCPA Dimethylamine 276 g/L (as Acid) SL

(from the Certificate of Analysis)

Batch number			:	SWEPL-42803

Internal number			:	2929431-001

Manufacturing date		:	July 18, 2015

Expiry date			:	July 17, 2017

2,4-D dimethylamine salt

Common name			:	2,4-D dimethylamine salt

Chemical name (IUPAC)	:	2,4-Dichlorophenoxyacetic acid, dimethylamine salt

Chemical structure		:

Chemical formula		:	C10H13Cl2NO3

Chemical Class			:	Herbicide

CAS number			:	2008-39-1

Molecular weight		:	266.1

MCPA dimethylamine salt

Common name of the a.i.	:	MCPA dimethylamine salt

Chemical name (IUPAC)	:	Dimethylammonium 4-chloro-o-tolyloxyacetate

Chemical formula		:	C11H16ClNO3

Chemical structure		:

CAS number			:	2039-46-5

Chemical Class			:	Herbicide

Molecular weight		:	245.7 g/mol

Reference materials:

- 2,4-D, analytical standard, CAS 94-75-7, Purity 99.9% w/w, Batch no. SZBD352XV, Sigma-Aldrich

- MCPA, analytical standard, CAS 94-74-6, Purity 99.8% w/w, Batch no. SZBC129XV, Sigma-Aldrich

Reagents:

- Water, HPLC grade obtained from the Laboratory Water Purification System.

- Acetonitrile, HPLC grade (Sigma-Aldrich)

- Phosphoric acid 85% (H3PO4), reagent grade (Merck)

Equipment:

- HPLC Agilent mod. 1200 equipped with UV detector, autosampler

and Agilent software for data processing, Internal code No. 366

- HPLC Column, Internal code No. LCN 240

- Analytical balance, Mettler ML204, Internal code No. 417

- Freezer Liebherr, mod. G 5216, Internal code No. 746

- Refrigerator Fiocchetti, mod. Labor, Internal code No. 239

- Laboratory Water Purification Systems, Sartorius Arium® 611, Internal code No. 355

- Volumetric glassware: pipettes, flasks, measuring cylinders (Class A)

- Usual laboratory glassware

Solutions preparation:

- The stock MCPA reference material solution was prepared by dissolving MCPA in acetonitrile.

- The stock 2,4-D reference material solution was prepared by dissolving 2,4-D in acetonitrile.

- The working standard solutions were prepared by dissolving the stock standard solutions in acetonitrile.

- The test item solutions were prepared by dissolving the test item in acetonitrile.

Chromatographic conditions

HPLC Column			: 	Hypercarb 100 x 3.00 mm i.d., 5 µm

Detector			: 	UV/Vis operating at 228 nm

Column temperature		: 	25°C

Eluent B			: 	Acetonitrile

Eluent D			: 	Phosphoric acid at 1 %v/v

Isocratic			: 	B:D: 85:15

Eluent flow			: 	1.00 mL/min

Volume of injection		: 	10 µL

Retention time of MCPA	: 	about 4.6 minutes

Retention time of 2,4-D		: 	about 5.8 minutes

Total Analysis Time		: 	12 minutes

Validation - Results and discussions

Specificity:

A comparison of the chromatograms of the solvent wash (acetonitrile), MCPA and 2,4-D reference materials, MCPA and 2,4-D test substances, Placebo and test item solutions, shows that, following the operating conditions recommended in the analytical method, all the peaks are well separated and there is not evidence of interferences with the test item peaks.

Therefore, by using the conditions stated in the method, interferences can be avoided and the active ingredients can be reliably determined in 2,4-D Dimethylamine 275 g/L (as Acid) + MCPA Dimethylamine 275 g/L (as Acid) SL formulation samples.

Representative chromatograms are presented in the study report.

Linearity and System Precision:

To check linearity and system precision, five working standard solutions were prepared as previously described in the experimental section and each solution was injected.

The peak areas obtained from five consecutive injections of the middle working standard solution were then used to determine the mean value, the standard deviation (S.D.) and the relative standard deviation (%RSD) of the system precision.

The range tested for MCPA, from 35.09 µg/mL to 81.88 µg/mL ( 40 % of the concentration used for the quantification analysis), was found to be linear (correlation coefficient > 0.99).

The range tested for 2,4-D, from 36.20 µg/mL to 84.48 µg/mL ( 40 % of the concentration used for the quantification analysis), was found to be linear (correlation coefficient > 0.99).

Repeatability (Precision):

The relative standard deviation was 0.42% for MCPA and the Horwitz RSDr was 1.67 at a MCPA concentration of 23.3 % w/w. Since the relative standard deviation was lower than the Horwitz RSDr, the repeatability test for this active ingredient was acceptable.

The relative standard deviation was 0.38 % for 2,4-D and the Horwitz RSDr was 1.67 at a 2,4-D concentration of 23.5 % w/w. Since the relative standard deviation was lower than the Horwitz RSDr, the repeatability test for this active ingredient was acceptable.

The value of 2 g/L for the precision of the analytical method for MCPA, calculated as twice the standard deviation, can be considered acceptable for this test item with a declared nominal purity of 276 g/L.

The value of 2 g/L for the precision of the analytical method for 2,4-D, calculated as twice the standard deviation, can be considered acceptable for this test item with a declared nominal purity of 276 g/L.

Data and results were used to determine the following precision:

MCPA		:	23.3  0.2 % w/w		273   2 g/L

2,4-D		:	23.5  0.2 % w/w		275   2 g/L

In order to obtain the precision expressed in g/L, a 1.1738 g/mL density value, calculated in GLP Study CH – 278/2016, was used.

Recovery (Accuracy):

The test was performed by spiking the Placebo with the MCPA and 2,4-D test substances materials at three levels in duplicate, corresponding to additions of 75 %, 100 % and 125 % of the nominal concentration of active ingredients. Each fortified sample was analysed in duplicate.

Since all mean recovery values were in the correct range, these criteria were fulfilled and therefore accuracy of the analytical method can be considered acceptable.

Table ‎5.2-1:	Methods suitable for the determination of active substances 2,4-D and MCPA  in plant protection product 2,4-D Dimethylamine 275 g/L (as Acid) + MCPA Dimethylamine 275 g/L (as Acid) SL

|  | 2,4-D | MCPA |
| --- | --- | --- |
| Author(s), year | Simona Nichetti, 2016 | Simona Nichetti, 2016 |
| Principle of method | HPLC-UV | HPLC-UV |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | 5 concentration points from 36.20 to 84.48 µg/mL ( 40 % of the concentration used for the quantification analysis) Linear regression: y = 276964x - 581928 r > 0.99 | 5 concentration points from 35.09 to 81.88 µg/mL ( 40 % of the concentration used for the quantification analysis) Linear regression: y = 229214x - 530859 r > 0.99 |
| Precision – Repeatability Mean n = 6 (2 times at 3 fortification levels) (% RSD) | 23.5% w/w % RSD = 0.38 % RSDr = 1.67 Hr = 0.23, Hr ≤ 1 | 23.3% w/w % RSD = 0.42 % RSDr = 1.67 Hr = 0.25, Hr ≤ 1 |
| Accuracy n = 6 (2 times at 3 fortification levels, corresponding to nominal addition of 75%, 100% and 125%)) (% Recovery) | 99.6% 100.0% 100.4% % Marginal Total mean recovery = 100.0 | 99.6% 100.0% 101.1% % Marginal Total mean recovery = 100.2 |
| Interference/ Specificity | No interference, the method is specific (according to blank samples chromatogram) | No interference, the method is specific (according to blank samples chromatogram) |
| Comment | - | - |

Conclusion

According to SANCO/3030/99 rev. 5 the analytical method was shown to be specific for each active ingredient in the 2,4-D Dimethylamine 275 g/L (as Acid) + MCPA Dimethylamine 275 g/L (as Acid) SL formulation samples.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The analytical method (UV/Vis spectrophotometry) for determination of 2,4-DCP in PLANET (2,4-D 27.5% +MCPA 27.5% SL) has been submitted. The method was validated and meets criteria of specificity, linearity and precision according to the requirements of SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1.2 |
| --- | --- |
| Report | 2,4 D 275 g/L + MCPA 275 g/L SL: Validation of the Analytical Method for the Determination of Free Phenols as Relevant Impurity Content, Mercedes Pardo Martinez, 2019, Report No. CH-391/2019 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The determination of 2,4-Dichlorophenol (2,4-DCP) was performed by external standard analysis using an UV/Vis spectrophotometer at 520 nm following the CIPAC method MT 69.1.

A sample of 2,4 D 275 g/L + MCPA 275 g/L SL, contained in a COEX bottle with screw cap, was received by the Test Facility on May 13, 2019. Information on the test item is as follows:

Test item identification	             :	2,4 D 275 g/L + MCPA 275 g/L SL

Nominal active ingredient	:	2,4-D 276 g/L + MCPA 276 g/L

Active ingredients content	:	276 g/L (expressed as 2,4 D acid) +

276 g/L (expressed as MCPA acid) SL

(from the Certificate of Analysis)

Batch number			:	SCL-46522

Internal number			:	1902794-002

Manufacturing date		:	January 20, 2019

Expiry date			:	January 19, 2021

Reference material:

- 2,4-Dichlorophenol (2,4-DCP), analytical standard, CAS 120-83-2, Purity 99.8%, Batch no. BCBS2340V, Sigma-Aldrich

Reagents:

- Water, HPLC grade, obtained by the Lab Water Purification System

- Acetone, pesticide residue analysis, Honeywell

- Ethanol, analytical grade, absolute alcohol, VWR Chemicals

- Ammonia solution 25%, reagent grade, Merck

- 4-Aminoantipyrine, ≥98.0% reagent grade, Fluka

- Potassium ferricyanide (III), reagent grade, Sigma-Aldrich

- 2,4-D, analytical standard, Sigma-Aldrich

Equipment:

- UV/Vis spectrometer, mod. HP 8453 equipped with polyethylene (10 mm) cells, managed by Chemstation software, Agilent

- Analytical balance, mod. Pioneer PA214C, OHAUS

- Laboratory Water Purification Systems, mod. Arium® 611, Sartorius

- Freezer, mod. G 5216, Liebherr

- Refrigerator, mod. Labor Lux 1000, Fiocchetti

- Falcon plastic test tubes (15 mL), VWR

- Usual laboratory glassware, Various

- Volumetric glassware: pipettes, flasks, cylinders (Class A), Various

Solutions preparation:

- The stock 2,4-DCP reference material solution (115.8 µg/mL) was prepared by adding 1 mL acetone and 11.6 mg reference material up to total volume of 100 mL with water.

- The working standard solutions were prepared by diluting the stock 2,4-DCP reference material solution to final volume with the 2,4-D stock solution (at about 500 µg/mL).

- The test item solutions were prepared by weighting about 400 mg of the test item into a 50.00 mL volumetric flask and dissolving to volume with water.

Reaction procedure:

Using a volumetric pipette, an aliquot of 5.00 mL of each working standard solution were transferred into a 15 mL falcon test tube, 2.50 mL of the 0.05 N ammonia solution, 2.50 mL of 4-Aminoantipyrine 0.2% w/v solution and 2.50 mL of potassium ferricyanide (III) 0.4% w/v solution were added shaking after each addition.

Reagents must be added in the exact order described above.

The final solution is shacked for 1 minute and after 5 minutes of waiting for colorimetric reaction the absorbance at 520 nm was recorded.

The absorbance at 520 nm is determined by a spectrophotometer in a 10 mm polyethylene cell.

For the blank reagents, it is used an aliquot of 5.00 mL of 2,4-D stock solution, without of 2,4-DCP reference material solution addition.

After the measurements of the working standard solutions, from the lowest to the highest concentration, a blank reagent was also measured in order to verify if memory signal is detected.

To test the system precision, the middle working standard solution (WSS 3) was then measured five times calculating the mean value, the standard deviation and the relative standard deviation.

Validation - Results and discussions

Specificity:

The method was specific for the determination of free phenols impurity content, expressed as 2,4-Dichlorophenol (2,4-DCP), in 2,4 D 275 g/L + MCPA 275 g/L SL sample by virtue of the absorption at 520 nm after colorimetric reaction.

A comparison of the absorption at 520 nm of the following samples presented in the report shows that, following the operating conditions recommended in the analytical method, the impurity is well detected and there is no evidence of interferences with the test item peak.

Confirmatory:

The identity of the free phenols impurity, a mixture of all possible phenols, which are quantified and expressed respect to 2,4-Dichlorophenol, is confirmed by the recovery results of and by the specificity of the CIPAC method MT 69.1.

Linearity:

Five working standard solutions were prepared as described in the Experimental section and each solution was analysed by UV spectrophotometer at 520 nm.

The absorbances obtained from five consecutive measurements of the middle working standard solution were then used to determine the mean value, the standard deviation and the relative standard deviation (RSD%) of the system precision.

The measurement range and the linearity range for the impurity are detailed in the table 5.2-2.

No significant memory absorbance was detected in the blank reagents’ measurement after the highest working standard solution and the range tested for free phenols impurity, expressed as 2,4-DCP, was found to be linear (correlation coefficient > 0.99).

Limit of quantification (LOQ) and Limit of detection (LOD):

Data from Linearity test were used to calculate the LOD whereas data from recovery test were used to calculate the LOQ.

The limit of detection (LOD), defined as half the lowest working standard solution, was a final measurement solution of 0.50 μg/mL, corresponding to 62.5 μg/g for the relevant impurity in the test item.

The limit of quantification (LOQ), defined as the lowest fortification level, was a final measurement solution of 2.00 μg/mL for relevant impurity, corresponding to 250 μg/g in the test item.

Impurity results calculated as < 62.5 μg/g (L.O.D.) are classified as not detected (n.d.).

Impurity results calculated as greater than the limit of detection but less than the limit of quantification, are designated as < 250 μg/g.

If an impurity content is calculated as greater than 1809 μg/g in the test item, the test item solution must be suitably diluted using volumetric glassware.

Precision:

The precision test was performed by six determinations of the test item.

From data obtained, the RSDr resulted to be greater than RSD% for the impurity and the Horwitz ratio Hr ≤ 1; therefore, precision of the analytical method is considered acceptable.

Recovery:

The accuracy test was performed by spiking the test item six times at two fortification levels.

For the accuracy, the SANCO/3030/99 rev. 5 guideline requires mean recovery values:

- in the range 80 to 120 % for impurity content between 0.1 % and 1.0 % w/w (1000 μg/g and 10000 μg/g).

Since all recovery values for the impurity resulted to be in the correct ranges, these criteria were fulfilled and therefore recovery of the analytical method can be considered acceptable.

Table ‎5.2-2:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) SHA 5148 A / PLANET

|  | 2,4-Dichlorophenol (2,4-DCP) in PPP |
| --- | --- |
| Author(s), year | Mercedes Pardo Martinez, 2019 |
| Principle of method | External standard analysis using an UV/Vis spectrophotometer, confirmed by the CIPAC method MT 69.1 |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | 5 concentration points from 1.16 to 28.94 µg/mL (145 – 3618 μg/g) Linear regression: y = 0.03362x – 0.02480 r > 0.99 |
| Precision – Repeatability Mean n = 6 (% RSD) | 2099 μg/g % RSD = 2.8 % RSDr = 3.39 Hr = 0.83, Hr ≤ 1 |
| Accuracy n = 12 (6 times at 2 fortification levels) (% Recovery) | Low level: (252 μg/g) %Recovery = 94 High level: (2520 μg/g) %Recovery = 99 % Marginal mean recovery = 96.5 |
| Interference/ Specificity | No interference, the method is specific according to blank sample chromatogram |
| LOQ n=6 | LOD = 62.5 μg/g LOQ = 250 μg/g (%RSD 0.4%; Hr = 0.84 < 1) |
| Comment | - |

Conclusion

According to SANCO/3030/99 rev. 5 the analytical method was shown to be specific for determining the relevant impurities (free phenols expressed as 2,4-DCP) content in the test item 2,4-D 275 g/L + MCPA 275 g/L SL.

| Comments of zRMS: | The analytical method (GC/MS) for determination of PCDD/F in PLANET (2,4-D 27.5% +MCPA 27.5% SL) has been submitted. The method was validated and meets criteria of specificity, linearity and precision according to the requirements of SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.1.3 |
| --- | --- |
| Report | A Study for the Analysis of 2,4-D (27.5%) MCPA (27.5%) for the Presence of Targeted Polychlorinated Dibenzodioxins and Dibenzofurans with Accelerated Storage, Nick Ordsmith, 2019, Report No. HAL19-281 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The procedure for sample preparation and analysis of the 17 targeted Dioxin/Furan congeners (2,3,7,8 containing tetra to octa chlorine) is based on USEPA method 1613 for environmental samples. Preparation of sample material for analysis did not require an extraction as solutions of the samples were made prior to chromatographic clean-up.

Determination of chlorinated Dibenzodioxin and Dibenzofuran compounds was by High Resolution GC-MS using an isotope dilution technique to quantify the 17 target analytes. The resulting data were processed through quantitation software against calibration standards. These results have been compared to accepted criteria.

Test item identification	             :	2,4 D 275 g/L + MCPA 275 g/L SL

Nominal active ingredient	:	2,4-D 276 g/L + MCPA 276 g/L

Active ingredients content	:	276 g/L (expressed as 2,4 D acid) +

276 g/L (expressed as MCPA acid) SL

(from the Certificate of Analysis)

Batch number			:	SCL-46522

Internal number			:	1902794-002

Manufacturing date		:	January 20, 2019

Expiry date			:	January 19, 2021

Control Substances (Analytical Standards) Dioxins/Furans:

- Precision and Recovery Standard EPA-1613 PAR, Batch no. 13PAR1115, Wellington Laboratories, Ontario, Canada

- Labelled Compound Stock Solution EPA-1613 LCS, Batch no. 13LCS0918, Wellington Laboratories, Ontario, Canada

- Internal Standard Spiking Solution EPA-1613ISS, Batch no. 13ISS0918, Wellington Laboratories, Ontario, Canada

Control Substances (Calibration Standards) Dioxins/Furans:

- Method 1613 Calibration Solutions (CS1-CS5) EPA-1613CVS, Batch no. 13CS10114, 13CS20114, 13CS30114, 13CS40114, 13CS50114, Wellington Laboratories, Ontario, Canada

At various stages in the procedure, specific standard compounds were added to the sample as part of the analytical method.

Sample preparation:

Approximately 1 g of 2,4-D 27.5% and MCPA 27.5% SL was weighed accurately.

A 2.5 ml aliquot of 1613 LCS standard (containing 13C labelled compounds only and diluted 1:250 from supplied concentration) was used to spike one sample of 2,4-D 27.5% and MCPA 27.5% SL prior to being made up in solution - unspiked sample.

A 50 µL aliquot of 1613 PAR standard and 2.5 mL of diluted 1613 LCS standard (containing 13C labelled compounds only) were used to spike a second sample of 2,4-D 27.5% and MCPA 27.5% SL prior to being made up in solution - spiked sample. This resulted in a sample spiked with native analytes at 2 ppb for tetra congeners; 10 ppb for penta - hepta congeners and 20 ppb for octa congeners, all with respect to active ingredient.

A method blank was also prepared that contained only 13C labelled compounds.

The blank and sample solutions were then cleaned up according to the in-house Hall Analytical UKAS accredited method.

Sample Clean up Procedure:

Glass Supelco columns (one per solution) with a fritted bottom were packed with 1 g ± 0.1 g of Florisil. Each column was washed (after attaching the reservoir, seals and washers) with 10 mL (± 1 mL) of DCM (dichloromethane). The reservoir was then dismantled and the columns activated by heating at 130°C ±5°C for a minimum of 16 hours.

Supelco dioxin sample preparation kits were assembled (one per solution) and each multi-layered silica column was pre-eluted (conditioned) with 50ml (±5ml) of hexane using a glass syringe, checked visually for leakage at connections and adjusted as required. The eluate was discarded.

A Florisil column was attached to each conditioned multi-layered silica column and mounted on the vacuum chamber.

The sample solutions were prepared in 22 mL vials (1 g sample in 10 mL toluene) and quantitatively transferred to Zymark tubes prior to reduction under nitrogen to a volume not less than 0.5 mL. Each concentrated solution was then quantitatively transferred to the top of an appropriately labelled silica column.

Each Zymark tube was then rinsed with 5 mL of hexane, sonicated and mixed with a Pasteur pipette for 10 - 15 seconds with these rinses added to the appropriate column. This step was repeated a further 4 times allowing each 5 mL to elute before adding the next and not allowing the column to dry out. A solvent reservoir containing 150 mL ± 2 mL of hexane was attached to elute the columns, applying a light vacuum if required ensuring there were no leaks at the reservoir connections. The eluate was discarded.

Each silica column was removed and replaced with an empty one to enable the Florisil column to be eluted with 50 mL ± 1 mL of DCM which was subsequently collected in individually labelled Zymark tubes.

The blank and sample extracts were concentrated to ~0.5 mL at 40°C ± 5°C under nitrogen, the Zymark tubes rinsed with 5ml ±1ml of hexane and concentrated again to ~0.5 mL. This was repeated a further 4 times, with the final extract evaporated to dryness.

Preparation of Cleaned-Up Sample for Analysis by GC-MS:

A 30 µL aliquot of 1613ISS Internal Standard Spiking Solution (diluted 1:5 from supplied concentration) was added to each of the dry residues to allow 13C labelled internal standard recovery to be measured.

The mixtures were sonicated for at least 15 seconds with repeated aspirating and expelling with a Pasteur pipette and transferred to labelled 300 µL clear glass vials. The vial was capped with a crimp seal autosampler cap and stored in the fridge at 5°C ± 3°C until analysis.

Chromatographic and MS conditions

GC Column			: 	DB-5MS 60m x 0.25mm x 0.25µm

Detector			: 	MS at 8000 resolution

Source temperature		: 	230°C

Eluent flow			: 	1.0 mL/min He

Oven Parameters		:	140°C, 4 min hold, 8°C/min to 220°C, 1.4°C/min to 260°C,

4°C/min to 310°C, hold 3 min

Injection temperature 		:	280°C

Volume of injection		: 	1 µL

Selected ion recording		:	2 ions per analyte

Validation - Results and discussions

Specificity:

Specificity was demonstrated by comparing the native analyte and 13C labelled internal standard chromatograms of an unspiked and spiked sample extract. The chromatograms show, for each chlorination level tetra to octa, the native analytes (upper two traces) and the 13C labelled internal standards (lower two traces).

The data demonstrated there are no interfering peaks and the assay method was specific and fit for purpose.

Linearity:

A set of commercially available standards at 5 concentrations was run with single injections. The calibration standards cover the concentration range 0.5 to 200 pg/µL for tetra congeners; 2.5 to 1000 pg/µL for penta - hepta congeners and 5 to 2000 pg/µL for octa congeners. This represents concentrations of 0.015 to 6 ppb for tetra congeners; 0.075 to 30 ppb for penta - hepta congeners and 0.15 to 60 ppb for octa congeners with respect to active ingredient.

The calibration uses the RRF method (as specified in the USEPA method 1613) with values calculated using the following equation:

RRF = (Response analyte / Response Internal Standard) x (Concentration Internal Standard / Concentration Analyte)

The % RSDs of the RRFs for each target analyte at the 5 concentrations from the calibration data were calculated and shown to be <35% (as specified in USEPA method 1613).

Although the calibration does not use the traditional linear regression method, curves can be generated and used for illustration.

Precision (Repeatability):

5 replicate injections of spiked batch SCL-46522 were made and the %RSD (coefficient of variation) of each target analyte concentration was calculated. This shows %RSD values over the 5 replicates of <5%.

For the blank prepared and analysed alongside the samples, the values reported are total ng, as there is no sample weight to take into account.

Accuracy:

The 13C labelled compounds in the EPA-1613 LCS solution added to the spiked sample prior to preparation were quantified against the EPA-16131SS standard. These results were used to determine the % recovery of each compound in the batch. Accuracy was based on these recoveries. The mean ranges from 80- 97%, within the specification of USEPA method 1613 QC limits.

Limit of quantification (LOQ):

The LOQ for the target impurities in each batch was measured using the data from the spiked sample. The LOQ is calculated by the instrument data system and is based on 8:1 signal to noise in the chromatogram for that specific analyte in that specific sample extract, therefore an accurate assessment on a sample by sample basis is determined, rather than applying a single value derived from a blank to all data. The data demonstrates limits in the ppt range.

The Toxic Equivalents Scheme (TEFs and TEQs)

Dioxins and Furans have several different congeners, each having its own degree of toxicity. To express the overall toxicity as a single value, the International Toxic Equivalents (TEQ) scheme has been developed. The procedure involves assigning individual toxicity equivalency factors (TEFs) to the Dioxin and Furan congeners in terms of their relative toxicity to 2,3,7,8-TCDD. To calculate the TEQ of a compound, the concentration of the toxic compound is multiplied by its TEF - using values accepted by the World Health Organisation (WHO).

Table ‎5.2-3:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) SHA 5148 A / PLANET

|  | Dioxins and Furans in PPP |
| --- | --- |
| Author(s), year | Nick Ordsmith, 2019 |
| Principle of method | Analysis of the 17 target congeners (2,3,7,8 containing tetra to octa chlorine) is based on USEPA method 1613 using high resolution GC/MS with an isotope dilution technique |
| Linearity (linear between mg/L) The calibration uses the RRF method RRF RSD% | 5 concentration points 0.5 to 200 pg/µL for tetra congeners (0.015 to 6 ppb) 2.5 to 1000 pg/µL for penta - hepta congeners (0.075 to 30 ppb) 5 to 2000 pg/µL for octa congeners (0.15 to 60 ppb) 2,3,7,8-TCDF: % RSD = 6.7 1,2,3,7,8-PeCDF: % RSD = 4.3 2,3,4,7,8-PeCDF: % RSD = 8.5 1,2,3,4,7,8-HxCDF: % RSD = 5.7 1,2,3,6,7,8-HxCDF: % RSD = 8.8 2,3,4,6,7,8-HxCDF: % RSD = 7.8 1,2,3,7,8,9-HxCDF: % RSD = 5.3 1,2,3,4,6,7,8-HpCDF: % RSD = 8.4 1,2,3,4,7,8,9-HpCDF: % RSD = 9.2 OCDF: % RSD = 9.3 2,3,7,8-TCDD: % RSD = 5.8 1,2,3,7,8-PeCDD: % RSD = 6 1,2,3,4,7,8-HxCDD: % RSD = 9.6 1,2,3,6,7,8-HxCDD: % RSD = 6.4 1,2,3,7,8,9-HxCDD: % RSD = 5.8 1,2,3,4,6,7,8-HpCDD: % RSD = 7.7 OCDD: % RSD = 9.9 |
| Precision – Repeatability Mean n = 5 (% RSD) | 2,3,7,8-TCDF: 1.6 ppb; % RSD = 1.1 1,2,3,7,8-PeCDF: 7.6 ppb; % RSD = 0.8 2,3,4,7,8-PeCDF: 7.7 ppb; % RSD = 1.3 1,2,3,4,7,8-HxCDF: 7.5 ppb; % RSD = 0.6 1,2,3,6,7,8-HxCDF:7.4 ppb; % RSD = 0.6 2,3,4,6,7,8-HxCDF:7.5 ppb; % RSD = 0.4 1,2,3,7,8,9-HxCDF: 7.4 ppb; % RSD = 1.1 1,2,3,4,6,7,8-HpCDF: 6.9 ppb; % RSD = 0.6 1,2,3,4,7,8,9-HpCDF:7.1 ppb; % RSD = 1.2 OCDF: 15.8 ppb; % RSD =0.7 2,3,7,8-TCDD: 1.4 ppb; % RSD = 4.3 1,2,3,7,8-PeCDD : 7.4 ppb; % RSD = 2.1 1,2,3,4,7,8-HxCDD: 7.3 ppb; % RSD = 1.3 1,2,3,6,7,8-HxCDD: 7.7 ppb; % RSD = 2.5 1,2,3,7,8,9-HxCDD: 7.8 ppb; % RSD = 2.3 1,2,3,4,6,7,8-HpCDD: 7.3 ppb; % RSD = 1.3 OCDD: 14.8 ppb; % RSD = 1.8 Horrat ratio < 1 |
| Accuracy n = 5 (% Recovery) | 13C12-2,3,7,8-TCDF: % Mean Recovery = 81 13C12-1,2,3,7,8-PeCDF: % Mean Recovery = 90 13C12-2,3,4,7,8-PeCDF: % Mean Recovery = 96 13C12-1,2,3,4,7,8-HxCDF: % Mean Recovery = 83 13C12-1,2,3,6,7,8-HxCDF: % Mean Recovery = 80 13C12-2,3,4,6,7,8-HxCDF: % Mean Recovery = 82 13C12-1,2,3,7,8,9-HxCDF: % Mean Recovery = 82 13C12-1,2,3,4,6,7,8-HpCDF: % Mean Recovery = 81 13C12-1,2,3,4,7,8,9-HpCDF: % Mean Recovery = 83 13C12-2,3,7,8-TCDD: % Mean Recovery = 88 13C12-1,2,3,7,8-PeCDD: % Mean Recovery = 97 13C12-1,2,3,4,7,8-HxCDD: % Mean Recovery = 84 13C12-1,2,3,6,7,8-HxCDD: % Mean Recovery = 82 13C12-1,2,3,4,6,7,8-HpCDD: % Mean Recovery = 85 13C12-OCDD: % Mean Recovery = 85 |
| Interference/ Specificity | No interference, the method is specific |
| LOQ | 2,3,7,8-TCDF: LOQ = 0.0020 ppt 1,2,3,7,8-PeCDF: LOQ = 0.0050 ppt 2,3,4,7,8-PeCDF: LOQ = 0.0044 ppt 1,2,3,4,7,8-HxCDF: LOQ = 0.0074 ppt 1,2,3,6,7,8-HxCDF: LOQ = 0.0073 ppt 2,3,4,6,7,8-HxCDF: LOQ = 0.0072 ppt 1,2,3,7,8,9-HxCDF: LOQ = 0.0087 ppt 1,2,3,4,6,7,8-HpCDF: LOQ = 0.0054 ppt 1,2,3,4,7,8,9-HpCDF: LOQ = 0.0065 ppt OCDF: LOQ = 0.0045 ppt 2,3,7,8-TCDD: LOQ = 0.0021 ppt 1,2,3,7,8-PeCDD: LOQ = 0.0055 ppt 1,2,3,4,7,8-HxCDD: LOQ = 0.0066 ppt 1,2,3,6,7,8-HxCDD: LOQ = 0.0067 ppt 1,2,3,7,8,9-HxCDD: LOQ = 0.0067 ppt 1,2,3,4,6,7,8-HpCDD: LOQ = 0.0046 ppt OCDD: LOQ = 0.0047 ppt |
| Comment | - |

Conclusion

According to SANCO/3030/99 rev. 5 the analytical method was shown to be specific for determining the relevant impurities (Dioxins and Furans) content in the test item 2,4-D 275 g/L + MCPA 275 g/L SL.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant, not necessary.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

A CIPAC method No. 1 is available for 2,4-D.

A CIPAC method No. 2 is available for MCPA.

### Methods for the determination of residues (KCP 5.1.2)

Please refer to the post-registration methods.

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues 2,4-D (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table ‎5.3-3:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | 2,4-D (sum of 2,4-D, its salts, its esters and its conjugates, expressed as 2,4-D) | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Plant, high acid content |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Plant, high protein/high starch content (dry commodities) |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Plant, high oil content |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.1 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Muscle | 2,4-D (sum of 2,4-D, its salts, its esters and its conjugates, expressed as 2,4-D) | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Milk |  | 0.01 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Eggs |  | 0.01 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Fat |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Liver, kidney |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2022/1363 |
| Soil (Ecotoxicology) | 2,4-D | 0.05 mg/kg | Common limit |
| Drinking water (Human toxicology) | 2,4-D | 0.1 µg/L | General limit for drinking water |
| Surface water (Ecotoxicology) | 2,4-D | 0.011 mg/L | Myriophyllum spicatum, EC50 EFSA Journal 2014;12(9):3812 |
| Air | 2,4-D | 6 µg/m3 | AOEL sys/AOEL inhal: 0.02 mg/kg bw/d Reg. (EU) 2015/2033 |
| Tissue (meat or liver) | - | not required | Not classified as T / T+ |
| Body fluids |  | not required | Not classified as T / T+ |

Table ‎5.3-4:	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of 2,4-D in plant matrices is given in the following table. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-5:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: 2,4-D (sum of 2,4-D, its salts, its esters and its conjugates, expressed as 2,4-D) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.1 Ravi Kumar C.R., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| High acid content | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| High oil content (soybean) | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.1 Ravi Kumar C.R., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| High protein/high starch content (dry) | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.1 Ravi Kumar C.R., 2020 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | KCP 5.2.1.1 Zarebska M., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

Table ‎5.3-6:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Not required as residues ≥ LOQ are not expected. |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of 2,4-D in animal matrices is given in the following table. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-7:	Validated methods for food and feed of animal origin (if appropriate)

| Component of residue definition: 2,4-D (sum of 2,4-D, its salts, its esters and its conjugates, expressed as 2,4-D) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2 Gundoju S., 2019 ammended KCP 5.2.2.1 Ravi Kumar C.R., 2020 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2.2 Zarebska M., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Eggs | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2 Gundoju S., 2019 ammended KCP 5.2.2.1 Ravi Kumar C.R., 2020 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2.2 Zarebska M., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Muscle | Primary | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2 Gundoju S., 2019 ammended KCP 5.2.2.1 Ravi Kumar C.R., 2020 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2.2 Zarebska M., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Fat | Primary (beef) | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2 Gundoju S., 2019 ammended KCP 5.2.2.1 Ravi Kumar C.R., 2020 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2.2 Zarebska M., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Kidney, liver | Primary (beef) | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.01 mg/kg | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2 Gundoju S., 2019 ammended KCP 5.2.2.1 Ravi Kumar C.R., 2020 |
|  | ILV | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2.3 Zarebska M., 2020 |
|  | Confirmatory (if required) | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

Table ‎5.3-8:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Not required as residues ≥ LOQ are not expected. |

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of 2,4-D in soil is given in the following table. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-9:	Validated methods for soil (if appropriate)

| Component of residue definition: 2,4-D |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.05 mg/kg | LC-MS/MS GC-MS (for 2,4-DCA) | EFSA Journal 2014;12(9):3812, EU agreed |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Primary | 0.05 mg/kg | LC-MS/MS (for 2,4-D, 2,4-DCP, 4-CP) GC-MS/MS (for 2,4-DCA) | KCP 5.2.3 Gundoju S., 2020 KCP 5.2.4 Ravi Kumar C.R., 2021 |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of 2,4-D in surface and drinking water is given in the following table. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-10:	Validated methods for water (if appropriate)

| Component of residue definition: 2,4-D |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking/Ground water | Primary | 0.1 µg/L | LC-MS/MS GC-MS (for 2,4-DCA) | EFSA Journal 2014;12(9):3812, EU agreed |
|  | ILV | 0.1 µg/L | LC-MS/MS GC-MS (for 2,4-DCA) | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.1 µg/L | LC-MS/MS (for 2,4-D, 2,4-DCP, 4-CP, 2,4-DCA) | KCP 5.2.5 Ayyavoo K., 2021 KCP 5.2.6 Ayyavoo K., 2022 KCP 5.2.7 Ayyavoo K., 2022 KCP 5.2.8 Ayyavoo K., 2021 |
|  | Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Surface water | Primary | 0.1 µg/L | LC-MS/MS GC-MS (for 2,4-DCA) | EFSA Journal 2014;12(9):3812, EU agreed |
|  | Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
|  | Primary | 0.1 µg/L | LC-MS/MS (for 2,4-D, 2,4-DCP, 4-CP, 2,4-DCA) | KCP 5.2.5 Ayyavoo K., 2021 KCP 5.2.6 Ayyavoo K., 2022 KCP 5.2.7 Ayyavoo K., 2022 KCP 5.2.8 Ayyavoo K., 2021 |
|  | Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of 2,4-D in air is given in the following table. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-11:	Validated methods for air (if appropriate)

| Component of residue definition: 2,4-D |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 4.5 μg/m3 | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Primary | 4.5 μg/m3 | LC-MS/MS | KCP 5.2.9 Gundoju S., 2020 |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

Not required since 2,4-D is not classified as toxic or highly toxic substance but an overview on the acceptable methods and possible data gaps for analysis of 2,4-D in body fluids and tissues is given in the following table. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-12:	Methods for body fluids and tissues (if appropriate)

| Component of residue definition: 2,4-D |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary (urine and blood) | 0.05 mg/L | LC-MS/MS | EFSA Journal 2014;12(9):3812, EU agreed |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Primary (blood and meat) | 0.01 mg/kg | LC-MS/MS | KCP 5.2.2 Gundoju S., 2019 ammended KCP 5.2.2.1 Ravi Kumar C.R., 2020 |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

#### Other studies/ information

Not relevant, not required.

### Description of analytical methods for the determination of residues MCPA (KCP 5.2)

### Description of analytical methods for the determination of residues MCPA (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table ‎5.3-13:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | MCPA and MCPB (MCPA, MCPB including their salts, esters and conjugates expressed as MCPA)* | 0.05 mg/kg | Reg. (EU) No 491/2014 |
| Plant, high acid content |  | 0.05 mg/kg | Reg. (EU) No 491/2014 |
| Plant, high protein/high starch content (dry commodities) |  | 0.05 mg/kg | Reg. (EU) No 491/2014 |
| Plant, high oil content |  | 0.1 0.05 mg/kg | Reg. (EU) No 491/2014 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.1 mg/kg | Reg. (EU) No 491/2014 |
| Muscle | MCPA, MCPB and MCPA thioethyl expressed as MCPA | 0.1 mg/kg | Reg. (EU) No 491/2014 |
| Milk |  | 0.05 mg/kg | Reg. (EU) No 491/2014 |
| Eggs |  | 0.05 mg/kg | Reg. (EU) No 491/2014 |
| Fat |  | 0.1 mg/kg | Reg. (EU) No 491/2014 |
| Liver, kidney |  | 0.1 mg/kg | Reg. (EU) No 491/2014 |
| Soil (Ecotoxicology) | MCPA | 0.05 mg/kg | Common limit |
| Drinking water (Human toxicology) | MCPA, MCPB and MCPP | 0.1 µg/L | General limit for drinking water |
| Surface water (Ecotoxicology) | MCPA, MCPB and MCPP | 15000 µg/L | Lowest NOEC for Pimephales promelas |
| Air | MCPA | 12 µg/m3 | AOEL sys: 0.04 mg/kg bw/d Dir 05/57 |
| Tissue (meat or liver) | - | not required | Not classified as T / T+ |
| Body fluids |  | not required | Not classified as T / T+ |

* The residue definition for monitoring in plants and animal products is proposed to be MCPA and MCPB (MCPA and MCPB including their salts esters and conjugates expressed as MCPA). There is however no reason to monitor MCPB in residue trials because it is not possible for MCPA to convert to MCPB under field conditions.

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of MCPA in plant matrices is given in the following tables.

zRMS:

ILV methods are missing. These methods were not provided during EU review. These data gaps can be covered after renewal of MCPA.

Table ‎5.3-14:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: MCPA and MCPB (MCPA, MCPB including their salts, esters and conjugates expressed as MCPA) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content | Primary | 0.05 mg/kg | GC-MS (MID) | Keller W., 1980, MCPA Addendum, 2003 |
|  | Primary | 0.05 mg/kg | GC-MS | Pfarl C., 1994, MCPA Addendum, 2003 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Selectivity of method is confirmed by additional fragment ion, therefore other confirmatory method is not required |
| High acid content | Primary | 0.05 mg/kg | GC-MS | Keller, W. 1980, MCPA Addendum, 2003 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Not provided during EU review |
| High protein/high starch content (dry) | Primary | 0.05 mg/kg | GC-MS (MID) | Keller W., 1980, MCPA Addendum, 2003 |
|  | Primary | 0.07 mg/kg | GC/ECD | S. G. Flynn, 1979, MCPA Addendum, 2003 |
|  | Primary | 0.05 mg/kg | GC-MS | Pfarl C., 1994, MCPA Addendum, 2003 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Selectivity of method is confirmed by additional fragment ion, therefore other confirmatory method is not required |

Table ‎5.3-15:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Not required as residues ≥ LOQ are not expected. |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of MCPA in animal matrices is given in the following tables.

zRMS:

ILV methods are missing. These methods were not provided during EU review. These data gaps can be covered after renewal of MCPA.

Table ‎5.3-16:	Validated methods for food and feed of animal origin (if appropriate)

| Component of residue definition: MCPA, MCPB, MCPA thioethyl expressed as MCPA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | - mg/kg 0.01 mg/kg | GC-MSD LC-MS | Koch, DA, 2007, FAO 2012 Johnson, T, and King, DL, 2001, FAO 2012 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Not provided during EU review |
| Eggs | Primary | - | - | Not provided during EU review |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Not provided during EU review |
| Muscle | Primary | 0.05 mg/kg 0.05 mg/kg | GC-MSD LC-MS | Koch, DA, 2007, FAO 2012 Johnson, T, and King, DL, 2001, FAO 2012 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Not provided during EU review |
| Fat | Primary | 0.05 mg/kg 0.05 mg/kg | GC-MSD LC-MS | Koch, DA, 2007, FAO 2012 Johnson, T, and King, DL, 2001, FAO 2012 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Not provided during EU review |
| Kidney, liver | Primary | 0.05 mg/kg 0.05 mg/kg | GC-MSD LC-MS | Koch, DA, 2007, FAO 2012 Johnson, T, and King, DL, 2001, FAO 2012 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory (if required) | - | - | Not provided during EU review |

Table ‎5.3-17:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Not provided during the EU review of MCPA. |

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of MCPA in soil is given in the following tables. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table ‎5.3-18:	Validated methods for soil (if appropriate)

| Component of residue definition: MCPA |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 50 mg/kg | GC-EC | Sattar M.A. and Pasasivirta J., 1979, MCPA Addendum, 2003 |
| Confirmatory | - | - | Not provided during EU review |
| Primary | 0.01 mg/kg | LC-MS (for MCPA and PCOC) | KCP 5.2.10 M. Rubino, 2018 KCP 5.2.11 M. Rubino, 2019 |
| Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of MCPA in surface and drinking water is given in the following tables. For the detailed evaluation of additional studies it is referred to Appendix 2.

zRMS:

ILV method for drinking water is missing. This method was not provided during EU review. This data gaps can be covered after renewal of MCPA.

Table ‎5.3-19:	Validated methods for water (if appropriate)

| Component of residue definition: MCPA, MCPB and MCPP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | 0.1 μg/L | GC-MS | HMSO Publication, 1997, MCPA Addendum, 2003 |
|  | ILV | - | - | Not provided during EU review |
|  | Confirmatory | 0.1 μg/L | GC-MS | HMSO Publication, 1997, MCPA Addendum, 2003 |
|  | Primary | 0.01 μg/L | LC-MS | KCP 5.2.12 M. Rubino, 2019 |
|  | Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |
| Surface water | Primary | 0.1 μg/L | GC-MS | HMSO Publication, 1997, MCPA Addendum, 2003 |
|  | Confirmatory | 0.1 μg/L | GC-MS | HMSO Publication, 1997, MCPA Addendum, 2003 |
|  | Primary | 0.01 μg/L | LC-MS | KCP 5.2.13 M. Rubino, 2019 |
|  | Confirmatory | - | - | This method is highly specific, validated on two mass transitions, so confirmatory is not required |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of MCPA in air is given in the following tables.

Table ‎5.3-20:	Validated methods for air (if appropriate)

| Component of residue definition: MCPA |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.6 µg/m3 | HPLC | Reichert N., 1994, MCPA Addendum, 2003 |
|  | 0.24 μg/m3 | HPLC-UV | Zangmeister W., 1995, MCPA Addendum, 2003 |
| Confirmatory | - | - | Not provided during EU review |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

Not required since MCPA is not classified as toxic or highly toxic substance.

zRMS:

Analytical methods for the determination of residues in body fluids and tissues are required (data requirement according to Regulation (EU) No 284/2013). This data gaps can be covered after renewal of MCPA.

#### Other studies/ information

Not relevant, not required.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1.1 | Simona Nichetti | 2016 | 2,4-D dimethylamine 275 g/L (as acid) + MCPA dimethylamine 275 g/L (as acid) SL: validation of the analytical method for the determination of the active ingredients content ChemService, Report No. CH-280/2016 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.1.1.2 | Mercedes Pardo Martinez | 2019 | 2,4 D 275 g/L + MCPA 275 g/L SL: Validation of the Analytical Method for the Determination of Free Phenols as Relevant Impurity Content ChemService, Report No. CH-391/2019 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.1.1.3 | Nick Ordsmith | 2019 | A Study for the Analysis of 2,4-D (27.5%) MCPA (27.5%) for the Presence of Targeted Polychlorinated Dibenzodioxins and Dibenzofurans with Accelerated Storage Hall Analytical Labolatories, Report No. HAL19-281 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.1 | Ravi Kumar C.R. | 2020 | Validation of analytical method for the determination of residues of 2,4-D in crop matrices: wheat, peas whole plant, peas dry grain, peas fresh grain, osr grain and osr whole plant by LC-MS/MS Bioneeds India Private Limited, Report no. BIO-ANM 1354 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.1.1 | Zarebska M. | 2020 | Independent laboratory Validation of the analytical procedure for the determination of residues of 2,4-D (CAS 94-75-7) in wheat grain by Liquid Chromatography Institute of Heavy Organic Synthesis ˝Blachownia˝, Report No. 52/2020 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.2 | Gundoju S. | 2019 | Validation of analytical method for the determination of residues of 2,4-D acid in animal and body fluid matrices: blood, meat, fat, liver, eggs and milk by LC-MS/MS Bioneeds, Report no. BIO-ANM 1355 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.2.1 | Ravi Kumar C.R. | 2020 | Amendment to study report Validation of analytical method for the determination of residues of 2,4-D acid in animal and body fluid matrices: blood, meat, fat, liver, eggs and milk by LC-MS/MS Bioneeds, Report no. BIO-ANM 1355 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.2.2 | Zarebska M. | 2020 | Independent laboratory Validation of the analytical procedure for the determination of residues of 2,4-D (CAS 94-75-7) in meat, fat, eggs and milk by Liquid Chromatography Institute of Heavy Organic Synthesis ˝Blachownia˝, Report No. 133/2020 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.2.3 | Zarebska M. | 2020 | Independent laboratory Validation of the analytical procedure for the determination of residues of 2,4-D (CAS 94-75-7) in liver by Liquid Chromatography Institute of Heavy Organic Synthesis ˝Blachownia˝, Report No. 41/2020 GLP, Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.3 | Gundoju S. | 2020 | Validation of analytical method for the determination of 2,4-D and its metabolites in soil by LC-MS/MS Bioneeds, Report no. BIO-ANM 1357 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.4 | Ravi Kumar C.R., | 2021 | Validation of analytical method for the determination of residues of 2,4-dichloroanisole (2,4-DCA) in soil Bioneeds, Report no. BIO-ANM 1348 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.5 | Ayyavoo K. | 2021 | Validation of the analytical methodology used for the determination of (2,4-dichlorophenoxy) acetic acid in surface water, drinking water and ground water Bioscience Research Fundation, Report no. 10034/2021 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.6 | Ayyavoo K. | 2022 | Validation of the analytical methodology used for the determination of 2,4-dichlorophenol in surface water, drinking water and ground water Bioscience Research Fundation, Report no. 9753/2021 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.7 | Ayyavoo K. | 2022 | Validation of the analytical methodology used for the determination of 4-chlorophenol in surface water, drinking water and ground water Bioscience Research Fundation, Report no. 9754/2021 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.8 | Ayyavoo K. | 2021 | Validation of the analytical methodology used for the determination of 2,4-dichloroanisole in surface water, drinking water and ground water Bioscience Research Fundation, Report no. 9752/2021 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.9 | Gundoju S. | 2020 | Validation of analytical method for the determination of 2,4-D acid in air by liquid chromatography-mass spectrometry Bioneeds, Report no. BIO-ANM 1356 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.10 | Rubino M. | 2018 | Validation of the analytical procedure for the determination of residues of MCPA (CAS: 94-74-6) in soil by LC/MS Chelab, Report No. 18.647485.0001 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.11 | Rubino M. | 2019 | Validation of the analytical procedure for the determination of PCOC (CAS: 1570-64-5) in soil by LC-MS Chelab, Report No. 19.509954.0001 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.12 | Rubino M. | 2019 | Validation of the analytical procedure for the determination of residues of MCPA (CAS: 94-74-6) in drinking water by LC-MS Chelab, Report No. 18.647484.0007 GLP Unpublished | N | Sharda Cropchem Ltd. |
| KCP 5.2.13 | Rubino M. | 2019 | Validation of the analytical procedure for the determination of residues of MCPA (CAS: 94-74-6) in surface water by LC-MS Chelab, Report No. 18.647484.0006 GLP Unpublished | N | Sharda Cropchem Ltd. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for 2,4-D

    - Methods used for the generation of pre-authorization data (KCP 5.1)

Please refer to the post-autorization methods.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is accepted. |
| --- | --- |

| Reference: | KCP 5.2.1 |
| --- | --- |
| Report | Validation of analytical method for the determination of residues of 2,4-D in crop matrices: wheat, peas whole plant, peas dry grain, peas fresh grain, OSR grain and OSR whole plant by LC-MS/MS, Ravi Kumar C.R., 2020, Report No. BIO-ANM 1354 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to validate an analytical method for the determination of 2,4-D in crop matrices: wheat grain, peas (whole plant, dry grain and fresh grain) and OSR (grain and whole plant) according to test guidelines SANCO/825/00-rev 8.1 and SANCO 3029/99 Rev.4 by LC-MS/MS.

Reference materials:

| Reference Standard 1 | 2,4-D |
| --- | --- |
| CAS No. | 94-75-7 |
| Physical Appearance | White powder |
| Lot No. | BCBX3937 |
| Formula | Cl2C6H3OCH2CO2H |
| Purity | 99.9% |
| Batch Produced by | Sigma-Aldrich |
| Date of Expiry | May 2023 |
| Storage Condition | Room Temperature |
| Reference Standard 2 | 2,4-D Ethylhexyl Ester |
| CAS No. | 1928-43-4 |
| Lot No. | G976505 |
| Purity | 99.29% (g/g) |
| Batch Produced by | LGC / Dr. Ehrenstorfer |
| Date of Expiry | 09.07.2024 |
| Storage Condition | 20°C ± 4°C |

Reagents:

| Name | Source | Batch/Lot No. |
| --- | --- | --- |
| Methanol | SDFCL | C19A/1819/2203/53 |
| Methanol | JT Baker | 0000199199 |
| Acetic acid | Sigma-Aldrich | BCBW1549 |
| Hydrochloric acid | SDFCL | B18A/1218/0905/13 |
| Sodium hydroxide | SDFCL | L18A/1018/1711/08 |
| Milli-Q water | In-house | - |

Equipment:

| Name | Make | Model |
| --- | --- | --- |
| Balance | Sartorius | CPA225D |
| Vortex shaker | Tarsons | SPINX-3020 |
| Ultrasonic cleaner | Servewell Instruments Pvt. Ltd. | GT-1860QTS |
| LC-MS/MS | Agilent Technologies | Ultivo 6465A |
| Column | Agilent Technologies | Eclipse Plus |
| Deep freezer | Carrier | HF DCHP |
| Refrigerator | LG | GL-335VVG4/2012 |
| Stop Clock | Labex | NA |
| Nitrogen Evaporator | Crescent Scientific | Rapid |
| Centrifuge | Thermo Scientific | SORVALL |
| Pipettes | Thermo Scientific | FINNPIPETTE®F2 |
| Homogenizer | Prestige | Stylo |
| Positive Pressure Unit | Orochem Technologies Inc. | EZYPRESSTM HT48 |
| Solid Phase Extraction Catridges | Thermo Scientific | Oasis® HLB 30μm |

Analytical method (LC-MS/MS Conditions):

| Instrument |  | Agilent HPLC 1260 Infinity II with Agilent Ultivo ESI-MS/MS system |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Column |  | Agilent Eclipse plus C18 RRHD [50 m × 2.1 mm (i.d.) × 1.8 μm film thickness] |  |  |  |  |
| Mobile phase A |  | 0.1% formic acid in Milli Q-Water (60%) |  |  |  |  |
| Mobile phase B |  | Acetonitrile (40%) |  |  |  |  |
| Flow rate |  | 0.3 mL/min |  |  |  |  |
| Injection volume |  | 5 μL |  |  |  |  |
| Sample compartment Temperature |  | 15°C |  |  |  |  |
| Run time |  | 4.0 min |  |  |  |  |
| Diluent |  | Methanol |  |  |  |  |
| Retention time |  | ~ 1.5 min |  |  |  |  |
| Mass Spectrometry Operating Conditions: |  |  |  |  |  |  |
| Inetrface | Electro Spray Ionizator |  |  |  |  |  |
| Polarity | Negative |  |  |  |  |  |
| Run Mode | MRM |  |  |  |  |  |
| Gas temperature | 250°C |  |  |  |  |  |
| Sheath Gas Temperature | 300°C |  |  |  |  |  |
| Compound | Transition Type |  | Precursor ion (m/z) | Product ion (m/z) | Fragment voltage (FV) | Collision Energy (CE) |
| 2,4 - D | Quantitative |  | 219 | 160.9 | 79 | 5 |
|  | Confirmatory |  | 219 | 124.9 | 79 | 25 |

Validation of Hydrolysis:

The procedure was designed to measure residues of 2,4-D EHE as 2,4-D. A base hydrolysis of 2,4-D EHE was performed to accomplish this, by virtue of shaking the samples for 60 minutes with a methanol:0.1 N NaOH (90:10, v/v) extraction solution. The completion of this hydrolysis was tested to demonstrate complete conversion to 2,4-D during sample analysis. Samples were spiked with 2,4-D EHE and analysed, over time, to evaluate the completeness of the hydrolysis procedure at time points 0, 5, 10, 20, 30 and 60 minutes. Following analysis, each sample was evaluated, monitored the amount of 2,4-D present, by LC-MS/MS.

Extraction procedure:

About 10 g of each homogenized sample (Wheat grain, Peas - whole plant, dry grain and fresh grain, OSR - whole plant and grain) was transferred into 50 mL falcon tube, fortified with 0.1 mL of stock standard solution hydrolysis was done by vortex extracted with 30 mL of extraction solution (methanol: 0.1 N NaOH, 90:10 v/v) for about 60 minutes at ambient room temperature.

Alter hydrolysis the samples were centrifuged at 5000 rpm for 10 minutes and the supernatant was transferred to another tube and acidified with 2 mL of 0.2 N HCl. Then the solutions were purified by using reverse phase solid phase extraction cartridges (cartridges were conditioned with 5 mL of methanol prior to loading the samples). Samples were loaded onto SPE cartridges further eluted with 5 mL methanol: water (90:10%, v/v) containing 0.5% acetic acid and dilute with 5 mL of methanol: water (10: 90%, v/v) containing 0.5% acetic acid. The extracts were then analyzed for residues of 2,4-D by LC-MS/MS, using the transition m/z 219 → 160.9 for primary quantification and the transition m/z 219-124.9 for qualitative confirmation.

Results and discussions

Specificity/Selectivity:

Specificity of the method was determined by analyzing diluent, mobile phase, standard solution, matrix blank and matrix at LOQ level.

Due to the selective nature of the detection of this method (two individual ion transitions monitored), interference peaks were not observed at the retention time of the analyte.

Matrix effect:

Matrix effect was evaluated and found to be 10.38%, 11.15%, 15.86%, 12.80%, 7.70% and 16.30% for Wheat grain, Peas dry grain, Peas whole plant, Peas fresh grain, OSR whole plant and OSR grain, respectively. Which are less than 20%. Hence no matrix effect was considered.

Linearity:

Linearity was assessed by preparing six different concentrations in order to cover the range from about 30% LOQ to 30% above the highest level. The calibration solution was prepared in solvent.

Linearity solutions prepared in solvent used for quantitative and confirmatory analysis of 2,4-D ranged from 0.005-0.30 mg/L in concentration.

Hydrolysis:

The results indicate an average hydrolysis completion recovery value of >90% after shaking for 60 minutes with methanol: 0.1 N NaOH. The results demonstrate complete conversion from 2,4-D EHE to 2,4-D in the analysis of crop samples with this procedure.

Recovery (Accuracy) and Precision (Repeatability):

Assay accuracy was checked at two concentration levels LOQ (0.01 mg/kg), 10 × LOQ (0.1 mg/kg) and two control samples (background level of <30% is used for LOQ). Five determinations were made at each concentration level for 2,4-D and 2,4-D EHE. The assay accuracy in terms of percentage recovery and the precision in terms of relative standard deviation were determined.

During method validation, acceptable recoveries were generated for the samples fortified level at LOQ and 10 × LOQ level. The % RSD (precision) was <20% at each fortification level. Recovery data from these samples demonstrated that 2,4-D residues were stable during analysis.

Stability of Extracts:

Sample extracts fortified at 10 × LOQ level were stored in refrigerator at 2 to 8°C up to 30 days. After this period, the sample were re-analysed and calculated recovery. The recoveries of all the samples analysed were in the range of 70-110% with %RSD ≤ 20%.

Table A 1:	Recovery results from method validation of 2,4-D using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat Grain (LOQ) | 2,4-D | 0.010 | 93.33 | 2.43 | - |
| Wheat Grain (10LOQ) |  | 0.102 | 93.01 | 2.35 | - |
| Peas dry grain (LOQ) | 2,4-D | 0.010 | 94.76 | 3.17 | - |
| Peas dry grain (10LOQ) |  | 0.102 | 95.79 | 2.51 | - |
| ORS Grain (LOQ) | 2,4-D | 0.010 | 91.16 | 2.56 | - |
| ORS Grain (10LOQ) |  | 0.102 | 93.21 | 1.91 | - |
| Peas fresh grain (LOQ) | 2,4-D | 0.010 | 93.12 | 1.12 | - |
| Peas fresh grain (10LOQ) |  | 0.100 | 94.09 | 1.82 | - |
| Peas whole plant (LOQ) | 2,4-D | 0.010 | 91.24 | 3.08 | - |
| Peas whole plant (10LOQ) |  | 0.100 | 91.85 | 2.12 | - |
| ORS whole plant (LOQ) | 2,4-D | 0.010 | 92.87 | 2.27 | - |
| ORS whole plant (10LOQ) |  | 0.100 | 93.34 | 3.11 | - |
| Wheat Grain (LOQ) | 2,4-D Ester | 0.010 | 91.51 | 2.32 | - |
| Wheat Grain (10LOQ) |  | 0.102 | 91.71 | 3.07 | - |
| Peas dry grain (LOQ) | 2,4-D Ester | 0.010 | 90.78 | 2.35 | - |
| Peas dry grain (10LOQ) |  | 0.102 | 91.11 | 1.73 | - |
| ORS Grain (LOQ) | 2,4-D Ester | 0.010 | 90.29 | 3.00 | - |
| ORS Grain (10LOQ) |  | 0.102 | 90.16 | 3.21 | - |
| Peas fresh grain (LOQ)- | 2,4-D Ester | 0.010 | 92.85 | 2.92 | - |
| Peas fresh grain (10LOQ) |  | 0.100 | 93.31 | 2.03 | - |
| Peas whole plant (LOQ) | 2,4-D Ester | 0.010 | 90.79 | 1.60 | - |
| Peas whole plant (10LOQ) |  | 0.100 | 91.44 | 2.14 | - |
| ORS whole plant (LOQ) | 2,4-D Ester | 0.010 | 89.52 | 2.27 | - |
| ORS whole plant (10LOQ) |  | 0.100 | 93.08 | 0.84 | - |

Table A 2:	Recovery results from method confirmation

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat Grain (LOQ) | 2,4-D | 0.010 | 92.35 | 3.34 | - |
| Wheat Grain (10LOQ) |  | 0.102 | 93.54 | 2.68 | - |
| Peas dry grain (LOQ) | 2,4-D | 0.010 | 94.99 | 5.08 | - |
| Peas dry grain (10LOQ) |  | 0.102 | 96.73 | 2.74 | - |
| ORS Grain (LOQ) | 2,4-D | 0.010 | 91.68 | 3.59 | - |
| ORS Grain (10LOQ) |  | 0.102 | 94.12 | 2.16 | - |
| Peas fresh grain (LOQ) | 2,4-D | 0.010 | 92.62 | 1.76 | - |
| Peas fresh grain (10LOQ) |  | 0.100 | 94.12 | 2.15 | - |
| Peas whole plant (LOQ) | 2,4-D | 0.010 | 92.62 | 2.85 | - |
| Peas whole plant (10LOQ) |  | 0.100 | 92.06 | 2.54 | - |
| ORS whole plant (LOQ) | 2,4-D | 0.010 | 92.66 | 2.77 | - |
| ORS whole plant (10LOQ) |  | 0.100 | 92.42 | 2.36 | - |
| Wheat Grain (LOQ) | 2,4-D Ester | 0.010 | 93.28 | 2.73 | - |
| Wheat Grain (10LOQ) |  | 0.102 | 94.37 | 2.77 | - |
| Peas dry grain (LOQ) | 2,4-D Ester | 0.010 | 91.84 | 2.32 | - |
| Peas dry grain (10LOQ) |  | 0.102 | 94.17 | 1.73 | - |
| ORS Grain (LOQ) | 2,4-D Ester | 0.010 | 93.38 | 4.63 | - |
| ORS Grain (10LOQ) |  | 0.100 | 93.22 | 5.18 | - |
| Peas fresh grain (LOQ) | 2,4-D Ester | 0.010 | 92.51 | 3.79 | - |
| Peas fresh grain (10LOQ) |  | 0.100 | 93.68 | 1.95 | - |
| Peas whole plant (LOQ) | 2,4-D Ester | 0.010 | 90.75 | 0.86 | - |
| Peas whole plant (10LOQ) |  | 0.100 | 91.62 | 2.43 | - |
| ORS whole plant (LOQ) | 2,4-D Ester | 0.010 | 90.07 | 3.49 | - |
| ORS whole plant (10LOQ) |  | 0.100 | 93.83 | 0.82 | - |

Table A 3:	Characteristics for the analytical method used for validation of 2,4-D residues in plant matrices

|  | 2,4-D |
| --- | --- |
| Specificity | No interference, the method is specific |
| Calibration (type, number of data points) | 5 concentration points Linear regression r > 0.99 |
| Calibration range | 0.005 – 0.30 mg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg |

Conclusion

This method for the determination of 2,4-D in Wheat grain, Peas dry grain, Peas whole plant, Peas fresh grain, OSR whole plant and OSR grain sample meets the specification of SANTE/2020/12830 rev. 1.

The LC-MS/MS method is free of interference at the retention time of 2,4-D in Wheat grain, Peas dry grain, Peas whole plant, Peas fresh grain, OSR whole plant and OSR grain samples. The method generated acceptable recoveries over the concentration levels expected in the matrices tested.

          - Independent laboratory validation

| Comments of zRMS: | Validation is acceptable. |
| --- | --- |

| Reference: | 5.2.1.1 |
| --- | --- |
| Report | Independent laboratory Validation of the analytical procedure for the determination of residues of 2,4-D (CAS 94-75-7) in wheat grain by Liquid Chromatography, Zarebska M, 2020, Report No. 52/2020 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to perform Independent Laboratory Validation Study of the analytical method for the ˝Validation of the analytical method for the determination of residues 2,4-D in crop matrices: wheat, peas whole plant, peas dry grain, peas fresh grain, ORS grain and ORS whole plant by LC/MS˝, Study No.: BIO-ANM 1354.

The samples were analyzed by LC-MS/MS according to the internal test procedure BA-AB/MS/SPO-1 ˝General principles of analysis by coupled system of high performance liquid chromatography and mass spectrometry˝.

The samples were extracted by hydrolysis step to transform any ester and conjugate forms into free 2,4-D. The residues of 2,4-D were separated on an reversed-phase column and detected by tandem mass spectrometry (MS/MS) by electrospray (ESI) operating with optimized conditions.

Results and discussions

Wheat grain samples were fortified at two recovery levels: 0.01 mg/kg (LOQ) and 0.1 mg/kg (10xLOQ), five replicates at each fortification level. In addition, control samples were analyzed for the presence of background. Fortification of matrix blanks with 2,4-D Acid and 2,4-D Ester standard was demonstrated and showed that the method is free from interference. The quantification was achieved using 5 points calibration curve obtained plotting standard solutions concentrations and the corresponding peak area.

Table A 4:	Recovery results from independent laboratory validation of 2,4-D using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat grain | 2,4-D | LOQ | 103 | 3.5 | - |
| Wheat grain |  | 10LOQ | 104 | 1.0 | - |
| Wheat grain | 2,4-D Ester | LOQ | 106 | 2.8 | - |
| Wheat grain |  | 10LOQ | 103 | 1.0 | - |

Table A 5:	Recovery results from method confirmation

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat grain | 2,4-D | LOQ | 101 | 2.3 | - |
| Wheat grain |  | 10LOQ | 102 | 1.1 | - |
| Wheat grain | 2,4-D Ester | LOQ | 103 | 4.0 | - |
| Wheat grain |  | 10LOQ | 102 | 0.7 | - |

Table A 6:	Characteristics for the analytical method used for independent laboratory validation of 2,4-D residues in wheat grain

|  | 2,4-D |
| --- | --- |
| Specificity | Matrix blank, reagent blank, mobil phase, mg/kg < 30 % LOQ |
| Calibration (type, number of data points) | 5 concentration points Linear regression y = 1.19e + 007x - 1.43e + 004 r > 0.99 |
| Calibration range | 0.005 – 0.3 mg/L (0.005 – 0.3 mg/kg) |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.003 mg/L (0.003 mg/kg) LOQ = 0.01 mg/L (0.01 mg/kg) |

Conclusion

The data for the validation of the method obtained in the present study were in the compliance with the acceptability criteria whereby it was considered a reliable analytical method to determine the residues of 2,4-D residue (2,4-D, its salts, esters and conjugates expressed as 2,4-D) in wheat grain samples.

No interferences from the matrices were observed and the analytical method worked adequately.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is accepted. |
| --- | --- |

| Reference: | KCP 5.2.2, amended KCP 5.2.2.1 |
| --- | --- |
| Report | Validation for analytical method for the determination of residues of 2,4-D Acid in animal and body fluid matrices: Blood, Meat, Fat, Liver, Eggs and Milk by LC-MS/MS, Suresh Gundoju, 2019, Report No. BIO-ANM 1355, ammended 2020, Ravi Kumar C.R. |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99, rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to validate analytical method for the determination of 2,4-D Acid in animal and body fluid matrices: Blood, Meat, Fat, Liver, Eggs and Milk by LC-MS/MS.

Reference materials:

| Reference Standard 1 | 2,4 - D |
| --- | --- |
| CAS No. | 94-75-7 |
| Physical appearance | White powder |
| Lot No. | BCBX3937 |
| Purity | 99.9% |
| Bach Produced by | Sigma-Aldrich |
| Date of Expiry | May 2023 |
| Storage Condition | Room temperature |
| Reference Standard 2 | 2,4 – D Ethylhexyl Ester |
| CAS No. | 1928-43-4 |
| Lot No. | G976505 |
| Purity | 99.29% (g/g) |
| Bach Produced by | LGC / Dr. Ehrenstorfer |
| Date of Expiry | 09.07.2024 |
| Storage Condition | 20°C ± 4°C |

Reagents:

| Name | Source | Batch/Lot No. |
| --- | --- | --- |
| Methanol | SDFCL | A19A/1019/2901/53 |
| Acetonitrile | JT Baker | 0000200489 |
| Magnesium sulphate | Sigma-Aldrich | SLBW8124 |
| Acetic acid | Sigma-Aldrich | BCBW1549 |
| Tri sodium citrate dehydrate | CDH | 4208018 |
| Sodium chloride | Himedia | 0000265538 |
| Sodium hydroxide | SDFCL | L18A/1018/1711/08 |
| Milli-Q-water | In-house | - |

Equipment:

| Name | Make | Model |
| --- | --- | --- |
| Balance | Sartorius | CPA225D |
| Vortex shaker | Tarsons | SPINX-3020 |
| Ultrasonic cleaner | Servewell Instruments Pvt. Ltd. | GT-1860QTS |
| LC-MS/MS | Agilent Technologies | Ultivo 6465A |
| Column | Agilent Technologies | Eclipse plus |
| Deep freezer | Carrier | HF 500CHP |
| Refrigerator | LG | GL-335VVG4/2012 |
| Digital timer | Sport line | L359 |
| Nitrogen evaporator | Crescent Scientific | RAPID |
| Tissue homogenizer | IKA | T10B |
| Centrifuge | Thermo Scientific | SORVALL |
| Water Purification System | Millipore Q8UV | SMARTPAY DIRECT |

Analytical method (LC-MS/MS Conditions):

| HPLC Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Instrument |  | Agilent HPLC 1260 Infinity II with Agilent Ultivo ESI-MS/MS system |  |  |  |  |
| Column |  | Agilent Eclipse plus C18 [50 m × 2.1 mm (i.d.) × 1.8 μm film thickness] |  |  |  |  |
| Mobile phase A |  | 0.1% formic acid in Milli Q-Water (60%) |  |  |  |  |
| Mobile phase B |  | Acetonitrile (40%) |  |  |  |  |
| Flow rate |  | 0.3 mL/min |  |  |  |  |
| Injection volume |  | 5 μL |  |  |  |  |
| Sample compartment temperature |  | 15°C |  |  |  |  |
| Run time |  | 4.0 min |  |  |  |  |
| Diluents |  | Methanol |  |  |  |  |
| Retention time |  | ~ 1.7 min |  |  |  |  |
| Mass Spectrometry Operating Conditions: |  |  |  |  |  |  |
| Inetrface | Electro Spray Ionization |  |  |  |  |  |
| Polarity | Negative |  |  |  |  |  |
| Run Mode | MRM |  |  |  |  |  |
| Gas temperature | 250°C |  |  |  |  |  |
| Sheath Gas Temperature | 300°C |  |  |  |  |  |
| Compound | Transition Type |  | Precursor ion (m/z) | Product ion (m/z) | Fragment voltage (FV) | Collision Energy (CE) |
| 2,4-D Acid | Quantitative |  | 219 | 161 | 79 | 5 |
|  | Confirmatory |  | 219 | 125 | 79 | 25 |

Validation of Hydrolysis:

The procedure was designed to measure residues of 2,4-D EHE as 2,4-D. A base hydrolysis of 2,4-D EHE was performed to accomplish this, by virtue of shaking the samples for 60 minutes with a metha-nol:0.1 N NaOH (90:10, v/v) extraction solution. The completion of this hydrolysis was tested to demonstrate complete conversion to 2,4-D during sample analysis. Samples were spiked with 2,4-D EHE and analysed, over time, to evaluate the completeness of the hydrolysis procedure at time points 0, 5, 10, 20, 30 and 60 minutes. Following analysis, each sample was evaluated, monitored the amount of 2,4-D present, by LC-MS/MS.

Extraction procedure:

Hydrolysis step was carried out by shaking the samples for 60 minutes with methanol: 0.1 N NaOH (90:10, v/v) extraction solution.

The above hydrolysis procedure is used for all matrices prior to extraction process discussed individually for each matrix.

Results and discussions

Specificity / Selectivity:

Specificity of the method was determined by analyzing diluent (reagent blank), mobile phase, standard solution, matrix blank and matrix at LOQ level.

Due to the selective nature of the detection of this method (two individual ion transitions monitored), interference were not observed at the retention time of the analyte.

Matrix effect:

Matrix effect was evaluated through the ratio of the matrix blank spiked area and the solvent spiked area.

Matrix effect was evaluated and found to be 6.94%, 0.97%, 1.81%, 0.73%, 2.36% and 1.5% for meat, fat. Liver, blood, milk and egg, respectively. Which are less than 20%. Hence no matrix effect was considered.

Linearity:

Linearity was assessed by preparing five different concentrations of the 2,4-D Acid standard in diluents. Linearity curve was plotted between concentrations versus detector response. The correlation coefficient, slope and intercept were calculated.

Linearity solutions prepared in matrix blank used for quantitative and confirmatory analysis of 2,4-D Acid ranged from 0.003 – 0.300 mg/L (0.003 to 0.300 mg/kg, w/w) in concentration.

Hydrolysis:

The results indicate an average hydrolysis completion recovery value of ˃90% after shaking for 60 minutes with methanol: 0.1 N NaOH. The results demonstrate complete conversion from 2,4-D EHE to 2,4-D in the analysis of meat, fat, liver, blood, milk and egg samples with this procedure. Therefore, the hydrolysis step is included in the extraction procedure for determining the residues of 2.4-D ester as 2,4-D.

Assay Accuracy (Recovery) and Precision (Repeatability):

Recovery of the method was determined at two fortification levels at LOQ (0.01 mg/kg) and 10 × LOQ (0.1 mg/kg) along with two control samples. Five determinations were made at each concentration level for 2,4-D and 2,4-D EHE. The assay accuracy was calculated in terms of percentage recovery and the precision in terms of relative standard deviation. All fortifications were made directly to the sample.

During method validation, acceptable recoveries were generated for the samples fortified at LOQ and 10 × LOQ level. The % RSD (precision) was <20% at each fortification level. Recovery data from these samples demonstrated that 2,4-D Acid residues were stable during analysis.

Stability of Extracts:

Sample extracts fortified at 10 × LOQ level were stored in refrigerator at 2 to 8°C up to 15 days. After this period, the sample were re-analysed and calculated recovery. The recoveries of all the samples analysed were in the range of 70-110% with %RSD ≤ 20%.

Table A 7:	Recovery results from method validation of 2,4-D Acid  using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Meat (LOQ) | 2,4-D Acid | 0.00940 | 94.26 | 6.47 | - |
| Meat (10LOQ) |  | 0.0933 | 93.35 | 3.10 | - |
| Fat (LOQ) | 2,4-D Acid | 0.00970 | 96.62 | 4.68 | - |
| Fat (10LOQ) |  | 0.0964 | 96.43 | 4.07 | - |
| Liver (LOQ) | 2,4-D Acid | 0.00930 | 92.94 | 4.10 | - |
| Liver (10LOQ) |  | 0.0929 | 92.87 | 3.37 | - |
| Meat (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.00850 | 85.34 | 2.25 | - |
| Meat (10LOQ) |  | 0.0897 | 88.82 | 1.81 | - |
| Fat (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0086 | 85.83 | 3.20 | - |
| Fat (10LOQ) |  | 0.0880 | 87.11 | 2.41 | - |
| Liver (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.00840 | 83.76 | 2.75 | - |
| Liver (10LOQ) |  | 0.0867 | 85.86 | 2.10 | - |
| Blood (LOQ) | 2,4-D Acid | 0.00930 | 93.18 | 3.20 | - |
| Blood (10LOQ) |  | 0.0928 | 92.82 | 2.89 | - |
| Milk (LOQ) | 2,4-D Acid | 0.00930 | 93.50 | 2.70 | - |
| Milk (10LOQ) |  | 0.0937 | 92.75 | 1.36 | - |
| Egg (LOQ) | 2,4-D Acid | 0.00940 | 93.51 | 3.35 | - |
| Egg (10LOQ) |  | 0.0949 | 93.92 | 2.81 | - |
| Blood (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.00870 | 86.71 | 3.96 | - |
| Blood (10LOQ) |  | 0.0928 | 90.97 | 2.80 | - |
| Milk (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.00900 | 90.47 | 3.47 | - |
| Milk (10LOQ) |  | 0.0911 | 89.30 | 3.67 | - |
| Egg (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.00880 | 88.22 | 1.64 | - |
| Egg (10LOQ) |  | 0.0893 | 87.55 | 3.70 | - |

Table A 8:	Recovery results from method confirmation

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Meat (LOQ) | 2,4-D Acid | 0.0095 | 95.11 | 6.88 | - |
| Meat (10LOQ) |  | 0.0942 | 94.17 | 2.97 | - |
| Fat (LOQ) | 2,4-D Acid | 0.0099 | 98.57 | 5.54 | - |
| Fat (10LOQ) |  | 0.0971 | 97.05 | 3.59 | - |
| Liver (LOQ) | 2,4-D Acid | 0.0094 | 93.56 | 4.25 | - |
| Liver (10LOQ) |  | 0.0935 | 93.50 | 3.86 | - |
| Meat (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0084 | 83.88 | 3.23 | - |
| Meat (10LOQ) |  | 0.0898 | 88.87 | 1.74 | - |
| Fat (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0086 | 85.86 | 2.54 | - |
| Fat (10LOQ) |  | 0.0884 | 87.54 | 2.68 | - |
| Liver (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0084 | 83.82 | 3.47 | - |
| Liver (10LOQ) |  | 0.0866 | 85.72 | 2.02 | - |
| Blood (LOQ) | 2,4-D Acid | 0.0093 | 92.96 | 3.29 | - |
| Blood (10LOQ) |  | 0.0928 | 92.80 | 2.67 | - |
| Milk (LOQ) | 2,4-D Acid | 0.0094 | 94.39 | 4.50 | - |
| Milk (10LOQ) |  | 0.0958 | 94.87 | 1.87 | - |
| Egg (LOQ) | 2,4-D Acid | 0.0095 | 95.02 | 5.64 | - |
| Egg (10LOQ) |  | 0.0975 | 96.53 | 3.86 | - |
| Blood (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0088 | 87.68 | 2.73 | - |
| Blood (10LOQ) |  | 0.0937 | 91.83 | 2.80 | - |
| Milk (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0092 | 91.92 | 4.18 | - |
| Milk (10LOQ) |  | 0.918 | 89.95 | 4.01 | - |
| Egg (LOQ) | 2,4-D Acid in 2,4-D Ester | 0.0089 | 88.67 | 2.50 | - |
| Egg (10LOQ) |  | 0.0900 | 88.19 | 4.09 | - |

Table A 9:	Characteristics for the analytical method used for validation of 2,4-D residues in animal matrices

|  | 2,4-D |
| --- | --- |
| Specificity | No interference, the method is specific |
| Calibration (type, number of data points) | 5 concentration points Linear regression r > 0.99 |
| Calibration range | 0.003 – 0.30 mg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg |

Conclusion

This method for the determination of 2,4-D Acid in animal and body fluid matrices (Blood, Meat, Fat, Liver, Eggs and Milk) meets the specification of SANTE/2020/12830 rev.1.

The LC-MS/MS method is free of interference at the retention time of 2,4-D Acid in animal and body fluid matrices tested . The method generated acceptable recoveries over the concentration levels expected in the matrices tested. Therefore it is concluded that the analytical method has been properly validated and it is adequate to determine 2,4-D residue (defined as 2,4-D, its salts,  esters and conjugates, expressed as 2,4-D) in animal and body fluid matrices.

          - Independent laboratory validation

| Comments of zRMS: | Independent laboratory Validation is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.2.2 |
| --- | --- |
| Report | Independent laboratory Validation of the analytical procedure for the determination of residues of 2,4-D (CAS 94-75-7) in meat, fat, eggs and milk by Liquid Chromatography, Zarebska M, 2020, Report No. 133/2020 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to perform Independent Laboratory Validation Study of the analytical method for determination of 2,4-D. The analysis was performed on meat, fat, milk and egg according the procedure delivered by Sharda ˝Validation of the analytical method for the determination of residues of 2,4-D acid in animal and body fluid matrices: blood, meat, fat, liver, eggs and milk by LC/MS˝, Study No.: BIO-ANM 1355.

The samples were analyzed by LC-MS/MS according to the internal test procedure BA-AB/MS/SPO-1 ˝General principles of analysis by coupled system of high performance liquid chromatography and mass spectrometry˝.

The samples were extracted by hydrolysis step to transform any ester and conjugate forms into free 2,4-D. The residues of 2,4-D were separated on an reversed-phase column and detected by tandem mass spectrometry (MS/MS) by electrospray (ESI) operating with optimized conditions.

Results and discussions

Meat, fat, eggs and milk samples were fortified at two recovery levels: 0.01 mg/kg (LOQ) and 0.1 mg/kg (10xLOQ), five replicates at each fortification level. In addition, control samples were analyzed for the presence of background. Fortification of matrix blanks with 2,4-D Acid and 2,4-D Ester standard was demonstrated and showed that the method is free from interference. The quantification was achieved using 5 points calibration curve obtained plotting standard solutions concentrations and the corresponding peak area.

Table A 10:	Recovery results from independent laboratory validation of 2,4-D using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Meat | 2,4-D | LOQ | 98 | 1.6 | - |
|  |  | 10LOQ | 104 | 2.8 | - |
|  | 2,4-D Ester | LOQ | 96 | 2.8 | - |
|  |  | 10LOQ | 104 | 1.9 | - |
| Fat | 2,4-D | LOQ | 83 | 0.9 | - |
|  |  | 10LOQ | 106 | 1.9 | - |
|  | 2,4-D Ester | LOQ | 88 | 2.5 | - |
|  |  | 10LOQ | 108 | 1.2 | - |
| Egg | 2,4-D | LOQ | 92 | 1.9 | - |
|  |  | 10LOQ | 104 | 0.7 | - |
|  | 2,4-D Ester | LOQ | 96 | 0.6 | - |
|  |  | 10LOQ | 103 | 1.3 | - |
| Milk | 2,4-D | LOQ | 93 | 2.1 | - |
|  |  | 10LOQ | 104 | 1.3 | - |
|  | 2,4-D Ester | LOQ | 93 | 1.3 | - |

Table A 11: 	Recovery results from method confirmation

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Meat | 2,4-D | LOQ | 97 | 5.3 | - |
|  |  | 10LOQ | 101 | 3.5 | - |
|  | 2,4-D Ester | LOQ | 97 | 1.6 | - |
|  |  | 10LOQ | 100 | 1.7 | - |
| Fat | 2,4-D | LOQ | 83 | 3.5 | - |
|  |  | 10LOQ | 104 | 2.2 | - |
|  | 2,4-D Ester | LOQ | 88 | 1.5 | - |
|  |  | 10LOQ | 107 | 2.4 | - |
| Egg | 2,4-D | LOQ | 96 | 3.7 | - |
|  |  | 10LOQ | 109 | 0.7 | - |
|  | 2,4-D Ester | LOQ | 105 | 2.3 | - |
|  |  | 10LOQ | 110 | 2.2 | - |
| Milk | 2,4-D | LOQ | 95 | 3.2 | - |
|  |  | 10LOQ | 102 | 1.5 | - |
|  | 2,4-D Ester | LOQ | 96 | 2.4 | - |
|  |  | 10LOQ | 103 | 1.7 | - |

Table A 12:	Characteristics for the analytical method used for independent laboratory validation of 2,4-D residues in meat, fat, eggs and milk

|  | 2,4-D |
| --- | --- |
| Specificity | Matrix blank, reagent blank, mobil phase, mg/kg < 30 % LOQ |
| Calibration (type, number of data points) | 5 concentration points Linear regression for meat y = 1.14e + 007x – 3.2e + 003 r > 0.99 Linear regression for fat y = 1.08e + 007x – 4.81e + 003 r > 0.99 Linear regression for egg y = 1.26e + 007x – 57.8 r > 0.99 Linear regression for milk y = 1.28e + 007x – 5.38e + 003 r > 0.99 |
| Calibration range | 0.003 – 0.3 mg/L (0.003 – 0.3 mg/kg) |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.003 mg/L (0.003 mg/kg) LOQ = 0.01 mg/L (0.01 mg/kg) |

Conclusion

The data for the validation of the method obtained in the present study were in the compliance with the acceptability criteria whereby it was considered a reliable analytical method to determine the residues of 2,4-D residue (2,4-D, its salts, esters and conjugates expressed as 2,4-D) in meat, fat, eggs and milk.

No interferences from the matrices were observed and the analytical method worked adequately.

          - Independent laboratory validation

| Comments of zRMS: | Independent laboratory Validation is acceptable. |
| --- | --- |

| Reference: | 5.2.2.3 |
| --- | --- |
| Report | Independent laboratory Validation of the analytical procedure for the determination of residues of 2,4-D (CAS 94-75-7) in liver by Liquid Chromatography, Zarebska M, 2020, Report No. 41/2020 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to perform Independent Laboratory Validation Study of the analytical method for determination of 2,4-D. The analysis was performed on liver according the procedure delivered by Sharda ˝Validation of the analytical method for the determination of residues of 2,4-D acid in animal and body fluid matrices: blood, meat, fat, liver, eggs and milk by LC/MS˝, Study No.: BIO-ANM 1355.

The samples were analyzed by LC-MS/MS according to the internal test procedure BA-AB/MS/SPO-1 ˝General principles of analysis by coupled system of high performance liquid chromatography and mass spectrometry˝.

The samples were extracted by hydrolysis step to transform any ester and conjugate forms into free 2,4-D. The residues of 2,4-D were separated on an reversed-phase column and detected by tandem mass spectrometry (MS/MS) by electrospray (ESI) operating with optimized conditions.

Results and discussions

Liver samples were fortified at two recovery levels: 0.01 mg/kg (LOQ) and 0.1 mg/kg (10xLOQ), five replicates at each fortification level. In addition, control samples were analyzed for the presence of background. Fortification of matrix blanks with 2,4-D Acid and 2,4-D Ester standard was demonstrated and showed that the method is free from interference. The quantification was achieved using 5 points calibration curve obtained plotting standard solutions concentrations and the corresponding peak area.

Table A 13:	Recovery results from independent laboratory validation of 2,4-D using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Liver | 2,4-D | LOQ | 94 | 3.5 | - |
|  |  | 10LOQ | 102 | 1.8 | - |
|  | 2,4-D Ester | LOQ | 89 | 3.0 | - |
|  |  | 10LOQ | 95 | 2.4 | - |

Table A 14:	Recovery results from method confirmation

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Liver | 2,4-D | LOQ | 91 | 5.1 | - |
|  |  | 10LOQ | 101 | 2.1 | - |
|  | 2,4-D Ester | LOQ | 87 | 3.3 | - |
|  |  | 10LOQ | 95 | 1.2 | - |

Table A 15:	Characteristics for the analytical method used for independent laboratory validation of 2,4-D residues in liver

|  | 2,4-D |
| --- | --- |
| Specificity | Matrix blank, reagent blank, mobil phase, mg/kg < 30 % LOQ |
| Calibration (type, number of data points) | 5 concentration points Linear regression y = 2.23e+007x – 4.85e+003 r > 0.99 |
| Calibration range | 0.003 – 0.3 mg/L (0.003 – 0.3 mg/kg) |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.003 mg/L (0.003 mg/kg) LOQ = 0.01 mg/L (0.01 mg/kg) |

Conclusion

The data for the validation of the method obtained in the present study were in the compliance with the acceptability criteria whereby it was considered a reliable analytical method to determine the residues of 2,4-D residue (2,4-D, its salts, esters and conjugates expressed as 2,4-D) in liver samples.

No interferences from the matrices were observed and the analytical method worked adequately.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.3 |
| --- | --- |
| Report | Validation of analytical method for the determination of 2,4-D and its metabolites in soil by LC-MS/MS, Suresh Gundoju, 2020, Report No. BIO-ANM 1357 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The analytical methods described in this report were developed for the detection, quantification and confirmation of 2,4-D and its metabolites in soil. This validation study was performed and documented following EU-guidance document SANCO/825/00-rev 8.1, 16/11/2010 and SANCO 3029/99 Rev.4. by LC-MS/MS.

Reference materials:

| Reference Standard 1 | 2,4-D |
| --- | --- |
| CAS No. | 94-75-7 |
| Physical Appearance | White powder |
| Lot No. | BCBX3937 |
| Formula | Cl2C6H3OCH2CO2H |
| Purity | 99.9% |
| Batch Produced by | Sigma-Aldrich |
| Date of Expiry | May 2023 |
| Storage Condition | Room Temperature |
| Reference Standard 2 | 2,4-D Ethylhexyl Ester (2,4-D EHE) |
| CAS No. | 1928-43-4 |
| Lot No. | G976505 |
| Purity | 99.29% (g/g) |
| Batch Produced by | LGC / Dr. Ehrenstorfer |
| Date of Expiry | 9 July 2024 |
| Storage Condition | 20°C ± 4°C |
| Reference Standard 3 | 4-Chlorophenol (metabolite 4-CP) |
| CAS No. | 108-48-9 |
| Physical Appearance | White Crystalline Solid |
| Batch No. | WXBC2551V |
| Purity | 99.89% |
| Batch Produced by | Sigma-Aldrich |
| Next Retesting Date | 24 March 2020 |
| Reference Standard 4 | 2,4- dichlorophenol (metabolite 2,4-DCP) |
| CAS No. | 120-83-2 |
| Physical Appearance | White Crystalline Solid |
| Batch No. | BCBT6806 |
| Purity | 99.38% |
| Batch Produced by | Sigma-Aldrich |
| Next Retesting Date | 24 March 2020 |

Reagents:

| Name | Source | Batch/Lot No. |
| --- | --- | --- |
| Acetonitrile | JT Baker | 0000221770 & 0000210475 |
| Methanol | SDFCL | C19A/1819/2203/53 |
| Acetic acid | Sigma-Aldrich | BCBW1549 |
| Hydrochloric acid | SDFCL | B18A/1218/0905/13 |
| Sodium hydroxide | SDFCL | L18A/1018/1711/08 |
| Milli-Q water | In-house | - |

Equipment:

| Name | Make | Model |
| --- | --- | --- |
| Balance | Sartorius | CPA225D |
| Vortex shaker | Tarsons | SPINX-3020 |
| Ultrasonic cleaner | Servewell Instruments Pvt. Ltd. | GT-1860QTS |
| LC-MS/MS | Agilent Technologies | Ultivo 6465A |
| Manifold | Phenomenex | - |
| Column | Agilent Technologies | Eclipse Plus |
| Refrigerator | LG | GL-335VVG4/2012 |
| Centrifuge | Thermo Fisher Scientific | Sorvall legendxtr |
| Nitrogen Evaporator | Crescent Scientific | Rapid |
| Milli-Q water | Millipore Q8UV | SMARTPAY DIRECT |
| Micropipettes | Thermo Scientific | FINNPIPETTE®F2 |
| Solid Phase Extraction Catridges | Thermo Scientific | Oasis® MCX 30μm |

Analytical method (LC-MS/MS Conditions):

| Instrument |  | Agilent HPLC 1260 Infinity II with Agilent Ultivo ESI-MS/MS system |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Column |  | Agilent Eclipse plus C18 RRHD [50 m × 2.1 mm (i.d.) × 1.8 μm film thickness] |  |  |  |  |
| Mobile phase A |  | 0.1% formic acid in Milli Q-Water (60%) |  |  |  |  |
| Mobile phase B |  | Acetonitrile (40%) |  |  |  |  |
| Flow rate |  | 0.3 mL/min |  |  |  |  |
| Injection volume |  | 5 μL |  |  |  |  |
| Column compartment |  | 40°C |  |  |  |  |
| Sample compartment Temperature |  | 15°C |  |  |  |  |
| Run time |  | 4.0 min |  |  |  |  |
| Diluent |  | Acetonitrile |  |  |  |  |
| Retention time |  | 2,4-D : 1.7 min 2,4-DCP : 2.5 min 2,4-CP : 1.5 min |  |  |  |  |
| Mass Spectrometry Operating Conditions for 2,4-D and 2,4-DCP: |  |  |  |  |  |  |
| Inetrface | Atmospheric Pressure Chemical Ionization (APCI) |  |  |  |  |  |
| Polarity | Negative |  |  |  |  |  |
| Run Mode | MRM |  |  |  |  |  |
| Gas temperature | 290°C |  |  |  |  |  |
| Nebulizer | 40 psi |  |  |  |  |  |
| Capillary | 4000 V |  |  |  |  |  |
| Corona Current | 25 μA |  |  |  |  |  |
| Vaporizer | 300°C |  |  |  |  |  |
| Compound | Transition Type |  | Precursor ion (m/z) | Product ion (m/z) | Fragment voltage (FV) | Collision Energy (CE) |
| 2,4-D | Quantitative |  | 219 | 160.9 | 79 | 18 |
|  | Confirmatory |  | 219 | 124.9 | 79 | 15 |
| 2,4-DCP | Quantitative |  | 161 | 125.0 | 95 | 10 |
|  | Confirmatory |  | 161 | 88.9 | 95 | 10 |
| Mass Spectrometry Operating Conditions for 4-CP: |  |  |  |  |  |  |
| Inetrface | AJS ESI |  |  |  |  |  |
| Polarity | Negative |  |  |  |  |  |
| Run Mode | MRM |  |  |  |  |  |
| Gas temperature | 250°C |  |  |  |  |  |
| Nebulizer | 45 psi |  |  |  |  |  |
| Capillary | 1000 V |  |  |  |  |  |
| Sheath Gas Temperature | 300°C |  |  |  |  |  |
| Sheath GAS Flow | 11 L/min |  |  |  |  |  |
| Compound | Transition Type |  | Precursor ion (m/z) | Product ion (m/z) | Fragment voltage (FV) | Collision Energy (CE) |
| 4-CP | Quantitative |  | 127 | 91.2 | 106 | 13 |
|  | Confirmatory |  | 127 | 34.9 | 106 | 9 |

Extraction procedure - Soil Sample Extraction for 2,4-D, 2,4-DCP and 4-CP:

About 50 g of homogenized soil sample were spiked with standard solutions at LOQ and 10 LOQ and extracted with a mixture of 100 mL of acetonitrile and 0.1 N HCl (90:10, v/v) for about two hours using end-over-end mechanical shaker and centrifuged at 10000 rpm for 10 minutes. The above extraction step was repeated with 100 mL of mixture of acetonitrile and 1 N HCl (90:10, v/v). An aliquot 10 mL of extract was diluted with 20 mL of HPLC grade water and acidified with 1 mL of 2 N HCl. The extract was purified using an Oasis MCX cartridge. 10 mL of extract sample was loaded and washed with 5 mL of 0.1 N HCl in water and discarded the eluent further eluted with 10 mL acetonitrile using manifold. The extract was concentrated to near dryness using nitrogen evaporatory and reconstituted with 5 mL of acetonitrile further diluted with 1 mL of water containing 0.1% acetic acid prior to analysis by LC-MS/MS.

Extraction procedure - Hydrolysis Procedure for 2,4-D EHE:

About 50 g of homogenized soil samples were spiked with 2,4-D EHE standard solution at LOQ and 10 LOQ concentration levels and hydrolysis step was carried out by shaking the samples for 60 minutes with methanol: 0.1N NaOH (90:10, v/v) extraction solvent. The above extraction procedure was used for soil samples.

Results and discussions

Specificity/Selectivity:

Specificity of the method was determined by analyzing diluent, mobile phase, standard solution, matrix blank and matrix at LOQ level.

Due to the selective nature of the detection of this method (two individual ion transitions monitored), interference peaks were not observed at the retention time of the analyte.

Matrix effect:

Matrix effect was evaluated for the analyte 2,4-D, 2,4-DCP, 4-CP in soil matrix and found to be 9.48%, 3.94% and 7.23% respectively, which are less than 20%. Hence, no matrix effect was considered.

Linearity:

Linearity was assessed by preparing six different concentrations of 2,4-D, 2,4-DCP and 4-CP standards in diluent. Linearity curve was plotted between concentrations versus detector response. The correlation coefficient, slope and intercept were calculated.

Hydrolysis:

The results indicate an average hydrolysis completion recovery value of >85% after shaking for 60 minutes with methanol: 0.1 N NaOH. The results demonstrate complete conversion from 2,4-D EHE to 2,4-D in the analysis of soil samples with this procedure. Therefore, hydrolysis step is included in the extraction procedure for determining the residues of 2,4-D EHE as 2,4-D.

Recovery (Accuracy) and Precision (Repeatability):

Recovery of the method was determined at two fortification levels at LOQ (0.05 mg/kg) and 10×LOQ (0.5 mg/kg) along with two control samples. Five determinations were made at each concentration level for 2,4-D, 2,4-D EHE, 2,4-DCP and 4-CP. The assay accuracy was calculated in terms of percentage recovery and the precision in terms of relative standard deviation. All fortifications were made directly to the soil.

During method validation, acceptable recoveries were generated for the samples fortified at LOQ and 10×LOQ level. The % RSD (precision) was < 20% at each fortification level. Recovery data from these samples demonstrated that 2,4-D, 2,4-DCP, 2,4-D EHE and 4-CP residues were stable during analysis. The recoveries of all the samples analysed were in the range of 70-110% with % RSD ≤ 20%.

Stability of Extracts:

Sample extracts fortified at 10 × LOQ level were stored in refrigerator at 2 to 8°C up to 15 days. After this period, the samples were re-analysed and calculated recovery. The recoveries of all the samples analysed were in the range of 70-110% with %RSD ≤ 20%.

Table A 16:	Recovery results from method validation of 2,4-D and its metabolites using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soil | 2,4-D | 0.05 | 99.05 | 3.50 | Quantifier transition |
|  |  | 0.5 | 97.86 | 2.65 |  |
|  |  | 0.05 | 93.55 | 2.11 | Confirmatory transition |
|  |  | 0.5 | 96.54 | 4.52 |  |
|  | 2,4-D EHE | 0.05 | 88.93 | 3.06 | Quantifier transition |
|  |  | 0.5 | 92.01 | 5.43 |  |
|  |  | 0.05 | 99.34 | 0.97 | Confirmatory transition |
|  |  | 0.5 | 91.79 | 1.05 |  |
|  | 2,4-DCP | 0.05 | 96.72 | 1.05 | Quantifier transition |
|  |  | 0.5 | 95.12 | 3.05 |  |
|  |  | 0.05 | 101.94 | 1.57 | Confirmatory transition |
|  |  | 0.5 | 94.35 | 2.20 |  |
|  | 4-CP | 0.05 | 94.06 | 0.31 | Quantifier transition |
|  |  | 0.5 | 94.71 | 4.27 |  |
|  |  | 0.05 | 99.14 | 0.23 | Confirmatory transition |
|  |  | 0.5 | 96.08 | 4.10 |  |

Table A 17:	Characteristics for the analytical method used for validation of 2,4-D residues in soil

|  | 2,4-D | 2,4-DCP | 4-CP |
| --- | --- | --- | --- |
| Specificity | No interference, the method is specific | No interference, the method is specific | No interference, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression y = 418635x + 11.312 r > 0.99 | 6 concentration points Linear regression y = 461091x - 818.37 r > 0.99 | 6 concentration points Linear regression y = 415465x - 106.43 r > 0.99 |
| Calibration range | 0.005 – 0.204 mg/L (0.003 – 0.136 mg/kg) | 0.005 – 0.204 mg/L (0.003 – 0.136 mg/kg) | 0.005 – 0.203 mg/L (0.003 – 0.135 mg/kg) |
| Assessment of matrix effects is presented | Yes | Yes | Yes |
| Limit of determination/quantification | LOD = 0.003 mg/kg LOQ = 0.05 mg/kg | LOD = 0.003 mg/kg LOQ = 0.05 mg/kg | LOD = 0.003 mg/kg LOQ = 0.05 mg/kg |

Conclusion

This method for the determination of 2,4-D, its conjugate and esters forms 2,4-DCP, 4-CP and 2,4-D EHE (as 2,4-D) in soil meets the specification of SANTE/2020/12830 rev. 1.

The LC-MS/MS method is free of interference at the retention times of 2,4-D, 2,4-DCP and 4-CP in soil matrix tested. The method generated acceptable recoveries over the concentration levels expected in the matrix tested. Therefore, it is concluded that analytical method has been properly validated and it is adequate to determine 2,4-D, 2,4-DCP and 4-CP residues in soil with a limit of quantification of 0.05 mg/kg.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.4 |
| --- | --- |
| Report | Validation of analytical method for the determination of residues of 2,4-Dichloroanisole (2,4-DCA) in soil, Ravi Kumar C.R., 2021, Report No. BIO-ANM 1348 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to validate an analytical method for the determination of 2,4-Dichloroanisole in Soil according to test guidelines SANCO/825/00-rev 8.1 and SANCO 3029/99 Rev.4. by GC-MS/MS.

Reference materials:

| Reference Standard 1 | 2,4-Dichloroanisole (metabolite 2,4-DCA) |
| --- | --- |
| CAS No. | 553-82-2 |
| Physical Appearance | Colourless liquid |
| Batch No. | AB126634 |
| Purity | 99.71% |
| Batch Produced by | Lubaq Corporate, S.L. |
| Retest Date | 14 July 2020 |
| Next Retesting Date | 14 July 2022 |
| Storage Condition | Ambient (21 to 29°C) |

Reagents:

| Name | Source | Batch/Lot No. |
| --- | --- | --- |
| Ethyl acetate | Spectrochem Pvt. Ltd. | E301171 |
| Sodium sulphate | Central Drug House (P) Ltd. | 120417 |
|  | SDFCL | H17A/1017/1008/08 |

Equipment:

| Name | Make | Model |
| --- | --- | --- |
| Balance | Sartorius | CPA 225D and GPA3202 |
| Ultrasonic cleaner | Servewell Instruments Pvt. Ltd. | GT-1860QTS |
| Vortex shaker | Tarsons | SPINX-3020 |
| Micro pipette | Eppendorf | Research ® Plus |
| GC-MS/MS | Thermo Scientific | Trace 1300 TSQ9000 |
| Column | Thermo Scientific | TG-5MS |
| Nitrogen Evaporator | Crescent Scientific | Rapid |
| Refrigerator | LG | GL-335VVG4/2012 |

Analytical method (GC-MS/MS Conditions):

| Instrument | Thermo Scientific Trace 1300 TSQ9000 |  |  |  |
| --- | --- | --- | --- | --- |
| Column | TG-5MS [30m × 0.25 mm (i.d.), 0.25 μm film thickness] |  |  |  |
| Column Flow | 1.0 mL/min |  |  |  |
| Carrier gas | Helium |  |  |  |
| Injection volume | 1 μL |  |  |  |
| Detector | MS/MS |  |  |  |
| Oven Temperature | Initial 80°C, hold for 2 minutes, and ramp 10°C/minute up to 150°C, hold for 8 minutes |  |  |  |
| Injector Temperature | 250°C |  |  |  |
| Split | 1:20 |  |  |  |
| Run time | 17 min |  |  |  |
| MS Transfer Line Temperature | 250°C |  |  |  |
| Ion source Temperature | 230°C |  |  |  |
| Diluent | Ethyl acetate |  |  |  |
| Compound | Transition Type | Precursor ion (m/z) | Product ion (m/z) | Collision Energy (CE) |
| 2,4-DCA | Quantitative | 176 | 160.9 | 20 |
|  | Confirmatory | 176 | 132.9 | 10 |

Extraction procedure:

For residues analysis of 2,4-Dichloroanisole, about 10 g of homogenized soil sample was taken in 50 mL polypropylene tubes. To the tubes containing soil samples, 10 mL of ethyl acetate was added followed by about 6 g of sodium sulphate and the tubes were vortexed thoroughly for the extraction of the residues of 2, 4-Dichloroanisole. About 3 mL of the upper aliquot of the ethyl acetate layer was taken and the samples were filtered with 0.2 μm syringe filter and the clear solution was injected into GC-MS/MS for analysis.

Results and discussions

Specificity/Selectivity:

Specificity of the method was determined by analyzing diluent, mobile phase, standard solution, matrix blank and matrix at LOQ level.

Specificity meets the acceptance criterion less than 30.0% of the LOQ.

Matrix effect:

Matrix effect was found to be less than 20% for both ion mass transitions.

Linearity:

Linearity solutions of six different concentrations were prepared in order to cover the range from about 30% of the LOQ to 20% above the highest level. Linearity curve was plotted between concentrations versus detector response. The correlation coefficient, slope and intercept were calculated.

Recovery (Accuracy) and Precision (Repeatability):

Assay accuracy was checked at two concentration levels, LOQ (0.05 mg/kg), 10 × LOQ (0.5 mg/kg) and 2 control samples were prepared. Five determinations were made at each concentration level for 2,4-DCA was added prior to extraction. The assay accuracy in terms of percentage recovery and the precision in terms of relative standard deviation were determined.

The recoveries of all the samples analysed were in the range of 70-110% with % RSD ≤ 20%.

Stability of Extracts:

Sample extracts fortified at the 10 × LOQ level (Five Replicates) were stored in a refrigerator at 2-8°C up to 15 days along with one control sample. After 15 days of storage, the 10 LOQ samples of five replicates and one control samples which were stored in refrigerator at 2-8ºC were taken out and allowed to attain room temperature and kept for analysis.

Stability meets the acceptance criterion for 15 days at 2-8ºC as the % recovery obtained are within the range of 70% to 120% of the nominal concentration.

Table A 18:	Recovery results from method validation of 2,4-DCA using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soil | 2,4-DCA | 0.05 | 87.98 | 4.66 | Quantifier transition |
|  |  | 0.5 | 87.82 | 1.84 |  |
|  |  | 0.05 | 88.40 | 4.43 | Confirmatory transition |
|  |  | 0.5 | 87.09 | 1.97 |  |

Table A 19:	Characteristics for the analytical method used for validation of 2,4-DCA residues in soil

|  | 2,4-DCA |
| --- | --- |
| Specificity | Interference < 30.0% to the LOQ level, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression y = 28348.6084x + 31.923 r > 0.99 |
| Calibration range | 0.0347 – 0.5977 mg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.03 mg/kg LOQ = 0.05 mg/kg |

Conclusion

The method validation parameters carried out for the analyte 2,4-Dichloroanisole meets the acceptance criteria of SANTE/2020/12830 rev. 1. The results obtained are within the specified limits, thus the method is suitable for the analysis of 2,4-Dichloroanisole in Soil.

      - Description of Methods for the Analysis of Water (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.5 |
| --- | --- |
| Report | Validation of the analytical methodology used for the determination of (2,4-dichlorophenoxy) acetic acid in surface water, drinking water and ground water, Ayyavoo K., 2021, Report No. 10034/2021 |
| Guideline(s): | SANTE/2020/12830 rev. 1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to determine the analytical methodology for 2,4-dichlorophenoxyacetic acid (2,4-D) in surface water, drinking water and ground water by LC-MS/MS following the guideline SANTE/2020/12830 rev. 1.

Reference materials:

- 2,4-dichlorophenoxyacetic acid, Lot no. G130560; CAS 94-75-7, Purity 99.93%, purchased from Dr. Ehrenstorfer

Reagents:

- Acetonitrile, grade LC-MS, Batch No. 0000252086, Purity ≥99.9%, purchased from J. T. Baker

- Formic Acid, grade AR, Batch No. R290F18, Purity 98-100%, purchased from Rankem

Equipment:

- Analytical Balance, Shimadzu, AUW220D

- Liquid Chromatograph Mass Spectrometer, Shimadzu, LC-MS-8045

- High  Performance Liquid Chromatograph, Shimadzu, LC-2030 plus

- Solid Phase Extraction Cartridge (SPE), 1 mL, Supelco, Discovery ® DSC-MCAX

- Sonicator, Enertech, USC-300

- Fume cupboard, Technico

Analytical method:

- Column: Shimadzu, 3µm C18, 4.6  x 150 mm

- Mobile phase A: Acetonitrile (70%)

- Mobile phase B: 0.1% Formic acid (30%)

- Flow: 1 mL/min

- Retention time: ~ 2.2 min

- Injection volume: 3 μL

- Column temperature: 40°C

- Detector: Mass Detector (triple-qutrapole)

- Ionization mode: ESI-

- Run mode: MRM (First transition: 221.00 > 163.10 m/z; Second transition (confirmatory): 219.00 > 161.10 m/z)

- Elution: Isoctratic

Solutions preparation:

- Mobile phase A

A volume of 1000 mL LC-MS grade acetonitrile was taken into 1000 mL mobile phase bottle and sonicated the mixture.

- Mobile phase B

A volume of 1 mL Formic acid was taken into 1000 mL mobile phase bottle and made upto the mark with 999 mL of water then sonicated the mixture. This solution was equivalent to 0.1% of Formic acid.

- Calibration solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into each six different conical flasks with six different concentrations (by adding 0.01 mL, 0.1 mL, 0.4 mL, 0.7 mL, 1.0 mL, 1.2 mL of standard stock solution - about 10 µg/L of reference material in acetonitrile), and the samples were passed and collected through a solid phase extraction cartridge (SPE), packed with Cl8 packing materials. The concentrations 0.01 µg/L, 0.1 µg/L, 0.4 µg/L, 0.7 µg/L, 1 µg/L, 1.2 µg/L standard solutions were prepared to different extracts. The extract was acidified with 80 µl of 5% Formic acid and the respective standards were injected into LC-MS/MS.

- Test item solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into separate 12 different conical flasks, two conical flasks were used for control (non spiked sample solution) and 0.1 mL of specificity standard stock solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for lower concentration, and 1 mL of specificity standard solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for higher concentration. These samples were mixed well for homogeneity. The mixture was passed and collected through a solid phase extraction cartridge (SPE), packed with C18 packing materials along with 10 mL of acetonitrile. The collected extraction was evaporated near to dryness under a stream of nitrogen at 40°C and collected 10 mL of water samples (spiked and non-spiked solutions). The extract was transferred into a volumetric flask and acidified with 80 µL of 5% Formic acid. The respective spiked solutions along with control were injected into LC-MS/MS.

Results and discussions

Table A 20:	Recovery results from method validation of 2,4-D using the analytical method

| Matrix | Analyte | Fortification level (μg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | 2,4-D | 0.1 | 99.19 | 0.86 | First mass transition |
|  |  | 1 | 98.62 | 0.71 |  |
|  |  | 0.1 | 98.38 | 0.75 | Second mass transition |
|  |  | 1 | 99.78 | 0.45 |  |
| Drinking water |  | 0.1 | 97.04 | 6.68 | First mass transition |
|  |  | 1 | 99.45 | 1.14 |  |
|  |  | 0.1 | 95.07 | 3.49 | Second mass transition |
|  |  | 1 | 97.66 | 1.52 |  |
| Ground water |  | 0.1 | 99.41 | 1.87 | First mass transition |
|  |  | 1 | 98.72 | 0.73 |  |
|  |  | 0.1 | 98.51 | 1.34 | Second mass transition |
|  |  | 1 | 99.66 | 0.48 |  |

Table A 21:	Characteristics for the analytical method used for validation of 2,4-D residues in water

|  | 2,4-D |
| --- | --- |
| Specificity | No interference, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression for surface water y = 6079082x + 25824 r > 0.99 Linear regression for drinking water y = 5296897x - 18473 r > 0.99 Linear regression for ground water y = 5322749x - 26835 r > 0.99 |
| Calibration range | 0.01 – 1.2 μg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.01 μg/L LOQ = 0.1 μg/L |

Conclusion

From the determined results, the 2,4-dichlorophenoxyacetic acid content in surface water, drinking water, ground water is in detection limits; LOQ = 0.1 µg/L (for surface water, drinking water, ground water). All the validation parameter results comply with SANTE/2020/12830 rev. 1.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.6 |
| --- | --- |
| Report | Validation of the analytical methodology used for the determination of 2,4-dichlorophenol in surface water, drinking water and ground water, Ayyavoo K., 2022, Report no. 9753/2021 |
| Guideline(s): | SANTE/2020/12830 rev. 1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to determine the analytical methodology for 2,4-dichlorophenol (2,4-DCP) in surface water, drinking water and ground water by LC-MS/MS following the guideline SANTE/2020/12830 rev. 1.

Reference materials:

- 2,4-dichlorophenol, Lot no. BCBT6806; CAS 120-83-2, Purity 98.97%, purchased from Lubaq

Reagents:

- Acetonitrile, grade LC-MS, Batch No. 0000252086, Purity ≥99.9%, purchased from J. T. Baker

- Formic Acid, grade AR, Batch No. R290F18, Purity 98-100%, purchased from Rankem

Equipment:

- Analytical Balance, Shimadzu, AUW220D

- Liquid Chromatograph Mass Spectrometer, Shimadzu, LC-MS-8045

- High  Performance Liquid Chromatograph, Shimadzu, LC-2030 plus

- Solid Phase Extraction Cartridge (SPE), 1 mL, Supelco, Discovery ® DSC-MCAX

- Sonicator, Enertech, USC-300

- Fume cupboard, Technico

Analytical method:

- Column: Shimadzu, 3µm C18, 4.6  x 150 mm

- Mobile phase A: Acetonitrile (90%)

- Mobile phase B: 0.1% Formic acid (10%)

- Flow: 0.9 mL/min

- Retention time: ~ 3.8 min

- Injection volume: 0.9 μL

- Column temperature: 40°C

- Detector: Mass Detector (triple-qutrapole)

- Ionization mode: APCI-

- Run mode: MRM (First transition: 161.10 > 125.00 m/z; Second transition (confirmatory): 161.10 > 89.10 m/z)

- Elution: Isoctratic

Solutions preparation:

- Mobile phase A

A volume of 1000 mL LC-MS grade acetonitrile was taken into 1000 mL mobile phase bottle and sonicated the mixture.

- Mobile phase B

A volume of 1 mL Formic acid was taken into 1000 mL mobile phase bottle and made upto the mark with 999 mL of water then sonicated the mixture. This solution was equivalent to 0.1% of Formic acid.

- Calibration solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into each six different conical flasks with six different concentrations (by adding 0.01 mL, 0.1 mL, 0.4 mL, 0.7 mL, 1.0 mL, 1.2 mL of standard stock solution - about 10 µg/L of reference material in acetonitrile), and the samples were passed and collected through a solid phase extraction cartridge (SPE), packed with Cl8 packing materials. The concentrations 0.01 µg/L, 0.1 µg/L, 0.4 µg/L, 0.7 µg/L, 1 µg/L, 1.2 µg/L standard solutions were prepared to different extracts. The extract was acidified with 80 µl of 5% Formic acid and the respective standards were injected into LC-MS/MS.

- Test item solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into separate 12 different conical flasks, two conical flasks were used for control (non spiked sample solution) and 0.1 mL of specificity standard stock solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for lower concentration, and 1 mL of specificity standard solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for higher concentration. These samples were mixed well for homogeneity. The mixture was passed and collected through a solid phase extraction cartridge (SPE), packed with C18 packing materials along with 10 mL of acetonitrile. The collected extraction was evaporated near to dryness under a stream of nitrogen at 40°C and collected 10 mL of water samples (spiked and non-spiked solutions). The extract was transferred into a volumetric flask and acidified with 80 µL of 5% Formic acid. The respective spiked solutions along with control were injected into LC-MS/MS.

Results and discussions

Table A 22:	Recovery results from method validation of 2,4-DCP using the analytical method

| Matrix | Analyte | Fortification level (μg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | 2,4-DCP | 0.1 | 94.21 | 2.52 | First mass transition |
|  |  | 1 | 92.59 | 1.39 |  |
|  |  | 0.1 | 92.36 | 2.64 | Second mass transition |
|  |  | 1 | 91.33 | 0.37 |  |
| Drinking water |  | 0.1 | 99.01 | 0.42 | First mass transition |
|  |  | 1 | 90.19 | 0.76 |  |
|  |  | 0.1 | 91.91 | 2.56 | Second mass transition |
|  |  | 1 | 93.61 | 1.12 |  |
| Ground water |  | 0.1 | 97.85 | 4.91 | First mass transition |
|  |  | 1 | 91.39 | 1.20 |  |
|  |  | 0.1 | 94.85 | 3.65 | Second mass transition |
|  |  | 1 | 92.97 | 3.96 |  |

Table A 23:	Characteristics for the analytical method used for validation of 2,4-DCP residues in water

|  | 2,4-DCP |
| --- | --- |
| Specificity | No interference, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression for surface water y = 10094x + 326.92 r > 0.99 Linear regression for drinking water y = 9409x + 756.22 r > 0.99 Linear regression for ground water y = 10003x + 630.83 r > 0.99 |
| Calibration range | 0.01 – 1.2 μg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.01 μg/L LOQ = 0.1 μg/L |

Conclusion

From the determined results, the 2,4-dichlorophenol content in surface water, drinking water, ground water is in detection limits; LOQ = 0.1 µg/L (for surface water, drinking water, ground water). All the validation parameter results comply with SANTE/2020/12830 rev. 1.

        - Analytical method 3

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.7 |
| --- | --- |
| Report | Validation of the analytical methodology used for the determination of 4-chlorophenol in surface water, drinking water and ground water, Ayyavoo K., 2022, Report no. 9754/2021 |
| Guideline(s): | SANTE/2020/12830 rev. 1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to determine the analytical methodology for 4-chlorophenol (4-CP) in surface water, drinking water and ground water by LC-MS/MS following the guideline SANTE/2020/12830 rev. 1.

Reference materials:

- 4-chlorophenol, Batch no. WXBC0117V; CAS 106-48-9, Purity 99.5%, purchased from Sigma-Aldrich

Reagents:

- Acetonitrile, grade LC-MS, Batch No. 0000252086, Purity ≥99.9%, purchased from J. T. Baker

- Formic Acid, grade AR, Batch No. R290F18, Purity 98-100%, purchased from Rankem

Equipment:

- Analytical Balance, Shimadzu, AUW220D

- Liquid Chromatograph Mass Spectrometer, Shimadzu, LC-MS-8045

- High  Performance Liquid Chromatograph, Shimadzu, LC-2030 plus

- Solid Phase Extraction Cartridge (SPE), 1 mL, Supelco, Discovery ® DSC- MCAX

- Sonicator, Enertech, USC-300

- Fume cupboard, Technico

Analytical method:

- Column: Shimadzu, 3µm C18, 4.6  x 150 mm

- Mobile phase A: Acetonitrile (90%)

- Mobile phase B: 0.1% Formic acid (10%)

- Flow: 0.9 mL/min

- Retention time: ~ 3.6 min

- Injection volume: 0.9 μL

- Column temperature: 40°C

- Detector: Mass Detector (triple-qutrapole)

- Ionization mode: APCI-

- Run mode: MRM (First transition: 127.20 > 125.30 m/z; Second transition (confirmatory): 127.20 > 91.10 m/z)

- Elution: Isoctratic

Solutions preparation:

- Mobile phase A

A volume of 1000 mL LC-MS grade acetonitrile was taken into 1000 mL mobile phase bottle and sonicated the mixture.

- Mobile phase B

A volume of 1 mL Formic acid was taken into 1000 mL mobile phase bottle and made upto the mark with 999 mL of water then sonicated the mixture. This solution was equivalent to 0.1% of Formic acid.

- Calibration solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into each six different conical flasks with six different concentrations (by adding 0.01 mL, 0.1 mL, 0.4 mL, 0.7 mL, 1.0 mL, 1.2 mL of standard stock solution - about 10 µg/L of reference material in acetonitrile), and the samples were passed and collected through a solid phase extraction cartridge (SPE), packed with Cl8 packing materials. The concentrations 0.01 µg/L, 0.1 µg/L, 0.4 µg/L, 0.7 µg/L, 1 µg/L, 1.2 µg/L standard solutions were prepared to different extracts. The extract was acidified with 80 µl of 5% Formic acid and the respective standards were injected into LC-MS/MS.

- Test item solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into separate 12 different conical flasks, two conical flasks were used for control (non spiked sample solution) and 0.1 mL of specificity standard stock solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for lower concentration, and 1 mL of specificity standard solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for higher concentration. These samples were mixed well for homogeneity. The mixture was passed and collected through a solid phase extraction cartridge (SPE), packed with C18 packing materials along with 10 mL of acetonitrile. The collected extraction was evaporated near to dryness under a stream of nitrogen at 40°C and collected 10 mL of water samples (spiked and non-spiked solutions). The extract was transferred into a volumetric flask and acidified with 80 µL of 5% Formic acid. The respective spiked solutions along with control were injected into LC-MS/MS.

Results and discussions

Table A 24:	Recovery results from method validation of 4-CP using the analytical method

| Matrix | Analyte | Fortification level (μg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | 4-CP | 0.1 | 94.93 | 3.99 | First mass transition |
|  |  | 1 | 97.1 | 1.17 |  |
|  |  | 0.1 | 97.38 | 1.61 | Second mass transition |
|  |  | 1 | 98.19 | 0.31 |  |
| Drinking water |  | 0.1 | 95.09 | 2.30 | First mass transition |
|  |  | 1 | 96.73 | 1.27 |  |
|  |  | 0.1 | 98.18 | 1.27 | Second mass transition |
|  |  | 1 | 98.57 | 1.01 |  |
| Ground water |  | 0.1 | 92.99 | 2.26 | First mass transition |
|  |  | 1 | 98.24 | 1.36 |  |
|  |  | 0.1 | 98.44 | 0.49 | Second mass transition |
|  |  | 1 | 96.76 | 0.08 |  |

Table A 25:	Characteristics for the analytical method used for validation of 4-CP residues in water

|  | 4-CP |
| --- | --- |
| Specificity | No interference, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression for surface water y = 1729x + 65.78 r > 0.99 Linear regression for drinking water y = 1644x + 14.73 r > 0.99 Linear regression for ground water y = 1003x + 17.93 r > 0.99 |
| Calibration range | 0.01 – 1.2 μg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.01 μg/L LOQ = 0.1 μg/L |

Conclusion

From the determined results, the 4-chlorophenol content in surface water, drinking water, ground water is in detection limits; LOQ = 0.1 µg/L (for surface water, drinking water, ground water). All the validation parameter results comply with SANTE/2020/12830 rev. 1.

        - Analytical method 4

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.8 |
| --- | --- |
| Report | Validation of the analytical methodology used for the determination of 2,4-dichloroanisole in surface water, drinking water and ground water, Ayyavoo K., 2021, Report no. 9752/2021 |
| Guideline(s): | SANTE/2020/12830 rev. 1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to determine the analytical methodology for 2,4-dichloroanisole (2,4-DCA) in surface water, drinking water and ground water by LC-MS/MS following the guideline SANTE/2020/12830 rev. 1.

Reference materials:

- 2,4-dichloroanisole, Lot no. AB126634; CAS 553-82-2, Purity 99.71%, purchased from Lubaq

Reagents:

- Acetonitrile, grade LC-MS, Batch No. 0000252086, Purity ≥99.9%, purchased from J. T. Baker

- Formic Acid, grade AR, Batch No. R290F18, Purity 98-100%, purchased from Rankem

Equipment:

- Analytical Balance, Shimadzu, AUW220D

- Liquid Chromatograph Mass Spectrometer, Shimadzu, LC-MS-8045

- High  Performance Liquid Chromatograph, Shimadzu, LC-2030 plus

- Solid Phase Extraction Cartridge (SPE), 1 mL, Supelco, Discovery ® DSC- MCAX

- Sonicator, Enertech, USC-300

- Fume cupboard, Technico

Analytical method:

- Column: Shimadzu, 3µm C18, 4.6  x 150 mm

- Mobile phase A: Acetonitrile (90%)

- Mobile phase B: 0.1% Formic acid (10%)

- Flow: 0.9 mL/min

- Retention time: ~ 3.8 min

- Injection volume: 0.9 μL

- Column temperature: 40°C

- Detector: Mass Detector (triple-qutrapole)

- Ionization mode: ACPI-

- Run mode: MRM (First transition: 178.00 > 163.00 m/z; Second transition (confirmatory): 178.00 > 135.10 m/z)

- Elution: Isoctratic

Solutions preparation:

- Mobile phase A

A volume of 1000 mL LC-MS grade acetonitrile was taken into 1000 mL mobile phase bottle and sonicated the mixture.

- Mobile phase B

A volume of 1 mL Formic acid was taken into 1000 mL mobile phase bottle and made upto the mark with 999 mL of water then sonicated the mixture. This solution was equivalent to 0.1% of Formic acid.

- Calibration solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into each six different conical flasks with six different concentrations (by adding 0.01 mL, 0.1 mL, 0.4 mL, 0.7 mL, 1.0 mL, 1.2 mL of standard stock solution - about 10 µg/L of reference material in acetonitrile), and the samples were passed and collected through a solid phase extraction cartridge (SPE), packed with Cl8 packing materials. The concentrations 0.01 µg/L, 0.1 µg/L, 0.4 µg/L, 0.7 µg/L, 1 µg/L, 1.2 µg/L standard solutions were prepared to different extracts. The extract was acidified with 80 µl of 5% Formic acid and the respective standards were injected into LC-MS/MS.

- Test item solutions (extraction and purification procedure)

A volume of 10 mL either surface, drinking or ground water was taken into separate 12 different conical flasks, two conical flasks were used for control (non spiked sample solution) and 0.1 mL of specificity standard stock solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for lower concentration, and 1 mL of specificity standard solution (about 10 µg/L of reference material in acetonitrile) was added into 5 conical flasks for higher concentration. These samples were mixed well for homogeneity. The mixture was passed and collected through a solid phase extraction cartridge (SPE), packed with C18 packing materials along with 10 mL of acetonitrile. The collected extraction was evaporated near to dryness under a stream of nitrogen at 40°C and collected 10 mL of water samples (spiked and non-spiked solutions). The extract was transferred into a volumetric flask and acidified with 80 µL of 5% Formic acid. The respective spiked solutions along with control were injected into LC-MS/MS.

Results and discussions

Table A 26:	Recovery results from method validation of 2,4-DCA using the analytical method

| Matrix | Analyte | Fortification level (μg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | 2,4-DCA | 0.1 | 96.11 | 7.75 | First mass transition |
|  |  | 1 | 108.9 | 2.72 |  |
|  |  | 0.1 | 95.94 | 6.24 | Second mass transition |
|  |  | 1 | 104.98 | 3.32 |  |
| Drinking water |  | 0.1 | 98.39 | 2.30 | First mass transition |
|  |  | 1 | 105.64 | 2.35 |  |
|  |  | 0.1 | 99.5 | 2.12 | Second mass transition |
|  |  | 1 | 105.44 | 3.38 |  |
| Ground water |  | 0.1 | 100.57 | 2.05 | First mass transition |
|  |  | 1 | 105.94 | 1.98 |  |
|  |  | 0.1 | 101.12 | 1.42 | Second mass transition |
|  |  | 1 | 107.55 | 1.70 |  |

Table A 27:	Characteristics for the analytical method used for validation of 2,4-DCA residues in water

|  | 2,4-DCA |
| --- | --- |
| Specificity | No interference, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression for surface water y = 415543x + 6466.6 r > 0.99 Linear regression for drinking water y = 410383x - 3346.8 r > 0.99 Linear regression for ground water y = 529591x + 5271.4 r > 0.99 |
| Calibration range | 0.01 – 1.2 μg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.01 μg/L LOQ = 0.1 μg/L |

Conclusion

From the determined results, the 2,4-dichloroanisole content in surface water, drinking water, ground water is in detection limits; LOQ = 0.1 µg/L (for surface water, drinking water, ground water). All the validation parameter results comply with SANTE/2020/12830 rev. 1.

      - Description of Methods for the Analysis of Air (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.9 |
| --- | --- |
| Report | Validation of Analytical Method for the Determination of 2,4-D Acid in Air by Liquid Chromatography-Mass Spectrometry, Suresh Gundoju, 2020, Report No. BIO-ANM 1356 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to validate analytical method for the determination of 2,4-D in Air according to test guidelines SANCO/825/00-rev 8.1 and SANCO 3029/99 Rev.4 by LC-MS/MS.

Reference materials:

| Reference Standard 1 | 2,4-D |
| --- | --- |
| CAS No. | 94-75-7 |
| Physical Appearance | White powder |
| Lot No. | BCBX3937 |
| Formula | Cl2C6H3OCH2CO2H |
| Purity | 99.9% |
| Batch Produced by | Sigma-Aldrich |
| Date of Expiry | May 2023 |
| Storage Condition | Room Temperature |

Reagents:

| Name | Source | Batch/Lot No. |
| --- | --- | --- |
| Acetonitrile | JT Baker | 0000221770 |
| Amberlite® XAD4 | Sigma-Aldrich | SLBT7634 |
| Acetic acid | Sigma-Aldrich | BCBW1549 |
| Milli-Q water | In-house | - |

Equipment:

| Name | Make | Model |
| --- | --- | --- |
| Balance | Sartorius | CPA225D |
| Vortex shaker | Tarsons | SPINX-3020 |
| Ultrasonic cleaner | Servewell Instruments Pvt. Ltd. | GT-1860QTS |
| LC-MS/MS | Agilent Technologies | Ultivo 6465A |
| Column | Agilent Technologies | Eclipse Plus |
| Deep freezer | Carrier | HF 500CHP |
| Flow Meter (Rota Meter) | MIGFITT | - |
| Vacuum pump | Unisource industrial | DT-75 |
| Water Purification System | Millipore Q8UV | SMARTPAY DIRECT |
| Pipettes | Thermo Scientific | FINNPIPETTE®F2 |

Analytical method (LC-MS/MS Conditions):

| Instrument |  | Agilent HPLC 1260 Infinity II with Agilent Ultivo ESI-MS/MS system |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Column |  | Agilent Eclipse plus C18 RRHD [50 m × 2.1 mm (i.d.) × 1.8 μm film thickness] |  |  |  |  |
| Mobile phase A |  | 0.1% formic acid in Milli Q-Water (60%) |  |  |  |  |
| Mobile phase B |  | Acetonitrile (40%) |  |  |  |  |
| Flow rate |  | 0.3 mL/min |  |  |  |  |
| Injection volume |  | 5 μL |  |  |  |  |
| Column compartment |  | 40°C |  |  |  |  |
| Sample compartment Temperature |  | 10°C |  |  |  |  |
| Run time |  | 4.0 min |  |  |  |  |
| Diluent |  | Acetonitrile |  |  |  |  |
| Retention time |  | 1.7 min |  |  |  |  |
| Mass Spectrometry Operating Conditions: |  |  |  |  |  |  |
| Inetrface | Electro Spray Ionization |  |  |  |  |  |
| Polarity | Negative |  |  |  |  |  |
| Run Mode | MRM |  |  |  |  |  |
| Gas temperature | 250°C |  |  |  |  |  |
| Sheath Gas Temperature | 300°C |  |  |  |  |  |
| Compound | Transition Type |  | Precursor ion (m/z) | Product ion (m/z) | Fragment voltage (FV) | Collision Energy (CE) |
| 2,4-D | Quantitative |  | 219 | 160.9 | 79 | 5 |
|  | Confirmatory |  | 219 | 124.9 | 79 | 25 |

Principle of the Method:

Air sampling was carried out with absorption tubes/cartridges filled with Amberlite® XAD4 adsorption material. Subsequent extraction was carried out with acetonitrile. Extracts were diluted suitably depending on fortification level using acetonitrile/water (50/50%, v/v) prior to LC-MS/MS analysis.

Extraction Procedure:

Sample cartridges were connected to the vacuum pump with flexible tubing. Absorption cartridge intake was positioned at 2 meters above ground level. After the absorption cartridge was correctly inserted and positioned, the power switch of vacuum pump was turned on and the sampling begins. The flow was controlled with calibrated rota meter. Air at defined temperature (such as 20±2°C and 35±2°C) was passed through cartridges at a flow rate of 2.5 L/min for a period of minimum 6 hours. After passing defined volume of air, the cartridges were extracted thrice with 10 mL of acetonitrile. The extract was further diluted suitably prior to LC-MS/MS analysis using acetonitrile/water (50/50%, v/v).

Extraction Efficiency:

Cartridges were fortified with 2,4-D standard solution at 10 × LOQ level. Five determinations were made along with a blank. In case of blank cartridges were fortified with acetonitrile containing no analyte. Subsequently, cartridges were extracted thrice with 10 mL acetonitrile and pooled extract was further diluted using acetonitrile/water (50/50%, v/v) prior to LC-MS/MS analysis.

Results and discussions

Specificity/Selectivity:

Specificity of the method was determined by analyzing diluent, mobile phase (50:50%, v/v), standard solution, matrix blank and matrix at LOQ level.

Due to the selective nature of the detection of this method (two individual ion transitions monitored), interference was found to be less than 30% to the peak area measured for target analyte at the limit of quantification level.

Linearity:

Linearity was assessed by preparing six different concentrations of 2,4-D standard in diluent. Linearity curve was plotted between concentrations versus detector response. The correlation coefficient, slope and intercept were calculated.

Recovery (Accuracy) and Precision (Repeatability):

Recovery of the method was determined at two fortification levels at LOQ (4.5 μg/m3) and 10×LOQ (45 μg/m3) along with two control samples. Five determination were made at each concentration level for 2,4-D. The assay accuracy was calculated in terms of percentage recovery and the precision in terms of relative standard deviation. All fortifications were made directly to the sorbent material. Recovery of the method was demonstrated at 20±2°C and 35±2°C.

During method validation, acceptable recoveries were generated for the samples fortified at LOQ and 10×LOQ level. The %RSD (precision) was < 20% at each fortification level. Recovery data from these samples demonstrated that 2,4-D residues were stable during analysis.

Extraction Efficiency:

Extraction efficiency was calculated in terms of recovery and the obtained recoveries were in the range of 104.83% - 93.32% (n=5) for quantitative transition.

Table A 28:	Recovery results from method validation of 2,4-D using the analytical method

| Matrix | Analyte | Fortification level (μg/m3) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Ambient air at 20±2°C | 2,4-D | 4.5 | 98.15 | 3.29 | Quantifier transition |
|  |  | 45 | 97.29 | 4.85 |  |
|  |  | 4.5 | 95.17 | 5.57 | Confirmatory transition |
|  |  | 45 | 96.35 | 5.09 |  |
| Warm humid air at 35±2°C |  | 4.5 | 98.80 | 4.11 | Quantifier transition |
|  |  | 45 | 103.81 | 2.89 |  |
|  |  | 4.5 | 91.60 | 6.69 | Confirmatory transition |
|  |  | 45 | 100.09 | 4.80 |  |

Table A 29:	Characteristics for the analytical method used for validation of 2,4-D residues in air

|  | 2,4-D |
| --- | --- |
| Specificity | Interference < 30.0% to the LOQ level, the method is specific |
| Calibration (type, number of data points) | 6 concentration points Linear regression y = 249.35x - 43.461 r > 0.99 |
| Calibration range | 5.0 – 205 ng/mL |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 4.5 μg/m3 |

Conclusion

The analytical method was validated for the determination of 2,4-D in ambient and in warm, humid air with a limit of quantification (LOQ) of 4.5 μg/m3. The results are within guideline requirements of SANTE/2020/12830 rev. 1.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted.

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted.

  - Analytical methods for MCPA

    - Methods used for the generation of pre-authorization data (KCP 5.1)

Please refer to the post-autorization methods.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.10 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of residues of MCPA (CAS: 94-74-6), in soil by LC/MS, M. Rubino, 2018, Report No. 18.647485.0001 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method, internally codified as SOPa-447-LABCHI rev.0 for the determination of MCPA (CAS: 94-74-6) in soil. Soil was used as representative matrix according to SANCO/3029/99 rev. 4 SANCO/825/00 Rev. 8.1 and OECD-204/2014 guidelines.

Reference materials:

- MCPA, IUPAC name: 2-(4-chloro-2-methylphenoxy)acetic acid, Batch No. 123941; CAS 94-74-6, Purity 99.5%, ID Tl-0013482, purchased from Dr. Ehrenstorfer

Reagents:

- milliQ water, SRA 787

- Methanol, ID: Tl-0033474 purchased from VWR

- Acetonitrile, ID: Tl-0033026 purchased from VWR

- Magnesium sulphate anhydrous, ID: Tl-0025582 purchased from Sigma-Aldrich

- Sodium chloride, ID: Tl-0024842 purchased from VWR

- Formic acid, ID: Tl-0018641, purchased from Merck

- Acetic acid glacial, ID: Tl-0012723, purchased from VWR

- Ammonium formate, ID: Tl-0023118, purchased Sigma-Aldrich

- Sulphuric acid solution 2 N, ID: Tl-0023103, purchased from VWR

- Sodium Hydroxide solution 1N, ID: Tl-0025146, purchased from VWR

Equipment:

- Common analytical glassware

- Fridge, SRA 7

- Analytical balance (± 0.01 mg), SRA 602

- Vortex

- Centrifuge, SRA 638

- Thermostatic bath equipped with N2 flow, SRA 66

- MS XEVO TQS (Waters-Micromass), SRA 470

- Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 µm (ID: LC 120)

Analytical method:

- Column: Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 μm

- Mobile phase A: 0.1% formic acid in water

- Mobile phase B: 0.1% formic acid in methanol

- Flow: 0.2 mL/min

- Retention time: ~ 3.9 min

- Injection volume: 5 μL

- Detector: MS XEVO TQS (Waters-Micromass), SRA 470

- Ionization mode: ESI-

- Source temp.: 150°C

- Nebulizer: 6 bar

- Cone gas: 150 L/h

- Desolvation gas: 800 L/h

- Run time: 8 min

- Run mode: MRM (First transition: 199 > 141 m/z; Second transition (confirmatory): 199 > 105 m/z)

- Elution: Gradien

Solutions preparation:

- Mobile phase A (0.1% formic acid in water)

1 mL of formic acid was added in a 1000 mL volumetric flask containing about 500 mL of water and then the solution was diluted to volume with milliQ water.

- Mobile phase B (0.1% formic acid in methanol)

1 mL of formic acid were added in a 1000 mL volumetric flask containing about 500 mL of methanol and the solution was diluted to volume with methanol.

- Blank solution (10mM ammonium formate pH 4: methanol, 50:50 (v/v))

About 0.62 g of ammonium formate were accurately weighed (± 0.01 g) into a 1000 ml volumetric flask and dissolved with about 500 ml of milliQ water. 0.22 ml of formic acid were added and then the solution was diluted to volume with milliQ water. pH was 4 (10mM ammonium formate pH 4). 10mM ammonium formate pH 4: methanol, 50:50 (v/v).

- Extraction mixture (0.5% acetic acid in acetonitrile)

In a 200 ml volumetric flask containing about 50 ml of acetonitrile, about 1 ml of acetic acid was introduced and then, diluted to volume with acetonitrile.

Extraction procedure:

About 6.00 g soil were introduced into a 50 mL falcon. 10 mL of milliQ water were introduced in order to hydrate the matrix. 1 mL of sodium hydroxide solution 1 N was added to the sample and vortexed for about 1 min. After 30 minutes the extract was neutralized with 2.7 mL of sulphuric acid solution 2N (pH ~ 1). Then, 10 mL of extraction mixture were added to the sample. After vortexing for about 1min, 1 g of sodium chloride and 4 g of magnesium sulphate anhydrous were added to the sample. Vortexed for about 1 min and centrifuged at 4700 rpm for 3 min. 1.5 mL of supernatant were transferred in 10 mL tube and dried by nitrogen flux. The dried sample was resuspended with 0.5 mL of blank solution. Vortexed for about 1min, then filtered and transferred into an HPLC vial and injected. The sample was prepared in duplicate.

Results and discussions

Table A 30:	Recovery results from method validation of MCPA using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soil | MCPA | 0.01 | 90 | 5 | First mass transition |
|  |  | 0.1 | 81 | 8 |  |
|  |  | 0.01 | 90 | 6 | Second mass transition |
|  |  | 0.1 | 82 | 8 |  |

Table A 31:	Characteristics for the analytical method used for validation of MCPA residues in soil

|  | MCPA |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) were detected at RT of the target analyte in the blank and test solution compared to the spiked test solution for both transitions. The method is specific |
| Calibration (type, number of data points) | 5 concentration points Linear regression for first mass transition y = 1306534x r > 0.99 Linear regression for second mass transition y = 8812x r > 0.99 |
| Calibration range | 0.003 – 0.35 mg/kg |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.003 mg/kg LOQ = 0.01 mg/kg |

Conclusion

According to SANTE/2020/12830 rev. 1 the method was successfully validated and is suitable for determination of MCPA residues in soil.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.11 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of PCOC (CAS: 1570-64-5) in soil by LC-MS, M. Rubino, 2019, Report No. 19.509954.0001 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method, internally codified as SOPa-491-LACHI rev.0 for the determination of residues of PCOC (CAS: 1570-64-5) in soil. Soil samples were used as representative matrix, according to SANCO/3029/99 rev. 4 SANCO/825/00 rev. 8.1 and OECD-204/2014 guidelines.

Reference materials:

- PCOC, IUPAC name: 4-chloro-2-methylphenol; Batch No. G159146, CAS 1570-64-5, Purity 99.23%, ID Tl-0043314, purchased from Dr. Ehrenstorfer

Reagents:

- milliQ water, SRA 787

- Methanol, ID: Tl-0038417 purchased from VWR

- Sodium Hydroxide (NaOH) 0.1N solution, ID: Tl-0042986, purchased from Merck

Equipment:

- Common analytical glassware

- Fridge, SRA 7

- Analytical balance (± 0.1 mg), SRA 602

- Analytical balance (± 0.01 mg), SRA 768

- Vortex

- Centrifuge, SRA 638

- Ultracentrifuge, SRA 38

- Electronic pipette. SRA 847

- MS XEVO TQS (Waters-Micromass), SRA 470

- Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 µm (ID: LC 350)

Analytical method:

- Column: Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 μm

- Mobile phase A: Water

- Mobile phase B: Methanol

- Flow: 0.2 mL/min

- Retention time: ~ 6.7 min

- Injection volume: 10 μL

- Detector: MS XEVO TQS (Waters-Micromass), SRA 470

- Ionization mode: ESI-

- Source temp.: 150°C

- Nebulizer: 6 bar

- Cone gas: 150 L/h

- Desolvation gas: 800 L/h

- Run time: 13 min

- Run mode: MRM (First transition: 141 > 105 m/z; Second transition (confirmatory): 141 > 35 m/z)

- Elution: Gradien

Solutions preparation:

- Extraction phase and blank solution

8:2 mixture of 0.01N NaOH solution and methanol (400 mL)

Extraction procedure:

About 2 g of soil were weighed into a 10 mL glass tube and extracted with exactly 2 mL of extraction phase. The sample was stirred and centrifuged (2000 rpm, 6 min) then the supernatant was transferred into a second glass tube. The extraction was performed twice with the same amount of solvent and the two extracts were combines. An aliquot was ultracentrifuged (15000 rpm, 5 min) and injected into LC-MS. The sample was prepared in duplicate.

Results and discussions

Table A 32:	Recovery results from method validation of PCOC using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soil | PCOC | 0.01 | 87 | 5 | First mass transition |
|  |  | 0.1 | 92 | 3 |  |
|  |  | 0.01 | 94 | 2 | Second mass transition |
|  |  | 0.1 | 97 | 3 |  |

Table A 33:	Characteristics for the analytical method used for validation of PCOC residues in soil

|  | PCOC |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) were detected at RT of the target analyte in the blank and test solution compared to the spiked test solution for both transitions. The method is specific |
| Calibration (type, number of data points) | 5 concentration points Linear regression for first mass transition y = 511927x r > 0.99 Linear regression for second mass transition y = 388098x r > 0.99 |
| Calibration range | 0.002 – 0.37 mg/kg |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.002 mg/kg LOQ = 0.01 mg/kg |

Conclusion

According to SANTE/2020/12830 rev. 1 the method was successfully validated and is suitable for determination of PCOC residues in soil.

      - Description of Methods for the Analysis of Water (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.12 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of residues of MCPA (CAS: 94-74-6), in drinking water by LC/MS, M. Rubino, 2019, Report No. 18.647484.0007 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method, internally codified as SOPa-446-LABCHI rev.0 for the determination of MCPA (CAS: 94-74-6) in drinking water. Drinking water was used as representative matrix according to SANCO/3029/99 rev. 4 SANCO/825/00 Rev. 8.1 and OECD-204/2014 guidelines.

Reference materials:

- MCPA, IUPAC name: 2-(4-chloro-2-methylphenoxy)acetic acid, Batch No. 123941, CAS 94-74-6, Purity 99.5%, ID Tl-0013482, purchased from Dr. Ehrenstorfer

Reagents:

- milliQ water, SRA 787

- Methanol, ID: Tl-0034938 purchased from Honeywell

- Acetic acid, ID: Tl-0012723, purchased from VWR

- Formic acid, ID: Tl-0018641, purchased from Merck

- Ammonium formate, ID: Tl-0023118, purchased from Sigma-Aldrich

- Sodium Acetate, ID: Tl-0025575, purchased from Merck

- Oasis MAX 6cc Catridge, ID: Tl-0035455, purchased from Waters

Equipment:

- Common analytical glassware

- Fridge, SRA 7

- Vortex

- Analytical balance (± 0.01 mg), SRA 768

- Centrifuge, SRA 638

- Thermostatic bath equipped with N2 flow, SRA 66

- MS XEVO TQS (Waters-Micromass), SRA 470

- Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 µm (ID: LC 120)

Analytical method:

- Column: Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 μm

- Mobile phase A: 0.1% formic acid in water

- Mobile phase B: 0.1% formic acid in methanol

- Flow: 0.2 mL/min

- Retention time: ~ 3.8 min

- Injection volume: 10 μL

- Detector: MS XEVO TQS (Waters-Micromass), SRA 470

- Ionization mode: ESI-

- Source temp.: 150°C

- Nebulizer: 6 bar

- Cone gas: 150 L/h

- Desolvation gas: 800 L/h

- Run time: 8 min

- Run mode: MRM (First transition: 199 > 141 m/z; Second transition (confirmatory): 199 > 105 m/z)

- Elution: Gradien

Solutions preparation:

- Mobile phase A (0.1% formic acid in water)

1 mL of formic acid was added in a 1000 mL volumetric flask containing about 500 mL of water and then the solution was diluted to volume with milliQ water.

- Mobile phase B (0.1% formic acid in methanol)

1 mL of formic acid were added in a 1000 mL volumetric flask containing about 500 mL of methanol and the solution was diluted to volume with methanol.

- Blank solution (10mM ammonium formate pH 4: methanol, 50:50 (v/v))

About 0.62 g of ammonium formate were accurately weighed (± 0.01 g) into a 1000 ml volumetric flask and dissolved with about 500 ml of milliQ water. 0.22 ml of formic acid were added and then the solution was diluted to volume with milliQ water. pH was 4 (10mM ammonium formate pH 4). 10mM ammonium formate pH 4: methanol, 50:50 (v/v).

- Extraction phase 2% formic acid in methanol):

In a 100 ml volumetric flask containing about 50 ml of acetonitrile, about 2 ml of formic acid were introduced and then, diluted to volume with acetonitrile.

- Washing phase (50 mM Sodium Acetate pH 7):

About 4.1 g of sodium acetate were accurately weighed (± 0.1 g) into a 1000 ml volumetric flask and dissolved with about 900 ml of milliQ water. Added acetic acid to pH 7 and then the solution was diluted to volume with milliQ water.

Extraction procedure:

800.0 ml (± 0.1 ml) of drinking water were introduced into a becker. Then, the water was percolated using a regular vacuum through a MAX 6cc cartridge (previously conditioned with 5 ml of methanol, and then 5 ml of milliQ water) with a flow rate of 2 drop/sec. The cartridge was washed with 3 ml of washing phase and dried using centrifugation technique (2000 rpm, 2 min). The analyte was eluted with 8 ml of extraction phase and recovered in 10 ml of tube. The eluate was dried under nitrogen flux and resuspended with 0.5 ml of blank solution. Vortexed for about 1 min and transferred into an HPLC vial and injected. The sample was prepared in duplicate.

Results and discussions

Table A 34:	Recovery results from method validation of MCPA using the analytical method

| Matrix | Analyte | Fortification level (µg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Drinking water | MCPA | 0.01 | 104 | 5 | First mass transition |
|  |  | 0.1 | 87 | 9 |  |
|  |  | 0.01 | 101 | 7 | Second mass transition |
|  |  | 0.1 | 86 | 10 |  |

Table A 35:	Characteristics for the analytical method used for validation of MCPA residues in drinking water

|  | MCPA |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) were detected at RT of the target analyte in the blank and test solution compared to the spiked test solution for both transitions. The method is specific |
| Calibration (type, number of data points) | 5 concentration points Linear regression for first mass transition y = 1110.9x r > 0.99 Linear regression for second mass transition y = 7.8x r > 0.99 |
| Calibration range | 0.003 – 0.45 µg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.003 µg/L LOQ = 0.01 µg/L |

Conclusion

According to SANTE/2020/12830 rev. 1 the method was successfully validated and is suitable for determination of MCPA residues in drinking water.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Method is acceptable. |
| --- | --- |

| Reference: | KCP 5.2.13 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of residues of MCPA (CAS: 94-74-6), in surface water by LC/MS, M. Rubino, 2019, Report No. 18.647484.0006 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method, internally codified as SOPa-445-LABCHI rev.0 for the determination of MCPA (CAS: 94-74-6) in surface water. Surface water was used as representative matrix according to SANCO/3029/99 rev. 4 SANCO/825/00 Rev. 8.1 and OECD-204/2014 guidelines.

Reference materials:

- MCPA, IUPAC name: 2-(4-chloro-2-methylphenoxy)acetic acid, Batch No. 123941, CAS 94-74-6, Purity 99.5%, ID Tl-0013482, purchased from Dr. Ehrenstorfer

Reagents:

- milliQ water, SRA 787

- Methanol, ID: Tl-0034938 purchased from Honeywell

- Formic acid, ID: Tl-0018641, purchased from Merck

- Ammonium formate, ID: Tl-0023118, purchased from Sigma-Aldrich

- Oasis HLB 6cc Catridge, ID: Tl-0036812, purchased from Waters

Equipment:

- Common analytical glassware

- Fridge, SRA 7

- Vortex

- Analytical balance (± 0.01 mg), SRA 768

- Centrifuge, SRA 638

- Thermostatic bath equipped with N2 flow, SRA 66

- MS XEVO TQS (Waters-Micromass), SRA 470

- Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 µm (ID: LC 120)

Analytical method:

- Column: Acquity UPLC BEH C18, 50 mm x 2.1 mm x 1.7 μm

- Mobile phase A: 0.1% formic acid in water

- Mobile phase B: 0.1% formic acid in methanol

- Flow: 0.2 mL/min

- Retention time: ~ 3.5 min

- Injection volume: 10 μL

- Detector: MS XEVO TQS (Waters-Micromass), SRA 470

- Ionization mode: ESI-

- Source temp.: 150°C

- Nebulizer: 6 bar

- Cone gas: 150 L/h

- Desolvation gas: 800 L/h

- Run time: 8 min

- Run mode: MRM (First transition: 199 > 141 m/z; Second transition (confirmatory): 201 > 143 m/z)

- Elution: Gradien

Solutions preparation:

- Mobile phase A (0.1% formic acid in water)

1 mL of formic acid was added in a 1000 mL volumetric flask containing about 500 mL of water and then the solution was diluted to volume with milliQ water.

- Mobile phase B (0.1% formic acid in methanol)

1 mL of formic acid were added in a 1000 mL volumetric flask containing about 500 mL of methanol and the solution was diluted to volume with methanol.

- Blank solution (10mM ammonium formate pH 4: methanol, 50:50 (v/v))

About 0.62 g of ammonium formate were accurately weighed (± 0.01 g) into a 1000 ml volumetric flask and dissolved with about 500 ml of milliQ water. 0.22 ml of formic acid were added and then the solution was diluted to volume with milliQ water. pH was 4 (10mM ammonium formate pH 4). 10mM ammonium formate pH 4: methanol, 50:50 (v/v).

Extraction procedure:

50.0 ml (± 0.1 ml) of surface water were introduced into a becker. Then, the water was percolated using a regular vacuum through a HLB 6cc cartridge (previously conditioned with 5 ml of methanol, and then 5 ml of milliQ water) with a flow rate of 2 drop/sec. The cartridge was washed with 3 ml of milliQ water and dried using centrifugation technique (2000 rpm, 2 min). The analyte was eluted with 4 ml of methanol and recovered in 10 ml of tube. The eluate was dried under nitrogen flux and resuspended with 0.5 ml of blank solution. Vortexed for about 1 min, filter if necessary and transferred into an HPLC vial and injected. The sample was prepared in duplicate.

Results and discussions

Table A 36:	Recovery results from method validation of MCPA using the analytical method

| Matrix | Analyte | Fortification level (µg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | MCPA | 0.01 | 94 | 11 | First mass transition |
|  |  | 0.1 | 87 | 4 |  |
|  |  | 0.01 | 87 | 13 | Second mass transition |
|  |  | 0.1 | 82 | 5 |  |

Table A 37:	Characteristics for the analytical method used for validation of MCPA residues in surface water

|  | MCPA |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) were detected at RT of the target analyte in the blank and test solution compared to the spiked test solution for both transitions. The method is specific |
| Calibration (type, number of data points) | 5 concentration points Linear regression for first mass transition y = 2247x r > 0.99 Linear regression for second mass transition y = 676x r > 0.99 |
| Calibration range | 0.003 – 0.36 µg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.003 µg/L LOQ = 0.01 µg/L |

Conclusion

According to SANTE/2020/12830 rev. 1 the method was successfully validated and is suitable for determination of MCPA residues in surface water.

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted.

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted.