FINAL REGISTRATION REPORT Part B Section 8 Environmental Fate Detailed summary of the risk assessment

Product code: SHA 5148 A Product name: PLANET Chemical active substances: 2,4-D, 275 g/L (in form of DMA salt) MCPA, 275 g/L (in form of DMA salt)

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Sharda Cropchem Ltd. Submission date: October 2022 MS Finalisation date: Maj 2023; May 2024

Version history

| When | What |
| --- | --- |
| 05.2023 | ZRMs evaluated dRR updated by Applicant |
| 05.2024 | ZRMs prepared fRR after Applicant’s RT comments |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	5

8.1	Critical GAP and overall conclusions	5

8.2	Metabolites considered in the assessment	7

8.3	Rate of degradation in soil (KCP 9.1.1)	8

8.3.1	Aerobic degradation in soil (KCP 9.1.1.1)	8

8.3.1.1	2,4 D and its metabolites	8

8.3.1.2	MCPA	9

8.3.2	Anaerobic degradation in soil (KCP 9.1.1.1)	9

8.3.2.1	2,4 D and its metabolites	9

8.3.2.2	MCPA	10

8.4	Field studies (KCP 9.1.1.2)	10

8.4.1	Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)	10

8.4.2	Soil accumulation testing (KCP 9.1.1.2.2)	10

8.5	Mobility in soil (KCP 9.1.2)	10

8.5.1	2,4 D and its metabolites	10

8.5.2	MCPA	13

8.5.3	Column leaching (KCP 9.1.2.1)	13

8.5.3.1	2,4 D and its metabolites	13

8.5.3.2	MCPA	13

8.5.4	Lysimeter studies (KCP 9.1.2.2)	13

8.5.4.1	2,4 D and its metabolites	13

8.5.4.2	MCPA	13

8.5.5	Field leaching studies (KCP 9.1.2.3)	13

8.5.5.1	2,4 D and its metabolites	13

8.5.5.2	MCPA	14

8.6	Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)	14

8.6.1	2,4 D and its metabolites	14

8.6.2	MCPA	15

8.7	Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)	15

8.7.1	Justification for new endpoints	15

8.7.2	Active substance(s) and relevant metabolite(s)	15

8.7.2.1	2,4 D and its metabolites	16

8.7.2.2	MCPA	18

8.7.2.3	PECsoil of PLANET	18

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	19

8.8.1	Justification for new endpoints	19

8.8.2	Active substance and relevant metabolites (KCP 9.2.4.1)	19

8.8.2.1	2,4 D and its metabolites	20

8.8.2.2	MCPA	22

8.9	Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)	23

8.9.1	Justification for new endpoints	24

8.9.2	Active substance, relevant metabolites and the formulation (KCP 9.2.5)	24

8.9.2.1	2,4 D and its metabolites	25

8.9.2.2	MCPA	29

8.9.2.3	PECsw/sed of PLANET	31

8.10	Fate and behaviour in air (KCP 9.3, KCP 9.3.1)	32

8.10.1	2,4 D	32

8.10.2	MCPA	32

Appendix 1	Lists of data considered in support of the evaluation	34

Appendix 2	Detailed evaluation of the new Annex II studies	35

Appendix 3	Additional information provided by the applicant (e.g. detailed modelling data)	35

# Fate and behaviour in the environment (KCP 9)

## Critical GAP and overall conclusions

Table 8.1-1:	Critical use pattern of the formulated product

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |  | 12 |  | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha | Conclusion |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | kg or L product/ha a) max. rate per appl. b) max. total rate per crop/season | g or kg as/ha a) max. rate per appl. b) max. total rate per crop/season |  | Water L/ha min/max |  |  |  | PECgw |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | CEU | Spring Barley | F | Broadleaved weeds | Foliar Spray | BBCH 25-32 | a) 1 b) 1 | NA | a) 1.3 b) 1.3 | a) 0.356 2,4-D + 0.357 MCPA b) 0.356 2,4-D + 0.357 MCPA |  | 200-400 |  |  |  | A |
| 2 | CEU | Winter wheat | F | Broadleaved weeds | Foliar Spray | BBCH 25-39 | a) 1 b) 1 | NA | a) 1.5 b) 1.5 | a) 0.412 2,4-D + 0.412 MCPA b) 0.412 2,4-D + 0.412 MCPA |  | 200-400 |  |  |  | A |
| 3 | CEU | Winter barley | F | Broadleaved weeds | Foliar Spray | BBCH 25-39 | a) 1 b) 1 | NA | a) 1.5 b) 1.5 | a) 0.412 2,4-D + 0.412 MCPA b) 0.412 2,4-D + 0.412 MCPA |  | 200-400 |  |  |  | A |
| Interzonal uses (use as seed treatment, in greenhouses (or other closed places of plant production), as post-harvest treatment or for treatment of empty storage rooms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor uses according to Article 51 (zonal uses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor uses according to Article 51 (interzonal uses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

Explanation for column 15 “Conclusion”

| A | Safe use |
| --- | --- |
| R | Further refinement and/or risk mitigation measures required |
| C | To be confirmed by cMS |
| N | No safe use |

Table 8.1-2:	Assessed (critical) uses during approval of 2,4 D concerning the Section Environmental Fate

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | kg or L product/ha a) max. rate per appl. b) max. total rate per crop/season | g or kg as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  |

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

## Metabolites considered in the assessment

Table 8.2-3:	Metabolites of 2,4 D potentially relevant for exposure assessment

| Metabolite | Molar mass | Chemical structure | Maximum observed occurence in compartements | Exposue assessment required due to |
| --- | --- | --- | --- | --- |
| 2,4 DCP | 163 |  | Soil: 8.7% Water/sediment: 32.1% Water:2.6% Sediment::31.8% | PECsoil PECgw: leaching potential to groundwater PECsw/sed |
| 2,4 DCA | 177 |  | Soil: 15% Water/sediment: 5.3% | PECsoil PECgw: leaching potential to groundwater PECsw/sed |
| 4-CP | 128.6 |  | Soil: 33% (anaerobic conditions) | PECsoil PECgw: leaching potential to groundwater PECsw/sed |
| 1,2,4 Benzenetriol | 126.1 |  | Water: 31.7% (photolysis) | PECsw/sed |

No metabolites for MCPA were considered as relevant for exposure.

## Rate of degradation in soil (KCP 9.1.1)

Studies on degradation in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Aerobic degradation in soil (KCP 9.1.1.1)

#### 2,4 D and its metabolites

Table 8.3-4:	Summary of aerobic degradation rates for 2,4 D - laboratory studies

| 2,4 D Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | Chi2 (%) | Kinetic model | Evaluated on EU level Reference |
| Mississippi | Silt loam | 7.4 | 25 | * | 28.9 | 195.6 | 94.6** |  |  | Yes, EFSA Journal 2014; 12(9): 3812 |
| Fayette | Clay loam | 6.2 | 20 | 50 | 7.5 | 24.8 | 5.3 |  |  |  |
| RefSol 03-G | Clay loam | 6.2 | 20 | 50 | 1.6 | 5.4 | 1.2 |  |  |  |
| Site E1 | Sandy loam | 6.7 | 20 | 50 | 2.2 | 7.4 | 1.6 |  |  |  |
| Site I2 | Sandy loam | 7.8 | 20 | 50 | 2.0 | 6.5 | 1.8 | 7.8 | SFO |  |
| Geometric mean (n=5) |  |  |  |  |  |  | 4.4 |  |  |  |
| pH-dependency: n |  |  |  |  |  |  |  |  |  |  |

*Not reported

**Normalized only for temperature

Table 8.3-5:	Summary of aerobic degradation rates for 2,4 DCP - laboratory studies

| 2,4 DCP, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | Chi2 (%) | Kinetic model | Evaluated on EU level Reference |
| Fayette | Clay loam | 6.2 | 20 | 50 | - |  |  |  |  | Yes, EFSA Journal 2014; 12(9): 3812 |
| RefSol 03-G | Clay loam | 6.2 | 20 | 50 | 15.5 |  | 11.1 | 6.3 | HS |  |
| Site E1 | Sandy loam | 6.7 | 20 | 50 | 6.2 |  | 4.4 | 9.2 | SFO |  |
| Site I2 | Sandy loam | 7.8 | 20 | 50 | 7.7 |  | 6.9 | 12.8 | FOMC |  |
| Geometric mean (n=3) |  |  |  |  |  |  | 7.0 |  |  |  |
| pH-dependency: n |  |  |  |  |  |  |  |  |  |  |

Table 8.3-3:	Summary of aerobic degradation rates for 2,4 DCA - laboratory studies

| 2,4 DCA, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (x) | pH (x) | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | Chi2 (%) | Kinetic model | Evaluated on EU level Reference |
| Fayette | Clay loam | 6.2 | 20 | 50 |  |  |  |  |  | Yes, EFSA Journal 2014; 12(9): 3812 |
| RefSol 03-G | Clay loam | 6.2 | 20 | 50 | 16.3 |  | 11.7 | 3.7 | SFO |  |
| Site E1 | Sandy loam | 6.7 | 20 | 50 | 13.7 |  | 9.8 | 6.3 |  |  |
| Site I2 | Sandy loam | 7.8 | 20 | 50 | 10.9 |  | 9.8 | 8.3 |  |  |
| Geometric mean (n=3) |  |  |  |  |  |  | 10.4 |  |  |  |
| pH-dependency: n |  |  |  |  |  |  |  |  |  |  |

#### MCPA

DT50 lab (20°C, aerobic): range from 7 to 41 d (14 soils, data from literature). The DT50 value of 24 d (25°C) chosen for PECsoil calculations was obtained from the route of degradation study (SANCO/4062/2001-final (11/07/2008)). The DT90 was 79 d (from the route of degradation study).

### Anaerobic degradation in soil (KCP 9.1.1.1)

#### 2,4 D and its metabolites

Table 8.3.2-6:	Summary of anaerobic degradation rates for 2,4 D - laboratory studies

| 2,4 D Laboratory studies, anaerobic conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | pH | t.oC | MWHC % | DT50 (d) | DT90 (d) | DT50 (d) 20°C pF2/10kPa | Chi2 (%) | Kinetic model | Evaluated on EU level Reference |
| RefSol 03-G | Clay loam | 6.9 | 20±2 | pF2 | 32 | 107 | 32 | 0.9861 (5.1% err.) | SFO | Yes, EFSA Journal 2014; 12(9): 3812 |
| Kenslow | Loam | 5.8 |  |  | 23 | 77 | 23 | 0.9778 (5.3% err.) |  |  |
| Chelmorton | Silt loam | 6.8 |  |  | 38 | 127 | 38 | 0.9824 (3.9% err.) |  |  |
| Longwoods | Sandy loam | 8.1 |  |  | 22 | 74 | 22 | 0.9031 (27.7% err.) |  |  |
| Geometric mean (n=) |  |  |  |  |  |  |  |  |  |  |
| pH-dependency: n |  |  |  |  |  |  |  |  |  |  |

#### MCPA

Not degraded at 20ºC (SANCO/4062/2001-final (11/07/2008).

## Field studies (KCP 9.1.1.2)

Not needed because the 2,4 D lab DT50 is lower than 60 days and not required for MCPA.

### Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)

Not needed because the 2,4 D lab DT50 is lower than 60 days and not required for MCPA.

### Soil accumulation testing (KCP 9.1.1.2.2)

Not needed because the 2,4 D lab DT90 is lower than 1 year and not relevant for MCPA.

## Mobility in soil (KCP 9.1.2)

Studies on mobility in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### 2,4 D and its metabolites

Table 8.5-7:	Summary of soil adsorption/desorption for 2,4 D

| 2,4-D |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type | OC (%) | pH (CaCl2) | Kf (mL/g) | Kfoc (mL/g) | 1/n | Evaluated on EU level y/n/ Reference |
| M800 | Clay loam | 1.3 | 7.1 | 0.55 | 42 | 0.83 | y/ EFSA Journal 2014; 12(9)3812 (Amended on March 2017) |
| M801 | Loamy sand | 1.1 | 5.2 | 0.45 | 41 | 0.83 |  |
| M802 | Loam | 2.5 | 5.0 | 0.42 | 17 | 0.82 |  |
| M803 | Silt loam | 3.6 | 5.9 | 0.83 | 23 | 0.87 |  |
| M804 | Sandy loam | 1.4 | 7.5 | 0.19 | 14 | 0.81 |  |
| M816 | Silt loam | 0.9 | 5.9 | 0.21 | 23 | 0.78 |  |
| M822 | Clay loam | 4.4 | 7.2 | 0.51 | 12 | 0.90 |  |
| Soil I | Loamy sand | 6.1 | 6.18 | 4.5 | 56.62 | 0.85 |  |
| Soil II | Silt loam | 1.7 | 5.56 | 2.42 | 44.85 | 0.59 |  |
| Soil III | Loamy sand | 1.4 | 4.04 | 4.18 | 126.79 | 0.63 |  |
| Soil VI | Silt | 1.5 | 5.65 | 3.21 | 50.38 | 0.56 |  |
| Soil V | Silt loam | 1.6 | 5.33 | 1.25 | 52.6 | 0.83 |  |
| Plainfield | Sand | 0.46 | 5.6 | 0.357 | 76 | 0.882 |  |
| California | Sandy loam | 0.58 | 6.7 | 0.167 | 70 | 0.677 |  |
| Arizona | Silty clay loam | 0.87 | 7.9 | 0.517 | 59 | 0.816 |  |
| Lorraine | Rendzina | 6.8-9.5 | 7.0 | 3.09 | 30.67 | 0.78 |  |
| Jura I | Humic Cambisol | 10.0-14.1 | 6.5-7.0 | 5.03 | 25.73 | 0.80 |  |
| Jura II | Mollic Cambisol | 4.5-9.2 | 6.8-7.8 | 4.99 | 39.42 | 0.72 |  |
| Ile de France | Calcic Cambisol | 0.9-1.4 | 7.0-7.5 | 0.54 | 26.09 | 0.78 |  |
| Lorraine/Jura | Dystric Cambisol | 1.4-2.6 | 4.5-5.4 | 1.19 | 40 | 0.73 |  |
| Lorraine | Gleyic Cambisol | 1.3-1.5 | 6.2-6.5 | 1.27 | 57.14 | 0.68 |  |
| Louisiana | Clay | 2.09 | 7.3 | - | 58.1 | 0.83 |  |
| Chromoxerert (0-10 cm) | Silty clay | 2.57 | 7.9 | 0.82 | 31.91 | 0.91 |  |
| Chromoxerert (10-20 cm) | Silty clay | 1.40 | 7.8 | 0.37 | 26.46 | 0.99 |  |
| Pelloxerert I (0-20 cm) | Clay | 0.97 | 7.6 | 0.62 | 68.79 | 0.90 |  |
| Pelloxerert II (0-10 cm) | Clay | 0.98 | 7.7 | 0.77 | 78.54 | 0.90 |  |
| Xerofluvent I (0-10 cm) | Clay loam | 1.29 | 7.7 | 0.77 | 59.79 | 0.96 |  |
| Xerofluvent II (10-20 cm) | Sandy loam | 0.71 | 7.2 | 0.93 | 131.42 | 0.90 |  |
| Xerofluvent II (10-20 cm) | Sandy loam | 0.37 | 6.9 | 0.78 | 382.42 | 0.97 |  |
| Xerofluvent III (0-20 cm) | Sandy clay loam | 2.73 | 6.3 | 3.08 | 112.97 | 0.88 |  |
| Haploxeralf (0-10 cm) | Loam | 2.41 | 6.5 | 1.64 | 68.11 | 0.93 |  |
| Soil No. 20 | - | - | - | 2.23 | 116.85 | 0.95 |  |
| Illinois | Silt loam | 2.23 | 5.9 | - | 41 | 0.896 |  |
| North Dakota | Loam | 3.09 | 6.8 | - | 35 | 0.930 |  |
| Missippi | Clay | 1.26 | 7.0 | - | 74 | 0.795 |  |
| Geometric mean (n=34) |  |  |  |  | 48.47 | - |  |
| Arithmetic mean (n=34) |  |  |  |  | - | 0.826 |  |
| pH-dependency |  |  |  |  | No |  |  |

Table 8.5-8:	Summary of soil adsorption/desorption for 2,4 DCP

| 2,4-DCP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH (CaCl2) | Kf (mL/g) | Kfoc (mL/g) | 1/n | Evaluated on EU level y/n/ Reference |
| M800 | Clay loam | 1.3 | 7.1 | 10 | 765 | 0.85 | y/ EFSA Journal 2014; 12(9):3812 (Amended on March 2017) |
| M801 | Loamy sand | 1.1 | 5.2 | 4 | 405 | 0.80 |  |
| M802 | Loam | 2.5 | 5.0 | 16 | 655 | 0.94 |  |
| M803 | Silt loam | 3.6 | 5.9 | 25 | 690 | 0.94 |  |
| M804 | Sandy loam | 1.4 | 7.5 | 3 | 244 | 0.88 |  |
| M816 | Silt loam | 0.9 | 5.9 | 5 | 574 | 0.83 |  |
| M822 | Clay loam | 4.4 | 7.2 | 11 | 250 | 0.93 |  |
| Plainfield | Sand | 0.46 | 5.9 | - | 368 | 0.906 |  |
| Arizona | Silty clay loam | 0.87 | 7.9 | - | 374 | 0.739 |  |
| Geometric mean (n=9) |  |  |  |  | 444 | - |  |
| Arithmetic mean (n=9) |  |  |  |  | - | 0.868 |  |
| pH-dependency |  |  |  |  | No |  |  |

Table 8.5-3:	Summary of soil adsorption/desorption for 2,4 DCA

| 2,4-DCA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Name | Soil Type | OC (%) | pH (CaCl2) | Kf (mL/g) | Kfoc (mL/g) | 1/n | Evaluated on EU level y/n/ Reference |
| M800 | Clay loam | 1.3 | 7.1 | 18 | 1386 | 0.85 | y/ EFSA Journal 2014; 12(9)3812 (Amended on March 2017) |
| M801 | Loamy sand | 1.1 | 5.2 | 18 | 1630 | 0.93 |  |
| M802 | Loam | 2.5 | 5.0 | 21 | 841 | 0.93 |  |
| M803 | Silt loam | 3.6 | 5.9 | 27 | 746 | 0.93 |  |
| M804 | Sandy loam | 1.4 | 7.5 | 12 | 836 | 0.95 |  |
| M816 | Silt loam | 0.9 | 5.9 | 10 | 1137 | 0.92 |  |
| M822 | Clay loam | 4.4 | 7.2 | 27 | 622 | 0.92 |  |
| Plainfield | Sand | 0.46 | 5.6 | - | 436 | 0.955 |  |
| California | Sandy loam | 0.58 | 6.7 | - | 667 | 0.978 |  |
| Arizona | Silty clay loam | 0.87 | 7.9 | - | 616 | 0.809 |  |
| Geometric mean (n=10) |  |  |  |  | 827 |  |  |
| Arithmetic mean (n=10) |  |  |  |  |  | 0.917 |  |
| pH-dependency |  |  |  |  | No |  |  |

### MCPA

Summary from SANCO/4062/2001-final (11/07/2008):

Soils 8 (USA)

Kf: 0.21 - 1.11

1/n: 0.50 – 0.72 (mean value 0.68).

Koc: 10 - 157 (mean value 74).

Kd: 0.05 - 1.99.

The adsorption/desorption process is pH dependent, less adsorption when pH increases, this effect is likely to be significant only at pH values < about 4.

### Column leaching (KCP 9.1.2.1)

#### 2,4 D and its metabolites

Not needed because the soil mobility of 2,4 D and its metabolites 2,4 DCP and 2,4 DCA has been well defined based on the adsorption/desorption studies.

#### MCPA

In an aged residue leaching study the radioactivity of aged MCPA residues was concentrated in the top soil layer. Leachate contamination: less than 1% of TAR (SANCO/4062/2001-final (11/07/2008)).

### Lysimeter studies (KCP 9.1.2.2)

#### 2,4 D and its metabolites

Aged for (d): 28 and 30 d (2 studies)

Study no 1: 96.73 % of AR in soil profile was found in the top soil layer (0-4.5 cm). Concentration of 2,4-D in the pooled leachate was 0.1 μg/L (equivalent to 0.27 % AR and 1.53 % of radioactivity submitted to leachate)

Study no 2: Concentration of 2,4-D in the pooled leachate was 0.035 μg/L (equivalent to 0.35 % AR and 2.15 % of radioactivity submitted to leachate).

#### MCPA

Less than 0.5% of MCPA residues were detected in the percolate in a 2 year period. Most of the residue (about 80% of the total found) was detected in the top soil layer 0-10 cm after 735 d (SANCO/4062/2001-final (11/07/2008)).

### Field leaching studies (KCP 9.1.2.3)

#### 2,4 D and its metabolites

Location: Borstel / Neustadf a.R./FRG (lower Saxony) (Study submitted in the dossier of 1991).

Study type (e.g. lysimeter, field): lysimeter

Soil properties: texture, pH =5.7, OC=1.5, MWHC =20-34

Dates of application : 15th June

Crop : /Interception estimated: Winter rye was sown on November

Number of applications: 1 application

Duration: 2 growing seasons

Application rate: 750 g/ha/year

2,4-D and its known metabolites were not detected in any of the leachates in both lysimeters. Up to three unknown fractions at low concentrations were detected. Unidentified metabolite M1 exceeded 0.1 μg/L in the leachate.

#### MCPA

Less than 0.5% of MCPA residues were detected in the percolate in a 2 year period. Most of the residue (about 80% of the total found) was detected in the top soil layer 0-10 cm after 735 d (SANCO/4062/2001-final (11/07/2008)).

## Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)

Studies on degradation in water/sediment systems with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### 2,4 D and its metabolites

Table 8.6-9:	Summary of degradation in water/sediment of 2,4 D

| 2,4 D Distribution (max. water 100% after 0 days /sediment 24.7 % after 7 days) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water/sediment system | pH water/ sed. | DegT50 whole syst. (d) | DegT90 whole syst. (d) | Kinetic, Fit | DissT50 water (d) | DissT90 water (d) | Kinetic, Fit | DissT50 sed. (d) | Kinetic, Fit | Evaluated on EU level Reference |
| Pond system (loamy sand) | 6.5/6.4 | 18 | 60 |  | 12.6 | 41.9 |  | 9.8 | SFO | Yes, EFSA Journal 2014; 12(9): 3812 |
| Pond system (Silt loam) | 8.3/7.8 | 6.4 | 21.1 |  | 4.7 | 15.7 |  |  |  |  |
| Pond system (Silty clay loam) | 6.9/7.8 | 29 (Normalized= 52d | 96.3 |  |  |  |  |  |  |  |
| Geometric mean (n=x) |  | 18.6 |  |  | 7.7 |  |  | 9.8 |  |  |

Table 8.6-10:	Summary of observed metabolites

| 2,4 DCP Water/sediment system | Max. in water/sediment: 32.1 Max in water: 2.6% after 26 days Max in sediment: 31.8 % after 13 d (Calculated in the kinetic evaluation of the water/sediment study) | Evaluated on EU level Reference Yes, EFSA Journal 2014; 12(9): 3812 |
| --- | --- | --- |
| 2,4 DCA Water/sediment system | Max. in water/sediment 5.3 % (Calculated in the kinetic evaluation of the water/sediment study ) |  |

### MCPA

Table 8.6-3:	Summary of degradation in water/sediment of MCPA

Only two studies were quoted in the SANCO/4062/2001-final (11/07/2008), DT50 and DT90 means for water and whole system were reported:

|  | water | Whole system |
| --- | --- | --- |
| DT50 (d) | 13.5 | 16.9 |
| DT90 (d) | 44.7 | 56.2 |

Less than 20% of MCPA was detected in sediment system.

## Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)

zRMS Comments 2,4 D The PECsoil calculations performed for active ingredient and its metabolites 2,4 DCP and 2,4 DCA can be considered acceptable. The input parameters were used in line EFSA Journal 2014; 12(9): 3812 (Amended on March 2017). PECsoil - 2,4 D: 0.459 mg/kg PECsoil - 2,4 DCP: 0.028 mg/kg (metabolite) PECsoil - 2,4 DCA: 0.053 mg/kg (metabolite) MCPA Input parameters and PECsoil calculations performed for active ingredient can be considered acceptable PECsoil - MCPA: 0.439mg/kg Product PLANET PECsoil - 1.878mg/kg (20% interception) PECsoil - 0.470 mg/kg (80 %interception) These values will be used in further risk assessment.

### Justification for new endpoints

The same endpoints as the EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) and SANCO/4062/2001-final (11/07/2008) were used in cereals crop.

### Active substance(s) and relevant metabolite(s)

Table 8.7-11:	Input parameters related to application for PECsoil calculations

| Use No. | 1 | 2 & 3 |
| --- | --- | --- |
| Crop | Spring Cereals | Winter Cereals* |
| Application rate (g as/ha) | 2,4 D: 356 MCPA: 357 | 2,4 D: 412 MCPA: 412 |
| Number of applications/interval | 1/- | 1/- |
| Crop interception (%) | 20 (BBCH 25) 80 (BBCH 30) | 20 (BBCH 25) worst case 80 (BBCH 30) |
| Soil density g/cm3 | 1.5 |  |
| Depth of soil layer (relevant for plateau concentration) (cm) | 20 cm |  |

*Winter Cereals dosage and interception will be used for PECsoil calculations as worst case possible.

#### 2,4 D and its metabolites

Table 8.7-12:	Input parameter for 2,4 D and its relevant metabolites for PECsoil calculation

| Compound | Molecular weight (g/mol) | Max. occurrence (%) | DT50 (days) | Value in accordance to EU endpoint Reference |
| --- | --- | --- | --- | --- |
| 2,4 D | 221 | - | 7.5 (maximum DT50 lab best fit) | Yes, EFSA Journal 2014; 12(9): 3812 Final Addendum to the Renewal Assessment Report 2014 |
| 2,4 DCP | 163 | 8.7 | 14 (maximum DT50 lab best fit) |  |
| 2,4 DCA | 177 | 15 | 15.4 (maximum DT50 lab best fit) |  |

Table 8.7-13:	PECsoil for 2,4 D on Winter Cereals (worst case)

| PECsoil (mg/kg) |  | Winter cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.439 | - | - | - |
| Short term | 24h | 0.401 | 0.420 | - | - |
|  | 2d | 0.365 | 0.401 | - | - |
|  | 4d | 0.304 | 0.367 | - | - |
| Long term | 7d | 0.230 | 0.324 | - | - |
|  | 14d | 0.121 | 0.247 | - | - |
|  | 21d | 0.063 | 0.194 | - | - |
|  | 28d | 0.033 | 0.157 | - | - |
|  | 50d | 0.004 | 0.094 | - | - |
|  | 100d | <0.001 | 0.048 | - | - |
| Plateau concentration (20 cm) after year x |  | - | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | - | - | - | - |

PECsoil of metabolites

PECsoil values for the metabolites were determined as for the parent with an application rate corrected taking into account the molecular weights (MW) and the maximum occurrence of the metabolite in soil as following:

Application ratemetabolite = (MWmetabolite/ MWparent) x (% maximum occurrence/100) x application rateparent

The corresponding application rates for each metabolite are summarized in the table below.

Table 8.7-14:	Corrected application rates for the metabolites

| Metabolite | Application rate of the parent (g/ha) | MWparent | MWmetabolite | Maximum occurrence in soil (%) | Corrected application rate (g/ha) |
| --- | --- | --- | --- | --- | --- |
| 2,4 DCP | 1 x 412 (worst case) | 221 | 163 | 8.7 | 26.43 |
| 2,4 DCA |  |  | 177 | 15 | 49.49 |

The results of PECsoil calculations are presented in the tables below.

Table 8.7-15:	PECsoil for 2,4 DCP on Winter Cereals (worst case)

| PECsoil (mg/kg) |  | Winter cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.028 | - | - | - |
| Short term | 24h | 0.027 | 0.028 | - | - |
|  | 2d | 0.026 | 0.027 | - | - |
|  | 4d | 0.023 | 0.026 | - | - |
| Long term | 7d | 0.020 | 0.024 | - | - |
|  | 14d | 0.014 | 0.020 | - | - |
|  | 21d | 0.010 | 0.018 | - | - |
|  | 28d | 0.007 | 0.015 | - | - |
|  | 50d | 0.002 | 0.010 | - | - |
|  | 100d | <0.001 | 0.006 | - | - |
| Plateau concentration (20 cm) after year x |  | - | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | - | - | - | - |

Table 8.7-5:	PECsoil for 2,4 DCA on Winter Cereals (worst case)

| PECsoil (mg/kg) |  | Winter cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.053 | - | - | - |
| Short term | 24h | 0.050 | 0.052 | - | - |
|  | 2d | 0.048 | 0.050 | - | - |
|  | 4d | 0.044 | 0.048 | - | - |
| Long term | 7d | 0.039 | 0.045 | - | - |
|  | 14d | 0.028 | 0.039 | - | - |
|  | 21d | 0.021 | 0.034 | - | - |
|  | 28d | 0.015 | 0.030 | - | - |
|  | 50d | 0.006 | 0.021 | - | - |
|  | 100d | 0.001 | 0.012 | - | - |
| Plateau concentration (20 cm) after year x |  | - | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | - | - | - | - |

#### MCPA

Table 8.7-6:	Input parameter for MCPA for PECsoil calculation

| Compound | Molecular weight (g/mol) | Max. occurrence (%) | DT50 (days) | Value in accordance to EU endpoint Reference |
| --- | --- | --- | --- | --- |
| MCPA | 201 | - | 24 (from route of degradation study) (laboratory study, not normalised) | Yes, SANCO/4062/2001-final (11/07/2008) |

Table 8.7-7:	PECsoil for MCPA on Winter Cereals (worst case)

| PECsoil (mg/kg) |  | Winter cereals |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0.439 | - | - | - |
| Short term | 24h | 0.427 | 0.433 | - | - |
|  | 2d | 0.415 | 0.427 | - | - |
|  | 4d | 0.392 | 0.415 | - | - |
| Long term | 7d | 0.359 | 0.398 | - | - |
|  | 14d | 0.293 | 0.361 | - | - |
|  | 21d | 0.240 | 0.330 | - | - |
|  | 28d | 0.196 | 0.301 | - | - |
|  | 50d | 0.104 | 0.233 | - | - |
|  | 100d | 0.024 | 0.144 | - | - |
| Plateau concentration (5/20 cm) after year x |  | - | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | - | - | - | - |

#### PECsoil of PLANET

Table 8.7-8:	PECsoil for PLANET on winter cereals (worst case)

| Active substance/ preparation | Application rate (g/ha) | Interception (%) | PECact (mg/kg) |
| --- | --- | --- | --- |
| 2,4 D + MCPA / PLANET | 1761* | 20 | 1.878 |
|  |  | 80 | 0.470 |

*Density 1.1738 g/cm3

## Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)

zRMS Comments 2,4 -D Input parameters and PECgw calculations performed for active ingredient and its metabolites can be considered acceptable. All used endpoints were agreed at the EU level A crop interception of 20% was used for winter and spring cereals. PECgw for active ingredient 2,4 - D and its metabolites (2,4 DCP and 2,4-DCA) were predicted to be less than 0.001 g/L. Therefore, there is not an unacceptable risk for ground water. MCPA Input parameters and PECgw calculations performed for MCPA can be considered acceptable. All used endpoints were agreed at the EU level .A crop interception of 20% was used for winter and spring cereals. PECgw for MCPA were predicted to be less than 0.001 g/L. Therefore, there is not an unacceptable risk for ground water.

### Justification for new endpoints

The same endpoints as the EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) and SANCO/4062/2001-final (11/07/2008) were used in cereals crop.

### Active substance and relevant metabolites (KCP 9.2.4.1)

Table 8.8-16:	Input parameters related to application for PECgw calculations

| Use No. | 1 | 2 & 3 |
| --- | --- | --- |
| Crop | Spring Cereals | Winter Cereals |
| Application rate (g as/ha) | 2,4 D: 356 MCPA: 357 | 2,4 D : 412 MCPA: 412 |
| Number of applications/interval (d) | 1/- |  |
| Crop interception (%) | 20 |  |
| Frequency of application | annual |  |
| Models used for calculation | FOCUS PEARL v5.5.5, FOCUS PELMO v6.6.4 |  |

Table 8.8-17:	Application dates used for groundwater risk assessment

| Scenario | Application dates (absolute)* |  |
| --- | --- | --- |
|  | Spring Cereals | Winter Cereals |
| Châteaudun | 07/04 | 10/04 |
| Hamburg | 21/04 | 29/04 |
| Jokioinen | 01/06 | 09/05 |
| Kremsmünster | 21/04 | 19/04 |
| Okehampton | 17/04 | 16/04 |
| Piacenza | - | 14/03 |
| Porto | 07/04 | 15/01 |
| Sevilla | - | 28/12 |
| Thiva | - | 06/01 |

*According to AppDate v3.06 28 June 2019

#### 2,4 D and its metabolites

Table 8.8-18:	Input parameters related to active substance 2,4 D and metabolites for PECgw calculations

| Compound | 2,4 D | 2,4 DCP | 2,4 DCA | Value in accordance with EU endpoint y/n/ Reference* |
| --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 221 | 163 | 177 | Yes, EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) |
| Water solubility (g/mol): | 24300 @ 25ºC 20180 @ 20ºC (calculated by Eva 3, rev.2h, 20/09/2017)* | 48700 @ 20ºC | 96.3 @ 20ºC |  |
| Saturated vapour pressure (Pa): | 9.9 x10-6 @ 20ºC | 8.7 @ 20ºC | 9.3 @ 20ºC |  |
| DT50 in soil (d) | 4.4 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n =5) | 7.0 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n =3) | 10.4 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n =3) |  |
| Transformation rate | - | Parent2,4 DCP 0.157533 | 2,4 DCP2,4 DCA 0.099021 |  |
| Kfoc (mL/g)/Kfom | 48.47 (geomean, n=34) / 28.11 | 444 (geomean, n=9) / 271.3 | 827 (geomean, n=10) / 565.6 |  |
| 1/n | 0.826 (arithmetic mean, n=34) | 0.868 (arithmetic mean, n=9) | 0.917 (arithmetic mean, n=10) |  |
| Plant uptake factor | 0 |  |  |  |
| Formation fraction | - | 1 from parent | 1 from 2,4 DCP |  |

Table 8.8-4:	PECgw for 2,4 D and metabolites on Spring Cereals (with FOCUS PEARL 5.5.5 / FOCUS PELMO 6.6.4)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2,4 D |  | 2,4 DCP |  | 2,4 DCA |  |
|  |  | PEARL | PELMO | PEARL | PELMO | PEARL | PELMO |
| Spring cereals | Châteaudun | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Jokioinen | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Okehampton | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Porto | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |

Table 8.8-5:	PECgw for 2,4 D and metabolites on Winter Cereals (with FOCUS PEARL 5.5.5 / FOCUS PELMO 6.6.4)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2,4 D |  | 2,4 DCP |  | 2,4 DCA |  |
|  |  | PEARL | PELMO | PEARL | PELMO | PEARL | PELMO |
| Winter cereals | Châteaudun | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Jokioinen | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Okehampton | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Piacenza | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Porto | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Sevilla | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |
|  | Thiva | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 |

Conclusions

No PECgw for 2,4D and its metabolites are predicted to occur in groundwater at concentrations above 0.1 µg/L. Therefore, there is not an unacceptable risk for ground water and the assessment of the relevance of these metabolites according to the stepwise procedure of the EC guidance document SANCO/221/2000 –rev.11 is not required.

#### MCPA

Table 8.8-6:	Input parameters related to active substance MCPA for PECgw calculations

| Compound | MCPA | Value in accordance with EU endpoint Reference* |
| --- | --- | --- |
| Molecular weight (g/mol) | 201 | Yes, SANCO/4062/2001-final (11/07/2008) |
| Water solubility (g/L): | 293.3 @ pH=7 and 25ºC 243.6 @ pH=7 and 20ºC* |  |
| Saturated vapour pressure (Pa): | 4.4 x 10-4 @ 32ºC 9.5 x 10-5 @20ºC* |  |
| DT50 in soil (d) | 24 (from route of degradation study) |  |
| Transformation rate | - |  |
| Kfoc (mL/g)/Kfom | 74 (mean value10 - 157) (less adsorption when pH increases, this effect is likely to be significant only at pH values < about 4) |  |
| 1/n | 0.68 (mean value 0.50 – 0.72) |  |
| Plant uptake factor | 0 |  |
| Formation fraction | - |  |

*Recalculated by UBA Excel tool EVA 3 rev. 2h of 20.09.201 and used for calculations

Table 8.8-7:	PECgw for MCPA on Spring Cereals (with FOCUS PEARL 5.5.5 / FOCUS PELMO 6.6.4)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |
| --- | --- | --- | --- |
|  |  | MCPA |  |
|  |  | PEARL | PELMO |
| Spring cereals | Châteaudun | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 |
|  | Jokioinen | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 |
|  | Okehampton | <0.001 | <0.001 |
|  | Porto | <0.001 | <0.001 |

Table 8.8-8:	PECgw for MCPA on Winter Cereals (with FOCUS PEARL 5.5.5 / FOCUS PELMO 6.6.4)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |
| --- | --- | --- | --- |
|  |  | MCPA |  |
|  |  | PEARL | PELMO |
| Winter cereals | Châteaudun | <0.001 | <0.001 |
|  | Hamburg | <0.001 | <0.001 |
|  | Jokioinen | <0.001 | <0.001 |
|  | Kremsmünster | <0.001 | <0.001 |
|  | Okehampton | <0.001 | <0.001 |
|  | Piacenza | <0.001 | <0.001 |
|  | Porto | <0.001 | <0.001 |
|  | Sevilla | <0.001 | <0.001 |
|  | Thiva | <0.001 | <0.001 |

Conclusions

No PECgw for MCPA are predicted to occur in groundwater at concentrations above 0.1 µg/L. Therefore, there is not an unacceptable risk for ground water.

Final Conclusions

No PECgw for 2,4D and its metabolites, nor for MCPA, are predicted to occur in groundwater at concentrations above 0.1 µg/L. Therefore, there is not an unacceptable risk for ground water and the assessment of the relevance of these metabolites, according to the stepwise procedure of the EC guidance document SANCO/221/2000 –rev.11 is not required.

## Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)

zRMS Comments

2,4-D

Input parameters and PECsw/sed calculations performed for the active ingredient and its metabolites can be considered acceptable. All used endpoints were agreed at the EU level. The application dates were accepted (and corrected in winter cereals, spring application).

PECSW and PECSED were assessed in accordance with FOCUS Surface Water guidance; Step 1 & 2 and Step 3 (SWASH model) and Step 4 (SWAN and VFSmod) were used.

PECsw/sed calculations were also provided for the anaerobic metabolite 4-CP. It’s the zRMS opinion that anaerobic condition following the use of PLANET are unlikely to occur. Therefore, PEC calculations for the metabolite 4-CP are not considered necessary.

The PECsw/sed: 2,4-D ; 4-DCP2; 4-DCA; 1,2,4 Benzenetriol have been presented in Table 8.9-4 to Table 8.9-12

MCPA

Input parameters and PECsw/sed calculations performed for MCPA can be considered acceptable. All used endpoints were agreed at the EU level. .PECsw and PECsed were assessed in accordance with FOCUS Surface Water guidance; Step 1 & 2 and Step 3 (SWASH model) and Step 4 (SWAN and VFSmod) were used.

The PECsw/sed: MCPA have been presented in: Table 8.9.14-15.

The PECsw/sed for product PLANET have been presented in: Table 8.9.17- 8.9.18

PECSW and PECSED were assessed in accordance with FOCUS Surface Water guidance; Step 1 & 2 and Step 3 (SWASH model) and Step 4 (SWAN and VFSmod and AGES) were used. However, according to the “Working Document of the central zone in the authorisation of plant protection products” (Section 8, Environmental Fate and Behaviour, Version 1 rev. 1 – June 2018) it has been decided that

other approaches for simulating run-off mitigation reductions (e.g. VSFMod and AGES) are not recommended for the Core Assessment. Such approaches will be acceptable for national authorisations; such approaches should only be presented in National Assessment Report.

### Justification for new endpoints

The same endpoints as the EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) and SANCO/4062/2001-final (11/07/2008) were used in cereals crop.

### Active substance, relevant metabolites and the formulation (KCP 9.2.5)

Table 8.9-19:	Input parameters related to application for PECSW/SED calculations

| Plant protection product | PLANET |  |
| --- | --- | --- |
| Use No. | 1 | 2 & 3 |
| Crop | Spring Cereals | Winter Cereals |
| Application rate (kg as/ha) | 2,4 D: 0.356 MCPA: 0.357 | 2,4 D: 0.412 MCPA: 0.412 |
| Number of applications/interval (d) | 1/- |  |
| Application window | March-May (average/intermediate crop cover) |  |
| Application method | Foliar spray |  |
| CAM (Chemical application method) | 2 |  |
| Soil depth (cm) | 4 |  |
| Models used for calculation | FOCUS STEP 1/2 v3.2, FOCUS SWASH v5.3, FOCUS PRZM v4.3.1, FOCUS MACRO v5.5.4, FOCUS TOXWA v5.5, SWAN 5.0.1 |  |

Table 8.9-20:	FOCUS Step 3 Scenario related input parameters for PECsw/sed calculations for the application of PLANET

| Scenario | Application window used in modelling* |  |
| --- | --- | --- |
|  | Spring Cereals | Winter Cereals |
| D1 | 22/05 | 20/03 |
| D2 | - | 30/03 |
| D3 | 21/04 | 11/04 |
| D4 | 13/05 | 13/03 |
| D5 | 03/04 | 10/03 |
| D6 | - | 16/01 |
| R1 | - | 19/04 |
| R3 | - | 14/03 |
| R4 | 03/04 | 06/01 |

*According to AppDate v3.06 28 June 2019

#### 2,4 D and its metabolites

Table 8.9-21:	Input parameters related to active substance 2,4 D and metabolites for PECsw/sed calculations STEP 1/2 and 3(/4) (if necessary)

| Compound | 2,4 D | 2,4 DCP | 2,4 DCA | Value in accordance to EU endpoint Reference |
| --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 221 | 163 | 177 | EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) |
| Saturated vapour pressure (Pa) | 9.9 x 10-6 @ 20ºC | Not needed for Steps 1/2 |  |  |
| Water solubility (mg/L) | 24300 @ 25ºC | 4870 @ 20ºC | 96.3 @ 20ºC |  |
| Diffusion coefficient in water (m²/d) | 4.3 x 10-5 | Not needed for Steps 1/2 |  | default |
| Diffusion coefficient in air (m²/d) | 0.43 |  |  |  |
| Kfoc (mL/g) | 48.47 (geomean, n=34) / 28.11 | 444 (geomean, n=9) / 271.3 | 827 (geomean, n=10) / 565.6 | EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) |
| Freundlich Exponent 1/n | 0.826 (arithmetic mean, n=34) | 0.868 (arithmetic mean, n=9) | 0.917 (arithmetic mean, n=10) |  |
| Plant Uptake | 0 | Not needed for Steps 1/2 |  | default |
| Wash-Off factor from Crop (1/mm) | 0.05 (MACRO) 0.50 (PRZM) |  |  |  |
| DT50,soil (d) | 4.4 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n =5) | 7 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n =3) | 10.4 (geomean, normalisation to pF2, 20 °C with Q10 of 2.58, n =3) | EFSA Journal 2014; 12(9): 3812 (Amended on March 2017) |
| DT50,water (d) | 18.16 (geomean, n=3) | 1000 (default) | 1000 (default) |  |
| DT50,sed (d) | 1000 (default) | 103.9 (geomean, n=2) |  |  |
| DT50,whole system (d) | 18.16 (geomean, n=3) | 103.9 (geomean, n=2) |  |  |
| Maximum occurrence observed (% molar basis with respect to the parent) | Sediment: 24.7 | Soil:8.7% Water/ Sediment: 32.1% | Soil: 15% Water/ Sediment: 5.3% |  |

PECsw/sed

Table 8.9-4:	FOCUS Step 1, 2 and 3 PECsw and PECsed for 2,4 D following single application of PLANET to Spring Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | - | 114.74 | - | 78.79 | 54.03 |
| Step 2 |  |  |  |  |  |
| Southern Europe | March-May | 21.70 | - | 15.18 | 10.49 |
| Northern Europe |  | 12.20 |  | 8.52 | 5.89 |
| Step 3 |  |  |  |  |  |
| D1 | ditch | 2.309 | Drainage | 1.663 | 2.110 |
| D1 | stream | 1.997 | Drainage | 0.096 | 0.509 |
| D3 | ditch | 2.256 | Drainage | 0.113 | 0.471 |
| D4 | pond | 0.078 | Drainage | 0.058 | 0.099 |
| D4 | stream | 1.846 | Drainage | 0.008 | 0.097 |
| D5 | pond | 0.078 | Drainage | 0.062 | 0.113 |
| D5 | stream | 1.792 | Drainage | 0.003 | 0.043 |
| R4 | stream | 5.064 | Run off | 0.248 | 1.003 |

Table 8.9-5:	FOCUS Step 1, 2 and 3 PECsw and PECsed for 2,4 D following single application of PLANET to Winter Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | - | 132.79 | - | 91.19 | 62.52 |
| Step 2 |  |  |  |  |  |
| Southern Europe | March-May | 25.11 | - | 17.57 | 12.14 |
| Northern Europe |  | 14.12 |  | 9.86 | 6.81 |
| Step 3 |  |  |  |  |  |
| D1 | Ditch | 25.34 | Drainage | 17.12 | 20.01 |
| D1 | Stream | 15.85 | Drainage | 9.652 | 10.32 |
| D2 | Ditch | 36.63 | Drainage | 12.73 | 12.70 |
| D2 | Stream | 23.25 | Drainage | 6.829 | 7.268 |
| D3 | Ditch | 2.608 | Drainage | 0.119 | 0.515 |
| D4 | Pond | 0.090 | Drainage | 0.073 | 0.140 |
| D4 | Stream | 1.930 | Drainage | 0.003 | 0.048 |
| D5 | Pond | 0.090 | Drainage | 0.072 | 0.130 |
| D5 | Stream | 2.084 | Drainage | 0.004 | 0.051 |
| D6 | Ditch | 3.880 | Drainage | 1.392 | 2.195 |
| R1 | Pond | 0.096 | Run off | 0.075 | 0.176 |
| R1 | Stream | 2.452 | Run off | 0.094 | 0.410 |
| R3 | Stream | 2.416 | Run off | 0.077 | 0.469 |
| R4 | Stream | 1.703 | Run off | 0.012 | 0.129 |

FOCUS Step 4

Table 8.9-6:	Global maximum PECsw values for 2,4 D following single application of PLANET to Spring Cereals according to the Central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | STEP 4 2,4 D |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | None | 5* | 10 | 15** | 20 |
|  | No spray buffer (m) | 5 | 5 | 10 | 15 | 20 |
| None | D1 ditch | 0.644 | - | - | - | - |
|  | D1 stream | 0.730 | - | - | - | - |
|  | D3 ditch | 0.612 | - | - | - | - |
|  | D4 stream | 0.674 | - | - | - | - |
|  | D5 stream | 0.655 | - | - | - | - |
|  | R4 stream | 5.064 | 3.288 | 2.286 | 1.751 | 1.193 |

*0.4 was used for used for run off reduction and erosion respectively in water and sediment according to the Austrian Environmental Agency (AGES).

**0.7 and 0.9 was used for used for run off reduction and erosion respectively in water and sediment according to the Austrian Environmental Agency (AGES).

Table 8.9-7:	Global maximum PECsw values for 2,4 D following single application of PLANET to Spring Cereals according to the Central EU zone GAP according to surface water Step 4 VFSMOD

| PECsw (µg/L) | Scenario | VFSMOD 2,4D PECsw (µg/L) |
| --- | --- | --- |
| Nozzle reduction | No-spray buffer (m) | 5 |
|  | Vegetated filter strip (m) | 5 |
| None | R4 stream | 0.544 |

Table 8.9-8:	Global maximum PECsw values for 2,4 D following single application of PLANET to Winter Cereals according to the Central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | STEP 4 2,4 D |  |  |
| --- | --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | None | 5* | 10 |
|  | No spray buffer (m) | 5 | 5 | 10 |
| None | D1 ditch | 25.34 | - | - |
|  | D1 stream | 15.85 | - | - |
|  | D2 ditch | 36.63 | - | - |
|  | D2 stream | 23.25 | - | - |
|  | D3 ditch | 0.707 | - | - |
|  | D4 stream | 0.705 | - | - |
|  | D5 stream | 0.761 | - | - |
|  | D6 ditch | 3.880 | - | - |
|  | R1 stream | 2.452 | 1.500 | 1.009 |
|  | R3 stream | 1.670 | 1.092 | - |
|  | R4 stream | 0.622 | - | - |

*0.4 was used for used for run off reduction and erosion respectively in water and sediment according to the Austrian Environmental Agency (AGES).

Metabolites of 2,4 D

Table 8.9-9:	FOCUS Step 1 & 2 PECsw and PECsed for 2,4 DCP following single applications to Spring Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominat entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 23.21 | - | 21.39 | 101.80 |

Table 8.9-10:	FOCUS Step 1 & 2 PECsw and PECsed for 2,4 DCP following single applications to Winter Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominat entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 26.86 | - | 24.76 | 116.98 |
| Step 2 |  |  |  |  |  |
| Southern Europe |  | 5.31 | - | 5.09 | 23.07 |
| Northern Europe |  | 2.97 |  | 2.82 | 12.77 |

Table 8.9-11:	FOCUS Step 1 & 2 PECsw and PECsed for 2,4 DCA following single applications to Spring Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominat entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 9.31 | - | 9.18 | 76.38 |

Table 8.9-12:	FOCUS Step 1 & 2 PECsw and PECsed for 2,4 DCA following single applications to Winter Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominat entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 10.78 | - | 10.62 | 88.39 |

#### MCPA

Table 8.9-13:	Input parameters related to active substance MCPA for PECsw/sed calculations STEP 1/2 and 3(/4) (if necessary)

| Compound | MCPA | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- |
| Molecular weight (g/mol) | 201 | Yes, SANCO/4062/2001-final (11/07/2008) |
| Saturated vapour pressure (Pa) | 4.4 x 10-4 @ 32ºC 9.5 x 10-5 @20ºC* |  |
| Water solubility (g/L) | 293.3 @ pH=7 and 25ºC |  |
| Diffusion coefficient in water (m²/d) | 4.3 x 10-5 | default |
| Diffusion coefficient in air (m²/d) | 0.43 | default |
| Kfoc (mL/g) | 74 (mean value10 - 157) (less adsorption when pH increases, this effect is likely to be significant only at pH values < about 4) | Yes, SANCO/4062/2001-final (11/07/2008) |
| Freundlich Exponent 1/n | 0.68 (mean value 0.50 – 0.72) |  |
| Plant Uptake | 0 | default |
| Wash-Off factor from Crop (1/mm) | 0.05 (MACRO) 0.50 (PRZM) |  |
| DT50,soil (d) | 24 (from route of degradation study at 25°C) | Yes, SANCO/4062/2001-final (11/07/2008) |
| DT50,water (d) | 13.5 (mean, n=2) |  |
| DT50,sed (d) | 1000 (by default) |  |
| DT50,whole system (d) | 16.9 (mean, n=2) |  |
| Maximum occurrence observed (% molar basis with respect to the parent) | Sediment: 20 (worst case) | - |
| Formation fraction in soil: | - | - |

*Recalculated by UBA Excel tool EVA 3 rev. 2h of 20.09.201 and used for calculations

PECsw/sed

Table 8.9-14:	FOCUS Step 1, 2 and 3 PECsw and PECsed for MCPA following single application of PLANET to Spring Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 111.60 | - | 74.62 | 80.15 |
| Step 2 |  |  |  |  |  |
| Southern Europe | March-May | 33.41 | - | 21.16 | 24.66 |
| Northern Europe |  | 17.97 |  | 11.36 | 13.24 |
| Step 3 |  |  |  |  |  |
| D1 | ditch | 3.476 | Drainage | 2.986 | 13.25 |
| D1 | stream | 2.705 | Drainage | 1.864 | 7.647 |
| D3 | ditch | 2.261 | Drainage | 0.112 | 0.892 |
| D4 | pond | 0.237 | Drainage | 0.221 | 1.128 |
| D4 | stream | 1.851 | Drainage | 0.175 | 0.637 |
| D5 | pond | 0.094 | Drainage | 0.071 | 0.460 |
| D5 | stream | 1.806 | Drainage | 0.023 | 0.199 |
| R4 | stream | 8.800 | Run off | 0.419 | 2.790 |

Table 8.9-15:	FOCUS Step 1, 2 and 3 PECsw and PECsed for MCPA following single application of PLANET to Winter Cereals

| Scenario FOCUS | Waterbody | Max PECsw (μg/L) | Dominant entry route | 21 d- PECsw,twa (µg/L) | Max PECsed (μg/kg) |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 128.79 | - | 86.12 | 92.50 |
| Step 2 |  |  |  |  |  |
| Southern Europe | March-May | 38.56 | - | 24.42 | 28.46 |
| Northern Europe |  | 20.74 |  | 13.11 | 15.28 |
| Step 3 |  |  |  |  |  |
| D1 | ditch | 10.14 | Drainage | 7.537 | 21.08 |
| D1 | stream | 6.417 | Drainage | 4.226 | 11.94 |
| D2 | ditch | 39.75 | Drainage | 11.63 | 31.41 |
| D2 | stream | 25.07 | Drainage | 6.495 | 19.20 |
| D3 | ditch | 2.608 | Drainage | 0.118 | 0.962 |
| D4 | pond | 0.202 | Drainage | 0.187 | 1.035 |
| D4 | stream | 1.930 | Drainage | 0.149 | 0.550 |
| D5 | pond | 0.113 | Drainage | 0.086 | 0.537 |
| D5 | stream | 2.100 | Drainage | 0.028 | 0.279 |
| D6 | ditch | 10.84 | Drainage | 2.406 | 7.571 |
| R1 | pond | 0.148 | Run off | 0.111 | 0.546 |
| R1 | stream | 2.609 | Run off | 0.149 | 0.977 |
| R3 | stream | 4.637 | Run off | 0.214 | 2.013 |
| R4 | stream | 1.703 | Run off | 0.036 | 0.414 |

FOCUS Step 4

Table 8.9-16:	Global maximum PECsw values for MCPA following single application of PLANET to Winter Cereals according to the Central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | STEP 4 2,4 D |
| --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | None |
|  | No spray buffer (m) | 5 |
| None | D2 ditch | 39.75 |
|  | D2 stream | 25.07 |

#### PECsw/sed of PLANET

The PECSW for PLANET was calculated using the following equation:

The application of PLANET is 1 x 1.5 L/ha for winter cereal (worst case), corresponding to 1761 g/ha (taking into account a density of 1.1738 g/cm3) for winter cereals. The depth of the static water body was assumed to be 30 cm. The resulting maximum instantaneous PECSW value is presented in the table 8.9-17.

Table 8.9-17:	PECsw for PLANET to winter cereals (worst case)

| Crop | Distance (m) | Drift (%) | Max PECsw (μg/L) |
| --- | --- | --- | --- |
| Winter cereals | 1 | 2.77 | 16.26 |

The PECsed for PLANET was calculated using the following equation:

The application of PLANET is 1 x 1.5 L/ha, corresponding to 1761 g/ha (taking into account a density of 1.1738 g/cm3) for winter cereals (worst case). The maximum percentage of 2,4D in the sediment is 24.7% but the content in the formulated product is 27.5%, thus the actual percentage in sediment is 24.7 x 0.275 = 6.8. Regarding MCPA, the maximum percentage in the sediment is 20% but the content in the formulated product is 27.5%, thus the actual percentage in sediment is 20 x 0.275 = 5.5. The height of the sediment was assumed to be 5 cm and the sediment density was assumed to be 1.3 g/cm3. The resulting maximum instantaneous PECsed value is presented in the table 8.9-18.

Table 8.9-18:	PECsed for PLANET to winter cereals (worst case)

| Crop | Distance (m) | Drift (%) | % in sediment | Max PECsed (μg/kg) (based on maximum occurrence) |
| --- | --- | --- | --- | --- |
| Winter cereals | 1 | 2.77 | 6.8 (2,4D) | 5.10 |
|  |  |  | 5.5 (MCPA) | 4.13 |

## Fate and behaviour in air (KCP 9.3, KCP 9.3.1)

### 2,4 D

Table 8.10-22:	Summary of atmospheric degradation and behaviour

| Compound | 2,4 D |
| --- | --- |
| Direct photolysis in air |  |
| Quantum yield of direct phototransformation |  |
| Photochemical oxidative degradation in air | DT50 = 1.6d (assuming 1.5 x 106 OH radicals/cm3) |
| Volatilisation | Vapour pressure (Pa): 9.9 x 10-6 @ 20ºC Henry constant (Pa m3/mol) : 4 x 10-6 @ 20ºC |

The vapour pressure at 20 °C of the 2,4 D is < 10-5 Pa. Hence 2,4 D is regarded as non-volatile. Therefore, exposure of adjacent surface waters and terrestrial ecosystems by 2,4 D due to volatilization with subsequent deposition should not be considered.

### MCPA

Table 8.10-2:	Summary of atmospheric degradation and behaviour

| Compound | MCPA |
| --- | --- |
| Direct photolysis in air | Degradation rate from OH-attack was found to be kOH = 12.63x 10-12 cm3 molecule-1 s-1. Based on this value the atmospheric degradation half-life was 0.78 d. |
| Quantum yield of direct phototransformation | - |
| Photochemical oxidative degradation in air | Not measured. Low volatility of MCPA means that atmospheric accumulation is unlikely. |
| Volatilisation | Vapour pressure (Pa): 4.4 x 10-4 @ 32ºC 4 x 10-3 @ 45ºC Henry's Law Constant (Pa.m3/mol): 5.5 x 10-5 @ 25ºC |
| Metabolites | - |

The vapour pressure at 32 °C of the active substance MCPA is /between 10-5 and 10-4 Pa. Hence the active substance MCPA is regarded as semivolatile (volatilisation only from plant surfaces). Therefore, according to the atmospheric DT50 of 0.78d exposure of adjacent surface waters and terrestrial ecosystems by the active substance MCPA due to volatilization with subsequent deposition should not be considered.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

The following tables are to be completed by MS

- Detailed evaluation of the new Annex II studies

- Additional information provided by the applicant (e.g. detailed modelling data)