FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: SHA 2600 E Product name: PENTAGON Chemical active substances: Pendimethalin 455 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: SHARDA Cropchem Ltd. Submission date: December 2023 Finalisation date: April 2024; October 2024

Version history

| When | What |
| --- | --- |
| December 2023 | Dossier submission for evaluation |
| April 2024 | RMS assessment |
| October 2024 | ZRMS made changes in dossier according to reviewed comments |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	Pendimethalin	5

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	19

4	Section 4: Further information on the plant protection product	20

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	20

4.2	Procedures for cleaning application equipment (KCP 4.4.2)	22

Appendix 1	Lists of data considered in support of the evaluation	24

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	28

A 2.1	Pendimethalin	28

State whether or not submitted data are sufficient for evaluation. Data gaps and conditions for registration should be listed, if appropriate.

Sufficient data on identity, physical and chemical properties and other information are not available for the plant protection product and the contained technical active substance(s).

Noticed data gaps are:

- None Particle size distribution study

- Relevant impurities content after two years storage study

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

### Producer(s) of the active substance(s)

Producer(s) of the active substance

Pendimethalin:

CONFIDENTIAL information is provided separately (Part C)

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Pendimethalin

| Pendimethalin | min. 980 g/kg Sharda source (assessment by UK) min. 970 g/kg Sharda source (assessment by Poland) min. 900 g/kg (Commission Implementing Regulation (EU) 2017/1114 (22 June 2017) and SANTE/11656/2016 (18 May 2017 rev. 2) |
| --- | --- |

Relevant impurities:

| 1,2-dichloroethane | ≤ 1 g/kg in the technical (max 25.613 ± 0.453 mg/kg in the formulation SHA 2600 E) |
| --- | --- |
| Total N-Nitroso compounds | max 100 mg/kg, of which N-Nitroso-pendimethalin: < 45 mg/kg in the technical (max 6.087 ± 0.098 µg/g in the formulation SHA 2600 E) |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | PENTAGON |
| --- | --- |
| Company code number: | SHA 2600 E |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Pendimethalin | 455 g/L | 432.25 - 477.75 g/L (± 5% of declared content) | 464.29 g/L | 39.67 |

* 	Based on the minimum purity of the active substance (98% w/w) declared for registration in the active substance dossiers

** 	Based on the density of the formulation = 1.1705 g/mL

Table 1.4-2:	Relevant impurities

| Relevant impurity | Maximum content (g/L or g/kg) |
| --- | --- |
| 1,2-dichloroethane | ≤ 1 g/kg in the technical (max 25.613 ± 0.453 mg/kg in the formulation SHA 2600 E) |
| Total N-Nitroso compounds | max 100 mg/kg, of which N-Nitroso-pendimethalin: < 45 mg/kg in the technical (max 6.087 ± 0.098 µg/g of N-Nitrosopendimethalin in the formulation SHA 2600 E) N-Nitroso compounds other than N-Nitroso Pendimethalin have not been detected. |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-3:	Information on Pendimethalin

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Pendimethalin |
| CAS No. | 40487-42-1 |
| EC No. | 254-938-2 |
| CIPAC No. | 357 |

Table 1.4-4:	Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Capsule Suspension | [Code: CS] |
| --- | --- |

## Function (KCP 1.6)

The product PENTAGON (Pendimethalin 45.5% CS) is an herbicide.

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable. The appearance of the product is yellow to brown viscous liquid having non-characteristic odour. orange suspension liquid with characteristic odor.  It is not explosive, has no oxidising properties. The product is not flammable. It has a self-ignition temperature of 375 °C. In aqueous solution, it has a pH value around 9.53 ± 0.01 9.42 at 20 °C. There is no effect of high temperature on the stability of the formulation, since after 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The stability data indicate a shelf life of 2 years at ambient temperature when stored in COEX (HDPE/EVOH).. Its technical characteristics are acceptable for a capsule suspension formulation. The product is surface active. Particle size distribution study and determination of relevant impurities content after two years storage study are set as data gaps.

The intended concentration of use is 0.11% to 1.75%.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

Neither classification or labelling is relevant for this section.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No risk and safety phrases are relevant for this section.

Compliance with FAO specifications:

The product Pendimethalin 45.5% CS complies with FAO specifications.

At the time of evaluation no FAO specification was allocated.

Formulation used for tests

The product used to determine the physical, chemical and technical properties is the one cited in Part C.

Some tests were performed on formulation Pendimethalin 45.5% CS. The Applicant made a statement that this formulation is identical to Pendimethalin 455 g/L CS. In order to confirm that the study was made on the formulation of that code, the applicant attached a written statement where he confirms that the batches were used in the studies of exactly that recipe, under the specified code.

Table 2-5:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | OCSPP 830.6302 OCSPP 830.6303 OCSPP 830.6304 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Orange suspension liquid with characteristic odor (20 °C) | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |
| Explosive properties (KCP 2.2.1) | EEC A.14 | Pendimethalin 455 g/L CS SCL-40026 | The test item was non-explosive when subjected to thermal sensitivity (flame) and mechanical sensitivity (shock) tests. | Y | H.S. Anand, 2017 Report No. G14435 | Accepted. |
| Oxidizing properties (KCP 2.2.2) | EEC A.21 | Pendimethalin 455 g/L CS SCL-40026 | Pendimethalin 455 g/L CS is non-oxidizing in nature | Y | Manisha K. Varma, 2017 Report No. 230-2-11-18398 | Accepted. |
| Flash point (KCP 2.3.1) | CIPAC MT 12.3 EEC A.9 | Pendimethalin 455 g/L CS SCL-40026 | The flash point of Pendimethalin 455 g/L CS ˃ 100 °C, the test item began to boil at 100 °C and spilled out from the sample cup at normal atmospheric pressure (i.e. 760 mm Hg). |  | Manisha K. Varma, 2017 Report No. 221-2-11-16339 | Accepted. |
| Flammability (KCP 2.3.2) |  |  | Please refer to KCP 2.3.1 |  |  |  |
| Self-heating (KCP 2.3.3) | EEC A.16 A.15 DIN 51794:2003-05 | Pendimethalin 455 g/L CS SCL-40026 | Test item Pendimethalin 455 g/L CS has got the auto-ignition temperature 375 °C (1004 ÷ 1006 hPa) | Y | Paulina Flasińska, 2017 Report no. BC-67/17 | Accepted. |
| Acidity or alkalinity and pH (KCP 2.4.1) |  |  | Please refer to KCP 2.4.2 |  |  |  |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | pH value (1 % w/v aqueous dilution), 20 °C = 9.42 (pH of used water = 6,18) pH value (neat form), 20 °C = 7.93 | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |
| Viscosity (KCP 2.5.1) | OECD 114 | Pendimethalin 455 g/L CS SCL-40026 | Dynamic viscosity Mean viscosity at 20 ± 0.2 °C = 768.5 cP (mPa·s) Mean viscosity at 40 ± 0.2 °C = 437.5 cP (mPa·s) Kinematic viscosity Mean viscosity at 20 ± 0.2 °C = 655 (m2/s) Mean viscosity at 40 ± 0.2 °C = 372.9 cP (m2/s) | Y | Manisha K. Varma, 2017 Report No. 214-2-11-16338 | Accepted. |
| Surface tension (KCP 2.5.2) | EEC A.5 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Surface tension (21.5 mL/L aqueous solution), 20 °C = 50.38 mN/m | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. The concentration used is slightly higher than the highest concentration according to the GAP (17.5 mL/L), the results can however be accepted. The product is surface active. |
| Relative density (KCP 2.6.1) | EEC A.3 | Pendimethalin 455 g/L CS SCL-40026 | The mean relative density of Pendimethalin 455 g/L CS was 1.1705 ± 0.0040 at 20 °C. | Y | Manisha K. Varma, 2017 Report No. 260-2-11-16335 | Accepted. |
| Bulk density (KCP 2.6.2) |  |  | Not relevant for a CS formulation. |  |  |  |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.3 CIPAC MT 46.3 LC-MS/GC-MS | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 Pendimethalin 455 g/L CS SCL-80067 | Pendimethalin 45.5 % CS. Active substance content test results (HPLC). 0 days; 54°C, 14 days (CIPAC MT 46.3). Pendimethalin Total; 38.40 %, (w/w) 384.01 g/kg 45.32 %, (w/v) 453.21 g/L; 40.12 %, (w/w) 401.25 g/kg 47.36 %, (w/v) 473.55 g/L. Free and Encapsulated Active substance content test results (CIPAC MT 189). Pendimethalin form; 0 days; 54°C, 14 days (CIPAC MT 46.3). Pendimethalin form relative contents, %. Relative Free Pendimethalin; 0.37 % relative active; 0.35 % relative active. Relative Encapsulated Pendimethalin; 99.63 % relative active; 99.65 % relative active. Pendimethalin form contents, % w/w. Total Pendimethalin; 38.40; 40.13. Free Pendimethalin; 0.14; 0.141. Encapsulated Pendimethalin; 38.26; 39.98. Pendimethalin form contents, g/kg. Total Pendimethalin; 384.01; 401.25. Free Pendimethalin; 1.41; 1.41. Encapsulated Pendimethalin; 382.60; 399.84. Pendimethalin 45.5 % CS - 54°C, 14 days (CIPAC MT 46.3). STUDY NAME; METHOD; RESULTS. Appearance (physical state, colour, odour, 20 °C); OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304; Orange suspension liquid with characteristic odor. pH value (neat form) , 20 °C; CIPAC MT 75.3; 7.73. pH value (1 % w/v aqueous dilution), 20 °C; 9.32 (pH of used water = 6,89). Suspensibility (dilution at 21,5 mL/L; CIPAC MT 184 Standard water D; 93.27 % w/w. Suspensibility (dilution at 3,33 mL/L); 91.14 % w/w. Spontaneity of dispersion (dilution at 21,5 mL/L); CIPAC MT 160 Standard water D; 99.56 % w/w. Wet sieve; CIPAC MT 185; 0.019 % of residue on a 75 µm mesh sieve. Pourability; CIPAC MT 148; Residue = 0.89 % w/w Rinsed residue = 0.22 % w/w. Stability of packaging and packaging/preparation interactions; Technical Monograph CropLife; Packaging material COEX After accelerated storage procedure: - no changes in appearance of sample was observed; - no changes in appearance of packaging was observed; - weight loss of the package with sample was 0,06 % (w/w).. Ambient sample; Aged sample (54 ± 2 °C, 14 days). Active ingredient content; 464.9 ± 1.4 g/L; 465.8 ± 3.1g/L. 1,2-Dichloroethane content; 25.613 ± 0.453 ppm (mg/kg); 26.117 ± 0.220 ppm (mg/kg). N-Nitroso Pendimethalin content; 6.087 ± 0.098 µg/g; 5.424 ± 0.061 µg/g. N-Nitrosodimethylamine content; BLOD (LOD = 0.092 µg/g). N-Nitrosomethylethylamine content; BLOD (LOD = 0.015 µg/g). N-Nitrosopyrrolidine content; BLOD (LOD = 0.009 µg/g). N-Nitrosodiethylamine content; BLOD (LOD = 0.004 µg/g). N-Nitrosodipropylamine content; BLOD (LOD = 0.005 µg/g). N-Nitrosopiperidine content; BLOD (LOD = 0.011 µg/g). N-Nitrosodibutylamine content; BLOD (LOD = 0.216 µg/g). The test of stability of packaging confirmed that the test item is non-corrosive for packaging material (HDPE containers) after 14 days storage at 54 ± 2 °C. | Y Y | Agata Krasuska, 2021 Report No.: 47/2021 Smita D. Revankar, 2021 Report No.: G21360 | Accepted. Accepted. |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | FTSP | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Pendimethalin 45.5 % CS. Active substance content test results (HPLC). 0 days; 4 storage cycles -10 ±2 °C, 18 hour / 20 ±2 °C, 6 hour. Pendimethalin Total; 38.40 %, (w/w) 384.01 g/kg 45.32 %, (w/v) 453.21 g/L; 40.10 %, (w/w) 401.02 g/kg 47.33 %, (w/v) 473.29 g/L. Free and Encapsulated Active substance content test results (CIPAC MT 189). Pendimethalin form; 0 days; 4 storage cycles -10 ±2 °C, 18 hour / 20 ±2 °C, 6 hour. Pendimethalin form relative contents, %. Relative Free Pendimethalin; 0.37 % relative active; 0.36 % relative active. Relative Encapsulated Pendimethalin; 99.63 % relative active; 99.64 % relative active. Pendimethalin form contents, % w/w. Total Pendimethalin; 38.40; 40.10. Free Pendimethalin; 0.14; 0.14. Encapsulated Pendimethalin; 38.26; 39.96. Pendimethalin form contents, g/kg. Total Pendimethalin; 384.01; 401.02. Free Pendimethalin; 1.41; 1.43. Encapsulated Pendimethalin; 382.60; 399.59. Pendimethalin 45.5 % CS - 4 storage cycles (-10 ±2 °C, 18 hour / 20 ±2 °C, 6 hour). STUDY NAME; METHOD; RESULTS. pH value (neat form) , 20 °C; CIPAC MT 75.3; 7.92. pH value (1 % w/v aqueous dilution), 20 °C; 9.40 (pH of used water = 6.39). Suspensibility (dilution at 21,5 mL/L; CIPAC MT 184 Standard water D; 92.35 % w/w. Suspensibility (dilution at 3,33 mL/L); 92.46 % w/w. Spontaneity of dispersion (dilution at 21,5 mL/L); CIPAC MT 160 Standard water D; 99.63 % w/w. Wet sieve; CIPAC MT 185; 0.022 % of residue on a 75 µm mesh sieve. Pourability; CIPAC MT 148; Residue = 0.92 % w/w Rinsed residue = 0.22 % w/w. | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |
| Minimum content after heat stability testing (KCP 2.7.3) |  |  | Please refer to KCP 2.7.1. |  |  |  |
| Effect of low temperatures on stability (KCP 2.7.4) |  |  | Please refer to KCP 2.7.2. |  |  |  |
| Ambient temperature shelf life (KCP 2.7.5) | OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 HPLC method CIPAC MT 75.3 CIPAC MT 47.3 EEC A.5 CIPAC MT 184 CIPAC MT 160 CIPAC MT 185 CIPAC MT 148 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS Batch: SCL-32690 | On-going Active substance content test results (HPLC). 0 days (study code 47/2021); 20 (±2)°C, 6 months; 20 (±2)°C, 1 year; 20 (±2)°C, 2 years. Pendimethalin Total; 38.40 %, (w/w) 384.01 g/kg 45.32 %, (w/v) 453.21 g/L; 40.21 % (w/w) 402.11 g/kg 47.46% (w/v) 474.58 g/L; 36.67% (w/w) 366.71 g/kg 43.28% (w/v) 432.79 g/L; 36.68 % (w/w) 366.77 g/kg 43.29 % (w/v) 432.86 g/L. Free and Encapsulated Active substance content test results (CIPAC MT 189). Pendimethalin form; 0 days (study code 47/2021); 20 (±2)°C, 6 months; 20 (±2)°C, 1 year; 20 (±2)°C, 2 years. Pendimethalin form relative contents, %. Relative Free Pendimethalin; 0.37% relative active; 0.37% relative active; 0.38% relative active; 0.40% relative active. Relative Encapsulated Pendimethalin; 99.63% relative active; 99.63% relative active; 99.62% relative active; 99.60% relative active. Pendimethalin form contents, % w/w. Total Pendimethalin; 38.40; 40.21; 36.67; 36.68. Free Pendimethalin; 0.14; 0.15; 0.14; 0.15. Encapsulated Pendimethalin; 38.26; 40.06; 36.53; 36.53. Pendimethalin form contents, g/kg. Total Pendimethalin; 384.01; 402.11; 366.71; 366.71. Free Pendimethalin; 1.41; 1.48; 1.39; 1.48. Encapsulated Pendimethalin; 382.60; 400.63; 365.32; 365.29. Other P&C test results: Study name; 0 days (study code 47/2021); 20 (±2)°C, 6 months; 20 (±2)°C, 1 year; 20 (±2)°C, 2 years. Appearance (physical state, colour, odour, 20 °C); Orange suspension liquid with characteristic odor; Orange suspension liquid with characteristic odor; Orange suspension liquid with characteristic odor; Orange suspension liquid with characteristic odor. pH value (neat form), 20 °C; 7.93; 7.89; 7.85; 7.76. pH value (1 % w/v aqueous dilution), 20 °C; 9.42; 9.29; 9.28; 9.28. Suspensibility using standard water D (dilution at 21.5 mL/L); 91.95 % w/w; 90.55 % w/w; 89.94% w/w; 87.13 % w/w. Suspensibility using standard water D (dilution at 3.33 mL/L); 90.81 % w/w; 93.55 % w/w; 93.93% w/w; 92.71 % w/w. Spontaneity of dispersion using standard water D (dilution at 21.5 mL/L); 101.78 % w/w; 99.78 % w/w; 99.61 %w/w; 99.98 %w/w. Wet sieve; 0.022 % of residue on a 75 μm mesh sieve; 0.021 % of residue on a 75 μm mesh sieve; 0.028 % of residue on a 75 μm mesh sieve; 0.022 % of residue on a 75 μm mesh sieve. Pourability; Residue = 1.05 % w/w Rinsed residue = 0.23 % w/w; Residue = 0.97 % w/w Rinsed residue = 0.24 % w/w; Residue = 0.88 % w/w Rinsed residue = 0.21 % w/w; Residue = 0.90 % w/w Rinsed residue = 0.20 % w/w. Stability of packaging and packaging/preparation interactions; No significant effect of the formulation on the package and vice versa was observed; - no changes in appearance of sample was observed; - no changes in appearance of packaging was observed; - weight loss of the package with sample was 0,07 % (w/w).; - no changes in appearance of sample was observed; - no changes in appearance of packaging was observed; - weight loss of the package with sample was 0,07 % (w/w).; - no changes in appearance of sample was observed; - no changes in appearance of packaging was observed; - weight loss of the package with sample was 0,10 % (w/w).. Material of packaging: COEX(HDPE/EVOH) | Y | B. Krzysiak-Warzała, 2023, Report No.: 48/2021 | Accepted. Data gap: Relevant impurities content after two years storage study |
| AShelf life in months (if less than 2 years) (KCP 2.7.6) |  |  | Not required |  |  |  |
| Wettability (KCP 2.8.1) |  |  | Not relevant for a CS formulation. |  |  |  |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.3 (Standard Water D) | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Persistent foaming (21.5 mL/L aqueous solution) = 1 mL of foam after 1 minute Persistent foaming (3.33 mL/L aqueous solution) = 0 mL of foam after 1 minute | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. The concentrations used are slightly different to the highest and lowest concentrations according to GAP (17.5 mL/L and 1,1 mL/L, respectively), however the differences are quite small and margin of safety is wide, so the study can be accepted. |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184 (Standard Water D) | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | 21.5 mL/L = 91.95 % w/w 3.33 mL/L = 90.81 % w/w | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. The concentrations used are slightly different to the highest and lowest concentrations according to GAP (17.5 mL/L and 1,1 mL/L, respectively), however the differences are quite small so the study can be accepted. |
| Spontaneity of dispersion (KCP 2.8.3.2) | CIPAC MT 160 (Standard water D) | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Spontaneity of dispersion (dilution at 21.5 mL/L) = 101.78 % w/w | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |
| Dispersion stability (KCP 2.8.3.3) |  |  | Not relevant for a CS formulation. |  |  |  |
| Degree of dissolution and dilution stability (KCP 2.8.4) |  |  | Not relevant for a CS formulation. |  |  |  |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) |  |  | Not relevant |  |  | Data gap |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | 0.022 % of residue on a 75 µm mesh sieve | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |
| Dry sieve test |  |  | Not relevant for a CS formulation. |  |  |  |
| Dust content (KCP 2.8.5.2.1) |  |  | Not relevant for a CS formulation. |  |  |  |
| Particle size of dust (KCP 2.8.5.2.2) |  |  | Not relevant for a CS formulation. |  |  |  |
| Attrition (KCP 2.8.5.3) |  |  | Not relevant for a CS formulation. |  |  |  |
| Hardness and integrity (KCP 2.8.5.4) |  |  | Not relevant for a CS formulation. |  |  |  |
| Emulsifiability (KCP 2.8.6.1) |  |  | Not relevant for a CS formulation. |  |  |  |
| Emulsion stability (KCP 2.8.6.2) |  |  | Not relevant for a CS formulation. |  |  |  |
| Re-emulsifiability (KCP 2.8.6.3) |  |  | Not relevant for a CS formulation. |  |  |  |
| Flowability (KCP 2.8.7.1) |  |  | Not relevant for a CS formulation. |  |  |  |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Residue = 1.05 % w/w Rinsed residue = 0.23 % w/w | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |
| Dustability following accelerated storage (KCP 2.8.7.3) |  |  | Not relevant for a CS formulation. |  |  |  |
| Physical compatibility of tank mixes (KCP 2.9.1) |  |  | Not relevant. |  |  |  |
| Chemical compatibility of tank mixes (KCP 2.9.2) |  |  | Not relevant. |  |  |  |
| Adhesion to seeds (KCP 2.10.1) |  |  | Not relevant, not used for seed treatment. |  |  |  |
| Distribution to seed (KCP 2.10.2) |  |  | Not relevant, not used for seed treatment. |  |  |  |
| Other/special studies (KCP 2.11) | CIPAC MT 190 | Pendimethalin 455 g/L CS Pendimethalin 45.5% CS SCL-32690 | Release rate of Pendimethalin CIPAC MT 190. Rt, % Percentage of Pendimethalin released relative to the Pendimethalin content of the formulation. Pendimethalin 45.5 % CS Initial sample. - extracted at time 15 minutes is 0.659 % - extracted at time 30 minutes is 0.938 % - extracted at time 180 minutes is 3.399 %. Pendimethalin 45.5 % CS 4 storage cycles (-10 ±2 °C, 18 hour / 20 ±2 °C, 6 hour). - extracted at time 15 minutes is 0.594 % - extracted at time 30 minutes is 0.865 % - extracted at time 180 minutes is 3.463 %. Pendimethalin 45.5 % CS 54°C, 14 days (CIPAC MT 46.3). - extracted at time 15 minutes is 0.777 % - extracted at time 30 minutes is 1.078 % - extracted at time 180 minutes is 3.144 %. | Y | Agata Krasuska, 2021 Report No.: 47/2021 | Accepted. |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

Table 4.1-1:	Packaging information for 100 mL

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 57 mm diameter x 85.8 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 250 mL

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 61 mm diameter x 138.8 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 500 mL

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 69.5 mm diameter x 188.5 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 1 L

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 239.5 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 5 L

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Jerrycan; 136 mm x 192 mm x 285 mm |
| Opening: | 54.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| UN/ADR | compliant |

Table 4.1-6:	Packaging information for 10 liter

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | jerrycan / approx. 313 mm x 192 mm x 232 mm |
| Opening: | 47 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-7:	Packaging information for 20 L

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | jerrycan / approx. 240.0 mm x 285.0 mm x 387.5 mm |
| Opening: | 47.0 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-8:	Packaging information for 1 liter

| Type | Description |
| --- | --- |
| Material: | HDPE/EVOH (COEX) |
| Shape/size: | Round bottle / approx. 89.0 mm diameter x 240.0 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-9:	Packaging information for 5 liter

| Type | Description |
| --- | --- |
| Material: | HDPE/PA (COEX) |
| Shape/size: | jerrycan / approx. 136.0 mm x 192.0 mm x 285 mm (± 3mm) |
| Opening: | 59.7±0.4 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-10:	Packaging information for 10 liter

| Type | Description |
| --- | --- |
| Material: | HDPE/PA (COEX) |
| Shape/size: | jerrycan / approx. 174.0 mm x 226.0 mm x 368.0 mm |
| Opening: | 54.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Proposed packagings are in line with the ones used for aging studies and they are proved to be stable. Proposed pakagings are accepted.

According to SANCO/10473/2003 –rev.5 21.10.2021 for aqueous based formulation types e.g. SL, SC, LS, CS or FS, extrapolation between any plastic material types is acceptable.

## Procedures for cleaning application equipment (KCP 4.4.2)

Experience in use of plant protection products based on Pendimethalin has not indicated any particular problems. Low levels of residues of PENTAGON (Pendimethalin 455 g/L, CS) in the equipment are not expected to present any particular risk to crops to be treated from a tank that has previously been used for the product.

The efficacy of cleaning of the application equipment with regard to impacts on “other” crops can be estimated on the basis of the PSD Efficacy Guideline 302 (December 2001). As worst case, the following prerequisites were considered:

Application rate:	3.5 L product/ha, (1.593 kg/ha of Pendimethalin)

Tank volume:			2000 L

Volume remaining in spray

lines and pump after spraying:	20 L

Spray volume:	200 L/ha (lowest spray volume corresponding to the maximum concentration of PENTAGON in diluted spray)

Based on these prerequisites and in consideration of 3 rinses with each 300 – 500 L of water based on good agricultural cleaning procedures, Pendimethalin residues remaining in the tank after spraying will be diluted to the following levels:

| Cleaning step | Water volume [L] | Concentration of residues |  |
| --- | --- | --- | --- |
|  |  | product [L PPP/L water] | active substance [kg as/L] |
| Tank filling: Residues after spraying: | 2000 20 | 0.0175 | 0.007695 |
| 1st step: 1/10 dilution of residual spray volume: Residues after spraying: | 200 20 | 0.00175 | 7.695‬×10-4 |
| 2nd step: 20% of tank volume added: Residues after spraying: | 400 20 | 8.75×10-5 | 3.8475‬×10-5 |
| 3rd step: 20% of tank volume added: Residues after spraying: | 400 20 | 4.375‬×10-6 | 1.92375‬×10-6 |
| Addition of fresh spray solution: Residues in the tank filling: | 2000 | 4.375‬×10-8 | 1.92375‬×10-8 |

PPP = PENTAGON	as = Pendimethalin

Residues remaining in the last cleaning solution were calculated to be 1.92375‬×10-6 kg/L of Pendimethalin resulting in residue concentration of 1.92375‬×10-8 g/L Pendimethalin after refilling the tank with 2000 L of water for another spray work. Assuming a range of spray volumes of 200 – 400 L/ha applied to succeeding crops, residues of 3.8475×10-6 – 7.695×10-6 g Pendimethalin will be applied per ha.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1 KCP 2.4.2 KCP 2.5.2 KCP 2.7.1 KCP 2.7.2 KCP 2.8.2 KCP 2.8.3.1 KCP 2.8.3.2 KCP 2.8.5.1.2 KCP 2.8.7.2 KCP 2.11 | Agata Krasuska | 2021 | Pendimethalin 45.5 % CS: Analysis of active substance content and physicochemical properties of initial preparation and preparation after accelerated and freeze/thaw storage procedures Report No.: 47/2021 Institute of Heavy Organic Synthesis „Blachownia” GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.2.1 | H.S. Anand | 2017 | DETERMINATION OF EXPLOSIVE PROPERTIES OF PENDIMETHALIN 45.5% CS Report No: G14435 Advinus Therapeutics Limited GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.2.2 | Manisha K. Varma | 2017 | OXIDIZING PROPERTIES OF PENDIMETHALIN 455 g/l CS Report No: 230-2-11-18398 JAI Research Foundation GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.3.1 | Manisha K. Varma | 2017 | FLASH POINT OF PENDIMETHALIN 455 g/L CS Report No: 221-2-11-16339 JAI Research Foundation GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.3.3 | Paulina Flasińska | 2017 | Pendimethalin 455 g/L CS: Determination of auto-ignition temperature Study No: BC-67/17 Institute of Industrial Organic Chemistry GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.5.1 | Manisha K. Varma | 2017 | VISCOSITY OF PENDIMETHALIN 455 g/L CS Report No: 214-2-11-16338 JAI Research Foundation GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.6.1 | Manisha K. Varma | 2017 | RELATIVE DENSITY TENSION OF PENDIMETHALIN 455 g/L CS Report No: 260-2-11-16335 JAI Research Foundation GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.7.1 | Smita D. Revankar | 2021 | Accelerated Storage Stability Test by Heating at elevated Temperature of Pendimethalin 455 g/L CS Report No.: G21360 Eurofins Advinus Limited GLP Unpublished | N | SHARDA Cropchem Limited |
| KCP 2.7.5 | B. Krzysiak-Warzała | 2023 | Pendimethalin 45.5 % CS Evaluation of stability of the product after storage in accordance with the CropLife Technical Monograph No. 17 (6 months, 1 year, 2 years at 20 ±2°C) Report No.: 48/2021 Institute of Heavy Organic Synthesis „Blachownia” GLP Unpublished | N | SHARDA Cropchem Limited |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Additional data on the physical, chemical and technical properties of the active substance

  - Pendimethalin

Not relevant. There is no additional data on the active substance Pendimethalin.