FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: SHA 2600 E Product name(s): PENTAGON Chemical active substances: Pendimethalin, 455 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Sharda Cropchem Limited Submission date: December 2023 Finalisation date: July 2024, November 2024

Version history

| When | What |
| --- | --- |
| December 2023 | Dossier submission for evaluation |
| July 2024 | RMS assessment |
| November 2024 | Final Registration Report |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	11

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	19

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	19

5.2.2	Methods for the determination of residues (KCP 5.1.2)	20

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	21

5.3.1	Analysis of the plant protection product (KCP 5.2)	21

5.3.2	Description of analytical methods for the determination of residues of pendimethalin (KCP 5.2)	21

5.3.2.1	Overview of residue definitions and levels for which compliance is required	21

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	22

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	24

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	24

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	25

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	25

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	26

5.3.2.8	Other studies/ information	26

Appendix 1	Lists of data considered in support of the evaluation	28

Appendix 2	Detailed evaluation of submitted analytical methods	35

A 2.1	Analytical methods for pendimethalin	35

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	35

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	51

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for the active substance(s) and relevant impurities in the plant protection product.

Noticed data gaps are:

- Method for determination of free pendimethalin content in the formulation

- None

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are:

- none

The applicant has provided a data matching dossier to support the application (NL, May 2018, May 2020; available on CIRCA database)

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Potato, ware potato (high water commodity) | Supported |
| Winter cereals – rye, oats, triticale (high starch commodity) | Supported |
| Winter oilseed rape (high oil commodity) | Supported |
| Onion, shallots (high water commodity) | Supported |
| Flower bulb and flower tuber crops | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of pendimethalin in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical method is suitable for the determination of pendimethalin in the plant protection product Pentagon (Pendimethalin 455 g/L CS). The proposed analytical method has been fully validated in terms of specificity, linearity, accuracy and precision. Proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | KCP 5.1.1 |
| --- | --- |
| Report | Validation of analytical method for determination of active ingredient content of pendimethalin 455 g/L CS. Manisha K. Varma, 2017. Document no.: 228-2-12-16345 |
| Guideline(s): | Yes SANCO/3030/99 rev. 4 ABNT NBR 14029 OCSPP 830.1800 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Principle of the method:

Active ingredient pendimethalin was detected by high performance liquid chromatography on a reversed phase column using internal standard.

Material:

Test item:

Name: 				Pendimethalin 455 g/L CS

Active ingredient content: 	455 g/L

Batch: 				SCL-40026

Preparation date: 		April 10, 2016

Expiry date: 			April 09, 2018

Reference item:

Name: 				Pendimethalin

CAS number: 			40487-42-1

Purity:				98.8%

Source:				Sigma - Aldrich

Batch: 				SZBD302XV

Preparation date: 		October 29, 2013

Expiry date:			October 29, 2018

Blank formulation:

Name: 				Pendimethalin 455 g/L CS Blank Formulation

Batch: 				SCL - 40146

Preparation date: 		June 27, 2016

Expiry date:			June 26, 2018

The active ingredient concentration of pendimethalin 455 g/L CS is determined by high performance liquid chromatography on a reversed phase column using Acetonitrile (90%): MilliQ Water (10%), v/v as mobile phase and UV detection at 240 nm.

Instrumental parameters

Instrument:	HPLC (Shimadzu: Prominience-I, LC2030 C with Lab Solutions Software)

Column:	C-18 [250 cm x 4.6 mm(i.d.) x 5.0 µm particle size]

Wave length:	240 nm

Flow rate:	1.0 mL/min

Injection volume:	5 µL

Mobile phase:	Acetonitrile (90%): Mili Q Water (10%) v/v

Specificity

The specificity of the method was studies by injecting solvent (acetonitrile), blank formulation solution,

Pendimethalin reference standard solution and sample solution. Since there was no interference between

the peaks of active ingredient in reference standard, sample, blank formulation solution as well as solvent,

the analytical method was considered to be specific for the analyte.

Linearity

The reference standard solutions (L1 – L5; 54.32 – 200.33 mg/L) were injected onto HPLC using the parameters above. The peak area was plotted against concentration (mg/L). The correlation coefficient (r), intercept with y-axis (a) and slope of the line (b) were calculated and the regression equation Y = bX + a was established.

Precision

The test item samples prepared and analysed in five replicates preparation. The content (%) in each replication, mean of active ingredient content and %RSD was calculated and reported. Repeatability of the method was evaluated two times, in the same ways but at different days.

Accuracy

Accuracy of the analytical method was determined by the method of standard addition to the blank formulation of the same test item. The blank formulation sample used for specificity was fortified with reference standard in three fortification levels. The fortification levels were near to 80, 100, 120 % of declared active ingredient content. The three replicates determination was analysed for each levels.

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of pendimethalin in plant protection product PENTAGON/SHA 2600 E

|  | Pendimethalin |
| --- | --- |
| Author(s), year | Manisha K. Varma, 2017 |
| Principle of method | HPLC-UV |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | 5 points 54.32 to 200.33 (mg/L) Y = 23082.84x – 66278.06 R2 = 0.999 |
| Precision – Repeatability Mean n = 5 (%RSD) | The repeatability was assessed by anaysing five samples on two different days. Mean content 1 = 39.08 %w/w (458.57 g/L) Mean content 2 = 39.08 %w/w (458.55 g/L) %RSD = 0.03 %RSDR = 2.30 %RSDr = 1.54 Hr = 0.019 ≤ 1 |
| Accuracy n = 9 (% Recovery) | The accuracy was assessed by analysing three samples on three fortification levels (n=9) 1st level (80.68 mg/L): Mean recovery = 98.76% %RSD = 0.03 2nd level (99.99 mg/L): Mean recovery = 98.47% %RSD = 0.11 3rd level (99.73 120.93 mg/L): Mean recovery = 99.73% %RSD = 0.011 Overall mean recover = 98.99% Overall standard deviation = 0.66% |
| Interference/ Specificity | No interference, the method is specific |
| Comment | - |

Conclusion

From the results of the analytical method validation, it is concluded that the analytical method is specific, sensitive, precise and accurate for the analysis of pendimethalin in the test item Pendimethalin 455 g/L CS. The results of validation criteria are within the specified limits of SANCO/3030/99 rev. 5.

| Comments of zRMS: | The test formulation name is Pendimethalin 45.5% CS, however the Applicant has included a statement, that the formulation is identical to formulation Pendimethalin 455 g/L CS. The proposed analytical method is suitable for the determination of pendimethalin in the plant protection product Pentagon (Pendimethalin 455 g/L CS). The proposed analytical method has been fully validated in terms of specificity, linearity, accuracy and precision. Proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. With regard to the proposed analytical method for determination of free active content, zRMS does not agree with the Applicant’s conclusion. In the opinion of zRMS, the method meets criteria with regard to the specificity, linearity and precision. However, the method does not meet criteria for accuracy. The content of free pendimethalin is determined as 0.14 % w/w. According to the SANCO/3030/99 rev.5 guidance, for the content of active substance 0.1-1 % w/w, the mean recovery should be 80-120%. For the method, the mean recovery is 127% and 127.9%, which is outside the acceptable range. The method is not acceptable for determination of free pendimethalin content. After additional clarifications from the Applicant, regarding the content of free pendimethalin, the analytical method for determination of free active content has been accepted. The method meets criteria of SANCO/3030/99 rev.5 guidance with regard to specificity, linearity, precision and accuracy and is suitable for determination of free pendimethalin content in the formulation. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | KCP 5.1.1-2 |
| --- | --- |
| Report | Pendimethalin 45.5% CS Analysis of active substance content and physicochemical properties of initial preparation and preparation after accelerated and freeze/thaw storage procedures. B. Krzysiak-Warzała, 2021, Report No. 47/2021 |
| Guideline(s): | Yes SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The test was carried out in Liquid Chromatography Laboratory.

In order to determine the concentration of active substance in the test samples the HPLC-UV method was used.

Standard solution preparation:

Pendimethalin: 14.9 mg of the standard was diluted to 10cm3 with acetonitrile.

Sample solution preparation:

Initial preparation: five independent weights (about 11 mg) of the test sample was diluted with 1cm3 water and 9cm3 acetonitrile. These solutions were analyzed by HPLC-UV under stable chromatographic conditions.

Blank sample solution preparation:

41.3 mg of the blank formulation sample was diluted with 1cm3 water and 9cm3 acetonitrile.

Chromatographic conditions:

column: 			SynergiTM, 4μm Hydro-RP 80Å, 250x4.6 mm

column temperature: 		35oC

flow rate: 			1,4 cm3/min

eluent: 90% acetonitrile: 	10% H2O

elution program: 		isocratic

injection volume: 		5μL

detector: 			UV 240 nm

run time: 			9 minutes

Linearity (calibration curve)

For Pendimethalin calibration 5 standard solutions were prepared in the concentration range about 80-120% of analyte content in tested sample. Each calibration solution was analyzed twice under stable chromatographic conditions. Linearity range of Pendimethalin was 346,6 – 519,8 mg/L

Specificity

The combination of the HPLC column, mobile phase composition and the column temperature was designed to ensure separation of the active ingredient from any impurities that possibly could interfere with the determination of the active ingredient.

The selectivity of the HPLC method was assessed by examination of peak homogeneity and peak purity using diode array detector.

Precision

For sample repeatability evaluation, five independent test item solutions prepared from five individual weights of the test item (initial preparation), were analyzed in duplicate using the same chromatographic conditions as during the calibration process.

Accuracy

For an accuracy estimation, two solutions with active substance concentration prepared from analytical standard in the blank formulation matrix were prepared in the range of 90 % and 110% of analyte concentration in the tested solutions.

Each solution was injected 7 times using the same chromatographic conditions as during the test item measurements and calibration process

Validation - Results and discussions

Table 5.2-2:	Methods suitable for the determination of active substances Pendimethalin in plant protection product PENTAGON/SHA 2600 E

|  | Pendimethalin |
| --- | --- |
| Author(s), year | B. Krzysiak-Warzała, 2021 |
| Principle of method | HPLC-UV |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | 347 – 520 mg/L y=0.2825x – 4.9096 R2=0.9995 0.9991 |
| Precision – Repeatability Mean n = 10 (%RSD) | RSD = 0.41% RSDR=2.25% RSDr=1.51% Hr=0.27 < 1 |
| Accuracy n = 7 (% Recovery) | 1st level - 90% (40.791 %w/v): mean recovery: 101.2% 2nd level - 110% (49.856 %w/v): mean recovery: 101.1% |
| Interference/ Specificity | No interference. The method is specific |

Conclusion

According to SANCO/3030/99 rev. 5 the method was successfully validated and is suitable for determination of Pendimethalin in the test item.

Free and encapsulated active substance content

The test consist of few following stages:

Extraction of free Pendimethalin

Extraction was carried out at temperature between 20 °C and 25 °C.

Sample containing 70 – 80 mg of Pendimethalin (about 0,2 g) was putted into glass bottle. The volume of formulation was calculated (by using the sample density 1,1802 g/mL). To this sufficient distilled water was added to give a total volume of 6 ml, and then 50 ml of hexane was added. The bottle was closed and placed horizontally on the roller. The bottle was rolled with a rotational speed of 70 ± 10 rpm for a period of 5 minutes (±10 s). After that clear hexane extract was taken to quantitative determination of free Pendimethalin content. Five independent extracts were prepared.

Free Pendimethalin content determination

The HPLC analysis was performed according to the internal test procedure BA-AB/SPO-3 “Pesticide analysis in the formulations of plant protection products” and research method No. BA-AB/MB-28 “Pendimethalin” validated before used.

Quantitative analysis of Pendimethalin was based on external calibration using certified analytical standard.

For calibration 5 – 7 calibration solutions from analytical standard were prepared (with different concentration active substance in suitable solvent). Each calibration solution was analyzed twice under stable chromatographic conditions.

Test sample extracts were analyzed in directly form by HPLC-UV under stable chromatographic conditions (the same as calibration conditions). Two measurements of content of Pendimethalin in the each obtained extract were made. The results was express in mg/ml.

Standard solution preparation:

Pendimethalin: 14,9 mg of the standard was diluted to 10cm3 with hexane. 1 cm3 of this solution was diluted to 10cm3 with hexane.

Sample solution preparation:

Five independent test sample, directly form, were analyzed by HPLC-UV under stable chromatographic conditions.

Chromatographic conditions:

column: 			SynergiTM, 4μm Hydro-RP 80Å, 250x4.6 mm

column temperature: 		35oC

flow rate: 			1,4 cm3/min

eluent: 80% acetonitrile: 	20% H2O

elution program: 		isocratic

injection volume: 		5μL

detector: 			UV 240 nm

run time: 			9 minutes

Specificity

The combination of the HPLC column, mobile phase composition and the column temperature was designed to ensure separation of the active ingredient from any impurities that possibly could interfere with the determination of the active ingredient.

The selectivity of the HPLC method was assessed by examination of peak homogeneity and peak purity using diode array detector.

Precision

For sample repeatability evaluation, five independent test item solutions (initial preparation) were injected twice using the same chromatographic conditions as during the calibration process.

Accuracy

For an accuracy estimation, two solutions with a concentration of analyte in the range of 90% and 110% level in the test item were prepared from test item extract spiked by analytical standard.

Each solution was injected 7 times using the same chromatographic conditions as during the test item measurements and calibration process.

Linearity

For Pendimethalin calibration 5 standard solutions were prepared in the concentration range about 80-120% of analyte content in tested sample. Each calibration solution was analyzed twice under stable chromatographic conditions. Linearity range of Pendimethalin was 4,44 – 6,66 mg/l

Validation - Results and discussions

Table 5.2-3:	Methods suitable for the determination of active substances Pendimethalin in plant protection product PENTAGON/SHA 2600 E

|  | Pendimethalin |
| --- | --- |
| Author(s), year | B. Krzysiak-Warzała, 2021 |
| Principle of method | HPLC-UV |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | 4.44-6.66 mg/L y=0.2867x - 0.033 R2=0.9978 0.9955 |
| Precision – Repeatability Mean n = 10 (%RSD) | RSD = 5.56% RSDR=12.34% RSDr=8.27% Hr=0.67 < 1 |
| Accuracy n = 7 (% Recovery) | 1st level - 90% (0.00051 %w/v): mean recovery: 127.9% 2nd level - 110% (0.00062 %w/v): mean recovery: 127.0% |
| Interference/ Specificity | No interference. The method is specific |

Conclusion

According to SANCO/3030/99 rev. 5 the method was successfully validated and is suitable for determination of free and encapsulated content of Pendimethalin in the test item.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical method is suitable for determination of relative impurities - 1,2-dichloroethane and N-nitrosopendimethalin in the plant protection product Pentagon (Pendimethalin 455 g/L CS). The proposed analytical method has been fully validated in terms of specificity, linearity, accuracy and precision. Proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | KCP 5.1.2 |
| --- | --- |
| Report | Pendimethalin 455 g/L CS. Methods validation for determination of the relevant impurities of pendimethalin. A. Chałas, 2017. Document no.: BA-45/17 |
| Guideline(s): | Yes SANCO/3030/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Materials:

Active substance

Name: 				Pendimethalin 455 g/L CS

Active ingredient content: 	455 g/L

Batch: 				SCL-40026

Preparation date: 		April 10, 2016

Expiry date: 			April 09, 2018

Impurity 1:

Name: 				1,2-Dichloroethane

IUPAC name: 			1,2-Dichloroethane

Chemical formula:		C2H4Cl2

CAS No: 			[107-06-2]

Molar mass: 			98.95 g/mol

Impurity 2:

Name: 				N-nitrosopendimethalin
IUPAC name: 			N-(3,4-dimethyl-2,6-dinitrophenyl)-N-pentan-3-ylnitrous amide

Chemical formula:		C13H18N4O5

CAS No: 			[68897-50-7]

Molar mass: 			310.3 g/mol

Reference item (impurities)

1,2-dichloroethane, 99.9 %, Sigma-Aldrich, Lot#BCBS5198V

N-nitrosopendimethalin, 99.2 ±0.1 % (m/m), IPO, Series No. 3A/16

- 1,2-dichloroethane

The method is based on determination of 1,2-dichlorethane by headspace analysis in combination with gas chromatography and flame ionization detection (HS-GC-FID) using external standard method with application of placebo of PENDIMETHALIN 455 g/l CS preparation as a matrix to which known amount of 1,2-dichloroethane was added to determine the individual validation parameters.

Preparation of standard solutions

About 100 mg of 1,2-dichloroethane standard was weighed with accuracy of 0.01 mg into the 10 mL volumetric flask. DMSO was added to 10 mL and content was mixed (solution A - ~10mg/mL). The solution A was diluted 10-times with DMSO (solution B - ~1 mg/mL). Solution B was diluted: 10-times, 20-times, 40-times, 50-times,80-times, 100-times and 200-times giving solutions of 1,2-dichloroethane from C-1 to C-7 with concentrations from 0.1000 to 0.0050 mg/mL

Preparation of the test item solutions

About 200 mg of PENDIMETHALIN 455 g/l CS preparation was weighted in headspace vial and then 1 mL DMSO and 6 mL deionized water were added. The vials were tightly closed and headspace of the solution was analyzed.

Specificity

To prove specificity of the developed method the headspace chromatograms of: mixture of DMSO and deionized water (sample solvents, 1 mL+6 mL), placebo solution in DMSO and deionized water (200 mg+1 mL+6 mL), sample solution (PENDIMETHALIN 455 g/l CS) in DMSO and deionized water (200 mg+1 mL+6 mL), standard solution of 1,2-dichloroethane in DMSO and deionized water (1 mL+6 mL), sample and placebo of PENDIMETHALIN 455 g/l CS solution in DMSO and deionized water (200 mg+1 mL+6 mL) fortified with 1,2-dichloroethane at LOQ were performed and superimposed.

There are no other peaks that could interfere with peak of the determined substance under specified chromatographic conditions.

Linearity

Detector response linearity was examined using 7 solutions in the range of 1,2-dichloroethane content from 0.0050 to 0.1000 mg what for 200 mg of preparation sample constitutes from around 7 to 130 % of maximum allowed 1,2-dichloroethane content in PENDIMETHALIN 455 g/l CS preparation (m/m).

Limit of quantification (LOQ)

The limit of quantification (LOQ) of 1,2-dichloroethane in PENDIMETHALIN 455 g/l CS preparation was defined as the lowest quantity of 1,2-dichloroethane standard used for calibration curve. Limit of quantification is 0.0050 mg what corresponds to 0.0025 % (0.025 g/kg) of 1,2-dichloroethane content in the preparation.

Repeatability (precision)

Content of 1,2-dichloroethane in the preparation was determined by analyzing of six samples of PENDIMETHALIN 455 g/l CS preparation. About 200 mg of the examined preparation were placed in headspace vials and then 1 mL DMSO and 6 mL water were added. The vials were tightly closed and analyzed. In none of the examined samples 1,2-dichloroethane was detected above LOQ. Therefore for the determination of repeatability six portions about 200 mg of placebo fortified with 1,2-dichlorethane at LOQ level (0.0050 mg) were analyzed.

Accuracy

The accuracy of determination of 1,2-dichloroethane content in PENDIMETHALIN 455 g/l preparation was assessed by recovery. Calculations were performed using the calibration curve determined with addition of placebo of PENDIMETHALIN 455 g/l CS.

- N-nitrosopendimethalin

The method is based on determination of N-nitrosopendimethalin using reversed phase high performance liquid chromatography (RP-HPLC) with UV detection at wavelength 206 nm using external standard method.

Preparation of standard solutions

About 5 mg of N-nitrosopendimethalin standard was weighted into a 10 mL volumetric flask and dissolved in acetonitrile. Acetonitrile was added to 10 mL and the content was mixed. The solution was diluted 250 times with acetonitrile.

Acetonitrile was used as solvent.

Preparation of the test item solutions

About 40 mg of PENDIMETHALIN 455 g/l CS preparation was weighted into a 10 mL volumetric flask, 2.0 mL of water was added and the content was mixed. Then about 7 mL of acetonitrile was added and the flask was put into an ultrasonic bath for 10 minutes. The solution was adjusted to room temperature. Acetonitrile was added up to 10 mL and the content was mixed.

Specificity

To prove specificity of the developed method chromatograms of: solvent (acetonitrile), placebo, standard solution and sample of PENDIMETHALIN 455 g/l CS preparation dissolved in acetonitrile were performed and superimposed.

There are no other peaks that could interfere with peaks of the analyte under specified chromatographic conditions.

Linearity

Detector response linearity was examined using 6 solutions at 6 levels of concentration in the range of N-nitrosopendimethalin concentration between 0.039 – 0.088 μg/mL. The detector response against the N-nitrosopendimethalin content was plotted.

Limit of quantification (LOQ)

The limit of quantification (LOQ) was defined as the lowest concentration of standard used for calibration curve – 0.039 μg/mL, which is 0.010 g/kg of preparation.

Repeatability (precision)

The content of N-nitrosopendimethalin in the preparation was determined by analyzing of three portions of the examined sample. In none of the examined samples N-nitrosopendimethalin was detected above LOQ. Therefore for the determination of repeatability, six portions about 30 mg of the placebo fortified with N-nitrosopendimethalin at concentration 0.040 μg/mL (0.001 %) were analyzed.

Accuracy

The accuracy of determination of N-nitrosopendimethalin content in PENIDMETHALIN 455 g/l CS preparation was assessed by recovery. Twelve 10 mL flasks containing about 30 mg of placebo were prepared. To six of them 0.20 mL of N-nitrosopendimethalin stock solution (0.0020 mg/mL) was added and to other six 0.25 mL of the stock solution. To each flask 2 mL water was added, about 7 mL acetonitrile and the flasks were placed in the ultrasonic bath for 10 minutes. Afterwards the flasks were filled up to 10 mL with acetonitrile and mixed. The solutions were analyzed and the concentration of N-nitrosopendimethalin and recovery were calculated using the calibration curve.

Validation - Results and discussions

Table 5.2-4:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) PENTAGON/SHA 2600 E

|  | 1,2-dichloroethane | N-nitrosopendimethalin |
| --- | --- | --- |
| Author(s), year | A. Chałas, 2017 |  |
| Principle of method | HS-GC-FID | RP-HPLC with UV-DAD |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | 7 points 0.0050 - 0.1000 mg/mL (0.025-0.5 g/kg) Y = 222183x +1.2 R2 =0.9999 | 6 points 0.039 – 0.088 μg/mL Y = 256042.6x – 535.0 R=0.9933 |
| Precision – Repeatability Mean n = 6 (%RSD) | RSD = 1.06% RSDR= 9.86% RSDr= 6.60% Hr= 0.16 < 1 | RSD = 3.38% RSDR= 11.31% RSDr= 7.58% Hr= 0.46 < 1 |
| Accuracy n = 12 (% Recovery) | The accuracy was assessed by analysing six samples on two fortification levels. Total mean recovery = 99.2% | The accuracy was assessed by analysing six samples on two fortification levels Total mean recovery = 97.6% |
| Interference/ Specificity | No interference, the method is specific | No interference, the method is specific |
| LOQ | 0.025 g/kg | 0.010 g/kg |
| Comment | - | - |

Conclusion

According to SANCO/3030/99 rev. 5 the method was successfully validated and is suitable for determination of relative impurities - 1,2-dichloroethane and N-nitrosopendimethalin in the test item.

The content of the analyzed relevant impurities, 1,2-dichloroethane and N-nitrosopendimethalin in PENDIMETHALIN 455 g/l CS preparation, determined by developed and validated method were below limit of quantification (LOQ), it means below 0.025 g/kg for 1,2-dichloroethane and below 0.010 g/kg for N-nitrosopendimethalin, therefore below permitted level (1,2-dichloroethane ≤ 0.386 0.401 g/kg, N-nitrosopendimethalin ≤ 0.017 0.018 g/kg).

| Comments of zRMS: | The proposed analytical method is suitable for determination of relative impurities - 1,2-dichloroethane, N-nitrosopendimethalin and N-nitrosoamines in the plant protection product Pentagon (Pendimethalin 455 g/L CS). The proposed analytical method has been fully validated in terms of specificity, linearity, accuracy and precision. Proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | KCP 5.1.2-2 |
| --- | --- |
| Report | Accelerated storage stability test by heating at elevated temperature of Pendimethalin 455 g/L CS. S. Revankar, 2021, Report No. G21360 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

- N-Nitroso Pendimethalin

Specificity

The specificity and selectivity of the method was established by injecting the diluent along with the impurity working standard solutions to the LC-MS/MS. The chromatograms of the diluent were checked for the absence of any interference at the retention time of the impurities.

Linearity

A known aliquot from each of the N-Nitroso pendimethalin reference standard stock solutions prepared was transferred into a volumetric flask and diluted with diluent to get five different working standard solutions for detector linearity check. Equal volumes each of the impurity working standard solutions were injected in triplicate into LC-MS/MS and the peak areas of N-NitrosoPendimethalin were recorded for each injection.

Precision

About 0.6 g of the test item, Pendimethalin 455 g/L CS was weighed in five replications to separate 100 mL volumetric flasks and the contents of the flask were dissolved in diluent. Later, the volume was made up to the mark with diluent, the solution was shaken thoroughly and used for the analysis of impurity content by injecting to the LC-MS/MS.

Accuracy

About 0.6 g of test item, Pendimethalin 455 g/L CS was weighed into separate 100 mL volumetric flasks (three for each of the three fortification levels) and the contents of the volumetric flasks were dissolved in diluent. Later, the test item solutions were fortified with N-Nitroso Pendimethalin reference standard stock solution. Further, the volume was made up to the mark with diluent, the solutions were shaken thoroughly and used for analysis of impurity content by injecting to LC-MS/MS.

Chromatographic conditions

Instrument:		API 3200 LC-MS/MS system with Shimadzu front end HPLC equipped with PC based data system

Column:		Waters Sunfire C18, 150x4.6 mm, particle size 3.5 μm

Mobile phase A: 	0.1% formic acid in Milli-Q water

Mobile phase B: 	Acetonitrile:Methanol (50:50%v/v)

Gradient:

| Time (min) | Mobile phase A (%) | Mobile phase B (%) |
| --- | --- | --- |
| 0.01 | 50 | 50 |
| 10.00 | 10 | 90 |
| 15.00 | 10 | 90 |
| 15.10 | 50 | 50 |
| 20.00 | 50 | 50 |

Flow rate:		1.0 mL/min with splitter

Sample temperature:	15℃

Mass spectrometer:	API 3200

Ion Source:		ESI

Polarity:		positive

Mode:			MRM

Ion monitored:		Q1: 311.1 and Q3: 211.1

Volume of injection: 	50 μL

Run time:		20min

Retention time: 	10.1 min.

- 1,2-Dichloroethane

Specificity

The specificity of the method was established by injecting the diluent along with the 1,2-Dichloroethane working standard solutions to the GC FID. The chromatogram of diluent was checked forabsence of any interference at the retention time of the 1,2-Dichloroethane.

Linearity

A known aliquot from the impurity reference standard stock solution prepared was transferred in to a volumetric flask and diluted with diluent as detailed below to get five different working standard solutions for detector linearity check.

Precision

About 0.1 g of the test item, Pendimethalin 455 g/L CS was weighed in  five replications in to separate head space vials and an aliquot of 1.0 mL of diluent was added. Further the vials were sealed with rubber septa and aluminium cap. The solution was used for the analysis of impurity content by injecting to GC-HS-FID.

Accuracy

About 0.1 g of the test item Pendimethalin 455 g/L CS was weighed into separate head space vials (three for each of the three fortification levels). Further, fortified with 1.0 mL of 1,2-Dichloroethane standard solution of different concentration for different fortification levels. Further the vials were sealed with rubber septa and aluminium cap and used for analysis of 1,2-Dichloroethane content by injecting to GC-FID

Chromatographic conditions

Instrument: 	Gas Chromatograph equipped with HS auto sampler, FID Detector and PC based data system

Column: 	DB-624, 30 m long, 0.32 mm internal diameter, 1.8 μm film thickness

Split ratio:	25:1

Column Oven: 	Initial: 40℃ hold for 5min

Ramp 2: 7℃/min to 100℃, hold for 0 min

Ramp 3: 60℃/min to 250℃, hold for 5min

Helium: 	2.0 mL/min

- N-Nitrosoamines

Specificity

Specificity and selectivity of the method was established by injecting duplicate preparations if diluent along with the impurity working standard solution to the GC-MS/MS. The chromatograms od the diluent were checked for the absence of any interference at the retention time of the impurities.

Linearity

A known aliquot from each of the impurity reference standard stock solutions prepared was transferred in to a volumetric flask and diluted with diluent (Acetonitrile) to get five different working standard solutions for detector linearity check. Equal volumes each of the impurity working standard solutions were injected in triplicate into GC-MS/MS. The peak areas of impurity were recorded for each injection.

Precision

The N-Nitrosoamine impurities were not detected in the test item. To calculate the precision of the method, the test item was fortified with known amount of N-Nitrosoamine impurities. About 0.1 g of the test item, Pendimethalin 455 g/L CS was weighed in five replications to separate 5 mL volumetric flasks and an aliquot of 0.25 mL of N-Nitrosoamine impurity mix standard solution of about 2μg/mL was added, and the contents of the flask were dissolved in diluent. Later, the volume was made up to the mark with diluent, the solution was shaken thoroughly, filtered and used for the analysis of impurity content by injecting to GC-MS/MS.

Accuracy

About 0.1 g of test item, Pendimethalin 455 g/L CS was weighed into separate 5 mL volumetric flasks (Three for each of the three fortification levels) and the contents of the volumetric flasks were dissolved in diluent. Later the℃  test item solutions were fortified with N-Nitrosoamine impurities reference standard stock solution. Later, the volume was made up to the mark with diluent, the solutions were shaken thoroughly, filtered and used for analysis of impurity content by injecting to GC-MS/MS.

Chromatographic conditions

Instrument:	Shimadzu Gas Chromatography for mass spectrometer equipped with HS-20 autosampler, GCMS-TQ8050 and PC based data system with Shimadzu GC-MS/MS software.

GC conditions

Column:		DB-35MS, 30 m long, 0.25 mm internal diameter and 0.25 μm film thickness

Split ratio:		5:1

Helium:		1.0 mL/min

Injection volume: 	1.0 μL

Run time: 21 min.

Column oven:		initial: 80℃ hold for 1.0 min

Ramp 1: 15℃/min to 150℃, hold for 0.0 min.

Ramp2: 25℃/min to 300℃, hold for 5 min.

Ion source temp:	230℃

Interface tempL		300℃

Acquisition mode: 	MRM

Validation - Results and discussions

Table 5.2-5:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) PENTAGON/SHA 2600 E

|  | N-Nitrosopendimethalin | 1,2-dichloroethane | N-Nitrosodimethylamine | N-Nitrosomethylamine N-Nitrosomethylethylamine |
| --- | --- | --- | --- | --- |
| Author(s), year | S. Revankar, 2021 |  |  |  |
| Principle of method | LC-MS/MS | GC-FID | GC-MS/MS | GC-MS/MS |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | 5 points 0.00605 to 0.24205 μg/mL First mass transition y=346748x-41 R=0.9998 0.9995 Second mass transition y=82890.15201x-51.713 R=0.99957 | 5 points 0.504 to 20.158 μg/mL First mass transition y=1684.36x-216.46 R=0.9999 | 5 points 0.0201 to 0.2510 0.2511 μg/mL First mass transition y=1505x+334165 R=0.9996 Second mass transition y=114988x+154.45 R=0.9996 | 5 points 0.0201 to 0.2511 μg/mL First mass transition y=1328x+370502 R=0.9995 Second mass transition y=182860.6884x+484.3878 R=0.9997 |
| Precision – Repeatability Mean n = 5 (%RSD) | First mass transition %RSD=3.458 %RSDR=12.26 %RSDr=8.21 Hr=0.42<1 Second mass transition %RSD=5.357 %RSDR=12.30 %RSDr=8.24 Hr=0.67<1 | First mass transition %RSD=2.113 %RSDR=9.78 %RSDr=6.55 Hr=0.32<1 | First mass transition %RSD=2.103 %RSDR=14.12 %RSDr=9.46 Hr=0.22<1 Second mass transition %RSD=4.141 %RSDR=14.12 %RSDr=9.46 Hr=0.44<1 | First mass transition %RSD=1.670 %RSDR=14.12 %RSDr=9.46 Hr=0.18<1 Second mass transition %RSD=2.509 %RSDR=13.93 %RSDr=9.33 Hr=0.27<1 |
| Accuracy 3 concentrations n = 3 (% Recovery) | First mass transition Total mean recovery = 88.932 ± 5.95% Seccond mass transition 86.390 ± 4.655% | Total mean recovery = 107.337 ± 8.684% | First mass transition Total mean recovery = 103.786 ± 8.035% Seccond mass transition 99.298 ± 5.062% | First mass transition Total mean recovery = 108.136 ± 5.552% Seccond mass transition 99.412 ± 3.08% |
| Interference/ Specificity | No interference. The method is specific. | No interference. The method is specific. | No interference. The method is specific. | No interference. The method is specific. |
| LOQ | 0.006 μg/mL | 0.504 μg/mL | 0.967 0.971 mg/kg | 0.968 0.967 mg/kg |
| Comment | - | - | - | - |

|  | N-Nitrosodiethylamine | N-Nitrosopyrrolidine | N-Nitrosodipropylamine | N-Nitrosopiperidine | N-Nitrosodibutylamine |
| --- | --- | --- | --- | --- | --- |
| Author(s), year | S. Revankar, 2021 |  |  |  |  |
| Principle of method | GC-MS/MS | GC-MS/MS | GC-MS/MS | GC-MS/MS | GC-MS/MS |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | 5 points 0.0201 to 0.2508 μg/mL First mass transition y=1067x+291975 R=0.9995 Second mass transition y=113220.3120x+354.6928 R=0.9995 | 5 points 0.0201 to 0.2508 μg/mL First mass transition y=184180x+490 R=0.9995 Second mass transition y=175963.8595x+552.1607 R=0.9995 | 5 points 0.0201 to 0.25109 0.2509 μg/mL First mass transition y=283845x+98 R=0.9974 Second mass transition y=166593.1973x+444.3638 R=0.9987 | 5 points 0.0201 to 0.2508 μg/mL First mass transition y=234217x+835 R=0.9995 Second mass transition y=124810.5807x+239.8151 R=0.9996 | 5 points 0.0201 to 0.2508 μg/mL First mass transition y=203337x+994 R=0.9996 Second mass transition y=131830.7235x-146.0902 R=0.9994 |
| Precision – Repeatability Mean n = 5 (%RSD) | First mass transition %RSD=1.650 %RSDR=14.12 %RSDr=9.46 Hr=0.17<1 Second mass transition %RSD=1.330 %RSDR=14.12 %RSDr=9.46 Hr=0.14<1 | First mass transition %RSD=2.223 %RSDR=14.03 %RSDr=9.40 Hr=0.24<1 Second mass transition %RSD=4.497 %RSDR=14.03 %RSDr=9.40 Hr=0.36<1 | First mass transition %RSD=6.205 %RSDR=13.74 %RSDr=9.21 Hr=0.67<1 Second mass transition %RSD=3.771 %RSDR=13.93 %RSDr=9.33 Hr=0.40<1 | First mass transition %RSD=1.014 %RSDR=13.93 %RSDr=9.33 Hr=0.11<1 Second mass transition %RSD=2.632 %RSDR=13.93 %RSDr=9.33 Hr=0.28<1 | First mass transition %RSD=3.494 %RSDR=14.03 %RSDr=9.40 Hr=0.37<1 Second mass transition %RSD=7.982 %RSDR=14.12 %RSDr=9.46 Hr=0.84<1 |
| Accuracy 3 concentrations n = 3 (% Recovery) | First mass transition Total mean recovery = 111.631 ± 5.661% Seccond mass transition 100.268 ± 5.82% | First mass transition Total mean recovery = 108.209 ± 6.712% Seccond mass transition 107.164 ± 4.565% | First mass transition Total mean recovery = 110.973 ± 3.196% Seccond mass transition 99.629 ± 12.29% | First mass transition Total mean recovery = 111.379 ± 6.018% Seccond mass transition 107.499 ± 5.42% | First mass transition Total mean recovery = 103.855 ± 5.261% Seccond mass transition 110.780± 4.191% |
| Interference/ Specificity | No interference. The method is specific. | No interference. The method is specific. | No interference. The method is specific. | No interference. The method is specific. | No interference. The method is specific. |
| LOQ | 0.966 0.968 mg/kg | 0.966 mg/kg | 0.967 mg/kg | 0.966 mg/kg | 0.967 mg/kg |
| Comment | - | - | - | - | - |

Conclusion

According to SANCO/3030/99 rev. 5 the method was successfully validated and is suitable for determination of Relevant impurities (1,2-dichloroethane, N-Nitroso compounds) of pendimethalin in the test item PENTAGON/SHA 2600 E.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

CPIAC method No 357 is available.

### Methods for the determination of residues (KCP 5.1.2)

An overview on the acceptable methods and possible data gaps for analysis of residues of Pendimethalin for the generation of pre-authorization data is given in the following table. For the detailed evaluation of new/ additional studies it is referred to Appendix 2.

Table 5.2-6:	Validated methods for the generation of pre-authorization data

| Component of residue definition: Pendimethalin |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Water – fish (Ecotoxicology) | Primary | 0.5 mg/L | GC-FID | KCP 5.1.2-3 Murali K., 2020 Report no. 7887/2020 |
|  | Confirmatory (if required) | - | - | Not required |
| Reconstituted Water Medium – Daphnia magna (Ecotoxicology) | Primary | 0.1 mg/L | GC-FID | KCP 5.1.2-4 Rajeshwari S. 2021 Report no. 9010/2021 |
|  | Confirmatory (if required) |  | - | Not required |
| OECD medium– algal growth inhibition test (Ecotoxicology) | Primary | 0.1 mg/L | GC-FID | KCP 5.1.2-5 Radha S., 2021 Report no. 9008/2021 |
|  | Confirmatory (if required) | - | - | Not required |
| 20xAAP Medium– Lemna gibba (Ecotoxicology) | Primary | 0.1 mg/L | GC-FID | KCP 5.1.2-6 Radha S., 2021 Report no: 9009/2021 |
|  | Confirmatory (if required) | - | - | Not required |
| Artifical Soil – Hypoaspis (Ecotoxicology) | Primary | 0.104 mg/kg | GC-FID | KCP 5.1.2-7 Angayarkanni V., 2021 Report no. 8925/2021 |
|  | Confirmatory (if required) | - | - | Not required |
| Artifical Soil – Folsomia candida (Ecotoxicology) | Primary | 4.375µg/L (corresponding to 0.05 mg/kg dry weight of soil) | GC-FID | KCP 5.1.2-8 Dec W. 2021 Report No.: EMI/4/17/2020 |
|  | Confirmatory (if required) | - | - | Not required |
| Bee diet - Chronic oral toxicity to Honey Bee (Apis mellifera L.) (Ecotoxicology) | Primary | 0.1 mg/L | HPLC-UV | KCP 5.1.2-9 Glanas, A. 2017 Report no. B/107/17 |
|  | Confirmatory (if required) | - | - | Not required |
| Larval diet, dose verification – Chronic honeybee larvae study (Apis mellifera L.) (Ecotoxicology) | Primary | 174.7 mg/L pendimethalin (DF 3.494 mg/L) | HPLC-DAD | KCP 5.1.2-10 Kleebaum, K. 2017 Report no. 17 48 BLC 0083 |
|  | Confirmatory (if required) | - | - | Not required |
| Deionized water - Seedling Emergence and Seedling Growth – aqueous solution (Ecotoxicology) | Primary | 0.1 mg/L | GC-MS/MS | KCP 5.1.2-11 Radha S., 2021 Report no. 9419/2021 |
|  | Confirmatory (if required) | - | - | Not required |
| Deionized water - Vegetative Vigour – aqueous solution (Ecotoxicology) | Primary | 0.1 mg/L | GC-MS/MS | KCP 5.1.2-12 Radha S., 2021 Report no. 9420/2021 |
|  | Confirmatory (if required) | - | - | Not required |

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of pendimethalin (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-7:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Pendimethalin | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Plant, high acid content |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Plant, high protein/high starch content (dry commodities) |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Plant, high oil content |  | 0.05 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.05 mg/kg | Reg. (EU) 2019/1791 |
| Muscle | Pendimethalin | 0.01 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Milk |  | 0.02 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Eggs |  | 0.01 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Fat |  | 0.01 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Liver, kidney |  | 0.01 mg/kg | Reg. (EU) 2019/1791 Reg. (EU) 2023/1049 |
| Soil (Ecotoxicology) | Pendimethalin | 0.05 mg/kg | common limit |
| Drinking water (Human toxicology) | Pendimethalin | 0.1 µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | Pendimethalin | 0.006 mg/L | Lowest NOEC from aquatic toxicity study (Long term toxicity fish – Pimephales promelas) |
| Air | Pendimethalin | 51.0 µg/m3 | AOEL: 0.17 mg/kg bw/d |
| Tissue (meat or liver) | Pendimethalin | Not required 0.01 mg/kg | Not classified as T / T+ SANTE/2020/12830, Rev.1 |
| Body fluids |  | Not required 0.01 mg/L | Not classified as T / T+ SANTE/2020/12830, Rev.1 |

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of pendimethalin in plant matrices is given in the following tables. For the detailed evaluation of new/ additional studies it is referred to Appendix 2.

Table 5.3-8:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: pendimethalin |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content | Primary | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Weber H., 2011 |
|  | ILV | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Scherthan D., 2012 |
|  | Primary | 0.01 mg/kg | HPLC-MS/MS | KCP 5.3.2.2.1 M. L. Greco, 2017 Report No. 16.566423.0002 |
|  | ILV | 0.01 mg/kg | HPLC-MS/MS | KCP 5.3.2.2.5 J. Kicińska, 2017 Report No. ZBBZ-2016/12/DPL/4DE |
|  | Confirmatory (if required) | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |
| High acid content | Primary | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Weber H., 2011 |
|  | ILV | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Scherthan D., 2012 |
|  | Primary | 0.01 mg/kg | HPLC-MS/MS | KCP 5.3.2.2.2 M. L. Greco, 2017 Report No. 16.566423.0003 |
|  | Confirmatory (if required) | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |
| High oil content | Primary | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Weber H., 2011 |
|  | ILV | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Scherthan D., 2012 |
|  | Primary (soy bean) | 0.01 mg/kg | HPLC-MS/MS | KCP 5.3.2.2.3 (Maria Laura Greco, 2017; Study N. 16.566423.0001) |
|  | ILV | 0.01 mg/kg | HPLC-MS/MS | KCP 5.3.2.2.6 J. Kicińska, 2018 Report No. ZBBZ-2016/12/DPL/8PL |
|  | Confirmatory (if required) | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |
| High protein/high starch content (dry) | Primary | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Weber H., 2011 |
|  | ILV | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Scherthan D., 2012 |
|  | Primary (cereals) | 0.01 mg/kg | HPLC-MS/MS | KCP 5.3.2.2.4 M. L. Greco, 2017 Report No. 16.566423.0005 |
|  | Confirmatory (if required) | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |

Table 5.3-9:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | Please refer to the RAR, 2015 |
| Not required, because: | - |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

Based on EFSA Journal 2016;14(3):4420:

Not required since no MRLS are proposed.

zRMS:

It should be noted that the applicant has provided a data matching dossier to support the renewal of the product. In the dossier, analytical method HPLC-MS/MS based on QUECHERS extraction was accepted according to SANCO/825/00 Rev. 8.1 for determination of residues in kidney with an LOQ of 0.01 mg/kg.  No ILV was deemed necessary as QUECHERS extraction has been used. Furthermore, for the other animal matrices (liver, fat, muscle, eggs and milk) an unprotected multiresidue method DFG S19 with GC-ECD with an LOQ of 0.01 mg/kg was found acceptable in the data matching dossier (NL, May 2018, May 2020).

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of pendimethalin in soil is given in the following tables. For the detailed evaluation of new/ additional studies it is referred to Appendix 2.

Table 5.3-10:	Validated methods for soil (if appropriate)

| Component of residue definition: pendimethalin |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.01 mg/kg | HPLC-MS/MS | RAR 2015, EFSA 2016 Heinz N.,2013; Class T., 2013 |
| Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.3.2.4.1 (Maria Laura Greco, 2017; Study N. 16.566423.0008) |
| Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.3.2.4.2 (Maria Laura Greco, 2017; Study N. 16.566423.0009) |
| Confirmatory | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of pendimethalin in surface and drinking water is given in the following tables. For the detailed evaluation of new/ additional studies it is referred to Appendix 2.

Table 5.3-11:	Validated methods for water (if appropriate)

| Component of residue definition: pendimethalin |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | 0.02 μg/L | HPLC-MS/MS | RAR 2015, EFSA 2016 Heinz N., 2013 |
|  | ILV | 0.02 μg/L | HPLC-MS/MS | RAR 2015, EFSA 2016 Wiesner F., Breyer N., 2013 |
|  | Primary | 0.1 μg/L | GC-MS | KCP 5.3.2.5.1 (G Andrews 2015; Study N. YV/13/034) |
|  | ILV | 0.1 μg/L | GC-MS | KCP 5.3.2.5.2 (K. Mclnerney 2016; Study N. 100041492B) |
|  | Confirmatory | - | - | Not required. |
| Surface water | Primary | 0.02 μg/L | HPLC-MS/MS | RAR 2015, EFSA 2016 Heinz N., 2013 |
|  | Primary | 0.02 μg/L | HPLC-MS/MS | KCP 5.3.2.5.5 M. L. Greco, 2017 Report No. 16.566423.0010 |
|  | Confirmatory | - | - | Not required. |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of pendimethalin in air is given in the following tables. For the detailed evaluation of new/ additional studies it is referred to Appendix 2.

Table 5.3-12:	Validated methods for air (if appropriate)

| Component of residue definition: pendimethalin |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 4 μg/m3 | LC-MS/MS | RAR 2015, EFSA 2016 Penning H., 2013 |
|  | 4 μg/m3 | LC-MS/MS | KCP 5.3.2.6.1 (Maria Laura Greco, 2017; Study N. 16.566423.00012) |
| Confirmatory | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of pendimethalin in body fluids and tissues is given in the following table. For the detailed evaluation of new/ additional studies it is referred to Appendix 2.

Table 5.3-13:	Methods for body fluids and tissues (if appropriate)

| Component of residue definition: pendimethalin |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary (blood) | 0.05 mg/L | LC-MS/MS | KCP 5.3.2.7.1 (Maria Laura Greco, 2017; Study N. 16.566423.0007) |
| Confirmatory | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |
| Primary (liver) | 0.01 mg/kg | LC-MS/MS | KCP 5.3.2.7.2 (Maria Laura Greco, 2017; Study N. 16.566423.0004) and amendment |
| Confirmatory | - | - | HPLC-MS/MS is highly selective method, therefore no confirmatory method is required. |

For any special comments or remarkable points concerning the analytical methods for body fluids and tissues please refer to Appendix 2.

#### Other studies/ information

Not relevant.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1 | Manisha K. Varma | 2017 | Validation of analytical method for determination of active ingredient content of pendimethalin 455 g/L CS. Document no.: 228-2-12-16345 Institute of Industrial Organic Chemistry GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.1-2 | B. Krzysiak-Warzała | 2021 | Pendimethalin 45.5% CS Analysis of active substance content and physicochemical properties of preparation and preparation after acelerated and freeze/that storage procedures. Report No. 47/2021 Institute of Heavy Organic Synthesis “Blachownia” GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2 | A. Chałas | 2017 | Pendimethalin 455 g/L CS. Methods validation for determination of the relevant impurities of pendimethalin. Document no.: BA-45/17 JAI Reesearch Foundation GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-2 | S. Revankar | 2021 | Accelerated storage stability test by heating at elevated temperature of Pendimethalin 455 g/L CS. Report No. G21360 Eurofins Advinus GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-3 | Murali, K. | 2020 | Rainbow trout (Oncorhynchus mykiss), acute toxicity test with Pendimethalin 45.5% CS Report No.: 7887/2020 Bioscience Research Foundation, India GLP Unpublished | Y | Sharda Cropchem Ltd |
| KCP 5.1.2-4 | Rajeshwari, S. | 2021 | Study of Daphnia magna acute immobilisation with Pendimethalin 45.5% CS Report No.: 9010/2021 Bioscience Research Foundation, India GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-5 | Radha, S. | 2021 | Study of algal growth inhibition with Pendimethalin 45.5% CS Report No.: 9008/2021 Bioscience Research Foundation, India GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-6 | Radha, S. | 2021 | Study of Lemna gibba growth inhibition with Pendimethalin 45.5% CS Report No.: 9009/2021 Bioscience Research Foundation, India GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-7 | Angayarkanni, V. | 2021 | Effect of Pendimethalin 45.5% CS on the Reproductive Output of the Predatory Soil Mite Hypoaspis (Geolaelaps) aculeifer Canestrini (Acari: Laelapidae) in Artificial Soil. Report No.: 8925/2021 Bioscience Research Foundation, India GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-8 | Dec W. | 2021 | Collembolan (Folsomia candida) Reproduction Test Report No.: EMI/4/17/2020 Ecomelius Institute Sp. z.o. o, Poland GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-9 | Glanas, A. | 2017 | Pendimethalin Technical Honeybees (Apis mellifera), chronic oral toxicity test Report No.: B/107/17 Institute of Industrial Organic Chemistry Branch Pszczyna, Poland GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2-10 | Kleebaum, K. | 2017 | Pendimethalin Technical – Repeated exposure of honeybee (Apis mellifera L.) larvae under laboratory conditions (in vitro) Report No.: 17 48 BLC 0083 BioChem agrar, Germany GLP Unpublished | N | Sharda Cropchem Ltd |
| KPC 5.1.2-11 | Radha, S. | 2021 | Effect of Pendimethalin 45.5% CS on seedling emergence and seedling growth of terrestrial plants Report No. 9419/2021 Bioscience Research Foundation, India GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.1.2.-12 | Radha, S. | 2021 | Effect of Pendimethalin 455 g/L on vegetative vigour of terrestrial plants Report No. 9420/2021 Bioscience Research Foundation, India GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.2.1 | M. L. Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1) in potato by liquid chromatography. M. L. Greco, 2017, Chelab Report No. 16.566423.0002 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.2.2 | M. L. Greco | 2017 | Validation of the analytical procedure for the determination of Pendimethalin (CAS: 40487-42-1) in oranges by liquid chromatography. M. L. Greco, 2017, Chelab Report No. 16.566423.0003 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.2.3 | Maria Laura Greco | 2014 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in soy beans by liquid chromatography. Study No. 16.566423.0001 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.2.4 | M. L. Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in wheat grains by liquid chromatography. M. L. Greco, 2017, Chelab Report No. 16.566423.0005 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.2.5 | J. Kicińska | 2017 | Determination of residues of Pendimethalin applied as “Pendimethalin 330 g/L” in potato at one site in Germany, 2016. J. Kicińska, 2017, Food Safety Laboratory Report No. ZBBZ-2016/12/DPL/4DE GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.2.6 | J. Kicińska | 2018 | Determination of residues of Pendimethalin applied as “Penddimethalin 330 g/L EC” in soybean at one site in Poland, 2016. Food Safety Laboratory Report No. ZBBZ-2016/12/DPL/8PL GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.4.1 | Maria Laura Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), loamy sand soil by liquid chromatography. Study No. 16.566423.0008 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.4.2 | Maria Laura Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in sandy loam soil by liquid chromatography. Study No. 16.566423.0009 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.5.1 | G Andrews | 2015 | Method Validation: Determination of Residues of Pendimethalin in Drinking Water Samples Study No. YV/13/034 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.5.2 | K. Mclnerney | 2016 | Independent Laboratory Validation (ILV) of Analytical Method for the Determination of Residues of Pendimethalin in Drinking Water Study No. 100041492B GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.5.5 | Maria Laura Greco | 2017 | Validation of the analytical procedure for the determination of Pendimethalin (CAS: 40487-42-1), in surface water by liquid chromatography. Report No. 16.566423.0010 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.6.1 | Maria Laura Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in air by liquid chromatography. Study No. 16.566423.0012 GLP Unpublished | N | Sharda Cropchem Ltd |
| KCP 5.3.2.7.1 | Maria Laura Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in blood by liquid chromatography. Study No. 16.566423.0007 GLP Unpublished | Y | Sharda Cropchem Ltd |
| KCP 5.3.2.7.2 and Amendment | Maria Laura Greco | 2017 | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in liver by liquid chromatography. Study No. 16.566423.0004 GLP Unpublished | Y | Sharda Cropchem Ltd |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for pendimethalin

    - Methods used for the generation of pre-authorization data (KCP 5.1)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-3 |
| --- | --- |
| Report | Rainbow trout (Oncorhynchus mykiss), acute toxicity test with Pendimethalin 455 g/l CS, Murali K., 2020 Report no. 7887/2020 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in ground water medium. The limit of quantification (LOQ) of the method was 0.5 mg/L. The quantitative measurements of pendimethalin were performed by GC-FID with external calibration.

Reference substance: Pendimethalin: batch no. G150077, Purity: 98.87%, Dr Ehrenstorfer

Standard stock solution:

An amount of 10.12 mg Pendimethalin reference standard (98.87%) was weighed into a 10 mL flask and was made up to the mark with acetonitrile. This solution was equivalent to 1000.56 mg/L. From this, an aliquot of 2.5 ml solution was diluted in 25 mL volumetric flask of acetonitrile. This solution was equivalent to 100 mg/L.

Preparation of sample solution

An amount of 10.0 mg test substance was weighed into a 100 mL flask, dissolved in acetonitrile and made up to the mark with acetonitrile. This solution was equivalent to 100 mg/L and used for determination of specificity, linearity.

Preparation of calibration solution

From the specificity standard stock solution (100 mg/L), the serial dilutions were made by using acetonitrile, to prepare further concentrations 0.5, 1, 2, 4, 8 and 16 mg/L.

Preparation of sample solution (Precision)

An amount of 21.38 and 80.75 mg test substance was weighed accurately into 10 mL volumetric flask and made up to the mark with acetonitrile. These solutions were equivalent 2138 and 8075 mg/L, respectively. An aliquot of 0.01 mL sample solution was taken into 10 mL volumetric flask, diluted with acetonitrile and made up to the mark with the acetonitrile. These solutions were equivalent to 2.138 and 8.075 mg/L.

GC-FID Conditions:

| GC | Shimadzu, Model GC-FID-2010 equipped with FID detector |
| --- | --- |
| Column | SH-Rxi-5ms (30 mm x 0.25 mm) |
| Injection temperature | 160 °C |
| Column Oven Temperature | 350°C |
| Solvent | Acetonitrile |
| Injection volume | 1 µl |
| Flow rate | 19.8 ml/min |
| Carrier gas | Nitrogen |
| Retention time | approximately 9.01 minutes |
| Run Time | 12 minutes |

- Results and discussions

Table A 1:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Ground water | Pendimethalin | 0.5 | 99.35 | 0.753 | - |
|  |  | 1.0 | 98.60 | 0.483 | - |

Table A 2:	Characteristics for the analytical method used for validation of pendimethalin in ground water

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity of GC method for Pendimethalin was determined by injecting the standard and sample solutions along with blank (acetonitrile) and observed that there was no interference found with main peak of interest. |
| Calibration (type, number of data points) | 6 points Linear calibration Y= 3522.8x + 972.04 R = 0.9995 |
| Calibration range | 0.5 mg/L to 16 mg/L. |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ= 0.5 mg/L |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin with an LOQ of 0.5 mg/L in ground water.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-4 |
| --- | --- |
| Report | Study of Daphnia magna acute immobilisation with Pendimethalin 455 g/l CS, Rajeshwari S., 2021 Report no. 9010/2021 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in reconstituted water medium. The limit of quantification (LOQ) of the method was 0.1 mg/L. The quantitative measurements of pendimethalin were performed by GC-FID with external calibration.

Reference substance: Pendimethalin: batch no. G998095, Purity: 97.34%, Dr Ehrenstorfer

Standard stock solution:

An amount of 10.28 mg Pendimethalin reference standard (97.34%) was weighed into a 10 mL flask and was made up to the mark with acetonitrile. This solution was equivalent to 1000.66 mg/L. From this, an aliquot of 2.5 ml solution was diluted in 25 mL volumetric flask of acetonitrile. This solution was equivalent to 100 mg/L.

Preparation of calibration solution

From the specificity standard stock solution (100 mg/L), the serial dilutions were made by using acetonitrile, to prepare further concentrations 0.05, 0.1, 0.5, 1, 3, 5 and 7 mg/L.

Preparation of sample solution (Precision)

An amount of 11.42 and 68.95 mg test substance was weighed into a 100 mL volumetric flask and made up to the mark with reconstituted water medium and the fortified samples were transferred into a clean and dry 100 mL flask and the content was evaporated near to dryness. The resultant residue was dissolved in ethyl acetate and the organic part was again dried near to dryness and the resulting residue was dissolved with acetonitrile. These solutions were equivalent to 114.2 and 689.5 mg/L respectively. An aliquot of each 0.1 sample solutions were taken into 10 mL volumetric flask diluted with acetonitrile and made up to the mark with the acetonitrile. These solutions were equivalent to 1.142 and 6.895 mg/L respectively. The prepared solutions were analysed by GC-FID.

GC-FID Conditions:

| GC | Shimadzu, Model GC-FID-2010 equipped with FID detector |
| --- | --- |
| Column: | SH-Rxi-5ms (30 mm x 0.25 mm) |
| Injection temperature: | 160 °C |
| Column Oven Temperature | 350°C |
| Solvent | Acetonitrile |
| Injection volume | 1 µl |
| Flow rate | 19.8 ml/min |
| Carrier gas | Nitrogen |
| Retention time | approximately 8.35 minutes |
| Run Time | 10 minutes |

- Results and discussions

Table A 3:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Reconstituted wter medium | Pendimethalin | 0.1 | 101.84 | 0.11 | - |
|  |  | 1.0 | 101.05 | 0.20 | - |

Table A 4:	Characteristics for the analytical method used for validation of pendimethalin in reconstituted water medium

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity of GC method for pendimethalin was determined by injecting the standard and sample solutions along with blank (solvent/diluent and medium in diluent (acetonitrile)) and observed that there was no interference found with main peak of interest at RT 8.3 min. |
| Calibration (type, number of data points) | 7 points Linear calibration Y= 10848x – 19.129 R = 0.9998 |
| Calibration range | 0.05 mg/L to 7 mg/L. |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ= 0.1 mg/L |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin in reconstituted water medium with an LOQ of 0.1 mg/L.

        - Analytical method 3

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-5 |
| --- | --- |
| Report | Study of algal growth inhibition with Pendimethalin 455 g/I CS, Radha S., 2021 Report no. 9008/2021 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in OECD medium. The limit of quantification (LOQ) of the method was 0.1 mg/L. The quantitative measurements of pendimethalin were performed by GC-FID with external calibration.

Reference substance: Pendimethalin: batch no. G998095, Purity: 97.34%, Dr Ehrenstorfer

Preparation of standard stock solution:

An amount of 10.28 mg Pendimethalin reference standard (97.34%) was weighed into a 10 mL flask and was made up to the mark with acetonitrile. This solution was equivalent to 1000.66 mg/L. From this, an aliquot of 2.5 ml solution was diluted in 25 mL volumetric flask of acetonitrile. This solution was equivalent to 100 mg/L.

Preparation of calibration solution

From the specificity standard stock solution (100 mg/L), the serial dilutions were made by using acetonitrile, to prepare further concentrations 0.05, 0.1, 0.5, 1, 3, 5 and 7 mg/L.

Preparation of sample solution (Precision)

An amount of 11.93 and 69.00 mg test substance was weighed into a 100 mL volumetric flask, fortified the contents with OECD medium and made up to the mark with OECD medium and the fortified samples was transferred into a clean and dry 100 mL Round Bottom (RB) flask and the content was evaporated near to dryness. The resultant residue was dissolved in ethyl acetate. The organic part was again dried and the resulting residue was dissolved with acetonitrile. These solutions were equivalent to 119.3 and 690.0 mg/L respectively. An aliquot of each 0.1 sample solutions were taken into 10 mL volumetric flask diluted with acetonitrile and made up to the mark with the acetonitrile. These solutions were equivalent to 1.193 and 6.900 mg/L respectively. The prepared solutions were analysed by GC-FID.

GC-FID Conditions:

| GC | Shimadzu, Model GC-FID-2010 equipped with FID detector |
| --- | --- |
| Column: | SH-Rxi-5ms (30 mm x 0.25 mm) |
| Injection temperature: | 160 °C |
| Column Oven Temperature | 350°C |
| Solvent | Acetonitrile |
| Injection volume | 1 µl |
| Flow rate | 19.8 ml/min |
| Carrier gas | Nitrogen |
| Retention time | approximately 8.35 minutes |
| Run Time | 10 minutes |

- Results and discussions

Table A 5:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| OECD medium | Pendimethalin | 0.1 | 99.81 | 0.27 | - |
|  |  | 1.0 | 100.08 | 0.34 | - |

Table A 6:	Characteristics for the analytical method used for validation of pendimethalin in OECD medium

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity of GC method for pendimethalin was determined by injecting the standard, OECD medium matrix and test media OECD medium sample solutions along with solvent blank/ diluent (acetonitrile) and observed that there was no interference found with the main peak of interest. |
| Calibration (type, number of data points) | 7 points Linear calibration Y= 10848x – 19.129 R = 0.9999 |
| Calibration range | 0.05 mg/L to 7 mg/L. |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ= 0.1 mg/L |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin in OECD medium with an LOQ of 0.1 mg/L.

        - Analytical method 4

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-6 |
| --- | --- |
| Report | Study of Lemna gibba growth inhibition with Pendimethalin 455 g/1 CS, Radha S., 2021 Report no. 9009/2021 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in 20xAAP Medium. The limit of quantification (LOQ) of the method was 0.1 mg/L. The quantitative measurements of pendimethalin were performed by GC-FID with external calibration.

Reference substance: Pendimethalin: batch no. G998095, Purity: 97.34%, Dr Ehrenstorfer

Preparation of standard stock solution:

An amount of 10.28 mg Pendimethalin reference standard (97.34%) was weighed into a 10 mL flask and was made up to the mark with acetonitrile. This solution was equivalent to 1000.66 mg/L. From this, an aliquot of 2.5 ml solution was diluted in 25 mL volumetric flask of acetonitrile. This solution was equivalent to 100 mg/L.

Preparation of calibration solution

From the specificity standard stock solution (100 mg/L), the serial dilutions were made by using acetonitrile, to prepare further concentrations 0.05, 0.1, 0.5, 1, 3, 5 and 7 mg/L.

Preparation of sample solution (Precision)

An amount of 11.50 and 69.00 100 test substance was weighed into a 100 mL volumetric flask, fortified the contents with 20xAAP medium and made up to the mark with 20xAAP medium and the fortified sample was transferred into a clean and dry 100 mL round bottom (RB) flask and the content was evaporated near to dryness. The resultant residue was dissolved in ethyl acetate. The organic part was again dried and the resulting residue was dissolved with acetonitrile. These solutions were equivalent to 115.0 and 691.1 mg/L respectively. An aliquot of each 0.1 sample solutions were taken into 10 mL volumetric flask diluted with acetonitrile and made up to the mark with the acetonitrile. These solutions were equivalent to 1.150 and 6.911 mg/L respectively. The prepared solutions were analysed by GC-FID.

GC-FID Conditions:

| GC | Shimadzu, Model GC-FID-2010 equipped with FID detector |
| --- | --- |
| Column | SH-Rxi-5ms (30 mm x 0.25 mm) |
| Injection temperature | 160 °C |
| Column Oven Temperature | 350°C |
| Solvent | Acetonitrile |
| Injection volume | 1 µl |
| Flow rate | 19.8 ml/min |
| Carrier gas | Nitrogen |
| Retention time | approximately 8.35 minutes |
| Run Time | 10 minutes |

- Results and discussions

Table A 7:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| 20xAAP medium | Pendimethalin | 0.1 | 101.9 | 0.07 | - |
|  |  | 1.0 | 99.85 | 0.46 | - |

Table A 8:	Characteristics for the analytical method used for validation of pendimethalin in 20xAAP medium

|  | Pendimethalin |
| --- | --- |
| Specificity | The Specificity of GC method for pendimethalin was determined by injecting the standard, 20xAAP medium matrix (in the Chromatogram mentioned as Control) and test media 20xAAP medium sample solutions along with solvent blank/diluent (acetonitrile) and observed that there was no interference found with the main peak of interest. |
| Calibration (type, number of data points) | 7 points Linear calibration Y= 10848x – 19.129 R = 0.9999 |
| Calibration range | 0.05 mg/L to 7 mg/L. |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ= 0.1 mg/L |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 4 and is suitable for the determination of pendimethalin in 20xAAP medium with an LOQ of 0.1 mg/L.

        - Analytical method 5

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-7 |
| --- | --- |
| Report | Effect of Pendimethalin 455g/L CS on the reproductive output of the predatory soil mite Hypoaspis (Geolaelaps) aculeifer Canestrini (Acari: Laelapidae) in artificial soil, Angayarkanni V., 2021 Report no. 8925/2021 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in soil medium. The limit of quantification (LOQ) of the method was 0.1 mg/kg. The quantitative measurements of pendimethalin were performed by GC-FID with external calibration.

Reference substance: Pendimethalin: batch no. G998095, Purity: 97.34%, Dr Ehrenstorfer

Preparation of standard stock solution:

An amount of 10.28 mg Pendimethalin reference standard (97.34%) was weighed into a 10 mL flask and was made up to the mark with acetonitrile. This solution was equivalent to 1000.66 mg/L. From this, an aliquot of 2.5 ml solution was diluted in 25 mL volumetric flask of acetonitrile. This solution was equivalent to 100 mg/L.

Preparation of calibration solution

From the specificity standard stock solution (100 mg/L), the serial dilutions were made by using acetonitrile, to prepare further concentrations 0.05, 0.1, 0.5, 1, 3, 5 and 7 mg/L.

Preparation of sample solution (Precision)

To 100g of soil an amount of 12.02 and 68.30 mg of test substance and 100g acetonitrile was into a 250 mL conical flasks separately. After mixing, the samples were filtered and the filtrate was passed through a glass column packed with silica gel and the extract was collected into a round bottom (RB) flask and concentrated using vacuum rotary evaporator near to dryness after that the residue was dissolved with 100g of acetonitrile. Which were equivalent to 1202 and 6830 mg/kg respectively. From this an aliquot 0.01ml sample solutions were taken into 10 mL volumetric flask and diluted with 10g of acetonitrile. Which were equivalent to 1.202 and 6.830 mg/kg respectively. The prepared solutions were analysed by GC-FID.

GC-FID Conditions:

| GC | Shimadzu, Model GC-FID-2010 equipped with FID detector |
| --- | --- |
| Column: | SH-Rxi-5ms (30 mm x 0.25 mm) |
| Injection temperature: | 160 °C |
| Column Oven Temperature | 350°C |
| Solvent | Acetonitrile |
| Injection volume | 1 µl |
| Flow rate | 19.8 ml/min |
| Carrier gas | Nitrogen |
| Retention time | approximately 8.35 minutes |
| Run Time | 10 minutes |

- Results and discussions

Table A 9:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soil | Pendimethalin | 0.1 | 99.70 | 0.31 | - |
|  |  | 1.0 | 100.51 | 0.21 | - |

Table A 10:	Characteristics for the analytical method used for validation of pendimethalin in soil

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity of GC method for pendimethalin was determined by injecting the standard, soil matrix and test media soil sample solutions along with solvent blank/diluent (acetonitrile) and observed that there was no interference found with the main peak of interest. |
| Calibration (type, number of data points) | 7 points Linear calibration Y= 10848x – 19.129 R = 0.9999 |
| Calibration range | 0.05 mg/L to 7 mg/L. |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ= 0.1 mg/L |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin in soil medium with an LOQ of 0.1 mg/kg.

        - Analytical method 6

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-8 |
| --- | --- |
| Report | Collembolan (Folsomia candida) Reproduction Test according to OECD Guideline No. 232 (2016), Dec W., 2021 Report no. EMI/4/17/2020 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in artificial soil. The limit of quantification (LOQ) of the method was 4.375µg/L (corresponding to 0.05 mg/kg dry weight of soil). The quantitative measurements of pendimethalin were performed by LC-MS/MS.

Test substances:

| Test substance | Pendimethalin 455 g/L CS |
| --- | --- |
| Batch no. | SCL- 14177 |
| Active ingredient content | Pendimethalin – 455.0 g/L |

Reference substance: Pendimethalin: batch no. BCBX2084, Purity: 98.7%

Artificial soil:  5% sphagnum peat, 20% kaolin clay, and 75% quartz sand

Sample preparation

A sample amount of about 1.25g were mixed with 4 mL of water, 8 mL acetonitrile and a salt mixture. The supernatant was further diluted and analysed by LC-MS/MS.

| HPLC | 1290 Agilent Infinity. 6460 Triple Quad Mass Spectrometer (ion source AJS ESI) |
| --- | --- |
| Column | Zorbax SB-C18 Guard column 2.1 x 5mm, 1.8µm Zorbax SB-C18 RRHT Column 2.1 x 50mm. 1.8µm |
| Injection volume | 10 µL |
| Column temperature | 40°C |
| Mobile phases: | A: water + ammonium formate (0.1%) B: acetonitrile + formic acid (0.05%) |
| Gradient: | Time (min); A (%); B (%); Flow (mL/min). 0.00; 30; 70; 0.20. 2.60; 20; 80; 0.40. 3.50; 0; 100; 0.40. 4.00; 30; 70; 0.40. |
| Mass conditions | Precursor (m/z); Product (m/z); Dwell; Frag (V); CE (V); Cell Acc (V). 282.2; 212; 250; 56; 10; 4. 282.2; 91.1; 250; 56; 30; 4. |
| Polarity: | Positive |
| Transition (Target) | 282.2 → 212 (quantification) 282:2 → 91.1 (confirmation) |
| Retention time | Approx. 2.1 min |

- Results and discussions

Table A 11:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 6) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Artifical soil | Pendimethalin | 0.05 | 99.2 | 2.8 | - |
|  |  | 0.5 | 91.4 | 3.3 | - |
|  |  | 500 (n=5) | 97.9 | 2.3 | - |

Table A 12:	Characteristics for the analytical method used for validation of pendimethalin in artificial soil

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity was determined on basis of chromatograms obtained for the control artificial soil samples and the fortified sample (LOQ) The analysis showed that no signal of the detected substance was overlapping with the matrix signal of the control samples. |
| Calibration (type, number of data points) | 10 points Linear calibration Y= 6720.8x – 3965 R² = 0.9985 |
| Calibration range | 1.5 µg/L to 150 µg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 4.375µg/L (corresponding to 0.05 mg/kg dry weight of soil) LOD = 1.31 µg/L (corresponding to 0.015 mg/kg dry weight of soil |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin in artificial soil with an LOQ of 4.375µg/L (corresponding to 0.05 mg/kg dry weight of soil).

        - Analytical method 7

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-9 |
| --- | --- |
| Report | Pendimethalin Technical: Honeybees (Apis mellifera L.), Chronic Oral Toxicity Test, Glanas, A. 2017, Report no. B/107/17. |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in sucrose solution. The limit of quantification (LOQ) of the method was 0.1 mg/L. The quantitative measurements of pendimethalin were performed by HPLC-UV/VIS with external calibration.

Reference substance: Pendimethalin standard, Purity: 99.3 ± 0.1%, IPO Warsaw

Sample Preparation

Each sample in a weight between 1 – 15 g was shaken twice with 20 mL of dichloromethane. The organic phases were filtered through anhydrous sodium sulphate (VI). The combined extracts were evaporated to dryness using vacuum rotary evaporator at 45°C. The dry residue was dissolved in acetonitrile and injected to the chromatographic column in a volume of 20.0 µL.

HPLC Conditions:

| HPLC | Varian Prostar liquid chromatograph with UV-Vis detector |
| --- | --- |
| Column | Luna 5μ C18, 100A, l = 250 x 4.6 mm |
| Mobile phase | Acetonitrile : 0.05% solution of orthophosphoric (V) acid, (90 : 10, v/v), |
| Injection volume | 20 µl |
| Wave length | 240 nm |
| Flow rate | 1.0 mL/min |

- Results and discussions

Table A 13:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| sucrose solution | Pendimethalin | 0.1 | 94.3 | 3.1 | - |
|  |  | 10.0 | 101.3 | 1.8 | - |

Table A 14:	Characteristics for the analytical method used for validation of pendimethalin in sucrose solution

|  | Pendimethalin |
| --- | --- |
| Specificity | The analytical method specificity was estimated on the basis of the analysis of the chromatograms obtained for the control sucrose solution samples and fortified samples. The analysis showed that no signal of the detected substance was overlapping with the matrix signal of the control samples. |
| Calibration (type, number of data points) | 6 points Linear calibration Y= 8.618464e+005x R= 0.9998 |
| Calibration range | 0.05 µg/mL to 10.0 µg/mL |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.1 mg/L; corresponds to the lowest validated fortification level. |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin in sucrose solution with an LOQ of 0.1 mg/L.

        - Analytical method 8

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-10 |
| --- | --- |
| Report | Pendimethalin Technical - Repeated exposure of honey bee (Apis mellifera L.) larvae under laboratory conditions (in vitro), Kleebaum K., 2017, report no. 17 48 BLC 0083 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in test item stock solutions. The limit of quantification (LOQ) of the method was 174.7 mg/L of pendimethalin. The quantitative measurements of pendimethalin were performed by HPLC-DAD with external calibration.

Reference substance: Pendimethalin Technical, Batch No.: SCL-5983; Analyzed purity: 98.09% (w/w)

Sample matrix: Acetone

Dilution medium: Equal volumes of acetone and water were mixed

Ana. Reference item stock solution 2017/0486-1:
11.58 mg of the analytical reference item pendimethalin (98.09%) were weighed into a 10 mL measuring flask and filled to the mark with acetone. (concentration of a.i. = 1135.9 mg/L).

Further dilutions were prepared: 2017/0486-1-Dil 1:

0.095 mL of stock solution 2017/0486-1 were pipetted into a 10 mL flask and filled to the mark with dilution medium (concentration of pendimethalin = 10.79 mg/L)

Sample Preparation

Samples were analysed after dilution with factor 50 and 1923 with dilution medium.

HPLC Conditions:

| HPLC | Shimadzu LC-20 HPLC system equipped with a diode-array detector |
| --- | --- |
| Column | Macherey Nagel Nucleoshell RP18, 2.0 mm x 100 mm, 2.7 µm |
| Mobile phase | A: Water with 0.1% (v/v) phosphoric acid B: Acetonitrile with 0.1% (v/v) phosphoric acid |
| Gradient | Time (min); Mobil phase A (%); Mobile phase B (%); Flow (mL/min). 0.00; 40; 60; 0.4. 5.00; 5; 95; 0.4. 6.00; 5; 95; 0.4. 6.01; 40; 60; 0.4. 8.00; Stop; -; -. |
| Wave length | 238 nm |
| Retention time | About 3.3 minutes |

- Results and discussions

Table A 15:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Honeybee | Pendimethalin | 174.7 | 99 | 2.3 | - |
|  |  | 14319 | 100 | 0.5 | - |

Table A 16:	Characteristics for the analytical method used for validation of pendimethalin in solution

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity of the method was assured by the following method: UV spectra from 200 to 300 nm were continuously recorded by the diode-array detector. Spectra of the peaks were compared to those of the reference. Similar spectra with approximately equal absorption maxima, a constant chromatographic retention time and no interfering peaks were observed. LOQ - blank values did not exceed 30% of the lowest validated concentration. Therefore, the specificity of the method was demonstrated. |
| Calibration (type, number of data points) | 5 points Linear calibration y = 111740x + 1456.55 R= 0.99999 |
| Calibration range | 2.158 – 10.79 mg/L of a.i. at a concentration range of approximately 62% of the lowest validation measuring concentration to approximately 145% of the highest validation measuring concentration. |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | 174.7 mg/L of pendimethalin, regarding DF 3.494 mg/L; corresponds to the lowest validated fortification level. |

- Conclusion

The analytical method fulfils the requirements of SANCO/3029/99 rev. 5 and is suitable for the determination of pendimethalin in solution with an LOQ of 174.7 mg/L.

        - Analytical method 9 and 10

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.1.2-11 |
| --- | --- |
| Report | Effect of Pendimethalin 455 g/l CS on seedling emergence and seedling growth of terrestrial plants, Radha S., 2021 Report no. 9419/2021 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

| Reference: | KCP 5.1.2-12 |
| --- | --- |
| Report | Effect of Pendimethalin 455 g/1 CS on vegetative vigour of terrestrial plants, Radha S., 2021 Report no. 9420/2021 |
| Guideline(s): | SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

The analytical method for pendimethalin determination was validated in deionized water. The limit of quantification (LOQ) of the method was 0.1 mg/L. The quantitative measurements of pendimethalin were performed by GC-MS/MS with external calibration.

Reference substance: Pendimethalin: batch no. G998095, Purity: 97.34%, Dr Ehrenstorfer

Preparation of standard stock solution:

An amount of 10.28 mg Pendimethalin reference standard (97.34%) was weighed into a 10 mL flask and was made up to the mark with acetonitrile. This solution was equivalent to 1000.66 mg/L. From this, an aliquot of 2.5 ml solution was diluted in 25 mL volumetric flask of acetonitrile. This solution was equivalent to 100 mg/L.

Preparation of calibration solution

From the specificity standard stock solution (100 mg/L), the serial dilutions were made by using acetonitrile, to prepare further concentrations 0.05, 0.1, 0.5, 1, 5, 10 and 15 mg/L. Since there is no effect in the matrix (control and solvent-acetonitrile): hence the linearity was performed in the solvent.

Preparation of sample solution

The matrix sample (deionized water, for matrix effect and specificity) or sample solution with c=1 mg/L (accuracy and precision) was placed into a 100 mL round bottom (RB) flask and the content was evaporated near to dryness. The resultant residue was dissolved in ethyl acetate and the organic part was again dried near to dryness. The resulting residue was dissolved with acetonitrile, the extract was filtered through 0.25µ nylon filter and injected into GC for quantification.

GC-MS Conditions:

| GC | Shimadzu, Model GC-FID-2010 |
| --- | --- |
| Column: | SH-Rxi-5ms (30 mm x 0.25 mm) |
| Injection temperature: | 160 °C |
| Column Oven Temperature | 350°C |
| Solvent | Acetonitrile |
| Injection volume | 1 µl |
| Flow rate | 19.8 ml/min |
| Carrier gas | Nitrogen |
| Retention time | approximately 7.32 minutes |
| Run Time | 10 minutes |
| Name of the Instrument | Gas Chromatography-Mass Spectroscopy (GC-MS) |
| Detector | Mass |
| Detector Type | Quadrupole |
| Analysis Mode | GC-MS |
| Injection Temperature | 250°C |
| Ionization Type | Electron Ionization (EI) |
| Ion Source Temperature | 300°C |
| Interface Temperature | 250°C |
| GC-MS monitoring fragment ions used 252, 266 and 281 m/z |  |

- Results and discussions

Table A 17:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Deionized water | Pendimethalin | 0.1 | 100.94 | 0.46 | - |
|  |  | 1.0 | 101.23 | 0.23 | - |

Table A 18:	Characteristics for the analytical method used for validation of pendimethalin in deionized water

|  | Pendimethalin |
| --- | --- |
| Specificity | The specificity of GC method for pendimethalin was determined by injecting the standard, deionized water matrix and test media deionized water sample solutions along with solvent blank/diluent (acetonitrile) and observed that there was no interference found with the main peak of interest. Hence, this method was considered to be specific for the analysis of the test substance. |
| Calibration (type, number of data points) | 7 points Linear calibration y = 8197.6x + 369.29 R= 0.9999 |
| Calibration range | 0.05 mg/L to 15 mg/L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.1 mg/kg mg/L; corresponds to the lowest validated fortification level |

- Conclusion

The analytical method fulfils the requirements SANTE/2020/12830, Rev.1 and is suitable for the determination of pendimethalin in deionized water with an LOQ of 0.1 mg/L.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.2.1 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1) in potato by liquid chromatography. M. L. Greco, 2017, Report No. 16.566423.0002 |
| Guideline(s): | Yes SANCO/3029/99 rev. 4 SANCO/825/00 rev. 8.1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method for the determination of Pendimethalin (CAS: 40487-42-1) in potato.

Mobile phase A (10 mM ammonium formate buffer pH 4)

About 0.62 g of ammonium formate were accurately weighed into a 1000 ml volumetric flask and dissolved with about 500 mL of milliQ water. 0.22 mL of formic acid were added and then the solution was diluted to volume with milliQ water. pH wass 4

Mobile phase B

Methanol

Blank solution

10 mM ammonium formate buffer: acetonitrile, 50:50

Extraction mixture (5% formic acid acetonitrile)

In a 200 mL volumetric flask containing about 50 mL of acetonitrile, about 10 mL of acid formic were introduced and then diluted to volume with acetonitrile.

Sample extraction

About 5 g of grinded potato were weighed into a 50 mL plastic falcon and 7.5 mL of milliQ water were added in order to hydrate the matrix. Then, 10 mL of extraction mixture were added to the sample. After vortexing for about 1 min, about 6 g of magnesium sulfate anhydrous and about 1.5 g of sodium acetate were added to the sample and vortexed again for about 1 min. The tube was centrifuged at 4750 rpm for 5 min and proceed to purification

5 mL of supernatant were transferred into a 10 mL plastic tube, containing about 450 mg of magnesium sulphate anhydrous and 150 mg of PSA resin. Vortexed for about 1 min and centrifuged at 4750 rpm for 5 min. The supernatants were recovered and transferred into an HPLC vial and injected. Test sample was prepared in triplicate.

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281,31 g/mol

Molecular Formula:				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17B074018 purchased from VWR;

• Acetonitrile, batch STBG5330V purchased from VWR;

• Magnesium sulfate anhydrous ID: 226.10 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.5 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 purchased from Sigma Aldrich

• PSA Resin 40 μm, ID: 306.6 purchased from Varian

• Formic acid, batch BCBR6503V, Fluka

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-284-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin. Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.003 mg/kg) to about 30xLOQ (0.3 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.003 mg/kg), and 10xLOQ (about 0.3 mg/kg). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.01 mg/kg.

Results and discussions

Table A 19:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n =5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Potato | Pendimethalin | 0.01 | 100.1 | 3 | First mass transition |
|  |  | 0.1 | 97.5 | 2 |  |
|  |  | 0.01 | 96.3 | 3 | Second mass transition |
|  |  | 0.1 | 96.2 | 3 |  |

Table A 20:	Characteristics for the analytical method used for validation of pendimethalin residues in potato

|  | Pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transitions. The method is specific. |
| Calibration (type, number of data points) | 5 points 0.002 to 0.4 mg/kg First mass transition y=18900395x R2=0.9959 Second mass transition y=2120709x R2=0.9964 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830 the method was successfully validated and is suitable for determination of residues of pendimethalin in potato.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.2.2 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Pendimethalin (CAS: 40487-42-1) in oranges by liquid chromatography. M. L. Greco, 2017, Report No. 16.566423.0003 |
| Guideline(s): | Yes SANCO/3029/99 rev. 4 SANCO/825/00 rev. 8.1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method for the determination of Pendimethalin (CAS: 40487-42-1) in oranges.

Mobile phase A (10 mM ammonium formate buffer pH 4)

About 0.62 g of ammonium formate were accurately weighed into a 1000 ml volumetric flask and dissolved with about 500 mL of milliQ water. 0.22 mL of formic acid were added and then the solution was diluted to volume with milliQ water. pH was 4

Mobile phase B

Methanol

Blank solution

10 mM ammonium formate buffer: acetonitrile, 50:50

Extraction mixture (5% formic acid acetonitrile)

In a 200 mL volumetric flask containing about 50 mL of acetonitrile, about 10 mL of acid formic were introduced and then diluted to volume with acetonitrile.

Sample extraction

About 5 g of grinded oranges were weighed into a 50 mL plastic falcon and 7.5 mL of milliQ water were added in order to hydrate the matrix. Then, 10 mL of extraction mixture were added to the sample. After vortexing for about 1 min, about 6 g of magnesium sulfate anhydrous and about 1.5 g of sodium acetate were added to the sample and vortexed again for about 1 min. The tube was centrifuged at 4750 rpm for 5 min and proceed to purification

5 mL of supernatant were transferred into a 10 mL plastic tube, containing about 450 mg of magnesium sulphate anhydrous and 150 mg of PSA resin. Vortexed for about 1 min and centrifuged at 4750 rpm for 5 min. The supernatants were recovered and transferred into an HPLC vial and injected. Test sample was prepared in triplicate.

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281,31 g/mol

Molecular Formula:				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17A054007 purchased from VWR;

• Acetonitrile, batch STBG5324V purchased from VWR;

• Magnesium sulfate anhydrous ID: 226.13 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 purchased from Sigma Aldrich

• PSA Resin 40 μm, ID: 306.6 purchased from Varian

• Formic acid, ID:961.1 from Suprapur

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-284-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin. Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.003 mg/kg) to about 30xLOQ (0.3 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.003 mg/kg), and 10xLOQ (about 0.3 mg/kg). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.01 mg/kg.

Results and discussions

Table A 21:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n =5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Orange | Pendimethalin | 0.01 | 101.9 | 2 | First mass transition |
|  |  | 0.1 | 99.5 | 2 |  |
|  |  | 0.01 | 99.1 | 1 | Second mass transition |
|  |  | 0.1 | 98.3 | 2 |  |

Table A 22:	Characteristics for the analytical method used for validation of pendimethalin residues in oranges

|  | Pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transitions. The method is specific. |
| Calibration (type, number of data points) | 5 points 0.002 to 0.398 mg/kg First mass transition y=35255318x R2=0.9999 Second mass transition y=3842521x R2=0.9999 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830 the method was successfully validated and is suitable for determination of residues of pendimethalin in oranges.

        - Analytical method 3

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.2.3 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in soy beans by liquid chromatography. Maria Laura Greco, 2017; Report number: 16.566423.0001. |
| Guideline(s): | Yes (SANCO 3029/99 Rev.4 SANCO/825/00 Rev 8.1 and OECD-204/2014) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281,31 g/mol

Molecular Formula:				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17Z0666 purchased from VWR;

• Acetonitrile, batch STBG5324V purchased from VWR;

• Magnesium sulfate anhydrous ID: 226.13 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 purchased from Sigma Aldrich

• PSA Resin 40 μm, ID: 306.6 purchased from Varian

• Formic acid, ID: 961.2, Suprapur

• Pendimethalin Reference Standard Solution, ID: 3422, logbook n°1045 pag 1/20 (Conc. 496.96 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI-

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-284-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin. Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

| MRM | Transition 1 | Transition 2 |  | Transition 1 | Transition 2 |
| --- | --- | --- | --- | --- | --- |
| Analytical session: | nº1 |  | nº2 |  |  |
| determination | area | area |  | area | area |
| 1 | 72845 | 8010 |  | 70475 | 7576 |
| 2 | 64317 | 6819 |  | 61727 | 6607 |
| 3 | 62536 | 7033 |  | 62190 | 6722 |
| 4 | 71852 | 7752 |  | 68209 | 7428 |
| Average | 67888 | 7404 |  | 65650 | 7083 |
| Std. Dev. | 5128 | 568 |  | 4366 | 490 |
| RSD | 8 | 8 |  | 7 | 7 |
| % RSD ≤ | 10 | 10 |  | 10 | 10 |
| Conformity | Yes | Yes |  | Yes | Yes |

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.003 mg/kg) to about 30xLOQ (0.3 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

- Results obtained and the statement of conformity to the acceptance criteria defined in the Study plan are listed below.

| ID RS | IRS-A volume ml | IRS-B volume ml | RS final ml | RS final conc. mg/l | Conc. Vs sample | Trans 1 area | Trans 2 area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | - | 0.4 | 50 | 0.001 | 0.003 | 20717 | 2283 |
| L2 | - | 0.6 | 20 | 0.003 | 0.010 | 63785 | 6985 |
| L3 | - | 3.0 | 10 | 0.030 | 0.099 | 645963 | 68291 |
| L4 | 0.6 | - | 10 | 0.060 | 0.199 | 1297527 | 138194 |
| L5 | 1.0 | - | 10 | 0.099 | 0.331 | 2052077 | 219730 |

Linearity Parameters of transition 1: Coefficient of determination (R2) – 0.9988

Linearity Parameters of transition 2: Coefficient of determination (R2) – 0.9991

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.003 mg/kg), and 10xLOQ (about 0.3 mg/kg). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.01 mg/kg.

Table A 23:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) |  | RSD (%) |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tran. 1 | Tran. 2 | Tran. 1 | Tran. 2 |  |
| Soy bean | pendimethalin | 0.01 | 92.4 | 96.5 | 3 | 4 |  |
| Soy bean | pendimethalin | 0.10 | 88.8 | 88.0 | 2 | 2 |  |

Table A 24:	Characteristics for the analytical method used for validation of pendimethalin residues in soy bean

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 0.003 to 0.331 mg/kg First mass transition y=20985706x R2=0.9988 Second mass transition y=2242212x R2=0.9991 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

- Conclusion

- The validation data demonstrate that the analytical method SOPa-284-LABCHI-Rev.0 internally developed is suitable to qualitatively and quantitatively determine pendimethalin in soy bean specimens, according to SANTE/12830/2020 and OECD-204/2014 guidelines and for the given concentration range.

        - Analytical method 4

          - Method validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.2.4 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in wheat grains by liquid chromatography. M. L. Greco, 2017, Report No. 16.566423.0005 |
| Guideline(s): | Yes SANCO/3029/99 rev. 4 SANCO/825/00 rev. 8.1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of an analytical method for the determination of Pendimethalin (CAS: 40487-42-1) in wheat grain.

Mobile phase A (10 mM ammonium formate buffer pH 4)

About 0.62 g of ammonium formate were accurately weighed into a 1000 ml volumetric flask and dissolved with about 500 mL of milliQ water. 0.22 mL of formic acid were added and then the solution was diluted to bolume with milliQ water. pH wass 4

Mobile phase B

Methanol

Blank solution

10 mM ammonium formate buffer: acetonitrile, 50:50

Extraction mixture (5% formic acid acetonitrile)

In a 200 mL volumetric flask containing about 50 mL of acetonitrile, about 10 mL of acid formic were introduced and then diluted to volume with acetonitrile.

Sample extraction

About 5 g of wheat grains were introduced into a 50 mL plastic tube, 7.5 mL of milliQ water and 10 mL of extraction mixture were added to the sample. After vortexing for about 1 min, about 6 g of magnesium sulphate anhydrous and about 1.5 g of sodium acetate were added to the sample and vortexed again for about 1 min. The tube was centrifuged at 4750 rpm for 5 min and kept at about - 20℃ for about 2 hours. Then, centrifuge the tube at 4750 rpm for 5 min and proceed to purification of the supernatant. 5 mL of supernatant were transferred into a 10 mL plastic tube, containing about 450 mg of magnesium sulphate anhydrous and 150 mg of PSA resin. Vortexed for about 1 min and centrifuged at 4750 rpm for 5 min. The supernatants of purified sample were recovered and transferred into an HPLC vial and inject. Test sample was prepared in duplicate.

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281,31 g/mol

Molecular Formula:				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17F074033 purchased from VWR;

• Acetonitrile, batch STBG5324V purchased from VWR;

• Magnesium sulfate anhydrous ID: 226.13 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 purchased from Sigma Aldrich

• PSA Resin 40 μm, ID: 306.6 purchased from Varian

• Formic acid, ID: 961.2, Suprapur

• Pendimethalin Reference Standard Solution, ID: 3422, logbook n°1045 pag 1/20 (Conc. 496.96 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI-

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

Procedure

The analytical method, internally developed and codified as SOPa-284-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin. Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.003 mg/kg) to about 30xLOQ (0.3 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.003 mg/kg), and 10xLOQ (about 0.3 mg/kg). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.01 mg/kg.

Results and discussions

Table A 25:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n =5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat grain | Pendimethalin | 0.01 | 100.3 | 2 | First mass transition |
|  |  | 0.1 | 100.2 | 2 |  |
|  |  | 0.01 | 108.2 | 1 | Second mass transition |
|  |  | 0.1 | 97.8 | 1 |  |

Table A 26:	Characteristics for the analytical method used for validation of pendimethalin residues in wheat grain

|  | Pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transitions. The method is specific. |
| Calibration (type, number of data points) | 5 points 0.002 to 0.0.358 mg/kg First mass transition y=9948520x R2=0.9980 Second mass transition y=1071544 R2=0.9981 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830 the method was successfully validated and is suitable for determination of residues of pendimethalin in wheat grain.

          - Independent laboratory validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.2.5 |
| --- | --- |
| Report | Determination of residues of Pendimethalin applied as “Pendimethalin 330 g/L” in potato at one site in Germany, 2016. J. Kicińska, 2017, Report No. ZBBZ-2016/12/DPL/4DE |
| Guideline(s): | SANCO/3029/99 rev. 4 SANCO/825/00 rev. 8.1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to determine the decline and the magnitude of residues of Pendimethalin in Potato samples taken from the field trial, after applications of Pendimethalin 330 g/L, under open field conditions. To achieve the objective appropriate analytical method for determination of Pendimethalin was validated in accordance to the guidance documents SANCO/825/00, rev. 8.1. and SANCO/3029/99, rev. 4 of the European Commission and to meet residue regulatory requirements. The validated limit of quantification is 0.01 mg/kg.

The general principles of the analytical procedure were based on the Final Report N. 16555423.002. In brief, samples of Potato were extracted with acidified acetonitrile. After addition of a buffer-salt mixture containing magnesium sulfate and sodium acetate the extract was shaken. After centrifugation, an aliquot of the upper acetonitrile phase was cleaned by primary secondary amine (PSA) and dehydrated by magnesium sulfate addition.

Selectivity and Confirmation of Residue Identity

Quantification was performed by use of highly selective liquid chromatography coupled with tandem mass spectrometry (LC-MS/MS). Two selected ion mass transitions were evaluated in order to demonstrate that the method achieves a high level of selectivity. The retention time of analyte in extracts correspond to that of the calibration standard with a tolerance of < ± 0.1 min. Also, confirmation ion ratios for Pendimethalin in all samples were within ± 30 % of the average found for the standards.

No significant interference above 30 % of LOQ was detected in any of the reagent blanks or control specimen extracts for Potato matrix, so that a high level of selectivity was demonstrated and an additional confirmatory method is not necessary.

Matrix Effects

Matrix effects on the detection of Pendimethalin in extracts of Potato were lesser than 20% and thus considered insignificant, according to SANCO/825/00, rev. 8.1. and SANCO/3029/99, rev. 4. Determination was performed using matrix-matched calibration standards.

Linearity

The correlation between the injected concentration of analyte standard and detector response was demonstrated to be linear by single determination of matrix-matched calibration standards at seven concentration levels ranging from 0.00025 μg/mL to 0.025 μg/mL. This range corresponds from 0.0025 mg/kg to 0.25 mg/kg for potato and thus covers the range from no more than 30 % of the LOQ and at least + 20 % of the highest analyte concentration level detected in samples.

The calibration curves obtained for both ion mass transitions of Pendimethalin were linear with the coefficients of correlation (R) greater than 0.99. Linear regression was performed with 1/x weighting.

Accuracy and precision

Accuracy was determined by fortification of control (untreated) samples with known amounts of the reference item and subsequent determination of the recoveries when applying the extraction procedure. Precision was determined by repeatability (relative standard deviation – RSD).

Five recovery determinations were performed at the LOQ (0.01 mg/kg) and at the 10 x LOQ (0.1 mg/kg), respectively. Analysis was performed by extraction and single injection.

Sample extraction

About 5 g of grinded potato were weighed into a 50 mL plastic falcon and 7.5 mL of milliQ water were added in order to hydrate the matrix. Then, 10 mL of extraction mixture were added to the sample. After vortexing for about 1 min, about 6 g of magnesium sulfate anhydrous and about 1.5 g of sodium acetate were added to the sample and vortexed again for about 1 min. The tube was centrifuged at 4750 rpm for 5 min and proceed to purification

5 mL of supernatant were transferred into a 10 mL plastic tube, containing about 450 mg of magnesium sulphate anhydrous and 150 mg of PSA resin. Vortexed for about 1 min and centrifuged at 4750 rpm for 5 min. The supernatants were recovered and transferred into an HPLC vial and injected. Test sample was prepared in triplicate.

Results and discussions

Table A 27:	Recovery results from independent laboratory validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Potato | Pendimethalin | 0.01 | 86 | 3.8 | First mass transition |
|  |  | 0.1 | 95 | 1.5 |  |
|  |  | 0.01 | 90 | 6.7 | Second mass transition |
|  |  | 0.1 | 94 | 1.3 |  |

Table A 28:	Characteristics for the analytical method used for independent laboratory validation of Pendimethalin residues in potato

|  | Potato |
| --- | --- |
| Specificity | No significant peaks (≤30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transitions. The method is specific. |
| Calibration (type, number of data points) | 7 points 0.0025 to 0.25 mg/kg First mass transition y=6656250.770882x+557.271913 R2=0.99961687 Second mass transition y=1101199.679722x+167.416310 R2=0.99883311 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.0025 mg/kg |

Conclusion

According to SANTE/2020/12830 the method was successfully validated and is suitable for determination of residues of pendimethalin in potato.

          - Independent laboratory validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.2.6 |
| --- | --- |
| Report | Determination of residues of Pendimethalin applied as “Pendimethalin 330 g/L EC” in soybean at one site in Poland, 2016. J. Kicińska, 2018, Report No. ZBBZ-2016/12/12/DPL/8PL |
| Guideline(s): | Yes SANCO/3029/99 rev 4 SANCO/825/00 rev. 8.1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to determine the residues of Pendimethalin in Soybean samples taken from the field trial, after applications of Pendimethalin 330 g/L EC, under open field conditions. To achieve the objective, appropriate analytical method for determination of Pendimethalin was validated in accordance to the guidance documents SANCO/825/00, rev. 8.1. and SANCO/3029/99, rev. 4 of the European Commission and to meet residue regulatory requirements. The validated limit of quantification is 0.01 mg/kg.

The general principles of the analytical procedure were based on the Final Report N. 16555423.0002 [3]. In brief, samples of Soybean were extracted with acidified acetonitrile. After addition of a buffer-salt mixture containing magnesium sulfate and sodium acetate the extract was shaken. After centrifugation, an aliquot of the upper acetonitrile phase was cleaned by primary secondary amine (PSA) and dehydrated by magnesium sulfate addition.

Selectivity and Confirmation of Residue Identity

Quantification was performed by use of highly selective liquid chromatography coupled with tandem mass spectrometry (LC-MS/MS). Two selected ion mass transitions were evaluated in order to demonstrate that the method achieves a high level of selectivity. The retention time of analyte in extracts correspond to that of the calibration standard with a tolerance of < ± 0.1 min. Also, confirmation ratios for Pendimethalin in all samples were within ± 30 % of the average found for the standards.

No significant interference above 30 % of LOQ was detected in any of the reagent blanks or control specimen extracts for Soybean matrix, so that a high level of selectivity was demonstrated and an additional confirmatory method is not necessary.

Matrix Effects

Matrix effects on the detection of Pendimethalin in extracts of Soybean were higher than 20% and thus considered significant, according to SANCO/825/00, rev. 8.1. and SANCO/3029/99, rev. 4. Determination was performed using matrix-matched calibration standards.

Linearity

The correlation between the injected concentration of analyte standard and detector response was demonstrated to be linear by single determination of matrix-matched calibration standards at seven concentration levels ranging from 0.00025 μg/mL to 0.025 μg/mL. This range corresponds from 0.0025 mg/kg to 0.25 mg/kg for Soybean and thus covers the range from no more than 30 % of the LOQ and at least + 20 % of the highest analyte concentration level detected in samples.

Accuracy and Precision

Accuracy was determined by fortification of control samples with known amounts of the reference item and subsequent determination of the recoveries when applying the extraction procedure. Precision was determined by repeatability (relative standard deviation – RSD).

The mean recovery values at the fortification levels of 0.01 mg/kg and 0.1 mg/kg for both ion mass transitions were all in the range 70 – 110 % and thus comply with the standard acceptance criteria of the guidance documents SANCO/825/00, rev. 8.1. and SANCO/3029/99, rev. 4. All precision values at the fortification levels of 0.01 mg/kg and 0.1 mg/kg for both ion mass transitions were < 20%.

Sample extraction

5.00 g ± 0.05 g of homogenized matrix was weighed into a 50 mL Teflon® centrifuge tube. Sample weight was recorded.

If necessary fortification of the concurrent recovery sample(s) by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken vigorously by QuEChERS Hand Motion Shaker for 1 min. and allowed to stand for about 5 min. Fortification details are below:

| Fortification level | Concentration (μg/mL) | Volume used (mL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 1 | 0.05 |
| 10 x LOQ (0.1 mg/kg) | 10 | 0.05 |

Using glass volumetric pipettes 7.5 mL of water and 10 mL of acidified acetonitrile (+5 Vol % formic acid) were added.

The Teflon® centrifuge tube was closed tightly and shaken vigorously by QuEChERS Hand Motion Shaker for 1 min.

Results and discussions

Table A 29:	Recovery results from independent laboratory validation of pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soybean | Pendimethalin | 0.01 | 74 | 5.7 | First mass transition |
|  |  | 0.1 | 73 | 11.2 |  |
|  |  | 0.01 | 72 | 6.2 | Second mass transition |
|  |  | 0.1 | 73 | 11.6 |  |

Table A 30:	Characteristics for the analytical method used for independent laboratory validation of Pendimethalin residues in soybean

|  | Pendimethalin |
| --- | --- |
| Specificity | No significant interference above 30 % of LOQ was detected in any of the reagent blanks or control specimen extracts for Soybean matrix, so that a high level of selectivity was demonstrated and an additional confirmatory method is not necessary. |
| Calibration (type, number of data points) | 7 points 0.00025 μg/mL to 0.025 μg/mL First mass transition y=12097357.433687x+673.276223 R=0.9963276476 Second mass transition y=1235894.921549x+239.521389 R=0.99604571 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.0025 mg/kg |

Conclusion

- According to SANTE/2020/12830 the method was validated and is suitable for determination of Pendimethalin residues in soybean.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Soil (KCP 5.2)

        - Analytical method 1

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.4.1 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), loamy sand soil by liquid chromatography. Maria Laura Greco, 2017; Report number: 16.566423.0008. |
| Guideline(s): | Yes (SANCO 3029/99 Rev.4 SANCO/825/00 Rev 8.1 and OECD-204/2014) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281.31 g/mol

Molecular Formula: 				C13H19N3O4

CHELAB ID:					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17G174025 purchased from VWR;

• Acetonitrile, batch STBG5699V purchased from Honeywell;

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Acetone, batch 17C294011 purchased from VWR;

• Hexane, batch 17C154010 purchased from VWR;

• Formic acid, ID: 961.1, Suprapur

• Pendimethalin Reference Standard Solution, ID: 3452, logbook n°1045 pag 11/20 (Conc. 497.95 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-291-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin Two SRM transitions were monitored

Pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

| MRM | Transition 1 | Transition 2 |  | Transition 1 | Transition 2 |
| --- | --- | --- | --- | --- | --- |
| Analytical session: | nº1 |  | nº2 |  |  |
| determination | area | area |  | area | area |
| 1 | 43207 | 4768 |  | 45644 | 4967 |
| 2 | 40610 | 4535 |  | 42474 | 4726 |
| 3 | 40334 | 4639 |  | 44084 | 5216 |
| 4 | 48971 | 5339 |  | 43790 | 4858 |
| Average | 43281 | 4820 |  | 43998 | 4942 |
| Std. Dev. | 4009 | 359 |  | 1302 | 208 |
| RSD | 9 | 7 |  | 3 | 4 |
| % RSD ≤ | 10 | 10 |  | 10 | 10 |
| Conformity | Yes | Yes |  | Yes | Yes |

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.0003 mg/kg) to about 30xLOQ (0.03 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

- Results obtained and the statement of conformity to the acceptance criteria defined in the Study plan are listed below.

| ID RS | IRS-A volume ml | IRS-B volume ml | RS final ml | RS final conc. mg/l | Conc. Vs sample | Trans 1 area | Trans 2 area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | - | 0.4 | 25 | 0.0016 | 0.0003 | 22834 | 2278 |
| L2 | - | 0.5 | 10 | 0.005 | 0.001 | 43066 | 4368 |
| L3 | 0.5 | - | 10 | 0.049 | 0.010 | 341773 | 37068 |
| L4 | 1.0 | - | 10 | 0.099 | 0.020 | 672177 | 71987 |
| L5 | 1.7 | - | 10 | 0.169 | 0.034 | 1157845 | 121123 |

Linearity Parameters of transition 1: Coefficient of determination (R2) – 0.9997

Linearity Parameters of transition 2: Coefficient of determination (R2) – 0.9996

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.001 ppm), and 10xLOQ (about 0.01 ppm). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

- All results comply with acceptance criteria defined in SANCO/3029/99 rev. 4 guidelines

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.001 mg/kg.

Table A 31:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) |  | RSD (%) |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tran. 1 | Tran. 2 | Tran. 1 | Tran. 2 |  |
| Loamy sand soil | pendimethalin | 0.001 | 103 | 104 | 2 | 3 |  |
|  |  | 0.010 | 80 | 84 | 6 | 6 |  |

Table A 32:	Characteristics for the analytical method used for validation of pendimethalin residues in soil

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 0.0016 mg/L to 0.169 mg/L First mass transition y=6820318x R=0.9997 Second mass transition y=719098x R=0.9996 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.001 mg/kg |

- Conclusion

The validation data demonstrate that the analytical method SOPa-291-LABCHI-Rev.0 internally developed is suitable to qualitatively and quantitatively determine pendimethalin in soil specimens, according to SANTE/2020/12830 and OECD-204/2014 guidelines and for the given concentration range.

        - Analytical method 2

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.4.2 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), sandy loam soil by liquid chromatography. Maria Laura Greco, 2017; Report number: 16.566423.0009. |
| Guideline(s): | Yes (SANCO 3029/99 Rev.4 SANCO/825/00 Rev 8.1 and OECD-204/2014) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch:						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281.31 g/mol

Molecular Formula: 				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17G174025 purchased from VWR;

• Acetonitrile, batch STBG5687V purchased from Honeywell;

• Magnesium sulfate anhydrous ID: 226.13 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 Sigma Aldrich

• PSA Resin 40 µm, ID: 306.6 - Varian

• Formic acid, ID: 961.1, Suprapur

• Pendimethalin Reference Standard Solution, ID: 3452, logbook n°1045 pag 11/20 (Conc. 497.95 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Grinder;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-291-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

| MRM | Transition 1 | Transition 2 |  | Transition 1 | Transition 2 |
| --- | --- | --- | --- | --- | --- |
| Analytical session: | nº1 |  | nº2 |  |  |
| determination | area | area |  | area | area |
| 1 | 13651 | 1404 |  | 15594 | 1664 |
| 2 | 13857 | 1398 |  | 15378 | 1701 |
| 3 | 13945 | 1508 |  | 15278 | 1728 |
| 4 | 15645 | 1606 |  | 15633 | 1654 |
| Average | 14275 | 1479 |  | 15471 | 1687 |
| Std. Dev. | 922 | 99 |  | 171 | 34 |
| RSD | 6 | 7 |  | 1 | 2 |
| % RSD ≤ | 10 | 10 |  | 10 | 10 |
| Conformity | Yes | Yes |  | Yes | Yes |

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0,0003 mg/kg) to about 30xLOQ (0,03 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

- Results obtained and the statement of conformity to the acceptance criteria defined in the Study plan are listed below.

| ID RS | IRS-A volume ml | IRS-B volume ml | RS final ml | RS final conc. mg/l | Conc. Vs sample | Trans 1 area | Trans 2 area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | - | 0.4 | 25 | 0.002 | 0.0003 | 5517 | 595 |
| L2 | - | 0.5 | 10 | 0.005 | 0.001 | 14100 | 1726 |
| L3 | 0.5 | - | 10 | 0.050 | 0.010 | 145995 | 15575 |
| L4 | 1.0 | - | 10 | 0.010 | 0.0199 | 294905 | 31818 |
| L5 | 1.7 | - | 10 | 0.169 | 0.0339 | 524035 | 56616 |

Linearity Parameters of transition 1: Coefficient of determination (R2) – 0.9991

Linearity Parameters of transition 2: Coefficient of determination (R2) – 0.9989

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.001 ppm), and 10xLOQ (about 0.01 ppm). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

- All results comply with acceptance criteria defined in SANCO/3029/99 rev. 4 guidelines

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.001 mg/kg.

Table A 33:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) |  | RSD (%) |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tran. 1 | Tran. 2 | Tran. 1 | Tran. 2 |  |
| Sandy loam | pendimethalin | 0.001 | 102 | 106 | 3 | 4 |  |
|  |  | 0.010 | 103 | 104 | 1 | 1 |  |

Table A 34:	Characteristics for the analytical method used for validation of pendimethalin residues in soil

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 0.0002 mg/L to 0.0169 mg/L First mass transition y=30528836x R=0.9991 Second mass transition y=3295107x R=0.9989 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.001 mg/kg |

- Conclusion
The validation data demonstrate that the analytical method SOPa-292-LABCHI-Rev.0 internally developed is suitable to qualitatively and quantitatively determine pendimethalin in soil specimens, according to SANTE/2020/12830 and OECD-204/2014 guidelines and for the given concentration range.

      - Description of Methods for the Analysis of Water (KCP 5.2)

        - Analytical method 1

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.5.1 |
| --- | --- |
| Report | Method Validation: Determination of Residues of Pendimethalin in Drinking Water Samples. G Andrews, 2015 Report number: YV/13/034 |
| Guideline(s): | Yes (SANCO/825/00 Rev 8.1) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Common name: 					Pendimethalin

Chemical name (IUPAC): 				N-(1-ethylpropyl)-2,6-dinitro-3,4-xylidine

CAS Number: 						40487-42-1

Molecular structure:					C13H19N3O4

Molecular weight: 					281.3

Source: 						Sigma Aldrich

Batch number: 						SZB8287XV

Expiry date: 						13th October 2015

Purity: 							98.8%

Storage conditions: 					Ambient

Principle of the Method and Extraction

The analytical method for the determination of pendimethalin in water samples consisted of a liquid liquid partition with dichloromethane (DCM) followed by evaporation of the organic layer. The samples were then reconstituted with 1mL of toluene before analysis by GC-MS. Calibration solutions for the determination of pendimethalin in water were prepared in matrix.

GC-MS Analysis

Method 1

Samples were analysed by Gas chromatography coupled with mass spectrometer (GC-MS).

Column: 			Agilent HP-5MS, 30 m x 0.25 mm (i.d.) x 0.25 μm (film thickness)

Injection: 			2 μL, pulsed splitless (pulse pressure 50 psi for 0.5 minutes)

Temperature: 			250°C.

Carrier: 			Helium, Constant Flow Mode, 1.0 mL/min

Oven: 				Initial temperature: 90°C, Hold time: 1 min

1. program rate: 65°C/min until 300°C, hold for 1.5 min

Mode of Ionisation: 		EI

Transfer Line: 			250°C

MS Source: 			230°C

MS Quad: 			150°C

Detector: 			MSD SIM mode, Mass monitored: m/z 252

Electron multiplier voltage: about 1600 V

Under these chromatographic conditions, the retention time for Pendimethalin was found to

be about 4.9 minutes.

Method 2

Samples were analysed by Gas chromatography coupled with mass spectrometer (GC-MS).

Column: 			Agilent Rxi-624 Sil MS, 30 m x 0.25 mm (i.d.) x 1.4 μm (film

thickness)

Injection: 			2 μL, pulsed splitless (pulse pressure 50 psi for 0.5 minutes)

Temperature: 			250°C.

Carrier: 			Helium, Constant Flow Mode, 1.0 mL/min

Oven: 				Initial temperature: 90°C, Hold time: 1.0 min

1. program rate: 65°C/min until 300°C, hold for 3.0 min

Mode of Ionisation: 		EI

Transfer Line: 			250°C

MS Source: 			250°C

MS Quad: 			150°C

Detector: 			MSD SIM mode, Mass monitored: m/z 252

Electron multiplier voltage: about 1600 V

Under these chromatographic conditions, the retention time for Pendimethalin was found to

be about 5.4 minutes.

Summary od method validation

The objective of this study was to validate an analytical method for the determination of residues of pendimethalin in drinking water to fulfil the requirements according to Regulations (EU) 544/2011 and 545/2011 implementing Regulation (EC) 1107/2009, EU Guidance Document SANCO/825/00 rev. 8.1, 16/11/10 and Directive 98/8/EC and Regulation (EU) 528/2012.

The analytical method for the determination of pendimethalin in water samples consisted of a liquid liquid partition with dichloromethane (DCM) followed by evaporation of the organic layer. The samples were then reconstituted with 1mL of toluene before analysis by GC-MS. Calibration solutions for the determination of pendimethalin in water were prepared in matrix. One ion was used for quantification on column HP-5MS (30m x 0.25mm x 0.25μm) and the same ion was used for confirmation on a Rxi-624 Sil (30m x 0.25mm x 1.4 μm) column. The analytical method was successfully validated for use in terms of linearity, specificity, accuracy and precision. The analytical method was shown to be linear for pendimethalin over the concentration range 3 ng/mL to 120 ng/mL. The regression coefficient (r2) was determined to be greater than 0.98. The limit of quantification (LOQ) was established at 0.1 μg/L for pendimethalin in drinking water.

No interference/contamination peak above 30% of the LOQ was detected at the retention time of pendimethalin in any control sample.

The average recoveries were found to lie within the required range of 70-110% at all levels of fortification and the relative standard deviation was less than 20% which demonstrates acceptable accuracy and precision of the method.

In conclusion, the method was successfully validated for the determination of residues of pendimethalin in drinking water with an LOQ of 0.1 μg/L.

Table A 35:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (µg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Drinking water | Pendimethalin | 0.1 | 86 | 16.4 | Method 1 |
|  |  | 1.0 | 95 | 13.5 | Method 1 |
|  |  | 0.1 | 83 | 8.4 | Method 2 |
|  |  | 1.0 | 98 | 11.9 | Method 2 |

Table A 36:	Characteristics for the analytical method used for validation of Pendimethalin residues in drinking water

|  | pendimethalin |
| --- | --- |
| Specificity | No interference/contamination peak above 30% of the LOQ was detected at the retention time of pendimethalin in any control sample. |
| Calibration range | Mehod 1, low range: 5 points 3.2 ng/mL to 63.5 ng/mL y=23.471x+3.129 R=0.9919 Mehod 1, high range: 5 points 42.4 ng/mL to 127 ng/mL y=25.534x-66.186 R=0.9957 Mehod 2, low range: 5 points 3.2 ng/mL to 63.5 ng/mL y=32.54x+34.454 R=0.9954 Mehod 2, high range: 5 points 42.4 ng/mL to 127 ng/mL y=34.884x-12.97 R=0.9934 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.1 µg/L |

- Conclusion
The validation data demonstrate that the analytical method internally developed is suitable to qualitatively and quantitatively determine pendimethalin in drinking water, according to SANTE/2020/12830 guideline and for the given concentration range.

          - Independent laboratory validation

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.5.2 |
| --- | --- |
| Report | Independent Laboratory Validation (ILV) of Analytical Method for the Determination of Residues of Pendimethalin in Drinking Water. Kevin McInerney, 2016 Report number: 100041492B |
| Guideline(s): | Yes (SANCO/825/00 Rev 8.1) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Substance name:			Pendimethalin

IUPAC Name: 				N‐(1‐ethylpropyl)‐2,6‐dinitro‐3,4‐xylidene

CAS No.: 				40487‐42‐1

Empirical formula: 			C13H19N3O4

Molecular weight: 			281.31 g/mol

Batch No.: 				SZBD302XV

Expiry Date: 				29 October 2018

Purity: 					98.8%

Laboratory Equipment

Balances:

•  Mettler MX5 balance, SN 1126452889 (used for preparation of the stock solution)

Extraction/Concentration:

•  250 ml separatory funnels (Labglass)

•  Zymark TurboVap 2, SN TV9233N6327

•  Pipette B1100300B

All reusable glassware was cleaned in a laboratory dishwasher, solvent rinsed, and air‐dried before use.

Consumable glassware (injection vials, glass pipettes) was baked at 400°C for at least 30 minutes before

use.

GC‐MS System

•  Agilent 6890 Gas Chromatograph

•  Agilent 5973N Mass Spectrometer

•  Agilent 7683 Autosampler

•  Agilent 7683 Injector

Solvents and Chemicals

•  Milli‐Q Water (Battelle‐Norwell).

•  Dichloromethane (Battelle reagent 151001‐05)

•  Toluene (Battelle reagent 150225‐03)

•  Drinking Water (Battelle reagent 151021‐12)

•  Sodium Sulfate

•  Acetonitrile (Battelle reagent 150901‐04)

Extraction

1. Placed 100mL of Water (drinking) into a 250mL separatory funnel. Recovery control specimens were fortified with the required volume of the appropriate fortification solution.

a. Samples spiked at the LOQ were fortified with 1000 μL of the 10.00 ng/mL fortification solution (Battelle ID: IQ71)

b. Samples spiked at 10x the LOQ were fortified with 1000 μL of the 100.0 ng/mL fortification solution (Battelle ID: IQ72)

2. 30 mL of dichloromethane was added to each sample.

3. Each sample was shaken vigorously for 1 minute evacuating gas.

4. Allowed layers to separate.

5. The lower layer was decanted into a concentrator tube.

6. Steps 2 through 5 were repeated twice with 20mL of dichloromethane.

7. The extracts were combined in one concentrator tube.

8. The extracts were evaporated on a turbovap at 38°C just to dryness.

9. The extracts were reconstituted at 1 mL with Toluene.

Method principle

Pendimethalin was extracted from drinking water with dichloromethane. The extract was evaporated to dryness and reconstituted with Toluene. Final determination of the analyte was achieved by GC‐MS in SIM mode, monitoring m/z 252 for quantitation.

The project team was able to confirm that the GC‐MS method afforded detection of the analyte at concentrations of 0.01 μg/L with a 2 μL injection, providing sufficient sensitivity to quantify residues of the analyte in the final extracts.

Matrix matched instrument calibrations were generated using concentrations of analyte in toluene, ranging from 3.00 to 120.0 ng/mL. Correlation coefficients (r) were ≥ 0.99. Quantitative determination was carried out by external standardization using matrix‐matched calibration solutions. As validated by this ILV, the method allows the determination of the analyte with a limit of quantification (LOQ) of 0.1 μg/L in drinking water. The limit of detection (LOD) of the method was set to 30 % of LOQ (i.e. to 0.003 μg/L).

For the independent laboratory validation, drinking water was fortified with pendimethalin at the LOQ (n=5, 0.1 μg/L) and at 10x the LOQ (n=5, 1.0 μg/L). Two additional samples of each matrix were not fortified and treated as blank controls.

Residues in all blank control specimens and reagent blanks were below 30 % of the LOQ.

The average recoveries for the pendimethalin detected were within the acceptable range of 70 % to 120% with relative standard deviations (RSD) ≤20%.

Table A 37:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level (µg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Drinking water | Pendimethalin | 0.1 | 83 | 8.2 | Method 1 |
|  |  | 1.0 | 84 | 9.1 | Method 1 |
|  |  | 0.1 | 81 | 6.2 | Method 2 |
|  |  | 1.0 | 82 | 7.3 | Method 2 |

Table A 38:	Characteristics for the analytical method used for validation of pendimethalin residues in drinking water

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 3.00 ng/mL to 120.0 ng/mL Y = 8.35e+001x – 4.35e+002 R2 = 0.996 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.1 µg/L LOD = 0.003 µg/L |

- Conclusion
The validation data demonstrate that the analytical method internally developed is suitable to qualitatively and quantitatively determine pendimethalin in drinking water, according to SANTE/2020/12830 guideline and for the given concentration range.

        - Analytical method 2

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.5.5 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Pendimethalin (CAS: 40487-42-1), in surface water by liquid chromatography. Maria Laura Greco, 2017, Report No. 16.566423.0010 |
| Guideline(s): | SANCO/3029/99 Rev. 4 SANCO/825/00 Rev. 8.1 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Short term study for the validation of analytical method, for the determination of Pendimethalin in surface water.

Preparation of solutions

Mobile phase A (10 mM ammonium formate buffer pH 4)

About 0.62 g of ammonium formate were accurately weighed into a 1000 mL volumetric flask and dissolved with about 500 mL of milliQ water. 0.22 mL of formic acid were added and then the solution was diluted to volume with milliQ water. pH was 4

Mobile phase B

Methanol

Blank solution:

Acetonitrile

Sample extraction

About 50 g of sample were exactly weighed into a 50 mL plastic tube. The solution was transferred into a 100 mL round bottomed flask by the help of a separating funnel and 8 mL of dichloromethane were added to the sample. The organic phase was recovered into a 20 mL glass tube. 0.2 mL of 99% formic acid were added to the aqueous phase and after shaking also 8 mL of diethyl ether were added. The aqueous phase was then recovered into a 100 mL beaker and added of about 1 g of Na2SO4 anhydrous. Then, the aqueous phase was added again to the organic phase and discarded by liquid phase separation by the help of a separating funnel. The organic phase was recovered in the glass tube together with the dichloromethane solution and dried by nitrogen flux bath. The dried sample was resuspended into 2 mL of blank solution, transferred into an HPLC vial and injected. Test sample was prepared in triplicate.

Reference Substance: 				Pendimethalin

IUPAC:					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281.31 g/mol

Molecular Formula:				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17G174025 purchased from VWR;

• Acetonitrile, batch 17F131395 purchased from VWR;

• Formic acid 99% v/v (LC-MS grade), batch 0B702170B purchased from Carlo Erba

• Sodium Sulfate anhydrous, ID: 874.1 purchased from Merck

• Diethyl Ether, batch S2BF1420V purchased from Fluka

• Dichloromethane, batch S2BG232OH purchased from Fluka

• Ammonium formate (LC-MS grade), ID: 237.5 purchased from Sigma Aldrich

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Vortex;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-284-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin. Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.000006 mg/kg) to about 30xLOQ (0.0006 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.00002 mg/kg), and 10xLOQ (about 0.0002 mg/kg). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.02 µg/L.

Results and discussions

Table A 39:	Recovery results from method validation of Pendimethalin using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | Pendimethalin | 0.00002 | 87.7 | 1.0 | First mass transition |
|  |  | 0.0002 | 87.9 | 1.0 |  |
|  |  | 0.00002 | 88.5 | 4.0 | Second mass transition |
|  |  | 0.0002 | 88.2 | 2.0 |  |

Table A 40:	Characteristics for the analytical method used for validation of Pendimethalin residues in Surface water

|  | Pendimethalin |
| --- | --- |
| Specificity | No significant peaks are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. The method is specific |
| Calibration (type, number of data points) | 5 points 0.000006 to 0.00072 mg/kg First mass transition y=34234760x R2=0.9998 Second mass transition y=3868167x R2=0.9997 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.00002 mg/kg LOD = 0.000006 mg/kg |

Conclusion

According to SANTE/2020/12830 the method was validated and is suitable for determination of residues of Pendimethalin in surface water.

      - Description of Methods for the Analysis of Air (KCP 5.2)

        - Analytical method 1

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.6.1 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1) in air by liquid chromatography. Maria Laura Greco, 2017; Report number: 16.566423.0012. |
| Guideline(s): | Yes (SANCO 3029/99 Rev.4 SANCO/825/00 Rev 8.1 and OECD-204/2014) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Reference Substance: 				Pendimethalin

IUPAC: 					3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 						SZBD302XV

CAS nr.:					40487-42-1

Origin: 						Sharda Cropchem Limited

Purity: 						98.8%

Molecular Weight: 				281.31 g/mol

Molecular Formula: 				C13H19N3O4

CHELAB ID: 					RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17B074018 purchased from VWR;

• Acetonitrile, batch STBG5330V purchased from Honeywell;

• Acetone, batch 16K234011 – VWR

• Hexane, batch STBG4421 - VWR

• Ammonium formate (LC-MS grade), ID: 237.5 purchased from Sigma Aldrich

• Formic acid, batch BCBR6503V - Fluka

• Pendimethalin Reference Standard Solution, ID: 3452, logbook n°1045 pag 11/20 (Conc. 497.95 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Raw Polyurethane Foam (PUF) plug, 22mm ODx 6.6 cm length;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Climatic chamber SRA 323;

• Thermostatic bath equipped with N2 flow, SRA 66;

• Sampler air;

• Ultrasonic bath, SRA 469

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI-

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-300-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

| MRM | Transition 1 | Transition 2 |  | Transition 1 | Transition 2 |
| --- | --- | --- | --- | --- | --- |
| Analytical session: | nº1 |  | nº2 |  |  |
| determination | area | area |  | area | area |
| 1 | 45171 | 51141 |  | 451400 | 51759 |
| 2 | 456907 | 51495 |  | 458337 | 51352 |
| 3 | 450941 | 50944 |  | 457126 | 52313 |
| 4 | 470504 | 53098 |  | 503188 | 56087 |
| Average | 458131 | 51670 |  | 467513 | 52878 |
| Std. Dev. | 8602 | 979 |  | 23975 | 2175 |
| RSD | 2 | 2 |  | 5 | 4 |
| % RSD ≤ | 10 | 10 |  | 10 | 10 |
| Conformity | Yes | Yes |  | Yes | Yes |

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.0012 ng/ml) to about 30xLOQ (0.12 ng/ml) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

- Results obtained and the statement of conformity to the acceptance criteria defined in the Study plan are listed below.

| ID RS | IRS-A volume ml | IRS-B volume ml | RS final ml | RS final conc. mg/l | Conc. Vs sample g/m3 | Trans 1 area | Trans 2 area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | - | 0.2 | 100 | 0.002 | 0.0000009 | 113831 | 12905 |
| L2 | - | 0.9 | 100 | 0.009 | 0.000004 | 451683 | 50638 |
| L3 | 1.0 | - | 100 | 0.100 | 0.000046 | 4727300 | 565998 |
| L4 | 1.0 | - | 50 | 0.199 | 0.000092 | 7797545 | 910245 |
| L5 | 0.7 | - | 25 | 0.279 | 0.000129 | 10600585 | 1306650 |

Linearity Parameters of transition 1: Coefficient of determination (R2) – 0.9904

Linearity Parameters of transition 2: Coefficient of determination (R2) – 0.9917

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.000004 g/m3), and 10xLOQ (about 0.00004 g/m3). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 4 ng/l.

Table A 41:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level ng/l) (n = 5) | Mean recovery (%) |  | RSD (%) |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tran. 1 | Tran. 2 | Tran. 1 | Tran. 2 |  |
| air | pendimethalin | 4.0 | 100.4 | 93.5 | 9 | 8 |  |
|  |  | 40 | 103.1 | 97.4 | 3 | 4 |  |

Table A 42:	Characteristics for the analytical method used for validation of pendimethalin residues in air

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 0.002 to 0.279 mg/L First mass transition y=39111796x R2=0.9904 Second mass transition y=4728027x R2=0.9917 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 4 ng/L |

- Conclusion 
The validation data demonstrate that the analytical method SOPa-300-LABCHI-Rev.0 internally developed is suitable to qualitatively and quantitatively determine pendimethalin in soil specimens, according to SANTE/2020/12830 and OECD-204/2014 guidelines and for the given concentration range.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

        - Analytical method 1

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.7.1 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in blood by liquid chromatography. Maria Laura Greco, 2017; Report number: 16.566423.0007. |
| Guideline(s): | Yes (SANCO 3029/99 Rev.4 SANCO/825/00 Rev 8.1 and OECD-204/2014) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Reference Substance: 					Pendimethalin

IUPAC	 						3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 							SZBD302XV

CAS nr.:						40487-42-1

Origin: 							Sharda Cropchem Limited

Purity: 							98,8%

Molecular Weight: 					281,31 g/mol

Molecular Formula: 					C13H19N3O4

CHELAB ID: 						RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17F074033 purchased from VWR;

• Acetonitrile, batch STBG5324V purchased from Honeywell;

• Magnesium sulfate anhydrous ID: 226.13 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 Sigma Aldrich

• PSA Resin 40 µm, ID: 306.6 - Varian

• Formic acid, ID: 961.1, Suprapur

• Pendimethalin Reference Standard Solution, ID: 3422, logbook n°1045 pag 1/20 (Conc. 496.95 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Grinder;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI-

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- • Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-290-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

| MRM | Transition 1 | Transition 2 |  | Transition 1 | Transition 2 |
| --- | --- | --- | --- | --- | --- |
| Analytical session: | nº1 |  | nº2 |  |  |
| determination | area | area |  | area | area |
| 1 | 429693 | 44406 |  | 432167 | 45411 |
| 2 | 407515 | 42262 |  | 414475 | 43688 |
| 3 | 401337 | 42091 |  | 412903 | 43007 |
| 4 | 447054 | 47757 |  | 453962 | 47245 |
| Average | 421400 | 44129 |  | 428377 | 44838 |
| Std. Dev. | 20994 | 2639 |  | 19163 | 1897 |
| RSD | 5 | 6 |  | 4 | 4 |
| % RSD ≤ | 10 | 10 |  | 10 | 10 |
| Conformity | Yes | Yes |  | Yes | Yes |

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0,015 mg/l) to about 30xLOQ (1.5 mg/l) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

- Results obtained and the statement of conformity to the acceptance criteria defined in the Study plan are listed below.

| ID RS | IRS-A volume ml | IRS-B volume ml | RS final ml | RS final conc. mg/l | Conc. Vs sample | Trans 1 area | Trans 2 area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | 0.4 | - | 10 | 0.004 | 0.013 | 118417 | 12481 |
| L2 | 1.5 | - | 10 | 0.015 | 0.050 | 404443 | 42116 |
| L3 | - | 1.5 | 10 | 0.149 | 0.497 | 4228624 | 447504 |
| L4 | - | 3.2 | 10 | 0.318 | 1.060 | 8100523 | 837765 |
| L5 | - | 4.8 | 10 | 0.477 | 1.590 | 11979157 | 1344107 |

Linearity Parameters of transition 1: Coefficient of determination (R2) – 0.9979

Linearity Parameters of transition 2: Coefficient of determination (R2) – 0.9972

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.015 mg/l), and 10xLOQ (about 1.5 mg/l). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0,05 mg/L.

Table A 43:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) |  | RSD (%) |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tran. 1 | Tran. 2 | Tran. 1 | Tran. 2 |  |
| blood | pendimethalin | 0.05 | 98.6 | 93.2 | 1 | 1 |  |
|  |  | 0.50 | 95.1 | 90.6 | 2 | 2 |  |

Table A 44:	Characteristics for the analytical method used for validation of pendimethalin residues in blood

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 0.004 to 0.477 mg/L First mass transition y=25420063x R2=0.9979 Second mass transition y=2776211x R2=0.9972 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.05 mg/L |

- Conclusion 
The validation data demonstrate that the analytical method SOPa-290-LABCHI-Rev.0 internally developed is suitable to qualitatively and quantitatively determine pendimethalin in soil specimens, according to SANTE/2020/12830 and OECD-204/2014 guidelines and for the given concentration range.

        - Analytical method 2

| Comments of zRMS: | Study is accepted |
| --- | --- |

| Reference: | KCP 5.3.2.7.2 and Amendment |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of pendimethalin (CAS: 40487-42-1), in liver by liquid chromatography. Maria Laura Greco, 2017; Report number: 16.566423.0004. |
| Guideline(s): | Yes (SANCO 3029/99 Rev.4 SANCO/825/00 Rev 8.1 and OECD-204/2014) |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

- Materials and methods

Reference Substance: 					Pendimethalin

IUPAC 						3,4-Dimethyl-2,6-dinitro-N-pentan-3-yl-aniline

Batch: 							SZBD302XV

CAS nr.:						40487-42-1

Origin: 							Sharda Cropchem Limited

Purity: 							98.8%

Molecular Weight: 					281.31 g/mol

Molecular Formula: 					C13H19N3O4

CHELAB ID: 						RS316512

Reagents

• milliQ water, SRA 35;

• Methanol, batch 17F074033 purchased from VWR;

• Acetonitrile, batch STBG5324V purchased from Honeywell;

• Magnesium sulfate anhydrous ID: 226.13 purchased from Sigma Aldrich

• Ammonium formate (LC-MS grade), ID: 237.7 purchased from Sigma Aldrich

• Sodium acetate ID: 701.3 Sigma Aldrich

• PSA Resin 40 µm, ID: 306.6 - Varian

• Formic acid, ID: 961.1, Suprapur

• Pendimethalin Reference Standard Solution, ID: 3422, logbook n°1045 pag 1/20 (Conc. 496.95 mg/l)

Materials and Apparatus

• Common analytical glassware;

• Fridge, SRA 7;

• Technical balance (± 0,01 g), SRA 49;

• Analytical balance (± 0,01 mg), SRA 192;

• Vortex;

• Grinder;

• Centrifuge, SRA 55;

• Thermostatic bath equipped with N2 flow, SRA 66;

• MS XEVO TQS (Waters-Micromass), SRA 470;

• Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 μm (ID: LC 23);

Instrumental Conditions

• Column:		Acquity UPLC BEH C18, 50 mm x 2,1 mm x 1,7 µm (LC 23)

• Mobile Phase A:	10 mM ammonium formate buffer pH 4,0

• Mobile Phase B:	Methanol

• Flow:			0,2 ml/min

• Injection Volume: 	5 µl

• Detector:		MS XEVO TQS (Waters-Micromass), SRA 470

• Source: 		ESI-

• Source temp.:		150 ºC

• Nebulizer.:		6 bar

• Cone gas:		150 l/h

• Desolvation gas:	400 l/ha

• Run time:		13 minutes

• Run mode:		MRM (see table below)

| Precursor ion m/z |  |  | m/z | Collision energy |
| --- | --- | --- | --- | --- |
| pendimethalin | 282.15 | Quantifier ion (trans 1) | 212 | 10 |
|  |  | Qualifier ion (trans 2) | 194 | 20 |

- Elution:		Gradient

| - Time (min) | - Mobile Phase A % | - Mobile Phase B % |
| --- | --- | --- |
| - 0 | - 100 | - 0 |
| - 0.5 | - 100 | - 0 |
| - 8.5 | - 0 | - 100 |
| - 11.5 | - 0 | - 100 |
| - 11.6 | - 100 | - 0 |
| - 13 | - 100 | - 0 |

- Results and discussions

Procedure

The analytical method, internally developed and codified as SOPa-287-LABCHI-Rev.0 was validated in terms of specificity, linearity, repeatability, accuracy and LOQ according to SANCO 3029/99 Rev.4 and OECD-204/2014 guidelines.

The validation was performed quantifying pendimethalin Two SRM transitions were monitored

pendimethalin:

• transition 1: 282.15 m/z (parent ion) > 212 m/z (daughter ion);

• transition 2: 282.15 m/z (parent ion) > 194  m/z (daughter ion).

System Suitability Test (SST)

System suitability test (SST) was performed in order to verify the suitability of the system at the beginning of each analytical sequence.

- For the purpose, the Reference Solution at a concentration corresponding to about LOQ in the sample, for pendimethalin was injected in triplicate at the beginning and single at the end of sequence. %RSD of area was calculated for the first and second transition. It was verified that it is to not higher than 10%, in accordance to the acceptance criteria for SST.

| MRM | Transition 1 | Transition 2 |  | Transition 1 | Transition 2 |
| --- | --- | --- | --- | --- | --- |
| Analytical session: | nº1 |  | nº2 |  |  |
| determination | area | area |  | area | area |
| 1 | 126116 | 13212 |  | 129369 | 13981 |
| 2 | 125040 | 13288 |  | 125554 | 13191 |
| 3 | 122466 | 13268 |  | 128027 | 13565 |
| Average | 124541 | 13256 |  | 127650 | 13579 |
| Std. Dev. | 1876 | 39 |  | 1935 | 395 |
| RSD | 2 | 0 |  | 2 | 3 |
| % RSD ≤ | 10 | 10 |  | 10 | 10 |
| Conformity | Yes | Yes |  | Yes | Yes |

Specificity

Blank solution, Reference solution at LOQ level, Test Solution and Spiked Test Solution (at LOQ level) were injected for specificity evaluation.

Based on the chromatograms, the method is able to determine the analyte in the presence of the sample matrix.

No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2.

Linearity

The method linearity was evaluated at 5 different levels of concentration, ranging from at least 30% LOQ (0.003 mg/kg) to about 30xLOQ (0.3 mg/kg) of analyte on the sample.

Using the experimental data of area ratio (y) and the corresponding theoretical concentrations (x, in mg/l), the slope (b), the intercept (a) of the regression lines (y = a + bx) and determination coefficient R2 were calculated.

- Results obtained and the statement of conformity to the acceptance criteria defined in the Study plan are listed below.

| ID RS | IRS-A volume ml | IRS-B volume ml | RS final ml | RS final conc. mg/l | Conc. Vs sample | Trans 1 area | Trans 2 area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | - | 0,6 | 50 | 0.01 | 0,02 | 33910 | 3936 |
| L2 | - | 1,0 | 20 | 0.05 | 0,1 | 1678970 | 13256 |
| L3 | - | 5,0 | 10 | 0.50 | 0,99 | 1299778 | 137815 |
| L4 | 1,2 | - | 10 | 1.16 | 2,39 | 3298234 | 350532 |
| L5 | 1,6 | - | 10 | 1.59 | 3,18 | 4362904 | 471392 |

Linearity Parameters of transition 1: Coefficient of determination (R2) – 0.9997

Linearity Parameters of transition 2: Coefficient of determination (R2) – 0.9995

Repeatability precision

Repeatability evaluation was performed on aliquots of sample spiked with Pendimethalin at LOQ (about 0.1 mg/kg), and 10xLOQ (about 1.0 mg/kg). 5 replicate analyses were performed for each spiking level.

- %RSD at each fortified level was calculated for both transitions.

Accuracy

The accuracy of the analytical method expresses the closeness of the consistency between the accepted true value and the value found.

%Recovery is included between 70% and 110% in all cases, in accordance with acceptance criteria. The extraction efficiency was evaluated by fortifying test system with reference item at LOQ and 10xLOQ and evaluating recovery%

Limit of Quantification (LOQ)

LOQ is the lowest concentration level where an acceptable degree of linearity, accuracy and precision is established. In this case LOQ corresponds to 0.01 mg/kg.

Table A 45:	Recovery results from method validation of pendimethalin n using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) |  | RSD (%) |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tran. 1 | Tran. 2 | Tran. 1 | Tran. 2 |  |
| liver | pendimethalin | 0.10 | 104.4 | 103.0 | 2 | 3 |  |
|  |  | 1.0 | 99.6 | 98.6 | 3 | 2 |  |

Table A 46:	Characteristics for the analytical method used for validation of pendimethalin residues in liver

|  | pendimethalin |
| --- | --- |
| Specificity | No significant peaks (≤ 30% LOQ) are detected at RT of the target analyte in the Blank and Test Solution with respect to the Spiked Test Solution for both transition 1 and 2. |
| Calibration range | 5 points 0.03 to 3.0 mg/L First mass transition y=27432263x R2=0.9997 Second mass transition y=2944247x R2=0.9995 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.1 mg/kg |

- Conclusion 
The validation data demonstrate that the analytical method SOPa-287-LABCHI-Rev.0 internally developed is suitable to qualitatively and quantitatively determine pendimethalin in soil specimens, according to SANTE/2020/12830 and OECD-204/2014 guidelines and for the given concentration range.

      - Other Studies/ Information

No new or additional studies have been submitted