FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: SAP250F Product name(s): INDOFIL Prothio 250 EC Chemical active substance: Prothioconazole, 250 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Indofil Industries (Netherlands) BV Submission date: 30/04/2021, 11/05/2022 Finalisation date: 08/2022 ; 02/2023 ; 02/2024

Version history

| When | What |
| --- | --- |
| April 2021 | V0 - Original version from applicant Indofil Industries (Netherlands) B.V. for submission to z-RMS in the frame of the PPP Authorization according to Article 33 of Regulation (EC) No 1107/2009. |
| May 2022 | Applicant’s revision (TDMs) |
| August 2022 | zRMS first evaluation |
| February 2023 | fRR after commenting period |
| January 2024 | Applicant dRR update according to Assessment of new residue study in honey |
| February 2024 | zRMS assessment of the completed Section 5 |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	6

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	9

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	9

5.2.2	Methods for the determination of residues (KCP 5.1.2)	9

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	12

5.3.1	Analysis of the plant protection product (KCP 5.2)	12

5.3.2	Description of analytical methods for the determination of residues of Prothioconazole (KCP 5.2)	12

5.3.2.1	Overview of residue definitions and levels for which compliance is required	12

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	13

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	15

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	18

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	19

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	20

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	21

5.3.2.8	Other studies/ information	21

Appendix 1	Lists of data considered in support of the evaluation	22

Appendix 2	Detailed evaluation of submitted analytical methods	28

A 2.1	Analytical methods for Prothioconazole	28

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	28

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	35

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for the active substance(s) and relevant impurities in the plant protection product.

Noticed data gaps are: no data gaps have been identified

Analytical methods proposed for the determination of a.s. Prothioconazole and relevant impurities – Prothioconazole-desthio and Toluene in the formulation INDOFIL Prothio 250 EC are acceptable and were validated according to SANCO/3030/99 rev.4 and 5.

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are:

- Method for determination prothioconazole residues in body fluids and tissues (at the re-evaluation of product)

- ILV method for determination prothioconazole residues in drinking water (at the re-evaluation of product)

New methods for determination prothioconazole residues in honey were provided by the applicant. Methods are accepted.

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Wheat (winter and spring) | Supported |
| Barley (spring and winter) | Supported |
| Oat | Supported |
| Rye | Supported |
| Triticale | Supported |
| Oilseed rape | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in plant protection product is provided as follows:

| Reference: | KCP 5.1.1/01 |
| --- | --- |
| Report | PROTHIOCONAZOLE 250 g/L EC (SAP250F) – Physical, chemical and technical properties of the plant protection product, Silva S., 2019, report No EF/297/19 – (interim report) |
| Guideline(s): | Yes, SANCO/3030/99 rev. 4 and 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Prothioconazole content in INDOFIL Prothio 250 EC is quantified using HPLC-UV. Test material is dissolved in acetonitrile, filtered and analysed without further processing. Concentration is determined against an external calibration range.

Description of the method

Preparation of standard solution:

25 ± 2.5 mg of prothioconazole reference material is weighed into a 25 mL volumetric flask. The material is then dissolved in acetonitrile. Prepare a calibration solution diluting 2.5mL of standard stock solution to a final volume of 10mL in acetonitrile.

Preparation of test item solutions:

108 ± 10 mg of the sample is weighed into a 100 mL volumetric flask, and the flask filled to the mark using acetonitrile. The flask is then placed in an ultrasonic bath, followed by filtration through a 0.20 μm disk filter.

Analysis conditions:

HPLC-UV:

| Mobile phase: | 50.0 % Acetonitrile : 50.0 % Water ultrapure with 0.25 % ammonium acetate |
| --- | --- |
| Flow: | 0.500 mL/min |
| Column: | Kinetex 5u C8; 4.6 mm x 150 mm; 4.7 µm |
| Column temperature: | 30.0 ºC |
| Detection wavelength: | 254 nm |
| Injection volume: | 2.00 µL |
| Retention time of Prothioconazole: | Around 7.0 minutes |
| Run time: | 15 minutes |

LC-MS:

| Mobile phase: | 50.0 % Acetonitrile : 50.0 % Water ultrapure with 0.05 % ortho-phosphoric acid |
| --- | --- |
| Flow: | 0.500 mL/min |
| Column: | Kinetex 5u C8; 4.6 mm x 150 mm; 4.7 µm |
| Column temperature: | 30.0 ºC |
| Detection wavelength: | 254 nm |
| Injection volume: | 2.00 µL |
| Retention time of Prothioconazole: | Around 10.6 minutes |
| Run time: | 15 minutes |
| MS ionization mode: | API-ES |
| Polarity: | Positive |
| Fragmentor: | 110 V |
| Gas temperature: | 350 ºC |
| Nebulizer pressure: | 35 psig |
| Drying gas: | 12.0 L/min |
| Scan range: | 50 m/z at 500 m/z |
| Vcap: | 3000 V |

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of active substance prothioconazole in plant protection product INDOFIL Prothio 250 EC

|  | Prothioconazole |
| --- | --- |
| Author(s), year | Silva S. 2019 |
| Principle of method | HPLC-UV/LC-MS |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | Range: 0.1439 mg/mL – 0.3399 mg/mL / 13.32 % – 31.47 % y= 7279.57.08x 7279.5708x – 15.2027 r = 0.9993 (n = 5 x 2) |
| Precision – Repeatability Mean n = 5 (%RSD) | RSD = 0.72 % At 23.15 23.06 % w/w RSDr = 1.67 % Hr = 0.43 |
| Accuracy n = 5 x 2 (% Recovery) | At 18.52 % w/w = 99.63 % ± 0.68 % At 23.15 % w/w = 99.82 % ± 0.49 % |
| Interference/ Specificity | There are no interfering peaks. Positive identification |
| Comment | The analytical method proposed for the determination of the Prothioconazole in the formulation INDOFIL Prothio 250 EC is acceptable and was validated according to SANCO/3030/99 rev.4 and 5 |

Conclusion

The analytical methods for the determination of active substance pyrimethanil in the plant protection product INDOFIL Prothio 250 EC has been described and validated according with SANCO/3030/99 rev. 4 and 5. The methods accomplish with the parameters validated:

- No peaks interfere with more than 3% to the total peak area

- The correlation coefficients are > 0.99

- The recovery values are between 98-102% and between 97-103% (SANCO/3030/99 rev. 5)

- The RSDs are < 20%

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Reference: | KCP 5.1.1/02 |
| --- | --- |
| Report | PROTIOCONAZOLE 250 g/L EC (SAP250F) – Physical, chemical and technical properties of the plant protection product, Silva S., 2019, report No EF/297/19 – (interim report) |
| Guideline(s): | Yes, SANCO/3030/99 rev. 4 and 5 |
| Deviations: | No |
| GLP: | No |
| Acceptability: | Yes |

Materials and methods

Prothioconazole-desthio and toluene content in INDOFIL Prothio 250 EC is quantified using HPLC-UV. Test material is dissolved in acetonitrile, filtered and analysed without further processing. Concentration is determined against an external calibration range.

Description of the method

Preparation of standard stock solution:

Prothioconazole-desthio: 10 ± 1.0 mg of prothioconazole-desthio (PTZ-desthio) reference material is weighed into a 10 mL volumetric flask. The material is then dissolved in acetonitrile.

Toluene: 25 ± 2.5 mg of toluene reference material is weighed into a 25 mL volumetric flask. The material is then dissolved in acetonitrile.

Calibration plot preparation:

Prepare the following calibration solutions by taking aliquots from the respective solutions and diluting with acetonitrile.

| Solution | PTZ-desthio stock solution volume (mL) | Toluene stock solution volume (mL) | Final solution volume (mL) |
| --- | --- | --- | --- |
| L1 | 0.050 | 0.050 | 5 |
| L2 | 0.125 | 0.125 | 5 |
| L3 | 0.200 | 0.200 | 5 |
| L4 | 0.250 | 0.250 | 5 |
| L5 | 0.400 | 0.400 | 5 |

Preparation of test item solutions:

2160 ± 10 mg of the sample is weighed into a 10 mL volumetric flask, and the flask filled to the mark using acetonitrile. The flask is then placed in an ultrasonic bath, followed by filtration through a 0.20 μm disk filter.

Analysis conditions:

HPLC-UV:

| Mobile phase: |  | Eluent B: Acetonitrile Eluent D: Water ultrapure with 0.05 % ortho-phosphoric acid |  |  |
| --- | --- | --- | --- | --- |
| Gradient: |  | Time (min) | Eluent B (%) | Eluent D (%) |
|  |  | 0.00 | 35.0 | 65.0 |
|  |  | 30.00 | 35.0 | 65.0 |
|  |  | 35.00 | 90.0 | 10.0 |
|  |  | 45.00 | 90.0 | 10.0 |
|  |  | 46.00 | 35.0 | 65.0 |
| Flow: |  | 0.300 mL/min |  |  |
| Column: |  | Zorbax Eclipse XDB-C8; 2.1 mm x 150 mm; 3.5 µm |  |  |
| Column temperature: |  | 30.0 ºC |  |  |
| Detection wavelength: |  | 216 nm |  |  |
| Injection volume: |  | 3.00 µL |  |  |
| Retention time | PTZ-desthio: | Around 17.3 minutes |  |  |
|  | Toluene: | Around 15.2 minutes |  |  |
| Run time: |  | 60 minutes |  |  |

LC-MS – protioconazole-desthio identification:

| MS ionization mode: | API-ES |
| --- | --- |
| Polarity: | Positive |
| Fragmentor: | 110 V |
| Gas temperature: | 350 ºC |
| Nebulizer pressure: | 35 psig |
| Drying gas: | 12.0 L/min |
| Scan range: | 50 m/z at 500 m/z |
| Vcap: | 3000 V |

GC-MS – toluene identification:

| Injector method: | Injection volume | 5.00 µL |
| --- | --- | --- |
|  | Pre-inj dwell time | 3.0 s |
|  | Post-inj dwell time | 3.0 s |
| GC method: | Oven Temperature | 50 ºC for 4.00 minutes 50.0 ºC/min until 255 ºC (maintain for 6.9 minutes) |
|  | Column: TG-1MS, 30 m x 0.320 mm x 0.25 µm, 100 % dimethyl polysiloxane |  |
|  | S/SL mode | Split |
|  | Temperature | 220 ºC |
|  | Split flow | 30.0 mL/min (constant flow) |
|  | Carrier flow | 1.500 mL/min |
| Detector method: (MS/MS) | Transfer line temperature | 280 ºC |
|  | Ion source temperature | 230 ºC |
|  | Start time | 2.8 min |
|  | Ionization mode | EI |
|  | Ion polarity | Positive |
|  | Acquisition mode | SCAN (m/z 30 – 150) |
| Retention time: | Toluene | Around 3.1 minutes |

Validation - Results and discussions

Table 5.2-2:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) INDOFIL Prothio 250 EC

|  | Prothioconazole-desthio max. content in PPP 0.075 g/L | Toluene max. content in PPP 0.125 g/L |
| --- | --- | --- |
| Author(s), year | Silva S. 2019 |  |
| Principle of method | HPLC-UV/LC-MS | HPLC-UV/GC-MS/MS |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | Range: 0.0098 mg/mL – 0.0789 mg/mL / 0.0045 % – 0.0365 % y= 25458.0287x – 170.0242 r = 0.9920 (n = 5 x 2) | Range: 0.0099 mg/mL – 0.0797 mg/mL / 0.0046 % – 0.0369 % y= 47857.8097x – 358.8733 r = 0.9952 (n = 5 x 2) |
| Precision – Repeatability Mean n = 5 (%RSD) | RSD = 0.52 % At 0.011 % w/w Horwitz RSDr: 5.28 % Hr = 0.098 | RSD = 0.38 % At 0.012 % w/w Horwitz RSDr: 5.21 % Hr = 0.073 |
| Accuracy n = 5 (% Recovery) | At 0.011 % w/w = 102.52 % ± 0.57 % At 0.023 % w/w = 97.07 % ± 1.14 % | At 0.012 % w/w = 102.97 % ± 0.41 % At 0.023 % w/w = 103.31 % ± 0.49 % |
| Interference/ Specificity | There are no interfering peaks. Mass spectra was obtained from standard and sample and confirm identity | There are no interfering peaks. Mass spectra was obtained from standard and sample and confirm identity. Furthermore, toluene mass spectra obtained in samples was compared with NIST/EPA/NIH mass spectral library |
| LOQ | 0.011 % w/w | 0.012 % w/w |
| Comment | Analytical methods proposed for the determination of relevant impurities are acceptable and validated according to SANCO/3030/99 rev.4 and 5 |  |

Conclusion

The analytical methods for the determination of impurities protioconazole-desthio and toluene in the plant protection product INDOFL Prothio 250F has been described and validated according with SANCO/3030/99 rev. 4 and 5. The methods accomplish with the parameters validated:

- No peaks interfere with more than 3% to the total peak area

- The correlation coefficients are > 0.99

- The recovery values are between 75-125%

- The RSDs are < 20%

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

There are no formulants or constituents of formulants within the preparation or formed during storage, that are of toxicological, ecotoxicological or environmental relevance. Therefore, this point is not relevant.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

There are no CIPAC available methods for the quantification of prothioconazole in emulsifiable concentrate applicable to INDOFIL Prothio 250 EC.

### Methods for the determination of residues (KCP 5.1.2)

An overview on the acceptable methods and possible data gaps for analysis of residues of prothioconazole for the generation of pre-authorization data is given in the following table. For the detailed evaluation of new / additional studies it is referred to Appendix 2.

Table 5.2-3:	Validated methods for the generation of pre-authorization data

| Component of residue definition: Prothioconazol: Prothioconazole-desthio (sum of isomers) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Grain/seed (Residues) | Primary | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2000a (00598) |
|  |  | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2000b (00598/M001) |
|  |  | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2001a (00647) |
|  | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | Confirmatory (if required) | - | - | - |
| Straw and green material (Residues) | Primary | 0.05 mg/kg | HPLC-MS/MS | Heinemann 2000a (00598) |
|  |  | 0.05 mg/kg | HPLC-MS/MS | Heinemann 2000b (00598/M001) |
|  |  | 0.05 mg/kg | HPLC-MS/MS | Heinemann 2001a (00647) |
|  | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | Confirmatory (if required) | - | - | - |
| Milk, meat, liver, kidney, fat (Residues) | Primary | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2001b (00655) |
|  |  | 0.004 mg/kg | HPLC-MS/MS | Heinemann 2001c (00655/M001) |
|  | Confirmatory (if required) | - | - | - |
| Component of residue definition: prothioconazole |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Grain/seed (Residues) | Primary | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2000a (00598) |
|  |  | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2000b (00598/M001) |
|  |  | 0.01 mg/kg | HPLC-MS/MS | Heinemann 2001a (00647) |
|  | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | Confirmatory (if required) | - | - | - |
| Straw and green material (Residues) | Primary | 0.05 mg/kg | HPLC-MS/MS | Heinemann 2000a (00598) |
|  |  | 0.05 mg/kg | HPLC-MS/MS | Heinemann 2000b (00598/M001) |
|  |  | 0.05 mg/kg | HPLC-MS/MS | Heinemann 2001a (00647) |
|  | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | Confirmatory (if required) | - | - | - |
| Water (Ecotoxicology) | Primary | Daphnia magna LOQ: 0.05 mg a.i./L Pseudokirchneriella subcapitata LOQ: 12.5 µg a.i./L after dilution by factor 2 Lemna gibba LOQ: 20 µg a.i./L | LC-MS/MS | Wydra, V., Siche, O., 2019a° Wydra, V., Siche, O., 2019b° Wydra, V., Siche, O., 2019c° |
|  | Confirmatory (if required) | - | - | - |
| Component of residue definition: 1,2,4-triazole |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Authors, year / missing / EU agreed |
| Plants, plant products,... (Residues) | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021b (KCP 5.1.2-02) |
| Component of residue definition: Triazole-alanine |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Authors, year / missing / EU agreed |
| Plants, plant products,... (Residues) | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021b (KCP 5.1.2-02) |
| Component of residue definition: Triazole-lactic acid |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Authors, year / missing / EU agreed |
| Plants, plant products,... (Residues) | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021b (KCP 5.1.2-02) |
| Component of residue definition: Triazole-acetic acid |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Authors, year / missing / EU agreed |
| Plants, plant products,... (Residues) | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021b (KCP 5.1.2-02) |

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

The applicant confirms that in its expert opinion, the methods submitted under point 5.2.1 can be applied for post-authorisation and monitoring and therefore additional methods under this point have not been submitted.

### Description of analytical methods for the determination of residues of Prothioconazole (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-4:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Prothioconazole-desthio (sum of isomers) | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Plant, high acid content |  | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Plant, high protein/high starch content (dry commodities) |  | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Plant, high oil content |  | 0.02* mg/kg | Reg. (EU) 2019/552 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.05* mg/kg | Reg. (EU) 2019/552 |
| Muscle | Prothioconazole-desthio (sum of isomers) | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Milk |  | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Eggs |  | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Fat |  | 0.01* mg/kg | Reg. (EU) 2019/552 |
| Liver, kidney (offal) |  | 0.1 mg/kg | Reg. (EU) 2019/552 |
| Soil (Ecotoxicology) | Prothioconazole and prothioconazole-desthio | 0.05 mg/kg | common limit |
| Drinking water (Human toxicology) | Prothioconazole and prothioconazole-desthio | - µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | Prothioconazole | 13 µg/L | Lowest EC50 from aquatic toxicity studies (Daphnia magna) using an AF of 100 |
|  | JAU6476-desthio | 7.3 µg/L | Lowest EyC50 from aquatic toxicity studies (Pseudokirchneriella subcapitata) using an AF of 10 |
|  | JAU6476-S-methyl | 18 µg/L | Lowest LC50 from aquatic toxicity studies (Onchorhynchus mykiss) using an AF of 100 |
|  | 1,2,4-triazole | 320 µg/L | Lowest NOEC from aquatic toxicity studies (Onchorhynchus mykiss) using an AF of 10 |
| Air | Prothioconazole | 60 µg/m3 | AOEL sys/AOEL inhal: 0.2 mg/kg bw/d |
| Tissue (meat or liver) | - | not required | not classified as T / T+ |
| Body fluids |  | not required | not classified as T / T+ |

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in plant matrices is given in the following tables. For the detailed evaluation of new / additional studies it is referred to Appendix 2.

Table 5.3-5:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: Prothioconazole-desthio |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content | Primary | 0.02 mg/kg | GC-MS | Weeren, R.D., Pelz, S. 2000 (00086/M033, DFG Method S19) |
|  | ILV | 0.02 mg/kg | GC-MS | Class 2001 (00086/M033, DFG Method S19, ILV) |
|  | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | Confirmatory (if required) | - | - | - |
| High acid content | Primary | - | - | Not required. |
|  | ILV | - | - | Not required. |
|  | Confirmatory (if required) | - | - | Not required. |
| High oil content | Primary | 0.02 mg/kg | GC-MS | Weeren, R.D., Pelz, S. 2000 (00086/M033, DFG Method S19) |
|  | ILV | - | - | Class 2001 (00086/M033, DFG Method S19, ILV) |
|  | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | ILV | Wheat straw and grain |  | Nichetti, S. 2022 |
|  | Confirmatory (if required) | - | - | - |
| High protein/high starch content (dry) | Primary | 0.05 mg/kg | GC-MS | Weeren, R.D., Pelz, S. 2000 (00086/M033, DFG Method S19) |
|  | Primary | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021a (KCP 5.1.2-01) |
|  | ILV | - | - | Class 2001 (00086/M033, DFG Method S19, ILV) |
|  | ILV | Rapeseed seeds and rapeseed whole plants |  | Nichetti, S. 2022 |
|  | Confirmatory (if required) | - | - | - |
| Difficult (if required, depends on intended use) | Primary | - | - | Not required. |
|  | ILV | - | - | Not required. |
|  | Confirmatory (if required) | - | - | Not required. |
| Component of residue definition: 1,2,4-triazole |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content/ High oil content | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021c (KCP 5.1.2-03) |
| Component of residue definition: Triazole-alanine |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content/ High oil content | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021c (KCP 5.1.2-03) |
| Component of residue definition: Triazole-lactic acid |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content/ High oil content | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021c (KCP 5.1.2-03) |
| Component of residue definition: Triazole-acetic acid |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content/ High oil content | Primary and confirmatory | 0.01 mg/kg wheat, barley, oilseed rape and processed commodities | HPLC-MS/MS | Longhi, D. 2021c (KCP 5.1.2-03) |

For any special comments or remarkable points concerning the analytical methods for the determination of residues in plant matrices, please refer to Appendix 2.

Table 5.3-6:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | Weeren, Pelz 2000 (00086/M033, DFG Method S19) |
| Not required, because: | - |

For the plant matrices the extraction method used in the metabolism study is comparable to the one validated in the Weeren/Pelz (2000) study, that has demonstrated suitability for the analysis of cereal grain and straw.  While the first one uses acetonitrile/water for extraction, the second one uses acetone/water. Acetonitrile and acetone are both polar protic solvents, with very similar polarity value (5.1 for acetone and 5.8 for acetonitrile). In conclusion, both extractions are comparable and extraction efficiency is demonstrated. No additional data is required.

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in animal matrices is given in the following tables. For the detailed evaluation of new / additional studies it is referred to Appendix 2.

Table 5.3-7:	Validated methods for food and feed of animal origin (if appropriate)

| Component of residue definition: Difenoconazole |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Matrix type | Method type |  | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Honey | Primary |  | 0.01 mg/kg | HPLC-MS/MS | Rovetto, I, 2023 (1111.4F.SAG23) |
|  | ILV |  | 0.01 mg/kg | HPLC-MS/MS | Mattioli, B, 2023 (0859-2023) |
| Component of residue definition: Prothioconazole |  |  |  |  |  |
| Matrix type | Method type |  | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Honey | Primary |  | 0.01 mg/kg | HPLC-MS/MS | Rovetto, I, 2023 (1111.4F.SAG23) |
|  | ILV |  | 0.01 mg/kg | HPLC-MS/MS | Mattioli, B, 2023 (0859-2023) |
| Component of residue definition: Triazole-derivate metabolites (TDM) |  |  |  |  |  |
| Matrix type | Method type |  | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Honey | Primary |  | 0.01 mg/kg | HPLC-MS/MS | Rovetto, I, 2023 (1111.4F.SAG23) |
|  | ILV |  | 0.01 mg/kg | HPLC-MS/MS | Mattioli, B, 2023 (0859-2023) |
| Component of residue definition: Prothioconazole-desthio |  |  |  |  |  |
| Matrix type | Method type | Method LOQ |  | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  |  | 0.004 mg/kg |  | HPLC-MS/MS | Heinemann 2001c (00655/M001) |
|  | ILV | 0.004 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Meat | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Fat | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Kidney, liver (offal) | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Honey | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Rovetto, I, 2023 (1111.4F.SAG23) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Mattioli, B, 2023 (0859-2023) |
| Component of residue definition: JAU6476-3-hydroxy-desthio |  |  |  |  |  |
| Matrix type | Method type | Method LOQ |  | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  |  | 0.004 mg/kg |  | HPLC-MS/MS | Heinemann 2001c (00655/M001) |
|  | ILV | 0.004 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Meat | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Fat | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Kidney, liver (offal) | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Component of residue definition: JAU6476-4-hydroxy-desthio |  |  |  |  |  |
| Matrix type | Method type | Method LOQ |  | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  |  | 0.004 mg/kg |  | HPLC-MS/MS | Heinemann 2001c (00655/M001) |
|  | ILV | 0.004 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Meat | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Fat | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |
| Kidney, liver (offal) | Primary | 0.01 mg/kg |  | HPLC-MS/MS | Heinemann 2001b (00655) |
|  | ILV | 0.01 mg/kg |  | HPLC-MS/MS | Dubey 2001 (ILV) |
|  | Confirmatory (if required) | - |  | - | - |

For any special comments or remarkable points concerning the analytical methods for the determination of residues in animal matrices, please refer to Appendix 2.

Table 5.3-8:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | Heinemann 2001b; Heinemann 2001c |
| Not required, because: | - |

The extraction efficiency of the residue method in animal matrices was previously demonstrated for the Annex I inclusion by Heinemann, O.; “Analytical Determination of Residues of JAU6476-3-hydroxydesthio, JAU6476-4-hydroxy-desthio, and JAU6476-desthio In/On Matrices of Animal Origin by HPLC-MS/MS”; document M-037709-01-1,using aged radioactive residues from the goat metabolism study (Weber, H., Weber, E. and Spiegel, K). In summary, the comparison of the residue analytical method of extraction for animal matrices with the extraction method used in the metabolism study demonstrated the suitability of the analytical method (extracting with an acetonitrile/water solvent system) for the determination of the relevant residue in animal matrices. No further consideration is necessary.

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in soil is given in the following tables. For the detailed evaluation of new / additional studies it is referred to Appendix 2.

Table 5.3-9:	Validated methods for soil (if appropriate)

| Component of residue definition: Prothioconazole |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | LOQ = 0.006 mg/kg | HPLC-MS/MS | Schramel, 2000 / EU agreed |
| Confirmatory | Not required. | - | - |
| Component of residue definition: JAU6476-desthio |  |  |  |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | LOQ = 0.006 mg/kg | HPLC-MS/MS | Schramel, 2000 / EU agreed |
|  | LOQ = 0.01 mg/kg | GC-MS | Steinhauer, 2001 / EU agreed |
| Confirmatory | Not required. | - | - |
| Component of residue definition: JAU6476-S-methyl* |  |  |  |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | LOQ = 0.006 mg/kg | HPLC-MS/MS | Schramel, 2000 / EU agreed |
| Confirmatory | Not required. | - | - |

*for monitoring not needed.

For any special comments or remarkable points concerning the analytical methods for soil please refer to Appendix 2.

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in surface and drinking water is given in the following tables. For the detailed valuation of new / additional studies, it is referred to Appendix 2.

Table 5.3-10:	Validated methods for water (if appropriate)

| Component of residue definition: Prothioconazole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | LOQ: 6 μg/L | HPLC-UV | Sommer, 1999 / EU agreed |
|  |  | LOQ: 0.1 μg/L | HPLC-MS/MS | Sommer, 2001b / EU agreed |
|  | ILV | - | - | - |
|  | Confirmatory | Not required. | - | - |
| Surface water | Primary | LOQ: 0.1 μg/L | HPLC-MS/MS | Sommer, 2001b / EU agreed |
|  | Confirmatory | Not required. | - | - |
| Component of residue definition: JAU6476-desthio |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | LOQ: 6 μg/L | HPLC-UV | Sommer, 1999 / EU agreed |
|  |  | LOQ: 0.05 μg/L | HPLC-MS/MS | Sommer, 2001b / EU agreed |
|  | ILV | - | - | - |
|  | Confirmatory | Not required. | - | - |
| Surface water | Primary | LOQ: 0.05 μg/L | HPLC-MS/MS | Sommer, 2001b / EU agreed |
|  | Confirmatory | Not required. | - | - |
| Component of residue definition: JAU6476-S-methyl |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | JAU6476-S-methyl LOQ: 10 μg/L | HPLC-UV | Sommer, 2001a / EU agreed |
|  | ILV | - | - | - |
|  | Confirmatory | Not required. | - | - |
| Surface water | Primary |  |  |  |
|  | Confirmatory | Not required. | - | - |

For any special comments or remarkable points concerning the analytical methods for water please refer to Appendix 2.

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in air is given in the following tables. For the detailed evaluation of new / additional studies please refer to Appendix 2.

Table 5.3-11:	Validated methods for air (if appropriate)

| Component of residue definition: Prothioconazole |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | LOQ: 0.015 mg/m3 | HPLC-MS/MS | Maasfeld, 2002 / EU agreed |
| Confirmatory | Not required. | - | - |
| Component of residue definition: JAU6476-desthio |  |  |  |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | LOQ: 0.0006 mg/m3 | HPLC-MS/MS | Maasfeld, 2002 / EU agreed |
| Confirmatory | Not required. | - | - |

For any special comments or remarkable points concerning the analytical methods for air it is referred to Appendix 2.

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Prothioconazole in body fluids and tissues is given in the following table. For the detailed evaluation of new/additional studies it is referred to Appendix 2.

Currently agreed EU endpoints for prothioconazole do not include a residue definition for monitoring in body fluids and tissues.

The development of monitoring methods for body fluids and tissues will be required once the active substance is renewed and the residue definitions in these matrices are finalized at EU level.

Table 5.3-12:	Methods for body fluids and tissues (if appropriate)

| Component of residue definition: JAU6476-desthio |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | Not required. | - | - |
| Confirmatory | Not required. | - | - |

For any special comments or remarkable points concerning the analytical methods for body fluids and tissues please refer to Appendix 2.

#### Other studies/ information

Not relevant.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1/01 KCP 5.1.1/02 | Silva, S. | 2019 | Prothioconazole 250 g/L EC (SAP250F) – Physical, chemical and technical properties of the plant protection product Study No EF/297/19 – Interim Report (T0) Ascenza Agro, S.A. GLP Unpublished | N | ASCENZA AGRO, S.A. |
| KCP 5.1/01 | Siche, O., Wydra, V. | 2019a | SAP250F: Acute toxicity to Daphnia magna in a semi-static 48-hour immobilisation test. Ibacon Study No 130811220 GLP Not published. | N | ASCENZA AGRO S.A. |
| KCP 5.1/02 | Siche, O., Wydra, V. | 2019b | SAP250F: Toxicity to Pseudokirchneriella subcapitata in an algal Growth Inhibition Test. Ibacon Study No 130811210 GLP Not published. | N | ASCENZA AGRO S.A. |
| KCP 5.1/03 | Siche, O., Wydra, V. | 2019c | SAP250F: Toxicity to the Aquatic Plant Lemna gibba in a semi-static growth inhibition test. Ibacon Study No 130811240 GLP Not published. | N | ASCENZA AGRO S.A. |
| KCP 5.1.2/01 | Longhi, D. | 2021a | Validation of an analytical method for the quantification of Difenoconazole and Prothioconazole-desthio in wheat, barley, oilseed rape and processed commodities Report No GLP-STUDY-21-31 LabAnalysis s.r.l. GLP Unpublished | N | INDOFIL Industries (Netherlands) B.V. |
| KCP 5.1.2/03 | Nichetti, S | 2022a | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Straw (wheat). Study No. CH-1081-2021 GLP Not Published | N | INDOFIL Industries (Netherlands) B.V. |
| KCP 5.1.2/04 | Nichetti, S | 2022b | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Grain (wheat). Study No. CH-1082-2021 GLP Not Published | N | INDOFIL Industries (Netherlands) B.V. |
| KCP 5.1.2/05 | Nichetti, S | 2022c | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Rapeseed seeds. Study No. CH-1083-2021 GLP Not Published | N | INDOFIL Industries (Netherlands) B.V. |
| KCP 5.1.2/06 | Nichetti, S | 2022d | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Whole plant (rapeseed). Study No. CH-1084-2021 GLP Not Published | N | INDOFIL Industries (Netherlands) B.V. |
| KCP 5.1.2/02 | Longhi, D. | 2021b | Validation of an analytical method for the quantification of Triazole Derivative Metabolites (TDMs) in wheat, barley, oilseed rape and processed commodities Report No GLP-STUDY-21-108 LabAnalysis s.r.l. GLP Unpublished | N | INDOFIL Industries (Netherlands) B.V. |
| 5.3.2.3/01 | Rovetto I. | 2023 | Magnitude of the residue of difenoconazole, prothioconazole, prothioconazole-desthio and triazole-derivative metabolites (TDMs) in honey after one application of IN233C1560 380 EC on Phacelia crop under semi field conditions in four trials in Northern Europe and Southern Europe – 2023 Study code: 1111.4F.SAG23 GLP Not Published | N | INDOFIL Industries (Netherlands) B.V. |
| 5.3.2.3/02 | Mattioli B. | 2023 | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole, Prothioconazole, Prothioconazole-desthio and Triazole Derivative Metabolites (TDMs) residue in Honey Study No. CH – 0859-2023 GLP Not Published | N | INDOFIL Industries (Netherlands) B.V. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCA 5.2.2 (KCP 5.1.2) | Heinemann, O. | 2000a | Analytical determination of residues of JAU 6476 and desthio-JAU 6476 in/ on cereals by HPLC/MS/MS. Bayer AG, Report nº: 00598. Date: 2000-03-20 | N | BAY |
| KCA 5.2.2 (KCP 5.1.2) | Heinemann, O. | 2000b | Analytical determination of residues of JAU6476 and JAU6476-desthio in/on cereals and canola by HPLC-MS/MS (method modification 00598/M001). Bayer AG, Report nº: 00598/M001, Date: 2000-08-28 | N | BAY |
| KCA 5.2.2 (KCP 5.1.2) | Heinemann, O. | 2001a | Analytical determination of residues of JAU6476-sulfonic acid and JAU6476-desthio in/on cereals and canola by HPLC-MS/MS. Bayer AG, Report nº: 00647, Date: 2001-04-20 | N | BAY |
| KCA 5.2.2 (KCP 5.1.2) | Heinemann, O. | 2001b | Analytical determination of residues of JAU6476-3-hydroxy-desthio, JAU6476-4-hydroxy-desthio, and JAU6476-desthio in/on matrices of animal origin by HPLC-MS/MS. Bayer AG, Report nº: 00655, Date: 2001-02-27 | N | BAY |
| KCA 5.2.2 (KCP 5.1.2) | Heinemann, O. | 2001c | Analytical determination of residues of JAU6476-3-hydroxy-desthio, JAY6476-4-hydroxy-desthio, and JAU6476-desthio in milk by HPLC-MS/MS (00655/M001). Bayer AG, Report nº: 00655/M001, Date: 2001-05-04 | N | BAY |
| KCA 5.3.2.2 (KCP 5.2) | Weeren, R. D.; Pelz, S. | 2000 | Modification M033 of method 00086: Validation of DFG method S 19 (extended revision) for the determination of residues of JAU 6476-desthio in materials of plant and animal origin. Dr. Specht & Partner, Chemische Laboratorien GmbH, Hamburg, Germany. Bayer AG, Report nº: 00086/M033, Date: 2000-11-20 | N | BAY |
| KCA 5.3.2.2 (KCP 5.2) | Class, Th. | 2001 | Independent laboratory validation of DFG method S19 (extended revision) for the determination of residues of JAU 6476-desthio (BAYER method 00086/M033) in plant materials PTRL Europe, Ulm, Germany. Bayer AG, Report nº: P/B 484 G, Date: 2001-05-15 | N | BAY |
| KCA 5.3.2.3 (KCP 5.2) | Dubey, L. | 2001 | Independent laboratory validation of bayer methods 00655 and 00655/M001 for the determination of residues of JAU6476-3-hydroxy-desthio, JAU6476-4-hydroxy-desthio, JAU6476-desthio in/on matreces of animal origin by HPLC-MS/MS. Battelle, Geneva Research Centres, Carouge/Geneva, Switzerland. Bayer AG, Report nº: A-14-01-01, Date: 2001-10-16 | N | BAY |
| KCA 5.3.2.4 (KCP 5.2) | Schramel, O. | 2000 | Residue analytical method 00610 (MR-643/99) for the determination of JAU 6476 and the metabolites JAU6476-desthio and JAU6476-S-methyl in soil by HPLC-MS/MS. Bayer AG, Report No.: 00610, Date:2000-07-13 | N | BAY |
| KCA 5.3.2.4 (KCP 5.2) | Steinhauer, S. | 2001 | Enforcement method 00086/M038 for the determination of the residues of JAU 6476-desthio in soil – validation of DFG method S 19 (extended revision) -Dr. Specht & Partner, Chemische Laboratorien GmbH, Hamburg, Germany Bayer AG, Report No.: 00086/M038, Date:2001-07-25 | N | BAY |
| KCA 5.3.2.5 (KCP 5.2) | Sommer, H. | 1999 | Method for the determination of JAU 6476 and SXX 0665 in test water from aquatic toxicity tests by HPLC [Tox/Ecotox method] Bayer AG, Report No.: 00586, Date:1999-05-28 | N | BAY |
| KCA 5.3.2.5 (KCP 5.2) | Sommer, H. | 2001b | Enforcement method 00684 for determination of JAU 6476 and JAU 6476-desthio in drinking and surface water by HPLC-MS/MS Bayer AG, Report No.: 00684, Date:2001-10-23 | N | BAY |
| KCA 5.3.2.5 (KCP 5.2 | Sommer, H. | 2001a | Tox/Ecotox method: Method for determination of JAU6476-S-methyl in test water from aquatic toxicity tests by HPLC-UV Bayer AG, Report No.: 00699, Date:2001-06-21 | N | BAY |
| KCA 5.3.2.6 (KCP 5.2 | Maasfeld, W. | 2002 | Method for the determination of JAU 6476 in air by HPLC-MS/MS Bayer AG, Report No.: 00724, Date:2002-01-22 | N | BAY |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for Prothioconazole

    - Methods used for the generation of pre-authorization data (KCP 5.1)

      - Analytical method 1

        - Method validation

| Comments of zRMS: | Method is acceptable |
| --- | --- |

| Reference: | KCP 5.1/01 |
| --- | --- |
| Report | SAP250F: Acute Toxicity to Daphnia magna in a semi-static 48-hour immobilisation test, Siche, O., Wydra, V., 2019a, Study No. 130811220. |
| Guideline(s): | Yes SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (part A; Section 5) of directive 91/414 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

| The content determination was performed in specimens following SAP250F application. The technique applied for Prothioconazole content determination was LC-MS/MS. For the determination of the stability of the test item under the test conditions and of the mainte-nance of the test item concentrations during the test period, duplicate samples from the test media of all test concentrations and the control (containing algae) were taken at the end of the test (after the 72 hours test period) by pouring together the contents of the test beakers of each treatment. All samples were diluted by a factor of two with acetonitrile. Additional samples of the control and of the dilution solvent were taken at each sampling without any sample treat-ment. Standards were prepared in solvent for the determination. The water sample are directly analysed so no extraction efficiency is needed. |  |
| --- | --- |
| LC-MS/MS Conditions |  |
| LC: | Agilent Series 1200 pump and autosampler |
| Column: | Kinetex C18 2.6 µm100 Å, 50 * 2.1 mm |
| Pre-Column: | Security Guard Cartridge C18 4 * 2 mm |
| Temperature: | 20 °C |
| Mobile Phase: | A: Acetonitrile containing 0.1 % formic acid B: HPLC water containing 0.1 % formic acid Isocratic: 70/30, A/B |
| Run Time: | 5 min |
| Flow Rate: | 0.3 mL/min |
| Injection Volume: | 40 µL |
| Detector: | Mass spectrometer API 4000 |
|  | Parameters: Ion Source: Electrospray ionization (ESI) positive Curtain gas (CUR): 10 psi Nebulizer gas (GS1): 15 psi Voltage (IS): 5500 V Heater Temperature (TEM) : 100 °C Turbo gas (GS2): 0 psi Collision gas (CAD): 4 Dwell time: 150 ms |
|  | MRM mass transitions: |
| Item; Precursor Ion; Product Ion; Declustering Potential (DP); Entrance Potential (EP); Collision Energy (CE); Collision Exit Potential (CXP). [m/z]; [m/z]; [V]; [V]; [V]; [V]. Prothioconanzole (quantifier); 344.3; 154.1; 46; 10; 35; 14. Prothioconanzole (qualifier); 344.3; 189.0; 46; 10; 15; 20. |

Results and discussions

Linearity range from 30% of the LOQ to 20% above the highest level

| Specificity: | No interference of total peak area for the target analyte was found. |
| --- | --- |
| Linearity: | Calibration Range: 0.005 – 0.074 mg a.i./L |
|  | Linearity of Response: Correlation of peak area of different standard solutions with their corresponding concentrations, using a linear regression |
|  | Correlation Coefficient: r = 0.9993 |
|  | Calibration Curve: y=677708*x-16 |
| Accuracy and Precision: | Mean Recovery Rates in the Fortified Samples: 99 % (n = 10, RSD 5 %) The values found for the precision (RSD) and for the accuracy (mean recovery rate) are acceptable |
| Identification: | Confirmation by high specificity of the mass transition and comparation of retention time |

Table A 1:	Recovery results from method validation of Prothioconazole using the analytical method

| Matrix | Analyte | Fortification level (mga.i./L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Test Item: SAP250F in Daphnia magna water | Prothioconazole | 0.025 | 97 | 3 | Overall mean value (n=10) 99%, RSD 5% |
| Test Item: SAP250F in Daphnia magna water | Prothioconazole | 0.050 | 102 | 5 |  |

Table A 2:	Characteristics for the analytical method used for validation of Prothioconazole residues in surface water

|  | Prothioconazole |
| --- | --- |
| Specificity | No interference of total peak area for the target analyte was found. |
| Calibration (type, number of data points) | Correlation of peak area of different standard solutions with their corresponding concentrations, using a linear regression y= 677708*x-16 r= 0.9993 n= 6 |
| Calibration range | 0.005 – 0.074 mg a.i./L |
| Assessment of matrix effects is presented | yes |
| Limit of determination/quantification | LOD = 0.3 μg a.i./ /L LOQ = 0.05 mg /L |

Conclusion

The validity criteria for the analytical method have been met.

      - Analytical method 2

        - Method validation

| Comments of zRMS: | Method is acceptable |
| --- | --- |

| Reference: | KCP 5.1/02 |
| --- | --- |
| Report | SAP250F: Toxicity to Pseudokirchneriella subcapitata in an algal growth inhibition test, Siche, O., Wydra, V., 2019b, Study No. 130811210. |
| Guideline(s): | Yes SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (part A; Section 5) of directive 91/414 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

| The content determination was performed in specimens following SAP250F application. The technique applied for Prothioconazole content determination was LC-MS/MS. For the determination of the stability of the test item under the test conditions and of the maintenance of the test item concentrations during the renewal periods, duplicate samples of all aged test media and control were sampled at each test medium renewal at the end of the test. Standards were prepared in solvent for the determination. The water sample are directly analysed so no extraction efficiency is needed. |  |
| --- | --- |
| LC-MS/MS Conditions |  |
| LC: | Agilent Series 1200 pump and autosampler |
| Column: | Kinetex C18 2.6 µm100 Å, 50 * 2.1 mm |
| Pre-Column: | Security Guard Cartridge C18 4 * 2 mm |
| Temperature: | 20 °C |
| Mobile Phase: | A: Acetonitrile containing 0.1 % formic acid B: HPLC water containing 0.1 % formic acid Isocratic: 70/30, A/B |
| Run Time: | 3 min |
| Flow Rate: | 0.3 mL/min |
| Injection Volume: | 20 µL |
| Detector: | Mass spectrometer API 4000 |
|  | Parameters: Ion Source: Electrospray ionization (ESI) positive Curtain gas (CUR): 10 psi Nebulizer gas (GS1): 15 psi Voltage (IS): 5500 V Heater Temperature (TEM) : 100 °C Turbo gas (GS2): 0 psi Collision gas (CAD): 4 Dwell time: 150 ms |
|  | MRM mass transitions: |
| Item; Precursor Ion; Product Ion; Declustering Potential (DP); Entrance Potential (EP); Collision Energy (CE); Collision Exit Potential (CXP). [m/z]; [m/z]; [V]; [V]; [V]; [V]. Prothioconanzole (quantifier); 344.3; 154.1; 46; 10; 35; 14. Prothioconanzole (qualifier); 344.3; 189.0; 46; 10; 15; 20. |

Results and discussions

Linearity range from 30% of the LOQ to 20% above the highest level

| Specificity: | No interference of total peak area for the target analyte was found. |
| --- | --- |
| Linearity: | Calibration Range: 1 – 100 µg a.i./L |
|  | Linearity of Response: Correlation of peak area of different standard solutions with their corresponding concentrations, using a linear regression |
|  | Correlation Coefficient: 1. r = 1.0000 2. r = 0.9995 |
|  | Calibration Curve: 1. y=706*x+56 2. y=475*x+409 |
| Accuracy and Precision: | Mean Recovery Rates in the Fortified Samples: 105 % (n = 10, RSD 3 %) The values found for the precision (RSD) and for the accuracy (mean recovery rate) are acceptable |
| Identification: | Confirmation by high specificity of the mass transition and comparation of retention time |

Table A 3:	Recovery results from method validation of Prothioconazole using the analytical method

| Matrix | Analyte | Fortification level (µg a.i/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Test Item: SAP250F in Daphnia magna water | Prothioconazole | 12.5 | 105 | 3 | Overall mean value (n=10) 105%, RSD 3% |
| Test Item: SAP250F in Daphnia magna water | Prothioconazole | 75 | 105 | 2 |  |

Table A 4:	Characteristics for the analytical method used for validation of Prothioconazole residues in surface water

|  | Prothioconazole |
| --- | --- |
| Specificity | No interference of total peak area for the target analyte was found. |
| Calibration (type, number of data points) | Correlation of peak area of different standard solutions with their corresponding concentrations, using a linear regression 1. y=706*x+56 2. y=475*x+409 1. r = 1.0000 2. r = 0.9995 n= 8 |
| Calibration range | 0.997 – 99.704 µg a.i./L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.3 μg a.i./L LOQ = 12.5 μg a.i./L |

Conclusion

The validity criteria for the analytical method have been met.

      - Analytical method 3

        - Method validation

| Comments of zRMS: | Method is acceptable |
| --- | --- |

| Reference: | KCP 5.1/03 |
| --- | --- |
| Report | SAP250F: Toxicity to the aquatic plant Lemna gibba in a semi-static growth inhibition test, Siche, O., Wydra, V., 2019c, Study No. 130811240. |
| Guideline(s): | Yes SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (part A; Section 5) of directive 91/414 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

| The content determination was performed in specimens following SAP250F application. The technique applied for Prothioconazole content determination was LC-MS/MS. For the determination of the stability of the test item under the test conditions and of the maintenance of the test item concentrations during the renewal periods, duplicate samples of all aged test media and control were sampled at each test medium renewal at the end of the test. Standards were prepared in solvent for the determination. The water sample are directly analysed so no extraction efficiency is needed. |  |
| --- | --- |
| LC-MS/MS Conditions |  |
| LC: | Agilent Series 1200 pump and autosampler |
| Column: | Kinetex C18 2.6 µm100 Å, 50 * 2.1 mm |
| Pre-Column: | Security Guard Cartridge C18 4 * 2 mm |
| Temperature: | 20 °C |
| Mobile Phase: | A: Acetonitrile containing 0.1 % formic acid B: HPLC water containing 0.1 % formic acid Isocratic: 70/30, A/B |
| Run Time: | 5 min |
| Flow Rate: | 0.3 mL/min |
| Injection Volume: | 20 µL |
| Detector: | Mass spectrometer API 4000 |
|  | Parameters: Ion Source: Electrospray ionization (ESI) positive Curtain gas (CUR): 10 psi Nebulizer gas (GS1): 15 psi Voltage (IS): 5500 V Heater Temperature (TEM) : 100 °C Turbo gas (GS2): 0 psi Collision gas (CAD): 4 Dwell time: 150 ms |
|  | MRM mass transitions: |
| Item; Precursor Ion; Product Ion; Declustering Potential (DP); Entrance Potential (EP); Collision Energy (CE); Collision Exit Potential (CXP). [m/z]; [m/z]; [V]; [V]; [V]; [V]. Prothioconanzole (quantifier); 344.3; 154.1; 46; 10; 35; 14. Prothioconanzole (qualifier); 344.3; 189.3; 46; 10; 21; 14. |

Results and discussions

Linearity range from 30% of the LOQ to 20% above the highest level

| Specificity: | No interference of total peak area for the target analyte was found. |
| --- | --- |
| Linearity: | Calibration Range: 1 – 100 µg a.i./L |
|  | Linearity of Response: Correlation of peak area of different standard solutions with their corresponding concentrations, using a linear regression |
|  | Correlation Coefficient: r = 0.9999 |
|  | Calibration Curve: y=164*x-10 |
| Accuracy and Precision: | Mean Recovery Rates in the Fortified Samples: 99 % (n = 10, RSD 4 %) The values found for the precision (RSD) and for the accuracy (mean recovery rate) are acceptable |
| Identification: | Confirmation by high specificity of the mass transition and comparation of retention time |

Table A 5:	Recovery results from method validation of Prothioconazole using the analytical method

| Matrix | Analyte | Fortification level (µg a.i./L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Test Item: SAP250F in Daphnia magna water | Prothioconazole | 20 | 94 | 5 | Overall mean value (n=10) 96%, RSD 4% |
| Test Item: SAP250F in Daphnia magna water | Prothioconazole | 100 | 97 | 3 |  |

Table A 6:	Characteristics for the analytical method used for validation of Prothioconazole residues in surface water

|  | Prothioconazole |
| --- | --- |
| Specificity | No interference of total peak area for the target analyte was found. |
| Calibration (type, number of data points) | Correlation of peak area of different standard solutions with their corresponding concentrations, using a linear regression y=164*x-10 r = 0.9999 n= 8 |
| Calibration range | 0.981 – 98.099 µg a.i./L |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOD = 0.95 μg a.i./L LOQ = 20 μg a.i./L |

Conclusion

The validity criteria for the analytical method have been met.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

        - Analytical method 1 (Wheat, barley, oilseed rape and processed commodities)

          - Method validation

| Comments of zRMS: | Method is acceptable |
| --- | --- |

| Reference: | KCP 5.1.2/01 |
| --- | --- |
| Report | Difenoconazole 250 g/L EC Greener – IN005B1570: Validation of an analytical method for the quantification of Difenoconazole and Prothioconazole-desthio in wheat, barley, oilseed rape and processed commodities Longhi, D. 2021 Study No. GLP-STUDY-21-31 |
| Guideline(s): | Yes. - European Commission, Guidance Document on Pesticide Analytical Methods for Risk Assessment and Post-approval Control and Monitoring Purposes, SANTE/2020/12830, Rev.1 (24/02/2021). - European Commission (2017): SANTE 2017/10632 rev. 3, dated 22 November 2017: Technical Guideline on the Evaluation of Extraction Efficiency of Residue Analytical Methods. - OECD Guidance Document on Pesticide Residue Analytical Methods ENV/JM/MONO(2007)17. - “European Committee for Standardisation (CEN) EN 15662:2018. Foods of plant origin - Multimethod for the determination of pesticide residues using GC- and LC-based analysis following acetonitrile extraction/partitioning and clean-up by dispersive SPE - Modular QuEChERS-method”. |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The analytical method for the determination of Difenoconazole and Prothioconazole-desthio in the tested matrices (AM-GLP-STUDY-21-31) was based on the QuEChERS method (EN 15662_2018). The instrumental determination was carried out using a HPLC-MS/MS system (high-performance liquid chromatography + triple quadrupole mass spectrometry).

Description of the method

Sample extraction

Plant matrices and processed commodities (whole plant, rapeseed seeds, wheat grain, white bread, straw):

Aliquots of 5 g of specimen (2.5 for straw) were taken from the homogenised frozen samples and put in a 50 mL screw capped centrifuge PE test tube followed by the addition of the following amounts of LC-MS grade water:

| Matrix | Water added (mL) |
| --- | --- |
| Whole plant(Rapeseed) | 0 |
| Rapeseed seeds | 10 |
| Grain (wheat) | 10 |
| Straw (wheat) | 10 |
| White bread (wheat) | 10 |

Then, 20 mL of acetonitrile were added and the obtained mixture was vigorously shaken for one minute. After that, a packet of QuEChERS extraction salt (4.0 g MgSO4, 1.0 g NaCl, 1.0 g trisodium citrate dehydrate, 0.5 g disodium hydrogen citrate sesquihydrate) was added and the mixture shaken again. The separation of the organic phase was achieved by centrifugation at 4500 rpm for 5 minutes. An aliquot of about 1 mL the organic supernatant was taken, transferred in a 2 mL HPLC glass vial and analysed with a HPLC-MS/MS system.

Beer:

Beer was analysed after a 5-fold dilution in a mixture of water/methanol 50:50 (about 5 g to 25 mL) and directly analysed with a HPLC-MS/MS system.

Rapeseed oil:

An aliquot of about 5 g of rapeseed oil was put in a 50 mL screw capped centrifuge PE test tube. Then, 15 mL of hexane were added, followed by 20 mL of a mixture of acetonitrile/water 80:20. The mixture was vigorously shaken for about one minute and then centrifuged at 4500 rpm for 5 minutes, in order to obtain 2 phases. An aliquot of the lower organic phase (acetonitrile) was taken and transferred to a 2 mL glass HPLC vial for final determination with a HPLC-MS/MS system.

Reference solutions preparation:

The following reference solutions were prepared:

| Solution | Starting material | Weight/Volume | Final volume (mL) (acetonitrile) | Actual concentration (mg/L) |
| --- | --- | --- | --- | --- |
| Difenoconazole stock solution (Difenoconazole ~ 2000 mg/L) | Difenoconazole reference material (purity: 95.4%) | 20.56 mg | 10 | 1961 |
| Prothioconazole-desthio stock solution (Prothioconazole-desthio ~ 1000 mg/L) | Prothioconazole-desthio reference material (purity: 99.55%) | 10.81 mg | 10 | 1076 |
| Solution A (~ 10 mg/L) | Difenoconazole stock solution + Prothioconazole-desthio stock solution | 51 µL + 93 µL | 10 | 10.0 |
| Solution B (~ 1 mg/L) | Difenoconazole stock solution + Prothioconazole-desthio stock solution | 5.1 µL + 9.3 µL | 10 | 1.00 |

Matrix-matched standard solutions for plant matrices, processed commodities, rapeseed oil were prepared from Solution B using the final extract of an unfortified aliquot on the basis of the following scheme:

| Solution | µL of Solution B | Final volume (mL) | Nominal concentration (µg/L) | Nominal concentration on the sample1 (mg/kg) |
| --- | --- | --- | --- | --- |
| L1 | 1 | 2 | 0.5 | 0.002 |
| L2 | 2.5 | 1 | 2.5 | 0.010 |
| L3 | 5 | 1 | 5 | 0.020 |
| L4 | 25 | 1 | 25 | 0.100 |
| L5 | 50 | 1 | 50 | 0.200 |

1: calculated considering the nominal sample preparation (5.0 g to a final volume of 20.0 mL)

Matrix-matched analytical standard solutions for straw were prepared from Solution B using the final extract of an unfortified aliquot on the basis of the following scheme:

| Solution | µL of Solution B | Final volume (mL) | Nominal concentration (µg/L) | Nominal concentration on the sample1 (mg/kg) |
| --- | --- | --- | --- | --- |
| L1 | 1.2 | 2 | 0.3 | 0.0024 |
| L2 | 1 | 1 | 2.5 | 0.008 |
| L3 | 2.5 | 1 | 5 | 0.020 |
| L4 | 10 | 1 | 10 | 0.080 |
| L5 | 25 | 1 | 25 | 0.200 |

1: calculated considering the nominal sample preparation (2.5 g to a final volume of 20.0 mL)

Matrix-matched analytical standard solutions for beer were prepared from Solution B using the final extract of an unfortified aliquot on the basis of the following scheme:

| Solution | µL of Solution B | Final volume (mL) | Nominal concentration (µg/L) | Nominal concentration on the sample1 (mg/kg) |
| --- | --- | --- | --- | --- |
| L1 | 1 | 2 | 0.5 | 0.0025 |
| L2 | 2.5 | 1 | 2.5 | 0.0125 |
| L3 | 5 | 1 | 5 | 0.025 |
| L4 | 25 | 1 | 25 | 0.125 |
| L5 | 50 | 1 | 50 | 0.250 |

1: calculated considering the nominal sample preparation (5.0 g to a final volume of 25.0 mL)

The analyses were carried out using a HPLC-MS/MS system according to the following conditions:

Instrument: 	HPLC Agilent 1290 Infinity II coupled with a triple quadrupole mass spectrometer Agilent 6470A

Column: 		Phenomenex Kinetex C18, 1.7 μm, 2.1 x 50 mm

Column temperature: 	40°C

Flow: 			0.6 mL/min

Injection volume: 	2.5 μL

Mobile phase A: 	LC-MS grade water with 0.2 % formic acid and 5 mM ammonium formate

Mobile phase B: 	LC-MS grade methanol with 0.2 % formic acid and 5 mM ammonium formate

Elution: 		gradient of the following composition:

| Time (min) | % A | % B |
| --- | --- | --- |
| 0 | 50 | 50 |
| 0.5 | 50 | 50 |
| 3 | 0 | 100 |

Stop time: 		5 min

Post time: 		1 min

Divert valve: 		0 min. to waste, 1.5 min. to MS, 3 min. to waste

Source type: 		ESI

Gas temperature: 	350°C

Gas flow (L/min): 	5

Nebulizer (psi): 	40

Sheath gas heater: 	400°C

Sheath gas flow (L/min): 12

Capillary: 		positive mode 3500 V, negative mode 3000 V

Vcharging:		 0

Acquiring mode: 	ESI positive and ESI negative, MRM (multi-reaction monitoring).

Calibration

The quantification of each analyte was made through the building of a calibration straight line with the external standard method. 5 matrix-matched analytical standard solutions were analysed in single injections in order to obtain a calibration curve (1/x weighed) interpolated with a linear regression.

Recovery (Accuracy) and Repeatability (Precision)

Recovery and repeatability (as precision, % RSD) data will be reported for the following fortification levels:

- LOQ level (5 replicates): 0.01 mg/kg

- 10xLOQ level (5 replicates): 0.1 mg/kg - or alternatively at a spiking level higher than maximum residue level found on field specimens.

Specificity (Selectivity)

This parameter will be evaluated in order to demonstrate that the applied method detects the right analyte and that the analytical signal is quantitatively correct and not affected by other analytes or by matrix interferences.

Using a MS/MS mass spectrometer detector the selectivity was evaluated comparing the following chromatograms: samples, fortified samples, reference solutions at the LOQ level in order to assess the presence or absence of interfering signals. No interfering signals were detected in the untreated matrix in amount higher than 30% of the LOQ level, that is in compliance with the guideline requirements.

Matrix effect

Assessment of matrix effects will be performed by comparing the analyte response of at least one individual standard prepared in solvent to at least one prepared in blank matrix, for all sample matrix used in the study. In order to nullify matrix effect, calibration curves for all matrices analysed will be prepared using matrix matched analytical standards.

Limit of detection (LOD)

The limit of detection (LOD) is defined as the lowest detectable concentration or amount of an analyte in a sample. It is the lowest concentration at which an analyte produces an instrumental signal at least 3 times higher than background noise of the chromatogram. It should be not higher than 30% of LOQ value and it will be considered as the lowest point of the instrumental calibration.

Limit of quantification (LOQ)

Limit of quantitation (LOQ) is defined as the lowest validated level with sufficient recovery and precision. Target LOQ for Difenoconazole and Prothioconazole-desthio in the sample analysed will be set to 0.01 mg/kg.

Confirmation:

A simultaneous confirmation to the primary detection was used using the HPLC-MS/MS, monitoring an additional SRM transition. The following data will be provided for a product ion scan of a 1 mg/L solution of Difenoconazole + Prothioconazole-desthio (Solution B), to justify the selection of the MS/MS transitions used: calibration data as recorded for primary detection, recovery and precision data as recorded for primary detection (at least for the 5 replicates at LOQ level) the recovery and precision mean values calculated on confirmatory ion must fulfill the same acceptable range reported above for primary detection.

Results and discussions

Method validation data can be summarised in the tables below.

Table A 7:	Recovery results from method validation of Difenoconazole using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | SD | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Whole plant (rapeseed) | Difenoconazole | 0.01 (LOQ) | 87.3 | 2.7 | 3.1 |  |
|  |  | 0.1 (10xLOQ) | 87.4 | 5.7 | 6.5 |  |
| Rapeseed seeds | Difenoconazole | 0.01 (LOQ) | 85.1 | 1.9 | 2.3 |  |
|  |  | 0.1 (10xLOQ) | 86.4 | 5.4 | 6.3 |  |
| Wheat grain | Difenoconazole | 0.01 (LOQ) | 88.9 | 2.1 | 2.4 |  |
|  |  | 0.1 (10xLOQ) | 91.2 | 3.9 | 4.3 |  |
| Straw (wheat) | Difenoconazole | 0.01 (LOQ) | 91.4 | 8.6 | 9.4 |  |
|  |  | 0.1 (10xLOQ) | 93.8 | 7.1 | 7.6 |  |
| Rapeseed oil | Difenoconazole | 0.01 (LOQ) | 88.2 | 3.8 | 4.3 |  |
|  |  | 0.1 (10xLOQ) | 87.9 | 1.7 | 2.0 |  |
| White bread (wheat) | Difenoconazole | 0.01 (LOQ) | 100.1 | 7.7 | 7.7 |  |
|  |  | 0.1 (10xLOQ) | 95.5 | 0.54 | 0.54 |  |
| Beer (barley) | Difenoconazole | 0.01 (LOQ) | 89.9 | 6.1 | 6.7 |  |
|  |  | 0.1 (10xLOQ) | 84.9 | 1.5 | 1.8 |  |

Table A 8:	Recovery results from method validation of Prothioconazole-desthio using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | SD | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Whole plant (rapeseed) | Prothioconazole-desthio | 0.01 (LOQ) | 85.4 | 7.2 | 8.5 |  |
|  |  | 0.1 (10xLOQ) | 82.7 | 6.7 | 8.1 |  |
| Rapeseed seeds | Prothioconazole-desthio | 0.01 (LOQ) | 89.3 | 5.1 | 5.7 |  |
|  |  | 0.1 (10xLOQ) | 81.5 | 5.3 | 6.5 |  |
| Wheat grain | Prothioconazole-desthio | 0.01 (LOQ) | 87.4 | 3.9 | 4.5 |  |
|  |  | 0.1 (10xLOQ) | 86.7 | 5.6 | 6.4 |  |
| Straw (wheat) | Prothioconazole-desthio | 0.01 (LOQ) | 80.1 | 2.9 | 3.6 |  |
|  |  | 0.1 (10xLOQ) | 86.9 | 5.4 | 6.3 |  |
| Rapeseed oil | Prothioconazole-desthio | 0.01 (LOQ) | 79.5 | 5.3 | 6.7 |  |
|  |  | 0.1 (10xLOQ) | 77.9 | 1.1 | 1.4 |  |
| White bread (wheat) | Prothioconazole-desthio | 0.01 (LOQ) | 97.7 | 1.7 | 1.8 |  |
|  |  | 0.1 (10xLOQ) | 96.0 | 0.41 | 0.42 |  |
| Beer (barley) | Prothioconazole-desthio | 0.01 (LOQ) | 87.3 | 9.5 | 10.8 |  |
|  |  | 0.1 (10xLOQ) | 80.4 | 1.9 | 2.3 |  |

Table A 3:	Characteristics for the analytical method used for validation of Difenoconazole and Prothioconazole-desthio residues in wheat, barley, oilseed rape and processed commodities

|  | Difenoconazole | Prothioconazole-desthio |
| --- | --- | --- |
| Specificity | MS spectrum provided: Yes Positive polarity m/z 406.2 – 251.1 m/z 406.2 – 188.4 | MS spectrum provided: Yes Negative polarity m/z 312.2 – 69.8 m/z 312.2 – 125.0 |
| Calibration (type, number of data points) | Rapeseed seeds : n = 5 y = 2133.411025x + 406.088099 R2 = 0.999 | Rapeseed seeds : n = 5 y = 1033.676304x + -7.425110 R2 = 0.999 |
|  | Whole Plant (Rapeseed) : n = 5 y = 2393.089220 x + 525.166810 R2 = 0.999 | Whole Plant (Rapeseed) : n = 5 y = 1240.335143 x + 41.225438 R2 = 0.999 |
|  | Grain (wheat) : n = 5 y = 2417.915699 x + 657.442734 R2 = 0.999 | Grain (wheat) : n = 5 y = 1086.126498 x + 28.886023 R2 = 0.999 |
|  | Straw (wheat) : n = 5 y = 3702.782310 x + 1318.496403 R2 = 0.999 | Straw (wheat) : n = 5 y = 1318.462397 x + 97.326629 R2 = 0.999 |
|  | Rapeseed oil : n = 5 y = 2778.122779 x + 355.419442 R2 = 0.999 | Rapeseed oil : n = 5 y = 1151.412873 x + 126.288342 R2 = 0.999 |
|  | White bread (wheat) : n = 5 2506.307430 x + 2064.880900 R2 = 0.999 | White bread (wheat) : n = 5 y = 1242.277359 x + 138.386250 R2 = 0.999 |
|  | Beer (barley) : n = 5 y = 2736.071421 x + 1929.069427 R2 = 0.999 | Beer (barley) : n = 5 y = 1096.261065 x + 153.749773 R2 = 0.999 |
| Calibration range | Range: 0.5 – 50.0 µg/L (0.00200 – 0.200 mg/kg) Beer range: 0.5 – 50.0 µg/L (0.00250 – 0.250 mg/kg) Straw (wheat) range: 0.3 – 25.0 µg/L (0.00240 – 0.200 mg/kg) | Range: 0.5 – 50.0 µg/L (0.00200 – 0.200 mg/kg) Beer range: 0.5 – 50.0 µg/L (0.00250 – 0.250 mg/kg) Straw (wheat) range: 0.3 – 25.0 µg/L (0.00240 – 0.200 mg/kg) |
| Assessment of matrix effects is presented | Matrix effects: Yes For wheat straw the matrix effect is considered significant, since it was found out of the acceptability ranges. | Matrix effects: Yes For wheat straw the matrix effect is considered significant, since it was found out of the acceptability ranges. |
| Limit of determination/quantification | LOD: 0.50 µg/L LOD for wheat: 0.3 µg/L LOQ: 0.01 mg/Kg | LOD: 0.50 µg/L LOD for wheat: 0.3 µg/L LOQ: 0.01 mg/Kg |

Conclusion

A mean recovery of 60-120% with a Relative Standard Deviation ≤ 30% was adopted as acceptability criteria.

The results obtained concerning matrix effects, linearity, selectivity, accuracy (recovery), precision (repeatability), specificity, limit of quantification and limit of detection are in compliance with requirements reported in guideline SANTE/2020/12830 rev. 1 for the analyte.

| Comments of zRMS: | Method is acceptable |
| --- | --- |
| Reference: | KCP 5.1.2/03 |
| Report | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Straw (wheat) Nichetti, S. 2022 Study No. CH-1081-2021 |
| Guideline(s): | Yes. - Council Decision [C(97)186/Final] amending Annex II to the council decision concerning the mutual acceptance of data in the assessment of chemicals [C(81) 30 (final)]. - ENV/MC/CHEM(98)17 OECD Principles on Good Laboratory Practice, No. 1 and all subsequent OECD consensus documents - Directive 2004/9/CE and directive 2004/10/CE of the Suropean Parliament and of the Council of February 11th, 2004 enforced by Italian Legislative Decree No. 50 of March 2nd, 2007 as published in G.U. No. 86 of April 13th, 2007 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Summary

Calibration

The quantification of each analyte was made through the building of a calibration straight line with the external standard method. 5 matrix-matched analytical standard solutions were analysed in single injections in order to obtain a calibration curve (1/x weighed) interpolated with a linear regression.

Recovery (Accuracy) and Repeatability (Precision)

Both repeatability and recovery tests were performed using Straw (wheat) samples, that were fortified five times at about 0.010 mg/kg (LOQ) and five times at 0.100 mg/kg (10 x LOQ), as nominal concentration.

The SANTE/2020/12830 rev. 1 (2021) guideline requires any interference present in the control matrix sample(s) to be lower than 30 % of the LOQ.

No interferences above the LOD were found in the control Straw (wheat) samples.

According the SANTE/2020/12830 rev. 1 guideline’s requirement, the mean recovery values must be in the range 70 to 120 %, with an RSD% lower than 20%. Since all recovery values for each analyte at both fortification levels (L.O.Q and 10 x LOQ) resulted to be in the correct range, these criteria were fulfilled and therefore the analytical method can be considered suitable to quantify Difenoconazole and Prothioconazole-desthio in Straw (wheat) samples with an established L.O.Q of 0.010 mg/kg

Confirmatory

Since the analysis by HPLC using external standards and MS triple quadrupole detector (HPLC/MS/MS) in MRM mode is highly specific and gave both quantification and identification data, a confirmatory test using another instrumental technique was not necessary. The selectivity of the primary method was demonstrated with the obtained data from the qualifier transitions.

Specificity (Selectivity)

The analytical method, using the HPLC/MS/MS instrument with quantification by external standard, was shown to be specific for Difenoconazole and Prothioconazole-desthio residues in in Straw (wheat) samples.

Matrix effect

Significant matrix effects for Difenoconazole and Prothioconazole-desthio residues in Straw (wheat) matrix were found (> ± 20%).

Therefore, the quantification should be performed using working standard solutions prepared

in Straw (wheat) (matrix matched calibration).

Limit of detection (LOD) and Limit of quantification

Data from Linearity test were used to calculate the LOD whereas data from Recovery test were used to calculate the LOQ.

The limit of detection (LOD), defined as the lowest working standard solution WSS 1, was a final injected solution of about 0.30 µg/L for each analyte, corresponding to about 0.0024 mg/kg in Straw (wheat) (30% of the LOQ).

The limit of quantification (LOQ), defined as the lowest fortification level with acceptable recovery and repeatability (70 – 120% with % RSD < 20%), was a final injected solution of about 1.25 µg/L for each analyte, corresponding to about 0.010 mg/kg in Straw (wheat).

The LOD and the LOQ was successfully established for each analyte and both mass transitions. Analyte results calculated as < 0.0024 mg/kg (LOD) are classified as not detectable (n.d.). Analyte results calculated as greater than the limit of detection but less than the limit of quantification, are designated as < 0.010 mg/kg. If the analyte content is calculated as greater than 0.200 mg/kg in Straw (wheat), the final solution must be suitably diluted using volumetric glassware to fit in the calibration range

Linearity in Straw (wheat) matrix

Five working standard solutions (WSS) were prepared in Straw (wheat) matrix as described in the Experimental section and each solution was analysed by HPLC/MS/MS.

No significant memory peak was detected in the wash matrix injected after the highest working standard solution and the range tested for Difenoconazole and Prothioconazole-desthio was found to be linear (each correlation coefficient > 0.99).

Residuals for both the proposed quantifier and confirmation transition were randomly distributed around the zero line with no trend visible.

Stability of stock, fortification and calibration solutions

Stability of stock, fortification and calibration solutions were not assessed since solutions were analysed within 24 hours after preparation

Stability of final extracts/dilution

The stability of final extracts/ dilutions was not assessed since extracts/ dilutions were analysed within 24 hours after preparation

Extraction efficiency

The extraction efficiency was demonstrated following the extraction procedure reported, and already judged acceptable in the GLP study Code GLP-STUDY-21-31 performed by LabAnalysis s.r.l.: “Analytical method validation to quantify Difenoconazole and Prothioconazole-desthio in wheat, barley, oilseed rape and processed commodities”.

Conclusion

The ILV was found to be acceptable.

| Comments of zRMS: | Method is acceptable |
| --- | --- |
| Reference: | KCP 5.1.2/04 |
| Report | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Grain (wheat) Nichetti, S. 2022 Study No. CH-1082-2021 |
| Guideline(s): | Yes. - Council Decision [C(97)186/Final] amending Annex II to the council decision concerning the mutual acceptance of data in the assessment of chemicals [C(81) 30 (final)]. - ENV/MC/CHEM(98)17 OECD Principles on Good Laboratory Practice, No. 1 and all subsequent OECD consensus documents - Directive 2004/9/CE and directive 2004/10/CE of the Suropean Parliament and of the Council of February 11th, 2004 enforced by Italian Legislative Decree No. 50 of March 2nd, 2007 as published in G.U. No. 86 of April 13th, 2007 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Summary

Calibration

The quantification of each analyte was made through the building of a calibration straight line with the external standard method. 5 matrix-matched analytical standard solutions were analysed in single injections in order to obtain a calibration curve (1/x weighed) interpolated with a linear regression.

Recovery (Accuracy) and Repeatability (Precision)

Both repeatability and recovery tests were performed using fortified Grain (wheat) samples, that were fortified five times at about 0.010 mg/kg (LOQ) and five times at 0.100 mg/kg (10 x LOQ), as nominal concentration. The SANTE/2020/12830 rev. 1 (2021) guideline requires any interference present in the control matrix sample(s) to be lower than 30 % of the LOQ. No interferences above the LOD were found in the control Grain (wheat) samples.

According the SANTE/2020/12830 rev. 1 guideline’s requirement, the mean recovery values must be in the range 70 to 120 %, with an RSD% lower than 20%. Since all recovery values for each analyte at both fortification levels (L.O.Q and 10 x LOQ) resulted to be in the correct range, these criteria were fulfilled and therefore the analytical method can be considered suitable to quantify Difenoconazole and Prothioconazole-desthio in Grain (wheat) samples with an established L.O.Q of 0.010 mg/kg.

Confirmatory

Since the analysis by HPLC using external standards and MS triple quadrupole detector (HPLC/MS/MS) in MRM mode is highly specific and gave both quantification and identification data, a confirmatory test using another instrumental technique was not necessary. The selectivity of the primary method was demonstrated with the obtained data from the qualifier transitions

Specificity (Selectivity)

The analytical method, using the HPLC/MS/MS instrument with quantification by external standard, was shown to be specific for Difenoconazole and Prothioconazole-desthio residues in Grain (wheat).

Matrix effect

Not significant matrix effects for Difenoconazole and Prothioconazole-desthio residues in Grain (wheat) matrix were found (< ± 20%). Therefore, the calibration standards could be prepared in solvent or in matrix. A matrix-matched calibration standards were used throughout the entire study

Limit of detection (LOD) and Limit of quantification

Data from Linearity test were used to calculate the LOD whereas data from Recovery test were used to calculate the LOQ. The limit of detection (LOD), defined as the lowest working standard solution WSS 1, was a final injected solution of about 0.50 µg/L for each analyte, corresponding to about 0.002 mg/kg in Grain (wheat) (30% of the LOQ). The limit of quantification (LOQ), defined as the lowest fortification level with acceptable recovery and repeatability (70 – 120% with % RSD < 20%), was a final injected solution of about 2.50 µg/L for each analyte, corresponding to about 0.010 mg/kg in Grain (wheat)

The LOD and the LOQ was successfully established for each analyte and both mass transitions. Analyte results calculated as < 0.002 mg/kg (LOD) are classified as not detectable (n.d.). Analyte results calculated as greater than the limit of detection but less than the limit of quantification, are designated as < 0.010 mg/kg. If the analyte content is calculated as greater than 0.200 mg/kg in Grain (wheat), the final solution must be suitably diluted using volumetric glassware to fit in the calibration range.

Linearity in Grain (wheat) matrix

Five working standard solutions (WSS) were prepared in Grain (wheat) matrix as described in the Experimental section and each solution was analysed by HPLC/MS/MS.

No significant memory peak was detected in the wash matrix injected after the highest working standard solution and the range tested for Difenoconazole and Prothioconazole-desthio was found to be linear (each correlation coefficient > 0.99). Residuals for both the proposed quantifier and confirmation transition were randomly distributed around the zero line with no trend visible.

Stability of stock, fortification and calibration solutions

Stability of stock, fortification and calibration solutions were not assessed since solutions were analysed within 24 hours after preparation.

Stability of final extracts/dilution

The stability of final extracts/ dilutions was not assessed since extracts/ dilutions were analysed within 24 hours after preparation

Extraction efficiency

The extraction efficiency was demonstrated following the extraction procedure reported, and already judged acceptable in the GLP study Code GLP-STUDY-21-31 performed by LabAnalysis s.r.l.: “Analytical method validation to quantify Difenoconazole and Prothioconazole-desthio in wheat, barley, oilseed rape and processed commodities”.

Conclusion

The ILV was found to be acceptable.

| Comments of zRMS: | Method is acceptable |
| --- | --- |
| Reference: | KCP 5.1.2/05 |
| Report | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Rapeseed seeds Nichetti, S. 2022 Study No. CH-1083-2021 |
| Guideline(s): | Yes. - Council Decision [C(97)186/Final] amending Annex II to the council decision concerning the mutual acceptance of data in the assessment of chemicals [C(81) 30 (final)]. - ENV/MC/CHEM(98)17 OECD Principles on Good Laboratory Practice, No. 1 and all subsequent OECD consensus documents - Directive 2004/9/CE and directive 2004/10/CE of the Suropean Parliament and of the Council of February 11th, 2004 enforced by Italian Legislative Decree No. 50 of March 2nd, 2007 as published in G.U. No. 86 of April 13th, 2007 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Summary

Confirmatory

Since the analysis by HPLC using external standards and MS triple quadrupole detector (HPLC/MS/MS) in MRM mode is highly specific and gave both quantification and identification data, a confirmatory test using another instrumental technique was not necessary. The selectivity of the primary method was demonstrated with the obtained data from the qualifier transitions

Recovery (Accuracy) and Repeatability (Precision)

Both repeatability and recovery tests were performed using fortified Rapeseed seeds samples, that were fortified five times at about 0.010 mg/kg (LOQ) and five times at 0.100 mg/kg (10 x LOQ), as nominal concentration. The SANTE/2020/12830 rev. 1 (2021) guideline requires any interference present in the control matrix sample(s) to be lower than 30 % of the LOQ. No interferences above the LOD were found in the control Rapeseed seeds samples, the analysis of fortified samples at low and high level gave the following results

According the SANTE/2020/12830 rev. 1 guideline’s requirement, the mean recovery values must be in the range 70 to 120 %, with an RSD% lower than 20%. Since all recovery values for each analyte at both fortification levels (L.O.Q and 10 x LOQ) resulted to be in the correct range, these criteria were fulfilled and therefore the analytical method can be considered suitable to quantify Difenoconazole and Prothioconazole-desthio in Rapeseed seeds samples with an established L.O.Q of 0.010 mg/kg

Specificity (Selectivity)

The analytical method, using the HPLC/MS/MS instrument with quantification by external standard, was shown to be specific for Difenoconazole and Prothioconazole-desthio residues in Rapeseed seeds samples.

Matrix effect

Not significant matrix effects for Difenoconazole and Prothioconazole-desthio residues in Rapeseed seed matrix were found (< ± 20%). Therefore, the calibration standards could be prepared in solvent or in matrix. A matrix-matched calibration standards were used throughout the entire study.

Limit of detection (LOD) and Limit of quantification

Data from Linearity test were used to calculate the LOD whereas data from Recovery test were used to calculate the LOQ. The limit of detection (LOD), defined as the lowest working standard solution WSS 1, was a final injected solution of about 0.50 µg/L for each analyte, corresponding to about 0.002 mg/kg in Rapeseed seeds (30% of the LOQ). The limit of quantification (LOQ), defined as the lowest fortification level with acceptable recovery and repeatability (70 – 120% with % RSD < 20%), was a final injected solution of about 2.50 µg/L for each analyte, corresponding to about 0.010 mg/kg in Rapeseed seeds.

The LOD and the LOQ was successfully established for each analyte and both mass transitions. Analyte results calculated as < 0.002 mg/kg (LOD) are classified as not detectable (n.d.). Analyte results calculated as greater than the limit of detection but less than the limit of quantification, are designated as < 0.010 mg/kg. If the analyte content is calculated as greater than 0.200 mg/kg in Rapeseed seeds, the final solution must be suitably diluted using volumetric glassware to fit in the calibration range.

Linearity in rapeseed seeds matrix

Five working standard solutions (WSS) were prepared in Rapeseed seeds matrix as described in the Experimental section and each solution was analysed by HPLC/MS/MS.

No significant memory peak was detected in the wash matrix injected after the highest working standard solution and the range tested for Difenoconazole and Prothioconazole-desthio was found to be linear (each correlation coefficient > 0.99)

Residuals for both the proposed quantifier and confirmation transition were randomly distributed around the zero line with no trend visible

Stability of stock, fortification and calibration solutions

Stability of stock, fortification and calibration solutions were not assessed since solutions were analysed within 24 hours after preparation.

Stability of final extracts/dilution

The stability of final extracts/ dilutions was not assessed since extracts/ dilutions were analysed within 24 hours after preparation

Extraction efficiency

The extraction efficiency was demonstrated following the extraction procedure reported, and already judged acceptable in the GLP study Code GLP-STUDY-21-31 performed by LabAnalysis s.r.l.: “Analytical method validation to quantify Difenoconazole and Prothioconazole-desthio in wheat, barley, oilseed rape and processed commodities”.

Conclusion

The ILV was found to be acceptable.

| Comments of zRMS: | Method is acceptable |
| --- | --- |
| Reference: | KCP 5.1.2/06 |
| Report | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole and Prothio-desthio in Whole Plant (Rapeseed) Nichetti, S. 2022 Study No. CH-1084-2021 |
| Guideline(s): | Yes. - Council Decision [C(97)186/Final] amending Annex II to the council decision concerning the mutual acceptance of data in the assessment of chemicals [C(81) 30 (final)]. - ENV/MC/CHEM(98)17 OECD Principles on Good Laboratory Practice, No. 1 and all subsequent OECD consensus documents - Directive 2004/9/CE and directive 2004/10/CE of the Suropean Parliament and of the Council of February 11th, 2004 enforced by Italian Legislative Decree No. 50 of March 2nd, 2007 as published in G.U. No. 86 of April 13th, 2007 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Summary

Confirmatory

Since the analysis by HPLC using external standards and MS triple quadrupole detector (HPLC/MS/MS) in MRM mode is highly specific and gave both quantification and identification data, a confirmatory test using another instrumental technique was not necessary.

The selectivity of the primary method was demonstrated with the obtained data from the qualifier transitions

Recovery (Accuracy) and Repeatability (Precision)

Both repeatability and recovery tests were performed using fortified Whole Plant (Rapeseed) samples, that were fortified five times at about 0.010 mg/kg (LOQ) and five times at 0.100 mg/kg (10 x LOQ), as nominal concentration. The SANTE/2020/12830 rev. 1 (2021) guideline requires any interference present in the control matrix sample(s) to be lower than 30 % of the LOQ. No interferences above the LOD were found in the control Whole Plant (Rapeseed) samples, the analysis of fortified samples at low and high level gave the following results.

According the SANTE/2020/12830 rev. 1 guideline’s requirement, the mean recovery values must be in the range 70 to 120 %, with an RSD% lower than 20%. Since all recovery values for each analyte at both fortification levels (L.O.Q and 10 x LOQ) resulted to be in the correct range, these criteria were fulfilled and therefore the analytical method can be considered suitable to quantify Difenoconazole and Prothioconazole-desthio in Whole Plant (Rapeseed) samples with an established L.O.Q of 0.010 mg/kg.

Specificity (Selectivity)

The analytical method, using the HPLC/MS/MS instrument with quantification by external standard, was shown to be specific for Difenoconazole and Prothioconazole-desthio residues in Whole Plant (Rapeseed).

Matrix effect

Not significant matrix effects for Difenoconazole and Prothioconazole-desthio residues in Whole Plant (Rapeseed) matrix were found (< ± 20%). Therefore, the calibration standards could be prepared in solvent or in matrix. A matrix-matched calibration standards were used throughout the entire study

Limit of detection (LOD) and Limit of quantification

Data from Linearity test were used to calculate the LOD whereas data from Recovery test were used to calculate the LOQ. The limit of detection (LOD), defined as the lowest working standard solution WSS 1, was a final injected solution of about 0.50 µg/L for each analyte, corresponding to about 0.002 mg/kg in Whole Plant (Rapeseed) (30% of the LOQ). The limit of quantification (LOQ), defined as the lowest fortification level with acceptable recovery and repeatability (70 – 120% with % RSD < 20%), was a final injected solution of about 2.50 µg/L for each analyte, corresponding to about 0.010 mg/kg in Whole Plant (Rapeseed).

The LOD and the LOQ was successfully established for each analyte and both mass transitions. Analyte results calculated as < 0.002 mg/kg (LOD) are classified as not detectable (n.d.). Analyte results calculated as greater than the limit of detection but less than the limit of quantification, are designated as < 0.010 mg/kg. If the analyte content is calculated as greater than 0.200 mg/kg in Whole Plant (Rapeseed), the final solution must be suitably diluted using volumetric glassware to fit in the calibration range.

Linearity in whole plant (rapeseed) matrix

Five working standard solutions (WSS) were prepared in Whole Plant (Rapeseed) matrix as described in the Experimental section and each solution was analysed by HPLC/MS/MS.

No significant memory peak was detected in the wash matrix injected after the highest working standard solution and the range tested for Difenoconazole and Prothioconazole-desthio was found to be linear (each correlation coefficient > 0.99).

Residuals for both the proposed quantifier and confirmation transition were randomly distributed around the zero line with no trend visible.

Stability of stock, fortification and calibration solutions

Stability of stock, fortification and calibration solutions were not assessed since solutions were analysed within 24 hours after preparation.

Stability of final extracts/dilution

The stability of final extracts/ dilutions was not assessed since extracts/ dilutions were analysed within 24 hours after preparation

Extraction efficiency

The extraction efficiency was demonstrated following the extraction procedure reported, and already judged acceptable in the GLP study Code GLP-STUDY-21-31 performed by LabAnalysis s.r.l.: “Analytical method validation to quantify Difenoconazole and Prothioconazole-desthio in wheat, barley, oilseed rape and processed commodities”.

Conclusion

The ILV was found to be acceptable.

        - Analytical method 2

          - Method validation

| Comments of zRMS: | Method is acceptable |
| --- | --- |

| Reference: | KCP 5.1.2/02 |
| --- | --- |
| Report | Validation of an analytical method for the quantification of Triazole Derivative Metabolites (TDMs) in wheat, barley, oilseed rape and processed commodities Longhi, D. 2021 Study No. GLP-STUDY-21-108 |
| Guidelines: | Yes. - European Commission, Guidance Document on Pesticide Analytical Methods for Risk Assessment and Postapproval Control and Monitoring Purposes, SANTE/2020/12830, Rev.1 (24/02/2021). - European Commission (2017): SANTE 2017/10632 rev. 3, dated 22 November 2017: Technical Guideline on the Evaluation of Extraction Efficiency of Residue Analytical Methods. - OECD Guidance Document on Pesticide Residue Analytical Methods ENV/JM/MONO(2007)17. - “Quick Method for the Analysis of Highly Polar Pesticides in Food Involving Extraction with Acidified Methanol and LC- or IC-MS/MS Measurement - Food of Plant Origin (QuPPe-PO-Method)- Method 8 (M8)” |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The validation of the analytical method was carried out under GLP compliance to SANTE/2020/12830 Rev.1 guideline. The analytical determination was carried out using a HPLC-MS/MS method validated in the GLP study coded GLP-STUDY-21-108.

Description of the method

Sample extraction

An appropriate aliquot of each specimen was taken from the homogenised frozen samples and put in a 50 mL screw capped centrifuge PE test tube followed by the addition of 100 μL of the internal standard solution TDM ISTD MIX (2 mg/L of each internal standard) and by the following amounts of deionized water (added on the basis of QuPPe-PO-Method and considering the theoretical water content of each matrix):

| Matrix | Matrix group | Sample weight (g) | Water added (mL) |
| --- | --- | --- | --- |
| Whole Plant (Rapeseed) | high water content commodity | 5 | 5 |
| Rapeseed seeds | high oil content commodity | 5 | 10 |
| Grain (wheat) | dry commodity | 5 | 10 |
| Straw (wheat) | dry commodity | 2.5 | 10 |
| Rapeseed oil | high oil content commodity | 5 | 10 |
| White bread (wheat) | dry commodity | 5 | 10 |
| Beer (barley) | high water content commodity | 10 | 0 |

Then, 10 mL of 1% formic acid in methanol were added and the obtained mixture was vigorously shaken for 3 minutes. The volume of the final extract is considered to be 20 mL: little variation due to the actual water content of each sample are corrected by the presence of the internal standard, that is added to produce a concentration in the final extract nominally of 10 μg/L of each compound.

The separation of the liquid phase from the solid one was achieved by centrifugation at 5000 rpm for 5 minutes. An aliquot of about 1 mL the supernatant was taken, filtered with a 0.45 μm PVDF filter and transferred in a 2 mL HPLC glass vial for the final analysis with a HPLC-DMS-MS/MS system.

Reference solutions preparation:

The following reference solutions were prepared:

| Stock solution | Preparation date (dd/mm/yyyy) | Starting material | Weight (mg) | Final volume (mL) (water) | Analyte | Actual concentration (mg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,2,4-TRZ stock solution | 25/08/2021 | 1,2,4-triazole reference material (purity: 100%) | 9.94 | 10 | 1,2,4-triazole | 994 |
| TA stock solution |  | Triazole-alanine reference material (purity: 98.3%) | 10.17 | 10 | Triazole-alanine | 999.7 |
| TLA stock solution |  | Triazole-lactic acid hydrochloride reference material (purity: 78.5%) | 15.10 | 10 | Triazole-lactic acid | 961.8* |
| TAA stock solution |  | Triazole-acetic acid reference material (purity: 95.7%) | 10.05 | 10 | Triazole-acetic acid | 961.8 |
| 1,2,4-TRZ stock solution | 27/09/2021 | 1,2,4-triazole reference material (purity: 100%) | 9.98 | 10 | 1,2,4-triazole | 998 |
| TA stock solution |  | Triazole-alanine reference material (purity: 98.3%) | 10.16 | 10 | Triazole-alanine | 998.7 |
| TLA stock solution |  | Triazole-lactic acid hydrochloride reference material (purity: 78.5%) | 15.07 | 10 | Triazole-lactic acid | 960.2* |
| TAA stock solution |  | Triazole-acetic acid reference material (purity: 95.7%) | 10.21 | 10 | Triazole-acetic acid | 977.1 |
| 1,2,4-TRZ stock solution | 11/11/2021 | 1,2,4-triazole reference material (purity: 100%) | 11.16 | 10 | 1,2,4-triazole | 1116 |
| TA stock solution |  | Triazole-alanine reference material (purity: 98.3%) | 10.68 | 10 | Triazole-alanine | 1050 |
| TLA stock solution |  | Triazole-lactic acid hydrochloride reference material (purity: 78.5%) | 14.18 | 10 | Triazole-lactic acid | 903.8* |
| TAA stock solution |  | Triazole-acetic acid reference material (purity: 95.7%) | 10.24 | 10 | Triazole-acetic acid | 980.0 |

* Concentration expressed as free acid (MW triazole lactic acid: 157.13 g/mol, MW triazole lactic acid HCl: 193.59 g/mol)

| Internal standard stock solutions | Preparation date (dd/mm/yyyy) | Starting material | Weight (mg) | Final volume (mL) (water) | Analyte | Actual concentration (mg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| TRZ ISTD stock solution | 19/08/2021 20/09/2021 | 1,2,4-triazole[13C2,15N3] reference material (purity: 98.4%) | 1.01 | 1 | 1,2,4-triazole[13C2,15N3] | 994 |
| TA ISTD stock solution |  | Triazole-alanine D2 reference material (purity: 95%) | 1.01 | 1 | Triazole-alanine D2 | 960 |
| TLA ISTD stock solution |  | Triazole[13C2,15N3] lactic acid reference material (purity: 98.42%) | 1.04 | 1 | Triazole[13C2,15N3] lactic acid | 1024 |
| TAA ISTD stock solution |  | Triazole[13C2,15N3] acetic acid reference material (purity: 98.03%) | 1.04 | 1 | Triazole[13C2,15N3] acetic acid | 1020 |

| Diluted Solution | Starting material | Volume (μL) | Final volume (mL) (water) | Analyte |  | Actual concentration (mg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| TDM Mix Solution A | 1,2,4-TRZ stock solution | 100* | 10 | 1,2,4-triazole |  | 10.0 |
|  | TA stock solution | 100* |  | Triazole-alanine |  | 10.0 |
|  | TLA stock solution | 100* |  | Triazole-lactic acid |  | 10.0 |
|  | TAA stock solution | 100* |  | Triazole-acetic acid |  | 10.0 |
| TDM Mix Solution B | TDM Mix Solution A | 1000 | 10 | 1,2,4-triazole |  | 1.00 |
|  |  |  |  | Triazole-alanine | 1.00 |  |
|  |  |  |  | Triazole-lactic acid | 1.00 |  |
|  |  |  |  | Triazole-acetic acid | 1.00 |  |
| TDM Mix Solution C | TDM Mix Solution A | 100 | 10 | 1,2,4-triazole |  | 0.100 |
|  |  |  |  | Triazole-alanine |  | 0.100 |
|  |  |  |  | Triazole-lactic acid |  | 0.100 |
|  |  |  |  | Triazole-acetic acid |  | 0.100 |
| TDM ISTD MIX | TRZ ISTD stock solution | 20 | 10 | 1,2,4-triazole[13C2,15N3] |  | 1.99 |
|  | TA ISTD stock solution | 21 |  | Triazole-alanine D2 |  | 2.02 |
|  | TLA ISTD stock solution | 20 |  | Triazole[13C2,15N3] lactic acid |  | 2.05 |
|  | TAA ISTD stock solution | 20 |  | Triazole[13C2,15N3] acetic acid |  | 2.04 |

* Depending on the actual stock solution concentration, an appropriate volume was diluted in 10 mL of water to obtain a concentration of 10 mg/L of each analyte.

Matrix-matched analytical standard solutions for whole plant (rapeseed) and rapeseed oiwere prepared from TDM Mix Solution B using the final extract of an unfortified aliquot on the basis of the following scheme:

| Matrix-matched standard solutions (for whole plant (rapeseed), rapeseed oil) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solution | μL of TDM Mix Solution B | μL of TDM ISTD MIX | Final volume (mL) | TDM concentration (each analyte) (μg/L) | ISTD concentration (each compound) (μg/L) | Nominal concentration on the sample1 (mg/kg) |
| L1 | 1 | 10 | 2 | 0.5 | 10 | 0.002 |
| L2 | 2.5 | 5 | 1 | 2.5 | 10 | 0.010 |
| L3 | 5 (or 10)2 | 5 | 1 | 5 (or 10)2 | 10 | 0.020 (or 0.040) 2 |
| L4 | 25 | 5 | 1 | 25 | 10 | 0.100 |
| L5 | 50 | 5 | 1 | 50 | 10 | 0.200 |

1: Calculated considering the nominal sample preparation (5.00 g to a final volume of 20.0 mL)

2: L3 with a different concentration was used for rapeseed oil. This level can be indifferently of 5 or 10 μg/L since its concentration has no impact on the extension of the calibration range

Analytical standard solutions for rapeseed seeds, grain (wheat), white bread (wheat) were prepared in solvent (water / 1% formic acid in methanol 50:50) on the basis of the following scheme:

| Standard solutions in solvent (for rapeseed seeds, grain (wheat), white bread (wheat)) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solution | μL of TDM Mix Solution B | μL of TDM ISTD MIX | Final volume (mL) | TDM concentration (each analyte) (μg/L) | ISTD concentration (each compound) (μg/L) | Nominal concentration on the sample1 (mg/kg) |
| L1 | 1 | 10 | 2 | 0.5 | 10 | 0.002 |
| L2 | 2.5 | 5 | 1 | 2.5 | 10 | 0.010 |
| L3 | 10 | 5 | 1 | 10 | 10 | 0.040 |
| L4 | 25 | 5 | 1 | 25 | 10 | 0.100 |
| L5 | 50 | 5 | 1 | 50 | 10 | 0.200 |

1: Calculated considering the nominal sample preparation (5.0 g to a final volume of 20.0 mL)

| Standard solutions in solvent (for beer(barley)) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solution | μL of TDM Mix Solution B | μL of TDM ISTD MIX | Final volume (mL) | TDM concentration (each analyte) (μg/L) | ISTD concentration (each compound) (μg/L) | Nominal concentration on the sample1 (mg/kg) |
| L1 | 1 | 5 | 1 | 1 | 10 | 0.002 |
| L2 | 5 | 5 | 1 | 5 | 10 | 0.010 |
| L3 | 10 | 5 | 1 | 10 | 10 | 0.020 |
| L4 | 50 | 5 | 1 | 50 | 10 | 0.100 |
| L5 | 100 | 5 | 1 | 100 | 10 | 0.200 |

1: Calculated considering the nominal sample preparation (10.0 g to a final volume of 20.0 mL)

| Standard solutions in solvent (for straw (wheat)) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solution | μL | μL of TDM ISTD MIX | Final volume (mL) | TDM concentration (each analyte) (μg/L) | ISTD concentration (each compound) (μg/L) | Nominal concentration on the sample1 (mg/kg) |
| L1 | 3.5 of TDM Mix Solution C | 5 | 1 | 0.35 | 10 | 0.0028 |
| L2 | 1 of TDM Mix Solution B | 5 | 1 | 1.00 | 10 | 0.0080 |
| L3 | 5 of TDM Mix Solution B | 5 | 1 | 5.00 | 10 | 0.040 |
| L4 | 1 of TDM Mix Solution A | 5 | 1 | 10.0 | 10 | 0.080 |
| L5 | 3.5 of TDM Mix Solution A | 5 | 1 | 35.0 | 10 | 0.280 |

1: Calculated considering the nominal sample preparation (2.5 g to a final volume of 20.0 mL)

Furthermore, the linearity in solvent in a more concentrated calibration range was verified (from 10 to 1000 μg/L). The preparation of the working standard solution used for this purpose is reported in the following table:

| Standard solutions in solvent |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solution | μL of TDM Mix Solution A | μL of TDM ISTD MIX | Final volume (mL) | TDM concentration (each analyte) (μg/L) | ISTD concentration (each compound) (μg/L) | Nominal concentration on the sample1 (mg/kg) |
| L1 | 1 | 5 | 1 | 10 | 10 | 0.040 |
| L2 | 5 | 5 | 1 | 50 | 10 | 0.200 |
| L3 | 10 | 5 | 1 | 100 | 10 | 0.400 |
| L4 | 50 | 5 | 1 | 500 | 10 | 2.00 |
| L5 | 100 | 5 | 1 | 1000 | 10 | 4.00 |

1: Calculated considering a generic nominal sample preparation of 5.0 g to a final volume of 20.0 mL

The analyses were carried out using a HPLC-MS/MS system according to the following conditions:

Instrument: 	HPLC Shimadzu LC-40 XR + Sciex API 6500 + equipped with SelexION+ (Differential Ion Mobility Device)

Column: 		Thermo Hypercarb 5 μm, 2.1 x 100 mm

Column temperature: 	40°C

Flow: 			0.6 mL/min

Injection volume: 	2 μL

Mobile phase A: 	LC-MS grade water with 1 % acetic acid

Mobile phase B: 	LC-MS grade methanol with 1 % acetic acid

Elution: 		gradient of the following composition:

| Time (min) | % A | % B |
| --- | --- | --- |
| 0 | 95 | 5 |
| 5 | 10 | 90 |
| 6 | 10 | 90 |
| 6.1 | 95 | 5 |

Stop time: 		10 min

Source type: 		ESI

Curtain gas flow: 	30 mL/min

Gas 1: 			55 mL/min

Gas 2: 			65 mL/min

Gas temperature: 	500°C

Capillary: 		positive mode 3500 V

Acquiring mode: 	ESI positive, MRM (multi-reaction monitoring).

Calibration

The quantification of each analyte was made through the building of a calibration straight line with the internal standard method. 5 analytical standard solutions for each analysed matrix were analysed in single injections in order to obtain a calibration curve (1/x weighed) interpolated with a linear regression.

Recovery (Accuracy) and Repeatability (Precision)

Recovery and repeatability (as precision, % RSD) data will be reported for the following fortification levels:

- LOQ level (5 replicates): 0.01 mg/kg

- 10xLOQ level (5 replicates): 0.1 mg/kg - or alternatively at a spiking level higher than maximum residue level found on field specimens.

Specificity (Selectivity)

This parameter will be evaluated in order to demonstrate that the applied method detects the right analyte and that the analytical signal is quantitatively correct and not affected by other analytes or by matrix interferences.

Using a differential ion mobility device and a MS/MS mass spectrometer detector the selectivity was evaluated comparing the following chromatograms: samples, fortified samples, reference solutions at the LOQ level, in order to assess the presence or absence of interfering signals. Blank values (non-fortified samples) will be determined from the matrices used in fortification experiments and should not be higher than 30% of the LOQ.

Matrix effect

Assessment of matrix effects will be performed by comparing the analyte response of at least one individual standard prepared in solvent to at least one prepared in blank matrix, for all sample matrix used in the study. In order to nullify matrix effect, calibration curves for all matrices analysed will be prepared using matrix matched analytical standards.

Stability of final extracts and standard

The samples extracts will be preferably injected the same day of the extraction, if not analysed immediately they will be stored at 5 ± 3°C in dark conditions.

Limit of detection (LOD)

The limit of detection (LOD) is defined as the lowest detectable concentration or amount of an analyte in a sample. It is the lowest concentration at which an analyte produces an instrumental signal at least 3 times higher than background noise of the chromatogram. It should be not higher than 30% of LOQ value and it will be considered as the lowest point of the instrumental calibration.

Limit of quantification (LOQ)

Limit of quantitation (LOQ) is defined as the lowest validated level with sufficient recovery and precision. Target LOQ for Triazole Derivative Metabolites (TDMs) in the sample analysed will be set to 0.01 mg/kg.

Confirmation:

A simultaneous confirmation to the primary detection was used using the HPLC-MS/MS, monitoring an additional SRM transition. The following data will be provided for the additional ion: calibration data as recorded for primary detection, recovery and precision data as recorded for primary detection (at least for the 5 replicates at LOQ level) the recovery and precision mean values calculated on confirmatory ion must fulfill the same acceptable range reported above for primary detection.

Results and discussions

Method validation data can be summarised in the tables below.

Table A 6:	Recovery results from method validation of 1,2,4-triazole (1,2,4-TRZ) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | SD | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Whole plant (rapeseed) | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 87.0 | 6.2 | 7.2 |  |
|  |  | 0.1 (10xLOQ) | 98.7 | 4.0 | 4.1 |  |
| Rapeseed seeds | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 99.0 | 2.3 | 2.3 |  |
|  |  | 0.1 (10xLOQ) | 93.9 | 4.0 | 4.3 |  |
| Wheat grain | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 87.5 | 7.5 | 8.6 |  |
|  |  | 0.1 (10xLOQ) | 97.7 | 5.3 | 5.4 |  |
| Straw (wheat) | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 93.9 | 6.2 | 6.6 |  |
|  |  | 0.1 (10xLOQ) | 95.6 | 2.1 | 2.2 |  |
| Rapeseed oil | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 94.2 | 7.9 | 8.4 |  |
|  |  | 0.1 (10xLOQ) | 98.4 | 2.4 | 2.5 |  |
| White bread (wheat) | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 97.7 | 5.3 | 5.5 |  |
|  |  | 0.1 (10xLOQ) | 98.7 | 3.4 | 3.4 |  |
| Beer (barley) | 1,2,4-triazole (1,2,4-TRZ) | 0.01(LOQ) | 102.6 | 6.3 | 6.2 |  |
|  |  | 0.1 (10xLOQ) | 100.6 | 3.1 | 3.1 |  |

Table A 7:	Recovery results from method validation of Triazole-alanine (TA) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | SD | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Whole plant (rapeseed) | Triazole-alanine (TA) | 0.01(LOQ) | 98.7 | 2.9 | 2.9 |  |
|  |  | 0.1 (10xLOQ) | 99.5 | 2.3 | 2.3 |  |
| Rapeseed seeds | Triazole-alanine (TA) | 0.01(LOQ) | 102.0 | 4.7 | 4.6 |  |
|  |  | 0.1 (10xLOQ) | 112.0 | 1.8 | 1.6 |  |
| Wheat grain | Triazole-alanine (TA) | 0.01(LOQ) | 91.2 | 5.7 | 6.3 |  |
|  |  | 0.1 (10xLOQ) | 106.5 | 1.6 | 1.5 |  |
| Straw (wheat) | Triazole-alanine (TA) | 0.01(LOQ) | 100.0 | 5.2 | 5.2 |  |
|  |  | 0.1 (10xLOQ) | 99.0 | 3.8 | 3.8 |  |
| Rapeseed oil | Triazole-alanine (TA) | 0.01(LOQ) | 100.6 | 1.5 | 1.5 |  |
|  |  | 0.1 (10xLOQ) | 100.0 | 1.5 | 1.5 |  |
| White bread (wheat) | Triazole-alanine (TA) | 0.01(LOQ) | 100.3 | 2.5 | 2.5 |  |
|  |  | 0.1 (10xLOQ) | 100.7 | 3.4 | 3.3 |  |
| Beer (barley) | Triazole-alanine (TA) | 0.01(LOQ) | 104.1 | 13.0 | 12.5 |  |
|  |  | 0.1 (10xLOQ) | 109.4 | 5.2 | 4.8 |  |

Table A 8:	Recovery results from method validation of Triazole-lactic acid (TLA) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | SD | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Whole plant (rapeseed) | Triazole-lactic acid (TLA) | 0.01(LOQ) | 101.3 | 1.8 | 1.8 |  |
|  |  | 0.1 (10xLOQ) | 99.8 | 1.5 | 1.5 |  |
| Rapeseed seeds | Triazole-lactic acid (TLA) | 0.01(LOQ) | 113.0 | 1.7 | 1.5 |  |
|  |  | 0.1 (10xLOQ) | 106.6 | 0.9 | 0.8 |  |
| Wheat grain | Triazole-lactic acid (TLA) | 0.01(LOQ) | 104.2 | 3.6 | 3.4 |  |
|  |  | 0.1 (10xLOQ) | 104.6 | 2.0 | 1.9 |  |
| Straw (wheat) | Triazole-lactic acid (TLA) | 0.01(LOQ) | 99.5 | 4.8 | 4.8 |  |
|  |  | 0.1 (10xLOQ) | 99.7 | 1.2 | 1.2 |  |
| Rapeseed oil | Triazole-lactic acid (TLA) | 0.01(LOQ) | 100.2 | 2.4 | 2.4 |  |
|  |  | 0.1 (10xLOQ) | 97.7 | 1.2 | 1.2 |  |
| White bread (wheat) | Triazole-lactic acid (TLA) | 0.01(LOQ) | 99.9 | 3.8 | 3.8 |  |
|  |  | 0.1 (10xLOQ) | 100.3 | 1.0 | 1.0 |  |
| Beer (barley) | Triazole-lactic acid (TLA) | 0.01(LOQ) | 97.6 | 3.0 | 3.1 |  |
|  |  | 0.1 (10xLOQ) | 97.5 | 3.7 | 3.8 |  |

Table A 9:	Recovery results from method validation of Triazole-acetic acid (TAA) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | SD | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Whole plant (rapeseed) | Triazole-acetic acid (TAA) | 0.01(LOQ) | 100.8 | 1.7 | 1.7 |  |
|  |  | 0.1 (10xLOQ) | 100.0 | 1.3 | 1.3 |  |
| Rapeseed seeds | Triazole-acetic acid (TAA) | 0.01(LOQ) | 102.9 | 3.3 | 3.2 |  |
|  |  | 0.1 (10xLOQ) | 99.1 | 1.3 | 1.3 |  |
| Wheat grain | Triazole-acetic acid (TAA) | 0.01(LOQ) | 100.0 | 4.2 | 4.2 |  |
|  |  | 0.1 (10xLOQ) | 101.9 | 2.2 | 2.1 |  |
| Straw (wheat) | Triazole-acetic acid (TAA) | 0.01(LOQ) | 98.9 | 6.6 | 6.7 |  |
|  |  | 0.1 (10xLOQ) | 102.2 | 1.0 | 1.0 |  |
| Rapeseed oil | Triazole-acetic acid (TAA) | 0.01(LOQ) | 103.0 | 1.7 | 1.6 |  |
|  |  | 0.1 (10xLOQ) | 99.4 | 0.8 | 0.8 |  |
| White bread (wheat) | Triazole-acetic acid (TAA) | 0.01(LOQ) | 101.1 | 2.5 | 2.5 |  |
|  |  | 0.1 (10xLOQ) | 98.5 | 0.9 | 0.9 |  |
| Beer (barley) | Triazole-acetic acid (TAA) | 0.01(LOQ) | 100.1 | 17.5 | 17.5 |  |
|  |  | 0.1 (10xLOQ) | 103.1 | 1.6 | 1.6 |  |

Table A 10:	Characteristics for the analytical method used for validation of TDMs residues in wheat, barley, oilseed rape and processed commodities

|  | 1,2,4-triazole (1,2,4-TRZ) | Triazole alanine (TA) | Triazole lactic acid (TLA) | Triazole acetic acid (TAA) |
| --- | --- | --- | --- | --- |
| Specificity | MS sppectrum provided: Yes m/z 70.1 – 43.1 m/z 70.1 – 70.0 | MS sppectrum provided: Yes m/z 157.0 – 70.0 m/z 157.0 – 88.0 | MS sppectrum provided: Yes m/z 158.0 – 70.0 m/z 158.0 – 43.0 | MS sppectrum provided: Yes m/z 128.0 – 70 m/z 128.0 – 73.0 |
| Calibration (type, number of data points) | Whole Plant (Rapeseed): n = 5 y= 1.03477x + 0.07821 R2 = 0.999 | Whole Plant (Rapeseed): n = 5 y= 0.51285x + 0.02730 R2 = 0.999 | Whole Plant (Rapeseed): n = 5 y= 1.20823 x + 0.02456 R2 = 0.999 | Whole Plant (Rapeseed): n = 5 y= 0.98409x + 0.01569 R2 = 0.999 |
|  | Rapeseed oil: n = 5 y = 0.94482x + 0.03984 R2 = 0.999 | Rapeseed oil: y = 2.37172x + 0.01580 R2 = 0.999 | Rapeseed oil: n = 5 y = 1.22940 x + -0.00528 R2 = 0.999 | Rapeseed oil: y = 0.098796x + 1.07526e-4 R2 = 0.999 |
|  | Rapeseed seeds/Grain (wheat)/White bread (wheat): n = 5 y = 0.89935x + 0.04423 R2 = 0.999 | Rapeseed seeds/Grain(wheat)/ White bread (wheat): n = 5 y = 2.76713 x + 0.02212 R2 = 0.999 | Rapeseed seeds/Grain(wheat)/ White bread (wheat): n = 5 y = 1.02675x +-9.58870e-4 R2 = 0.999 | Rapeseed seeds/Grain(wheat)/ White bread (wheat): n = 5 y = 1.00664 x + 0.00803 R2 = 0.999 |
|  | Straw (wheat) n = 5 y = 0.97019 x + 0.00769 R2 = 0.999 | Straw (wheat) n = 5 y = 5.15827 x + 0.09160 R2 = 0.999 | Straw (wheat) n = 5 y = 1.19902 x + 0.01208 R2 = 0.999 | Straw (wheat) n = 5 y = 1.03861 x + 4.1612e-4 R2 = 0.999 |
|  | Beer (barley) n = 5 y = 0.94954 x + 0.00154 R2 = 0.999 | Beer (barley) n = 5 y = 3.83597 x + 0.16176 R2 = 0.999 | Beer (barley) n = 5 y = 1.31066 x + 0.07797 R2 = 0.999 | Beer (barley) n = 5 y = 0.95844 x + 0.01940 R2 = 0.999 |
| Calibration range | Range: 0.50 – 50.0 µg/L (0.00200 – 0.200mg/kg) Beer range: 1.00 – 100 µg/L (0.0020 – 0.200 mg/kg) Straw (wheat) range: 0.35 – 35.0µg/L (0.0028 – 0.280mg/kg) | Range: 0.50 – 50.0 µg/L (0.00200 – 0.200mg/kg) Beer range: 1.00 – 100 µg/L (0.0020 – 0.200 mg/kg) Straw (wheat) range: 0.35 – 35.0µg/L (0.0028 – 0.280mg/kg) | Range: 0.50 – 50.0 µg/L (0.00200 – 0.200mg/kg) Beer range: 1.00 – 100 µg/L (0.0020 – 0.200 mg/kg) Straw (wheat) range: 0.35 – 35.0µg/L (0.0028 – 0.280mg/kg) | Range: 0.50 – 50.0 µg/L (0.00200 – 0.200mg/kg) Beer range: 1.00 – 100 µg/L (0.0020 – 0.200 mg/kg) Straw (wheat) range: 0.35 – 35.0µg/L (0.0028 – 0.280mg/kg) |
| Assessment of matrix effects is presented | Yes Except for triazole-alanine in whole plant, the matrix effect is considered not significant according to the SANTE/2020/12830 rev.1 guideline, since the analytes responses in matrix matched calibration solutions are < ± 20% of the analytes responses in the calibration solutions prepared in solvent (water/methanol 50:50 + 1% formic acid). Since the matrix effect is negligible, the calibration can also be done using standards prepared in solvent. |  |  |  |
| Limit of determination/quantification | LOD: 0.50 µg/L LOD: 1 µg/L for beer LOD for wheat: 0.35 µg/L LOQ: 0.01 mg/kg | LOD: 0.50 µg/L LOD: 1 µg/L for beer LOD for wheat: 0.35 µg/L LOQ: 0.01 mg/kg | LOD: 0.50 µg/L LOD: 1 µg/L for beer LOD for wheat: 0.35 µg/L LOQ: 0.01 mg/kg | LOD: 0.50 µg/L LOD: 1 µg/L for beer LOD for wheat: 0.35 µg/L LOQ: 0.01 mg/kg |

Conclusion

A mean recovery of 60-120% with a Relative Standard Deviation  ≤ 30% was adopted as acceptability criteria.

The results obtained concerning matrix effects, linearity, selectivity, accuracy (recovery), precision (repeatability), specificity, limit of quantification and limit of detection are in compliance with requirements reported in guideline SANTE/2020/12830 rev. 1 for the analyte.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted

      - Other Studies/ Information

No new or additional studies have been submitted

        - ILV Analytical method 1 (Honey)

          - Method Validation 1

| Comments of zRMS: | Method is acceptable |
| --- | --- |

| Reference: | KCP 5.1.2/03 |
| --- | --- |
| Report | Analytical Method for the Determination of Difenoconazole, Prothioconazole, Prothioconazole-desthio and Triazole Derivative Metabolites (TDMs) residue in Honey Ivo Rovetto. 2023 Study code. 1111.4F.SAG23 |
| Guidelines: | Yes. • European Community Guidelines 7029/VI/95 – Rev. 5, 22/07/97: General recommendations for the design, preparation and realization of residue trials. • Regulation (EC) no. 1107/2009 of the European Parliament and of the Council of 21st October 2009 concerning the placing of plant protection products on the market and repealing Council Directives 79/117/EC and 91/414/EEC, 21/10/2019 • Regulation (EU) No. 283/2013 setting out the data requirements for active substances,in accordance with Regulation (EC) No 1107/2009 of the European Parliament and of the Council concerning the placing of plant protection products on the market, 01/03/2013 • OECD Test Guideline 509: Crop field trial • European Community Guidelines SANTE/2019/12752 on data requirements for setting maximum residue levels, comparability of residue trials and extrapolation of residue data on products from plant and animal origin (Repealing and replacing the existing Guidance Document SANCO 7525/VI/95 Rev. 10.3) • SANTE/11956/2016 rev. 9, Technical guidelines for determining the magnitude of pesticide residues in honey and setting Maximum Residue Levels in honey, 14 September 2018. • SANTE/2020/12830, Rev.2 14 February 2023, Guidance Document on Pesticide Analytical Methods for Risk Assessment and Post-approval Control and Monitoring Purposes. • Guidance document on pesticide residue analytical methods [ENV/JM/MONO(2007)17]. • GLP compliance monitoring of international multi-site studies, Ref. Ares(2017)75658. • Directive 2004/10/EC of the European Parliament and of the Council (L 50/44). |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Principle of the test

The validation of the analytical method was carried out under GLP compliance to SANTE/11956/2016 rev. 9 (Appendix V: tunnel residue trials for MRL setting in Honey). The analyses were carried out according to the analytical method for the determination of difenoconazole, prothioconazole and prothioconazole-desthio and according to the analytical method for the determination of TDMs (triazole-derivative metabolites) validated under GLP compliance according to SANTE/12830/2020 rev. 2 guideline.

Description of the method

Determination of difenoconazole, prothioconazole and prothioconazole-desthio:

The analytical method to quantify Difenoconazole, Prothioconazole and Prothioconazole-desthio in honey

was based on the QuEChERS method (EN 15662_2018). The instrumental determination was carried out

using a HPLC-MS/MS system (high-performance liquid chromatography + triple quadrupole mass

spectrometry).

Sample preparation:

Aliquots of 5 g of specimen were taken from the frozen samples and put in a 50 mL screw capped centrifuge PE test tube (aliquots for recovery evaluation were spiked at this stage), followed by the addition of 5 mL of an aqueous solution of 0.1 g/L of cysteine hydrochloride (to prevent the degradation of Prothioconazole) and 0.1% formic acid.

Then, 20 mL of acetonitrile were added and the obtained mixture was vigorously shaken for one minute. After that, a packet of unbuffered QuEChERS extraction salt (4.0 g MgSO4, 1.0 g NaCl) was added and the mixture shaking again. The separation of the organic phase was achieved by centrifugation at 5000 rpm for 5 minutes.  Finally, an aliquot of the organic supernatant was filtered, transferred in a 2 mL HPLC glass vial and analysed with a HPLC-MS/MS system.

Reference solutions preparation:

The following reference solutions were prepared:

| Solution | Starting material | Weight /Volume | Final volume (mL) (acetonitrile) | Analyte | Actual Conc. (mg/L)* | Preparation date (dd/mm/ yyyy) | Expiry date (dd/mm/ yyyy)** |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Difenoconazole stock solution (Difenoconazole ~ 1000 mg/L) | Difenoconazole reference material (purity: 95.4%) | 8.52 mg | 10 | Difenoconazole | 813 | 14/09/2023 | 19/12/2023 |
| Prothioconazole stock solution (Prothioconazole ~ 1000 mg/L) | Prothioconazole Reference material (purity: 98.2 %) | 6.63 mg | 10 | Prothioconazole | 651 | 14/09/2023 | 08/11/2023 |
| Prothioconazole-desthio stock solution (Prothioconazole- desthio ~ 1000 mg/L) | Prothioconazole-desthio Reference material (purity: 98.3 %) | 5.75 mg | 10 | Prothioconazoledesthio | 565 | 14/09/2023 | 19/12/2023 |
| Solution A (~ 10 mg/L each analyte) | Difenoconazole stock solution | 123 µL | 10 | Difenoconazole | 10.0 | Each Analytical Session | Daily |
|  | Prothioconazole stock solution | 154 µL |  | Prothioconazole | 10.0 |  |  |
|  | Prothioconazole- desthio stock solution | 177 µL |  | Prothioconazoledesthio | 10.0 |  |  |
| Solution B (~ 1 mg/L each analyte) | Solution A | 1000 µL | 10 | Difenoconazole | 1.00 |  |  |
|  |  |  |  | Prothioconazole | 1.00 |  |  |
|  |  |  |  | Prothioconazoledesthio | 1.00 |  |  |

* Values corrected for the purity

** For stability data of stock solutions refer to the validation study LBN-0092-2023 (stability of 96 days from the preparation for Difenoconazole and Prothioconazole-desthio, 55 days for Prothioconazole)

Matrix-matched standard solutions:

matrix-matched analytical standard solutions were prepared at each analytical session from Solution B and A using the final extract of an unfortified sample extract on the basis of the following scheme:

|  | Matrix-matched standard solutions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solution | µL of solution |  | Final volume (mL) | Concentration in solution (each analyte) (µg/L) | Nominal concentration on sample (each analyte)1 (mg/kg) |
| L1 | 1.0 | Solution B | 2 | 0.500 | 0.0020 |
| L2 | 2.5 | Solution B | 1 | 2.50 | 0.0100 |
| L3 | 5.0 | Solution B | 1 | 5.00 | 0.020 |
| L4 | 2.5 | Solution A | 1 | 25.0 | 0.100 |
| L5 | 5.0 | Solution A | 1 | 50.0 | 0.200 |

1 Calculated considering the nominal sample preparation (5 g to a final volume of 20 mL)

Instrumental determination

The analysis of Difenoconazole, Prothioconazole and Prothioconazole-desthio was carried out using a HPLCMS/MS system operating in the following conditions:

Instrument: HPLC Agilent 1290 Infinity II coupled with a triple quadrupole mass spectrometer Agilent 6470A

Column: Phenomenex Kinetex C18, 1.7 µm, 2.1 x 50 mm

Column temperature: 40°C

Flow: 0.6 mL/min

Injection volume: 2.5 µL

Mobile phase A: LC-MS grade water with 0.2 % formic acid and 5 mM ammonium formate - 	Mobile phase B: LC-MS grade methanol with 0.2 % formic acid and 5 mM ammonium formate - 	Elution: gradient of the following composition:

| Time (min) | % A | % B |
| --- | --- | --- |
| 0 | 70 | 30 |
| 0.5 | 70 | 30 |
| 3.0 | 0 | 100 |

Stop time: 5 min

Post time: 1 min

Divert valve: 0 min. to waste, 2 min. to MS, 3.5 min. to waste

Source type: ESI

Gas temperature: 350°C

Gas flow (L/min): 5

Nebulizer (psi): 40

Sheath gas heater: 400°C

Sheath gas flow (L/min): 12

Capillary: positive mode 3500 V, negative mode 3000 V

Vcharging: 0

Acquiring mode: ESI positive and ESI negative, MRM (multi-reaction monitoring).

|  |  | MRM monitored transitions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analyte | Retention time (approx, min) | Detection | Polarity | Precursor ion (m/z) | Product ion (m/z) | Dwell (mS) | Fragmentor (V) | Collision energy (eV) |
| Difenoconazole | 2.9 | Primary | Positive | 406.2 | 251.1 | 50 | 115 | 22 |
|  |  | Confirmatory |  | 406.2 | 188.4 | 50 |  | 54 |
| Prothioconazole- desthio | 2.6 | Primary | Positive | 312.2 | 125 | 50 | 100 | 38 |
|  |  | Confirmatory |  | 312.2 | 69.8 | 50 |  | 26 |
| Prothioconazole | 2.8 | Primary | Negative | 342 | 100 | 50 | 115 | 25 |
|  |  | Confirmatory |  | 342 | 264 | 50 |  | 21 |

Calibration:

The quantification of each analyte was made through the building of a calibration straight line with the external standard method. 5 matrix-matched analytical standard solutions were analysed in single injections in order to obtain a calibration curve (1/x weighed) interpolated with a linear regression.

Recovery (Accuracy)

During the analytical session for the determination of Difenoconazole, Prothioconazole and Prothioconazole-desthio, two recovery check tests were carried out fortifying aliquots of an untreated honey sample (CDS23-1469 and CDS-23-1565) at the following fortification levels:

- LOQ (0.01 mg/kg), adding 50 µL of Solution B (containing 1.00 mg/L of each analyte) to aliquots of about 5 g of untreated sample.

- 10xLOQ (0.1 mg/kg), adding 50 µL of Solution A (containing 10.0 mg/L of each analyte) to aliquots of about 5 g of untreated sample.

Procedural recoveries were handled and stored in the same way and for the same time period as the analytical specimens that have been prepared within the same analytical set. Procedural recovery values were evaluated against the SANTE 2020/12830 rev.2 requirements

Determination of TDM

Principle of the test:

The applied analytical method (AM-LBN-0093-2023) allows the determination of the following TDM (triazole derivative metabolites):

-	1,2,4-triazole (TRZ)

-	Triazole-alanine (TA)

-	Triazole-lactic acid (TLA)

-	Triazole-acetic acid (TAA) in honey.

The analytical method was based on the method “Quick Method for the Analysis of Highly Polar Pesticides in Food Involving Extraction with Acidified Methanol and LC- or IC-MS/MS Measurement - Food of Plant Origin (QuPPe-PO-Method) - Method 8 (M8)”. The instrumental determination was carried out using a HPLC-MS/MS system (high-performance liquid chromatography + triple quadrupole mass spectrometry) equipped with a differential mobility separation device (DMS).

Sample preparation:

An aliquot of about 5 g of honey was taken from the frozen sample and put in a 50 mL screw capped centrifuge PE test tube (aliquots for recovery evaluation were spiked at this stage) followed by the addition of 100 μL of the internal standard solution TDM ISTD MIX (2 mg/L of each internal standard) and of 7.5 mL of deionized water. After a vigorous shake, 10 mL of 1% formic acid in methanol were added and the obtained mixture was shaken for about one minute. An aliquot of about 1 mL the supernatant was taken, filtered with a 0.45 μm PVDF filter and transferred in a 2 mL HPLC glass vial for the final analysis with a HPLC-DMS-MS/MS system.

Stock standard

| Solution | Starting material | Weight (mg) | Final volume (mL) (water) | Analyte | Actual concentration (mg/L)* | Preparation date (dd/mm/ yyyy) | Expiry date (dd/mm/ yyyy)*** |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2,4-TRZ stock solution | 1,2,4-triazole reference material (purity: 99.7%) | 10.05 | 10 | 1,2,4-triazole | 1002 | 14/09/2023 | 01/12/2023 |
| TA stock solution | Triazole-alanine reference material (purity: 98.3%) | 10.63 | 10 | Triazole-alanine | 1045 | 14/09/2023 | 01/12/2023 |
| TLA stock solution | Triazole-lactic acid hydrochloride reference material (purity: 78.5%) | 11.73 | 10 | Triazole-lactic acid | 747.4** | 14/09/2023 | 01/12/2023 |
| TAA stock solution | Triazole-acetic acid reference material (purity: 97%) | 10.75 | 10 | Triazole-acetic acid | 1043 | 14/09/2023 | 01/12/2023 |

Internal standard stock solutions

| Solution | Starting material | Weight (mg) | Final volume (mL) (water) | Analyte | Actual concentration (mg/L)* | Preparation date (dd/mm/ yyyy) | Expiry date (dd/mm/ yyyy)** |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TRZ ISTD stock solution | 1,2,4-triazole[13C2,15N3] reference material (purity: 98.4%) | 1.31 | 1 | 1,2,4triazole[13C2,15N3] | 1289 | 14/09/2023 | 01/12/2023 |
| TA ISTD stock solution | Triazole-alanine D2 homogeneous solution in water/methanol 3:1 v/v | / | / | Triazole-alanine D2 | 999.68 | / | 14/08/2024 |
| TLA ISTD stock solution | Triazole[13C2,15N3] lactic acid reference material (purity: 98.42%) | 1.46 | 1 | Triazole[13C2,15N3] lactic acid | 1437 | 14/09/2023 | 01/12/2023 |
| TAA ISTD stock solution | Triazole[13C2,15N3] acetic acid reference material (purity: 98.03%) | 1.04 | 1 | Triazole[13C2,15N3] acetic acid | 1020 | 14/09/2023 | 01/12/2023 |

Diluted solutions

| Solution | Starting material | Volume (µL) | Final volume (mL) (water) | Analyte | Actual concentration (mg/L) | Preparation date (dd/mm/ yyyy) | Expiry date (dd/mm/ yyyy)** |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDM Mix Solution A | 1,2,4-TRZ stock solution | 100 | 10 | 1,2,4-triazole | 10.0 | Each Analytical Session | Daily |
|  | TA stock solution | 96 |  | Triazole-alanine | 10.0 |  |  |
|  | TLA stock solution | 134 |  | Triazole-lactic acid | 10.0 |  |  |
|  | TAA stock solution | 96 |  | Triazole-acetic acid | 10.0 |  |  |
| TDM Mix Solution B | TDM Mix Solution A | 1000 | 10 | 1,2,4-triazole | 1.00 |  |  |
|  |  |  |  | Triazole-alanine | 1.00 |  |  |
|  |  |  |  | Triazole-lactic acid | 1.00 |  |  |
|  |  |  |  | Triazole-acetic acid | 1.00 |  |  |
| TDM ISTD MIX | TRZ ISTD stock solution | 18.1 | 10 | 1,2,4-triazole[13C2,15N3] | 2.00 |  |  |
|  | TA ISTD stock solution | 20 |  | Triazole-alanine D2 | 2.00 |  |  |
|  | TLA ISTD stock solution | 13.7 |  | Triazole[13C2,15N3] lactic acid | 2.00 |  |  |
|  | TAA ISTD stock solution | 16.3 |  | Triazole[13C2,15N3] acetic acid | 2.00 |  |  |

Working standard solutions

|  |  |  | Working standard solutions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solution | µL of TDM Mix Solution B | µL of TDM ISTD MIX | Final volume (mL) | TDM concentration (each analyte) (µg/L) | ISTD concentration (each compound) (µg/L) | Nominal concentration on the sample1 (mg/kg) |
| L1 | 1 | 10 | 2 | 0.5 | 10 | 0.002 |
| L2 | 2.5 | 5 | 1 | 2.5 | 10 | 0.010 |
| L3 | 10 | 5 | 1 | 10 | 10 | 0.040 |
| L4 | 25 | 5 | 1 | 25 | 10 | 0.100 |
| L5 | 50 | 5 | 1 | 50 | 10 | 0.200 |

Instrumental conditions

The analyses were carried out using a HPLC-DMS-MS/MS system according to the following conditions:

Instrument: HPLC Shimadzu LC-40 XR + Sciex API 6500 + equipped with SelexION+ (Differential Ion Mobility Device)

Column: Thermo Hypercarb 5 µm, 2.1 x 100 mm

Column temperature: 40°C

Flow: 0.6 mL/min

Injection volume: 2 µL

Mobile phase A: LC-MS grade water with 1 % acetic acid - 	Mobile phase B: LC-MS grade methanol with 1 % acetic acid - 	Elution: gradient of the following composition:

| Time (min) | % A | % B |
| --- | --- | --- |
| 0 | 100 | 0 |
| 6.00 | 10 | 90 |
| 7.00 | 10 | 90 |
| 7.10 | 100 | 0 |

Stop time: 10 min

Source type: ESI

Curtain gas flow: 30 mL/min

Gas 1: 55 mL/min

Gas 2: 65 mL/min

Gas temperature: 500°C

Capillary: positive mode 3500 V

Acquiring mode: ESI positive, MRM (multi-reaction monitoring).

Transitions: the optimised MS/MS fragmentation parameters are reported in the table below:

MRM monitored transitions:

| Compound | Rt (min) | Detection | Associated internal standard | Precursor ion (m/z) | Product ion (m/z) |
| --- | --- | --- | --- | --- | --- |
| 1,2,4-triazole (1,2,4-TRZ) | 1.0 | Primary | TRZ ISTD | 70.1 | 43.1 |
|  |  | Confirmatory | TRZ ISTD | 70.1 | 70.0 |
| Triazole alanine (TA) | 1.05 | Primary | TAL ISTD | 157.0 | 70.0 |
|  |  | Confirmatory | TAL ISTD | 157.0 | 88.0 |
| Triazole lactic acid (TLA) | 3.4 | Primary | TLA ISTD | 158.0 | 70.0 |
|  |  | Confirmatory | TLA ISTD | 158.0 | 43.0 |
| Triazole acetic acid (TAA) | 3.8 | Primary | TAA ISTD | 128.0 | 70.0 |
|  |  | Confirmatory | TAA ISTD | 128.0 | 73.0 |
| 1,2,4-Triazole- [13C2,15N3] | 1.0 | Primary | - | 75.0 | 46.0 |
| 1,2,4-Triazole Alanine [D2] | 1.05 | Primary | - | 159.0 | 72.0 |
| 1,2,4-Triazole- [13C2, 15N3] Lactic Acid | 3.4 | Primary | - | 163.0 | 75.0 |
| 1,2,4-Triazole acetic acid [13C2, 15N3] | 3.8 | Primary | - | 133.0 | 75.0 |

Calibration

The quantification of each analyte was made through the building of a calibration straight line with the

internal standard method. 5 analytical standard solutions were analysed in single injections in order to

obtain a calibration curve (1/x weighed) interpolated with a linear regression.

Recovery (Accuracy)

During the analytical session for the determination of TDM (1,2,4-triazole, triazole-alanine, triazole-lactic acid and triazole-acetic acid), two recovery check tests were carried out fortifying aliquots of an untreated honey sample (CDS-23-1469 and CDS-23-1565) at the following fortification levels:

- LOQ (0.01 mg/kg), adding 50 μL of TDM Mix solution B (containing 1.00 mg/L of each TDM) to aliquots of about 5 g of untreated sample.

- 10xLOQ (0.1 mg/kg), adding 50 μL of TDM Mix solution A (containing 10.0 mg/L of each TDM) to aliquots of about 5 g of untreated sample.

Results and discussions

Method validation data can be summarised in the tables below.

Table A 6:	Recovery results from method validation of Difenoconazole using the analytical method

|  | Difenoconazole - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.0100 | 2.6912 | 0.0108 | 108 |
| CDS-23-1469 RC2 | 10xLOQ | 0.100 | 29.2820 | 0.117 | 117 |
| CDS-23-1565 RC1 | LOQ | 0.00988 | 2.3309 | 0.00921 | 93.2 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0998 | 24.9968 | 0.0998 | 100 |

Table A 7:	Recovery results from method validation of Prothioconazole using the analytical method

|  | Prothioconazole - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.0100 | 1.9831 | 0.00793 | 79.3 |
| CDS-23-1469 RC2 | 10xLOQ | 0.100 | 17.5604 | 0.0702 | 70.2 |
| CDS-23-1565 RC1 | LOQ | 0.00988 | 2.4439 | 0.00966 | 97.8 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0998 | 26.9483 | 0.108 | 108 |

Table A 8:	Recovery results from method validation of Prothioconazole-desthio using the analytical method

|  | Prothioconazole-desthio - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.0100 | 2.4187 | 0.00967 | 96.7 |
| CDS-23-1469 RC2 | 10xLOQ | 0.100 | 27.1363 | 0.109 | 109 |
| CDS-23-1565 RC1 | LOQ | 0.00988 | 2.3981 | 0.00948 | 95.9 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0998 | 25.2165 | 0.101 | 101 |

Table A 10:	Characteristics for the analytical method used for validation of Difenoconazole, Prothioconazole, Prothioconazole-desthio residues in Honey

|  | Difenoconazole | Prothioconazole | Prothioconazole-desthio |
| --- | --- | --- | --- |
| Calibration (range, number of data points) | Exp. Range = 0.5000-50.0000 R2 = 0.99988849 y = 1578.707297x + 326.624111 (weighting: 1/x) | Exp. Range = 0.5000-50.0000 R2 = 0.99826763 y = 18.433477x - 1.901495 (weighting: 1/x) | Exp. Range = 0.5000-50.0000 (n=5) R2 = 0.99971858 Y=517.190437x + 274.256963 (weight:1/x) |
| Limit of determination/quantification | LOD: 0.002 mg/kg LOQ: 0.010 mg/kg |  |  |

Table A 9:	Recovery results from method validation of 1,2,4-triazole (1,2,4-TRZ) using the analytical method

|  | 1,2,4-triazole (TRZ) - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.00988 | 2.342 | 0.00926 | 93.7 |
| CDS-23-1469 RC2 | 10xLOQ | 0.0990 | 23.924 | 0.0947 | 95.7 |
| CDS-23-1565 RC1 | LOQ | 0.0101 | 2.022 | 0.00817 | 80.9 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0994 | 24.699 | 0.0982 | 98.8 |

Table A 10:	Recovery results from method validation of Triazole-alanine (TA) using the analytical method

|  | Triazole alanine (TA) - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.00988 | 2.470 | 0.00976 | 98.8 |
| CDS-23-1469 RC2 | 10xLOQ | 0.0990 | 21.860 | 0.0866 | 87.4 |
| CDS-23-1565 RC1 | LOQ | 0.0101 | 1.929 | 0.00779 | 77.2 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0994 | 21.878 | 0.0870 | 87.5 |

Table A 11:	Recovery results from method validation of Triazole-lactic acid (TLA) using the analytical method

|  | Triazole lactic acid (TLA) - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.00988 | 2.361 | 0.00933 | 94.4 |
| CDS-23-1469 RC2 | 10xLOQ | 0.0990 | 25.383 | 0.101 | 102 |
| CDS-23-1565 RC1 | LOQ | 0.0101 | 1.970 | 0.00796 | 78.8 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0994 | 25.164 | 0.100 | 101 |

Table A 12:	Recovery results from method validation of Triazole-acetic acid (TAA) using the analytical method

|  | Triazole acetic acid (TAA) - Procedural recovery check results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specimen identification | Fortification level | Actual spiked Conc. (mg/kg) | Measured conc. in the analyzed extract (µg/L) | Measured conc. in the sample (mg/kg) | Recovery (%) |
| CDS-23-1469 RC1 | LOQ | 0.00988 | 2.403 | 0.00950 | 96.1 |
| CDS-23-1469 RC2 | 10xLOQ | 0.0990 | 25.193 | 0.0998 | 101 |
| CDS-23-1565 RC1 | LOQ | 0.0101 | 2.571 | 0.0104 | 103 |
| CDS-23-1565 RC2 | 10xLOQ | 0.0994 | 25.314 | 0.101 | 101 |

Table A 10:	Characteristics for the analytical method used for validation of TDMs residues in Honey

|  | 1,2,4-triazole (1,2,4-TRZ) | Triazole alanine (TA) | Triazole lactic acid (TLA) | Triazole acetic acid (TAA) |
| --- | --- | --- | --- | --- |
| Calibration (range, number of data points) | Range = 0.511-50.169 (n=5) R2 = 0.9984805776 y = 0.64316 x + 0.00558 (r = 0.99924) (weighting: 1/x) | Range = 0.60-51.869 (n=5) R2 = 0.9950661009 14.22980x + 0.04992 (r = 0.99753) (weighting: 1/x) | Range = 0.513-50.20 (n=5) R2 = 0.9989602704 y = 0.17746x + 0.00104 (r = 0.99948) (weighting: 1/x) | Range = 0.543-50.748 (n=5) R2 = 0.9993601024 y = 1.06430 x + 0.00787 (r = 0.99968) (weighting: 1 x) |
| Limit of determination/quantification | LOD: 0.002 mg/kg LOQ: 0.010 mg/kg |  |  |  |

Conclusion

The test item was applied, and specimens were generated and analysed according to the recommendations of the sponsor and the study objectives. Results of residue analysis may therefore be used in order to predict the residue behaviour of Difenoconazole, Prothioconazole and Prothioconazole-desthio after one foliar application of the test item IN233C1560 380 EC when applied as per study indications.

        - Independent Laboratory Validation (ILV) Analytical method 2

          - Method validation

| Comments of zRMS: | Method is accepted. |
| --- | --- |

| Reference: | KCP 5.1.2/04 |
| --- | --- |
| Report | Independent Laboratory Validation (ILV) of the Analytical Method for the Determination of Difenoconazole, Prothioconazole, Prothioconazole-desthio and Triazole Derivative Metabolites (TDMs) residue in Honey Mattioli B. 2023 GLP Study No. CH – 0859-2023 |
| Guideline(s): | Yes. - Council Decision [C(97)186/Finai] amending Annex II to the council decision concerning the mutual acceptance of data in the assessment of chemicals [C(81) 30 (final)]. - ENV/MC/CHEM(98)17 OECD Principles on Good Laboratory Practice, No. 1 and all subsequent OECD consensus documents. - Directive 2004/9/CE and directive 2004/10/CE of the European Parliament and of the Council of February 11th, 2004 enforced by Italian Legislative Decree No. 50 of March 2nd, 2007 as published in G.U. No. 86 of April 13th, 2007. |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Description of the method for the quantification of Prothioconazole, Prothioconazole-desthio and difenoconazole

Recovery and Repeatability (Precision)

The recovery and the repeatability (as precision, % RSD) of the analytical method were determined using freshly fortified control samples of honey. Since the validation of the linearity and calibration test was not a requirement, but the calibration is necessary to quantify the fortified matrix samples, it is presented below how the calibration was prepared and the obtained results. Fortified samples were quantified using the equation of the calibration curve, with the standard solutions injected in the same run with the samples: five matrix-matched standard solutions (from MSS1 to MSS5) were prepared in matrix (blank honey extract) and they were injected before the fortified samples.  Fortification levels were chosen at LOQ 0.01 mg/kg and at 10xLOQ 0.1 mg/kg in honey, as nominal concentrations.

Sample extraction

Reference solutions preparation:

The following reference solutions were prepared:

Instrument conditions:

The analyses were carried out using a HPLC-MS/MS system according to the following conditions:

| Method for the quantification of Prothioconazole, Prothioconazole-desthio and difenoconazole |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Column | HPLC column, Kinetex C18 100 Å, 1.7 µm, 50 x 2.1 mm i.d. (+ pre-column); Internal code LCN 431 (or equivalent) |  |  |  |  |
| Detector | MS Triple quadrupole (Scan in MRM mode) |  |  |  |  |
| Eluent C | Methanol with formic acid 0.2% and ammonium formate 5 mM |  |  |  |  |
| Eluent D | Water with formic acid 0.2% and ammonium formate 5 mM |  |  |  |  |
| Eluent flow | 0.6 mL/min |  |  |  |  |
| Elution mode | gradient condition |  |  |  |  |
| Solvent composition | Time (min) | % C |  | % D |  |
|  | 0 | 30 |  | 70 |  |
|  | 0.5 | 30 |  | 70 |  |
|  | 3 | 10 |  | 100 |  |
|  | 5 | 30 |  | 70 |  |
| Column temperature | 40°C |  |  |  |  |
| Volume of injection | 2.5 μL |  |  |  |  |
| Retention time | Prothioconazole - About 3.2 minutes |  |  |  |  |
|  | Prothioconazole-desthio - About 3.0 minutes |  |  |  |  |
|  | Difenoconazole – About 3.3 |  |  |  |  |
| Total analysis time | 5 minutes + 1.0 minute as post time |  |  |  |  |
| Mass scan parameters |  |  |  |  |  |
| Ion mode | ESI, positive polarity |  |  |  |  |
| Scan type | MRM |  |  |  |  |
| Dry gas temperature (°C) | 300 |  |  |  |  |
| Dry gas flow (L/min) | 8 |  |  |  |  |
| Nebuliser (psi) | 40 |  |  |  |  |
| Sheath Gas Temp (°C) | 400 |  |  |  |  |
| Sheath Gas Flow (L/min) | 12 |  |  |  |  |
| Capillary current (V) | 3000 |  |  |  |  |
| Nozzle Voltage/Charging (V) | 1500 |  |  |  |  |
| Compound | Prothioconazole |  | Prothioconazole-desthio |  | Difenoconazole |
| Precursor ion (m/z) - (fragmentor, V) | 342 – (115) |  | 312.2 – (100) |  | 406.2 – (115) |
| Product ions (m/z) – (Collision Energy, V) | quantifier 100.0 – (25) |  | quantifier 125.0 – (38) |  | quantifier 251.1 – (22) |
|  | qualifier 264.0 – (21) |  | qualifier 69.8 (*) - (26) |  | qualifier 188.4 - (54) |
| Dwell time (msec) | 50 |  |  |  |  |

(*) Note: for the analyte Prothioconazole-desthio it was not possible to find both the product ions with m/z > 100, as required by SANTE/2020/12830, rev.2; this analyte fragmented only in two product ions, one of them with m/z < 100.

Results and discussions

Calibration (matrix-matched standard solutions)

Prothioconazole, Prothioconazole-desthio and difenoconazole linearity was checked by a 5-points calibration curve using the standard solutions. The analytical calibration was prepared in the range 0.500 – 50.0 μg/L, as nominal concentrations, for each analyte (corresponding to 0.0020 – 0.200 mg/kg).

Linearity was considered acceptable since the regression residuals plots were randomly distributed.

Recovery (Accuracy) and Repeatability (Precision)

| Analyte | Prothioconazole |  | Prothioconazole-desthio |  | Difenoconazole |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ion | Primary (342 m/z -> 100 m/z) | Confirmatory (342 m/z -> 264 m/z) | Primary (312 m/z -> 125 m/z) | Confirmatory (312 m/z -> 69.8 m/z) | Primary (406.2 m/z -> 251.1 m/z) | Confirmatory (406.2 m/z -> 188.4 m/z) |
| Linear calibration | Y=91.5x+123.3 r2=0.99708 | y=47.2+48.4 r2=0.99879 | Y=189.3x+190.6 r2=0.99991 | y=625.8+753.8 r2=0.99997 | y=676.5+4755.5 r2=0.99979 | y=71.4 + 517.8 r2=0.99976 |
| % Recovery ± (% RSD) | LOQ = 76.4 ± 3.2 % 10xLOQ= 98.1 ± 9.3 % | LOQ = 86.3 ± 2.7 % 10xLOQ= 96.5 ± 9.7 % | LOQ = 107.4 ± 10.0 % 10xLOQ= 89.1 ± 14.7 % | LOQ = 105.1 ± 16.5 % 10xLOQ= 87.2 ± 15.0 % | LOQ = 94.5 ± 5.6 % 10xLOQ= 92.4 ± 3.8 % | LOQ = 92.9 ± 10.5 % 10xLOQ= 89.9 ± 4.7 % |

Matrix effect

The matrix effect measured resulted not significant (lower than 20%) in Primary GLP study code LBN-0092-2023. In any case, matrix matched standard solutions were used to compensate the slight matrix effect.

| Matrix Effects Results |  |  |
| --- | --- | --- |
| GLP Primary study | Analyte | Matrix Effect (%) |
| Primary GLP study code LBN-0092-2023 | Prothioconazole | - 0.06 % (not significant) |
|  | Prothioconazole-desthio | -10.5 % (not significant) |
|  | Difenoconazole | - 7.93 % (not significant) |

Limit of detection (LOD)

The limit of detection (LOD) is the lowest calibration level standard solution at 0.500 μg/L (corresponding to 0.002 mg/kg) of each analyte (≤ 30% LOQ).

Limit of quantification (LOQ)

The limit of quantification (LOQ) is the lowest fortification level solution at 0.01 mg/kg of each analyte in the honey samples.

Confirmation:

The Prothioconazole, Prothioconazole-desthio and difenoconazole determination was conducted by HPLC-MS/MS in MRM mode, monitoring two MS/MS ion mass transitions: 342.0 → 100.0, 312.2 → 125.0 and 406.2 → 251.1 primary detection for quantification, 342.0 → 264.0, 312.2 → 69.8(*) and 406.2 → 188.4 for qualitative purpose (confirmation).

(*) Note: for the analyte Prothioconazole-desthio it was not possible to find both the product ions with m/z > 100, as required by SANTE/2020/12830, rev.2; this analyte fragmented only in two product ions, one of them with m/z < 100.

Stability:

Samples extract: Verified for all the analytes for 5 days at 5 ± 3°C in the dark (stable with recovery values ranging from 60 to 120%).

Standard solutions: The stability of the Prothioconazole-desthio and Difenoconazole in the stock standard solutions was verified for 96 days at 5 ± 3°C in the dark for stock solutions in acetonitrile (stable with difference between stored and fresh solution lower than 10%).

Description of the method for the quantification of triazole-derivative metabolites (TDMs

Recovery and Repeatability (Precision)

The recovery and the repeatability (as precision, % RSD) of the analytical method were determined using freshly fortified control samples of honey.  Since the validation of the linearity and calibration test was not a requirement, but the calibration is necessary to quantify the fortified matrix samples, it is presented below how the calibration was prepared and the obtained results.  Fortified samples were quantified using the equation of the calibration curve, with the standard solutions injected in the same run with the samples: five working standard solutions (from WSS1 to WSS5) were prepared in solvent and they were injected before the fortification levels.  Fortification levels were chosen at LOQ 0.01 mg/kg and at 10xLOQ 0.1 mg/kg in honey, as nominal concentrations.

| Method for the quantification of triazole-derivative metabolites (TDMs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Column |  |  | HPLC column, Hypercarb, 5 µm, 100 x 2.1 mm i.d.; Internal code LCN 421 (or equivalent) |  |  |  |  |  |  |  |  |
| Detector |  |  | MS Triple quadrupole (Scan in MRM mode) |  |  |  |  |  |  |  |  |
| Eluent A |  |  | Water with acetic acid 1% |  |  |  |  |  |  |  |  |
| Eluent B |  |  | Methanol with acetic acid 1% |  |  |  |  |  |  |  |  |
| Eluent flow |  |  | 0.6 mL/min |  |  |  |  |  |  |  |  |
| Elution mode |  |  | gradient condition |  |  |  |  |  |  |  |  |
| Solvent composition |  |  | Time (min) |  |  | % A |  |  | % B |  |  |
|  |  |  | 0 |  |  | 100 |  |  | 0 |  |  |
|  |  |  | 1 |  |  | 100 |  |  | 0 |  |  |
|  |  |  | 6 |  |  | 10 |  |  | 90 |  |  |
|  |  |  | 7 |  |  | 10 |  |  | 90 |  |  |
|  |  |  | 7.1 |  |  | 100 |  |  | 0 |  |  |
| Column temperature |  |  | 40°C |  |  |  |  |  |  |  |  |
| Volume of injection |  |  | 2 μL |  |  |  |  |  |  |  |  |
| Retention time |  |  | TRZ - About 0.90 minutes |  |  |  |  |  |  |  |  |
|  |  |  | TAL - About 1.05 minutes |  |  |  |  |  |  |  |  |
|  |  |  | TLA - About 3.50 minutes |  |  |  |  |  |  |  |  |
|  |  |  | TAA - About 3.80 minutes |  |  |  |  |  |  |  |  |
| Total analysis time |  |  | 10 minutes |  |  |  |  |  |  |  |  |
| Mass scan parameters |  |  |  |  |  |  |  |  |  |  |  |
| Ion mode |  |  | ESI, positive polarity |  |  |  |  |  |  |  |  |
| Scan type |  |  | MRM |  |  |  |  |  |  |  |  |
| Curtain gas flow (mL/min) |  |  | 30.00 |  |  |  |  |  |  |  |  |
| Gas 1 flow (mL/min) |  |  | 55.00 |  |  |  |  |  |  |  |  |
| Gas 2 flow (mL/min) |  |  | 65.00 |  |  |  |  |  |  |  |  |
| Gas Temperature (°C) |  |  | 500.00 |  |  |  |  |  |  |  |  |
| Capillary current (V) |  |  | 3500.00 |  |  |  |  |  |  |  |  |
| MRM transitions parameters |  |  |  |  |  |  |  |  |  |  |  |
| Compound | Precursor ion (m/z) | Product ion (m/z) | DP | EP | CE |  | CXP | COV |  | DMO | SV |
| TRZ | 70.1 | 43.1 (quantifier) | 90 | 6 | 28 |  | 20 | -13.3 |  | -12 | 3000 |
|  |  | 70.0 (qualifier) |  |  | 13 |  | 10 |  |  |  |  |
| TAL | 157.0 | 70.0 (quantifier) | 13 | 7 | 18 |  | 10 | -0.6 |  | -30 | 2500 |
|  |  | 88.0 (qualifier) |  |  | 18 |  | 10 |  |  |  |  |
| TLA | 158.0 | 70.0 (quantifier) | 40 | 10 | 25 |  | 10 | 0.15 |  | -35 | 3500 |
|  |  | 43.0 (qualifier) |  |  | 55 |  | 10 |  |  |  |  |
| TAA | 128.0 | 70.0 (quantifier) | 60 | 10 | 24 |  | 8 | -3.8 |  | -30 | 3000 |
|  |  | 73.0 (qualifier) |  |  | 20 |  | 9 |  |  |  |  |

| Compound | Precursor ion (m/z) | Product ion (m/z) | DP | EP | CE | CXP | COV | DMO | SV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TRZ ISTD | 75.0 | 46.0 | 90 | 6 | 28 | 20 | -13.1 | -12 | 3000 |
| TAL ISTD | 159.0 | 72.0 | 20 | 10 | 20 | 10 | 1.15 | -35 | 3400 |
| TLA ISTD | 163.0 | 75.0 | 40 | 10 | 25 | 10 | 0.15 | -35 | 3500 |
| TAA ISTD | 133.0 | 75.0 | 60 | 10 | 26 | 8 | -3.6 | -30 | 3000 |
| TAL ISTD_2 (*) | 159.0 | 89.0 | 20 | 10 | 18 | 10 | 1.15 | -35 | 3400 |

TRZ: 1,2,4-Triazole; TAL: 1,2,4-Triazole Alanine; TLA: 1,2,4-Triazole lactic acid; TAA: 1,2,4-Triazole acetic acid; DP: declustering potential. EP: entry potential. CE: collision energy. CXP: collision cell exit potential. COV: compensation voltage. DMO: DMS offset. SV: separation voltage.

(*) Note: this transition will not be taken into account for calculations. It is used only as confirmatory for TAL ISTD.

Results and discussions

Confirmation:

The analytical method for the TDMs (triazole-derivative metabolites) determination consisted in the addition of a proper amount of ILIS (isotope-labelled internal standards) and the analysis of the samples extract with HPLC-MS/MS, equipped with a differential mobility separation device (DMS), in MRM mode, monitoring the MS/MS ion mass transitions:

- 1,2,4-Triazole: 70.1 → 43.1 (quantification) 70.1 → 70 (confirmation)

- Triazole Alanine: 157 → 70 (quantification) 157 → 88 (confirmation)

- Triazole Lactic Acid: 158 → 70 (quantification) 158 → 43 (confirmation)

- Triazole Acetic Acid: 128 → 70 (quantification) 128 → 73 (confirmation)

Calibration (working standard solutions)

The triazole-derivative metabolites linearity was checked by a 5-points calibration curve using the standard solutions. The analytical calibration was prepared in the range 0.500 – 50.0 μg/L, as nominal concentrations, for each analyte (corresponding to 0.0020 – 0.200 mg/kg). Linearity was considered acceptable since the regression residuals plots were randomly distributed

Recovery (Accuracy) and Repeatability (Precision)

| Analyte | 1,2,4-Triazole |  | 1,2,4-Triazole acetic acid |  | 1,2,4-Triazole Alanine |  | 1,2,4-Triazole lactic acid |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ion | Primary (primary, 70.1 m/z -> 43.1 m/z) | Confirmatory (70.1 m/z -> 70.0 m/z) | Primary (primary, 128.0 m/z -> 70.0 m/z) | Confirmatory (128.0 m/z -> 73.0 m/z) | Primary (157.0 m/z -> 70.0 m/z) | Confirmatory (157.0 m/z -> 88.0 m/z) | Primary (primary, 158.0 m/z -> 70.0 m/z) | Confirmatory (158.0 m/z -> 43.0 m/z) |
| Linear calibration | y=0.0622 – 0.0077 r2=0.99994 | y=0.5586+0.0687 r2=0.99999 | y=0.1130 – 0.0223 r2=0.99989 | y=0.0049-0.021 r2=0.99968 | y=1.3679x+7700 r2=0.99816 | y=0.7341+0.3037 r2=0.99926 | Y=0.0171x+0.0009 r2=0.99991 | y=0.0021+0.0011 r2=0.99869 |
| % Recovery ± (% RSD) | LOQ = 109.9 ± 6.1 % 10xLOQ= 101.4 ± 2.8 % | LOQ = 101.8 ± 3.3 % 10xLOQ= 101.5 ± 2.0% | LOQ = 104.2 ± 4.4 % 10xLOQ= 97.6 ± 1.4 % | LOQ = 98.1 ± 7.1 % 10xLOQ= 94.6 ± 4.8 % | LOQ = 80.5 ± 9.1 % 10xLOQ= 85.8 ± 4.7 | LOQ = 80.5 ± 9.1 % 10xLOQ= 87.1 ± 3.9 | LOQ = 103 ± 5.8 % 10xLOQ= 102.9 ± 2.5 % | LOQ = 111.9 ± 5.8 % 10xLOQ= 112.1 ± 2.6% |

Matrix effect

The matrix effect measured resulted not significant (lower than 20%) in Primary GLP study code LBN-0092-2023. In any case, matrix matched standard solutions were used to compensate the slight matrix effect.

| Matrix Effects Results |  |  |
| --- | --- | --- |
| GLP Primary study | Analyte | Matrix Effect (%) |
| Primary GLP study code LBN-0093-2023 | 1,2,4-Triazole | + 2.4 % (not significant) |
|  | Triazole Alanine | -7.2 % (not significant) |
|  | Triazole Lactic Acid | + 2.1 % (not significant) |
|  | Triazole Acetic Acid | - 2.5 % (not significant) |

Limit of Detection

The limit of detection (LOD) is the lowest calibration level standard solution at 0.500 μg/L (corresponding to 0.002 mg/kg) of each analyte (≤ 30% LOQ).

Limit of Quantification

The limit of quantification (LOQ) is the lowest fortification level solution at 0.01 mg/kg of each analyte in the honey samples.

Stability:

Analytes in the samples extract: Not required since Isotope-Labelled Internal Standards (ILIS) were used.

Stability of the analytes in the standard solutions: Verified for 78 days at 5 ± 3°C in the dark for stock solutions in water (stable with difference between stored and fresh solution lower than 10%).

Conclusion

The primary method, validated in GLP studies Code LBN-0092-2023 (Determination of Difenoconazole, Prothioconazole and Prothioconazole-desthio in Honey) and LBN-0093-2023 (Determination of Triazole Derivative Metabolites (TDMs) in Honey) performed by LabAnalysis s.r.l., was fully validated. The study CH - 0859-2023 validated and confirmed the LOQ of the primary method according to the guideline SANTE/2020/12830 rev. 2 dated 14/02/23 and it can be considered suitable for the determination of Prothioconazole Prothioconazole-desthio, Difenoconazole 1,2,4-triazole, 1,2,4-triazole alanine, 1,2,4-triazole lactic acid and 1,2,4-triazole acetic acid residues in the tested matrix (honey).