FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: SAP250F Product name(s): INDOFIL Prothio 250 EC Chemical active substance: Prothioconazole, 250 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Indofil Industries (Netherlands) BV Submission date: 30/04/2021 MS Finalisation date: 08/2022, 02/2023

Version history

| When | What |
| --- | --- |
| April 2021 | V0 - Original version from applicant Indofil Industries (Netherlands) B.V. for submission to z-RMS in the frame of the PPP Authorization according to Article 33 of Regulation (EC) No 1107/2009. |
| August 2022 | zRMS first evaluation |
| February 2023 | fRR after commenting period |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	Prothioconazole	5

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	6

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	14

4	Section 4: Further information on the plant protection product	15

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	15

Appendix 1	Lists of data considered in support of the evaluation	17

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	21

A 2.1	Active substance 1	21

zRMS summary:

Submitted data are sufficient for evaluation. The stability data indicate a shelf life of at least 2 years when stored in COEX. Packaging: Based on the stability tests, proposed packaging type – COEX is appropriate and accepted.

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Submitted by:

Company:	Kerona Scientific Ltd.

Address:	14 Thomas Hand Street

Skerries, Co. Dublin,

Ireland

Name:	XXXXX

Telephone:	XXXXX

E-mail: 	XXXXX

On behalf of the applicant:

Company:	Indofil Industries (Netherlands) BV

Address:	Piet Heinkade 55

1019 GM Amsterdam

The Netherlands

Name:	XXXXX

Telephone:	XXXXX

E-mail: 	XXXXX

Contact point:	Dr. Irene McGrath

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Prothioconazole

| Prothioconazole | min. 970 g/kg (according to the Reg. 540/2011) min. 980 g/kg (Indofil source) |
| --- | --- |

| Toluene | max. 5.0 g/kg |
| --- | --- |
| Prothioconazole-desthio | max. 0.5 g/kg |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Please refer to Registration Report Part A for the relevant country (or) |
| --- | --- |
| Trade name: | INDOFIL Prothio 250 EC |
| Code numbers: | SAP250F (Prothioconazole 250 g/L EC) |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance | Declared content of the pure active substance (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Prothioconazole | 250 | 235 - 265 | 255.1 g/L | 26.04 23.62 |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers: 98.0%

** 	Based on the density of the formulation = 0.98  1.08

Table 1.4-2:	Safener and synergists

Neither safener nor synergist were used for the formulation.

Table 1.4-3:	Relevant impurities

| Relevant impurity | Maximum content in PPP (g/L or g/kg) |
| --- | --- |
| Prothioconazole-desthio | 0.125 g/L |
| Toluene | 1.25 g/L |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-4:	Information on Prothioconazole

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Prothioconazole |
| CAS No. | 178928-70-6 |
| EC No. | 605-841-2 |
| CIPAC No. | 745 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

No safener/synergist are present in the formulation.

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Emulsifiable concentrate | [Code: EC] |
| --- | --- |

## Function (KCP 1.6)

Fungicide

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.  The appearance of the product is that of a clear tan liquid, with an aromatic odour. It has no oxidising properties. The product has a flash point of 151.5 °C. It has a self-ignition temperature of 335 °C. In aqueous solution, it has a pH value around 4.8 at 25 °C. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0 °C and 14 days at 54 °C, neither the active substance content nor the technical properties were changed. The stability data indicate a shelf life of at least 2 years at ambient temperature when stored in COEX materials.  Its technical characteristics are acceptable for an emulsifiable concentrate formulation. The intended concentration of use is 1.75 mL/L to 5.33 mL/L.

The products is not intended to be tank mixed.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

No classification or labelling for physical chemical properties is proposed.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No risk or safety phrases for physical chemical properties are proposed.      P280

Compliance with FAO specifications:

The product SAP250F complies with FAO specifications.

Formulation used for tests

The formulation used for tests has the same composition as the one cited in Part C.

Table 2-5:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | Internal Method AM190 (based in EPA method OPPTS 830.6302 to 4) | Prothioconazole 250 g/L EC Batch: B-ACA | Initial and after 2 weeks at 54ºC: -Physical state: Liquid (emulsifiable concentrate) -Colour: Clear tan Odour (T0 and after 2 weeks): aromatic | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted |
| Explosive properties (KCP 2.2.1) | Theoretical assessment | Prothioconazole 250 g/L EC | No explosive properties of the preparation |  | KCP 2.1/02 Martins, V., 2019 Report No. TR0119 | Accepted Not explosive |
| Oxidizing properties (KCP 2.2.2) | Test O.2: Oxidising properties of liquids | Prothioconazole 250 g/L EC Batch: B-ACA | The test item had no oxidising properties. According with these results, the substance does not have to be classified as an “oxidising liquid”, class 5.1. | Y | KCP 2.1/03 Dornhagen, J., 2019 Report No. PS20190086-3 | Accepted No oxidising properties |
| Flash point (KCP 2.3.1) | EC A.9 and DIN EN ISO 3679:2015 (determination of flash point – rapid equilibrium closed cup method) | Prothioconazole 250 g/L EC Batch: R-DAA | The flash point of the test item was 151.5ºC under atmospheric conditions (1013 hPa). | Y | KCP 2.1/04 Dornhagen, J., 2019 Report No. PS20190004-1 | Accepted |
| Flammability (KCP 2.3.2) | - | - | - | - | - | Not required |
| Self-heating (KCP 2.3.3) | EC A.15 | Prothioconazole 250 g/L EC Batch: B-ACA | The auto-ignition temperature of the test item was 335ºC. |  | KCP 2.1/05 Dornhagen, J., 2019 Report No. PS20190086-2 | Accepted |
| Acidity or alkalinity and pH (KCP 2.4.1) | - | - | Acidity or alkalinity is not required, because the pH of SAP250F preparation in higher than 4 and lower than 10. | - | - | Not required |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | Internal method AM063 (based on CIPAC MT 75.3) | Prothioconazole 250 g/L EC Batch: B-ACA | Initial: 4.8 at 24.7ºC After 2 weeks at 54ºC: 4.7 at 24.6ºC | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted |
| Viscosity (KCP 2.5.1) | CIPAC MT 192 OECD Test Guideline 114 | Prothioconazole 250 g/L EC Batch: B-ACA | Kinematic Viscosity (cP) results at 20 and 40ºC: Kinematic viscosity (m2.s-1) [20ºC ¦ 40ºC]; 0.5 rpm; [2.56E-04 / 2.59E-04] ¦ [9.56E-05 / 9.68E-05]. 1.0 rpm; [2.48E-04 / 2.48E-04] ¦ [7.56E-05 / 7.50E-05]. 2.0 rpm; [2.44E-04] ¦ [7.31E-05]. | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted It is difficult to make a definite statement on the flow behaviour. Still, based on the formulation composition there is no implication for classification |
| Surface tension (KCP 2.5.2) | Internal method AM243 (based on OECD Guideline No. 115 and EC A.5) | Prothioconazole 250 g/L EC Batch: B-ACA | Highest application concentration 5.33 mL/L (20ºC ± 0.5): 41.7 mN/m. Since the surface tension is below 60 mN/m, the preparation should be regarded as surface active. | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted The highest recommended concetration was tested Surface active |
| Relative density (KCP 2.6.1) | Internal method AM 017 (based on ASTM D 4052-96 and OECD Guideline 109) | Prothioconazole 250 g/L EC Batch: B-ACA | Initial and after 2 weeks at 54ºC: 1.08 at 20ºC | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted |
| Bulk density (KCP 2.6.2) | - | - | Not applicable as the preparation is a liquid. | - | - | Not required |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.3 HPLC-UV | Prothioconazole 250 g/L EC Batch: B-ACA | The sample is considered to be stable at 54ºC for two weeks. The physical-chemical properties are similar between samples before and after storage. Packaging used was CoEx (HDPE/EVOH). This is the same packaging used in shelf life study therefore no extrapolation is needed. Please see points KCP 2.1, 2.4.2, 2.6.1, 2.7.3, 2.8.2, 2.8.6.1, 2.8.6.2, 2.8.6.3, 2.11 of the present Table 2.1 for detailed results, after 2 weeks storage at 54ºC. Prothioconazole: - T0: 249.1 g/L (23.06% w/w) - After 2 weeks: 249.0 g/L (23.06% w/w) Prothioconazole-desthio: - T0: < LOQ (0.011 %w/w) - After 2 weeks: < LOQ (0.011 %w/w) Toluene: - T0: < LOQ (0.012 % w/w) - After 2 weeks: < LOQ (0.012 % w/w) | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Packaging (COEX bottle) – unchanged (KCP 2.11). All other results after storage are acceptable Accepted |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | GIFAP Technical Monograph 17 | Prothioconazole 250 g/L EC Batch: B-ACA | A 2-year storage study is underway and will be provided when complete. | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 | Not required |
| Minimum content after heat stability testing (KCP 2.7.3) | Internal method AM270 | Prothioconazole 250 g/L EC Batch: B-ACA | Prothioconazole content; Initial; After 2 weeks at 54ºC. % w/w; 23.06; 23.06. g/L; 249.1; 249.0. | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 CIPAC MT 36.3 | Prothioconazole 250 g/L EC Batch: B-ACA | It was noticed any separated material. Emulsion characteristics and re-emulsification properties: (Lowest and highest application: 1.75 mL/L and 5.33mL/L) at 30ºC Standard Water A; Standard Water D. Initial; Uniform and spontaneously. 30’ (cream/oil); (0/0); (0/0). 2h (cream/oil); (0/0); (0/0). 24h (cream/oil); (0/0); (0/0). Re-emulsion; Uniform. 24h+30’ (cream/oil); (0/0); (0/0). | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Not affected by low temperature. The lowest and the highest recommended concetrations were tested Accepted |
| Ambient temperature shelf life (KCP 2.7.5) | GIFAP Technical Monograph 17 HPLC-UV OPPTS 830.6302 to 4 Internal method AM063, based on CIPAC MT 75.3 CIPAC MT 36.3 | Prothioconazole 250 g/L EC Batch: B-ACA | A 2-year storage study is underway and will be provided when complete. Analytical results after 24 months: Property; T0; T24. Prothioconazole; g/L; 23.06; 23.06. % w/w; 249.1; 249.0. Prothioconazole-desthio; % w/w; < LOQ (0.011); < LOQ (0.011). Toluene; % w/w; < LOQ (0.012); < LOQ (0.012). Appearance (T24): clear tan liquid with aromatic odour pH of 1%, temp. 24.6ºC (T24): 4.7 Emulsion characteristics and re-emulsification (T24) Lowest and highest application: 1.75 mL/L and 5.33mL/L: Water A and D: Initial emulsion – uniform and spontaneously 30’ (cream/oil); (0/0). 2h (cream/oil); (0/0). 24h (cream/oil); (0/0). Re-emulsion; Uniform. 24h+30’ (cream/oil); (0/0). Packaging (COEX (HDPE/EVOH) bottle) after storage – no visual degradation (deformation, leaking etc.) A 3-year storage study is on-going | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 | Formulation remained stable after storage. All results are acceptable The lowest and the highest recommended concetrations were tested Accepted |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Not required. A shelf life of at least 2 years is proposed based on results of accelerated storage testing (point 2.7.1). | - | - | Not required |
| Wettability (KCP 2.8.1) | - | - | Not required as the preparation is a liquid. | - | - | Not required |
| Persistence of foaming (KCP 2.8.2) | Internal method AM078 (based in CIPAC MT 47.3) | Prothioconazole 250 g/L EC Batch: B-ACA | Lowest and highest application: 1.75 mL/L and 5.33mL/L Type of water: CIPAC water D at 25ºC ± 5ºC Persistent Foam (mL) – Initial. Time; Lowest appl.; Highest appl.. 1 minute; 10; 8. 12 minutes; 10; 6. Persistent Foam (mL) – After 14 days at 54ºC. Time; Lowest appl.; Highest appl.. 1 minute; 10; 10. 12 minutes; 10; 8. | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted The lowest and the highest recommended concetrations were tested |
| Suspensibility (KCP 2.8.3.1) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Spontaneity of dispersion (KCP 2.8.3.2) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Dispersion stability (KCP 2.8.3.3) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Wet sieve test (KCP 2.8.5.1.2) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Dust content (KCP 2.8.5.2.1) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Particle size of dust (KCP 2.8.5.2.2) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Attrition (KCP 2.8.5.3) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Emulsifiability (KCP 2.8.6.1) | CIPAC MT 36.3 | Prothioconazole 250 g/L EC Batch: B-ACA | Initial and 2 weeks after 54ºC: (Lowest and highest application: 1.75 mL/L and 5.33mL/L) at 30ºC Standard Water A; Standard Water D. Initial emulsion; Uniform and spontaneously. 30’ (cream/oil); (0/0); (0/0). 2h (cream/oil); (0/0); (0/0). 24h (cream/oil); (0/0); (0/0). Re-emulsion; Uniform. 24h+30’ (cream/oil); (0/0); (0/0). | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted The lowest and the highest recommended concetrations were tested |
| Emulsion stability (KCP 2.8.6.2) | - | - | Please see KCP 2.8.6.1. | - | - | Not required |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Please see KCP 2.8.6.1. | - | - | Not required |
| Flowability (KCP 2.8.7.1) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Pourability (KCP 2.8.7.2) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not applicable for an EC formulation. | - | - | Not required |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | Not required as tank mixtures are not recommended in the label. | - | - | Not required |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | Not required as tank mixtures are not recommended in the label. | - | - | Not required |
| Adhesion to seeds (KCP 2.10.1) | - | - | Not required as the preparation is not for seed treatment. | - | - | Not required |
| Distribution to seed (KCP 2.10.2) | - | - | Not required as the preparation is not for seed treatment. | - | - | Not required |
| Other/special studies (KCP 2.11) | Package stability study | Prothioconazole 250 g/L EC Batch: B-ACA | Initial and after 2 weeks at 54ºC: No visual degradation observed in COEX (HDPE/EVOH) bottles. | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted |
|  | Internal method AM006 (based on CIPAC MT 30) | Prothioconazole 250 g/L EC Batch: B-ACA | Initial: 0.96 % (w/w) After 2 weeks at 54ºC: 0.98 % (w/w) | Y | KCP 2.1/01 Silva, S., 2019 Study EF/297/19 – Interim Report (T0) | Accepted |
|  | Efficacy Guideline 302, PSD Data Requirements Handbook | Prothioconazole 250 g/L EC Batch: R-DAA | After single rinse procedure, the residue of Prothioconazole in the tank was determined to be ≤ 0.12 % of the initial concentration (three samplings each, after 20 %, 50 % and 80 % of the capacity was drained off) | Y | Fiesler A., 2019 Study No. 130811361 | Accepted Concertation tested: 1.6 L of the test item / 150 L of water |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

Table 4.1-6:	Packaging information for 1 L bottle

| Type | Description |
| --- | --- |
| Material: | COEX- HDPE/barrier material/adhesive HDPE/EVOH |
| Shape/size: | cylindrical / approx. 89.5 (± 0.5) mm diameter x 231 (± 3) mm |
| Opening thread type: | 63 mm |
| Closure: | HDPE screw cap |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 1 L jerrican

| Type | Description |
| --- | --- |
| Material: | COEX- HDPE/barrier material/adhesive HDPE/EVOH |
| Shape/size: | Length (bottom): approx. 160 (± 1) mm; width (bottom): approx. 105 (± 1) mm; Height: approx. 259 (± 1) mm |
| Opening thread type: | 63 mm |
| Closure: | HDPE screw cap |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 5 L jerrican

| Type | Description |
| --- | --- |
| Material: | COEX- HDPE/barrier material/adhesive HDPE/EVOH |
| Shape/size: | Length (bottom): approx. 191 (± 1) mm; width (bottom): approx. 135 (± 1) mm; Height: approx. 282 (± 1) mm |
| Opening thread type: | 63 mm |
| Closure: | HDPE screw cap |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 10 L bottle

| Type | Description |
| --- | --- |
| Material: | COEX- HDPE/barrier material/adhesive HDPE/EVOH |
| Shape/size: | Length (bottom): approx. 232 (± 4) mm; width (bottom): approx. 192 (± 4) mm; Height: approx. 313 (± 5) mm |
| Opening: | 47 mm inner diameter |
| Closure: | HDPE screw cap |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 20 L jerrican

| Type | Description |
| --- | --- |
| Material: | COEX- HDPE/barrier material/adhesive HDPE/EVOH |
| Shape/size: | Length (bottom): approx. 270 (± 2) mm; width (bottom): approx. 220 (± 2) mm; Height: approx. 371 (± 2) mm |
| Opening: | Neck heigh: 25 (± 1) mm; Max. neck diameter: 68 (± 2) mm |
| Closure: | HDPE screw cap |
| UN/ADR | compliant |

Compatibility with the preparation

The materials proposed for use are known from experience to be very resistant to influences of chemicals. The contents have no negative effect on the packet integrity.

The packages were examined for any visible changes before and after accelerated storage for 14 days at 54º C and they did not suffer any change. Please see point KCP 2.11.

Findings:

Visual observations for package stability

| Damage | Visual Observations |  |
| --- | --- | --- |
|  | Initial | After 14 days at 54º C |
| Deformation | No | No |
| Attack by the content | No | No |
| Leaking | No | No |
| Label damage | No | No |
| Conclusion | No visual degradation | No visual degradation |

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1, KCP 2.4.2, KCP 2.5.1, KCP 2.5.2, KCP 2.6.1, KCP 2.7.1, KCP 2.7.2, KCP 2.7.3, KCP 2.7.4, KCP 2.7.5, KCP 2.8.2, KCP 2.8.6.1, KCP 2.11 | Silva, S. | 2019 | Prothioconazole 250 g/L EC (SAP250F) – Physical, chemical and technical properties of the plant protection product Study No EF/297/19 – Interim Report (T0) Ascenza Agro, S.A. GLP Unpublished | N | ASCENZA AGRO, S.A. |
| KCP 2.2.1 | Martins, V. | 2019 | Theoretical assessment of explosive properties of Prothioconazole 250 EC ASCENZA AGRO, S. A. Report No. TR0119 Not published | N | ASCENZA AGRO, S.A. |
| KCP 2.2.2 | Dornhagen, J. | 2019 | SAP250F – Oxidizing Properties (Liquids) O.2 Report No. PS20190086-3 Siemens AG GLP Unpuplished | N | ASCENZA AGRO, S.A. |
| KCP 2.3.1 | Dornhagen, J. | 2019 | Prothioconazole 250 g/L EC – SAP250F – Flash Point A.9 Report No. PS20190004-1 Siemens AG GLP Unpublished | N | ASCENZA AGRO, S.A. |
| KCP 2.3.3 | Dornhagen, J. | 2019 | SAP250F - Auto-Ignition Temperature (Liquids and Gases) A.15 Report No. PS20190086-2 Siemens AG GLP Unpublished | N | ASCENZA AGRO, S.A. |
| KCP 2.7.5 | Silva, S. | 2021 | Prothioconazole 250 g/L EC (SAP250F) – Physical, chemical and technical properties of the plant protection product Study No EF/297/19 – Interim Report (T24) Ascenza Agro, S.A. GLP Unpublished | N | ASCENZA AGRO, S.A. |
| KCP 2.11 | Fiesler A. | 2019 | SAP250F: Effectiveness of Cleaning Procedures Study No. 130811361 Ibacon GLP Unpublished | N | ASCENZA AGRO, S.A. |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report No Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Additional data on the physical, chemical and technical properties of the active substance

  - Active substance 1

None

2-(1-chlorocyclopropyl)-1-(2-chlorophenyl)-3-(1H-1,2,4-triazol-1-yl)-2-propan-2-ol