FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: SHA 105000 A Product name(s): SUPER Chemical active substance: Ferric phosphate, 29.7 g/kg

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Sharda Cropchem Ltd Submission date: March 2024 MS Finalisation date: September 2024; December 2024

Version history

| When | What |
| --- | --- |
| 07.2021 | RMS Assessment |
| 10.2021 | The Final Version of the RR |
| 05.2023 | Assessment of Applicant’s update (equivalence and RI determination and analytical methods validation) by ZRMS |
| March 2024 | This application for approval of SUPER, Iron phosphate 2.97% GB, an Ganular bait formulation containing 29.7g/kg of Ferric phosphate for nonprofessional use. Registration Report for FERROCIUS cover application for SUPER, therefore the Applicant would like to refer to already evaluated Registration Report (Section B124 below) finalized in 2023. |
| September 2024 | zRMS assessment of dRR (non-professional users, assessment of compliance with current regulations and guidelines). |
| December 2024 | The final Registration Report |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	Ferric phosphate	5

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	6

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	15

4	Section 4: Further information on the plant protection product	16

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	16

Appendix 1	Lists of data considered in support of the evaluation	18

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	21

A 2.1	Ferric phosphate	21

Sufficient data on identity, physical and chemical properties and other information are available for the plant protection product.

Noticed data gap is:

- none

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

### Producer(s) of the active substance(s)

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Ferric phosphate

| Ferric phosphate | min. 778.2 g/kg Sharda source min. 703 g/kg (equivalent to 260 g/kg iron and 144 g/kg phosphorus) (Commission Implementing Regulation (EU) 2015/1166 and according to SANTE/10385/2015 Rev 1 (29 May 2015)) RMS Comment: The equivalence source assessment of active substance (ferric phosphate) has not been completed. |
| --- | --- |

Relevant impurities (Commission Implementing Regulation (EU) 2015/1166 and according to SANTE/10385/2015 Rev 1 (29 May 2015)):

| Lead | max. 3 mg/kg in the active substance |
| --- | --- |
| Mercury | max. 0.1 mg/kg in the active substance |
| Cadmium | max. 1 mg/kg in the active substance |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Please refer to Registration Report Part A for the relevant country (or) |
| --- | --- |
| Trade name: | FEROCIOUS SUPER |
| Company code number: | SHA 105000 A Iron phosphate 2.97% GB Ferric phosphate 2.97% GB |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Ferric phosphate | 29.7 | 26.73 – 32.67 g/kg (± 10% of the declared content) | 38.17 g/kg | 3.82 % w/w |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

Table 1.4-2:	Safener and synergists

| Safener / synergist | Declared content of the safener / synergist (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| - | - | - | - | - |

* 	Based on the minimum purity of the safener/synergist declared for registration

Table 1.4-3:	Relevant impurities

| Relevant impurity | Maximum content (g/L or g/kg) |
| --- | --- |
| Lead | max. 3 mg/kg in the active substance (0.115 mg/kg in formulation) |
| Mercury | max. 0.1 mg/kg in the active substance (0.0038 mg/kg in formulation) |
| Cadmium | max. 1 mg/kg in the active substance (0.038 mg/kg in formulation) |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-4:	Information on Ferric phosphate

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Ferric phosphate |
| CAS No. | 10045-86-0 |
| EC No. | 233-149-7 |
| CIPAC No. | 629 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Ganular bait | [Code: GB] |
| --- | --- |

## Function (KCP 1.6)

Iron phosphate 2.97% GB is intended to be used as molluscicide.

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.

The appearance of the product is that of Pale blue granules with weakly odour. It is not explosive, has no oxidising properties. The product is not flammable and it has a self-ignition temperature of 250 °C. In aqueous solution, it has a pH value around 4.15 at 20 °C. There is no effect of  high temperature on the stability of the formulation, since after 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The stability data indicate a shelf life of at least 2 years at ambient temperature when stored in HDPE.  Its technical characteristics are acceptable for a granular bait formulation.

The intended concentration of use is 7 kg/ha 0.7 g/m2 (60-70 granular baits per m2 per application).

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

No risk and safety phrases are relevant for this section.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No risk and safety phrases are relevant for this section.

Compliance with FAO specifications:

The product Iron phosphate 2.97% GB complies with FAO specifications.

At the time of assessment, there is no FAO specification for ferric phosphate.

Formulation used for tests

The product used to determine the physical, chemical and technical properties is the one cited in Part C.

Table 2-5:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 | Iron phosphate 2.9% GB, Batch no SCL-58946 | Pale blue granules with weakly odor |  | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Explosive properties (KCP 2.2.1) | EEC A.14 | Iron phosphate 2.9% GB, Batch no SCL-58946 | Has no explosive properties. | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD Buczkowski D., 2017 Study N° BW-03/17 | Accepted |
| Oxidizing properties (KCP 2.2.2) | EEC A.17 | Iron phosphate 2.9% GB, Batch no SCL-58946 | Has no oxidising properties | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Flash point (KCP 2.3.1) | - | - | Not required for solid formulations. | - | - | Statement accepted |
| Flammability (KCP 2.3.2) | EEC A.10 | Iron phosphate 2.9% GB, Batch no SCL-58946 | Not considered to be highly flammable. | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Self-heating (KCP 2.3.3) | EEC A.16 | Iron phosphate 2.9% GB, Batch no SCL-58946 | 250 °C. | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Acidity or alkalinity and pH (KCP 2.4.1) | - | - | Since the pH value ranged from 4 to 10, the acidity or alkalinity test was not performed. | - | - | Statement accepted |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | Iron phosphate 2.9% GB, Batch no SCL-58946 | 4.15 (20 ºC). | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Viscosity (KCP 2.5.1) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Surface tension (KCP 2.5.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Relative density (KCP 2.6.1) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Bulk density (KCP 2.6.2) | CIPAC MT186 | Iron phosphate 2.9% GB, Batch no SCL-58946 | Pour density = 0.792 g/ml Tap density = 0.792 g/ml | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.3 CIPAC MT 75.3 CIPAC MT 170 CIPAC MT 171 CIPAC MT 178 CIPAC MT 190 ASTM E394-15 OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 Technical Monograph CropLife ISCO Procedure No.BA-AH/PB-01 XRF Spectrometry | Iron phosphate 2.9% GB, Batch no SCL-58946 | Results after storage the product at 54 ºC for 14 days: Active substance content 2.65% (w/w) 26.5 g/kg Amount of FePO4 released at time -15 minutes is 120 µg/g -30 minutes is 215 µg/g -180 minutes is 1551 µg/g Percentage FePO4 relative to the FePO4 content of the formulation-extracted at time: -15 minutes is 0.49% -30 minutes is 0.86% -180 minutes is 6.20% Appearance (physical state, coulour, odour) Pale blue granules with weakly odour pH value (1% aqueous extract) 4.22 (20 ºC) Particle size distribution 1st method: x1=1000µm where rx=0.11% x2=2000µm where rx=99.74% x1=1000µm where Rx≥99% x2=2000µm where Rx≥99% 2nd method: 99.74% of the particles have size between 2000-3350 µm 0.11% of the particles have size between 1000-2000µm Dust content Dust content is 1.2mg; thus sample is nearly dust free Attrition characteristics 99.75% Stability of packaging and packaging/preparation interactions -no significant effect of the formulation on the package and vice versa was observed; no changes in the appearance of the sample and package were observed; -Insignificant effect on the weight of the package with sample was observed;after accelerated storage procedure weight loss was in the range 0.20-0.25% (w/w). Relevant impurities content: Before storage: Cadmium: 0.00347 mg/kg Lead: 0.00360 mg/kg Mercury: 0.000103 mg/kg After storage: Iron Phosphate is inorganic compound with relevant impurities of (eco)toxicological concern: lead, cadmium, mercury which are elements. From the scientific point of view elements are not expected to form or increase their content in the formulation during the storage therefore no determination after storage is required for this product. | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD Vasu K., 2023, study No 11256/2022 | Accepted Active substance content and stability of packaging were tested. The change of the a.s. after storage was 0%. The a.s. content was determined with a method validated according to SANCO/3030/99 rev.5 (see dRR Part B5). All physical and chemical properties remained stable after the test and accepted. Test carried out in HDPE/PA coex bottles (commercial packaging). The packaging remained stable during the storage. The content of relevant impurities before storage was below the levels of SANTE/ 10385/2015 Rev 1 (29 May 2015). The content of RI was determined with a method validated according to SANCO/3030/99 rev.5 (see dRR Part B5). The content of the RI after the storage was not determined. However, as RIs do not form on the storage and the content of the a.s. was stable during the storage it can be accepted. Accepted. |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | - | - | Not required. | - | - | Statement accepted |
| Minimum content after heat stability testing (KCP 2.7.3) | - | - | Please refer KCP 2.7.1. | - | - | Statement accepted |
| Effect of low temperatures on stability (KCP 2.7.4) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Ambient temperature shelf life (KCP 2.7.5) | CIPAC MT 75.3 CIPAC MT 170 CIPAC MT 171 CIPAC MT 178 CIPAC MT 190 ASTM E394-15 OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 Technical Monograph CropLife ISCO Procedure No.BA-AH/PB-01 XRF Spectrometry | Iron phosphate 2.9% GB, Batch no SCL-58946 | Results after storage the product 2 years at room temperature: Active substance content 2.67% (w/w) 26.7 g/kg Amount of FePO4 released at time -15 minutes is 60 µg/g -30 minutes is 167 µg/g -180 minutes is 663 µg/g Percentage FePO4 relative to the FePO4 content of the formulation-extracted at time: -15 minutes is 0.23% -30 minutes is 0.64% -180 minutes is 2.54% Appearance (physical state, coulour, odour) Pale blue granules with weakly odour pH value (1% aqueous extract) 4.10 (20 ºC) Particle size distribution 1st method: x1=1000µm where rx=0.02% x2=2000µm where rx=99.87% x1=1000µm where Rx≥99% x2=2000µm where Rx≥99% 2nd method: 99.87% of the particles have size between 2000-3350 µm 0.02% of the particles have size between 1000-2000µm Dust content Dust content is 0.1mg; thus sample is nearly dust free Attrition characteristics 99.77% Stability of packaging and packaging/preparation interactions -no significant effect of the formulation on the package and vice versa was observed; no changes in the appearance of the sample and package were observed; -Insignificant effect on the weight of the package with sample was observed;after accelerated storage procedure weight loss was in the range 0.56% (w/w). | Y | Krzysiak-Warzala B., 2019. Study N° 21/2017/BA-AD | Accepted Active substance content and stability of packaging were tested. The change in content of a.s. was 0.01%. The a.s. content was determined with a method validated according to SANCO/3030/99 rev.5 (see dRR Part B5). All physical and chemical properties remained stable after the test and accepted. Test carried out in HDPE/PA coex bottles. The packaging remained stable (no corrosion, no leakage, malformation, no change in appearance, goodl condition of closure and seal). Storage stability – 2 years. |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Not relevant. | - | - | Statement accepted |
| Wettability (KCP 2.8.1) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Persistence of foaming (KCP 2.8.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Suspensibility (KCP 2.8.3.1) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Spontaneity of dispersion (KCP 2.8.3.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Dispersion stability (KCP 2.8.3.3) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 170 | Iron phosphate 2.9% GB, Batch no SCL-58946 | 1st method: x1=1000µm where rx=0.08% x2=2000µm where rx=99.80% x1=1000µm where Rx≥99% x2=2000µm where Rx≥99% 2nd method: 99.80% of the particles have size between 2000-3350 µm 0.08% of the particles have size between 1000-2000µm | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Wet sieve test (KCP 2.8.5.1.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Dust content (KCP 2.8.5.2.1) | CIPAC MT 171 | Iron phosphate 2.9% GB, Batch no SCL-58946 | Dust content is 0.3 mg; thus sample is nearly dust free. | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Particle size of dust (KCP 2.8.5.2.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Attrition (KCP 2.8.5.3) | CIPAC MT 178 | Iron phosphate 2.9% GB, Batch no SCL-58946 | 99.66% | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Emulsifiability (KCP 2.8.6.1) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Emulsion stability (KCP 2.8.6.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Flowability (KCP 2.8.7.1) | CIPAC MT 172 | Iron phosphate 2.9% GB, Batch no SCL-58946 | After storage procedure (54°C, 14 days, pressure on the sample of 25g/cm²)sample drops through the sieve spontaneously | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Pourability (KCP 2.8.7.2) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not requiered for GB formulation. | - | - | Statement accepted |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | No recommended tank mixes. |  |  | Statement accepted |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | No recommended tank mixes.. | - | - | Statement accepted |
| Adhesion to seeds (KCP 2.10.1) | - | - | Not a seed treatment formulation . | - | - | Statement accepted |
| Distribution to seed (KCP 2.10.2) | - | - | Not a seed treatment formulation. | - | - | Statement accepted |
| Other/special studies (KCP 2.11.1) | CIPAC MT 190 ASTM E394-15 | Iron phosphate 2.9% GB. Batch no SCL-58946 | Amount of FePO4 released at time -15 minutes is 58 µg/g -30 minutes is 94 µg/g -180 minutes is 538 µg/g Percentage FePO4 relative to the FePO4 content of the formulation-extracted at time: -15 minutes is 0.22% -30 minutes is 0.35% -180 minutes is 2.20% | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |
| Other/special studies (KCP 2.11.2) | ISCO Procedure No.BA-AH/PB-01 XRF Spectrometry | Iron phosphate 2.9% GB, Batch no SCL-58946 | Active substance content 2.65% (w/w) 26.5 g/kg | Y | Krzysiak-Warzala B., 2017. Study N° 20/2017/BA-AD | Accepted |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

| zRMS comments: | During accelerated storage and ambient stability tests, the formulation was stored in commercial packaging (1 litre HDPE/PA bottles). The packaging remained stable. According to SANCO/10473/2003 –rev.5, for granular formulations the extrapolation is possible between all container types. The proposed packaging is accepted. |
| --- | --- |

Table 4.1-1:	Packaging information for 250 g (500 mL bottle)

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Bottle / approx. 75 mm x 75 mm x 161 mm |
| Opening: | approx. 42.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 500 g (750 mL bottle)

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Bottle / approx. 85 mm x 85 mm x 186 mm |
| Opening: | approx. 42.5 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 1.75 kg (2.5 litre bottle)

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Bottle / approx. 122 mm x 122 mm x 258 mm |
| Opening: | approx. 69 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 3.5 kg (5.5 litre bottle)

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Bottle / approx. 150 mm x 150 mm x 335 mm |
| Opening: | approx. 69 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 5 kg (7.4 litre bottle)

| Type | Description |
| --- | --- |
| Material: | PP |
| Shape/size: | Rounded cube / approx. 174 mm x 255 mm |
| Opening: | approx. 255 mm inner diameter |
| Closure: | PP cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

Table 4.1-6:	Packaging information for 10 kg (13.75 litre bottle)

| Type | Description |
| --- | --- |
| Material: | PP |
| Shape/size: | Rounded cube / approx. 303 mm x 260 mm |
| Opening: | approx. 260 mm inner diameter |
| Closure: | PP cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

RMS Comments:

Recommended packagings have been accepted

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1 KCP 2.2.1 KCP 2.2.2 KCP 2.3.2 KCP 2.3.3 KCP 2.4.2 KCP 2.6.2 KCP 2.7.1 KCP 2.8.5.1.1 KCP 2.8.5.2.1 KCP 2.8.5.3 KCP 2.8.7.1 KCP 2.11.1 KCP 2.11.2 | Krzysiak-Warzala B. | 2017 | Iron phosphate 2.9% GB: Analysis of active substances content and physicochemical properties of initial preparation and preparation after accelerated storage procedure, Institute of heavy Organic Synthesis, Blachownia Study N° 20/2017/BA-AD GLP Unpublished | N | Sharda Cropchem Limited |
| KCP 2.2.1 | Buczkowski D. | 2017 | Iron phosphate 2.9% GB: Determination of explosive properties Institute of industrial organic chemistry Study N° BW-03/17 GLP Unpublished | N | Sharda Cropchem Limited |
| KCP 2.7.5 | Krzysiak-Warzala B. | 2019 | Iron phosphate 2.9% GB: Evaluation of the product after storage in accorance with the CropLife Technical Monograph No.17 (6months, 1year, 2 years) Institute of heavy Organic Synthesis, Blachownia Study N° 21/2017/BA-AD GLP Unpublished | N | Sharda Cropchem Limited |
| KCP 2.7.6 | Krzysiak-Warzala B. | 2018 | Iron phosphate 2.9% GB: Evaluation of the product after storage in accorance with the CropLife Technical Monograph No.17 (6months, 1year, 2 years) Institute of heavy Organic Synthesis, Blachownia Study N° 21/2017/BA-AD/1 GLP Unpublished | N | Sharda Cropchem Limited |
| KCP 2.7.6 | Krzysiak-Warzala B. | 2018 | Iron phosphate 2.9% GB: Evaluation of the product after storage in accorance with the CropLife Technical Monograph No.17 (6months, 1year, 2 years) Institute of heavy Organic Synthesis, Blachownia Study N° 21/2017/BA-AD/2 GLP Unpublished | N | Sharda Cropchem Limited |
| KCP 2.7.1 | K. Vasu | 2023 | Method validation and determination of relevant impurities lead, mercury and cadmium in Iron Phosphate 2.9% GB. Bioscience Research Foundation Report No. 11256/2022 GLP Unpublished | N | Sharda Cropchem Limited |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Additional data on the physical, chemical and technical properties of the active substance

  - Ferric phosphate

Not required.