FINAL REGISTRATION REPORT Part B Section 8 Environmental Fate Detailed summary of the risk assessment

Product code: CHR/H/IZOXACYP 250 SC Product name(s): Metida Plus 250 SC, Taizza Plus 250 SC Chemical active substance(s): Isoxaflutole, 250 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Innvigo Sp. Z o. o. Submission date: April 2022 MS Finalisation date: September 2022; August 2023

Version history

| When | What |
| --- | --- |
| April 2022 | Initial Invigo submisson |
| September 2022 | ZRMS assessment dRR for comments |
| August 2023 | Final version of RR after commenting period. |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	5

8.1	Critical GAP and overall conclusions	6

8.2	Metabolites considered in the assessment	8

8.3	Rate of degradation in soil (KCP 9.1.1)	9

8.3.1	Aerobic degradation in soil (KCP 9.1.1.1)	9

8.3.1.1	Isoxaflutole and its metabolites	9

8.3.2	Anaerobic degradation in soil (KCP 9.1.1.1)	10

8.4	Field studies (KCP 9.1.1.2)	10

8.4.1	Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)	10

8.4.1.1	Isoxaflutole and its metabolites	10

8.4.2	Soil accumulation testing (KCP 9.1.1.2.2)	12

8.5	Mobility in soil (KCP 9.1.2)	12

8.5.1	Isoxaflutole and its metabolites	12

8.5.3	Column leaching (KCP 9.1.2.1)	13

8.5.4	Lysimeter studies (KCP 9.1.2.2)	14

8.5.5	Field leaching studies (KCP 9.1.2.3)	14

8.6	Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)	14

8.6.1	Isoxaflutole and its metabolites	14

8.7	Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)	15

8.7.1	Justification for new endpoints	15

8.7.2	Active substance(s) and relevant metabolite(s)	15

8.7.2.1	Isoxaflutole and its metabolites	16

8.7.2.3	PECsoil of Izoxaflutole 250 SC	17

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	18

8.8.1	Justification for new endpoints	18

8.8.2	Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)	18

8.8.2.1	Isoxaflutole and its metabolites	19

8.9	Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)	20

8.9.1	Justification for new endpoints	20

8.9.2	Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)	20

8.9.2.1	Isoxaflutole and its metabolites	21

8.9.2.3	PECsw/sed of Izoxaflutole 250 SC	27

8.10	Fate and behaviour in air (KCP 9.3, KCP 9.3.1)	27

Appendix 1	Lists of data considered in support of the evaluation	28

Appendix 2	Detailed evaluation of the new Annex II studies	31

A 2.1	Study 1	31

Appendix 3	Additional information provided by the applicant (e.g. detailed modelling data)	31

# Fate and behaviour in the environment (KCP 9)

Data matching studies for isoxaflutole have been evaluated by RMS and as a result of the assessment all reports were accepted and considered as equivalent to protected studies. Therefore, to support the authorization of CHR/H/IZOXACYP 250 SC (Taizza Plus 250 SC, Metida Plus 250 SC) INNVIGO is allowed to refer to EU approved reports.

- zRMS comments:

Report dRR was prepared by Applicant. All comments and conclusions of the zRMS are presented in grey. Minor changes are introduced directly in the text and highlighted in grey. Not agreed or not relevant information is struck through and shaded for transparency.

## Critical GAP and overall conclusions

Table 8.1-1:	Critical use pattern of the formulated product

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha | Conclusion |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | L product/ha a) max. rate per appl. b) max. total rate per crop/season | kg as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  | Groundwater |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PL | Maize, sweet corn | F | Mono and dicots weeds | Spray | Pre-emergence BBCH 00-09 | a)1 b)1 | N/A | a)0,4 b)0,4 | a)0,1 b)0,1 | 200-300 | N/A | N/A |  |
| 2 | PL | Maize, sweet corn | F | Mono and dicots weeds | Spray | Post-emergence BBCH 11-13 | a)1 b)1 | N/A | a)0,4 b)0,4 | a)0,1 b)0,1 | 200-300 | N/A | N/A |  |
| 3 | HU, RO | Maize, sweet corn | F | Mono- and dictos weeds | Spray | Pre-emergence BBCH 00-09 | a)1 b)1 | N/A | 0.26 - 0.4 L product/ha | 0.65 - 0.1 kg as/ha | 200-300 | N/A | 3 |  |
| 4 | HU, RO | Maize, sweet corn | F | Mono- and dictos weeds | Spray | Post-emergence BBCH 11-13 | a)1 b)1 | N/A | 0.26 - 0.4 L product/ha | 0.65 - 0.1 kg as/ha | 200-300 | N/A | 4 |  |
| Interzonal uses (use as seed treatment, in greenhouses (or other closed places of plant production), as post-harvest treatment or for treatment of empty storage rooms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor uses according to Article 51 (zonal uses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minor uses according to Article 51 (interzonal uses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

Explanation for column 15 “Conclusion”

| A | Safe use |
| --- | --- |
| R | Further refinement and/or risk mitigation measures required |
| C | To be confirmed by cMS |
| N | No safe use |

Table 8.1-2:	Assessed (critical) uses during approval of isoxaflutole concerning the Section Environmental Fate

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

## Metabolites considered in the assessment

Table 8.2-3:	Metabolites of isoxaflutole potentially relevant for exposure assessment

| Metabolite | Molar mass | Chemical structure | Maximum observed occurence in compartements | Exposue assessment required due to |
| --- | --- | --- | --- | --- |
| RPA202248 | 359.32 g/mol |  | Soil: 100% Groundwater: 100% (ff isoxaflutole to RPA202248 = 1) Surface water and sediment: 70,3% (water: 63,9)% | PECsoil: the risk to soil organisms was concluded as low; PECgw: the risk to aquatic organisms was concluded as low; PECsw: the risk to aquatic organisms was concluded as low |
| RPA203328 | 268.22 g/mol |  | Soil: 90,02% Groundwater: 100% (ff isoxaflutole to RPA 203328 = 1) Surface water and sediment: 10,8% | PECsoil: the risk to soil organisms was concluded as low; PECgw: the risk to aquatic organisms was concluded as low; PECsw: high risk to aquatic organisms could not be excluded |
| RPA205834 | 359.32 g/mol |  | Surface water and sediment: 26,4% | PECsw: the risk to aquatic organisms was concluded as low |

zRMS comments:

The metabolites of  isoxaflutole are in line EFSA Journal 2016;14(3):4416.

## Rate of degradation in soil (KCP 9.1.1)

Studies on degradation in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Aerobic degradation in soil (KCP 9.1.1.1)

#### The rates of dissipation and degradation in the environmental matrices investigated were estimated using FOCUS (2006) kinetics guidance. In soil laboratory incubations under aerobic conditions in the dark, isoxaflutole exhibited very low to low persistence (transformation being quicker with increasing pH), forming the major (>10% applied radioactivity (AR)) metabolites RPA 202248 (max. 96.4% AR sum of isomers, expected to be racemic as the evidence is that its formation is primarily a chemical hydrolysis) which exhibited moderate persistence being transformed to RPA 203328 (max. 90% AR), which exhibited low to very high persistence.

Table 8.3-4:	Summary of aerobic degradation rates for isoxaflutole - laboratory studies

| Isoxaflutole, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (x) | pH (x)a | t.oC | MWHC % | DT50 (d) DT90 (d) # | DT50 (d) 20°C pF2/10kPab | Chi2 (%) | Kinetic model $ | Evaluated on EU level y/n/ Reference |
| 95/02 (Ferreira et al., 1996) | Loam | 4.8 | 20oC | 60% | 3.9 18.6 | 4 | 9.3 | SFO | EFSA Journal 2016;14(3):4416 |
| 95/04 (Burr, 1996) | Sandy loam | 5.3 | 20oC | 60.28% | 2.4 8.1 | 2.4 | 3.8 | SFO |  |
| 95/05 (Ferreria et al., 1996) | Clay loam | 8.2 | 20oC | 77.79% | 0.28 0.94 | 0.4 | 4.9 | SFO |  |
| 95/06 (Ferreira et al., 1996) | Loamy Sand | 7.4 | 20oC | 38.82% | 0.57 1.9 | 0.4 | 4.9 | SFO |  |
| 95/02 10oC (Ferreira et al., 1996) | Loam | 4.8 | 10oC | 59.91% | 5.9 35.8 | - | 10.4 | SFO |  |
| Geometric mean/Median (n=x) |  |  |  |  |  | 0.9 |  |  |  |
| pH-dependency: y/n |  |  |  |  |  | Yes |  |  |  |
| a measured In water b normalised using a Q10 of 2.58 and Walker equation coefficient of 0.7 # non-normalised for trigger evaluation $ for modelling purposes |  |  |  |  |  |  |  |  |  |

Table 8.3-5:	Summary of aerobic degradation rates for RPA202248 - laboratory studies

| RPA202248, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (x) | pHa) (x) | t.oC MWHC % | DT50 (d) DT90 (d) # | DT50 (d) 20°C pF2/10kPab) | Chi2 (%) | Kinetic model $ | Evaluated on EU level y/n/ Reference |
| Hoefchen am Hohenseh (Heinemann, 2012) | Silty Loam | 6.5 | 20 oC / 51.8% | 14 47 | 14.2 | 5.1 | SFO | EFSA Journal 2016;14(3):4416 |
| 95/02 (Ferreira et al., 1996) | Loam | 4.8 | 20 oC / 60% | 43.4 144 | 44.9 | 11. 3 | SFO+SFO |  |
| 95/04 (Burr, 1996) | SandyLoam | 5.3 | 20 oC / 60.28% | 10.4 34.4 | 10.4 | 14.7 | SFO+SFO |  |
| 95/05 (Ferreira et al., 1996) | Clay Loam | 8.2 | 20°C/77.79 % | 41.7 139 | 27.8 | 5.6 | SFO+SFO |  |
| 95/06 (Ferreira et al., 1996) | Loamy Sand | 7.4 | 20°C/38.82% | 29.4/97.6 | 21 | 13. 5 | SFO+SFO |  |
| 95/02 10°C (Ferreira and Jones, 1996) | Loam | 4.8 | 100C/59.91 | 44.8 149 | - | 17 | SFO+SFO |  |
| Geometric mean/Median (n=x) |  |  |  |  | 20.8 |  |  |  |
| pH-dependency: y/n |  |  |  |  | No |  |  |  |
| - Measured in water - Normalised using a Q10 of 2.58 and Walker equation coefficient of 0.7 # non-normalised for trigger evaluation $ for modelling purposes |  |  |  |  |  |  |  |  |

Table 8.3-3:	Summary of aerobic degradation rates for RPA203328 - laboratory studies

| RPA203328, Laboratory studies, aerobic conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil name | Soil type (x) | pHa) (x) | t.oC MWHC % | DT50 (d) DT90 (d) # | DT50 (d) 20°C pF2/10kPab) | Chi2 (%) | Kinetic model $ | Evaluated on EU level y/n/ Reference |
| 95/02 (Ferreira et al., 1996) | Loam | 4.8 | 20 oC / 60% | 204 578 | 210 | 3.8 | SFO+SFO+SFO | EFSA Journal 2016;14(3):4416 |
| 95/04 (Burr, 1996) | Sandy loam | 5.3 | 20 oC / 60.28% | >1000 >1000 | 1000 | 4.8 | SFO+SFO+SFO |  |
| 95/05(Ferreira et al., 1996) | Clay Loam | 4.8 | 20oC/77.79% | 3.5 11.6 | 2.3 | 16.9 | SFO+SFO+SFO |  |
| 95/06 (Ferreira et al., 1996) | Loamy Sand | 7.4 | 20°C/38.82% | >1000 >1000 | 1000 | 19.4 | SFO+SFO+SFO |  |
| 95/02 10°C (Ferreira and Jones, 1996) | Loam | 4.8 | 10oC/59.91% | 112 379 | - | 4.8 | SFO+SFO+SFO |  |
| Geometric mean/Median (n=x) |  |  |  |  | 148.2 |  |  |  |
| pH-dependency: y/n |  |  |  |  | No |  |  |  |
| - Measured in water - Normalised using a Q10 of 2.58 and Walker equation coefficient of 0.7 # non-normalised for trigger evaluation $ for modelling purposes |  |  |  |  |  |  |  |  |

### Anaerobic degradation in soil (KCP 9.1.1.1)

In anaerobic incubations isoxaflutole exhibited very low persistence forming the major transformation products RPA 205834 (max. 28% AR at 0.25 days) and RPA 202248 (max. 83% AR, sum of isomers, at 274 days).

Table 8.3-4: Rate of degradation in soil (anaerobic) laboratory studies active substance (Regulation (EU) N° 283/2013, Annex Part A, point 7.1.2.1.3 and Regulation (EU) N° 284/2013, Annex Part A, point 9.1.1.1) – reference: EFSA Journal 2016;14(3):4416

Table 8.3-5: Rate of degradation in soil (anaerobic) of RPA 202248 (laboratory studies)  – reference: EFSA Journal 2016;14(3):4416

Table 8.3-6: Rate of degradation in soil (anaerobic) of RPA 203328 (laboratory studies)  – reference: EFSA Journal 2016;14(3):4416

#### zRMS comments:

#### The anaerobic degradation rates for isoxaflutole and its metabolites are in line EFSA Journal 2016;14(3):4416.

## Field studies (KCP 9.1.1.2)

### Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)

The field dissipation of isoxaflutole has been evaluated, full details of these studies are provided in the respective EU DRAR and related documents and summarised in the EFSA Journal 2016;14(2):4416).  No additional studies have been performed.

Table 8.4-6:	Summary of aerobic degradation rates for isoxaflutole - field studies: Triggering endpoints [EFSA Journal 2016;14(3):4416]

Table 8.4-7:

Summary of aerobic degradation rates for RPA202248 - field studies: Triggering endpoints [EFSA Journal 2016;14(3):4416]

Table 8.4-3: Summary of aerobic degradation rates for RPA203328 - field studies: Triggering endpoints [EFSA Journal 2016;14(3):4416]

#### zRMS comments:

#### The field degradation rates for Isoxaflutole and its metabolites studies are in line EFSA Journal 2016;14(3):4416.

### Soil accumulation testing (KCP 9.1.1.2.2) [EFSA Journal 2016;14(3):4416]

## Mobility in soil (KCP 9.1.2)

Studies on mobility in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Isoxaflutole and its metabolites

Table 8.5-8:	Summary of soil adsorption/desorption for isoxaflutole [EFSA Journal 2016;14(3):4416]

Table 8.5-9:	Summary of soil adsorption/desorption for RPA202248 [EFSA Journal 2016;14(3):4416]

Table 8.5-3:	Summary of soil adsorption/desorption for RPA203328 [EFSA Journal 2016;14(3):4416]

### Column leaching (KCP 9.1.2.1) [EFSA Journal 2016;14(3):4416]

Active substance - isoxaflutole

Transformation products

- zRMS comments:

- The column leaching study for isoxaflutole is in line EFSA Journal 2016;14(3):4416.

### Lysimeter studies (KCP 9.1.2.2) [EFSA Journal 2016;14(3):4416]

### Field leaching studies (KCP 9.1.2.3) [EFSA Journal 2016;14(3):4416]

## Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)

Studies on degradation in water/sediment systems with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

### Isoxaflutole and its metabolites

Table 8.6-10:	Summary of degradation in water/sediment of isoxaflutole [EFSA Journal 2016;14(3):4416]

Table 8.6-11:	Summary of observed metabolites: a) RPA 202248, b) RPA 205834 [EFSA Journal 2016;14(3):4416]

a)

- zRMS comments:

- The degradation sudies in water/sediment of isoxaflutole and its metabolites  are in line EFSA Journal 2016;14(3):4416.

## Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)

### Justification for new endpoints

No new endpoints were established.

### Isoxaflutole and relevant metabolites

Table 8.6-12:	Input parameters related to application for PECsoil calculations

| Use No. | 1 | 2 |
| --- | --- | --- |
| Crop | Maize, sweet corn | Maize, sweet corn |
| Application rate (g as/ha) | Isoxaflutole: 100 g as/ha RPA202248: 100 g as/ha RPA203328: 67,2 g as/ha | Isoxaflutole: 100 g as/ha RPA202248: 100 g as/ha RPA203328: 67,2 g as/ha |
| Number of applications/interval | 1 | 1 |
| Crop interception (%) | 0% | 25% |
| Depth of soil layer (relevant for plateau concentration) (cm) | 5/20/x cm (no tillage/tillage) | 5/20/x cm (no tillage/tillage) |

Table 8.6-13:	Input parameter for isoxaflutole and its metabolites for PECsoil calculation

| Compound | Molecular weight (g/mol) | Max. occurrence (%) | DT50 (days) | Value in accordance to EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- |
| Parent: isoxaflutole | 359.32 g/mol | 100% | 3.9 days | Y EFSA Journal 2016;14(3):4416 |
| Metabolite 1: RPA202248 | 359,32 g/mol | 100% | 53.4 days | Y EFSA Journal 2016;14(3):4416 |
| metabolite 2: RPA203328 | 268,22 g/mol | 90,02% | None | Y EFSA Journal 2016;14(3):4416 |

#### Isoxaflutole and its metabolites

Notice: PECsoil for metabolite RPA203328 and PECsoil for CHR/H/IZOXACYP are determined using following formulas:

Determined from following formulas:

PECsoil = app. Rate∙(1-crop interception)/ d* q * 150;

app. Rate for metabolite = Mmetabolite/Mparent ∙ PECsoil for parent ∙ max. occurrence

d = 5 cm

q= 1.5 g/ml

Crop interception: 0% (worst case)

because this is the worst case for this formulation and RPA203328 have not DT50 value determined.

Table 8.6-14:	PECsoil for isoxaflutole on maize – crop interception 0%

| PECsoil (mg/kg) |  | maize |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0,1333 | - | - | - |
| Short term | 24h | 0,1116 | 0,1225 | - | - |
|  | 2d | 0,0934 | 0,1125 | - | - |
|  | 4d | 0,0655 | 0,0957 | - | - |
| Long term | 7d | 0,0384 | 0,0765 | - | - |
|  | 14d | 0,0111 | 0,0493 | - | - |
|  | 21d | 0,0032 | 0,0350 | - | - |
|  | 28d | 0,0009 | 0,0267 | - | - |
|  | 50d | <0,0001 | 0,0160 | - | - |
|  | 100d | <0,0001 | 0,0075 | - | - |
| Plateau concentration (5 cm) after year 10. |  | <0,0001 | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | 0,1333 |  | - |  |

PECsoil of metabolites

Table 8.6-15:	PECsoil for RPA202248 on maize – crop interception 0%

| PECsoil (mg/kg) |  | maize |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0,1092 | - | - | - |
| Short term | 24h | 0,1091 | 0,1092 | - | - |
|  | 2d | 0,1087 | 0,1092 | - | - |
|  | 4d | 0,1076 | 0,1090 | - | - |
| Long term | 7d | 0,1050 | 0,1087 | - | - |
|  | 14d | 0,0947 | 0,1072 | - | - |
|  | 21d | 0,0894 | 0,1051 | - | - |
|  | 28d | 0,0817 | 0,1025 | - | - |
|  | 50d | 0,0615 | 0,0931 | - | - |
|  | 100d | 0,0321 | 0,0733 | - | - |
| Plateau concentration (5 cm) after year 10. |  | 0,0010 | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | 0,1102 | - | - |  |

Table 8.6-5:		PECsoil for RPA203328 on maize –occurrence

Determined from following formulas:

PECsoil = app. Rate∙(1-crop interception)/ d* q * 150;

app. Rate for metabolite = Mmetabolite/Mparent ∙ PECsoil for parent ∙ max. occurrence

d = 5 cm

q= 1.5 g/ml

Crop interception: 0% (worst case)

| PECsoil (mg/kg) |  | maize |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single application |  | Multiple applications |  |
|  |  | Actual | TWA | Actual | TWA |
| Initial |  | 0,0896 | - | - | - |
| Short term | 24h | - | - | - | - |
|  | 2d | - | - | - | - |
|  | 4d | - | - | - | - |
| Long term | 7d | - | - | - | - |
|  | 14d | - | - | - | - |
|  | 21d | - | - | - | - |
|  | 28d | - | - | - | - |
|  | 50d | - | - | - | - |
|  | 100d | - | - | - | - |
| Plateau concentration (5 cm) after year 10. |  | <0,0001 | - | - | - |
| PECaccumulation (PECact +PECsoil plateau) |  | 0,0896 | - | - |  |

#### PECsoil of CHR/H/IZOXACYP 250 SC

Table 8.6-6:	PECsoil for CHR/H/IZOXACYP 250 SC on maize – determined by the formula: PECsoil = app. Rate∙(1-crop interception)/750

Determined from following formulas:

PECsoil = app. Rate∙(1-crop interception)/ d* q * 150;

app. Rate= 473.6 g/ha

d = 5 cm

q= 1.5 g/ml

Crop interception: 0% (worst case)

| Active substance/ reparation | Application rate (g/ha) | PECact (mg/kg) | PECtwa21 d (mg/kg) | Tillage depth (cm) | PECsoil,plateau (mg/kg) | PECaccu = PECact + PECsoil,plateau (mg/kg) |
| --- | --- | --- | --- | --- | --- | --- |
| CHR/H/IZOXACYP | 473,6 | 0,6315 | - | 5 | <0,0001 | 0,6315 |

## Based on the density of the formulation

zRMS comments:

The calculations of PECs for active substance isoxaflutole (1x100g/ha) and its metabolites submitted  by the Applicant for the  proposed use in maize  has been accepted. Calculation  for RPA202248 metabolite were remade by zRMS in the dRR proposed by the applicant.

All input parameters agreed by ZRMS for isoxaflutole and metabolites are stated in the Table 8.6-2. Agreed endpoints in line EFSA Journal 2016;14(3):4416 were used as input parameters.

Calculations were performed according FOCUS EU guidance.  Calculations were performed for  proposed  use in GAP. The interception according to FOCUS is 0% (worst case) for maize, growth stage BBCH 00.

The following PECs values can be used in the risk assessment:

Isoxaflutole: 0.1333 (mg/kg)

RPA202248: 0.1092 (mg/kg)

RPA203328: 0.0896 (mg/kg)

CHR/H/IZOXCYP: 0.6315 (mg/kg)

## Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)

### Justification for new endpoints

No new endpoints were established.

### Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)

Table 8.8-16:	Input parameters related to application for PECgw calculations

| Use No. | 1 | 2 |
| --- | --- | --- |
| Crop | Maize, sweet corn | Maize, sweet corn |
| Application rate (g as/ha) | Isoxaflutole: 100 g as/ha | Isoxaflutole: 100 g as/ha |
| Number of applications/interval (d) | 1 | 1 |
| Relative application date | 15 days before emergence | 4 days after emergence |
| Crop interception (%) | 0% | 25% |
| Frequency of application | Annual, Triennial | Annual, Triennial |
| Models used for calculation | FOCUS PEARL v4.4.4, FOCUS PELMO v5.5.3, FOCUS MACRO v5.5.3 | FOCUS PEARL v4.4.4, FOCUS PELMO v5.5.3, FOCUS MACRO v5.5.3 |

#### Isoxaflutole and its metabolites

Table 8.8-2:	Input parameters related to active substance isoxaflutole and its metabolites for PECgw calculations

| Compound | Isoxaflutole | RPA202248 | RPA203328 | Value in accordance with EU endpoint y/n/ Reference |
| --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 359,32 g/mol | 359,32 g/mol | 268 g/mol | Y - EFSA Journal 2016;14(2):4416 |
| Water solubility (g/mol): | 6,2 mg/L | 6,2 mg/L | 6,2 mg/L | Y - EFSA Journal 2016;14(2):4416 |
| Saturated vapour pressure (Pa): | 3,22 x 10-7 Pa (20oC) | N/D | N/D | Y - EFSA Journal 2016;14(2):4416 |
| DT50 in soil (d) |  |  |  |  |
| DT50 in soil (d) lab/field | 0.9 d (geomean, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =0.7) | 15.8 d (geomean, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =0.7) | 12 d (geomean, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =0.7) | Y - EFSA Journal 2016;14(2):4416 |
| Transformation rate | 0.770164 per day | 0.04387 per day | 0.057762 per day |  |
| Kfoc (mL/g)/Kfom | 79.8 mL/g (arithmetic mean)/46.3 mL/g | 12.3 mL/g (arithmetic mean, pH>6.5)/7.1 mL/g | 0 (arithmetic mean)/0 | Y - EFSA Journal 2016;14(2):4416 |
| 1/n | 0.935 (arithmetic mean) | 0.89 (arithmetic mean) | 1 (arithmetic mean) | Y - EFSA Journal 2016;14(2):4416 |
| Plant uptake factor | 0 | Tier 1: 0 Tier 2: 0.5* | Tier 1: 0 Tier 2: 0.5* | Y - EFSA Journal 2016;14(2):4416 |
| Formation fraction | - | 1 from isoxaflutole | 1 from RPA202248 | Y - EFSA Journal 2016;14(2):4416 |

*	In accordance to peer review: EFSA Journal 2016;14(2):4416

FOCUS PEARL Tier 1

Table 8.8-3:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,1 kg as/ha yearly pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize - pre-emergence | Châteaudun | <0.000001 | 0.189847 | 0.949965 |
|  | Hamburg | <0.000001 | 0.702662 | 3.524463 |
|  | Kremsmünster | <0.000001 | 0.465484 | 1.365440 |
|  | Okehampton | <0.000001 | 0.564882 | 1.749203 |
|  | Piacenza | <0.000001 | 0.161631 | 0.544553 |
|  | Porto | <0.000001 | 0.060059 | 0.284710 |
|  | Sevilla | <0.000001 | 0.002661 | 0.021775 |
|  | Thiva | <0.000001 | 0.046102 | 0.220052 |

Table 8.8-4:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,08 kg as/ha yearly pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.000001 | 0.146236 | 0.753838 |
|  | Hamburg | <0.000001 | 0.541036 | 2.811781 |
|  | Kremsmünster | <0.000001 | 0.362082 | 1.083712 |
|  | Okehampton | <0.000001 | 0.441830 | 1.393255 |
|  | Piacenza | <0.000001 | 0.125090 | 0.431741 |
|  | Porto | <0.000001 | 0.045932 | 0.225692 |
|  | Sevilla | <0.000001 | 0.002035 | 0.017290 |
|  | Thiva | <0.000001 | 0.035655 | 0.174257 |

Table 8.8-5:	 PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,06 kg as/ha yearly pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.000001 | 0.104646 | 0.558737 |
|  | Hamburg | <0.000001 | 0.385585 | 2.101219 |
|  | Kremsmünster | <0.000001 | 0.261615 | 0.804468 |
|  | Okehampton | <0.000001 | 0.321537 | 1.038976 |
|  | Piacenza | <0.000001 | 0.089790 | 0.320027 |
|  | Porto | <0.000001 | 0.032127 | 0.163065 |
|  | Sevilla | <0.000001 | 0.001441 | 0.012840 |
|  | Thiva | <0.000001 | 0.025565 | 0.128964 |

Table 8.8-6:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,1 kg as/ha 3 years interval pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize - pre-emergence | Châteaudun | <0.000001 | 0.080225 | 0.370465 |
|  | Hamburg | <0.000001 | 0.222791 | 1.024934 |
|  | Kremsmünster | <0.000001 | 0.147389 | 0.590645 |
|  | Okehampton | <0.000001 | 0.230260 | 0.646107 |
|  | Piacenza | <0.000001 | 0.091425 | 0.256530 |
|  | Porto | <0.000001 | 0.015055 | 0.083893 |
|  | Sevilla | <0.000001 | 0.001014 | 0.009207 |
|  | Thiva | <0.000001 | 0.013912 | 0.071544 |

Table 8.8-7:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,08 kg as/ha 3 years interval pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.000001 | 0.061522 | 0.294247 |
|  | Hamburg | <0.000001 | 0.171192 | 0.817526 |
|  | Kremsmünster | <0.000001 | 0.113308 | 0.469969 |
|  | Okehampton | <0.000001 | 0.179148 | 0.514777 |
|  | Piacenza | <0.000001 | 0.070951 | 0.203211 |
|  | Porto | <0.000001 | 0.011581 | 0.066618 |
|  | Sevilla | <0.000001 | 0.000775 | 0.007332 |
|  | Thiva | <0.000001 | 0.010664 | 0.056579 |

Table 8.8-8:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,06 kg as/ha 3 years interval pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.000001 | 0.043611 | 0.218621 |
|  | Hamburg | <0.000001 | 0.121679 | 0.610768 |
|  | Kremsmünster | <0.000001 | 0.080601 | 0.350009 |
|  | Okehampton | <0.000001 | 0.129486 | 0.384028 |
|  | Piacenza | <0.000001 | 0.051113 | 0.150456 |
|  | Porto | <0.000001 | 0.008245 | 0.049480 |
|  | Sevilla | <0.000001 | 0.000547 | 0.005466 |
|  | Thiva | <0.000001 | 0.007556 | 0.041800 |

Table 8.8-9:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,1 kg as/ha yearly post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize - post-emergence | Châteaudun | <0.000001 | 0.204356 | 0.922851 |
|  | Hamburg | <0.000001 | 0.714728 | 3.507243 |
|  | Kremsmünster | <0.000001 | 0.349750 | 1.178578 |
|  | Okehampton | <0.000001 | 0.595036 | 1.620850 |
|  | Piacenza | <0.000001 | 0.126218 | 0.437534 |
|  | Porto | <0.000001 | 0.040050 | 0.239767 |
|  | Sevilla | <0.000001 | 0.003298 | 0.025249 |
|  | Thiva | <0.000001 | 0.060890 | 0.273692 |

Table 8.8-10:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,08 kg as/ha yearly post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize –post-emergence | Châteaudun | <0.000001 | 0.136472 | 0.641917 |
|  | Hamburg | <0.000001 | 0.472947 | 2.501229 |
|  | Kremsmünster | <0.000001 | 0.256164 | 0.849886 |
|  | Okehampton | <0.000001 | 0.451036 | 1.166583 |
|  | Piacenza | <0.000001 | 0.073316 | 0.291669 |
|  | Porto | <0.000001 | 0.030556 | 0.168288 |
|  | Sevilla | <0.000001 | 0.001897 | 0.017196 |
|  | Thiva | <0.000001 | 0.038123 | 0.180398 |

Table 8.8-11:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,1 kg as/ha 3 years interval post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize - post-emergence | Châteaudun | <0.000001 | 0.076312 | 0.347727 |
|  | Hamburg | <0.000001 | 0.225857 | 0.959628 |
|  | Kremsmünster | <0.000001 | 0.123244 | 0.477463 |
|  | Okehampton | <0.000001 | 0.265877 | 0.642814 |
|  | Piacenza | <0.000001 | 0.050371 | 0.183924 |
|  | Porto | <0.000001 | 0.014003 | 0.082174 |
|  | Sevilla | <0.000001 | 0.001328 | 0.011238 |
|  | Thiva | <0.000001 | 0.020165 | 0.091563 |

Table 8.8-12:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,08 kg as/ha in 3 years interval post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Châteaudun | <0.000001 | 0.053999 | 0.256406 |
|  | Hamburg | <0.000001 | 0.154519 | 0.710651 |
|  | Kremsmünster | <0.000001 | 0.101361 | 0.383002 |
|  | Okehampton | <0.000001 | 0.161059 | 0.434774 |
|  | Piacenza | <0.000001 | 0.029722 | 0.113077 |
|  | Porto | <0.000001 | 0.008494 | 0.056946 |
|  | Sevilla | <0.000001 | 0.000796 | 0.007331 |
|  | Thiva | <0.000001 | 0.012231 | 0.060260 |

FOCUS PEARL Tier 2

Table 8.8-13:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,08 kg as/ha in 3 years interval pre-emergence – plant uptake factor for metabolites: 0.5

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.000001 | 0.038416 | 0.157242 |
|  | Hamburg | <0.000001 | 0.114467 | 0.463531 |
|  | Kremsmünster | <0.000001 | 0.081740 | 0.305731 |
|  | Okehampton | <0.000001 | 0.117215 | 0.298962 |
|  | Piacenza | <0.000001 | 0.041389 | 0.114705 |
|  | Porto | <0.000001 | 0.006278 | 0.029878 |
|  | Sevilla | <0.000001 | 0.000092 | 0.001074 |
|  | Thiva | <0.000001 | 0.004078 | 0.019453 |

Table 8.8-14:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,08 kg as/ha in 3 years interval post-emergence – plant uptake factor for metabolites: 0.5

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Châteaudun | <0.000001 | 0.032592 | 0.139039 |
|  | Hamburg | <0.000001 | 0.105585 | 0.422786 |
|  | Kremsmünster | <0.000001 | 0.076220 | 0.258601 |
|  | Okehampton | <0.000001 | 0.107413 | 0.249972 |
|  | Piacenza | <0.000001 | 0.017394 | 0.058844 |
|  | Porto | <0.000001 | 0.004254 | 0.021547 |
|  | Sevilla | <0.000001 | 0.000088 | 0.001075 |
|  | Thiva | <0.000001 | 0.004863 | 0.021704 |

Table 8.8-15:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0,06 kg as/ha in 3 years interval pre-emergence – plant uptake factor for metabolites: 0.5

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.000001 | 0.027471 | 0.117100 |
|  | Hamburg | <0.000001 | 0.081737 | 0.347490 |
|  | Kremsmünster | <0.000001 | 0.058383 | 0.228304 |
|  | Okehampton | <0.000001 | 0.085263 | 0.223876 |
|  | Piacenza | <0.000001 | 0.030089 | 0.085178 |
|  | Porto | <0.000001 | 0.004501 | 0.022165 |
|  | Sevilla | <0.000001 | 0.000066 | 0.000816 |
|  | Thiva | <0.000001 | 0.002916 | 0.014455 |

Table 8.8-16:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0.07 kg as/ha per 3 years - pre-emergence – Hamburg

| Maize - pre-emergence | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
|  | Hamburg | <0.000001 | 0.098558 | 0.396438 |

Table 8.8-17:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0.075 kg as/ha per 3 years - post-emergence – Hamburg

| Maize - post-emergence | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
|  | Hamburg | <0.000001 | 0.098526 | 0.396309 |

Table 8.8-18:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0.07 kg as/ha per 3 years –pre-emergence – Okehampton

| Maize - pre-emergence | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
|  | Okehampton | <0.000001 | 0.089768 | 0.241150 |

Table 8.8-19:	PECgw for isoxaflutole and its metabolites on maize (with FOCUS PEARL 5.5.5) – dosage 0.075 kg as/ha per 3 years – post-emergence – Okehampton

| Maize - post-emergence | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
|  | Okehampton | <0.000001 | 0.099972 | 0.268562 |

FOCUS PELMO Tier 1

Table 8.8-20:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,1 kg as/ha yearly pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize - pre-emergence | Châteaudun | <0.001 | 0.136 | 0.589 |
|  | Hamburg | <0.001 | 0.369 | 2.086 |
|  | Kremsmünster | <0.001 | 0.470 | 1.575 |
|  | Okehampton | <0.001 | 0.585 | 1.679 |
|  | Piacenza | <0.001 | 0.198 | 0.656 |
|  | Porto | <0.001 | 0.064 | 0.300 |
|  | Sevilla | <0.001 | 0.006 | 0.053 |
|  | Thiva | <0.001 | 0.030 | 0.111 |

Table 8.8-21:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,08 kg as/ha yearly pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.105 | 0.468 |
|  | Hamburg | <0.001 | 0.288 | 1.663 |
|  | Kremsmünster | <0.001 | 0.361 | 1.249 |
|  | Okehampton | <0.001 | 0.457 | 1.337 |
|  | Piacenza | <0.001 | 0.154 | 0.521 |
|  | Porto | <0.001 | 0.049 | 0.237 |
|  | Sevilla | <0.001 | 0.004 | 0.042 |
|  | Thiva | <0.001 | 0.023 | 0.088 |

Table 8.8-22:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,06 kg as/ha yearly pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.075 | 0.349 |
|  | Hamburg | <0.001 | 0.209 | 1.242 |
|  | Kremsmünster | <0.001 | 0.257 | 0.926 |
|  | Okehampton | <0.001 | 0.332 | 0.996 |
|  | Piacenza | <0.001 | 0.110 | 0.387 |
|  | Porto | <0.001 | 0.035 | 0.175 |
|  | Sevilla | <0.001 | 0.003 | 0.031 |
|  | Thiva | <0.001 | 0.017 | 0.065 |

Table 8.8-23:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,1 kg as/ha every 3 year pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.050 | 0.238 |
|  | Hamburg | <0.001 | 0.139 | 0.729 |
|  | Kremsmünster | <0.001 | 0.160 | 0.651 |
|  | Okehampton | <0.001 | 0.266 | 0.670 |
|  | Piacenza | <0.001 | 0.081 | 0.249 |
|  | Porto | <0.001 | 0.014 | 0.076 |
|  | Sevilla | <0.001 | 0.002 | 0.018 |
|  | Thiva | <0.001 | 0.008 | 0.042 |

Table 8.8-24:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,08 kg as/ha every 3 year pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.038 | 0.189 |
|  | Hamburg | <0.001 | 0.107 | 0.581 |
|  | Kremsmünster | <0.001 | 0.124 | 0.519 |
|  | Okehampton | <0.001 | 0.207 | 0.533 |
|  | Piacenza | <0.001 | 0.063 | 0.197 |
|  | Porto | <0.001 | 0.011 | 0.060 |
|  | Sevilla | <0.001 | 0.001 | 0.015 |
|  | Thiva | <0.001 | 0.006 | 0.033 |

Table 8.8-25:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,06 kg as/ha every 3 year pre-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.027 | 0.140 |
|  | Hamburg | <0.001 | 0.077 | 0.433 |
|  | Kremsmünster | <0.001 | 0.089 | 0.387 |
|  | Okehampton | <0.001 | 0.149 | 0.396 |
|  | Piacenza | <0.001 | 0.046 | 0.146 |
|  | Porto | <0.001 | 0.008 | 0.044 |
|  | Sevilla | <0.001 | 0.001 | 0.011 |
|  | Thiva | <0.001 | 0.004 | 0.025 |

Table 8.8-26:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,1 kg as/ha yearly post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Châteaudun | <0.001 | 0.164 | 0.702 |
|  | Hamburg | <0.001 | 0.373 | 2.166 |
|  | Kremsmünster | <0.001 | 0.474 | 1.469 |
|  | Okehampton | <0.001 | 0.704 | 1.871 |
|  | Piacenza | <0.001 | 0.178 | 0.681 |
|  | Porto | <0.001 | 0.043 | 0.224 |
|  | Sevilla | <0.001 | 0.005 | 0.047 |
|  | Thiva | <0.001 | 0.039 | 0.142 |

Table 8.8-27:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,08 kg as/ha yearly post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Châteaudun | <0.001 | 0.126 | 0.558 |
|  | Hamburg | <0.001 | 0.289 | 1.727 |
|  | Kremsmünster | <0.001 | 0.366 | 1.165 |
|  | Okehampton | <0.001 | 0.546 | 1.491 |
|  | Piacenza | <0.001 | 0.138 | 0.542 |
|  | Porto | <0.001 | 0.033 | 0.178 |
|  | Sevilla | <0.001 | 0.004 | 0.038 |
|  | Thiva | <0.001 | 0.030 | 0.112 |

Table 8.8-28:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,1 kg as/ha every 3 year post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – po.e. | Châteaudun | <0.001 | 0.056 | 0.268 |
|  | Hamburg | <0.001 | 0.156 | 0.802 |
|  | Kremsmünster | <0.001 | 0.175 | 0.636 |
|  | Okehampton | <0.001 | 0.288 | 0.709 |
|  | Piacenza | <0.001 | 0.069 | 0.232 |
|  | Porto | <0.001 | 0.016 | 0.068 |
|  | Sevilla | <0.001 | 0.001 | 0.019 |
|  | Thiva | <0.001 | 0.011 | 0.052 |

Table 8.8-29:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,08 kg as/ha every 3 year post-emergence

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Châteaudun | <0.001 | 0.043 | 0.213 |
|  | Hamburg | <0.001 | 0.120 | 0.639 |
|  | Kremsmünster | <0.001 | 0.136 | 0.506 |
|  | Okehampton | <0.001 | 0.224 | 0.565 |
|  | Piacenza | <0.001 | 0.054 | 0.185 |
|  | Porto | <0.001 | 0.012 | 0.054 |
|  | Sevilla | <0.001 | 0.001 | 0.015 |
|  | Thiva | <0.001 | 0.008 | 0.042 |

FOCUS PELMO Tier 2

Table 8.8-30:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,08 kg as/ha every 3 year pre-emergence with plant uptake factor 0.5 for metabolites

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.019 | 0.084 |
|  | Hamburg | <0.001 | 0.049 | 0.211 |
|  | Kremsmünster | <0.001 | 0.069 | 0.228 |
|  | Okehampton | <0.001 | 0.125 | 0.287 |
|  | Piacenza | <0.001 | 0.034 | 0.108 |
|  | Porto | <0.001 | 0.005 | 0.036 |
|  | Sevilla | <0.001 | <0.001 | 0.002 |
|  | Thiva | <0.001 | 0.001 | 0.008 |

Table 8.8-31:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,08 kg as/ha every 3 year post-emergence with plant uptake factor 0.5 for metabolites

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Châteaudun | <0.001 | 0.018 | 0.078 |
|  | Hamburg | <0.001 | 0.046 | 0.212 |
|  | Kremsmünster | <0.001 | 0.063 | 0.230 |
|  | Okehampton | <0.001 | 0.129 | 0.279 |
|  | Piacenza | <0.001 | 0.022 | 0.064 |
|  | Porto | <0.001 | 0.005 | 0.021 |
|  | Sevilla | <0.001 | <0.001 | 0.001 |
|  | Thiva | <0.001 | 0.002 | 0.010 |

Table 8.8-32:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,06 kg as/ha every 3 year pre-emergence with plant uptake factor 0.5 for metabolites

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Châteaudun | <0.001 | 0.014 | 0.063 |
|  | Hamburg | <0.001 | 0.036 | 0.160 |
|  | Kremsmünster | <0.001 | 0.050 | 0.170 |
|  | Okehampton | <0.001 | 0.091 | 0.214 |
|  | Piacenza | <0.001 | 0.025 | 0.080 |
|  | Porto | <0.001 | 0.003 | 0.027 |
|  | Sevilla | <0.001 | <0.001 | 0.001 |
|  | Thiva | <0.001 | 0.001 | 0.006 |

Table 8.8-33:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,065 kg as/ha every 3 year pre-emergence with plant uptake factor 0.5 for metabolites in Okehampton scenario

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – pre-emergence | Okehampton | <0.001 | 0.099 | 0.250 |

Table 8.8-34:	PECgw for isoxaflutole and its metabolites on crop (with FOCUS PELMO 6.6.4) – dosage 0,065 kg as/ha every 3 year post-emergence with plant uptake factor 0.5 for metabolites in Okehampton scenario

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Isoxaflutole | RPA202248 | RPA203328 |
| Maize – post-emergence | Okehampton | <0.001 | 0.094 | 0.211 |

PECgw values for application rates 0.75, 0.70, 0.065 per year and per 3 years pre-emergence and post-emergence were calculated because of trigger value 0.1 µg/L of PEC for isoxaflutole and RPA 202248. In tier 1 plant uptake factor for metabolites was accepted as 0, in tier 2 – 0.5.

zRMS comments:

The calculations of PECgw for active substance isoxaflutole and its metabolites submitted  by the Applicant for the  proposed use in maize  has been accepted.  Calculations were performed according FOCUS EU guidance.  Calculations were performed for  proposed  use in GAP. The interception according to FOCUS is 0% (maize, growth stage BBCH 00). Agreed endpoints in line EFSA Journal 2016;14(3):4416 were used as input parameters.

The PECgw for isoxaflutole are ˂ 0.1 µg/L. However, its  metabolite exceed trigger value 0.1 g /L.

Assessment of relevance of ground water metabolite is performed and presented in section B 10 of dRR.

## Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)

### Justification for new endpoints

No new endpoints were established.

### Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)

Table 8.9-17:	Input parameters related to application for PECSW/SED calculations

| Plant protection product | CHR/H/IZOXACYP 250 SC | CHR/H/IZOXACYP 250 SC |
| --- | --- | --- |
| Use No. | 1 | 2 |
| Crop | Maize, sweet corn | Maize, sweet corn |
| Application rate (kg as/ha) | 0,1 kg as/ha | 0,1 kg as/ha |
| Number of applications/interval (d) | 1 | 1 |
| Application window | March-May (relevant for STEP 1 and 2 only) | April-June (relevant for STEP 1 and 2 only) |
| Application method | Spray | Spray |
| CAM (Chemical application method) | Cannopy | Cannopy |
| Soil depth (cm) | 5 cm/20 cm | 5 cm/20 cm |
| Models used for calculation | FOCUS SWASH v3.1, FOCUS PRZM v3.3.1, FOCUS MACRO v5.5.3, FOCUS TOXWA v3.3.1 | FOCUS SWASH v3.1, FOCUS PRZM v3.3.1, FOCUS MACRO v5.5.3, FOCUS TOXWA v3.3.1 |

Table 8.9-18:	FOCUS Step 3 Scenario related input parameters for PECsw/sed calculations for the application of product

| Crop | Scenario | Application window used in modelling |
| --- | --- | --- |
|  | D3 | pre-emergence: 25.04-25.05; post-emergence: 9.05-8.06 |
|  | D4 | pre-emergence: 30-04-30.05; post-emergence: 14.05-13.06 |
|  | D5 | pre-emergence: 30.04-30.05; post-emergence: 13.05-12.06 |
|  | D6 | pre-emergence: 10.04-10.05; post-emergence: 23.04-23.05 |
|  | R1 | pre-emergence: 23.04-23.05; post-emergence: 7.05-6.06 |
|  | R2 | pre-emergence: 21.04-21.05; post-emergence: 6.05-5.06 |
|  | R3 | pre-emergence: 21.04-21.05; post-emergence: 5.05-4.06 |
|  | R4 | pre-emergence: 31.03-30.04; post-emergence: 13.04-13.05 |

#### Isoxaflutole and its metabolites

Table 8.9-19:	Input parameters related to active substance isoxaflutole and its metabolites for PECsw/sed calculations STEP 1/2 and 3(/4) (if necessary)

| Compound | Isoxaflutole |  | RPA202248 | RPA203328 |  | RPA205834 |  | Value in accordance to EU endpoint y/n/ Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Molecular weight (g/mol) | 359,32 g/mol |  | 359,32 g/mol | 268,22 g/mol |  | 361,34 g/mol |  | EFSA Journal 2016;14(2):4416 |  |
| Saturated vapour pressure (Pa) | not required for Step 1+2/3,22 x 10-7 Pa at 20oC |  | 0 Pa at 20oC | 0 |  | 0 |  | EFSA Journal 2016;14(2):4416 |  |
| Water solubility (mg/L) | 6,2 mg/L |  | 22600 mg/L | 6,2 mg/L |  | 6,2 mg/L |  | EFSA Journal 2016;14(2):4416 |  |
| Diffusion coefficient in water (m²/d) | not required for Step 1+2/ 4.3 x 10-5 |  | not required for Step 1+2/ 4.3 x 10-5 | -not required for Step 1+2/ 4.3 x 10-5 |  | -not required for Step 1+2/ 4.3 x 10-5 |  | default |  |
| Diffusion coefficient in air (m²/d) | not required for Step 1+2/0.43 |  | not required for Step 1+2/0.43 | not required for Step 1+2/0.43 |  | not required for Step 1+2/0.43 |  | default |  |
| Kfoc (mL/g) | 79.8 mL/g(arithmetic mean) |  | 12.3 mL/g (arithmetic mean) | 0 |  | 0 |  | EFSA Journal 2016;14(2):4416 |  |
| Freundlich Exponent 1/n | 0.935 (arithmetic mean) |  | 0.89 (arithmetic mean) | 1 |  | 1 |  | EFSA Journal 2016;14(2):4416 |  |
| Plant Uptake | not required for Step 1+2/0 |  | 0.5 | 0 |  | 0 |  | EFSA Journal 2016;14(2):4416 |  |
| Wash-Off factor from Crop (1/mm) | not required for Step 1+2/ 0.05 (MACRO) 0.50 (PRZM) |  | not required for Step 1+2/ 0.05 (MACRO) 0.50 (PRZM) | not required for Step 1+2/ 0.05 (MACRO) 0.50 (PRZM) |  | not required for Step 1+2/ 0.05 (MACRO) 0.50 (PRZM) |  | default |  |
| DT50,soil (d) | 0.9 d (geomean, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =0.7) |  | 15.8 d (geomean, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =0.7) | 12 d (geomean, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =0.7) |  | 1000 days (default) |  | EFSA Journal 2016;14(2):4416 |  |
| DT50,water (d) | 0.36d |  | 237d | 1000 days (default) |  | 76.2d |  | EFSA Journal 2016;14(2):4416 |  |
| DT50,sed (d) | 0.36 d 1000 d (step 3) |  | 237d 1000d (step 3) | 1000 days (default) |  | 76.2d |  | EFSA Journal 2016;14(2):4416 |  |
| DT50,whole system (d) | 0.36d |  | 237d | 1000 days (default) |  | 76.2d |  | EFSA Journal 2016;14(2):4416 |  |
| Maximum occurrence observed (% molar basis with respect to the parent) |  | - | Soil: 100% Water: 63.9% Sediment: N/D Total system: 70.3% | Soil: 62% Water: N/D Sediment: N/D Total system: 10.8% |  | Soil: 2.3% Water: N/D Sediment: N/D Total system: 26.4% |  | EFSA Journal 2016;14(2):4416 |  |
| Formation fraction in soil: |  | - | 1 fom parent |  | N/D |  | N/D |  | EFSA Journal 2016;14(2):4416 |

Regulatory acceptable concentration (RAC)

Parent (isoxaflutole): 0.1 µg/L (Americamysis bahia)

Met. 1. RPA202248: 5.5 µg/L (Lemna gibba)

Met. 2. RPA203328: 980 µg/L (Lemna gibba)

Met. 3. RPA205834: 110 µg/L (Lemna gibba)

PECsw/sed

Table 8.9-20:	FOCUS Step 1,2 and 3 PECsw and PECsed for isoxaflutole following single application of CHR/H/IZOXACYP 250 SC to maize

| Scenario FOCUS | Waterbody | Max PECsw (μg/L)* | Dominant entry route | 21 d- PECsw,twa (µg/L)** | Max PECsed (μg/kg)* |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 31.0474 | Drainage/Run-off | 0.9584 | 24.0419 |
| Step 2 |  |  |  |  |  |
| Pre-emergence | March-May | 0.9197 | Drainage/Run-off | 0.04 | 0.2211 |
| Post-emergence | March-May | 0.9197 | Drainage/Run-off | 0.04 | 0.1658 |
| Step 3 |  |  |  |  |  |
| D3 | ditch | 0.5247 | Drainage | 0.01348 | 0.06200 |
| D4 | pond | 0.02117 | Drainage | 0.001150 | 0.003993 |
| D4 | stream | 0.4493 | Drainage | 0.001832 | 0.02099 |
| D5 | pond | 0.02116 | Drainage | 0.000926 | 0.003383 |
| D5 | stream | 0.4478 | Drainage | 0.000887 | 0.01142 |
| D6 | ditch | 0.5249 | Drainage | 0.01557 | 0.06624 |
| R1 | pond | 0.02116 | Run-off | 0.001556 | 0.004386 |
| R1 | stream | 0.3625 | Run-off | 0.004185 | 0.02888 |
| R2 | stream | 0.4819 | Run-off | 0.001999 | 0.02224 |
| R3 | stream | 0.5128 | Run-off | 0.007733 | 0.05499 |
| R4 | stream | 0.3624 | Run-off | 0.009507 | 0.04311 |

* 	single applications should be marked.

**	twa-time as required by ecotox

FOCUS Step 4

Table 8.9-21:	Global maximum PECsw values for isoxaflutole, following single application of CHR/H/IZOXACYP 250 SC to maize according to the central EU zone GAP according to surface water Step 4

| PECsw (µg/L) | Scenario | Isoxaflutole 100 g as/ha |  |
| --- | --- | --- | --- |
| Nozzle reduction | Vegetative strip (m) | 10 | 20 |
|  | No spray buffer (m) | 10 | 20 |
| None | D3 ditch | 0.09118 | 0.04740 |
| None | D4 pond | 0.01358 | 0.009086 |
| None | D4 stream | 0.1002 | 0.05221 |
| None | D5 pond | 0.01358 | 0.009085 |
| None | D5 stream | 0.09984 | 0.05202 |
| None | D6 ditch | 0.09124 | 0.04744 |
| None | R1 pond | 0.01357 | 0.009083 |
| None | R1 stream | 0.08083 | 0.04212 |
| None | R2 stream | 0.1075 | 0.05599 |
| None | R3 stream | 0.1144 | 0.05959 |
| None | R4 stream | 0.08124 | 0.04255 |

RAC for isoxaflutole: 0.1µg/L

Conclusion: There was necessary to calculate all Steps (1, 2, 3, 4). There was necessary to apply 20 m buffer zone.

Metabolites of active substance

Table 8.9-22:	FOCUS Step 1, 2 and 3 PECsw and PECsed for RPA202248 following single application to maize

| Scenario FOCUS | Waterbody | Max PECsw (μg/L)* | Dominat entry route | 21 d- PECsw,twa (µg/L)** | Max PECsed (μg/kg)* |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 56.4972 | Drainage/Run-off | 54.7874 | 6.8696 |
| Step 2 |  |  |  |  |  |
| Pre-emergence | March-May | 6.3473 | Drainage/Run-off | 6.1531 | 0.7780 |
| Post-emergence | March-May | 4.9185 | Drainage/Run-off | 4.7673 | 0.6028 |
| Step 3 |  |  |  |  |  |
| D3 | ditch | 0.2727 | Drainage | 0.04753 | 0.8194 |
| D4 | pond | 0.09207 | Drainage | 0.09057 | 0.1382 |
| D4 | stream | 0.09882 | Drainage | 0.04574 | 0.05096 |
| D5 | pond | 0.04161 | Drainage | 0.04003 | 0.05447 |
| D5 | stream | 0.08022 | Drainage | 0.01424 | 0.01532 |
| D6 | ditch | 0.2715 | Drainage | 0.03848 | 0 .06438 |
| R1 | pond | 0.02651 | Run-off | 0.02460 | 0.02552 |
| R1 | stream | 1.460 | Run-off | 0.03156 | 0.1250 |
| R2 | stream | 1.160 | Run-off | 0.03164 | 0.1079 |
| R3 | stream | 0.1473 | Run-off | 0.003092 | 0.01154 |
| R4 | stream | 4.526 | Run-off | 0.1807 | 0.4999 |

*	single applications should be marked.

**	twa-time as required by ecotox

RAC for RPA202248: 5.5 µg/L

Conclusion: There is not necessary to calculate Step 4 because Steps 1, 2 and 3 results are lower than RAC.

Table 8.9-7:	FOCUS Step 1, 2 and 3 PECsw and PECsed for RPA203328 following single application to maize

| Scenario FOCUS | Waterbody | Max PECsw (μg/L)* | Dominat entry route | 21 d- PECsw,twa (µg/L)** | Max PECsed (μg/kg)* |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 18.1884 | Drainage/Run-off | 18.0566 | <0.0001 |
| Step 2 Not required |  |  |  |  |  |

RAC for RPA203328: 980 µg/L

Conclusion: There is not necessary to calculate Steps 2, 3 and 4 because Step 1 results are lower than RAC.

Table 8.9-8:	FOCUS Step 1, 2 and 3 PECsw and PECsed for RPA205834 following single application to maize

| Scenario FOCUS | Waterbody | Max PECsw (μg/L)* | Dominat entry route | 21 d- PECsw,twa (µg/L)** | Max PECsed (μg/kg)* |
| --- | --- | --- | --- | --- | --- |
| Step 1 | --- | 9.8646 | Drainage/Run-off | 8.9797 | <0.0001 |
| Step 2 Not required |  |  |  |  |  |

RAC for RPA205834: 110 µg/L

Conclusion: There is not necessary to calculate Steps 2, 3 and 4 because Step 1 results are lower than RAC.

#### PECsw/sed of formulation

zRMS comments:

The calculations of PECsw/sed for active substance isoxaflutole (1x100g/ha) and its metabolites submitted  by the Applicant for the  proposed use in maize  has been accepted.  Calculations were performed according FOCUS EU guidance and by models Step1 & 2 and 3 & 4. Calculations were performed for  proposed  use in GAP. Agreed endpoints in line EFSA Journal 2016;14(3):4416 were used as input parameters. The mitigations measure was considered: 10m and 20m  buffer zone (drift and vegetative) . The mitigation measures based on FOCUS Landscape and Mitigation are considered acceptable for further STEP 4 refinement.

## Fate and behaviour in air (KCP 9.3, KCP 9.3.1)

Table 8.10-23 Summary of atmospheric degradation and behaviour

| Compound | Isoxaflutole |
| --- | --- |
| Direct photolysis in air | Not studied – no data requested due to low volatility |
| Photochemical oxidative degradation in air | Isoxaflutole: DT50 of 1.83 days derived by the computer program AOPWIN (version 1.91) and the methodology derived by Atkinson. OH (12h) concentration assumed = 1.5x106 molecule cm-3 . RPA 202248 DT50: 0.62 – 1.42 days derived by the computer program AOPWIN (version 1.91) and the methodology derived by Atkinson. OH (12h) concentration assumed = 1.5×106 molecule cm-3 . |
| Volatilisation | From plant surfaces (BBA guideline): <1% after 24 hours |
|  | From soil surfaces (BBA guideline): negligible after 24 hours |
| Metabolites | RPA 202248 |

The vapour pressure at 20 °C of the active substance isoxaflutole is < 10-5 Pa. Hence the active substance isoxaflutole is regarded as non-volatile. Therefore exposure of adjacent surface waters and terrestrial ecosystems by the active substance isoxaflutole due to volatilization with subsequent deposition should be considered.

ZRMS comments:

Accepted.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 9.1.3 | Spankowski W. | 2022 | CHR/H/IZOXACYP 250 SC Predicted environmental concentration of isoxaflutole and its metabolites in soil Non-GLP Unpublished | N | Chemirol |
| KCP 9.2.4 | Spankowski W. | 2022 | CHR/H/IZOXACYP 250 SC Predicted environmental concentration of isoxaflutole and its metabolites in ground water Non-GLP Unpublished | N | Chemirol |
| KCP 9.2.5 | Spankowski W. | 2022 | CHR/H/IZOXACYP 250 SC Predicted environmental concentration of isoxaflutole and its metabolites in surface water and sediment Non-GLP Unpublished | N | Chemirol |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCA 7.1.1.1 /01 | Ferreira, E. M.; Jones, M. K.; Newby, S. E. | 1994 | RPA201772: Aerobic soil metabolism Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R000347, Edition Number: M-158435-01-1 Date: 1994-10-13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.1 /02 | Burr, C. M. | 1995 | Herbicides: RPA 201772 Route of degradation (aerobic metabolism) in one soil (interim report) Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C022444, Edition Number: M-213110-01-1 Date: 1995-12-18 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.1 /03 | Burr, C. M. | 1996 | Herbicides: Isoxaflutole Route of Degradation (Aerobic Metabolism) in one Soil Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: B003826, Edition Number: M-240821-01-1 EPA MRID No.: 45658804 Date: 1996-08-12 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.1 /04 | Ferreira, E. M.; Bullus, C. M.; Jones, M. K.; Simmonds, M. B. | 1996 | RPA 201772: Rate of Degradation under Aerobic Conditions in Three Soil Types Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: B003827, Edition Number: M-240822-01-1 EPA MRID No.: 45658805 Date: 1996-01-10 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.1 /05 | Hardy, I. A. J. | 2001 | Isoxaflutole: Kinetic modelling analysis of a European terrestrial field soil dissipation study Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C015653, Edition Number: M-200916-01-1 EPA MRID No.: 45658806 Date: 2001-12-04 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.1.1 /06 | Gatzweiler, E. W. | 1996 | RPA 201772: terrestrial field soil dissipation study Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: C034014, Report includes Trial Nos.: AR53-94 Edition Number: M-234225-01-1 Date: 1996-02-14 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.1 /07 | Heinemann, O. | 2012 | [Cyclopropyl-1-14C]RPA 202248:: Aerobic degradation/metabolism in one European soil Bayer CropScience, Report No.: MEF-11/486, Edition Number: M-427583-01-1 Date: 2012-03-12 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.2/01 | Burr C. M.; Jones, M. K.; Newby, S. E. | 1995 | RPA201772 Anaerobic Aquatic Metabolism Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R016760, Edition Number: M-192288-01-1 Date: 1995-01-30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.1.3 /01 | Ferreira, E. M. | 1994 | RPA201772 - Soil photolysis Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R000311, Edition Number: M-158351-01-1 Date: 1994-02-04 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2 /01 | Hardy, I. A. J. | 2001 | Isoxaflutole: Kinetic modelling analysis of a European terrestrial field soil dissipation study Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C015653, Edition Number: M-200916-01-1 EPA MRID No.: 45658806 Date: 2001-12-04 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2 /02 | Gatzweiler, E. W. | 1996 | RPA 201772: terrestrial field soil dissipation study Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: C034014, Report includes Trial Nos.: AR53-94 Edition Number: M-234225-01-1 Date: 1996-02-14 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.1 /01 | Ferreira, E. M.; Jones, M. K.; Newby, S. E. | 1994 | RPA201772: Aerobic soil metabolism Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R000347, Edition Number: M-158435-01-1 Date: 1994-10-13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.1 /02 | Burr C. M. | 1995 | Herbicides: RPA 201772 Route of degradation (aerobic metabolism) in one soil (interim report) Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C022444, Edition Number: M-213110-01-1 Date: 1995-12-18 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.1 /03 | Burr, C. M. | 1996 | Herbicides: Isoxaflutole Route of Degradation (Aerobic Metabolism) in one Soil Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: B003826, Edition Number: M-240821-01-1 EPA MRID No.: 45658804 Date: 1996-08-12 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.1 /04 | Ferreira, E. M.; Bullus, C. M.; Jones, M. K.; Simmonds, M. B. | 1996 | RPA 201772: Rate of Degradation under Aerobic Conditions in Three Soil Types Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: B003827, Edition Number: M-240822-01-1 EPA MRID No.: 45658805 Date: 1996-01-10 GLP/GEP: yes, unpublishe | N | Bayer CropScience |
| KCA 7.1.2.1.1/05 | Ferreira, E. M. | 1996 | RPA 201772: Rate of degradation under aerobic conditions in one soil at 10 degrees C Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C022445, Edition Number: M-213113-01-1 Date: 1996-01-24 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.1/06 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling evaluation of aerobic soil degradation study data to derive trigger endpoints Battelle UK Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: VC/13/007C, Edition Number: M-464601-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.1/07 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling evaluation of aerobic soil degradation study data to derive modelling endpoints Battelle UK Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: VC/13/007C, Edition Number: M-464601-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/01 | Ferreira, E. M.; Jones, M. K.; Newby, S. E. | 1994 | RPA201772: Aerobic soil metabolism Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R000347, Edition Number: M-158435-01-1 Date: 1994-10-13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/02 | Burr, C. M. | 1995 | Herbicides: RPA 201772 Route of degradation (aerobic metabolism) in one soil (interim report) Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C022444, Edition Number: M-213110-01-1 Date: 1995-12-18 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/03 | Burr, C. M. | 1996 | Herbicides: Isoxaflutole Route of Degradation (Aerobic Metabolism) in one Soil Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: B003826, Edition Number: M-240821-01-1 EPA MRID No.: 45658804 Date: 1996-08-12 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/04 | Ferreira, E. M.; Bullus, C. M.; Jones, M. K.; Simmonds, M. B. | 1996 | RPA 201772: Rate of Degradation under Aerobic Conditions in Three Soil Types Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: B003827, Edition Number: M-240822-01-1 EPA MRID No.: 45658805 Date: 1996-01-10 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/05 | Ferreira, E. M | 1996 | RPA 201772: Rate of degradation under aerobic conditions in one soil at 10 degrees C Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C022445, Edition Number: M-213113-01-1 Date: 1996-01-24 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/06 | Heinemann, O. | 2012 | [Cyclopropyl-1-14C]RPA 202248:: Aerobic degradation/metabolism in one European soil Bayer CropScience, Report No.: MEF-11/486, Edition Number: M-427583-01-1 Date: 2012-03-12 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/07 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling evaluation of aerobic soil degradation study data to derive trigger endpoints Battelle UK Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: VC/13/007C, Edition Number: M-464601-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.2/08 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling evaluation of aerobic soil degradation study data to derive modelling endpoints Battelle UK Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: VC/13/007C, Edition Number: M-464601-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.3 /01 | Burr C. M.; Jones, M. K.; Newby, S. E. | 1995 | RPA201772 Anaerobic Aquatic Metabolism Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R016760, Edition Number: M-192288-01-1 Date: 1995-01-30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.1.4 /01 | Burr C. M.; Jones, M. K.; Newby, S. E. | 1995 | RPA201772 Anaerobic Aquatic Metabolism Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: R016760, Edition Number: M-192288-01-1 Date: 1995-01-30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.2.1/01 | Gatzweiler, E.W. | 1996 | RPA 201772: terrestrial field soil dissipation study Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: C034014, Report includes Trial Nos.: AR53-94 Edition Number: M-234225-01-1 Date: 1996-02-14 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.2.1/02 | Gatzweiler, E.W. | 1996 | RPA 201772: terrestrial field soil dissipation study on four European soils Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: R000410, Report includes Trial Nos.: P94/009 Edition Number: M-158573-01-1 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2.2.1/03 | Heinemann, O. | 2012 | Determination of the residues of AE 0540092 in/on soil after spraying of AE 0540092 WP 20 in the field in Germany, United Kindom, France (North), Italy and Spain Bayer CropScience, Report No.: 10-2702, Report includes Trial Nos.: 10-2702-01 10-2702-02 10-2702-03 10-2702-04 10-2702-05 10-2702-06 Edition Number: M-428564-01-1 Date: 2012-03-19 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.2.1/04 | Kieken, J. L. | 1997 | Storage stability in soils from the European terrestrial field soil dissipation isoxaflutole and metabolites study P94/009 Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: C034021, Report includes Trial Nos.: 95-85 Edition Number: M-234239-01-1 Date: 1997-06-17 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.2.2.1/05 | Hardy, I. A. J. | 2013 | Kinetic modelling analysis of AE 0540092 and AE B197555 from a field soil residue study conducted in Europe (Normalisation to 20degree and pF2) Battelle UK Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: VC/13/007A, Edition Number: M-464592-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.2.2.1/06 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling endpoint summary Battelle UK Ltd., Chelmsford, United Kingdom Bayer CropScience, Report No.: VC/13/007J, Edition Number: M-464899-01-1 Date: 2013-09-18 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.1 /01 | Lowden, P.; Cooper, I.; Simmonds, M.; Burr, C. M.; Newby, S. E. | 1993 | [ 14C]-RPA 201722: Adsorption/desorption to and from four soils and an aquatic sediment - Addendum report Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: M-065963-03-1, Edition Number: M-065963-03-1 Date: 1993-11-25 ...Amended: 2004-04-08 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.1/02 | Burr C. M.; Newby, S. E. | 1993 | RPA 201772: Adsorption / desorption to and from four soils and an aquatic sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000307, Edition Number: M-158343-01-1 Date: 1993-11-25 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.1 /03 | Burr, C. M. | 1996 | 14C)-RPA202248: Adsorption / desorption to and from four soils and an aquatic sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Report No.: R016762, Edition Number: M-192293-01-1 Date: 1996-07-17 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /01 | Lowden, P.; Cooper, I.; Simmonds, M.; Burr, C. M. | 1995 | [ 14C]-RPA 202248: Adsorption/desorption to and from four soils - Addendum report Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom BCS, Report No.: CX/03/070A, Edition Number: M-066355-03-1 Date: 1995-11-20 ...Amended: 2004-04-08 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /02 | Burr, C. M. | 1995 | RPA202248: Adsorption / desorption to and from four soils Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Report No.: R016761, Edition Number: M-192291-01-1 Date: 1995-11-20 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /03 | Burr, C. M. | 1996 | ( 14C)-RPA202248: Adsorption / desorption to and from four soils and an aquatic sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Report No.: R016762, Edition Number: M-192293-01-1 Date: 1996-07-17 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /04 | Burr, C. M. Burrs, C. M. | 1996 | RPA 203328: Adsorption/desorption to and from four soils Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C025651, Edition Number: M-209743-02-1 Date: 1996-01-02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /05 | Burr, C. M. | 1996 | [14C]-RPA203328: Adsorption / desorption to and from four soils and a sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000447, Report includes Trial Nos.: 11487 Edition Number: M-158651-01-1 Date: 1996-07-30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /06 | Lowden, P.; Cooper, I.; Simmonds, M.; Burr, C. M. | 1996 | [ 14C]-RPA 202248: Adsorption/desorption to and from four soils and an aquatic sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: 11486, Edition Number: M-066483-01-1 Date: 1996-07-17 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /07 | Burr, C. M. | 1996 | RPA 203328: Adsorption/desorption to and from four soils Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Report No.: C026341, Edition Number: M-211226-01-1 Date: 1996-01-02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /08 | Hein, H.; Moendel, M. | 2012 | [cyclopropyl-1-14C] AE 0540092: Adsorption/desorption in five different soils RLP AgroScience GmbH, Neustadt a. d. Weinstraße, Germany Bayer CropScience, Report No.: AS200, Edition Number: M-429399-01-1 Date: 2012-03-14 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.3.1.2 /09 | Mills, E. A. M.; Simmonds, M. B. | 2004 | (14C)-RPA 203328: Adsorption / desorption in five soils Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C040534, Edition Number: M-229091-01-1 EPA MRID No.: 46801704 Date: 2004-04-06 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.1.4 /01 | Gatzweiler, E. W. | 1996 | Isoxaflutole: Computer simulation of the behaviour in soil using the PELMO 2.01 model Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000415, Edition Number: M-158583-01-1 Date: 1996-01-24 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.4 /02 | Gatzweiler, E. W. | 1996 | RPA 202248: Computer simulation of the behaviour in soil using the PELMO 2.01 model Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000413, Edition Number: M-158581-01-1 Date: 1996-01-24 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.4 /03 | Hardy, I. A. J. | 2001 | Isoxaflutole: Leaching risk assessment for isoxaflutole and two metabolites using the European FOCUS groundwater scenarios (AE B197278, AE 0540092, AE B197555) Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: MO-04-004596, Edition Number: M-066960-01-1 Date: 2001-09-14 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.1.4.1.1/01 | Newby, S. E.; Godward, P. J.; Jones, M. K. | 1995 | RPA 201772: Aged leaching study in four soils and a sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000361, Edition Number: M-158471-01-1 Date: 1995-01-10 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.1.1 /01 | Corgier, M. M.; Robin, J. M.; Plewa, A. P. | 1994 | Hydrolysis 14C-RPA201772 Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: R002384, Report includes Trial Nos.: 93-180 Edition Number: M-162558-01-1 Date: 1994-05-02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.1.2 /01 | Corgier, M. M.; Plewa, A. P. | 1995 | Photodegradation in water 14CRPA201772 (isoxaflutole) Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: R002507, Report includes Trial Nos.: 94-11 Edition Number: M-162794-01-1 Date: 1995-01-13 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.1.2 /02 | Cousin, J. A. | 1993 | Amendment No. 1 - RPA201772 active ingredient - Physical and chemical characteristics - Part E: Stability Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: R002181, Report includes Trial Nos.: 93-129 R&D/CRLD/AN/9516351 Edition Number: M-162149-02-1 Date: 1993-11-15 ...Amended: 1993-11-15 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.1.2 /03 | Roohi, A.; Caine, J. | 2002 | Photolysis in distilled water RPA202248 Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Report No.: C027919, Report includes Trial Nos.: 36662 CX/02/066 Edition Number: M-214288-01-1 Date: 2002-12-11 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.1 /01 | DesmaresKoopmans, M. J. E. | 1996 | Determination of ready biodegrability: Carbon dioxide (CO2) evolution test (modified Sturm test) with isoxaflutole Notox B.V., 's-Hertogenbosch, Netherlands Bayer CropScience, Report No.: C026345, Edition Number: M-211235-01-1 Date: 1996-01-23 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.1 /02 | DesmaresKoopmans, M. J. E. | 1996 | Determination of 'ready' biodegradability carbon dioxyde (CO2) - Evolution test (modified sturm test) with RPA201772 Notox B.V., 's-Hertogenbosch, Netherlands Bayer CropScience, Report No.: R002745, Report includes Trial Nos.: 58077 96-22 Edition Number: M-163261-01-1 Date: 1996-01-23 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.2 /01 | Walther, D. | 2013 | [14C] Isoxaflutole: Aerobic mineralization in surface water Harlan Laboratories Ltd., Itingen, Switzerland Bayer CropScience, Report No.: D62847, Edition Number: M-459354-02-1 Date: 2013-06-26 ...Amended: 2013-07-02 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.3 /01 | Ayliffe, J. M.; Newby, S. E. | 1995 | RPA 201772: Degradation and retention in two water / sediment systems Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000377, Edition Number: M-158508-01-1 Date: 1995-05-04 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.3 /02 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling evaluation of water sediment degradation study data to derive total system DT50 values Battelle UK Ltd., Ongar, United Kingdom Bayer CropScience, Report No.: VC/13/008A, Edition Number: M-464901-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.2.2.3 /03 | Hardy, I. A. J. | 2013 | Isoxaflutole: Kinetic modelling evaluation of water sediment degradation study data to derive water phase DT50 values Battelle UK Ltd., Ongar, United Kingdom Bayer CropScience, Report No.: VC/13/008B, Edition Number: M-464903-01-1 Date: 2013-07-17 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.2.2.4 /01 | Ayliffe, J. M.; Newby, S. E. | 1995 | RPA 201772: Degradation and retention in two water / sediment systems Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000377, Edition Number: M-158508-01-1 Date: 1995-05-04 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.4 /02 | Lowden, P.; Cooper, I.; Simmonds, M.; Burr, C. M. | 1996 | [ 14C]-RPA 202248: Adsorption/desorption to and from four soils and an aquatic sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: 11486, Edition Number: M-066483-01-1 Date: 1996-07-17 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.2.2.4 /03 | Burr, C. M. | 1996 | [ 14C]-RPA203328: Adsorption / desorption to and from four soils and a sediment Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000447, Report includes Trial Nos.: 11487 Edition Number: M-158651-01-1 Date: 1996-07-30 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.3.1 /01 | Maestracci, M. P | 1996 | Isoxaflutole - Estimation of the rate of photochemical transformation in the atmosphere under tropospheric conditions Rhone-Poulenc Agro, Lyon, France Bayer CropScience, Report No.: C022447, Report includes Trial Nos.: 95-116 Edition Number: M-213115-01-1 Date: 1996-01-08 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.3.1 /02 | Kubiak R. | 1997 | Investigation of the volatilization of 14C-isoxaflutole formulated corresponding to EXP31130A from plant and soil surfaces under laboratory conditions SLFA Neustadt, Neustadt, Germany Bayer CropScience, Report No.: R014793, Report includes Trial Nos.: 97-18 97-19 Edition Number: M-189768-01-1 Date: 1997-02-26 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 7.3.1 /03 | Buntain, I. G. | 2003 | Isoxaflutole: Estimation of degradation by photo-oxidation in air Model calculation according to Atkinson Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C038406, Report includes Trial Nos.: CX/03/081 Edition Number: M-224922-01-1 Date: 2003-12-03 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.3.1 /04 | Buntain, I. G. | 2003 | RPA202248: Estimation of degradation by photo-oxidation in air Model calculation according to Atkinson Battelle AgriFood Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C038405, Report includes Trial Nos.: CX/03/080 Edition Number: M-224920-01-1 Date: 2003-12-03 GLP/GEP: no, unpublished | N | Bayer CropScience |
| KCA 7.4.1 /01 | Hampton, R. E.; Cappy, J. J. | 1995 | Rationale for site of 14C-labelling of isoxaflutole (RPA201772) Supporting data Rhone-Poulenc Ag Company, RTP, NC, USA Bayer CropScience, Report No.: R004910, Edition Number: M-166773-01-1 GLP/GEP: no, unpublished | N | Bayer CropScience |

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP XX | Author | YYYY | Title Company Report N Source GLP/non GLP/GEP/non GEP Published/Unpublished | Y/N | Owner |

- Detailed evaluation of the new Annex II studies

  - Study 1

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

Results and discussions

Conclusion

- Additional information provided by the applicant (e.g. detailed modelling data)