FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: CHR/H/IZOXACYP 250 SC Product name(s): Metida Plus 250 SC, Taizza Plus 250 SC Chemical active substance: Isoxaflutole, 250 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Innvigo Sp. z o. o. Submission date: April 2022 Update: February 2024 MS Finalisation date: November 2022; August 2023; March 2024; April 2024

Version history

| When | What |
| --- | --- |
| November 2022 | Assessment by ZRMS |
| August 2023 | Final version of RR after commenting period. |
| February 2024 | Applicant’s update (information about tests after the 2nd year of storage and product specification) |
| March 2024 | zRMS assessment of Applicant’s update |
| April 2024 | zRMS assessment of Applicant’s proposal of technical specification |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	4

1.2.3.1	Active substance 1	4

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	5

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	22

4	Section 4: Further information on the plant protection product	23

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	23

Appendix 1	Lists of data considered in support of the evaluation	47

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	56

A 2.1	Isoxaflutole	56

This document was prepared to add information regarding the 2nd year of storage and to add product specifications. New data is highlighted in yellow.

Sufficient data on identity, physical and chemical properties and other information are not available for the plant protection product and the contained technical active substance(s).

Noticed data gaps are:

- 2-years ambient temperature stability none

# Section 1: Identity of the plant protection product

Data matching studies for isoxaflutole have been evaluated by RMS and as a result of the assessment all reports were accepted and considered as equivalent to protected studies. Therefore, to support the authorization of CHR/H/IZOXACYP 250 SC (Taizza Plus 250 SC, Metida Plus 250 SC) INNVIGO is allowed to refer to EU approved reports.

## Applicant (KCP 1.1)

Name:	Innvigo Sp. Z o. o.

Address:	178, Aleje Jerozolimskie

02-486 Warsaw , Poland

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Active substance 1

| Isoxaflutole | min. 972 g/kg |
| --- | --- |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Metida Plus 250 SC (or) |
| --- | --- |
| Trade name: | Taizza Plus 250 SC |
| Company code number: | CHR/H/IZOXACYP 250 SC |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Isoxaflutole | 250 g/L | 235-265 g/L | 250 257.2 g/L | 21,1 21.7% |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

** 	Based on the density of the formulation = 1.184  (Note: only applies if a liquid formulation – delete this comment if not needed)

Table 1.4-2:	Safener and synergists

| Safener / synergist | Declared content of the safener / synergist (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Cyprosulfamide | 250 g/L | 235-265 g/L | - | 21,1% |

* 	Based on the minimum purity of the safener/synergist declared for registration

** 	Based on the density of the formulation = 1.184  (Note: only applies if a liquid formulation – delete this comment if not needed)

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-3:	Information on isoxaflutole

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Isoxaflutole |
| CAS No. | 141112-29-0 |
| EC No. | Not Allocated |
| CIPAC No. | 575 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

Table 1.4-4:	Information on safeners/synergists

| Type | Name/Code number |
| --- | --- |
| Safener | Cyprosulfamide |
| ISO common name | Cyprosulfamide |
| CAS No. | 221667-31-8 |
| EC No. | Not yet assessed at EU level 485-320-2 |

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Suspension concentrate | [Code: SC] |
| --- | --- |

## Function (KCP 1.6)

Herbicide (H)

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable. The appearance of the product is that of light-brown, with a delicate odour. It is not explosive, has no oxidising properties. The product is not flammable. It has not a self auto-ignition temperature determined of 610°C. In aqueous solution, it has a pH value around 4.3 at 20 °C. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0 °C and 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The stability data after 1 2 years of storage indicate a shelf life of at least 1 2 years at ambient temperature when stored in HDPE. Its technical characteristics are acceptable for a herbicide formulation.

The intended concentration of use is 0.1% to 0.2%.

The 2-year storage stability studies on CHR/H/IZOXACYP 250 SC are ongoing.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

It is not necessary

Notifier Proposals for Risk and Safety Phrases (KCP 12)

Compliance with FAO specifications:

The product CHR/H/IZOXACYP 250 SC complies with FAO specifications.

At the time of evaluation, no FAO specification was allocated for formulations containing isoxaflutole.

Formulation used for tests

Izoxaflutole 250 SC (CHR/H/IZOXACYP 250 SC)

Date of production: 01.09.2020

Batch No: 202002

Technical specification of the product in accordance with FAO/WHO Manual.

The following table presents technical specifications as listed in the “Manual on development and use of FAO and WHO specifications for pesticides”, Third Revision, November 2016.

The product Taizza Plus 250 SC / Metida Plus 250 SC / Izoxacyp 250 SC containing active substance izoxaflutole 250  g/kg g/L and safener cyprosulfamide 250 g/L in the formulation of suspension concentrate (SC). Studies on the physicochemical properties of the preparation were conducted and are presented in full in this section of the draft Registration Report.

| No. | Quality parameters | Method | Required |
| --- | --- | --- | --- |
| 1. | Appearance | Visual | Homogeneous liquid with a characteristic odor, color ranging from white to light brown |
| 2. | Content of isoxaflutole, g/l | HPLC | 250 ± 15 |
| 3. | Content of cyprosulfamide, g/l | HPLC | 250 ± 15 |
| 4. | Density 20⁰C, g/ml | CIPAC MT 3 | 1,185 ± 0,02 1.1839 ± 0.02 |
| 5. | pH in 1% (m/v) suspension in distilled water, 20⁰C | CIPAC MT 75.3 | 4.0 – 6.0 4.0 – 5.0 |
| 6. | Suspensibility Stability of 0.1 and 0.2% (v/v) suspension in water CIPAC D after 0.5h in 30⁰C, min. content determined with chemical method, min., % | CIPAC MT 184 | 95 |
| 7. | Particle size Dv(50), µm | CIPAC MT 187 | 1,5-2,5 |
| 7. | Pourability, max. residue, % | CIPAC MT 148.1 | 5 |
| 8. | Spontaneity of dispersion after 5 min in CIPAC standard water D at 30 ± 2°C, min. content determined with chemical method, % | CIPAC MT 160 | Isoxaflutole - 90 Cyprosulfamide - 90 |
| 8. 9. | Residue on the wet sieve 75µm, max., % | CIPAC MT 185 | 0.1 |
| 9. 10. | Foaming of 0.2% (m/v) suspension in CIPAC D water after 1 minute, max., ml | CIPAC MT 47.3 | 20 |

| zRMS comment | Applicant submitted proposal for technical specification for formulation Taizza Plus 250 SC / Metida Plus 250 SC. According to “Manual on development and use of FAO and WHO specifications for pesticides”, the parameters which best describe the performance characteristics of SC formulations are: - Pourability test (to ensure that the SC can be poured from its container), - Water dispersibility (spontaneity of dispersion), suspensibility, wet sieve and persistent foam tests (to ensure the sprayability of the diluted suspension). The proposed required values for parameters a.s. and safener content, suspensibility, pourability, spontaneity of dispersion, residues on wet sieve, and persistent foam are all within limits set by SANCO/10473/2003, rev.5. Therefore, they are acceptable. In case of density, the deviation proposed by the Applicant (± 0.02) results from previous experiences of Applicant with SC formulations (results of density tests of various batches of SC formulations). It is therefore acceptable. In case of pH, in the expert’s opinion, the range proposed by the Applicant (4.0-6.0) is too large and should be narrowed down to 4.00-5.00. |
| --- | --- |

Table 2-5:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings |  |  |  | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | OPPTS 830.6302 830.6303 830.6304 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial preparation: Colour (Munsell’s notation) - 5YR 9/1 (light-brown) Physical state – liquid Odour – delicate After accelerated storage test (2 weeks at 54oC): Colour (Munsell’s notation) – 5YR 9/1 (light-brown) Odour – characteristic, delicate After shelf-life test (1 year, 20oC): Colour (Munsell’s notation) - 5YR 9/1 (light-brown) Physical state – liquid Odour – characteristic, delicate After shelf-life test (2 year, 20oC) Colour (Munsell’s notation) - 5YR 9/1 (light-brown) Physical state – liquid Odour – characteristic, delicate |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | Accepted. |
| Explosive properties (KCP 2.2.1) | EC A.14 | CHR/H/IZOXACYP 250 SC | No explosive properties |  |  |  | Y | Buczkowski D. BW-13/21 | In thermal sensitivity test no explosion occurred. In the mechanical sensitivity test (shock) no explosion occurred. Accepted. |
| Oxidizing properties (KCP 2.2.2) | EC A.21 | CHR/H/IZOXACYP 250 SC | No oxidizing properties |  |  |  | Y | Flasińska P. BC-18/21 | Accepted. |
| Flash point (KCP 2.3.1) | EEC A.9 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Flashpoint was not observed. The test flame was extinguished by vapours of the test item at temperatures 72°C. |  |  |  | Y | Patrzałek M. ICB/11/2021 | The closed cup method was used (Pensky-Martens apparatus). The preparation is not classified as flammable. Accepted. |
| Flammability (KCP 2.3.2) | EC A.15 | CHR/H/IZOXACYP 250 SC | Auto-ignition temperature: 610oC |  |  |  | Y | Flasińska P. BC-18/21 | Accepted. |
| Self-heating (KCP 2.3.3) | N/R |  | N/R |  |  |  |  |  |  |
| Acidity or alkalinity and pH (KCP 2.4.1) | CIPAC MT 75.3 CIPAC MT 191 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial pH: 4.31 pH after accelerated storage test: 3.10 pH after shelf-life test (1 year, 20 oC): 3.82 pH after shelf-life test (2 year, 20 oC): 3.61 Acidity after accelerated storage test : 0.45% Acidity after shelf-life test (1 year, 20oC): 0.56% Acidity after shelf-life test (2 year, 20oC): 0.75% |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | Accepted. |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial: 1% (w/v) suspension – 4.41 After accelerated storage test (2 weeks at 54oC): 1% (w/v) suspension – 4.27 After shelf-life test (1 year, 20oC): 4.51 After shelf-life test (2 year, 20oC): 4.39 |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | Accepted. |
| Viscosity (KCP 2.5.1) | OECD 114 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Determination of dynamic viscosity at 20oC: 5 s-1: 645 mPa s 10 s-1: 407 mPa s 25 s-1: 232 mPa s 50 s-1: 159 mPa s Determination of dynamic viscosity at 40 oC: 5 s-1: 499 mPa s 10 s-1: 312 mPa s 25 s-1 173 mPa s 50 s-1: 115 mPa s |  |  |  | Y | Arévalo E. BF-21/21 | The viscosity was determined by the use of Brookfield Test Method. The formulation is non-Newtonian liquid. As the formulation does not contain formulants classified as a Category 1 aspiration hazard the kinematic viscosity does not have to be determined. Accepted. |
| Surface tension (KCP 2.5.2) | EEC.A5 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Temp. 20±1°C: 0.1% (w/v) – 42.84 mN/m 0.2% (w/v) – 40.59 mN/m |  |  |  | Y | Patrzałek M. ICB/11/2021 | The product is surface active. Accepted. |
| Relative density (KCP 2.6.1) | EEC A.3 | CHR/H/IZOXACYP 250 SC Batch: 202002 | 1.1840 |  |  |  | Y | Patrzałek M. ICB/11/2021 | Accepted. |
| Bulk density (KCP 2.6.2) | N/R |  | N/R |  |  |  |  |  |  |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | Standard Operational Procedure SPB/38 CIPAC MT 46.3 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Test Type |  | At initial preparation | After accelerated storage | Y | Knapik I. ICB/58/2021 Patrzałek M. ICB/11/2021 | The test item was stored in commercial packaging made of HDPE. The temperature ranged from 54.4 to 55.5°C. The change of a.s. content: 4.2%. Accepted. |
|  |  |  | Appearance |  | 5YR 9/1 light-brown, delicate odour | 5YR 9/1 light-brown, characteristic delicate odour |  |  |  |
|  |  |  | pH of 1% suspension |  | 4.41 | 4.27 |  |  |  |
|  |  |  | Acidity |  | N/D | 0.45% |  |  |  |
|  |  |  | pH of undiluted suspension |  | 4.31 | 3.10 |  |  |  |
|  |  |  | Suspensibility in Standard Water D (30±2OC) | 0.1% (w/v) | Isoxaflutole 98% | Isoxaflutole 98% |  |  |  |
|  |  |  |  | 0.2% (w/v) | Isoxaflutole 98% | Isoxaflutole 96% |  |  |  |
|  |  |  | Spontaneity of dispersion in Standard Water C (30±2OC) |  | Isoxaflutole 98% | Isoxaflutole 97% |  |  |  |
|  |  |  | Wet sieve test |  | Residue (sieve 75 µm) – 0.00% | Residue (sieve 75 µm) – 0% |  |  |  |
|  |  |  | Stability of package After accelerated storage test (54OC) |  | Packaging 1/3 Change in packaging weight – 0.06% Change in gross weight – 0.038% Packaging 2/3 Change in packaging weight – 0.08% Change in gross weight – 0.037% |  |  |  |  |
|  |  |  | Content of isoxaflutole |  | 259.9 g/L | 249.06 g/L |  |  |  |
|  |  |  | Pourability |  | Residue – 2.29% | Residue – 2.28% |  |  |  |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | N/R |  | N/R |  |  |  |  |  |  |
| Minimum content after heat stability testing (KCP 2.7.3) | Isoxaflutole: Standard Operational Procedure SPB/205 Cyprosulfamide: Standard Operational Procedure SPB/237 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial: Isoxaflutole: 259.9 g/L Cyprosulfamide: 261.9 g/L After accelerated storage test (2 weeks at 54oC): Content of isoxaflutole: 249.06 g/L After shelf-life test (1 year at 20oC): Content of isoxaflutole: 253.28 g/L Content of cyprosulfamide: 263.23 g/L After shelf-life test (2 year at 20oC): Content of isoxaflutole: 253.235 g/L Content of cyprosulfamide: 253.342 g/L |  |  |  | Y | Patrzałek M. ICB/11/2021 Łebek B. CB/6/2022 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | After accelerated storage, the change in content of isoxaflutole was 4.2%. After shelf-life storage (1 year), the change in content of isoxaflutole was 2.5%, and of safener cyprosulfamide was 0.5%. After shelf-life storage (2 years), the change in the content of isoxaflutole was 2.6%, and of safener cyprosulfamide was 3.3%. Accepted. |
| Effect of low temperatures on stability (KCP 2.7.4) | Low temeprature stability of liquid formulations: CIPAC MT 39.3 CIPAC MT 185 Suspensibility: CIPAC MT 184 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Low temperature stability of liquid formulations: After low temperature stability 0⸰C for 7 days: 0.05 mL sediment After 24 h in room temperature and one invert: 0.05 mL sediment Residue on the wet sieve – 0.02% Suspensibility after 7 days at 0OC: 0.1% (w/v) in Standard Water D (30±2oC) – isoxaflutole 98% 0,2% (w/v) in Standard Water D (30±2oC) – isoxaflutole 99% |  |  |  | Y | Patrzałek M. ICB/11/2021 | Accepted. |
| Ambient temperature shelf life (KCP 2.7.5) | Shelf-life test: Own method SPB/40 | CHR/H/IZOXACYP 250 SC Batch: 202002 | After 1 year at 20 oC. Appearance; OPPTS 830.6302, 830.6303 and 830.6304; Colour (Munsell’s notation) – 5YR 9/1 (light brown) Physical state – liquid Odour – characteristic, delicate. pH; CIPAC MT 75.3; 1% (w/v) suspension – 4.51 Undiluted – 3.82. Suspensibility; CIPAC MT 184.1; 0.10% (w/v) in Standard Water D (30±2°C): - isoxaflutole 99% - cyprosulfamide 99% 0.20% (w/v) in Standard Water D (30±2°C): - isoxaflutole 98% -cyprosulfamide 99%. Spontaneity of dispersion; CIPAC MT 160; Standard Water C (30±2°C): - isoxaflutole 98% - cyprosulfamide 98%. Wet sieve; CIPAC MT 185; Residue (sieve 75 µm) – 0%. Pourability; CIPAC MT 148.1; Residue – 2.60%. Stability of package; Standard Operational Procedure SPB/38; Packaging 4/7 Change in packaging weight – 1.85 [%] Change in gross weight – 0.0125 [%]. Content of isoxaflutole; Standard Operational Procedure SPB/205; Isoxaflutole – 253.28 g/L. Content of cyprosulfamide; Standard Operational Procedure SPB/237; Cyprosulfamide – 263.23 g/L. Acidity; CIPAC MT 191; 0.56%. Particle size distribution; CIPAC MT 187 and Standard Operational Procedure SPB/32; d10-0.425 [µm] d20-0.663 [µm] d30-1.012 [µm] d40-1.335 [µm] d50-1.714 [µm] d60-2.183 [µm] d70-2.790 [µm] d80-3.670 [µm] d90-5.338 [µm] d99-18.564[µm]. After 2nd years at 20 oC. Appearance; OPPTS 830.6302, 830.6303 and 830.6304; Colour (Munsell’s notation) – 5YR 9/1 (light brown) Physical state – liquid Odour – characteristic, delicate. pH; CIPAC MT 75.3; 1% (w/v) suspension – 4.39 undiluted – 3.61. Suspensibility; CIPAC MT 184.1; 0.10% (w/v) in Standard Water D (25±5°C): - isoxaflutole 99% - cyprosulfamide 99% 0.20% (w/v) in Standard Water D (25±5°C): - isoxaflutole 100% -cyprosulfamide 100%. Spontaneity of dispersion; CIPAC MT 160; Standard Water C (30±2°C): - isoxaflutole 98% - cyprosulfamide 98%. Wet sieve; CIPAC MT 185; Residue (sieve 75 µm) – 0%. Pourability; CIPAC MT 148.1; Residue – 2.59%. Stability of package (HDPE); Standard Operational Procedure SPB/38; Packaging 5/7 Change in packaging weight – 1.37 [%] Change in gross weight – 0.0365 [%]. Content of isoxaflutole; Standard Operational Procedure SPB/205; Isoxaflutole – 253.235 g/L. Content of cyprosulfamide; Standard Operational Procedure SPB/237; Cyprosulfamide – 253.342 g/L. Acidity; CIPAC MT 191; 0.75%. Particle size distribution; CIPAC MT 187 and Standard Operational Procedure SPB/32; d10-0.437 [µm] d20-0.719 [µm] d30-1.049 [µm] d40-1.377 [µm] d50-1.758 [µm] d60-2.220 [µm] d70-2.818 [µm] d80-3.688 [µm] d90-5.337 [µm] d99-18.794[µm]. | Y | Łebek B, ICB/13/2021 Łebek B, ICB/14/2021 | The test item was stored in 1 L commercial packaging made of HDPE. The temperature ranged from 19.8 to 20.6°C. After shelf-life storage (1 year), the change in the content of isoxaflutole was 2.5%, and of safener, cyprosulfamide was 0.5%. After shelf-life storage (2 years), the change in the content of isoxaflutole was 2.6%, and of safener cyprosulfamide was 3.3%. The packaging remained stable. There were no significant changes to chemical and technical characteristics of the formulation. Accepted. |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | N/R |  | N/R |  |  |  |  |  |  |
| Wettability (KCP 2.8.1) | N/R |  | N/R |  |  |  |  |  |  |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.3 | CHR/H/IZOXACYP 250 SC Batch: 202002 | 0.1% (w/v): -after 1 minute - 4 mL -after 12 minutes – 2mL 0.2% (w/v): -after 1 minute – 6 mL -after 12 minutes – 4 mL |  |  |  | Y | Patrzałek M. ICB/11/2021 | Accepted. |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial: 0.1% (w/v) in Standard Water D (30±2⸰C): isoxaflutole 98% 0.2% (w/v) in Standard Water D (30±2⸰C): Isoxaflutole 98% After accelerated storage test: 0.10% (w/v) in Standard Water D (30±2°C): isoxaflutole 98% 0.20% (w/v) in Standard Water D (30±2°C): isoxaflutole 96% After shelf-life test (1 year, 20oC): 0.10% (w/v) in Standard Water D (30±2°C): - isoxaflutole 99% - cyprosulfamide 99% 0.20% (w/v) in Standard Water D (30±2°C): - isoxaflutole 98% - cyprosulfamide 99% After shelf-life test (2 year, 20oC): 0.10% (w/v) in Standard Water D (25±5°C): - isoxaflutole 99% - cyprosulfamide 99% 0.20% (w/v) in Standard Water D (25±5°C): - isoxaflutole 100% -cyprosulfamide 100% |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B, ICB/14/2021 | Accepted. |
| Spontaneity of dispersion (KCP 2.8.3.2) | CIPAC 160 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Standard Water C (30±2⸰C): Isoxaflutole 98% After accelerated storage test: Isoxaflutole 97% After shelf-life test (1 year, 20oC): Isoxaflutole 98% Cyprosulfamide 98% After shelf-life test (2 year, 20oC): Isoxaflutole 98% Cyprosulfamide 98% |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B, ICB/14/2021 | Accepted. |
| Dispersion stability (KCP 2.8.3.3) | N/R |  | N/R |  |  |  |  |  |  |
| Degree of dissolution and dilution stability (KCP 2.8.4) | N/R |  | N/R |  |  |  |  |  |  |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 187 and Standard Operational Procedure SPB/32 | CHR/H/IZOXACYP 250 SC Batch: 202002 | After shelf-life test (1 year at 20⸰C): d10-0.425 [µm] d20-0.663 [µm] d30-1.012 [µm] d40-1.335 [µm] d50-1.714 [µm] d60-2.183 [µm] d70-2.790 [µm] d80-3.670 [µm] d90-5.338 [µm] d99-18.564[µm] After shelf-life test (2 year at 20⸰C): d10-0.437 [µm] d20-0.719 [µm] d30-1.049 [µm] d40-1.377 [µm] d50-1.758 [µm] d60-2.220 [µm] d70-2.818 [µm] d80-3.688 [µm] d90-5.337 [µm] d99-18.794[µm] |  |  |  | Y | Łebek B. ICB/13/2021 Łebek B, ICB/14/2021 | As there is no specification for SC formulations containing isoxaflutole, the limit values for size ranges were not specifed. The resutls of the wet sieve analysis, suspensibility, pourability, and spontaneity of dispersion are acceptable so it should be accepted. Accepted. |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial: Residue (sieve 75 µm) – 0.00% After accelerated storage test: Residue (sieve 75 µm) – 0% After shelf-life test (1 year, 20oC): Residue (sieve 75 µm) – 0% After shelf-life test (2 year, 20oC): Residue (sieve 75 µm) – 0% |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | Accepted. |
| Dust content (KCP 2.8.5.2.1) | N/R |  | N/R |  |  |  |  |  |  |
| Particle size of dust (KCP 2.8.5.2.2) | N/R |  | N/R |  |  |  |  |  |  |
| Attrition (KCP 2.8.5.3) | N/R |  | N/R |  |  |  |  |  |  |
| Hardness and integrity (KCP 2.8.5.4) | N/R |  | N/R |  |  |  |  |  |  |
| Emulsifiability (KCP 2.8.6.1) | N/R |  | N/R |  |  |  |  |  |  |
| Emulsion stability (KCP 2.8.6.2) | N/R |  | N/R |  |  |  |  |  |  |
| Re-emulsifiability (KCP 2.8.6.3) | N/R |  | N/R |  |  |  |  |  |  |
| Flowability (KCP 2.8.7.1) | N/R |  | N/R |  |  |  |  |  |  |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148.1 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Initial: Residue – 2.29% After accelerated storage test: Residue – 2.28% After shelf-life test (1 year, 20oC): Residue – 2.60% After shelf-life test (2 year, 20oC): Residue – 2.59% |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | Accepted. |
| Dustability following accelerated storage (KCP 2.8.7.3) | N/R |  | N/R |  |  |  |  |  |  |
| Physical compatibility of tank mixes (KCP 2.9.1) | N/R |  | N/R |  |  |  |  |  |  |
| Chemical compatibility of tank mixes (KCP 2.9.2) | N/R |  | N/R |  |  |  |  |  |  |
| Adhesion to seeds (KCP 2.10.1) | N/R |  | N/R |  |  |  |  |  |  |
| Distribution to seed (KCP 2.10.2) | N/R |  | N/R |  |  |  |  |  |  |
| Other/special studies (KCP 2.11) | Effectiveness of cleaning: Efficacy guideline 305 | CHR/H/IZOXACYP 250 SC Batch: 202002 | Test concentration of 0.2% (w/v). The amount of a.s. removed: Single rinse procedure: 99.88 % isoxaflutole Double rinse procedure: 99.91 % isoxaflutole Triple rinse procedure: 99.91 % isoxaflutole |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 | The content of isoaxaflutole was determined by HPLC. Accepted. |
|  | Stability of package: Standard Operational Procedure SPB/38 | CHR/H/IZOXACYP 250 SC | Stability after accelerated storage test: Packaging 1/3 Change in packaging weight – 0.06 [%] Change in gross weight – 0.038 [%] Packaging 2/3 Change in packaging weight – 0.08 [%] Change in gross weight – 0.037 [%] Stability after shelf-life test: Packaging 4/7 Change in packaging weight – 1.85 [%] Change in gross weight – 0.0125 [%] Stability after shelf-life test (after 2 year) Packaging 5/7 Change in packaging weight – 1.37 [%] Change in gross weight – 0.0365 [%] |  |  |  | Y | Patrzałek M. ICB/11/2021 Knapik I. ICB/58/2021 Łebek B. ICB/13/2021 Łebek B. ICB/14/2021 | Accepted. |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

| Comments of zRMS: | In the accelerated storage and 1-year 2-years shelf-life stability study, the formulation was stored in commercial packaging (bottles made of HDPE) and the packaging remained stable during the storage, therefore, the proposed commercial packs are considered acceptable. |
| --- | --- |

HDPE bottles/jars

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 64 mm± 2 mm/130 mm ± 3 mm |
| Opening: | 40 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 72 mm± 2 mm/111,8 mm ± 3 mm |
| Opening: | 38 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 72±1 mm/111.8±2 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 69 mm± 2 mm/186.5 mm ± 2 mm |
| Opening: | 45.65± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 564 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 90,5 mm± 2 mm/151 mm ± 3 mm |
| Opening: | 40,6 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 77,6 mm± 2 mm/160,6 mm ± 3 mm |
| Opening: | 38 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 77.6 ±1 mm/160.6±2 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

The jar is set in an inner box (cardboard box). The inner box is grouped into an outer box

Material: HDPE

Type of construction: jar

Size: approximate diameter/height: 80 mm/138 mm

Capacity: 510 ml overflow

Type of closure: screw-cap with seal

Size of opening: 46 mm minimum

Accessories: one measuring device per each jar

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 145.5mm± 2 mm/78mm ± 2 mm |
| Opening: | 56mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 600 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | JAR |
| --- | --- |
| Material: | HDPE |
| size: | 79/80 mm/ 201 mm |
| Opening: | 46 mm minimum |
| Closure: | screw cap with seal |
| Capacity | 800 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 80 mm± 2 mm/201 mm ± 2 mm |
| Opening: | 64 mm |
| Closure: | screw cap with seal |
| Capacity | 800 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 88.5 mm± 2 mm/283.5 mm ± 2 mm |
| Opening: | 45.30 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 88 mm± 4 mm/242 mm ± 6 mm |
| Opening: | 39mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 238 mm± 2 mm/90mm ± 2 mm |
| Opening: | 39 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 234 mm± 2 mm/88.5mm ± 2 mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 84 mm± 2 mm/248.2 mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 234 mm± 2 mm/88.5mm ± 2 mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | cap with seal |
| Capacity | 1200 ± 50 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 84 ± 1.5 mm/230.1 ± 3 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 157,2 mm± 2 mm/101mm ± 2 mm |
| Opening: | 72 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | JAR |
| --- | --- |
| Material: | HDPE |
| size: | 108/110 mm/ 266 mm |
| Opening: | 46 mm minimum |
| Closure: | screw cap with seal |
| Capacity | 2000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 232 mm± 2 mm/195mm± 2 mm/130mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 3000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 94 ± 1 mm/103 ± 1 mm/272.5 ± 3 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 2000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 224,1 mm± 2 mm/122mm ± 2 mm |
| Opening: | 73 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 2000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

HDPE containers/cannisters

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 232 mm± 2 mm/195mm± 2 mm/130mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 3000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 96 ± 3 mm/195 ± 3.5 mm/297.2 ± 4 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 4000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 305mm± 5 mm/193 mm± 5 mm/142 mm ± 5 mm |
| Opening: | 59.20 mm minimum ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5850 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 336 mm± 5 mm/195mm± 5 mm/130mm ± 5 mm |
| Opening: | 50 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 310,5 mm± 5 mm/195mm± 5 mm/130mm ± 5 mm |
| Opening: | 63 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 190 mm± 5 mm /140 mm± 5 mm/ 314 mm ± 5 mm |
| Opening: | 54.5 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 127±2 mm/192±2 mm/285±5 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 145±2 mm/190.8±3/294±4 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 6000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 375mm± 5 mm/240 mm± 5 mm/179 mm ± 5 mm |
| Opening: | 63 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 11220±50 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 312 mm± 5 mm/225mm± 5 mm/185mm ± 5 mm |
| Opening: | 40.8 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 375 mm± 5 mm/230± 5 mm/165 mm ± 5 mm |
| Opening: | 54.5 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 377,7 mm± 5 mm/239,5± 5 mm/178 mm ± 5 mm |
| Opening: | 54 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 192±3 mm/228±7/313±7 mm |
| Opening: | 52 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 185±2 mm/225±2/312±3 mm |
| Opening: | 40.8 mm ± 0.3 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 443mm/288mm/230mm |
| Opening: | 44mm (internal) 60mm (external) |
| Closure: | screw cap with seal |
| Capacity | 22000ml ± 50 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 376.3±3 mm/295±3mm/246±3mm |
| Opening: | 50 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 20000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 378±5 mm/288±5/258±5 mm |
| Opening: | 53.7±1.5 mm |
| Closure: | screw cap with seal |
| Capacity | 22000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 376±8 mm/257,5±5/376±8 mm |
| Opening: | 52 mm± 3 |
| Closure: | screw cap with seal |
| Capacity | 20000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 257.5±6 mm/292±8/376±8 mm ± 5 mm |
| Opening: | 52 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 20000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Size: | 90000mm/59000mm ± 2 mm |
| Opening: | 45mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 200 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

HDPE/PA bottles

According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) storage stability study can be extrapolated to new packaging material HDPE/PA and HDPE/PA COEX from provided and evaluated storage stability studies of packing CHR/H/IZOXACYP to HDPE. Therefore, no further studies are required for the additional packaging materials.

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 62.5±1 mm/131.3±1 mm |
| Opening: | 45.65±3 mm |
| Closure: | screw cap with seal |
| Capacity | 323 ± 5 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA |
| size: | 59 ± 1 mm/143 ± 1 mm/ |
| Opening: | 41.7±0.7 mm |
| Closure: | screw cap with seal |
| Capacity | 275 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA |
| size: | 69 mm ± 2 mm/186.5 mm ± 2 mm |
| Opening: | 45.65±3 mm |
| Closure: | screw cap with seal |
| Capacity | 574 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 74± 1 mm/177 ± 1 mm/ |
| Opening: | 41.7±0.7 mm |
| Closure: | screw cap with seal |
| Capacity | 550 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 88 mm ± 2 mm/238 mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA |
| size: | 248.5±3 mm/84±1.5mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA |
| size: | 234 mm± 2 mm/88.5mm ± 2 mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 238± 1 mm/88 ± 1 mm/ |
| Opening: | 41.7±0,7 mm |
| Closure: | screw cap with seal |
| Capacity | 1100 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 84± 1.5 mm/248.5 ± 3 mm |
| Opening: | 50 mm ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 233.5± 1.5 mm/88.5 ± 1 mm/ |
| Opening: | 39 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1100 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | CONTAINER |
| Material: | HDPE/PA COEX |
| size: | 305mm± 5 mm/193 mm± 5 mm/142 mm ±5 mm |
| Opening: | 63 mm minimum ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5850 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 193 ± 3/ 142 ± 5 mm/320 mm± 5 mm |
| Opening: | 63,3 ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 5500 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 195 ± 3/ 130 ± 5 mm/310,5 mm± 5 mm |
| Opening: | 63,3 ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

Coextruded HDPE/PA containers/cannisters

According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) storage stability study can be extrapolated to new packaging material HDPE/PA and HDPE/PA COEX from provided and evaluated storage stability studies of packing CHR/H/IZOXACYP to HDPE. Therefore, no further studies are required for the additional packaging materials.

| Packaging information |  |
| --- | --- |
| Type | CANNISTER |
| Material: | HDPE/PA COEX |
| size: | 313± 5mm/190±3/140±5mm |
| Opening: | 50 mm ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | CONTAINER |
| Material: | HDPE/PA COEX |
| size: | 305mm/193 mm/142 mm ± 5 mm |
| Opening: | 63 mm minimum ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | CONTAINER |
| Material: | HDPE/PA COEX |
| size: | 377,7mm/178 mm/239,5 mm ± 5 mm |
| Opening: | 54 mm min ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | coextruded |
| UN/ADR | compliant |

HDPE/F bottles

According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) storage stability study can be extrapolated to new packaging material HDPE/F from provided and evaluated storage stability studies of packing CHR/H/IZOXACYP to HDPE. Therefore, no further studies are required for the additional packaging materials.

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 63.5±1 mm/126±1 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 318 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 63.5±1 mm/126±1 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 312 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 585 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 580 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 575 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 570 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1150 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1160 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1185 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1200 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Cannister |
| --- | --- |
| Material: | HDPE/F |
| size: | 193±2 mm/142±2mm/305±3mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 5880 ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Cannister |
| --- | --- |
| Material: | HDPE/F |
| size: | 193±2 mm/142±2mm/305±3mm |
| Opening: | 63 mm |
| Closure: | screw cap with seal |
| Capacity | 5880 ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 54,2 mm ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 63.4 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 67,5 mm ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

HDPE/F cannisters

According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) storage stability study can be extrapolated to new packaging material HDPE/F from provided and evaluated storage stability studies of packing CHR/H/IZOXACYP to HDPE. Therefore, no further studies are required for the additional packaging materials.

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 54,2 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 63,4 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 67,5 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Type | Cannister |
| --- | --- |
| Material: | HDPE/F |
| size: | 240±2 mm/179±2mm/375±3mm |
| Opening: | 63 mm |
| Closure: | screw cap with seal |
| Capacity | 10 000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

HDPE/EvOH bottles

According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) storage stability study can be extrapolated to new packaging material HDPE/EvOH from provided and evaluated storage stability studies of packing CHR/H/IZOXACYP to HDPE. Therefore, no further studies are required for the additional packaging materials.

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| Body diameter / total height: | 62,50 +- 0,50 / 126,50 +- 1,50 |
| External thread diameter: | 49,65 +- 0,35 |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | N/A |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| Body diameter / total height: | 59 +- 1 mm / 143 +- 1 mm |
| External thread diameter: | 41.7 +- 0,4 mm |
| Closure: | screw cap with seal |
| Capacity | 310 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | N/A |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| size: | 69 mm± 2 mm/186.5 mm ± 2 mm |
| Opening: | 42±3 mm |
| Closure: | screw cap with cutter |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| size: | 65 mm/234.8 mm ± 2 mm |
| Opening: | 27.4 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| size: | 69 mm ±1 mm/190 mm ±1.5 mm |
| Opening: | 49.5 mm ±0.3 mm |
| Closure: | screw cap with seal |
| Capacity | 579 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| size: | 234±3 mm/88.5±2mm |
| Opening: | 50 mm ± 3 mm |
| Closure: | screw cap with cutter |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EvOH |
| size: | 242±1.5 mm/88.5±1mm |
| Opening: | 49.5 mm ± 0.3 mm |
| Closure: | screw cap with cutter |
| Capacity | 1200± 50 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

HDPE/EvOH containers/cannisters

According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) storage stability study can be extrapolated to new packaging material HDPE/EvOH from provided and evaluated storage stability studies of packing CHR/H/IZOXACYP to HDPE. Therefore, no further studies are required for the additional packaging materials.

| Packaging information |  |
| --- | --- |
| Type | CONTAINER |
| Material: | HDPE/EvOH |
| size: | 165 mm ± 2 mm/195 mm ± 2 mm/228mm± 2 mm |
| Opening: | 48 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | CANNISTER |
| Material: | HDPE/EvOH |
| size: | 142 mm ± 1.5 mm/193 mm ± 2 mm/307mm± 3 mm |
| Opening: | 63.3 mm ± 0.3 mm |
| Closure: | screw cap with cutter |
| Capacity | 5650 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | CONTAINER |
| Material: | HDPE/EvOH |
| size: | 195 mm ± 2 mm/225mm± 2 mm/306mm± 2 mm |
| Opening: | 48 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

| Packaging information |  |
| --- | --- |
| Type | CONTAINER |
| Material: | HDPE/EvOH |
| size: | 375 mm ± 2 mm/290mm± 2 mm/245mm± 2 mm |
| Opening: | 85mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 20000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1/01 | Patrzałek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.1/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.1/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.2.1 | Buczkowski D. | 2021 | Determination of explosive properties BW-13/21 Testing facility: Łukasiewicz Research Network – Institute of Industrial Organic Chemistry, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.2.2 | Flasińska P. | 2021 | Determination of oxidizing properties and auto-ignition temertature BC-18/21 Testing facility: Łukasiewicz Research Network – Institute of Industrial Organic Chemistry, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.3.1 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.3.2 | Flasińska P. | 2021 | Determination of oxidizing properties and auto-ignition temertature BC-18/21 Testing facility: Łukasiewicz Research Network – Institute of Industrial Organic Chemistry, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.1/01 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.1/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.1/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.2/01 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.2/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.2/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.5.1 | Arévalo E. | 2021 | Viscosity determination BF-21/21 Testing facility: Łukasiewicz Research Network – Institute of Industrial Organic Chemistry, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.5.2 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.6.1 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.1/01 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.1/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.1/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.2 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.3/01 | Patrzałek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.3/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.3/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.3/04 | Łebek B. | 2022 | Validation of analytical method for Izoxaflutole 250 SC (CHR/H/IZOXACYP 250 SC) for determination of cyprosulfamide Company Report No ICB/6/2022 ICB Pharma, 10 Lema Street, 43-600 Jaworzno Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.4 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.2 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.1 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.2 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.1 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.2/01 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.2/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.2/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.2/01 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.2/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.2/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11/01 | Patrzalek M. | 2021 | Determination of physicochemical properties ICB/11/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11/02 | Knapik I. | 2021 | Determination of physicochemical properties after accelerated storage test ICB/58/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11/03 | Łebek B. | 2022 | Determination of physicochemical properties after shelf-life test ICB/13/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.1 KCP 2.4.1 KCP 2.4.2 KCP 2.7.3 KCP 2.7.5 KCP 2.8.3.1 KCP 2.8.3.2 KCP 2.8.5.1.1 KCP 2.8.5.1.2 KCP 2.8.7.2 KCP 2.11 | Łebek B. | 2023 | Determination of physicochemical properties after shelf-life test ICB/14/2021 Testing Facility: ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Additional data on the physical, chemical and technical properties of the active substance

  - Isoxaflutole

Not relevant