REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: GWN-10616 Chemical active substances: Zoxamide, 60 g/L Potassium phosphonates, 755 g/L Phosphonic acid equivalents, 500 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (new authorization)

Applicant: XXXX Submission date: 31/10/2023 (update September 2024) Evaluation date: 07/2024 MS Finalisation date: 11/2024

Version history

| When | What |
| --- | --- |
| July 2024 | Version submitted by the applicant and evaluated by zRMS PL |
| September 2024 | Update based on zRMS request of July 2024: Chapter 4.2 and 4.3 |
| November 2024 | Updated based on zRMS request of July 2024 and evaluated by zRMS taking into accounts comments received from cMSs and the applicant. |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	4

1.2.3.1	Zoxamide	4

1.2.3.2	Potassium phosphonates	4

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	5

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	19

4	Section 4: Further information on the plant protection product	19

4.1	Safety intervals and other precautions to protect humans, animals and the environment	19

4.2	Recommended methods and precautions	19

4.3	Emergency measures in the case of an accident	21

4.4	Packaging and Compatibility with the Preparation (KCP 4.4)	23

4.5	Procedures for destruction or decontamination of the plant protection product and its packaging	24

4.5.1	Neutralisation procedures	24

4.5.2	Controlled incineration	24

Appendix 1	Lists of data considered in support of the evaluation	25

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	31

Sufficient data on identity, physical and chemical properties and other information are available for the plant protection product and the contained technical active substance(s).

Noticed data gaps are:

- None

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name:	XXXX

Address:	XXXX

XXXX

Contact:	XXXX

Telephone:	XXXX

E-mail:	XXXX

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Zoxamide

| Zoxamide | min. 953 g/kg |
| --- | --- |

According to COMMISSION IMPLEMENTING REGULATION (EU) 2018/692
of 7 May 2018, the minimum purity of Zoxamide is 953 g/kg. There is no relevant impurity of toxicological/ecotoxicological concern.

#### Potassium phosphonates

| Potassium phosphonates | Specification for TK: 31.6 to 32.6 % phosphonate ions (sum of hydrogen phosphonate and phosphonate ions) 17.8 to 20.0 % potassium min. 990 g/kg on dry weight basis |
| --- | --- |

According to COMMISSION IMPLEMENTING REGULATION (EU) No 369/2013
of 22 April 2013, the minimum purity of potassium phosphonate is 990 g/kg on dry weight basis. The content of phosphonate ions is 31.6 – 32.6% (sum of hydrogen phosphonate and phosphonate ions) and 17.8 – 20.0% potassium. There is no relevant impurity of toxicological/ecotoxicological concern.

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Please refer to Registration Report Part A for the relevant country (or) |
| --- | --- |
| Company code number: | GWN-10616 (the other used code is GOW F716). |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

GWN-10616 was not the representative formulation in the evaluation of the active substances.

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L) | FAO Limits (min – max) | Technical content* (g/L) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Zoxamide | 60 | 54 – 66 | 63 | 4.42 |
| Potassium phosphonate | 755 | 730 – 780 | 763*** | 53.54 |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

** 	Based on the density of the formulation = 1.425 g/mL

***    Based on minimum purity of 990 g/kg based on dry weight

There are no safeners or synergists in the formulation. Please refer to part C.

There are no relevant impurities in the formulation.

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-2:	Information on Zoxamide

| Type | Name/Code Number |  |
| --- | --- | --- |
| ISO common name | Zoxamide | Variant: Not applicable |
| CAS No. | 156052-68-5 | Not applicable |
| EC No. | Not allocated | Not applicable |
| CIPAC No. | 640 | Not applicable |

Information on potassium phosphonate

| Type | Name/Code Number |  |
| --- | --- | --- |
| ISO common name | Potassium phosphonate | Variant: Not applicable |
| CAS No. | 13977-65-6 for potassium hydrogen phosphonate 13492-26-7 for dipotassium phosphonate Mixture: none | Not applicable |
| EC No. | - | Not applicable |
| CIPAC No. | 756 (for potassium phosphonates) | Not applicable |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

There are no safeners or synergists in the formulation. There are no relevant co-formulants in the formulation.

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Suspension concentrate | [Code: SC] |
| --- | --- |

## Function (KCP 1.6)

Fungicide

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable. The appearance of the product is that of homogenous liquid with yellow colour, with a characteristic odour. It is not explosive, has no oxidising properties. The product is not flammable. It has a self ignition temperature of 342°C. In aqueous solution, it has a pH value around 4.5 at 25°C. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0°C and 14 days at 54°C, neither the active ingredient content nor the technical properties were changed. The stability data indicate a shelf life of at least 2 years at ambient temperature when stored in HDPE/PA HDPE material.  Its technical characteristics are acceptable for a SC formulation.

The intended concentration of use is 0.3 0.25% to 1.5% v/v.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

None, as the product is not classified for the physical chemistry part.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

None, as the product is not classified for the physical chemistry part.

Compliance with FAO specifications:

The product GWN-10616 complies with FAO specifications.

Formulation used for tests

The product used in the tests summarised below has the same composition as the one cited in part C.

Table 2-3:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | Visual inspection | GWN-10616 Batch No. L1704669001 | Homogeneous and viscous liquid Sand yellow colour with characteristic odour | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.1/01 | Accepted |
| Explosive properties (KCP 2.2.1) | Theoretical assessment | - | Based on the composition of the formulation, no explosive properties are expected. | N | Anonymous (2023), Doc. No. 281-001 C-KCP 2.2.1/01 | Accepted |
| Oxidizing properties (KCP 2.2.2) | Theoretical assessment | - | Based on the composition of the formulation, no oxidising properties are expected. | N | Anonymous (2023), Doc. No. 281-001 C-KCP 2.2.2/01 | Accepted |
| Flash point (KCP 2.3.1) | EC 440/2008 No. A.9 | GWN-10616 Batch No. P2102669001 | No Flash point was observed until 130 °C | Y | Pardo Martinez, M. (2021), Report No. CH – 0385/2021, Doc. No. 242-001 KCP 2.3.1/01 | Accepted |
| Flammability (KCP 2.3.2) | - | - | Not required for this formulation type | - | - |  |
| Self-heating (KCP 2.3.3) | EC 440/2008 No. A.15 | GWN-10616 Batch No. P2102669001 | The auto-ignition temperature was determined at 342°C |  | Pardo Martinez, M. (2021), Report No. CH – 0386/2021, Doc. No. 242-002 KCP 2.3.3/01 | Accepted |
| Acidity or alkalinity and pH (KCP 2.4.1) | - | - | Not required as pH is between 4 – 10 | - | - |  |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 |  | Undiluted: 4.65 (25.0 °C) Diluted to 1% w/v: 4.40 (25.0 °C) | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.4.2/01 | Accepted |
| Viscosity (KCP 2.5.1) | OECD 114 Rotational viscometer | GWN-10616 Batch No. L1704669001 | Viscosity at 20°C: 5070.9 mPa*s at 4 rpm spindle speed. 523.4 mPa*s at 100 rpm spindle speed. Viscosity at 40°C: 4928.1 mPa*s at 4 rpm spindle speed. 457.0 mPa*s at 100 rpm spindle speed. Viscosity was found to decrease with increasing spindle speed. | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.5.1/01 | Accepted The PPP is a non-Newtonian liquid |
| Surface tension (KCP 2.5.2) | OECD 115 Ring corrected method | GWN-10616 Batch No. L1704669001 | Surface tension at 1.5% v/v (corresponds to the highest application rate): 32.94 mN/m at 20°C Surface tension of undiluted product: 64.29 mN/m at 20°C | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.5.2/01 | Accepted The PPP is a surface active |
| Relative density (KCP 2.6.1) | OECD 109 Pycnometer method | GWN-10616 Batch No. L1704669001 | Density = 1.425 g/mL at 20°C Relative density = 1.425 | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.6.1/01 | Accepted |
| Bulk density (KCP 2.6.2) | - | - | Not required for this formulation type | - | - |  |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.3 | GWN-10616 Batch No. L1704669001 | The test item remained unaltered after accelerated storage in HDPE-PA packaging for 14 days at 54°C. Please refer to table 2-2 for the detailed results. | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.7.1/01 | Accepted An analytical method is accepted in KCP 5.1.1 (Part B section 5) The applicant confirmed that GOW F716 was a different manufacturer code used for formulation GWN-10616. |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | - | - | Please refer to point KCP 2.7.1 above | - | - |  |
| Minimum content after heat stability testing (KCP 2.7.3) | - | - | Please refer to point KCP 2.7.1 above | - | - |  |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 | GWN-10616 Batch No. L1704669001 | Results after storage at 7 days at 0°C: Homogenous, beige formulation with characteristic odour. No separation or precipitation was observed. Technical properties remained unchanged. Please refer to table 2-3. | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.7.4/01 | Accepted The analytical method is accepted in KCP 5.1.1 (Part B section 5). The method was used for analysing active substances’ contents during the suspensibility testing. The applicant confirmed that GOW F716 was a different manufacturer code used for formulation GWN-10616. |
| Ambient temperature shelf life (KCP 2.7.5) | GIFAP No. 17 | GWN-10616 Batch No. L1704669001 | The test item remained unaltered after storage in HDPE-PA packaging for 3 2 years at ambient temperature. Please refer to table 2-4 for the detailed results. | Y | Aversa S. (2017 2019), Report No. BT 166/17, Doc. No. 245-002 KCP 2.7.5/01 | Accepted The analytical method is accepted in KCP 5.1.1 (Part B section 5) Applicant has provided only the two-year storage stability study. So, a two-year shelf life is granted for this PPP. The applicant confirmed that GOW F716 was a different manufacturer code used for formulation GWN-10616. |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Please refer to point KCP 2.7.5 above | - | - |  |
| Wettability (KCP 2.8.1) | - | - | Not required for this formulation type | - | - |  |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.3 | GWN-10616 Batch No. L1704669001 | Foam at 0.25% v/v (lowest application rate): 1 min: 0 mL 12 min: 0 mL Foam at 1.5% v/v (l highest application rate): 1 min: 0 mL 12 min: 0 mL | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.8.2/01 | Accepted |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184 | GWN-10616 Batch No. L1704669001 | Results for T0 at 0.25% v/v (lowest application rate): Zoxamide: 100% Phosphonic acid: 100% Results for T0 at 1.5% v/v (highest application rate): Zoxamide: 100% Phosphonic acid: 101% | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.8.3.1/01 | Accepted The analytical method is accepted in KCP 5.1.1 (Part B section 5). |
| Spontaneity of dispersion (KCP 2.8.3.2) | CIPAC MT 160 | GWN-10616 Batch No. L1704669001 | Tested at 1.5% v/v (highest application rate): 103% at T0 104% after two weeks at 54°C | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.8.3.2/01 | Accepted |
| Dispersion stability (KCP 2.8.3.3) | - | - | Not required for this formulation type | - | - |  |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not required for this formulation type | - | - |  |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 187 | GWN-10616 Batch No. P2102669001 | Dv 10: 0.640 μm; Dv 50: 1.95 μm; Dv 90: 5.75 μm; Volume < 50 μm: 100.00 % Volume > 75 μm: 0 % | Y | Pardo Martinez, M. (2021), Report No. CH – 0387/2021, Doc. No. 214-001 KCP 2.8.5.1.1/01 | Accepted |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | GWN-10616 Batch No. L1704669001 | Residue at 75 µm sieve: 0.016% at T0 0.002% after two weeks at 54°C | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.8.5.1.2/01 | Accepted |
| Dust content (KCP 2.8.5.2.1) | - | - | Not required for this formulation type | - | - |  |
| Particle size of dust (KCP 2.8.5.2.2) | - | - | Not required for this formulation type | - | - |  |
| Attrition (KCP 2.8.5.3) | - | - | Not required for this formulation type | - | - |  |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not required for this formulation type | - | - |  |
| Emulsifiability (KCP 2.8.6.1) | - | - | Not required for this formulation type | - | - |  |
| Emulsion stability (KCP 2.8.6.2) | - | - | Not required for this formulation type | - | - |  |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Not required for this formulation type | - | - |  |
| Flowability (KCP 2.8.7.1) | - | - | Not required for this formulation type | - | - |  |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148.1 | GWN-10616 Batch No. L1704669001 | Residue: 3.69% at T0 3.38% after two weeks at 54°C | Y | Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001 KCP 2.8.7.2/01 | Accepted |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not required for this formulation type | - | - |  |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | Not required as the product is not intended for use in tank mixes. | - | - |  |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | Not required as the product is not intended for use in tank mixes. | - | - |  |
| Adhesion to seeds (KCP 2.10.1) | - | - | Not used for seed treatment. | - | - |  |
| Distribution to seed (KCP 2.10.2) | - | - | Not used for seed treatment. | - | - |  |
| Other/special studies (KCP 2.11) | - | - | No other product studies submitted. | - | - |  |

Table 2-2:	Results of the accelerated storage stability study in HDPE-PA packaging (Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001, KCP 2.7.1/01)

| Tests |  | Methods |  | Initially |  |  |  |  | After storage for 14 days at 54°C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Appearance |  | Visual assessment |  | Homogeneous and viscous liquid, Colour: RAL 1002, Sand yellow Odour: characteristic |  |  |  |  | Homogeneous and viscous liquid, Colour: RAL 1002, Sand yellow Odour: characteristic |  |  |  |
| Stability of test substance container |  | Visual assessment |  | 1L HDPE-PA bottle |  |  |  |  | No modification observed |  |  |  |
| Weight change of test substance container |  | Weighing |  | - |  |  |  |  | 0.015% |  |  |  |
| Active substance content |  | HPLC-DAD for Zoxamide HPLC-MS/MS for phosphonic acid |  | Zoxamide: 4.49% w/w 63.92 g/L Phosphonic acid: 36.36% w/w 518.051 g/L |  |  |  |  | Zoxamide: 4.38% w/w 62.467 g/L Phosphonic acid: 36.40% w/w 518.707 g/L |  |  |  |
| pH value |  | CIPAC MT 75.3 |  | Undiluted: 4.65 (25.0°C) Diluted 1% w/v: 4.40 (25.0°C) |  |  |  |  | Undiluted: 4.61 (25.1°C) Diluted 1% w/v: 4.38 (25.1°C) |  |  |  |
| Spontaneity of dispersion 1.5 % v/v Std Water D |  | CIPAC MT 160 |  | 103% |  |  |  |  | 104% |  |  |  |
| Wet sieve test (Residue at 75 μm) |  | CIPAC MT 185 |  | 0.016% |  |  |  |  | 0.002% |  |  |  |
| Pourability Residue |  | CIPAC MT 148.1 |  | 3.69% |  |  |  |  | 3.38% |  |  |  |
| Suspensibility Std Water D | CIPAC MT 184 |  | 0.25% v/v |  |  | 1.5% v/v |  | 0.25% v/v |  |  | 1.5% v/v |  |
|  |  |  | Zoxamide |  | 100% | Zoxamide | 101% | Zoxamide |  | 100% | Zoxamide | 100% |
|  |  |  | Phosphonic acid |  | 100% | Phosphonic acid | 101% | Phosphonic acid |  | 100% | Phosphonic acid | 100% |

Table 2-3:	Results of the cold storage stability study (Aversa S. (2017), Report No. BT 165/17, Doc. No. 245-001, KCP 2.7.4/01)

| Tests | Methods | After storage at 0°C for 7 days |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Appearance | Visual assessment | Homogenous; beige colour (RAL 1001) with characteristic odour; no separation or precipitation |  |  |  |
| pH value | CIPAC MT 75.3 | Undiluted: 4.67 (25.0°C) Diluted 1% w/v: 4.42 (25.2°C) |  |  |  |
| Wet sieve test (Residue at 75 μm) | CIPAC MT 185 | 0.001% |  |  |  |
| Suspensibility Std Water D | CIPAC MT 184 | 0.25% v/v |  | 1.5% v/v |  |
|  |  | Zoxamide | 100% | Zoxamide | 101% |
|  |  | Phosphonic acid | 99% | Phosphonic acid | 101% |

Table 2-4:	Results of the ambient shelf life study in HDPE-PA packaging (Aversa S. (2017), Report No. BT 166/17, Doc. No. 245-002, KCP 2.7.5/01)

| Tests |  | Methods |  | Initially |  |  |  |  | 6 months |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Appearance |  | Visual assessment |  | Homogeneous and viscous liquid, Colour: RAL 1002, Sand yellow Odour: characteristic |  |  |  |  | Homogeneous and viscous liquid, Colour: RAL 1002, Sand yellow Odour: characteristic |  |  |  |
| Stability of test substance container |  | Visual assessment |  | 1L HDPE-PA bottle |  |  |  |  | No modification observed |  |  |  |
| Weight change of test substance container |  | Weighing |  | - |  |  |  |  | 0.0019% |  |  |  |
| Active substance content |  | HPLC-DAD for Zoxamide HPLC-MS/MS for phosphonic acid |  | Zoxamide: 4.49% w/w 63.92 g/L Phosphonic acid: 36.36% w/w 518.051 g/L |  |  |  |  | Zoxamide: 4.25% w/w 60.558 g/L Phosphonic acid: 34.68% w/w 494.268 g/L |  |  |  |
| pH value |  | CIPAC MT 75.3 |  | Undiluted: 4.65 (25.0°C) Diluted 1% w/v: 4.40 (25.0°C) |  |  |  |  | Undiluted: 4.46 (21.6°C) Diluted 1% w/v: 4.31 (21.5°C) |  |  |  |
| Spontaneity of dispersion 1.5 % v/v Std Water D |  | CIPAC MT 160 |  | 103% |  |  |  |  | 104% |  |  |  |
| Wet sieve test (Residue at 75 μm) |  | CIPAC MT 185 |  | 0.016% |  |  |  |  | 0.04% |  |  |  |
| Pourability Residue |  | CIPAC MT 148.1 |  | 3.69% |  |  |  |  | 3.62% |  |  |  |
| Suspensibility Std Water D | CIPAC MT 184 |  | 0.25% v/v |  |  | 1.5% v/v |  | 0.25% v/v |  |  | 1.5% v/v |  |
|  |  |  | Zoxamide |  | 100% | Zoxamide | 101% | Zoxamide |  | 101% | Zoxamide | 100% |
|  |  |  | Phosphonic acid |  | 100% | Phosphonic acid | 101% | Phosphonic acid |  | 100% | Phosphonic acid | 100% |

Table 2-4 (continued):	Results of the ambient shelf life study in HDPE-PA packaging (Aversa S. (2017), Report No. BT 166/17, Doc. No. 245-002, KCP 2.7.5/01)

| Tests |  | Methods |  | 12 months |  |  |  |  | After storage for 2 years at ambient temperature |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Appearance |  | Visual assessment |  | Homogeneous and viscous liquid, Colour: RAL 1002, Sand yellow Odour: characteristic |  |  |  |  | Homogeneous and viscous liquid, Colour: RAL 1002, Sand yellow Odour: characteristic |  |  |  |
| Stability of test substance container |  | Visual assessment |  | 1L HDPE-PA bottle |  |  |  |  | No modification observed |  |  |  |
| Weight change of test substance container |  | Weighing |  | - |  |  |  |  | 0.015% |  |  |  |
| Active substance content |  | HPLC-DAD for Zoxamide HPLC-MS/MS for phosphonic acid |  | Zoxamide: 4.38% w/w 62.401 g/L Phosphonic acid: 36.01% w/w 513.166 g/L |  |  |  |  | Zoxamide: 4.36% w/w 62.688 g/L Phosphonic acid: 35.96% w/w 512.433 g/L |  |  |  |
| pH value |  | CIPAC MT 75.3 |  | Undiluted: 4.56 (24.8°C) Diluted 1% w/v: 4.40 (24.6°C) |  |  |  |  | Undiluted: 4.79 (22.3°C) Diluted 1% w/v: 4.28 (23.4°C) |  |  |  |
| Spontaneity of dispersion 1.5 % v/v Std Water D |  | CIPAC MT 160 |  | 104% |  |  |  |  | 103% |  |  |  |
| Wet sieve test (Residue at 75 μm) |  | CIPAC MT 185 |  | 0.02% |  |  |  |  | 0.02% |  |  |  |
| Pourability Residue |  | CIPAC MT 148.1 |  | 3.27% |  |  |  |  | 3.28% |  |  |  |
| Suspensibility Std Water D | CIPAC MT 184 |  | 0.25% v/v |  |  | 1.5% v/v |  | 0.25% v/v |  |  | 1.5% v/v |  |
|  |  |  | Zoxamide |  | 99% | Zoxamide | 99% | Zoxamide |  | 99% | Zoxamide | 97% |
|  |  |  | Phosphonic acid |  | 100% | Phosphonic acid | 100% | Phosphonic acid |  | 99% | Phosphonic acid | 98% |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Safety intervals and other precautions to protect humans, animals and the environment

Pre-harvest interval (in days) for each relevant crop:

Please refer to the intended uses given in the GAP section, part B, section 0 of this dRR (dRR part B0, Appendix 1).

Re-entry period (in days) for livestock, to areas to be grazed:

Not relevant as livestock is not expected to enter treated areas.

Re-entry period (in hours or days) for man to crops, buildings or spaces treated:

Please refer to the worker exposure estimated in the toxicological section of this dRR (dRR part B6, 
point 6.6.4).

Withholding period (in days) for animal feeding stuffs:

Please refer to the residue section of this dRR (dRR part B7, point 7.1.2).

Waiting period (in days) between application and handling of treated products:

Please refer to the worker exposure estimated in the toxicological section of this dRR (dRR part B6, 
point 6.6.4).

Waiting period (in days) between last application and sowing or planting succeeding crops:

Not relevant as no phytotoxicity in succeeding crops has been observed.

Information on specific conditions under which the preparation may or may not be used:

No restrictions; when use of product is in compliance with label instructions and normal Good Agricultural Practice.

## Recommended methods and precautions

| Reference: | KCP 4.2/01 |
| --- | --- |
| Report: | SAFETY DATA SHEET - GWN-10616, Anonymous, 2023, report No. Version number 1.0, Doc. No. 954-001 |
| Guideline(s): | 2020/878 |
| Deviations: | No |
| GLP: | No, not required for MSDS |
| Acceptability: | Yes |

| Reference: | KCP 4.2/03 |
| --- | --- |
| Report: | SAFETY DATA SHEET - ELECTIS® K-PLUS, Anonymous, 2024, report No. Version number 1.0, Doc. No. 954-003 |
| Guideline(s): | EC/1272/2008/EC |
| Deviations: | No |
| GLP: | No, not required for MSDS |
| Acceptability: | Yes |

Handling and storage

Precautions for safe handling

Ensure good ventilation/exhaustion at the workplace.

Prevent formation of aerosols.

Conditions for safe storage, including any incompatibilities

Storage:

Requirements to be met by storerooms and receptacles: Store in a cool, dry, well-ventilated area.

Information about storage in one common storage facility: Keep away from food, drink and feed.

Further information about storage conditions: Keep only in the original container.

Methods and material for containment and cleaning up

Methods for containment:

Contain spill. Reclaim material if possible.

Methods for clean-up:

Absorb with liquid-binding material (sand, diatomite, acid binders, universal binders, sawdust).

Ensure adequate ventilation.

Effectiveness of cleaning procedures

The formulation GWN-10616 is a water based SC formulation with 755 g/L potassium phosphonates. As potassium phosphonates are inorganic salts with a high water solubility, cleaning with water is sufficient to remove residues of potassium phosphonates. The efficiency of cleaning for Zoxamide was investigated in the following study.

| Reference: | KCP 4.2/02 |
| --- | --- |
| Report: | GWN-10392: EFFECTIVENESS OF CLEANING PROCEDURES, Fieseler, A., 2022, report No. 164781361, Doc. No. 247-001 |
| Guideline(s): | OECD No. 302 (2001) |
| Deviations: | no |
| GLP: | Yes |
| Acceptability: | Yes |

The study investigated the cleaning efficiency of a formulation containing Zoxamide and Benalaxyl-M. The following summary focuses on the results for Zoxamide.

Principle of the method

A spray tank was filled with the test formulation and water. After mixing, the content was sprayed until the tank was empty. Two cleaning procedures (2 cleaning steps and 3 cleaning steps) were tested. For each cleaning step, the tank was filled with tap water to approx. 10% of the tank capacity. The tank was shaken and the washing liquid was disposed of by spraying. The remaining residue was drained from the tank. After the last cleaning step, the tank was refilled to capacity with acetonitrile and water (10 / 90) and shaken. The solution was sprayed and the three samples were taken after approx. 20 %, 50 % and 80 % of the capacity drained. These samples were analysed for the concentration of the active substance. The analytical method was validated according to SANTE/2020/12830 (please refer to dRR section B5).

Results

The detailed results of the cleaning efficiency are shown in the following table.

Table 4.2-1 	Efficiency of cleaning procedure for Zoxamide

| Sample | Found [mg a.s./L] | Dilution factor | Calculated [mg a.s./L] | Initial concentration in spraying solution [mg a.s./L] | % of initial concentration |
| --- | --- | --- | --- | --- | --- |
| First experiment (two cleaning steps) |  |  |  |  |  |
| Sample 1 | 0.249 | 2 | 0.498 | 812 | 0.06 |
| Sample 2 | 0.174 | 2 | 0.348 | 812 | 0.04 |
| Sample 3 | 0.302 | 2 | 0.640 | 812 | 0.08 |
| Second experiment (two cleaning steps) |  |  |  |  |  |
| Sample 1 | 0.366 | 2 | 0.732 | 813 | 0.09 |
| Sample 2 | 0.368 | 2 | 0.737 | 813 | 0.09 |
| Sample 3 | 0.376 | 2 | 0.752 | 813 | 0.09 |

After both cleaning experiments, the residual Zoxamide content was below 0.1%. It was therefore shown that cleaning with tap water has sufficient cleaning efficiency for Zoxamide.

Conclusion

The study demonstrated that Zoxamide residues in the spray tank can be efficiently removed by cleaning with tap water.

## Emergency measures in the case of an accident

| Reference: | KCP 4.3/01 |
| --- | --- |
| Report: | SAFETY DATA SHEET - GWN-10616, Anonymous, 2023, report No. Version number 1.0, Doc. No. 954-001 |
| Guideline(s): | 2020/878 |
| Deviations: | No |
| GLP: | No, not required for MSDS |
| Acceptability: | Yes |

| Reference: | KCP 4.3/02 |
| --- | --- |
| Report: | SAFETY DATA SHEET - ELECTIS® K-PLUS, Anonymous, 2024, report No. Version number 1.0, Doc. No. 954-003 |
| Guideline(s): | EC/1272/2008/EC |
| Deviations: | No |
| GLP: | No, not required for MSDS |
| Acceptability: | Yes |

First aid measures:

General information:

Immediately remove any clothing soiled by the product.

Symptoms of poisoning may even occur after several hours; therefore medical observation for at least 48 hours after the accident.

After inhalation:

Remove person from dangerous area. Supply the person with fresh air and, depending on the symptoms, consult a doctor.

Supply fresh air and to be sure call for a doctor.

In case of unconsciousness place patient stably in side position for transportation.

After skin contact: Wash thoroughly with plenty of water, remove contaminated, soaked clothing immediately, consult a doctor if skin irritation occurs (reddening, etc.). Immediately wash with water and soap and rinse thoroughly.

After eye contact: Rinse opened eye for several minutes under running water. Hold eye open and rinse slowly and gently with water for several minutes.

Remove contact lenses, if present, after first 5 minutes, then continue rinsing eyes.

Call a poison control centre or doctor for treatment advice.

After swallowing: Call for a doctor immediately. Do not induce vomiting; call for medical help immediately.

Firefighting measures

Suitable extinguishing media: CO2, powder or water spray. Fight larger fires with water spray or alcohol resistant foam. Use fire extinguishing methods suitable to surrounding conditions.

Special hazards arising from the substance or mixture: Formation of toxic gases is possible during heating or in case of fire.

Advice for firefighters

Protective equipment: Wear self-contained respiratory protective device. Mouth respiratory protective device.

## Packaging and Compatibility with the Preparation (KCP 4.4)

RMS comment:

The accelerated and the two-year study were done using a HDPE/PA packaging. For this SC formulation an extrapolation on other plastic materials like proposed HDPE one is accepted.

Table 4.4-4:	Packaging information for 5 liter jerry can (first supplier)

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Rectangular / height: 370±3 mm, width: 141±3 mm, length: 189±3 mm |
| Opening: | 63±0.5 mm |
| Closure: | Screw cap 63 |

Table 4.4-5:	Packaging information for 5 liter jerry can (second supplier)

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Rectangular / height: 305±2 mm, width: 140±3 mm, length: 190±3 mm |
| Opening: | 63.4 mm |
| Closure: | Screw cap 63 |

Table 4.4-6:	Packaging information for 10 liter jerry can

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | Rectangular / height: 363 mm, width: 178 mm, length: 222 mm |
| Opening: | 61 mm |
| Closure: | Cap closing torque min: 20 Nm |

Table 4.4-7:	Packaging information for 220 liter plastic drum

| Type | Description |
| --- | --- |
| Material: | HDPE |
| Shape/size: | cylindrical / height: 935 mm, width: 553 mm, length: 583 mm |
| Opening: | 65 mm inner diameter |
| Closure: | Bung House 2’ +2’ Mauser closure (screw plug) |

## Procedures for destruction or decontamination of the plant protection product and its packaging

### Neutralisation procedures

No special measures for neutralisation are required as the product is neither strongly acidic nor strongly alkaline.

### Controlled incineration

Not applicable as the active substances contain less than 60% halogen. Small amounts of the product and uncleaned empty packaging should be transferred to a suitable incinerator equipped with an afterburner and a flue gas scrubber. Empty containers should not be used for other purposes.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| C-KCP 2.2.1/01 | Anonymous | 2023 | STATEMENT ON EXPLOSIVE AND OXIDISING PROPERTIES OF GWN-10616 (281-001) Scientific Consulting Company, Bad Kreuznach, Germany Not GLP, unpublished | N | XXXX |
| C-KCP 2.2.2/01 | Anonymous | 2023 | STATEMENT ON EXPLOSIVE AND OXIDISING PROPERTIES OF GWN-10616 (281-001) Scientific Consulting Company, Bad Kreuznach, Germany Not GLP, unpublished | N | XXXX |
| KCP 2.3.1/01 | Pardo Martinez, M. | 2021 | GWN-10616: DETERMINATION OF THE FLASH POINT CH - 0385/2021 (242-001) ChemService S.r.l., Novate Milanese, Italy GLP, unpublished | N | XXXX |
| KCP 2.3.3/01 | Pardo Martinez, M. | 2021 | GWN-10616: DETERMINATION OF THE AUTO-IGNITION TEMPERATURE FOR LIQUIDS CH - 0386/2021 (242-002) ChemService S.r.l., Novate Milanese, Italy GLP, unpublished | N | XXXX |
| KCP 2.4.2/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.5.1/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.5.2/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.6.1/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.7.1/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.7.4/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.7.5/01 | Aversa, S. | 2019 | PHYSICAL-CHEMICAL PROPERTIES OF TEST ITEM GOW F716 AFTER 2 YEARS SHELF LIFE (2 YEARS/20±2°C) BT166/17 (245-002) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.8.2/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.8.3.1/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.8.3.2/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.8.5.1.1/01 | Pardo Martinez, M. | 2021 | GWN-10616: DETERMINATION OF THE PARTICLE SIZE DISTRIBUTION CH - 0387/2021 (214-001) ChemService S.r.l., Novate Milanese, Italy GLP, unpublished | N | XXXX |
| KCP 2.8.5.1.2/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 2.8.7.2/01 | Aversa, S. | 2017 | PHYSICAL-CHEMICAL CHARACTERIZATION AND ACCELERATED STORAGE STABILITY (2 WEEKS/54±2°C) OF TEST ITEM GOW F716 BT165/17 (245-001) BIOTECNOLOGIE BT S.r.l., Todi, Italy GLP, unpublished | N | XXXX |
| KCP 4.2/01 | Anonymous | 2023 | SAFETY DATA SHEET - GWN-10616 Version number 1.0 (954-001) XXXX, Rothamsted Research, Harpenden, United Kingdom Not GLP, unpublished | N | XXXX |
| KCP 4.2/02 | Fieseler, A. | 2022 | GWN-10392: EFFECTIVENESS OF CLEANING PROCEDURES 164781361 (247-001) Ibacon GmbH, Rossdorf, Germany GLP, unpublished | N | XXXX |
| KCP 4.2/03 | Anonymous | 2024 | SAFETY DATA SHEET - ELECTIS® K-PLUS Version number 1.0 (954-003) XXXX, Rothamsted Research, Harpenden, United Kingdom Not GLP, unpublished | N | XXXX |
| KCP 4.3/01 | Anonymous | 2023 | SAFETY DATA SHEET - GWN-10616 Version number 1.0 (954-001) XXXX, Rothamsted Research, Harpenden, United Kingdom Not GLP, unpublished | N | XXXX |
| KCP 4.3/02 | Anonymous | 2024 | SAFETY DATA SHEET - ELECTIS® K-PLUS Version number 1.0 (954-003) XXXX, Rothamsted Research, Harpenden, United Kingdom Not GLP, unpublished | N | XXXX |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

Not relevant.

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Additional data on the physical, chemical and technical properties of the active substance

  - Zoxamide

None

  - Potassium phosphonates

None