FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: MEZ-HER 100 SC Product name(s): MECORN 100 SC Chemical active substance: mesotrione, 100 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Pestila Sp. z o. o. Submission date: October 2023, March 2024 MS Finalisation date: May 2024, August 2024

Version history

| When | What |
| --- | --- |
| 03/2024 | Applicant’s update |
| 05/2024 | dRR assessment by zRMS PL |
| 08/2024 | The final Registration Report after 1st commenting period |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	7

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	13

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	14

5.2.2	Methods for the determination of residues (KCP 5.1.2)	14

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	15

5.3.1	Analysis of the plant protection product (KCP 5.2)	15

5.3.2	Description of analytical methods for the determination of residues of active substance mesotrione (KCP 5.2)	16

5.3.2.1	Overview of residue definitions and levels for which compliance is required	16

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	17

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	18

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	18

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	19

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	20

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	20

5.3.2.8	Other studies/ information	21

Appendix 1	Lists of data considered in support of the evaluation	22

Appendix 2	Detailed evaluation of submitted analytical methods	24

A 2.1	Analytical methods for the active substance	24

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	24

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	24

# Analytical methods

## Conclusion and summary of assessment

There are no data gaps related to analytical methods for the determination of a.s. and relevant impurities and their validations in formulation Mecorn 100 SC.

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are:

• none

Note: The table List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review should be completed before registration

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Maize | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of active substance mesotrione in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical method is suitable for the determination of the active substance mesotrione in the plant protection product Mecorn 100 SC. The proposed analytical method has been fully validated in terms of interference, specificity, linearity, accuracy and precision. The proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | 5.1.1 |
| --- | --- |
| Report | Analytical Method Validation for Active Ingredient and impurities Content Determination of the MEZ-HER 100 SC, Digrandi S., 2023, report No. 23214-01C |
| Guideline(s): | Yes, 3030/99 rev.5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The content of active substance in the examined sample was determined by high performance liquid chromatography HPLC-DAD and external standard method.

Name: MEZ-HER 100 SC

Active ingredient name: Mesotrione

Active ingredient (IUPAC name): 2-(4-Mesyl-2-nitrobenzoyl)-1,3-cyclohexanedione

CAS Number: 104206-82-8

Nominal content: 100 g/L

Formulation type: Suspension concentrate (SC)

Batch number: MEZ/01/23

Manufacturing date: 02/2023

Expiry date: 02/2025

Equipment and chromatographic conditions for mesotrione analysis

| LC system | HPLC Series 1100 Agilent |  |  |
| --- | --- | --- | --- |
| Analytical column | Phenomenex, Hypersil ODS, C18 5 µm 250x4.6 mm |  |  |
| Solvent A: | Acetonitrile |  |  |
| Solvent C: | Water 0.1% Phosphoric acid |  |  |
| Flow: | 1 mL/min |  |  |
| Pump Parameters | Time [min] | A (%) | C (%) |
|  | 0.00 | 55 | 45 |
|  | 7.00 | 55 | 45 |
| Oven temperature : | 30 °C |  |  |
| Injection volume: | 2 µL |  |  |
| Detector wavelength: | 272 nm |  |  |
| Chromatogram time: | 7.00 min |  |  |
| Retention times: | ~4.0 min (Mesotrione) |  |  |

The preparation of standard and sample solutions

Standard solution

Taking into account the analytical standard purity, two sets of reference item stock solutions in acetonitrile were prepared as follows:

Table 1: Stock solutions preparation for Mesotrione determination

| Reference item |  | Purity | Weight | Final Volume | Conc.1 | Id. Code |
| --- | --- | --- | --- | --- | --- | --- |
| Id. Code and batch | Operation n°. | [%] | [mg] | [mL] | [mg/mL] | - |
| SR 237 Batch BCCD6166 | 20 | 99.6 | 60.7 | 25 | 2.4183 | SM 237-20 |
|  | 21 | 99.6 | 23.1 | 10 | 2.3008 | SM 237-21 |

1: Stock solution concentration is calculated as

Table 2: Reference item working solutions used in the calibration and determination of Mesotrione

| Id Code | Initial Conc. | Volume taken | Final volume | Final Conc.1 | % w/w 2 | Id. Code |
| --- | --- | --- | --- | --- | --- | --- |
|  | [mg/mL] | [mL] | [mL] | [mg/mL] | [%] |  |
| SM 237-20 | 2.4183 | 0.70 | 5 | 0.3386 | 6.2 | SL 237-20 A |
| SM 237-20 | 2.4183 | 0.90 | 5 | 0.4353 | 7.9 | SL 237-20 B |
| SM 237-20 | 2.4183 | 1.10 | 5 | 0.5320 | 9.7 | SL 237-20 C |
| SM 237-20 | 2.4183 | 1.40 | 5 | 0.6771 | 12.3 | SL 237-20 D |
| SM 237-20 | 2.4183 | 1.80 | 5 | 0.8706 | 15.8 | SL 237-20 E |
| SM 237-21 | 2.3008 | 1.20 | 5 | 0.5522 | 10.0 | SL 237-21 QC |

1: Reference item solution concentration is calculated as

2: % w/w (analyte weight/sample weight) based on nominal test item solution concentration of 5.5 mg/mL, calculated as

Sample solution

Test item was accurately weighed into a class A volumetric flask, then made up to volume with acetonitrile and opportunely diluted to reach concentration of 5.5 mg/mL and analysed by HPLC-DAD. Five replicates were prepared for method validation and determination.

Validation - Results and discussions

Table 5.2-3:	Methods suitable for the determination of active substance mesotrione in plant protection product MEZ-HER 100 SC

|  | mesotrione |
| --- | --- |
| Author(s), year | Digrandi S., 2023 |
| Principle of method | SANCO/3030/99 rev.5 |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) n = 5 | The linearity of analytical method was assessed using mesotrione standard solutions with concentration range from 0.3386 mg/mL to 0.8706 mg/mL (from 6.2% to 15.8% of nominal mesotrione content in the sample). Correlation coefficient: R2 = 0.9999 Required: R2 ≥ 0.98. |
| Precision – Repeatability Mean n = 5 (%RSD) | RSD = 0.44% RSDr = 1.89% Horrat 0.23. |
| Accuracy (% Recovery) | Marginal recovery 97.5% (range: 98.9% - 100.5%) Required: 90-110 %. |
| Interference/ Specificity | No interference. |
| Comment | No comments. |

Conclusion

The HPLC-DAD method, used to quantify mesotrione in MEZ-HER 100 SC was fully validated. Method validation included linearity, non-analyte interference, precision, accuracy and specificity. All measured parameters meet the criteria given in SANCO/3030/99 rev.5.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical methods are suitable for the determination of the relevant impurities R287431, R287432, and 1,2 dichloroethane in the plant protection product Mecorn 100 SC. The proposed analytical methods have been fully validated in terms of interference, specificity, linearity, accuracy, precision, and LOQs. The proposed methods fulfil the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical methods has been accepted. |  |
| --- | --- | --- |
| Reference: |  | 5.1.1/02 |
| Report: |  | Analytical Method Validation for Active Ingredient and impurities Content Determination of the MEZ-HER 100 SC, Digrandi S., 2023, report no. BF – 23214-01C |
| Guideline(s): |  | Yes, SANCO/3030/99 rev.5 |
| Deviations: |  | No |
| GLP: |  | Yes |
| Acceptability: |  | Yes |

Materials and methods for R287432 impurity content

Determination of R287432 in MEZ-HER 100 SC was performed with HPLC-MS/MS instrument for specificity analysis. Spectra of analyte is the same in standard, samples and recovery samples.

Equipment and chromatographic conditions for R287432 impurity analysis

| Chromatographic System: | HPLC Series 1290 AGILENT |  |  |
| --- | --- | --- | --- |
|  | TRIPLE TOF 4600 AB SCIEX detector |  |  |
| Analytical Column: | Acilent, ZORBAX Eclipse Plus C18, C18 5 µm 50x2.1 mm |  |  |
| Solvent A: | Water 1% Formic Acid |  |  |
| Solvent B: | ACN |  |  |
| Flow: | 0.1 mL/min |  |  |
| Pump parameters | Time [min] | Phase A (%) | Phase B (%) |
|  | 0.00 | 90 | 10 |
|  | 20 | 90 | 10 |
|  | 21 | 70 | 30 |
|  | 23 | 70 | 30 |
|  | 23.01 | 90 | 10 |
|  | 25 | 90 | 10 |
| Temp Column Temperature: | 35 °C |  |  |
| Injection volume: | 3 μL |  |  |
| Mass Detector: | Ionisation mode: APCI Positive product ion |  |  |
|  | Temperature (TEM): 650 °C |  |  |
|  | Curtain gas (CUR): 30 psi |  |  |
|  | Collision gas (CAD): 5 psi |  |  |
|  | Ion Spray Voltage (IS): 4500 V |  |  |
|  | Gas 1: 45 psi |  |  |
|  | Gas 2: 40 psi |  |  |
|  | DP: 50 |  |  |
|  | CE: 20 |  |  |
| Retention times: | ~21.75 min (R287432) |  |  |

Test Item Solutions Preparation

Test item was weighted in a volumetric class A flask and made up to volume with Methanol to reach a concentration of 35.0 mg/mL. Samples were accurately homogenised by vigorous hand shaking, sonicated for 10 minutes and filtered with PTFE filters.

Reference Item Solutions Preparation

Taking into account the analytical standard purity, one set of reference item stock solution in methanol was prepared as follows.

Table 3: Stock solutions for R287432 determination

| Stock solution | Purity | Weight | Volume | Conc. | Id. Code |
| --- | --- | --- | --- | --- | --- |
| Id. code and batch | [%] | [mg] | [mL] | [mg/mL] |  |
| SR 904 Batch EPPJOW 34.1 | 99.80 | 13 | 20 | 0.6487 | SM 904-3 |

From the above stock solution were prepared the following reference working solutions for linearity.

Table 4: Reference item working solutions for R287432 determination

| Id Code | Initial Conc. | Volume taken | Final volume | Final Conc.1 | g/Kg referred to technical material2 | g/L referred to test Item3 | Id. Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | [mg/mL] | [mL] | [mL] | [mg/mL] | [g/kg] | [g/L] |  |
| SM 904-3 | 0.6487 | 1.00 | 20 | 0.0324 | 8.5331 | 0.9573 | SL 904-3 INT |
| SL 904-3 INT | 0.0324 | 0.65 | 25 | 0.0008 | 0.2219 | 0.0249 | SL 904-3 A |
| SL 904-3 INT | 0.0324 | 0.55 | 10 | 0.0018 | 0.4693 | 0.0527 | SL 904-3 B |
| SL 904-3 INT | 0.0324 | 0.85 | 10 | 0.0028 | 0.7253 | 0.0814 | SL 904-3 C |
| SL 904-3 INT | 0.0324 | 0.65 | 5 | 0.0042 | 1.1093 | 0.1244 | SL 904-3 D |
| SL 904-3 INT | 0.0324 | 0.85 | 5 | 0.0055 | 1.4506 | 0.1627 | SL 904-3 E |
| SL 904-3 INT | 0.0324 | 0.90 | 3 | 0.0097 | 2.5599 | 0.2872 | SL 904-3 F |

1: Reference item solution concentration is calculated as

2: g/Kg referred to technical material based on nominal test item solution concentration of 35.0 mg/mL, %purity of technical material of 93%, Content of AI % of 10.1 %; calculated as

3: g/L referred to the test item based on nominal test item solution concentration of 35.0 mg/mL, Density of test item of 1.033 g/mL; calculated as

Materials and methods for R287431 impurity content

The analysis was performed by the highly specific HPLC-TOF detection system; the molecular ion 344 for quantification and two fragmentation ions: 314 and 281 were detected simultaneously for confirmatory purposes.

The retention time of the analyte in the reference item matched the retention time of the analyte in the samples and recoveries solution.

The specificity of the method is confirmed by the ratio of qualifier transition to target transition: for all analytical samples with analyte the ratio was between ± 30 % of the averaged ratio calculated for the analytical standards.

Equipment and chromatographic conditions for R287431 impurity analysis

| LC System: | HPLC Series 1290 AGILENT |  |  |
| --- | --- | --- | --- |
| MS/MS detector System | TRIPLE TOF 4600 AB SCIEX deterctor |  |  |
| Analytical Column: | Agilent ZORBAX Eclipse Plus Column C18 1.8 µm 50x2.1mm |  |  |
| Solvent A: | Water |  |  |
| Solvent B: | MeOH |  |  |
| Pump parameters | Time [min] | Phase A (%) | Phase B (%) |
|  | 0.00 min | 80 | 20 |
|  | 3.00 min | 20 | 80 |
|  | 4.00 min | 20 | 80 |
|  | 4.10 min | 5 | 95 |
|  | 5.00 min | 5 | 95 |
|  | 5.10 min | 80 | 20 |
|  | 7.00 min | 80 | 20 |
| Flow rate: | 0.3 mL/min |  |  |
| Column Temperature: | 35 °C |  |  |
| Injection volume: | 20 μL |  |  |
| Retention time: | R287431: 2.75 min |  |  |
| Mass Detector: | Ionisation mode: APCI negative product ion |  |  |
|  | Temperature (TEM): 650 °C |  |  |
|  | Curtain gas (CUR): 30 psi |  |  |
|  | Collision gas (CAD): 5 psi |  |  |
|  | Ion Spray Voltage (IS): -4500 V |  |  |
|  | Gas 1: 45 psi |  |  |
|  | Gas 2: 40 psi |  |  |
|  | DP: -60 |  |  |
|  | CE: -22 |  |  |
| Transitions: | R287431-1 (344/314) R287431-2 (344/281) |  |  |

Test Item Solutions Preparation

The sample was accurately homogenised by vigorous hand shaking then (0.65 ± 0.05) g of the were accurately weighed into a 15 mL plastic tube. Fortification into the spike recoveries was made at this point.

Then the sample was made up to the 10 mL mark with methanol solvent, the tube was tightly sealed and vigorously shaken for 1 minute, then centrifuged at 4000 rpm for 5 minutes.

Taking care not to move the test item residue on the bottom of the tube, 5 mL of the supernatant limpid layer were transferred into a second 15 mL plastic tube, made up to the 10 mL mark with pure water (1:1 dilution) and mixed thoroughly after sealing the tube.

The white emulsion in the tube was then centrifuged at 4000 rpm for 5 minutes: a yellow precipitate separetes at the bottom of the tube. An aliquot of 0.75 mL of the supernatant limpid layer was transferred into a 2-mL Eppendorf microcentrifuge tube and 0.75 mL of methanol/water 40:60 solvent mixture was added (final sample volume after dilutions = 40 mL). The tube was sealed, mixed thoroughly and ultracentrifuged at 13500 rpm for 5 minutes.

The final extract was finally sampled from the upper layer of the tube and gently filtered by 0.22 µm, 12 mm diameter PTFE filter. The filter was previously conditioned with methanol solvent (around 10 mL) and then dried with air (around 2 x 10 mL).

Reference Item Solutions Preparation

Taking into account the analytical standard purity, one set of reference item stock solution in matrix (placebo) with solvent mixture methanol/water was prepared as follows.

Table4: Stock solutions for R287431 determination

| Stock solution | Purity | Weight | Volume | Conc. | Id. Code |
| --- | --- | --- | --- | --- | --- |
| Id. code and batch | [%] | [mg] | [mL] | [µg/mL] |  |
| SR 893 Batch 13-GUY-162-1 | 96.3 | 5.4 | 50 | 104.05 | SM 893-4 |

From the above stock solution were prepared the following reference working solutions for linearity.

Table 5: Reference item working solutions for R287431 determination

| Id Code | Initial Conc. | Volume taken | Final volume | Final Conc.1 | mg/Kg referred to technical material2 | mg/L referred to test Item3 | Id. Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | [µg/mL] | [µL] | [mL] | [ng/mL] | [mg/kg] | [mg/L] |  |
| SM 893-4 | 104.047 | 950 | 10 | 9884.4650 | - | - | SL INT 1 |
| SL INT 1 | 9.8845 | 400 | 20 | 197.6893 | - | - | SL INT 2 |
| SL INT 2 | 0.1977 | 5.0 | 1 | 0.9884 | 0.5601 | 0.0628 | Stm 1.0 ppb |
| SL INT 2 | 0.1977 | 7.0 | 1 | 1.3838 | 0.7841 | 0.0880 | Stm 1.4 ppb |
| SL INT 2 | 0.1977 | 10 | 1 | 1.9769 | 1.1202 | 0.1257 | Stm 2.0 ppb |
| SL INT 2 | 0.1977 | 15 | 1 | 2.9653 | 1.6803 | 0.1885 | Stm 3.0 ppb |
| SL INT 2 | 0.1977 | 20 | 1 | 3.9538 | 2.2404 | 0.2513 | Stm 4.0 ppb |
| SL INT 2 | 0.1977 | 25 | 1 | 4.9422 | 2.8005 | 0.3142 | Stm 5.0 ppb |

1: Reference item solution concentration is calculated as

2: mg/Kg referred to technical material based on nominal test item solution concentration of 65.0 mg/mL, % purity of technical material of 93%, Content of AI % of 10.1%; calculated as

3: mg/L referred to the test item based on nominal test item solution concentration of 65.0 mg/mL, Density of test item of 1.033 g/mL; calculated as

Materials and methods for 1,2 dichloroethane content

Solutions for standard and test item were analysed with GC-MS instrument for specificity analysis. Coherently with precision quantification analysis, no peak is seen in test item samples.

Solutions were prepared by diluting the sample in Methanol.

Equipment and chromatographic conditions for 1,2 dichloroethane analysis

| Instrumental system: | Agilent 7890B GC with Mass Selective Detector 7000B |
| --- | --- |
| Analytical Column: | Agilent J&W, DB-624 UI, Chromatographic column 30m *0.25mm*1,40µm |
| Oven: | 100°C hold 1.0 min 15°C/min to 280°C hold 1.0 min |
| Injector temperature: | 240 °C |
| Injection volume | 1 µL |
| Split ratio: | 4:1 |
| Carrier gas: | Helium |
| Flow(s): | 1.2 mL/min |
| Transfer line temperature: | 280°C |
| Mass Detector: | Quadrupole temperature 150 °C |
|  | Source temperature 230 °C |
|  | Electron ionization |
|  | Positive mode |
| Run time: | 14.0 min |
| Retention time: | 3.85 min (1,2-Dichloroethane) |

Test Item Solutions Preparation

Test item was weighted in a volumetric class A flask and made up to volume with Methanol to reach a concentration of 35.0 mg/mL. Samples were accurately homogenised by vigorous hand shaking, sonicated for 10 minutes and filtered with PTFE filters.

Reference Item Solutions Preparation

Taking into account the analytical standard purity, one set of reference item stock solution in methanol was prepared as follows.

Table 7: Stock solutions for 1,2 dichloroethane determination

| Stock solution | Purity | Weight | Volume | Conc. | Id. Code |
| --- | --- | --- | --- | --- | --- |
| Id. code and batch | [%] | [mg] | [mL] | [mg/mL] |  |
| SR 534 Batch LRAC7143 | 99.00 | 33.5 | 50 | 0.6633 | SM 534-9 |
|  | 99.00 | 10.3 | 50 | 0.2039 | SM 534-10 |

From the above stock solution were prepared the following reference working solutions for linearity.

Table 8: Reference item working solutions for 1,2 dichloroethane determination

| Id Code | Initial Conc. | Volume taken | Final volume | Final Conc.1 | g/Kg referred to technical material2 | g/L referred to test Item3 | Id. Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | [mg/mL] | [mL] | [mL] | [mg/mL] | [g/kg] | [g/L] |  |
| SM 534-9 | 0.6633 | 0.50 | 20 | 0.0166 | 4.3626 | 0.4894 | SL 534-9 INT |
| SL 534-9 INT | 0.0166 | 1.00 | 10 | 0.0017 | 0.4363 | 0.0489 | SL 534-9 A |
| SL 534-9 INT | 0.0166 | 1.30 | 10 | 0.0022 | 0.5671 | 0.0636 | SL 534-9 B |
| SL 534-9 INT | 0.0166 | 1.50 | 10 | 0.0025 | 0.6544 | 0.0734 | SL 534-9 C |
| SL 534-9 INT | 0.0166 | 0.90 | 5 | 0.0030 | 0.7853 | 0.0881 | SL 534-9 D |
| SL 534-9 INT | 0.0166 | 1.20 | 5 | 0.0040 | 1.0470 | 0.1175 | SL 534-9 E |
| SM 534-10 | 0.2039 | 0.30 | 25 | 0.0024 | 0.6438 | 0.0722 | SL 534-10 QC |

1: Reference item solution concentration is calculated as

2: g/Kg referred to technical material based on nominal test item solution concentration of 35.0 mg/mL, %purity of technical material of 93%, Content of AI % of 10.1%; calculated as

3: g/L referred to the test item based on nominal test item solution concentration of 35.0 mg/mL, Density of test item of 1.033 g/mL; calculated as

Table 5.2-6:	Methods suitable for the determination of the relevant impurities: R287432, R287431 and 1,2-Dichloroethane in plant protection product (PPP) 
MEZ-HER 100 SC

|  | R287432 | R287431 | 1,2-Dichloroethane |
| --- | --- | --- | --- |
| Author(s), year | Digrandi S., 2023 |  |  |
| Principle of method | SANCO/3030/99 rev.5 |  |  |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | The linearity of the analytical method was assessed using standards solutions of R287432 in the concentration range from 0.0008 mg/mL to 0.0097 mg/mL (corresponding to R287432 conc. in test item of 0.0249 - 0.2872 g/L). n = 6 Correlation coefficient: R2 = 0.9899 Required: R2 ≥ 0.99. | The linearity of the analytical method was assessed using standards solutions of R287431 in the concentration range from 0.9884 ng/mL to 4.9422 ng/mL. (corresponding to R287431 conc. in test item of 0.0628 - 0.3142 mg/L). n = 6 Correlation coefficient: R2 = 0.9988 Required: R2 ≥ 0.99. | The linearity of the analytical method was assessed using standards solutions of 1,2-Dichloroethane in the concentration range from 0.0017 mg/mL to 0.0040 mg/mL. (corresponding to 1,2-dichloroethane conc. in test item of 0.0489 - 0.1175 g/L). n = 5 Correlation coefficient: R2 = 0. 0.9994 Required: R2 ≥ 0.99. |
| Precision – Repeatability (%RSD) | n = 5 Hr = 0.26 Required: Hr ≤ 1 RSD = 1.63% RSDr = 6.14% | n = 5 Hr = 0.08 Required: Hr ≤ 1 RSD = 1.24% RSDr = 15.47% | n = 5 Hr = 0.27 Required: Hr ≤ 1 RSD = 1.55% RSDr = 5.77% |
| Accuracy (% Recovery) | Marginal recovery 99.1% (range: 93.7% - 102.4%) for LOQ level (n = 5) 103.6% (range: 100.6% - 106.6%) for higher level (n = 2) Required: 75% - 125%. | Marginal recovery 83.7% (range: 80.7% - 86.3%) for LOQ level (n = 5) 86.2% (range: 85.0% - 87.5%) for higher level (n = 2) Required: 75% - 125%. | Marginal recovery 118.1% (range: 115.6% - 119.9%) for LOQ level (n = 5) 114.5% (range: 113.2% - 115.9%) for higher level (n = 2) Required: 75% - 125%. |
| Interference/ Specificity | Fulfilled. | Fulfilled. | Fulfilled. |
| LOQ | 0.0026% (0.0009 mg/mL) of the preparation), 0.0268 g/L. | 0.0000085% (1.3838 ng/mL) of the preparation, 0.0880 mg/L. | 0.0062% (0.0022 mg/mL) of the preparation), 0.0636 g/L. |
| Comment | No comments. | No comments. | No comments. |

Conclusion

Determination of residues of relevant impurities: R287432, R287431 and 1,2-Dichloroethane was fully validated. The methods for determination are specific. The validation parameters for linearity, instrument precision, limit of quantification, repeatability and accuracy are within the acceptance range. There are not any interferences between relevant impurities and other ingredients of the samples. The methods had good precision, accuracy and the linearity and fulfil requirements of SANCO/3030/99 rev.5.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

There are no relevant formulants in product MEZ-HER 100 SC therefore no methods are required.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

There is no CIPAC method available for the determination of mesotrione in SC formulations.

Information from Renewal RR for Callisto 100 SC that are also relevant for evaluation of MEZ-HER 100 SC is provided below. Since data protection for this information expired, summary of relevant information was copied from Renewal RR for Callisto 100 SC. Information copied form Renewal RR for Callisto 100 SC were evaluated and accepted in renewal process of Callisto 100 SC and are deemed to be acceptable for MEZ-HER 100 SC.

### Methods for the determination of residues (KCP 5.1.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to the methods described in Renewal RR for Callisto 100 SC, which are, in applicant’s opinion, fulfil requirements of “Guidance Document on Pesticide Analytical Methods for Risk Assessment and Post-approval Control and Monitoring Purposes” (SANTE/2020/12830, Rev.2, 14. February 2023).

An overview on the acceptable methods and possible data gaps for analysis of residues of mesotrione for the generation of pre-authorization data is given in the following tables.

Table 5.2-7:	Validated methods for the generation of pre-authorization data

| Component of residue definition: Mesotrione (and MNBA) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High protein/high starch content (dry) Maize forage, maize grain | TMR0643B | 0.01 mg/kg | HPLC-FL | Method: *Alferness, 1996 Report: TMR0643B Validation: *Bolygo, 1996 Report: RJ0689B EU agreed (UK, 2015, 2015a) |
| High water content Maize fodder |  |  |  |  |
| High protein/high starch content (dry) Maize grain, maize silage, maize stover | RAM 366/01 | 0.01 mg/kg | HPLC-MS/MS (2 transitions) | Method: Crook, 2002 Report: RAM 366/01 Validation: Hill, 2004 Report: RJ3253B Bruns et al. 2001 Report: ZA1296/0656 EU agreed (UK, 2015, 2015a) |
| High water content Maize whole plant |  |  |  |  |
| High water content Oilseed rape whole plant |  | 0.01 mg/kg |  | Validation: Malet & Allard, 2010 Report: RXCO00307 EU agreed (UK, 2015, 2015a) |
| High oil content Oilseed rape seed |  |  |  |  |
| High oil content Linseed seed |  | 0.01 mg/kg |  | Validation: Simon, 2004 Report: gli058003 EU agreed (UK, 2015, 2015a) |
| High oil content Poppyseed seed |  | 0.01 mg/kg |  | Validation: Simon, 2004a Report: gpp067003 EU agreed (UK, 2015, 2015a) |
| High protein/high starch content (dry) Maize grain whole | GRM007.11A (update to RAM 366/01) | 0.01 mg/kg | HPLC-MS/MS (2 transitions) | Method: Watson & Crook, 2013 Report: GRM007.11A Validation: Watson, 2013 Report: S12-03629 Amic, 2013 Report: S13-02460 EU agreed (UK, 2015, 2015a) |
| High water content Maize forage |  |  |  |  |
| High oil content Oilseed rape seed |  |  |  |  |
| High acid content Whole orange |  |  |  |  |

* Methods have been inserted also in the post-authorization section

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of active substance mesotrione (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-8:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| High water content Lettuce | Mesotrione | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| High acid content Orange |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| High oil content Sunflower seed |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| High protein/high starch content (dry) Maize grain, dry broad bean |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| Difficult matrices |  | 0.05 mg/kg | Regulation (EU) No 2017/626 |
| Muscle | (Mesotrione)(a) | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| Milk |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| Eggs |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| Fat |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| Liver, kidney |  | 0.01 mg/kg | Regulation (EU) No 2017/626 |
| Soil (Ecotoxicology) | Mesotrione MNBA AMBA | 0.05 mg/kg 0.002 mg/kg | common limit EFSA Journal 2016;14(3):4419 |
| Drinking water (Human toxicology) | Mesotrione MNBA AMBA | 0.1 µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | Mesotrione MNBA AMBA | 0.0077 mg/L (EC50 Lemna gibba) | EFSA Journal 2016;14(3):4419 |
| Air | Mesotrione | 1.5 µg/m3 (AOEL sys = 0.005 mg/kg bw/day (EFSA 2016)) | Calculated according to SANTE/2020/12830, Rev.2 14. February 2023 |
| Tissue (meat or liver) | Mesotrione | 0.01 mg/kg | Validation: Watson, 2013b Report S12-03250 EU agreed (UK, 2015, 2015a) |
| Body fluids |  | 0.01 mg/kg |  |

(a) No residue definition has been set for products of animal origin.

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

An overview on the acceptable methods and possible data gaps for analysis of mesotrione in plant matrices is given in the following tables. No new data are submitted in the framework of this application.

Table 5.3-9:	Validated methods for food and feed of plant origin

| Component of residue definition: mesotrione |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content Maize forage | QuEChERS | 0.01 mg/kg | LC-MS/MS (multi-residue) | QuEChERS Method and validation: Watson, 2013a Report: S12-03251 ILV: Tessier, 2013 Report: S12-04607 EU agreed (UK, 2015, 2015a) |
|  | ILV (QuEChERS) | 0.01 mg/kg |  |  |
| High acid content Whole orange | QuEChERS | 0.01 mg/kg |  |  |
| High oil content Oilseed rape seed | QuEChERS | 0.01 mg/kg |  |  |
| High protein/high starch content (dry) Maize kernel | QuEChERS | 0.01 mg/kg |  |  |
|  | ILV (QuEChERS) | 0.01 mg/kg |  |  |
| High protein/high starch content (dry) Maize forage, maize grain | TMR0643B | 0.01 mg/kg | HPLC-FL | Method: Alferness, 1996 Report: TMR0643B Validation: Bolygo, 1996 Report: RJ2149B EU agreed (UK, 2015, 2015a) |
| High water content Maize fodder |  |  |  |  |

Table 5.3-10:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | Studies on the metabolism of mesotrione in maize incorporate a number of different extraction steps, one of which is extraction with acetonitrile/water in a 1:1 ratio. Residues of mesotrione in grain were extremely low and therefore no data are available to address the extraction efficiency in grain however data are available from samples of maize fodder and forage leaf as these samples contained the majority of the radioactivity. These data indicate that the majority of the total radioactive residue obtained via solvent extraction from these matrices was extracted via use of acetonitrile/water and subsequent characterisation indicated that these extracts contained the residue of mesotrione. This would therefore indicate that the use of acetonitrile/water in a 1:1 v/v ratio is effective for extraction of residues of mesotrione. Wei & Dohn, 1997 Report: RR 96-026B Tarr & van Neste, 1997 Report: RR96-007B EU agreed (UK, 2015, 2015a) |
| Not required, because: | - |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

An overview on the acceptable methods and possible data gaps for analysis of mesotrione in animal matrices is given in the following tables. No new data are submitted in the framework of this application.

Table 5.3-11:	Validated methods for food and feed of animal origin

| Component of residue definition: (mesotrione)(a) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Muscle | QuEChERS | 0.01 mg/kg | QuEChERS LC-MS/MS (multi-residue) | QuEChERS Method and validation: Watson, 2013b Report: S12-03250 EU agreed (UK, 2015, 2015a) ILV: Bernal, 2013 Report: S12-04608 EU agreed (UK, 2015, 2015a) |
| Eggs Fat | QuEChERS | 0.01 mg/kg |  |  |
| liver | QuEChERS | 0.01 mg/kg |  |  |
|  | ILV | 0.01 mg/kg |  |  |
| Kidney | QuEChERS | 0.01 mg/kg |  |  |
| Milk | QuEChERS | 0.01 mg/kg |  |  |
|  | ILV | 0.01 mg/kg |  |  |
| Eggs | QuEChERS | 0.01 mg/kg |  |  |
|  | ILV | 0.01 mg/kg |  |  |
| Whole blood | QuEChERS | 0.01 mg/kg |  |  |

(a) No residue definition has been set for products of animal origin.

Table 5.3-12:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Taking into account that intake of mesotrione is not significant, studies on active substance metabolism and extraction efficiency in animal matrices are not required. No monitoring method for residues in animal products is necessary. No MRLs have been set for products of animal origin. |

#### Description of methods for the analysis of soil (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

An overview on the acceptable methods and possible data gaps for analysis of mesotrione in soil is given in the following tables. No new studies have been submitted with this application.

Table 5.3-13:	Validated methods for soil

| Component of residue definition: mesotrione, MNBA and AMBA |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| GRM007.10A | 0.002 mg/kg | HPLC-MS/MS | Method: Jutsum & Williams, 2013 GRM007.10A Validation: Jutsum, 2013 CEMR-5657-REG EU agreed (UK, 2015, 2015a) |

#### Description of methods for the analysis of water (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

An overview on the acceptable methods and possible data gaps for analysis of mesotrione in sur-face and drinking water is given in the following tables. No new studies have been submitted with this application.

Table 5.3-14:	Validated methods for water

| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| --- | --- | --- | --- | --- |
| Component of residue definition: mesotrione, MNBA and AMBA |  |  |  |  |
| Drinking water | GRM007.09A | 0.05 (surface water) μg/L | HPLC-MS/MS | Method: Jutsum & Chamkesam, 2013 Report GRM007.09A Validation: Jutsum, 2013a Report CEMR-5658-REG EU agreed (UK, 2015, 2015a) |
|  | ILV (GRM007.09A) | 0.05 μg/L | HPLC-MS/MS | ILV Wiesner & Breyer, 2013 Report S13-04185 EU agreed (UK, 2015, 2015a) |
| Ground water | GRM007.09A | 0.05 μg/L | HPLC-MS/MS | Method: Jutsum & Chamkesam, 2013 Report GRM007.09A Validation: Jutsum, 2013a Report CEMR-5658-REG EU agreed (UK, 2015, 2015a) |
| Component of residue definition: Mesotrione and MNBA |  |  |  |  |
| Drinking water | TMR0707B | 0.05 μg/L | GC-MSD | Method: Meyers, 1997 Report TMR0707B EU Agreed (UK, 2015, 2015a) |

#### Description of methods for the analysis of air (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

An overview on the acceptable methods and possible data gaps for analysis of mesotrione in air is given in the following tables. No new studies have been submitted with this application.

Table 5.3-15:	Validated methods for air

| Component of residue definition: mesotrione |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| RR 97-031B | 0.01 μg/m3 | HPLC-UV | Method: Leung, 1997 Report RR 97-031B EU agreed (UK, 2015, 2015a) |
| GRM007-08B | 0.45 μg/m3 | HPLC-MS/MS | Method: Jutsum, 2013b GRM007.08B Validation: Jutsum, 2013c CEMR-5403-REG EU agreed (UK, 2015, 2015a) |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

Not relevant for new registration according to art. 34 of Reg. 1107/2009 based on data which protection period has expired. For the purpose of evaluation of MEZ-HER 100 SC please refer to Renewal RR for Callisto 100 SC.

The following methods can be used to determine residue levels of mesotrione in body fluids and tissues.

No new data are submitted in the framework of this application.

Table 5.3-16:	Methods for body fluids and tissues

| Component of residue definition: mesotrione |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Whole blood | QuEChERS Method | 0.01 mg/kg | HPLC-MS/MS | Validation: Watson, 2013b Report S12-03250 EU agreed (UK, 2015, 2015a) |

#### Other studies/ information

Not relevant.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1 KCP 5.1.1/02 | Digrandi S. | 2023 | Analytical Method Validation for Active Ingredient and impurities Content Determination of the MEZ-HER 100 SC. Report No 23214-01C Renolab S.r.l. GLP: Yes Unpublished | N | Pestila* |

* Pestila spółka z ograniczoną odpowiedzialnością

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for the active substance

    - Methods used for the generation of pre-authorization data (KCP 5.1)

New analytical method for determination of active substance and its relevant metabolites in plant protection product have been submitted and summarised in points 5.2.1.1. and 5.2.1.2. Submitted methods can be used for post-authorisation monitoring.

      - Description of analytical methods for the determination of residues in support of environmental fate studies (KCP 5.1.2.1)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of efficacy studies (KCP 5.1.2.2)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of residues studies (KCP 5.1.2.5)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of ecotoxicological studies (KCP 5.1.2.6)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2.1)

No new data have been submitted in the framework of this application.

      - Description of analytical methods for the determination of residues in support of toxicological studies

No new or additional studies have been submitted.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted.

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted.