FINAL REGISTRATION REPORT Part B Section 8 Environmental Fate Detailed summary of the risk assessment

Product code: AF-777 Product name(s): To be defined later Chemical active substance: 1-Methylcyclopropene (1-MCP), 20 g/kg (2.0 %)

Interzonal Interzonal Rapporteur Member State: Poland (PL)

CORE ASSESSMENT Authorisation according to Article 33

Applicant: AgroFresh Holding France S.A.S. Submission date: 18.01.2021 MS Finalisation date: 10.2021; 03.2022

Version history

| When | What |
| --- | --- |
| January 2021 | Original AgroFresh submission |
| October 2021 | Draft assessment prepared by zRMS for commenting |
| 03/2022 | The final version of RR |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	4

8.1	Critical GAP and overall conclusions	7

8.2	Metabolites considered in the assessment	9

8.3	Rate of degradation in soil (KCP 9.1.1)	9

8.3.1	Aerobic degradation in soil (KCP 9.1.1.1)	10

8.3.2	Anaerobic degradation in soil (KCP 9.1.1.1)	10

8.4	Field studies (KCP 9.1.1.2)	10

8.4.1	Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)	10

8.4.2	Soil accumulation testing (KCP 9.1.1.2.2)	10

8.5	Mobility in soil (KCP 9.1.2)	10

8.5.1	Column leaching (KCP 9.1.2.1)	11

8.5.2	Lysimeter studies (KCP 9.1.2.2)	11

8.5.3	Field leaching studies (KCP 9.1.2.3)	11

8.6	Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)	11

8.7	Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)	11

8.7.1	Justification for new endpoints	11

8.7.2	Active substance(s) and relevant metabolite(s)	13

8.7.2.1	PECsoil of AF-777	16

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	16

8.8.1	Justification for new endpoints	16

8.8.2	Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)	17

8.9	Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)	19

8.9.1	Justification for new endpoints	19

8.9.2	Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)	20

8.9.2.1	PECsw/sed of AF-777	21

8.10	Fate and behaviour in air (KCP 9.3, KCP 9.3.1)	24

Appendix 1	Lists of data considered in support of the evaluation	26

Appendix 2	Detailed evaluation of the new Annex II studies	28

Appendix 3	Additional information provided by the applicant (e.g. detailed modelling data)	29

A.3.1 	Drinking water criterion	29

A.3.2 	Monitoring data surface water	29

# Fate and behaviour in the environment (KCP 9)

This document supports the application for authorisation according to Article 33 of the Regulation (EC) No 1107/2009 and reviews the environmental fate studies and modelling of the tablet (TB) formulation AF 777, containing 2.0% of 1-methylcyclopropene (1-MCP) to be used as a post-harvest plant growth regulator on stored potatoes.

AF-777 is already registered in several EU countries under the name of SmartFreshTM ProTabs on different crops such as apples, pears, plums, tomatoes, kiwifruits, persimmons...to maintain crop quality during storage. The treatment in apples is considered as the worst case scenario with a maximum of three applications at 1000 ppb (2.24 mg active ingredient/m3) of 1-MCP with minimum interval of 24 h between applications.

The new intended use on potato with one application at a maximum use rate of 936 ppb (2.10 mg a.i/m3) is covered by the authorized use on apples, therefore the risk envelope approach is applicable.

AF-777 is the product code which is used in all sections of the core assessment. The commercial product name for the intended use on potato will be defined at a later stage.

Text is highlighted in yellow, when new information are included, not submitted as part of the renewal dossier of the product AF-777.

1-MCP was included into Annex I of Directive 91/414 (Regulation 2006/19/EC) and is renewed in the AIR procedure (Reg. (EU) 2019/1085).

A full risk assessment according to Uniform Principles is provided which demonstrates that the product is safe for the environment.

Where appropriate this document refers to the conclusions of the EU review of the 1-Methylcylcopropene. This will be where:

- the active substance data is relied upon in the risk assessment of  the formulation

- the EU review concluded that additional data/information should be considered at national re-registration.

Note: this Part B document only reviews data (Annex II or Annex III) and additional information that has not previously been considered within the EU review process, as part of the Annex I inclusion decision. New annex II data must only be included if they are considered essential for the evaluation and in this case a full study summary must be provided. In the case where the formulation has been previously evaluated, at European level, detailed summaries have not been provided.

Introduction

Concentrations of AF-777 in various environmental compartments are predicted following the proposed use pattern. The predicted environmental concentrations (PEC values) in soil, surface water, sediment, groundwater and air are provided. The long-term concentrations are based on results obtained for the active substance contained in the formulation.

The critical GAP for AF-777 is based on three applications at 1 ppm, it’s a worst case scenario used for all model calculations and risk assessments. This worst case scenario covers the intended use in potatoes.

Full details of all the proposed uses pattern that will be assessed is summarised in Table 8.1-1.

The impact of formulants is limited to short-term effects such as formation of stable spray dispersions or to facilitate uptake by target organisms, while their influence on long-term processes, such as degradation and distribution is negligible. Therefore, for the purposes of this risk assessment it is assumed that formulants do not influence the fate and behaviour of the active substance in the environment and are not considered further.

Properties considered relevant in assessing the fate of 1-MCP are shown in Table 8-1.

Table 8-1:	Agreed EU Physical chemical properties used in the Evaluation for 1-MCP (2018)

| Property | 1-MCP | Metabolites | Source |
| --- | --- | --- | --- |
| Molar mass [g/mol] | 54 | n.a. | (1), (2) |
| Molecular formula | C4H6 | n.a. | (1), (2) |
| Solubility in water (at 20 °C) [mg/L] | pH 7: 137 mg/L | n.a. | (1), (2) |
| Vapour pressure (at 25 °C) [Pa] | 2x105 Pa (calculation) | n.a. | (1), (2) |
| log POW (at 26 °C) (n-Octanol/water partition coefficient) | pH ≈ 7: 2.4 | n.a. | (1), (2) |
| Henry’s Law Constant (at 25C) [Pa m3/mol] | Not required for a gas at room temperature | n.a. | (1), (2) |
| Photolytic stability | Not required | n.a. | (1), (2) |
| Hydrolytic stability | At 50 °C at pH 4-9, 1-MCP is unstable in water (>70% degradation in 2.4 hours). In a ready biodegradability study (20-22 °C), the compound was stable for up to 28 days (in both sterile control samples and in the presence of viable sewage sludge inoculum). Stable to hydrolysis | n.a. | (1), (2) |
| n.a. = no data available and not required; (1) SANCO/ 2005/1094 - rev. 6; (2) EFSA conclusion of the AIR process (EFSA 2018;16(7):5308); RAR 2018; Final Renewal Report 2018 (SANTE/11631/2018 Rev 5) |  |  |  |

This product was not the representative formulation in the Annex I renewal, however, AF-777 has been already evaluated in several European Member States according to Uniform Principles (see table 8-2):

Table 8-2:	List of the countries, where the identical SmartFreshTM ProTabs is currently registered

| Product code | Product name(s) | MS | Authorization No. | Date of initial registration | Date of the last re-registration |
| --- | --- | --- | --- | --- | --- |
| AF-777 | SmartFreshTM ProTabs | Austria | 3651 | 10.12.2015 | --- |
| AF-777 | SmartFreshTM ProTabs | Belgium | 10414P/B | 09.03.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Bulgaria | 01621-ПРЗ-1/13.09.2018 г. | 13.09.2018 | --- |
| AF-777 | SmartFreshTM ProTabs | Croatia | UP/I-320-20/15-01/344 | 12.04.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Czech Republic | 5399-0 | 02.06.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | France | 2150057 | 29.06.2015 | --- |
| AF-777 | SmartFreshTM ProTabs | Germany | 008368-00 | 30.03.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Greece | 8253/31-5-2016 | 31.05.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Hungary | 04.2/1181-3/2017 | 30.06.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Ireland | 05779 | 05.02.2018 | --- |
| AF-777 | SmartFreshTM ProTabs | Italy | 16244 | 09.07.2015 | --- |
| AF-777 | SmartFreshTM ProTabs | Luxembourg | L02191-181 | 08.03.2018 | --- |
| AF-777 | SmartFreshTM ProTabs | Netherlands | 14648N | 31.10.2014 | --- |
| AF-777 | SmartFreshTM ProTabs | Poland | R-27/2016 | 04.11.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Portugal | 00962 | 27.06.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Romania | 380PC | 29.11.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Slovakia | 16-14-1840 | 20.10.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Slovenia | U34330-102/2016/3 | 26.10.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Spain | ES-00179 | 30.03.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Switzerland | W7067 (ex. W6975) | 21.10.2014 | --- |
| AF-777 | SmartFreshTM ProTabs | UK | 16546 | 08.07.2014 | --- |

The Renewal assessment report (RAR, 2018), the renewal report for 1-Methylcylcopropene (SANTE/11631/2018 Rev 5) and the Peer Review Report (EFSA 2018;16(7):5308) are considered to provide the relevant review information or a reference to where such information can be found. The relevant EU agreed endpoints to be used in this evaluation are included in the respective Annex points of this section.

On the basis of the proposed and supported uses, no particular issues have been identified as requiring short term attention from the Member States.

Information on the detailed composition of the product can be found in the confidential dossier of this submission (Registration Report - Part C).

## Critical GAP and overall conclusions

Table 8.1-1	Critical use pattern of the formulated product

| 1 | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 |  | 9 | 10 |  | 11 |  | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop)** | F, Fn, Fpn G, Gn, Gpn or I *** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  |  |  | Application rate |  |  |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha | Conclusion |
|  |  |  |  |  | Method / Kind |  | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season |  | Min. interval between applications (days) | g product/m3 a) max. rate per appl. b) max. total rate per crop/season |  | mg as/m3 a) max. rate per appl. b) max. total rate per crop/season |  | Water L/ha min/max |  |  | Groundwater |
| Interzonal uses (use as seed treatment, in greenhouses (or other closed places of plant production), as post-harvest treatment or for treatment of empty storage rooms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 **** | Interzonal | Apple [MABSD] | I | N/A (post-harvest plant growth regulator) | Proprietary kit (1) | Preferably as soon as possible after harvest BBCH 99 |  | 3 | 24 hours |  |  | a) 0.112 b) 0.336 |  | a) 1000 ppb [2.24 mg/m3] b) 3000 ppb [ 6.72 mg/m3] | n.r | n.r | See Nota Bene |  |
| 1 | Interzonal | Potato [SOLTU] | I | N/A (post-harvest plant growth regulator) | Proprietary kit (1) | After harvest and before ethylene application BBCH 99 |  | 1 | n.r |  |  | a) 0.105 |  | a) 936 ppb [2.10 mg/m3] | n.r. | n.r | See Nota Bene | A |

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	Use also code numbers according to Annex I of Regulation (EU) No 396/2005

*** 	I: indoor application

**** 	Use in apples represents the worst case scenario

(1)	Yellow or pink tablets of AF-777 have to be applied in combination with the AF-777 ProActivator Tablets and the AF-777 Activator Solution

N.B The relationship between 1-MCP in ppb vs. mg is expressed by the law of gases, thus following the formula:

Milligrams 1-MCP/cubic meter = selected concentration of 1-MCP in the air in ppm x molecular weight 1-MCP (=54 g/mol)/volume occupied by 1 mole of gas at 20°C (24.06 liters).

e.g. A use rate of 936 ppb 1-MCP is equivalent to 0.936*54/24.06 = 2.10 mg/m3

Storage room filling density considered is 250 kg/m3 for apples and between 350 - 500 kg/m3 for potatoes.

Explanation for column 15 “Conclusion”

| A | Safe use |
| --- | --- |
| R | Further refinement and/or risk mitigation measures required |
| C | To be confirmed by cMS |
| N | No safe use |

Table 8.1-2:	Assessed (critical) uses during approval of 1-MCP concerning the Section Environmental Fate

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. * | Member state(s) | Crop and/or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I ** | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/ synergist per ha |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | kg product/m3 a) max. rate per appl. b) max. total rate per crop/season | mg as/m3 a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min/max |  |  |
| --- | EU | Apples [MABSD] | I | N/A (plant growth regulator) | Proprietary kit*** | Preferably as soon as possible after harvest BBCH 99 | 1 | n.r. | 0.112 | 2.24 | n.r. | n.r. |  |

* 	Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1

** 	F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application

***	Yellow or pink tablets of AF-777 (SmartFresh™ ProTabs) have to be applied in combination with SmartFresh™ ProActivators and SmartFresh™ ProPack

## Metabolites considered in the assessment

As exposure of soil, groundwater, surface water and air to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible. Any metabolites formed will also be of a small fragment size. Therefore, any metabolites formed are considered not relevant.

This was confirmed during the Annex I listing process and the AIR process by EFSA (EFSA 2005; 30, 1-46 and EFSA 2018; 16(7):5308) that contamination of natural soils and thus a subsequent leaching to groundwater due to the proposed use of 1-MCP may be precluded. This is explained because the use of 1-MCP will be restricted to indoor use.

## Rate of degradation in soil (KCP 9.1.1)

1-MCP is to be used indoors as a post-harvest treatment on potato. The use in apples is considered as worst case scenario with a maximum application rate of 3 times 1000 ppb of 1-MCP at minimum intervals of 24 h each. Thus, the use in potato with an application rate of 1 time 936 ppb of 1-MCP is covered by this worst case scenario. The product is to be used within a sealed storage room in a controlled or normal atmosphere and thus there will be no exposure of the environment during use. Exposure will only occur once treatment rooms are vented after the treatment process. Exposure will primarily be to air, will be low and the compound will dissipate rapidly in air via photo-oxidative degradation (calculated DT50 from reaction with OH radicals - 4.4 hours calculated DT50 from reaction with OH radicals - 4.4 hours and estimated using the Atkinson model, with a half-life of 1.474 h) and diffusion.

It is confirmed during the Annex I listing process and the AIR process by EFSA (EFSA 2005; 30, 1-46 and EFSA 2018;16(7):5308) that contamination of natural soils due to the proposed use of 1-MCP may be precluded. This is explained because the use of 1-MCP will be restricted to indoor use in post-harvest storage. Therefore, studies to investigate the route of degradation in soil are not required.

As exposure of soil to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible.  Any metabolites formed will also be of a small fragment size.

Studies to investigate the rate of degradation in soil are not required.

QSARs (EPIWIN Suite 3.04 package) estimates of soil half-life were provided by the notifier in the context of the ground water risk assessment. The models employed to produce these estimates have not been peer reviewed at the EU level and the results should be taken as indicative. In general, QSAR estimates can be considered to have an approximate 95 % confidence interval of ± 10 fold. Therefore, these estimates would not be appropriate to perform the risk assessment of uses were soil contamination by 1-MCP was possible.

Estimates of the initial PEC in soil were produced based on a model that considers the limited exposure via ventilation of the CAF and subsequent deposition from the released air. The model has not been Peer Reviewed at EU level since EU models to calculate soil contamination through volatilization / deposition processes are expected to be provided by FOCUS air. However, it is considered that assumptions made in the calculation are conservative. The values obtained may be considered illustrative to confirm that no further data are needed to support the proposed uses.

### Aerobic degradation in soil (KCP 9.1.1.1)

Studies on aerobic degradation in soil with the formulation were not performed, as exposure of soil to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible.

### Anaerobic degradation in soil (KCP 9.1.1.1)

Studies on anaerobic degradation in soil with the formulation were not performed, as exposure of soil to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible. Please refer to chapter 8.3.

## Field studies (KCP 9.1.1.2)

Field studies with the formulation were not performed, as exposure of soil to the parent compound is negligible. Please refer to chapter 8.3.

### Soil dissipation testing on a range of representative soils (KCP 9.1.1.2.1)

Studies on field dissipation rates with the formulation were not performed, as exposure of soil to the parent compound is negligible. Please refer to chapter 8.3.

### Soil accumulation testing (KCP 9.1.1.2.2)

Soil accumulation testing is not required, as exposure of soil to the parent compound is negligible. This is confirmed in the AIR evaluation (see RAR 2018; section CP-B.8.1.1.2.2.). Please refer to point 8.3 above.

## Mobility in soil (KCP 9.1.2)

Use of 1-MCP will be restricted to indoor use in post-harvest storage. Therefore, contamination of natural soils due to the proposed use of 1-MCP may be precluded.

QSARs (EPIWIN Suite 3.04 package) estimates of adsorption coefficients to soil were provided by the notifier in the context of the ground water risk assessment. The models employed to produce this estimates have not been peer reviewed at EU level and the results should be taken as indicative. In general, QSAR estimates can be considered to have an approximate 95 % confidence interval of ± 10 fold. Therefore, these estimates would not be appropriate to perform the risk assessment of uses were soil contamination by 1-MCP was possible.

### Column leaching (KCP 9.1.2.1)

During the Annex I inclusion no data were submitted. Data are not required as worst case soil exposure is predicted to be very low (see DAR, section B.8.3). Please refer to chapter 8.3.

### Lysimeter studies (KCP 9.1.2.2)

During the Annex I inclusion no data were submitted. Lysimeter studies are not required as worst case soil exposure is predicted to be very low (see DAR, section B.8.3). Please refer to chapter 8.3.

### Field leaching studies (KCP 9.1.2.3)

During the Annex I inclusion no data were submitted. Field leaching studies are not required as worst case soil exposure is predicted to be very low (see DAR, section B.8.3). Please refer to chapter 8.3.

## Degradation in the water/sediment systems (KCP 9.2, KCP 9.2.1, KCP 9.2.2, KCP 9.2.3)

During the Annex I listing process EFSA (EFSA Scientific Report (2005) 30, 1-46) confirmed that contamination of natural surface water due to the proposed use of 1-MCP may be precluded since the use of 1-MCP will be restricted to indoor use in post-harvest storage. Therefore, water/sediment studies are not required. This is confirmed in the AIR evaluation (see RAR 2018; section CP-B8).

## Predicted Environmental Concentrations in soil (PECsoil) (KCP 9.1.3)

### Justification for new endpoints

AF-777 is a plant growth regulator with authorized uses in apple, pear, plum, tomato, kiwifruit, persimmon, peach, nectarine, apricot, pomegranate and melon. This application is for a new product registration with use in potatoes. The worst-case conditions (i.e. the highest application rates and lowest interception rates) were chosen for apples from the GAP (see table 8.1-1) to cover all uses (risk envelope approach) with respect to exposure assessment.

Product studies on degradation in soil with the formulation were not performed, since it is possible to extrapolate from data obtained with the active substance.

It is confirmed during the Annex I listing process and the AIR process by EFSA (EFSA 2005; 30, 1-46 and EFSA 2018; 16(6):5308) that contamination of natural soils due to the proposed use of 1-MCP may be precluded. This is explained due to the fact that the use of 1-MCP will be restricted to indoor use.

Thus there will be negligible exposure of the environment. Exposure will primarily be to air, will be low and the compound will dissipate rapidly in air via photo-oxidative degradation (calculated DT50 from reaction with OH radicals - 4.4 hours and estimated using the Atkinson model, with a half-life of 1.474 h) and diffusion.

Therefore, studies to investigate the route of degradation in soil are not required as the soil exposure is expected to be very low and conservative default values were used in the exposure assessment in the RAR. This was accepted in the Annex I inclusion process and confirmed in the AIR procedure for the active substance.

However, it is considered that assumptions made in the calculation for the PECsoil values are conservative (please refer to DAR, B.8.3). The values obtained may be considered illustrative to confirm that no further data are needed to support the proposed uses.

For the renewal of 1-MCP, the calculations were performed with using an airborne concentration of 1 ppm. In addition the RMS has used the approach of that considered exposure to soil from both venting and sewage treatment plants, following the processing of wastewater that may contain 1-MCP residues from the washing of treated apples. In the absence of measured values and the uncertainty of the QSAR estimates highlighted in the EFSA conclusion (2005) the conservative default values from FOCUS (2006) have been used for the calculation of PEC values plus the sewage sludge risk in the RAR 2018. This was also accepted by EFSA (EFSA conclusion 2018).

This approach was also considered for the indoor post-harvest use of AF-777.

The following table provides the EU endpoints to be used for the PEC soil calculations.

EU End-points for 1-MCP

| End-Point | 1-MCP |
| --- | --- |
|  | EU agreed endpoints [EFSA 2005; 30, 1-46 and EFSA 2018;16(7):5308] |
| Worst-case DT50 [days] | DT50lab (20°C, aerobic): no measured data available. Soil contamination precluded for the uses proposed. QSAR estimate of 15 days (first order). Only illustrative since a 10 fold uncertainty is estimated. DT90lab (20°C, aerobic): no measured data available. QSAR estimate of 50 days. Only illustrative since a 10 fold uncertainty is estimated. Soil contamination precluded for the uses proposed. DT50lab (10°C, aerobic): 33 days, calculated using a Q10 of 2.2 from a QSAR estimate. Only illustrative since a 10 fold uncertainty is estimated. DT50lab (20°C, anaerobic): No data submitted, not required. |

Short-term PECs values are covered by initial PECs values as these take into account three applications within 3 days as a conservative assumption. For the use in potato only one application is proposed and hence this use is covered by the authorized use in apples.

Long-term PECs values need not to be taken into account, as under the proposed use scenario, the release of 1-methylcyclopropene from controlled atmosphere facilities (CAF) is infrequent, and of negligible magnitude. Subsequently, deposition and accumulation in soil are anticipated to be similarly negligible. In addition, the DT50 of 1-MCP in air is 4.4 hours (or estimated using the Atkinson model, of 1.474 h) indicating that long-term exposure to soil organism is unlikely.

### Active substance(s) and relevant metabolite(s)

Environmental concentrations in soil are predicted as a first approach to estimate the exposure of soil macro- and micro-organisms.  The initial predicted environmental concentrations of 1-MCP in soil (PECs) were calculated using Tier I and Tier II approaches. The basic calculation can be found in the DAR (please refer to section B.8.3).

In the AIR procedure, the RMS considers the approach from the original DAR, 2005 (section B.8.3) to still be valid. In addition to the original calculation done in the DAR for the Annex I inclusion, in the RAR for the AIR process of 1-MCP, several conservative assumptions were estimated (see section CP B8). However, the RMS confirmed that the risk to soil form use of 1-MCP has been adequately addressed by building upon the approach used in the DAR (2005). The following calculations consider the new calculations done in the RAR for the Annex I renewal of 1-MCP.

In addition to the deposition calculation via air, soil can be exposed to 1-MCP via application of sewage sludge from a sewage treatment plant following washing of apples by the end consumer or as part of the industrial process and this route of exposure is considered here.

The routes of exposure to soil assessed can be summarised as follows:

1a. 	Deposition from air to soil from venting of the storage facility –Tier I

1b. 	Deposition from air to soil from venting of the storage facility –Tier II

2. 	Application of sewage sludge from apple washing (domestic & industrial) - Tier I

PECsoil values are presented for each scenario and then a combined PECsoil value that considers simultaneous contributions from either scenario 1a or 1b together with scenario 2 are presented

Route 1a. Tier I - Air to Soil Deposition

The Tier I analysis for one application with 1 ppm was evaluated and accepted during the Annex I inclusion of 1-MCP (please refer to DAR section B.8.3). The Tier I analysis for the EU review was based on the assumptions that the maximum concentration of 1-MCP of 1 ppm results in a 0.004 kg release of 1-MCP from a single CAF of approximately 1800 m3 (Shade 2001). For the AIR procedure (RAR, 2018), the RMS has made a correction and it is assumed that the total amount of generated 1-MCP from a storage room is deposited on an area of 1 ha. Thus, for a storage room of 1824 m3, it results in 4.395 g 1-MCP, deposited on an area of 1 ha. In addition, the worst-case assumption was made that no degradation in soil occurred between the deposition events and that there is no interception by plants.

The resulting estimated soil core predicted environmental concentration (PEC) for one application equals 0.0059 mg/kg (5.86 μg/kg). In order to cover the worst-case conditions for the product AF-777 (i.e. the highest application rates and lowest interception rates) for all uses in all zones (risk envelope approach) 3 applications with 1000 ppb were used for the following calculations. For the use in potato one application with 936 ppb is intended and thus, it is covered by the worst-case conditions mentioned above.

Taking into account 3 treatments, the predicted initial environmental concentration (PEC) equals 17.56 µg/kg (which is equivalent to 13.185 g/ha). The worst-case assumption was made that no degradation in soil occurred between the deposition events and that the total amount of generated 1-MCP from a 1800 m3 CAF is deposited on an area of 1 ha. In addition, no interception by plants has been assumed.

Route 1b. Tier II - Deposition from air to soil from venting of the storage facility

Tier II - Air to Soil Wet Deposition

A Tier II analysis was performed to refine the previous exposure estimates by accounting for wet (precipitation scouring) and dry deposition of 1-MCP to soil.  The calculated deposition values were based on the appropriate environmental characteristics of 1-MCP including Henry’s Law constant.  This approach contrasts with that taken for the Tier I analysis in which 100% deposition of the released 1-MCP to the soil was assumed.  The calculation of the wet and dry deposition was after the methods presented in Thibodeaux, 1996 and was evaluated during the Annex I inclusion of 1-methylcyclopropene. The whole calculation can be found in the DAR (please refer to section B.8.3).

Based on the information it has been estimated that the cumulative soil core concentration of 1-MCP resulting from wet deposition of 1-MCP released from a single CAF equals 3.59 x 10-4 μg/kg soil, assuming a 1 hectare deposition area. Taking into account three releases, the predicted environmental concentration of 1-MCP is 1.08 x 10-3 µg/kg. The conservative worst-case assumption was made that no degradation in soil occurred between the deposition events. Deposition areas greater than 1 hectare would result in lower accumulated soil core concentrations.  Rainfall events of less than that assumed for the purposes of the calculation would also result in appreciably lower soil core concentrations of 1-MCP.  These calculations are conservative in that potential soil degradation of 1-MCP was not accounted for.

Tier II Air to Soil Dry Deposition

Deposition of 1-MCP to the soil via dry deposition mechanisms has also been evaluated after the methods found in Thibodeaux (1996)1 during the Annex I inclusion of 1-MCP.

The cumulative soil core concentration of 1-MCP resulting from dry deposition of 1-MCP released from a single CAF and based on one single treatment would approximate 0.140 μg/kg soil, assuming a 1 hectare deposition area. Taking into account three treatments, the predicted environmental concentration (PEC) equals 0.419 µg/kg. This assumption was made that no degradation in soil occurred between the deposition events.

Deposition areas greater than 1 hectare would result in significantly lower accumulated soil core concentrations.  Because the deposition velocity of 0.05 cm/s selected for the calculation is conservative, deposition velocities of less than 0.05 cm/s would result in appreciably lower soil core concentrations of 1-MCP. It was also assumed that the atmospheric concentration of 1-MCP during each of the air exchanges equalled the maximum one hour average for each air exchange.  In practice this steady state maximum would not occur.  In addition, the inclusion of soil core degradation of 1-MCP in the analysis would appreciably lower predicted soil core concentrations. Taken in total these assumptions ensure the conservatism of the exposure estimate.

Tier II Air to Soil - wet and dry deposition

Accounting for both wet and dry deposition of 1-MCP following the release from a single CAF and based on one single treatment the total cumulative maximum 1-MCP soil core concentration equals 0.1399 μg/kg. Taking into account three treatments, the predicted environmental concentration (PEC) equals 0.4197 µg/kg (which is equivalent to 0.315 g/ha). The predominant deposition mechanism appears to be through dry deposition rather than through rainfall scouring. However, this estimate is likely to be highly conservative.  Deposition velocities of molecules such as 1-MCP are lacking.  The deposition velocity selected for 1-MCP was specifically chosen for its conservatism and is likely to be significantly lower.

The values obtained may be considered illustrative to confirm that no further data are needed to support the proposed uses. The PECsoil value from the Tier II approach was used in the risk assessment for soil organisms (see section 9 of the dRR CP 9.8).

Route 2. Tier I - Application of sewage sludge after exposure of the sewage treatment plant from apple washing (domestic & industrial)

In addition to exposure of 1-MCP to soil from venting, in the AIR procedure of 1-MCP, the RMS has considered exposure to soil following sewage treatment plant processing of 1-MCP wastewater from washing apples as worst case with 3 applications.

The RAR calculation is based on the assumption that the whole MRL for 1-MCP (0.01 mg/kg) was washed away from the apples and that for the industrial washing of apples, 500 tonnes of apples are washed in a single day.

As this is an unrealistic worst case scenario, this calculation also covers the intended use of AF-777 in potato. The detailed assessment for application of sewage sludge can be found in section CP B8.7 of the RAR.

At Tier I, the sewage treatment plant continuously produces sludge containing 6.33 x 10-2 mg 1-MCP per kg (section B.8.7 Table B.8.20) therefore at Tier I, the application rate (AR) of 1-MCP contained in the sludge would be:

AR1-MCP, sludge– Tier I = (5000 kgdry sludge/ha) x (6.33 x 10-2 mg1-MCP/kgdry sludge) / 1000 = 0.3165 g/ha

Predicted Environmental Concentration in soil (PECsoil)

For Tier I a default DT50 value of 1000 days as applied since no DT50 was available for 1-MCP in soil. The maximum PECsoil value resulting from an application rate of 4.395 g/ha.

The Tier II calculation considers for the estimated PECsoil value a conservative DT50 of 1-MCP in soil of 1000 days (as done in the RAR 2018), and the same assumptions for Tier II as for Tier I (but with the Tier II application rate. The total soil exposure at Tier I and Tier II was determined as follows:

Table 8.7-3:	Predicted Environmental Concentration in soil (PECsoil) - Tier I and Tier II

| Exposure scenario |  | Exposure [g/ha] | PEC [mg/kg soil] |
| --- | --- | --- | --- |
| Tier I |  |  |  |
| 1a. | Air-to-soil deposition at Tier I | 13.185 | 0.01756 |
| 2. | Application of sewage sludge from a sewage treatment plant at Tier I | 0.317 | 0.000423 |
|  | Total soil exposure at Tier I | 13.494 | 0.01799 |
| Tier II |  |  |  |
| 1b. | Air-to-soil deposition at Tier II (wet and dry) | 0.315 | 0.0004 |
| 2. | Application of sewage sludge from a sewage treatment plant at Tier I | 0.317 | 0.000423 |
|  | Total soil exposure at Tier II | 0.63178 | 0.000843 |

The values obtained may be considered illustrative to confirm that no further data are needed to support the proposed use in potato. The PECsoil value from the Tier II approach (0.000843 mg/kg soil) was used in the risk assessment for soil organisms (see section 9 of the dRR CP 9.8).

PECsoil of metabolites

As exposure of soil to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible. Therefore, no metabolites are considered potentially relevant and accordingly estimates of concentrations have not been presented.

#### PECsoil of AF-777

Studies on aerobic degradation in soil with the formulation were not performed, as exposure of soil to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible.

zRMS comments:

The submitted calculations were accepted as Tier I and  Tier II approach accepted.

Taking into consideration not typical formulation use in permanent structure, zRMS is in the opinion, that the EU approach was used in soil exposure assessment.

The ways of soil exposure were considered: via atmosphere and deposition on soil surface and

via sewage sludge storage on soil surface (after washing).

The application rate recalculated into g a.s./ha was agreed at the EU level.

Tier I PECs =  0.01799 mg/kg

Tier II PECs = 0.000843 mg/kg

These values will be used in further risk assessment.

## Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)

According to the most recent EFSA Guidance document on emissions for active substances from protected crops (greenhouses etc.; EFSA Journal 2014; 12(3):3615) to relevant compartments, it is not necessary to perform risk assessment for the leaching to groundwater for soil less cultivation systems which can be regarded as an appropriate surrogate for the indoor post-harvest use of AF-777. Assessments for leaching behaviour are not relevant. As 1-Methylcyclopropene is not regarded as persistent in soil, no assessment is necessary and no PECGW calculation needs to be conducted. Therefore, no appropriate emission routes are proposed in this EFSA guidance document which would fit properly to the representative AF-777 uses.

Nevertheless and for sake of completeness, a single treatment with 3x 1 ppm is assumed for PECGW calculations considering a possible deposition of emissions from the storage room by means of FOCUS PELMO and PEARL estimates with the same input parameters as for the EU renewal procedure.

1-MCP is to be used indoors as a post-harvest treatment on potatoes with a maximum application of 1 times 936 ppb of 1-MCP.

The use on apples with 3 x 1 ppm is the worst case and covers also the use on potato.

Due to the indoor use, there will be negligible exposure of the environment. As demonstrated in section 8.7.2, in which a number of conservative assumptions are made, exposure of soil will be negligible. Subsequent significant exposure of soil from the formation of metabolites will also not be plausible. Thus, the risk of contamination of groundwater by 1-Methylcyclopropene or any subsequent metabolites will be extremely low. This was confirmed during the Annex I listing process and the Annex I renewal by EFSA (EFSA 2005; 30, 1-46 and EFSA 2018;16(7):5308) that contamination of natural soils and thus a subsequent leaching to groundwater due to the proposed use of 1-Methylcyclopropene may be precluded. This is explained because the use of 1-MCP will be restricted to indoor use. Therefore, no PECGW simulations are needed for the use intended, and thus no further data were required in the EU review.

Further information about the drinking water criterion can be found in Appendix 3.

However, as a worst case scenario the calculation using the FOCUS PELMO 5.5.3 and FOCUS PEARL 4.4.4 are made with the maximum rate of 3 applications of 1000 ppb. The product is to be used within a sealed storage room in a controlled or normal atmosphere and thus there will be no exposure of the environment during use. This covers also the use in potatoes.

### Justification for new endpoints

No new endpoints are proposed.

### Active substance(s) and relevant metabolite(s) (KCP 9.2.4.1)

A FOCUS PELMO and FOCUS PEARL modelling has been performed using best modelling practice, using worst-case assumptions for the deposition from emissions from the storage room. The values obtained may be considered illustrative to confirm that no further data are needed to support the proposed use.

Both models FOCUS PELMO 5.5.3 and FOCUS PEARL 4.4.4 were used to estimate the potential groundwater concentrations of 1-Methylcyclopropene resulting from the release of the chemical from a single storage room (CAF). The modelling encompasses CAF releases of 1-Methylcyclopropene occurring each year for 26 consecutive years. Modelling rationale, including default inputs, were consistent with recommendations made by FOCUS (2000) and FOCUS (2009) for conducting simulation calculations for the registration procedure.

With respect to the PECGW calculations these were done according to the PECgw calculations performed by EFSA (EFSA Journal 2018;16(7):5308).

For these calculations the deposition of 1-MCP from air to soil from the venting of a storage facility was considered under worst case conditions in a Tier I approach. It was assumed that the total amount of the maximum use rate of 3x 1 ppm of 1-MCP from a storage room of 1824 m³ is entirely deposited onto a field area of 1 ha close to the storage facility (100 % air to soil transfer coefficient).

The deposition rate used for this groundwater assessment was calculated based on the calculations found in the original DAR (2005, section B.8.7 a) and presented in the RAR (2018). In the DAR, 1-MCP was handled as perfect gas with a standard atmospheric pressure and a storage temperature of 0 °C was taken into account. At that low temperature, the volume occupied by one 1 mole of 1-MCP would be:

V = n x R x T = 1 x 8.314 x 273.15 = 0.02241 m3

p 					101325

with:

V: volume occupied by 1-MCP (m3)

n: amount of 1-MCP in mol

R: gas constant (8.314 J K−1 mol-1)

T: temperature (K)

p: pressure (Pa)

At 1 ppm v/v, which is the maximum concentration of 1-MCP in the storage room, the theoretical total volume of pure 1-MCP for one application would be:

Vpure1MCP = 1824 x 1 x 10-6 = 1.824x 10-3 m3

The total amount of 1-MCP contained in the storage room with 1 application is therefore:

n1MCP = 1.824 x 10-3 = 0.08139 mol

0.02241

The total mass of 1-MCP in the warehouse with 1 application would be:

m1MCP = 0.08139 x 54 = 4.395 g

Thus, for a storage room of 1824 m3 and a maximum use rate of 3 x 1 ppm an application rate of 3 x 4.395 g = 13.185g  1-MCP, deposited evenly onto an area of 1 ha field, was assumed for the PECGW calculations. This approach covers the use in potatoes, too.

In addition, the worst-case assumption was made that no degradation and adsorption in soil occurred between the deposition events and that there is no interception by plants. The FOCUS crop scenario “winter wheat” was selected since this scenario comprises all nine available FOCUS groundwater scenarios which represent realistic worst-case agricultural areas in European and is in compliance with the crop scenario used by EFSA. The input parameter and application dates used for the PECGW calculations are presented below and were according to the PECGW calculations performed by EFSA.

Table 8.8.2-1:	Input parameters related to application for PECgw calculations

| Crop | Winter wheat (deposition from storage room) |
| --- | --- |
| Application rate (g as/ha) | 4.395 x 3 =13.185 |
| Number of applications | 3, calculated as 1 |
| Relative application date | Post-harvest |
| Crop interception (%) | 0 |
| Frequency of application | Annual |
| Models used for calculation | FOCUS PEARL v4.4.4, FOCUS PELMO v5.5.3 |

Table 8.8.2-2:	Application dates used for groundwater risk assessment

| Crop | Scenario | Application dates (absolute) |
| --- | --- | --- |
| Winter wheat | Châteaudun | 08.11. |
|  | Hamburg | 08.11. |
|  | Jokioinen | 08.11. |
|  | Kremsmünster | 08.11. |
|  | Okehampton | 08.11. |
|  | Piacenza | 08.11. |
|  | Porto | 08.11. |
|  | Sevilla | 08.11. |
|  | Thiva | 08.11. |

Table 8.8.2-3:	Input parameters related to 1-MCP for PECgw calculations

| Compound | 1-Methylcyclopropene | Value in accordance with EU endpoint [y/n]/ Reference |
| --- | --- | --- |
| Molecular weight (g/mol) | 54 | y / EFSA Journal 2018;16(7):5308 |
| Water solubility (mg/L) at 20 °C: at 25 °C: | pH 7: 137 pH 7: 164.98 | y / EFSA Journal 2018;16(7):5308 2nd temp. calculated by Van’t Hoff equation |
| Saturated vapour pressure (Pa) at 20 °C: at 25 °C: | 142000 200000 | y / EFSA Journal 2018;16(7):5308 y / EFSA Journal 2018;16(7):5308 |
| DT50 in soil (d) | 1000 | Default y / EFSA Journal 2018;16(7):5308 |
| Kfoc (mL/g)/Kfom | 0/0 | Default y / EFSA Journal 2018;16(7):5308 |
| 1/n | 1.0 | Default y / EFSA Journal 2018;16(7):5308 |
| Plant uptake factor | 0 | Default y / EFSA Journal 2018;16(7):5308 |
| Formation fraction | - | - |

Results are presented in the following Table 8.8.2-4. All relevant raw data to the additionally assessment of predicted environmental concentration in groundwater are provided in KCP 9.2.4.1/01.

Table 8.8.2-4:	PECgw for 1-MCP deposition from storage room (with FOCUS PEARL 4.4.4/ PELMO 5.5.3)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |  |
| --- | --- | --- | --- |
|  |  | 1-Methylcyclopropene |  |
|  |  | PELMO 5.5.3 | PEARL 4.4.4 |
| Winter wheat | Châteaudun | < 0.001 | 0.081 |
|  | Hamburg | < 0.001 | 0.004 |
|  | Jokioinen | < 0.001 | 0.024 |
|  | Kremsmünster | < 0.001 | 0.006 |
|  | Okehampton | < 0.001 | 0.023 |
|  | Piacenza | < 0.001 | 0.002 |
|  | Porto | < 0.001 | 0.008 |
|  | Sevilla | < 0.001 | < 0.001 |
|  | Thiva | < 0.001 | 0.027 |

The PECGW re-assessment, considering unrealistic worst case conditions, resulted in groundwater concentrations below the trigger of 0.1 µg/L for all scenarios. Therefore, the maximum use rate of 3x 1 ppm of 1-MCP in a storage room of 1824 m3 results in a safe use with respect to groundwater contamination.

zRMS comments:

The submitted calculations were accepted. In accordance with EU approach the winter wheat was used as a representative crop in PECgw assessment.

The endpoints for active substance were agreed at the EU level.

The PECgw assessment was done in accordance with FOCUS Groundwater guidance. Both FOCUS models: PEARL and PELMO were used.

The highest application rate was used in PECgw assessment.

The PECgw for active substance and its metabolite are below the trigger value of 0.1 µg/L

## Predicted Environmental Concentrations in surface water (PECsw) (KCP 9.2.5)

### Justification for new endpoints

No new endpoints are proposed.

### Active substance(s), relevant metabolite(s) and the formulation (KCP 9.2.5)

Estimates of the initial PEC in surface water were produced based on a model that considers the limited exposure via ventilation of the CAF and subsequent deposition from the released air (please see below).  The initial predicted environmental concentrations of 1-MCP in the aquatic environment (PECsw) were calculated using Tier I and Tier II approaches. It is considered by EFSA that assumptions made in the calculation for the PECsurface water values are conservative. The values obtained may be considered illustrative to confirm that no further data are needed to support the proposed uses. The RMS of the AIR procedure confirmed that concentrations in sediment were considered to be negligible.

In the AIR procedure the RMS has included an assessment of the exposure of 1-MCP to surface water and sediment via sewage treatment plants to provide a comprehensive assessment.

Several new routes of exposure of 1-MCP to surface water following washing of treated apples are considered. In addition new FOCUS surface water modelling using STEPs 1-2 has been included, which have been summarised into three main routes of exposure by the RMS with up to two tiers within each exposure route as shown in the following:

Table 8.9-4:	Summary of the possible routes of exposure used to predict PECSW and PECSED values

| Route | Description | Tiered assessment | Calculation |
| --- | --- | --- | --- |
| A | Venting to the atmosphere followed by deposition from the air onto soil – subsequent exposure of surface water from drainflow and run-off | Tier I - 1-MCP contained in a storage room of 1824 m3 was entirely deposited onto a 1-ha field Tier II - accounts for air to soil wet deposition and air to soil dry deposition surface water bodies. This approach contrasts with that taken for the Tier I analysis, where 100% deposition of the released 1-MCP to the field was assumed. | RAR (2018) refers to DAR (2005) |
| B | Venting to the atmosphere followed by deposition from the air directly onto surface water | Tier I - 1-MCP contained in a storage room of 1824 m3 was entirely deposited onto a 1-ha pond, without taking into account volatilisation. Tier II - accounts for wet (precipitation scouring) and dry deposition to surface water bodies. This approach contrasts with that taken for the Tier I analysis, where 100% deposition of the released 1-MCP to the surface water body was assumed. | RAR (2018) refers to DAR (2005) |
| C | Exposure via the sewage treatment plant from transfer of washing solutions from the end consumer or from industrial washing facilities. The active substance partitions between sludge, sewage effluent, and air. Indirect exposure of surface water occurs after sludge is applied to agricultural land and subsequently there is drainflow and / or run-off into a water body. Discharge of sewage effluent leads to direct exposure of surface water.* | The concentration of 1-MCP in the effluent water from a sewage treatment plant daily after industrial washing (500 T of apples) accounting for the domestic and the industrial source applying a dilution factor of 10. | RAR (2108) |
| *Deposition from the air is not considered as it is low compared to the scenario where venting leads to deposition from the air |  |  |  |

This approach was also considered for the indoor post-harvest use of AF-777.

PECSW for static water bodies represent the worst-case. PECSW values for slow moving water bodies would be lower than those for static systems and thus have not been calculated.

#### PECsw/sed of AF-777

AF-777 is a plant growth regulator which is used in apple, pear, plum, tomato, kiwifruit, persimmon, peach, nectarine, apricot, pomegranate and melon. The worst-case conditions (i.e. the highest application rates and lowest interception rates) were chosen for apples from the GAP (see table 8.1-1) to cover all uses (risk envelope approach) with respect to exposure assessment. This approach also covers the new use of AF-777 in potato.

Route A - Venting to the atmosphere followed by deposition from the air onto soil

For route A, at STEP 1, inputs of run-off, erosion and/or drainage are evaluated as a single loading to the water body and "worst-case" surface water and sediment concentrations are calculated. The loading to surface water is based upon the number of applications multiplied by the maximum single use rate unless 3 x DT50 in sediment/water systems (combined water + sediment) is less than the time between individual applications. In such a case the maximum individual application rate is used to derive the maximum PEC as there is no potential for accumulation in the sediment/water system. For first order kinetics the value of 3 x DT50 is comparable to the DT90 value and therefore is used as an estimate for the DT90.

At STEP 2, inputs of run-off, erosion and/or drainage are evaluated as a series of individual loadings comprising drift events (number, interval between applications and rates of application as defined in STEP 1) followed by a loading representing a run-off, erosion and/or drainage event, four days after the final application. This assumption is similar to that developed by the United States EPA in their GENEEC model (Parker, 1995). Degradation is assumed to follow first-order kinetics in soil, surface water and sediment and the registrant also has the option of using different degradation rates in surface water and sediment.

The in the RAR accepted input parameters used for the FOCUS model are shown in Table 8.9-1.

Table 8.9-5:	FOCUS input parameters for 1-MCP in FOCUS STEPs 1-2

| Parameter | Value | Source |
| --- | --- | --- |
| Physico-Chemical parameters |  |  |
| Molecular weight (g/mol) | 54 | RAR 2018, EFSA conclusion 2018 |
| Water solubility (mg/L) (20 °C, pH 7) | 137 | RAR 2018, EFSA conclusion 2018 |
| Degradation in soil |  |  |
| DT50 lab, norm. in soil (day) | 1000 | RAR 2018 (conservative default) |
| Sorption parameters |  |  |
| Soil Organic Carbon-Water Partitioning Coefficient: Kf,oc (mL/g) | 0 | RAR 2018 (worst case for PECSW) |
| Kf,oc (mL/g) | 1000* | RAR 2018 (conservative for PECSED) |
| Degradation in water |  |  |
| DT50 water/sediment system (day) | 1000 | RAR 2018 (conservative default) |
| DT50 water (day) | 1000 | RAR 2018 (conservative default) |
| DT50 sediment (day) | 1000 | RAR 2018 (conservative default) |
| * Conservative as QSAR estimate was 42.7 mL/g with 10 fold uncertainty (EFSA conclusion, 2005) therefore at its upper limit the Koc would be 427 mL/g which is lower than 1000 mL/g |  |  |

For the crop and use pattern parameters used in the RAR (2018), the pseudo application rate used was a sum contribution from venting and application of sewage sludge. For Tier I it was determined as 13.494 g/ha, for Tier II it was 0.632 g/ha (see table 7.7-2). For Tier I, the air-to-soil deposition was based on the assumptions that 1-MCP in the storeroom was entirely deposited onto a neighbouring 1-ha field after venting. For Tier II, the air-to-soil deposition was based on the assumptions that 1-MCP released from the storeroom was subject to both wet and dry deposition to surface water bodies. The region and season of application was “Oct-Feb, Northern Europe” in FOCUS STEPs 1-2. The application rate and crop interception values used in the PECSW and PECSED calculations are shown in Table B.8.13.

Table 8.9-6:	FOCUS input parameters for 1-MCP in FOCUS STEPs 1-2 (RAR 2018)

| Crop | Stage and/or BBCH | Crop Interception rate | Number of applications | Application rate (g/ha) |
| --- | --- | --- | --- | --- |
| ‘Incorporation / seed treatment’ * | not applicable | 0 % | 1 | Tier I: 13.494 Tier II: 0.632 |
| *Application is not targeted at the crop but incidental via contaminated sludge or air-to-soil deposition. |  |  |  |  |

As accepted in the RAR (2018), this is the worst-case regarding run-off/drainage input into surface water (according to table 2.4.3-1 in SANCO/4802/2001) with 5% of soil residues inputted into surface water via run-off/drainflow (FOCUS, 2001).

Results are presented in the following Table 8.9-4. All relevant raw data to the additionally assessment of predicted environmental concentration in groundwater are provided in KCP 9.2.5/01.

Table 8.9-7:	Summary of PECSW and PECSED outputs from STEPs 1-2

|  | Tier I |  | Tier II |  |
| --- | --- | --- | --- | --- |
|  | Step 1 | Step 2 | Step 1 | Step 2 |
| PECSW [μg/L] | 4.498 | 2.243 | 0.211 | 0.105 |
| PECSED [μg/kg] | 19.277 | 9.612 | 0.903 | 0.405 |

At tier I, Step 2 PECSW was 2.243 μg/L and PECSED was 9.612 μg/kg, whilst at tier II PECSW was 0.105 μg/L and PECSED was 0.405 μg/kg these results were then added to the PECSW and PECSED from the other routes that 1-MCP could reach surface water and sediment. These other routes are route B (Deposition from air directly to surface water) and route C (Exposure via sewage treatment plant).

Route B - Venting to the atmosphere followed by deposition from the air directly onto surface water

For route B, the estimation of PECSW due to air to water deposition at Tier I was based on the hypothesis that 1-MCP contained in a storage room of 1824 m3 was entirely deposited onto a 1-ha pond, without taking into account volatilisation. This was calculated in the DAR (2005) and confirmed with a small correction by the RAR (2018) for the Annex I renewal of the active substance 1-MCP. In the RAR a correction factor is introduced in order to take into account the possibility of a lower temperature in the storage room of 0 °C (see B.8.7.1 above). Thus the total amount of 1-MCP in the storeroom is 4.395 g (as described in the RAR). The modified calculated PECSW value in the RAR (2018) was 1.483 μg/L. A threefold application (3x 1 ppm) assuming no degradation in the 1 day between the applications results in a PECsw value of 4.449 μg/L.

For Tier I, the PECSED from this route of entry was estimated via the PECSED/ PECSW ratio observed using STEPs 1-2 calculation above (see table 8.9-4) and results in 4.29. This ratio was applied to the PECSW from Route B (air-to-water deposition, PECSW = 4.449 µg/L) and results in a PECSED of 19.086 µg/kg.

For Tier II- PECSW, all required data and calculations are available in the DAR (2005) and are still acceptable which was confirmed in the RAR (2018). The estimation of PECSW due to air to water deposition is based on the estimation of PECair. The Tier II results are 8.18 x10-5 μg/L for wet deposition and 6.22 x10-4 μg/L for the dry deposition for the application of 1 ppm. Thus, the contribution of air-to-water deposition to PECSW of 1-MCP at Tier II is 7.038 x10-4 μg/L for 1 ppm. A threefold application (3x 1 ppm) assuming no degradation in the 1 day between the applications results in a PECsw value of  2.111 x10-3 μg/L.

For Tier II the PECSED from this route of was estimated via the PECSED/ PECSW ratio observed in the STEPs 1-2 calculation above and results in 4.29. This ratio is applied to the PECSW from Route B (air-to-water deposition: PECSW = 2.111 x10-3 μg/L) and results in a PECSED of 9.056 x10-3 µg/kg.

Route C - Exposure via sewage treatment plant

For route C, the BPR (2015) assumes a 10 fold dilution for effluent entering surface water (BPR, 2015, page 77) and this has been applied by the RMS at tier I in the RAR 2018. No tier II assessment is presented in the RAR 2018.

For PECSW, the concentration of 1-MCP in the effluent water from a sewage treatment plant, resulting from the washing of apples by the end consumers and the industrial washing of 500 tonnes of apples daily, was 0.126 μg/L (see RAR 2018, Table B.8.20, section B.8.7). A dilution factor in the watercourse of 10 was applied leading to a PECsw value of 0.0126 μg/L.  As this is an unrealistic worst case scenario, this calculation also covers the intended use of AF-777 in potato. The detailed assessment for application of sewage sludge can be found in section CP B8.7 of the RAR.

The maximum PECSED in the solids effluent of a sewage treatment plant is 6.23 x 10-3 mg/kg or 6.23 μg/kg (see RAR 2018, Table B.8.20, section B.8.7). A dilution factor in the watercourse of 10 was applied leading to a PECSED value of 0.623 μg/kg.

Conclusions

The final PEC values combining the contributions of 1-MCP via routes A, B and C are shown in Table B.8.9-5 which present the worst case scenario. The use in potatoes is fully covered by these values.

Table 8.9-8:	Final PECSW and PECSED values from application of 1-MCP considering routes A, B and C

| Route of exposure | PECSW [μg/L] |  | PECSED (μg/kg) |  |
| --- | --- | --- | --- | --- |
|  | Tier I | Tier II | Tier I | Tier II |
| Route A | 2.243 | 0.105 | 9.612 | 0.405 |
| Route B | 4.449 | 2.111 x10-3 | 19.086 | 9.056 x10-3 |
| Route C | 0.0126 | 0.0126 | 0.623 | 0.623 |
| Total | 6.705 | 0.120 | 29.321 | 10.084 |

As outlined in the RAR, there were several conservative assumptions applied at tier II in the calculation of PECSW and PECSED including; no degradation of 1-MCP, no volatilization during transport in the sewage system, the complete loss of 1-MCP from the apples (apple is used as worst case scenario and covers the use in potato, too) during washing, a conservative amount of 1-MCP on the apples, a conservative domestic consumption model, a conservative air-to-soil and air-to-water deposition model, a conservative amount of water used to wash the apples, a KOC of 0 mL/g or 1,000 mL/g, the simultaneous events of sewage sludge application, venting and industrial washing.

The conclusion of the RAR 2018 was that there is a limited exposure of surface water to 1-MCP during use. Thus, the use in potatoes is fully covered.

Metabolite(s) of 1-MCP

As exposure of soil to the parent compound is negligible, subsequent significant exposure from metabolite formation will not be plausible. Therefore no metabolites are considered potentially relevant and accordingly estimates of concentrations have not been presented.

zRMS comments:

The submitted calculations were accepted.

For calculation/incorporation/seed treatment scenario was used.

The endpoints for active substance were agreed at the EU level.

These values will be used in further risk assessment.

## Fate and behaviour in air (KCP 9.3, KCP 9.3.1)

Currently there is no harmonised guidance to assess the risk to air from post-harvest treatments, and there has been no new guidance since the first Annex I listing.

Data on the fate and behaviour of 1-MCP in air was evaluated during the original EU review. No additional studies have been performed or are required for the original EU review and the Annex I renewal. The estimation of the PECair is given below. Table B.8.10-1 shows the endpoints that are relevant to the fate of 1-MCP in air.

Table 8.10-1:	Summary of atmospheric degradation and behaviour [Agreed EU Endpoints 
(RAR 2018 and SANCO/2005/1094 - rev. 6)]

| Compound | 1-MCP |
| --- | --- |
| Direct photolysis in air | Not studied – no data requested |
| Quantum yield of direct phototransformation | Not studied – no data requested |
| Photochemical oxidative degradation in air | Average rate of the reaction of 1-MCP with OH· was measured to be = 2.7 ± 1.3 x 10- 11 cm3/s. Assuming a typical 12-hour average concentration of OH· of 1.5 x 106 molecules/cm3, an atmospheric half-life of 4.4 hours is calculated (Rodriguez, J.M., 2001) DT50 of 1.47 hours derived by the Atkinson model (version AOPWIN v. 1.92). OH (12 h) concentration assumed = 1.56 x 106 / cm3 |
| Volatilisation | from plant surfaces: it is a gas from soil: not applicable |

1-MCP is a gas under environmental relevant conditions. Emission from a CAF through ventilation after the treatment processes may not be precluded. An estimation of the discharge from apple stores, which represents the worst case scenario, was conducted using the Industrial Source complex dispersion model (ISCST3). This model was used to estimate the maximum concentration of 1-MCP in air following venting of a controlled treatment facility. Using a number of conservative assumptions, the maximum predicted concentration of 1-MCP in air is approximately 1.1 ppb v/v (first 80 min of venting at approximately 50 to 75 m downwind).

The actual rate coefficient for the reaction of 1-MCP with hydroxyl radicals was measured using laser induced fluorescence. From this experiment, a half-life of 4.4 h may be deduced for the reaction of 
1-MCP with tropospheric OH radicals. Due to the negligible absorbance at wavelengths above 290 nm direct photolysis is assumed to be negligible. However, other dissipation and degradation routes may contribute to further reduce the half-life of 1-MCP in air.

It may be concluded that concentrations of 1-methylcyclopropene in air resulting from the proposed use will be low and unlikely to persist. Based on theoretical considerations ozone, formic acid, acetaldehyde and formaldehyde are likely the breakdown products in air. The quantities of these products produced by the expected maximum use of 1-MCP was compared to other anthropogenic sources of these compounds and was considered negligible and unlikely to impact the chemistry of the upper atmosphere globally or air quality nearer ground level at the local scale.

zRMS comments:

No comments.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

None

All data mentioned in this section were already submitted for the renewal process (Art. 43 of the Regulation (EC) No 1107/2009) of the product AF-777 (SmartFreshTM ProTabs) including the critical use in apple.

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

The studies listed below are not submitted herewith as these reports have been either evaluated during the EU review of 1-MCP or during national registration of AF-777.

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 9.1.3/01 | Thibodeaux, LJ | 1996 | In: Environmental Chemodynamics, The Movement of Chemicals in Air, Water and Soil. --- John Wiley and Sons, Inc. New York. pp 161; 391-392 Non GLP Published | N | --- |
| KCP 9.1.3/02 | Shade, WD. | 2001 | 1-Methylcyclopropene (1-MCP): industrial source complex (ISCST3) dispersion modeling of air emissions from controlled atmosphere facilities Report No. 00R-1066 Rohm and Haas Company Non GLP Not published | N | AgroFresh |
| KCP 9.2.4.1/01 | Ivanov, K., Lobe, I. | 2019 | Raw data to PEC in groundwater (PECgw) for 1-MCP using FOCUS Models related to the dRR – Part B Section 8 – for the product AF-777 Doc. No. 782-002, Scientific Consulting Company, Bad Kreuznach, Germany, non-GLP, not published | N | AgroFresh |
| KCP 9.2.5/01 | Ivanov, K., Lobe, I. | 2019 | Raw data to PEC in surface water (PECSW) for 1-MCP using FOCUS Models related to the dRR – Part B Section 8 – for the product AF-777 Doc. No. 782-003, Scientific Consulting Company, Bad Kreuznach, Germany, non-GLP, not published | N | AgroFresh |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Detailed evaluation of the new Annex II studies

No additional studies submitted in framework of this application.

- Additional information provided by the applicant (e.g. detailed modelling data)

## A.3.1 	Drinking water criterion

According to the dutch web site (www.vewin.nl/probleemstoffen), 1-MCP is not mentioned. Furthermore, several 1-MCP products have been on the Dutch market for more than 3 years (authorised since 2005). This period is sufficiently large to consider the market share to be established. From the general scientific knowledge collected by the CTGB about the product and its active substance, the CTGB concludes that there are in this case no concrete indications for concern about the consequences of this product for surface water from which drinking water is produced, when used in compliance with the directions for use. The CTGB does under this approach expect no exceeding of the drinking water criterion. The standards for surface water destined for the production of drinking water are met. Since this application is for the interzonal, indoor use of AF-777 in potatoes, we consider that the conclusions drawn by the Dutch competent authority as representative for drinking water safety across countries in the EU. Besides, 1-MCP is also not mentioned as chemical compound of concern in the German drinking water ordinance.

## A.3.2 	Monitoring data surface water

There are no data available in the Pesticide Atlas regarding the presence of 1-MCP in surface water.

Thibodeaux, L.J (1996), In: Environmental Chemodynamics, The Movement of Chemicals in Air, Water and Soil.  John Wiley and Sons, Inc. New York. pp 161; 391-392.