REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: A17960B Product name(s): FORTENZA Chemical active substance: Cyantraniliprole, 600 g/L

Interzonal Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (New Authorization)

Applicant: Syngenta Submission date: 30/10/2020 MS Finalisation date: August 2021 (initial Core Assessment) December 2021 (final Core Assessment)

Version history

| When | What |
| --- | --- |
| October 2020 | Initial dRR – Bayer submission |
| February 2021 | Update of GAP table for minor use (Noctuinae) |
| August 2021 | Initial zRMS assessment The report in the dRR format has been prepared by the Applicant , therefore all comments, additional evaluations and conclusions of the zRMS are presented in grey commenting boxes. Minor changes are introduced directly in the text and highlighted in grey. Not agreed or not relevant information are struck through and shaded for transparency. |
| December 2021 | Final report (Core Assessment updated following the commenting period) No additional information or assessments after the commenting period. |

Table of Contents

5	Analytical methods	5

5.1	Conclusion and summary of assessment	5

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	6

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	6

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	6

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	7

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	15

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	15

5.2.2	Methods for the determination of residues of cyantraniliprole (KCP 5.1.2)	15

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	17

5.3.1	Analysis of the plant protection product (KCP 5.2)	17

5.3.2	Description of analytical methods for the determination of residues of cyantraniliprole (KCP 5.2)	17

5.3.2.1	Overview of residue definitions and levels of cyantraniliprole for which compliance is required	17

5.3.2.2	Description of analytical methods for the determination of residues of cyantraniliprole in plant matrices (KCP 5.2.1)	19

5.3.2.3	Description of analytical methods for the determination of residues of cyantraniliprole in animal matrices (KCP 5.2.2)	19

5.3.2.4	Description of methods for the analysis of cyantraniliprole in body fluids and tissues (KCP 5.2.3)	21

5.3.2.5	Description of methods for the analysis of cyantraniliprole in soil (KCP 5.2.4)	21

5.3.2.6	Description of methods for the analysis of cyantraniliprole in water (KCP 5.2.5)	21

5.3.2.7	Description of methods for the analysis of Cyantraniliprole in air (KCP 5.2.6)	22

5.3.2.8	Other studies/ information	22

5.4	References	22

Appendix 1	Lists of data considered in support of the evaluation	23

Appendix 2	Detailed evaluation of submitted analytical methods	33

A 2.1	Analytical methods for cyantraniliprole	33

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	33

A 2.1.1.1	Description of analytical methods for the determination of residues in support of environmental fate studies (KCP 5.1.2.1)	33

A 2.1.1.2	Description of analytical methods for the determination of residues in support of toxicological studies (KCP 5.1.2.3)	33

A 2.1.1.3	Description of analytical methods for the determination of residues in support of operator, worker, resident and bystander exposure studies (KCP 5.1.2.4)	33

A 2.1.1.4	Description of analytical methods for the determination of residues in support of residues studies (KCP 5.1.2.5)	33

A 2.1.1.5	Description of analytical methods for the determination of residues in support of ecotoxicological studies (KCP 5.1.2.6)	33

A 2.1.1.6	Description of analytical methods for the determination of residues in support of physical and chemical properties tests (KCP 5.1.2.7)	47

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	47

A 2.1.2.1	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2.1)	47

A 2.1.2.2	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2.2)	47

A 2.1.2.3	Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2.3)	47

A 2.1.2.4	Description of Methods for the Analysis of Soil (KCP 5.2.4)	50

A 2.1.2.5	Description of Methods for the Analysis of Water (KCP 5.2.5)	50

A 2.1.2.6	Description of Methods for the Analysis of Air (KCP 5.2.6)	50

A 2.1.2.7	Other Studies/ Information	50

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for the active substance and relevant impurities in the plant protection product.

Noticed data gaps are: none

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are: None

zRMS conclusions: In EFSA Journal 2014;12(9):3814 “Peer review of the pesticide risk assessment of the active substance cyantraniliprole” it is stated that The residue definition for all matrices is cyantraniliprole only. LC-MS/MS methods are available for products of animal and plant origin, soil, water and air. A method of analysis for body fluids and tissues is not required as the active substance is not proposed for classification as toxic or very toxic. Monitoring methods for the determination of residues in crop commodities, products of animal origin and environmental matrices have been evaluated during the EU review of cyantraniliprole, where they were considered adequate and acceptable. Food/feed of plant origin (analytical technique and LOQ for methods for monitoring purposes); LC-MS/MS LOQ = 0.01 mg/kg for cyantraniliprole in plants (matrices with high water content, cereal and dry product, fatty product and acidic matrices) ILV is also available Method and ILV is also available and validated at the same LOQ for some metabolites. Food/feed of animal origin (analytical technique and LOQ for methods for monitoring purposes); LC-MS/MS LOQ = 0.01 mg/kg for cyantraniliprole in foodstuff of animal origin (milk, eggs, liver, kidney, meat and fat) ILV is also available Method and ILV is also available and validated at the same LOQ for some metabolites. Soil (analytical technique and LOQ); LC-MS/MS LOQ = 1.0 μg/kg for cyantraniliprole Method is also available and validated at the same LOQ for some environmental degradation products. Water (analytical technique and LOQ); LC-MS/MS LOQ = 0.1 μg/L for cyantraniliprole Method is also available and validated at the same LOQ for some environmental degradation products Analytical methods are also available for the determination of cyantraniliprole and environmental degradation products in sediment with LOQ equal to 1.0 μg/kg for each analyte. Air (analytical technique and LOQ); LC-MS/MS LOQ = 0.9 μg/m3 for cyantraniliprole Method is also available and validated at the same LOQ for some environmental degradation products. Body fluids and tissues (analytical technique and LOQ); Not required. Furthermore the Applicant submitted a number of methods for analysis of residues of cyantraniliprole and its metabolite for the generation of pre-authorization data (in support of ecotoxicological studies). The details of the evaluation of new and additional studies are referred in Appendix 2. According to the EFSA Journal 2014;12(9):3814 a method of analysis for body fluids and tissues is not required as the active substance is not proposed for classification as toxic or very toxic. However in Reg (EU) No 283/2013 it is stated that “methods, with a full description, shall be submitted for the analysis in body fluids and tissues for active substance and relevant metabolites”. A method for the determination of residues of cyatraniliprole in body fluid was submitted by Applicant. The details of the evaluation of new study is referred in Appendix 2. Analytical method FMC-49234 for the determination of residues of cyantraniliprole and chlorantraniliprole in plasma and urine was successfully validated at a limit of quantification (LOQ) of 1 µg/L. No additional data are required.

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Maize and sweet corn | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

The plant protection product A17960B has not been reviewed at EU level as a consequence of the review of Cyantraniliprole.

An overview on the acceptable methods and possible data gaps for analysis of Cyantraniliprole in plant protection product is provided as follows:

| Comments of zRMS: | The study has been already evaluated and accepted by the zRMS in the course of the zonal evaluation of Fortenza performed for purposes of authorisation as a seed treatment in oilseed rape and finalised in July 2020. Although no authorisation for uses in oilseed rape could be granted, zRMS evaluation of the study remain valid and presented below conclusions were taken from the Core Assessment, Part B, Section 5 of July 2020. Acceptable validation data for the method have been provided. The method is considered to be sufficient to determine the cyantraniliprole active within A17960B – the method has been validated in accordance with the SANCO 3030/99 rev 4 guidance. |
| --- | --- |

| Reference: | KCP 5.1.1/01 |
| --- | --- |
| Report | Das R., Jenniches D. (2011). A17960A/B - SYN545377 in formulation FS (600) by HPLC. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No ST-18/1. Issued date: 13.01.2011 Syngenta File No. A17960A_10000 |
| Guideline(s): | none |
| Deviations: | none |
| GLP: | no |
| Acceptability: | Yes |

| Reference: | KCP 5.1.1/02 |
| --- | --- |
| Report | Das R. (2011) A17960A - Validation of analytical method ST-18/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No 122951. Issued date: 24.05.2011 Syngenta File No. A17960A_10010 |
| Guideline(s): | SANCO 3030/99 rev. 4 |
| Deviations: | None |
| GLP: | Yes |
| Acceptability: | Yes |

| Reference: | KCP 5.1.1/03 |
| --- | --- |
| Report | Das R. (2011a) Cyantraniliprole FS (600) - Statement on the applicability of the validation of the analytical method SF-18/1 to A17960B. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No 10463209. Issued date: 26.05.2011 Syngenta File No. A17960B_10007 |
| Guideline(s): | none |
| Deviations: | none |
| GLP: | no |
| Acceptability: | Yes |

Materials and methods

Cyantraniliprole is determined in A17960A by liquid chromatography using a reversed phase column, using an acetonitrile: water (0.1% phosphoric acid) eluent and UV detection at 254 nm. The peak area for cyantraniliprole and the reference standard are measured by a data handling system and used to calculate the cyantraniliprole content of the sample.

The chromatograms of the blank formulation and the technical formuation demonstrate that no significant difference between A17960A and A17960B and that there is no interference in A17960B could be detected.

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of active substances cyantraniliprole in plant protection product A17960B

|  | Cyantraniliprole |
| --- | --- |
| Author(s), year | Das R., Jenniches D. (2011) |
| Principle of method | HPLC and UV detection |
| Linearity n = 5 Tested between 50 % and 150 % of declared content | r = 0.99993 y = 0.996*X+2.7 |
| Precision – Repeatability Mean n = 10 | Srel (%RSD) = 0.539 mean concentration = 49.02 % |
| Accuracy 3 samples Tested between 75 % - 125 % of declared content | mean recovery = 100.3 % |
| Interference/ Specificity | no significant interference |
| Comment | the method is acceptably validated |

Conclusion

The method is suitable for the specific, accurate and precise determination of Cyantraniliprole in product A17960B.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

IN-RYA13-001, IN-Q6S09-003, n-heptane, 3-picoline, methanesulfonic acid and acetonitrile are relevant impurities in A17960B.

These relevant impurities may be present in trace amounts in cyantraniliprole technical material. These impurities cannot be formed from cyantraniliprole or from other formulation components of A17960B and will not increase in concentration during storage of the formulation. Therefore, it is not necessary to provide data for the content of these relevant impurities in A17960B after storage.

Analytical method for IN-RYA13-001

Analytical method SD-1794/1 has been developed for the determination of the relevant impurity IN- IN-RYA13-001 in A17960B.  IN-RYA13-001 is an impurity which may be found in A17960B at trace levels as a result of the cyantraniliprole active ingredient manufacturing process. IN-RYA13-001 is not formed during manufacture or storage of cyantraniliprole.  The analytical method for the determination of IN-RYA13-001 in A17960B has not previously been reviewed and is provided in support of this assessment.

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The study has been already evaluated and accepted by the zRMS in the course of the zonal evaluation of Fortenza performed for purposes of authorisation as a seed treatment in oilseed rape and finalised in July 2020. Although no authorisation for uses in oilseed rape could be granted, zRMS evaluation of the study remain valid and presented below conclusions were taken from the Core Assessment, Part B, Section 5 of July 2020. The method validation was performed under GLP, for formulation A18351B (cyantraniliprole/acibenzolar-S-methyl SC (100/012.5)). The validation has demonstrated accuracy (recovery, linearity), precision (repeatability), specificity and limit of quantification of the method. Method SD-1794/1 will also be used for the determination of IN-RYA13-001 in A17960B. The different nominal concentrations of cyantraniliprole in formulations A18351B and A17960B are not of relevance, as the sample weighing is adapted accordingly. Thus the concentration of cyantraniliprole and its relevant impurity remains the same in the test solutions. Summary below reliably described the method procedure SD-1794/1 and validation data contained within the studies reports. Acceptable validation data for the method have been provided. The method is considered to be sufficient to determine the IN-RYA13-001 within A17960B – the method has been validated in accordance with the SANCO 3030/99 rev 4 guidance. |
| --- | --- |

| Reference: | KCP 5.1.1/04 |
| --- | --- |
| Report | Heintz K., Burkhard R. (2015), A18351B - Analytical Method SD-1794/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. 300029873. Issued date: 15.01.2015 Syngenta File No. A18351B_10416 |
| Guideline(s): | No |
| Deviations: | No |
| GLP: | No |
| Acceptability: | Yes |

| Reference: | KCP 5.1.1/05 |
| --- | --- |
| Report | Heintz K. (2015) A18351B - Validation of Analytical Method SD-1794/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. CHMU140761. Issued date 12.01.2015 Syngenta File No. A18351B_10417 |
| Guideline(s): | SANCO 3030/99 rev. 4 |
| Deviations: | no |
| GLP: | yes |
| Acceptability: | Yes |

| Reference: | 5.1.1/06 |
| --- | --- |
| Report | Heintz K. (2015a) Statement on Validation of the Analytical Method SD-1794/1 for the determination of IN-RYA13-001 in A17960B cyantraniliprole FS (600). Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No 300033315. Issued date: 21.01.2015 Syngenta File No. A17960B_10083 |
| Guideline(s): | no |
| Deviations: | no |
| GLP: | no |
| Acceptability: | Yes |

- Materials and methods

The relevant impurity IN-RYA13-001 is determined in FS formulation such as A17960B by liquid chromatography on a Nucleosil C18 column using an acetonitrile: water (0.1% trifluoroacetic acid) eluent and positive MS2 scan of ion m/z 493 (most intense peak of MH+ isotopic pattern). Quantification is by comparison of peak areas ratios to those of a reference solution.

- Validation - Results and discussions

Full validation of the method SD-1794/1 has been conducted. The method has been shown to be specific for the determination of IN-RYA13-001 in product A17960B and no significant interference was observed. Based on the results for repeatability, recovery, linearity and specificity, precision and accuracy of the method are established. Therefore, the method is suitable for the specific, accurate and precise determination of IN-RYA13-001 in product A17960B.

Table 5.2-2:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) A17960B

| Relevant impurities in cyantraniliprole | IN-RYA13-001 max. content in PPP ≤ 20 mg/kg |
| --- | --- |
| Author(s), year | Heintz K. 2015 |
| Principle of method | HPLC and MS detection |
| Linearity n = 6 Tested between 10-29 p.p.m of IN-RYA13-001 relative to the amount of Cyantraniliprole in A17960B | r = 0.9959 y = 2374.51*X-2077 |
| Precision as repeatability n = 6 (duplicate injections of A18351B) | mean conc.= 16.48 ppm Srel (%RSD) = 4.80 |
| Accuracy n = 5 Tested between 10-29 p.p.m of IN-RYA13-001 relative to the amount of Cyantraniliprole in A17960B | mean recovery = 98.1 % |
| Interference/Specificity | no significant interference |
| LOQ | 0.05% w/w |
| Comment | the method is acceptably validated |

Conclusion

The method is suitable for the specific, accurate and precise determination of IN-RYA13-001 in formulation A17960B.

Analytical method for IN-Q6S09-003

Analytical method SD-1767/1 has been developed for the determination of the relevant impurity IN-Q6S09-003 in A17960B.  IN-Q6S09-003 is an impurity which may be found in A17960B at trace levels as a result of the Cyantraniliprole active ingredient manufacturing process. IN-Q6S09-003 is not formed during manufacture or storage of Cyantraniliprole.  The analytical method for the determination of IN-Q6S09-003 in A17960B has not previously been reviewed and is provided in support of this assessment.

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The study has been already evaluated and accepted by the zRMS in the course of the zonal evaluation of Fortenza performed for purposes of authorisation as a seed treatment in oilseed rape and finalised in July 2020. Although no authorisation for uses in oilseed rape could be granted, zRMS evaluation of the study remain valid and presented below conclusions were taken from the Core Assessment, Part B, Section 5 of July 2020. Summary below reliably described the method procedure SD-1794/1 and validation data contained within the studies reports. Acceptable validation data for the method have been provided. The method is considered to be sufficient to determine the IN-Q6S09-003 within A17960B – the method has been validated in accordance with the SANCO 3030/99 rev 4 guidance. |  |  |
| --- | --- | --- | --- |
| Reference:; KCP 5.1.1/07. Report; Heintz K., Burkhard R. (2015a), A18351B - Analytical Method SD-1767/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. 300027621. Issued date: 04.02.2015 Syngenta File No. A18351B_10420. Guideline(s):; No. Deviations:; No. GLP:; No. Acceptability:; Yes. Reference:; KCP 5.1.1/08. Report; Heintz K. (2015b) A18351B - Validation of Analytical Method SD-1767/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. CHMU140762. Issued date 04.02.2015 Syngenta File No. A18351B_10419. Guideline(s):; SANCO 3030/99 rev. 4. Deviations:; no. GLP:; yes. Acceptability:; Yes. Reference:; 5.1.1/09. Report; Heintz K. (2015c) A17960B - Statement Validation Analytical Method SD-1767/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No 300033876. Issued date: 04.02.2015 (Syngenta File No. A17960B_10085). Guideline(s):; no. Deviations:; no. GLP:; no. Acceptability:; Yes. |  |

- Materials and methods

The relevant impurity IN-Q6S09-003 is determined in FS formulation such as A17960B by liquid chromatography on a Kinetext C8 column using an acetonitrile: water (0.1% formic acid) eluent and negative MS2 scan of ion m/z 327 (M-). Quantification is by comparison of peak areas ratios to those of a reference solution.

- Validation - Results and discussions

Full validation of the method SD-1767/1 has been conducted. The method has been shown to be specific for the determination of IN-Q6S09-003 in product A17960B and no significant interference was observed. Based on the results for repeatability, recovery, linearity and specificity, precision and accuracy of the method are established. Therefore, the method is suitable for the specific, accurate and precise determination of IN-Q6S09-003 in product A17960B.

Table 5.2-3:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) A17960B

| Relevant impurities in Cyantraniliprole | IN-Q6S09-003 max. content in PPP ≤ 1 mg/kg |
| --- | --- |
| Author(s), year | Heintz K. (2015) |
| Principle of method | HPLC and MS detection |
| Linearity n = 5 Tested between 0.24 – 2.38 ppm of IN-Q6S09-003 relative to the amount of cyantraniliprole in A17960B | r = 0.9992 y = |
| Precision as repeatability n = 6 (duplicate injections of A18351B) | mean conc.= 0.59 ppm Srel (%RSD) = 1.98 |
| Accuracy n = 3 Tested between 0.24 – 2.38 ppm of IN-Q6S09-003 relative to the amount of cyantraniliprole in A17960B | mean recovery = 97.9 % |
| Interference/Specificity | no significant interference |
| LOQ | 0.05% w/w |
| Comment | The method is acceptably validated |

Conclusion

The method is suitable for the specific, accurate and precise determination of IN-Q6S09-003 in formulation A17960B.

Analytical method for methanesulfonic acid

Analytical method SD-1782/1 has been developed for the determination of the relevant impurity SD-1782/1 in A17960B.  SD-1782/1 is an impurity which may be found in A17960B at trace levels as a result of the cyantraniliprole active ingredient manufacturing process. SD-1782/1 is not formed during manufacture or storage of cyantraniliprole.  The analytical method for the determination of SD-1782/1 in A17960B has not previously been reviewed and is provided in support of this assessment.

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The study has been already evaluated and accepted by the zRMS in the course of the zonal evaluation of Fortenza performed for purposes of authorisation as a seed treatment in oilseed rape and finalised in July 2020. Although no authorisation for uses in oilseed rape could be granted, zRMS evaluation of the study remain valid and presented below conclusions were taken from the Core Assessment, Part B, Section 5 of July 2020. Summary below reliably described the method procedure SD-1794/1 and validation data contained within the studies reports. Acceptable validation data for the method have been provided. The method is considered to be sufficient to determine the methanesulfonic acid within A17960B – the method has been validated in accordance with the SANCO 3030/99 rev 4 guidance. |
| --- | --- |

| Reference:; KCP 5.1.1/10. Report; Heppel C., Gold S. (2015), Determination of Methanesulfonate in technical and formulated products, by Ion chromatography. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. 300028414. Issued date: 05.01.2015 Syngenta File No. A18351B_10343. Guideline(s):; No. Deviations:; No. GLP:; No. Acceptability:; Yes. Reference:; KCP 5.1.1/11. Report; Heppel C. (2015) A18351B – Validation of Analytical Method SD-1782/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. CHMU140725. Issued date 05.01.2015 Syngenta File No. A18351B_10345. Guideline(s):; SANCO 3030/99 rev. 4. Deviations:; No. GLP:; yes. A17960B_10085; Yes. Reference:; KCP 5.1.1/12. Report; Heppel C. (2015a) Statement on Validation of Analytical Method SD-1782/1 Specificity for formulation A17960B (Cyantraniliprole FS (600)). Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No 300030631. Issued date: 19.11.2014 Syngenta File No. A17960B_10082. Guideline(s):; no. Deviations:; no. GLP:; no. Acceptability:; Yes. |  |
| --- | --- |

- Materials and methods

The relevant impurity methanesulfonic acid is determined in A17960B by ion chromatography as the form of methanesulfonate salt on an AS11-HC column, EGC III NaOH eluent, and using a conductivity detector (suppressor type ASRS 300 (4 mm)). Quantification is by comparison of peak areas ratios to those of a reference solution.

- Validation - Results and discussions

Full validation of the method SD-1782/1 has been conducted. The method has been shown to be specific for the determination of methanesulfonic acid in product A17960B and no significant interference was observed. Based on the results for repeatability, recovery, linearity and specificity, precision and accuracy of the method are established. Therefore, the method is suitable for the specific, accurate and precise determination of methanesulfonic acid in product A17960B.

Table 5.2-4:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) A17960B

| Relevant impurities in Cyantraniliprole | methanesulfonic acid max. content in PPP ≤ 2 g/kg |
| --- | --- |
| Author(s), year | Heppel C., Gold S. (2015) |
| Principle of method | Ion chromatography and conductivity detection |
| Linearity n = 5 Tested between 0.11 – 0.37 % of methanesulfonic acid relative to the amount of cyantraniliprole | r = 1.0000 y = 0.1097*X + 0.1918 |
| Precision as repeatability n = 6 (duplicate injections) | mean conc.= 0.0182 % w/w Srel (%RSD) = 1.031 |
| Accuracy n = 6 Tested between 0.11 – 0.37 % of methanesulfonic acid relative to the amount of difenoconazole | mean recovery = 100.0% |
| Interference/Specificity | no significant interference |
| LOQ | 0.1 % w/w |
| Comment | The method is acceptably validated |

Conclusion

The method is suitable for the specific, accurate and precise determination of methanesulfonic acid in formulation A17960B.

- Analytical method for n-heptane, 3-picoline, and acetonitrile

Analytical method SD-1769/1 has been developed for the determination of the relevant impurity n-heptane, 3-picoline, and acetonitrile in A17960B.  n-heptane, 3-picoline, and acetonitrile is an impurity which may be found in A17960B at trace levels as a result of the cyantraniliprole active ingredient manufacturing process. n-heptane, 3-picoline, and acetonitrile is not formed during manufacture or storage of cyantraniliprole.  The analytical method for the determination of n-heptane, 3-picoline, and acetonitrile in A17960B has not previously been reviewed and is provided in support of this assessment.

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | The study has been already evaluated and accepted by the zRMS in the course of the zonal evaluation of Fortenza performed for purposes of authorisation as a seed treatment in oilseed rape and finalised in July 2020. Although no authorisation for uses in oilseed rape could be granted, zRMS evaluation of the study remain valid and presented below conclusions were taken from the Core Assessment, Part B, Section 5 of July 2020. Summary below reliably described the method procedure SD-1794/1 and validation data contained within the studies reports. Acceptable validation data for the method have been provided. The method is considered to be sufficient to determine the n-heptane, 3-picoline, and acetonitrile within A17960B – the method has been validated in accordance with the SANCO 3030/99 rev 4 guidance. |
| --- | --- |

| Reference: | KCP 5.1.1/13 |
| --- | --- |
| Report | Heppel C. (2014a), Determination of Cyantraniliprole relevant impurities Heptane, Acetonitrile and 3-Picoline in technical and formulated products, by Gas chromatography. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. 300027843. Issued date: 14.11.2014 Syngenta File No. A18351B_10342 |
| Guideline(s): | No |
| Deviations: | No |
| GLP: | No |
| Acceptability: | Yes |

| Reference: | KCP 5.1.1/14 |
| --- | --- |
| Report | Heppel C. (2015a) A18351B – Validation of Analytical Method SD-1769/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No. CHMU140661. Issued date 26.01.2015 Syngenta File No. A18351B_10344 |
| Guideline(s): | SANCO 3030/99 rev. 4 |
| Deviations: | no |
| GLP: | yes |
| Acceptability: | Yes |

| Reference: | KCP 5.1.1/15 |
| --- | --- |
| Report | Heppel C. (2015b) A18351B - Validation Statement Analytical Method SD-1769/1. Syngenta Crop Protection Münchwilen AG, Münchwilen, Switzerland. Unpublished Report No 300030309. Issued date: 13.11.2014 Syngenta File No. A18351B_10427 |
| Guideline(s): | no |
| Deviations: | no |
| GLP: | no |
| Acceptability: | Yes |

- Materials and methods

The relevant impurities n-heptane, 3-picoline, and acetonitrile is determined in A17960B by gas chromatography on a fused silica column with a DB-Wax stationary phase with detector temperature of 300°C. Quantification is by comparison of peak areas ratios to those of a reference solution.

- Validation - Results and discussions

Full validation of the method SD-1769/1 has been conducted. The method has been shown to be specific for the determination of n-heptane, 3-picoline, and acetonitrile in product A17960B and no significant interference was observed. Based on the results for repeatability, recovery, linearity and specificity, precision and accuracy of the method are established. Therefore, the method is suitable for the specific, accurate and precise determination of n-heptane, 3-picoline, and acetonitrile in product A17960B.

Table 5.2-5:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) A17960B

| Relevant impurities in cyantraniliprole | n-heptane max. content in PPP ≤ 7 g/kg | 3-picoline max. content in PPP ≤ 3 g/kg | acetonitrile max. content in PPP ≤ 2 g/kg |
| --- | --- | --- | --- |
| Author(s), year | Heppel C. (2014) | Heppel C. (2014) | Heppel C. (2014) |
| Principle of method | headspace GC and FID detection | headspace GC and FID detection | headspace GC and FID detection |
| Linearity n = 6 | r = 0.99932 y = test range = 0.15 – 0.91% | r = 0.99954 y = test range = 0.078 - 0.467% | r = 0.99794 y = test range = 0.073% - 0.441% |
| Precision as repeatability n = 5 (duplicate injections) | mean conc.= 0.54 % w/w Srel (%RSD) = 2.59 | mean conc.= 0.28 % w/w Srel (%RSD) = 1.79 | mean conc.= 0.24 % w/w Srel (%RSD) = 2.08 |
| Accuracy n = 6 | mean recovery = 100 % test range = 0.15 – 0.91% | mean recovery = 101 % test range = 0.078 - 0.467% | mean recovery = 97 % test range = 0.073% - 0.441% |
| Interference/Specificity | no significant interference | no significant interference | no significant interference |
| LOQ | 0.15% w/w | 0.075% w/w | 0.075% w/w |
| Comment | The method is acceptably validated | The method is acceptably validated | The method is acceptably validated |

Conclusion

The method is suitable for the specific, accurate and precise determination of n-heptane, 3-picoline, and acetonitrile in formulation A17960B.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

There are no CIPAC methods for the determination of cyantraniliprole.

### Methods for the determination of residues of cyantraniliprole (KCP 5.1.2)

An overview on the acceptable methods and possible data gaps for analysis of residues of cyantraniliprole for the generation of pre-authorization data is given in the following tables. For the detailed evaluation of new/additional studies it is referred to Appendix 2.

Table 5.2-6:	Validated methods for the generation of pre-authorization data for cyantraniliprole in soil, water, air (KCP 5.1.2.1 in support of environmental fate studies)

| Method type | Matrix type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| --- | --- | --- | --- | --- |
| Component of residue definition: cyantranilliprole and its metabolites IN-K7H19, IN-JCZ38, IN-JSE76, IN-K5A77, IN-K5A78, IN-K5A79, IN-J9Z38, and IN-PLT97 |  |  |  |  |
| DuPont-15440 | Soil | 0.1 mg/kg 1.0 µg/kg | LC/MS/MS | Henze, R.M., Devine, T.J., McClory, J.P. (2006) /EU agreed |
| DuPont-18847 | Soil, ILV | 0.1 mg/kg 1.0 µg/kg | LC/MS/MS | Schwarz, T. (2010) / EU agreed |
| Component of residue definition: cyantranilliprole and its photodegradates IN-NXX70, IN-QKV54, and IN-RNU71 |  |  |  |  |
| DuPont-29747 | Soil | 0.1 mg/kg 1.0 µg/kg | LC/MS/MS | Vogl, E. (2010) / EU agreed |
| DuPont-30770 | Soil, ILV | 0.1 mg/kg 1.0 µg/kg | LC/MS/MS | Swaim, L. (2010) / EU agreed |
| Component of residue definition: active ingredient and metabolites IN-J9Z38, IN-JCZ38, IN-JSE76, IN-K5A77, IN-K5A78, IN-K5A79, and IN-PLT97 |  |  |  |  |
| DuPont-18850 | Surface and drinking water Primary | 0.1 μg/L | LC/MS/MS | Vogl, E. (2010)/ EU agreed |
| DuPont-27357 | Surface and drinking water ILV | 0.1 μg/L | LC/MS/MS | Weber, H., Schernikau, N. (2010)/ EU agreed |
| Component of residue definition: cyantraniliprole |  |  |  |  |
| DuPont-21330 | Air | 0.9 μg/m3 | LC/MS/MS | Bacher, R., Class, T. (2009)/ EU agreed |

No specific analytical methods were used to support the efficacy data generated on this product.

No analytical methods were used to support the toxicology data generated on this product.

No specific operator, worker, resident or bystander exposure studies were conducted to support this product. Consequently no analytical methods were required.

Table 5.2-3:	Validated methods for the generation of pre-authorization data for cyantraniliprole in plant and animal products (KCP 5.1.2.5 in support of residues studies)

| Component of residue definition for plant products: cyantraniliprole Component of residue definition for processed commodities: sum cyantraniliprole and IN-J9Z38 expressed as cyantraniliprole Component of residue definition for animal products: sum cyantraniliprole, IN-J9Z38, IN-MLA84 and IN-N7B69, expressed as cyantraniliprole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Method type | Matrix type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Dupont-15736 | High oil content: almond, oilseed rape seed | 0.01 mg/kg | LC-MS/MS | Method: McClory J.P., Stevens-Shreve S., Henze R.M., 2011 Report Dupont-15736, Revision No. 1 (Report IIA 4.3/08) Validation: McClory J.P., Stevens-Shreve S., Henze R.M., 2011 Report Dupont-15736, Revision No. 1 (Report IIA 4.3/08) Kinney J. 2008 Report DuPont-18846; E. I. du Pont de Nemours and Company; Wilmington, Delaware. 2008 EU agreed (United Kingdom, 2014) |
|  | High water content: cucumber, tomato, lettuce, apple, peach | 0.01 mg/kg |  |  |
|  | High acid content: lime, lemon, grapes | 0.01 mg/kg |  |  |
|  | High starch content: wheat grain, potato | 0.01 mg/kg |  |  |
|  | No group (dry): wheat straw | 0.01 mg/kg |  |  |
| Dupont-15736 | Tomato processed fractions: tomato paste, sundried tomatoes | 0.01 mg/kg | LC-MS/MS | Method: Vogl E., 2009 Report: Dupont-15736, Supplement No. 1 (Report IIA 4.3/07) Validation: Vogl E., 2009 Report Dupont-15736, Supplement No. 1 (Report IIA 4.3/07) EU agreed (United Kingdom, 2014) |
| Dupont-18844 | Milk, eggs, muscle, fat, kidney and liver | 0.01 mg/kg | LC-MS/MS | Method: Henze R.M., McClory J.P., 2010 Report Dupont-18844-Report IIA 4.3/03 Validation: Ward C., 2010 Report Dupont -25543 - Report IIA 4.3/12 EU agreed (United Kingdom, 2014) |

No specific analytical methods were used to support the ecotoxicology data generated on this product.

No specific analytical methods were used to support the physical and chemical properties generated on this product.

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of cyantraniliprole (KCP 5.2)

#### Overview of residue definitions and levels of cyantraniliprole for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-4:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Cyantraniliprole | 0.05 0.01 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Bulb vegetables Apricots |
| Plant, high acid content |  | 1.5 0.01 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Strawberries Dewberries |
| Plant, high protein/high starch content (dry commodities) |  | 0.05 0.01 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Root and tuber vegetables Peas (dry) |
| Plant, high oil content |  | 0.04 mg/kg (0.80 mg/kg) 0.01 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Tree nuts (rapeseeds) Linseeds, Peanuts, Poppy seeds |
| Plant, difficult matrices (hops, spices, tea) |  | 0.05 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Default LOQ as currently do not have a use |
| Muscle | Cyantraniliprole | 0.02 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Poultry muscle |
| Milk |  | 0.02 mg/kg | Reference (MRL Regulation (EU) No 2020/856) |
| Eggs |  | 0.15 mg/kg | Reference (MRL Regulation (EU) No 2020/856) |
| Fat |  | 0.04 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Poultry fat |
| Liver, kidney |  | 0.15 mg/kg | Reference (MRL Regulation (EU) No 2020/856) Poultry liver,kidney |
| Soil (Ecotoxicology) | Cyantraniliprole | 0.008 mg/kg | 28d chronic NOEC F. candida with TER ≥ 10 |
| Drinking water (Human toxicology) | Cyantraniliprole | 0.1 µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | Cyantraniliprole | 1.0 µg/L | Long-term aquatic invertebrate NOEC (10 µg/L) with assessment factor of 10. |
| Air | Cyantraniliprole | 0.1 mg/L | 28-day rat NOAEC |
| Body fluids | Cyantraniliprole | 1.0 ug/L | (LOQ) FMC-49234 |

#### Description of analytical methods for the determination of residues of cyantraniliprole in plant matrices (KCP 5.2.1)

An overview on the acceptable methods and possible data gaps for analysis of cyantraniliprole in plant matrices is given in the following tables.

Table 5.3-5:	Validated methods for food and feed of plant origin

| Component of residue definition: cyantraniliprole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content: tomato | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) | Method: Bacher,R,2009 Report Dupont-21328 (Report IIA4.3/02) Validation: Bacher,R, 2009 Report Dupont-21328 (Report IIA 4.3/02) ILV : Weber H, 2010 Report DuPont-21326 (Report IIA 4.3/13) EU agreed (United Kingdom, 2014) |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| High acid content: orange | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) |  |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| High oil content: almond | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) |  |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| High protein/high starch content (dry): wheat grain | DFG S 19 | 0.01 mg/kg | LC-MS/MS (multi-residue) |  |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |

For any special comments or remarkable points concerning the analytical methods for the determination of residues in plant matrices, please refer to Appendix 2.

Table 5.3-6:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | Extraction efficiency of the German DFG S19 method was not demonstrated due to the fact that multi-residue methods are intended to analyze many classes of compounds. The extraction solvents used for this procedure are consistent with solvent used to extract cyantraniliprole incurred 14C residues from crops. The extraction procedure used for the German DFG S19 procedure is a pre-soak in water followed by an acetone extraction. The cyantraniliprole metabolism study DuPont-16986 provided has demonstrated that incurred residues could be completely extracted using an acetonitrile:water, 9:1 extraction. The extraction solvent is very similar and based on the physicochemical properties of the technical grade cyantraniliprole test compound; acetone is a suitable solvent for the extraction of incurred cyantraniliprole residues. |

#### Description of analytical methods for the determination of residues of cyantraniliprole in animal matrices (KCP 5.2.2)

An overview on the acceptable methods and possible data gaps for analysis of cyantraniliprole in animal matrices is given in the following tables.

Table 5.3-7:	Validated methods for food and feed of animal origin

| Component of residue definition: cyantraniliprole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) | Method: Bacher,R,2010 Report Dupont-25544- Report IIA4.3/02 Validation: Bacher,R,2010 Report Dupont-25544- Report IIA 4.3/02 ILV : WeberH,2010 Report DuPont-17942 - Report IIA 4.3/14 EU agreed (United Kingdom, 2014) |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| Eggs | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) |  |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| Meat | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) |  |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| Kidney, liver | DFG S19 | 0.01 mg/kg | LC-MS/MS (multi-residue) |  |
|  | ILV (DFG S19) | 0.01 mg/kg |  |  |
| Fat | Dupont-18844 | 0.01 mg/kg | LC-MS/MS (single-residue) | Method: Henze R.M., McClory J.P., 2010 Report Dupont-18844-Report IIA 4.3/03 Validation: Ward C., 2010 Report Dupont -25543 - Report IIA4.3/12 ILV: Schwarz T., 2010 Report DuPont-26458 - Report IIA 4.3/10 EU agreed (United Kingdom, 2014) |
|  | ILV (Dupont-18844) | 0.01 mg/kg |  |  |

For any special comments or remarkable points concerning the analytical methods for the determination of residues in animal matrices, please refer to Appendix 2.

Table 5.3-8:	Statement on extraction efficiency

|  | Method for products of animal origin |
| --- | --- |
| Required, available from: | DAR (United Kingdom, 2014; DuPont-27356 - Report IIA 4.3/05) |
| Not required, because: | Reference: United Kingdom, 2014 Extraction efficiency of the German DFG S19 method was not demonstrated due to the fact that multi-residue methods are intended to analyze many classes of compounds. The extraction solvents used for this procedure are consistent with solvents used to extract cyantraniliprole incurred 14C residues from tissues. The extraction procedure used for the German DFG S19 procedure is a pre-soak in water followed by an acetone extraction. The cyantraniliprole metabolism study DuPont-16987 demonstrated that incurred residues could be extracted using an acetonitrile:water, (9:1) extraction. The extraction solvent is very similar and based on the physicochemical properties of the technical grade cyantraniliprole test compound, acetone is a suitable solvent for the extraction of incurred cyantraniliprole residues. |

#### Description of methods for the analysis of cyantraniliprole in body fluids and tissues (KCP 5.2.3)

Cyantraniliprole is not classified as toxic or highly toxic, therefore analytical methods for the determination of residues in body fluids and tissues are not required. However the following method can be used to determine residues of Cyantraniliprole in body fluids and tissues.

Table 5.3-6:	Methods for body fluids and tissues

| Component of residue definition: cyantraniliprole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Plasma | FMC-49234 | 1.0 ug/L | LC-MS/MS (single-residue) | Method: Pentz, A.M., Cabusas, M.E.Y.,2018 Report FMC-49234 New data |
| Urine |  | 1.0 ug/L |  |  |

#### Description of methods for the analysis of cyantraniliprole in soil (KCP 5.2.4)

An overview on the acceptable methods and possible data gaps for analysis of cyantraniliprole in soil is given in the following tables.

Table 5.3-9:	Validated methods for soil

| Component of residue definition: cyantranilliprole and its metabolites IN-K7H19, IN-JCZ38, IN-JSE76, IN-K5A77, IN-K5A78, IN-K5A79, IN-J9Z38, and IN-PLT97 |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Soil | 0.1 mg/kg | LC/MS/MS | Henze, R.M., Devine, T.J., McClory, J.P. (2006) /EU agreed |
| Soil, ILV | 0.1 mg/kg | LC/MS/MS | Schwarz, T. (2010) / EU agreed |
| Component of residue definition: cyantranilliprole and its photodegradates IN-NXX70, IN-QKV54, and IN-RNU71 |  |  |  |
| Soil | 0.1 mg/kg | LC/MS/MS | Vogl, E. (2010) / EU agreed |
| Soil, ILV | 0.1 mg/kg | LC/MS/MS | Swaim, L. (2010) / EU agreed |

For any special comments or remarkable points concerning the analytical methods for soil please refer to Appendix 2.

#### Description of methods for the analysis of cyantraniliprole in water (KCP 5.2.5)

An overview on the acceptable methods and possible data gaps for analysis of cynatraniliprole in surface and drinking water is given in the following tables.

Table 5.3-10:	Validated methods for water

| Component of residue definition: active ingredient and metabolites IN-J9Z38, IN-JCZ38, IN-JSE76, IN-K5A77, IN-K5A78, IN-K5A79, and IN-PLT97 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Surface and drinking water | Primary | 0.1 μg/L | LC/MS/MS | Vogl, E. (2010)/ EU agreed |
|  | ILV | 0.1 μg/L | LC/MS/MS | Weber, H., Schernikau, N. (2010)/ EU agreed |

For any special comments or remarkable points concerning the analytical methods for water please refer to Appendix 2.

#### Description of methods for the analysis of Cyantraniliprole in air (KCP 5.2.6)

An overview on the acceptable methods and possible data gaps for analysis of cyantraniliprole in air is given in the following tables.

Table 5.3-11:	Validated methods for air

| Component of residue definition: cyantraniliprole |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Air | 0.9 μg/m3 | LC/MS/MS | Bacher, R., Class, T. (2009)/ EU agreed |

For any special comments or remarkable points concerning the analytical methods for air it is referred to Appendix 2.

#### Other studies/ information

None.

## References

EFSA (European Food Safety Authority), 2014. Conclusion on the peer review of the pesticide risk assessment of the active substance cyantraniliprole. EFSA Journal 2014;12(9):3814, [249 pp.] doi:10.2903/j.efsa.2014.3814.

United Kingdom, 2014. OECD Global Joint Review Project, Volume 3, Annex B to the Report and Proposed Decision of the United Kingdom (RMS) and France (Co-RMS) made to the European Commision under Article 11 of Commission Regulation 1107/2009, April 2014.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1 | Das, R. Jenniches, D. | 13/01/2011 | A17960A/B - SYN545377 in formulation FS (600) by HPLC Report No. ST-18/1 Document No. VV-127928 , A17960A_10000 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Das, R. | 24/05/2011 | A17960A - Validation of analytical method ST-18/1 Report No. 10462267 Document No. VV-396516 , A17960A_10010 Test Facility Syngenta Crop Protection GLP Unpublished | N | SYN |
| KCP 5.1.1 | Das, R. | 26/05/2011 | Cyantraniliprole FS (600) - Statement on the applicability of the validation of the analytical method SF-18/1 to A17960B Report No. 10463209 Document No. VV-26306 , A17960B_10007 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Burkhard, R. Heintz, K. | 15/01/2015 | A18351B - Analytical Method SD-1794/1 Report No. 300029873 Document No. VV-128428 , A18351B_10416 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heintz, K. | 12/01/2015 | A18351B - Validation of Analytical Method SD-1794/1 Report No. CHMU140761 Document No. VV-411266 , A18351B_10417 Test Facility Syngenta Crop Protection GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heintz, K. | 21/01/2015 | Statement on Validation of the Analytical Method SD-1794/1 for the determination of IN-RYA13-001 in A17960B cyantraniliprole FS (600) Report No. 300033315 Document No. VV-28614 , A17960B_10083 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Burkhard, R. Heintz, K. | 04/02/2015 | A18351B - Analytical Method SD-1767/1 Report No. 300027621 Document No. VV-128432 , A18351B_10420 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heintz, K. | 04/02/2015 | A18351B - Validation of Analytical Method SD-1767/1 Report No. CHMU140762 Document No. VV-411440 , A18351B_10419 Test Facility Syngenta Crop Protection GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heintz, K. | 04/02/2015 | A17960B - Statement Validation Analytical Method SD-1767/1 Report No. 300033876 Document No. VV-28625 , A17960B_10085 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Gold, S. Heppel, C. | 05/01/2015 | Determination of Methanesulfonate in technical and formulated products, by Ion chromatography Report No. 300028414 Document No. VV-128426 , A18351B_10343 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heppel, C. | 05/01/2015 | A18351B – Validation of Analytical Method SD-1782/1 Report No. CHMU140725 Document No. VV-410835 , A18351B_10345 Test Facility Syngenta Crop Protection GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heppel, C. | 19/11/2014 | Statement on Validation of Analytical Method SD-1782/1 Specificity for formulation A17960B (Cyantraniliprole FS (600)) Report No. 300030631 Document No. VV-28861 , A17960B_10082 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heppel, C. | 14/11/2014 | Determination of Cyantraniliprole relevant impurities Heptane, Acetonitrile and 3-Picoline in technical and formulated products, by Gas chromatography Report No. 300027843 Document No. VV-128425 , A18351B_10342 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heppel, C. | 26/01/2015 | A18351B – Validation of Analytical Method SD-1769/1 Report No. CHMU140661 Document No. VV-410834 , A18351B_10344 Test Facility Syngenta Crop Protection GLP Unpublished | N | SYN |
| KCP 5.1.1 | Heppel, C. | 13/11/2014 | A18351B - Validation Statement Analytical Method SD-1769/1 Report No. 300030309 Document No. VV-28640 , A18351B_10427 Test Facility Syngenta Crop Protection Not GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Kuhl, R. Wydra, V. | 16/03/2011 | Cyantraniliprole FS (A17960A) – Acute toxicity to rainbow trout (Oncorhynchus mykiss) in a 96-hour static test Report No. 62993230 Document No. VV-396240 , A17960A_10004 Test Facility IBACON GmbH GLP Unpublished | Y | SYN |
| KCP 5.1.2.6 | Kuhl, R. Wydra, V. | 20/06/2011 | Amended Cyantraniliprole FS (A17960A) – Acute toxicity to Daphnia magna in a static 48-hour immobilisation test Report No. 62992220 Document No. VV-397034 , A17960A_10055 Test Facility IBACON GmbH GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Kuhl, R. Wydra, V. | 20/06/2011 | Cyantraniliprole FS (A17960A) – Toxicity to Pseudokirchneriella subcapitata in an algal growth inhibition test Report No. 62991210 Document No. VV-397173 , A17960A_10054 Test Facility IBACON GmbH GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Ruhland, S. | 05/01/2018 | Cyantraniliprole (SYN545377) - Chronic toxicity to the honey bee Apis mellifera L. in a 10 day continuous laboratory feeding study Report No. 17 48 BAC 0038 Document No. VV-468965 , SYN545377_12259 Test Facility BioChem agrar GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Ruhland, S. | 25/05/2018 | Cyantraniliprole (SYN546190) - Chronic toxicity to the honey bee Apis mellifera L. in a 10 day continuous laboratory feeding study Report No. 16 10 48 129 B Document No. VV-469913 , SYN546190_10003 Test Facility BioChem agrar GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Ruhland, S. | 26/02/2018 | Cyantraniliprole (SYN545871) - Chronic toxicity to the honey bee Apis mellifera L. in a 10 day continuous laboratory feeding study Report No. 16 10 48 131 B Document No. VV-469865 , SYN545871_10012 Test Facility BioChem GmbH GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Kleebaum, K. | 08/01/2018 | Cyantraniliprole (SYN545377) - Repeated Exposure to the Honey Bee (Apis mellifera) Larvae under laboratory Conditions (until Adult Emergence up to Day 22) Report No. 16 10 48 127 B Document No. VV-468966 , SYN545377_12260 Test Facility BioChem agrar GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Kleebaum, K. | 21/09/2018 | Cyantraniliprole (SYN546190) – Repeated Exposure to the Honey Bee (Apis mellifera) Larvae under laboratory Conditions (until Adult Emergence up to Day 22) Report No. 16 10 48 128 B Document No. VV-470531 , SYN546190_10004 Test Facility BioChem agrar GLP Unpublished | N | SYN |
| KCP 5.1.2.6 | Kleebaum, K. | 18/06/2018 | Cyantraniliprole (SYN545871) - Repeated Exposure to the Honey Bee (Apis mellifera) Larvae under laboratory Conditions (until Adult Emergence up to Day 22) Report No. 16 10 48 130 B Document No. VV-470102 , SYN545871_10013 Test Facility BioChem agrar GLP Unpublished | N | SYN |
| KCP 5.2.3 | Pentz, A.M., Cabusas, M.E.Y | 2018 | Analytical Method for the Determination of Cyantraniliprole (DPX-HGW86) and Chlorantraniliprole (DPX-E2Y45) in Plasma and Urine by HPLC/ESI-MS/MS FMC FMC Corporation, Stine Research Centre, Newark, Delaware 19711, GLP Not published | N | SYN/FMC |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.2.5 KCP 5.2.2 | Joseph P. McClory Sheila Stevens-Shreve Robert M. Henze | 2007 | Analytical Method for the Determination of DPX-HGW86 and Metabolites in Crops using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: Dupont-15736 E. I. du Pont de Nemours and Company DuPont Crop Protection Global Technology Division Stine-Haskell Research Center Newark, Delaware 19714-0030 GLP not published Syngenta File No VV-127753 | N | Dupont |
| KCP 5.1.2.5 KCP 5.2.2 | Robert M. Henze Joseph P. McClory | 2010 | Analytical Method for the Determination of DPX-HGW86 and Metabolites in Bovine Tissues, Diary and Eggs using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: Dupont-18844 E. I. du Pont de Nemours and Company DuPont Crop Protection Global Technology Division Stine-Haskell Research Center Newark, Delaware 19714-0030 GLP not published Syngenta File No VV-127909 | N | Dupont |
| KCP 5.1.2.5 KCP 5.2.2 | Timo Schwarz | 2010 | Independent Laboratory Validation of DuPont-18844 Method for the Determination of Residues of DPX-HGW86 and Metabolites in Milk and Bovine Tissues using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-26458 PTRL Europe Helmholtzstr. 22, Science Park D-89081 Ulm, Germany Project Identification Number PTRL Europe: Study P 1841 G GLP not published Syngenta File No VV-401091 | N | DuPont |
| KCP 5.1.2.5 KCP 5.2.2 | Reiner Bacher | 2009 | Method Assessment and Validation for the Determination of Residues of DPX-HGW86 in Plants using Multi-Residue Method DFG S19 (LC_MS_MS Module) E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: Dupont-21328 PTRL Europe Helmholtzstr. 22, Science Park D-89081 Ulm, Germany Project Identification Number PTRL Europe: Study No. P 1839 G GLP not published Syngenta File No VV-393976 | N | DuPont |
| KCP 5.1.2.5 KCP 5.2.2 | Harald Weber | 2010 | Independent Laboratory Validation for the Determination of Residues of DPX-HGW86 in Plants using Multi-Residue Method DFG S19 (LC-MS/MS Module) E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: Dupont-21326 Eurofins Dr. Specht GLP GmbH Großmoorbogen 25 21079 Hamburg, Germany Specht Study Plan: DUP-1005V GLP not published Syngenta File No VV-400943 | N | DuPont |
| KCP 5.1.2.5 KCP 5.2.2 | Reiner Bacher | 2010 | Method Assessment and Validation for the Determination of Residues of DPX-HGW86 and Metabolites in Animal Tissues using Multi-Residue Method DFG S19 (LC_MS_MS Module) E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: Dupont-25544 PTRL Europe Helmholtzstr. 22, Science Park D-89081 Ulm, Germany PTRL Europe: Study No. P 1854 G GLP not published Syngenta File No VV-393501 | N | DuPont |
| KCP 5.1.2.5 KCP 5.2.2 | Harald Weber | 2010 | Independent Laboratory validation for the Determination of Residues of DPX-HGW86 in Animal Tissues using Multi-Residue Method DFGS19 (LC-MS/MS Module E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-17942 Eurofins Dr. Specht GLP GmbH Großmoorbogen 25 21079 Hamburg, Germany Specht Study Plan: DUP-1006V GLP not published Syngenta File No VV-395318 | N | DuPont |
| KCP 5.1.2.5 KCP 5.2.2 | Clive Ward | 2010 | Validation of an Analytical Method for the Determination of DPX-HGW86, IN-HGW87, IN-N7B69, IN-K7H19, IN-JCZ38, IN-MYX98, IN-J9Z38 and IN-MLA84 in Bovine Tissues, Milk and Eggs E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: Dupont -25543 Charles River Tranent Edinburgh EH33 2NE UK Charles River Project No.: 215529 (unique identifier) GLP not published Syngenta File No VV-392957 | N | DuPont |
| KCP 5.2.2 | Harald Weber | 2010 | Independent Laboratory validation for the Determination of Residues of DPX-HGW86 in Animal Tissues using Multi-Residue Method DFGS19 (LC-MS/MS Module) E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-17942 Eurofins Dr. Specht GLP GmbH Großmoorbogen 25 21079 Hamburg, Germany Project Identification Numbers Specht Study Plan: DUP-1006VGLP not published Syngenta File No VV-395318 | N | DuPont |
| KCP 5.2.2 | Joseph Kinney Serena Melville | 2010 | Extraction Efficiency of [14C]DPX-HGW86 from Muscle, Liver and Milk from Goats and Eggs from Hens E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-27356 Charles River Tranent Edinburgh EH33 2NE UK Charles River Project No.: 217238 GLP not published Syngenta File No VV-401093 | N | DuPont |
| KCP 5.1.2.1 | Timo Schwarz | 2010 | Independent Laboratory Validation of DuPont-15440 Method for the Determination of Residues of DOX-HGW86 and Metabolites in Soil using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-15440 PTRL Europe Helmholtzstr. 22, Science Park D-89081 Ulm, Germany Project Identification Number PTRL Europe: Study No. P 1840 G GLP not published Syngenta File No VV-393642 | N | DuPont |
| 5.1.2.1 | Timo Schwarz | 2010 | Independent Laboratory Validation of DuPont-15440 Method for the Determination of Residues of DOX-HGW86 and Metabolites in Soil using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-18847 PTRL Europe Helmholtzstr. 22, Science Park D-89081 Ulm, Germany Project Identification Number PTRL Europe: Study No. P 1840 GGLP GLP not published Syngenta File No VV-393642 | N | DuPont |
| 5.1.2.1 | Emily Vogl | 2010 | Analytical Method for the Determination of DPX-HGW86 and Photoproducts in Soil using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-29747 ABC Laboratories, Inc. 7200 East ABC Lane Columbia, Missouri 65202 ABC Laboratories, Inc.: ABC-65533 GLP not published Syngenta File No VV-127905 | N | DuPont |
| 5.1.2.1 | Lisa Swaim | 2010 | Independent Laboratory Validation of "Analytical Method for Determination of DPX-HGW86 and Photoproducts in Soil using LC/MS/MS" E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-30770 ABC Laboratories, Inc. 7200 East ABC Lane Columbia, Missouri 65202 ABC Laboratories Inc. ID: 65936 GLP not published Syngenta File No VV-393653 | N | DuPont |
| 5.1.2.1 | Emily Vogl | 2010 | Analytical Method for the Determination of DPX-HGW86 and Metabolites in Water ( Pond, Stream, Well, and Tap) using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-18850 ABC Laboratories, Inc. 7200 East ABC Lane Columbia, Missouri 65202 ABC Laboratories, Inc.: ABC-64921 GLP not published Syngenta File No VV-127584 | N | DuPont |
| 5.1.2.1 | Harald Weber Nina Schernikau | 2010 | Independent Laboratory Validation of an Analytical Method for the Determination of DPX-HGW86 and Metabolites in Water (Surface, Ground and Tap) using LC-MS-MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-27357 Eurofins Dr. Specht GLP GmbH Großmoorbogen 25 21079 Hamburg, Germany Project Identification Numbers Specht Study Plan: DUP-1004V GLP not published Syngenta File No VV-394973 | N | DuPont |
| 5.1.2.1 | Reiner Bacher Thomas Class | 2009 | Analytical Method for the Determination of DPX-HGW86 in Air using LC/MS/MS E.I. Du Pont de Nemours, Newark, DE, USA, DuPont Study Number: DuPont-21330 PTRL Europe Helmholtzstr. 22, Science Park D-89081 Ulm, Germany Project Identification Numbers PTRL Europe: Study No. P 1838 G GLP not published Syngenta File No VV-127621 | N | DuPont |

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for cyantraniliprole

    - Methods used for the generation of pre-authorization data (KCP 5.1)

      - Description of analytical methods for the determination of residues in support of environmental fate studies (KCP 5.1.2.1)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of toxicological studies (KCP 5.1.2.3)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of operator, worker, resident and bystander exposure studies (KCP 5.1.2.4)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of residues studies (KCP 5.1.2.5)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in support of ecotoxicological studies (KCP 5.1.2.6)

No new or additional studies have been submitted.

        - Analytical method used for validation of cyantraniliprole residues in rainbow trout Oncorhynchus mykiss

          - Method validation

| Comments of zRMS: | Water samples from the study were analyzed to determine the concentrations of cyantraniliprole. The analyses were performed using Liquid Chromatography (LC) coupled with UV-Vis detection. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole in water samples. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 5 mg/L for cyantraniliprole. The overall recovery was calculated at 90% with RSD=9%. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Kuhl R., Wydra V. (2011) Cyantraniliprole FS (A17960A) - Acute Toxicity to Rainbow Trout (Oncorhynchus mykiss) in a 96-hour Static TestReport No: 62993230. Institut für Biologische Analytik, und Consulting IBACON GmbH, Arheilger Weg 17, 64380 Rossdorf, Germany (Syngenta File No. A17960A_10004, VV-396240)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:		No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Water samples are analysed using Liquid Chromatography (LC) coupled with UV-Vis detection. The limit of quantification (LOQ) is 5 mg/L. The analytical method was validated in water as a representative complex aqueous matrix that would demonstrate suitability of the analytical method for analysis of cyantraniliprole.

Detailed information on the analytical method validation can be found on page 17 of the report.

Specificity

LC-UV provides high specificity for the analysis and detection of cyantraniliprole for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (5, 15 and 100 mg/L, respectively) samples exhibit a peak at approximately 5.0 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of cyantraniliprole

| Matrix | Fortification Level (mg/L) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Water | 5 | 98, 97, 99, 97 | NR | NR | 97-99 |
|  | 15 | 93, 93, 96, 93 | NR | NR | 93-96 |
|  | 100 | 78, 81, 79, 80 | NR | NR | 78-81 |
|  | Overall | - | 90 | 9 | 78-99 |

Residues control samples and reagent blanks were less than 30% of the LOQ, % Mean calculated using un-rounded values

NR: Not Reported

Linearity

The linearity was determined by single analysis of 7 calibration points in the range of 2.04-50.92 mg/L. The correlation coefficient is 0.9997.

Recovery

Fortified samples were analysed at low, intermediate and high (5, 15 and 100 mg/L) fortification levels. The overall recovery was calculated at 90%.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 9% overall.

Limit of quantification (LOQ)

The LOQ was established at 5 mg/L. No interfering compounds were found to elute at the same time as cyantraniliprole in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of cyantraniliprole.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of cyantraniliprole with a limit of quantification of 5 mg/L.

(Kuhl R., Wydra V. 2011)

        - Analytical method used for validation of cyantraniliprole residues in waterflea Daphnia magna

          - Method validation

| Comments of zRMS: | Water samples from the study were analyzed to determine the concentrations of cyantraniliprole. The analyses were performed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.98 µg cyantraniliprole/L. The overall recovery was calculated at 96% with RSD=13%. Remark: Fortified samples of 1 μg test item/L were not taken into account for the calculation. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole in water samples with LOQ of 0.98 µg/L The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Kuhl R., Wydra V. (2011a) Cyantraniliprole FS (A17960A) - Acute Toxicity to Daphnia magna in a Static 48-hour Immobilisation Test. Report No: 62992220. Institut für Biologische Analytik, und Consulting IBACON GmbH, Arheilger Weg 17, 64380 Rossdorf, Germany (Syngenta File No. A17960A_10055, VV-397034)

Guidelines:	No guidelines were explicitly followed for this method.

Deviations:		No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Water samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.98 µg cyantraniliprole/L. The analytical method was validated in water as a representative complex aqueous matrix that would demonstrate suitability of the analytical method for analysis of cyantraniliprole.

Detailed information on the analytical method validation can be found on page 15 of the report.

Specificity

LC-MS/MS provides high specificity for the analysis and detection of cyantraniliprole for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (1, 2, 10 and 50 µg/L, respectively) samples exhibit a peak at approximately 2.3 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of cyantraniliprole

| Matrix | Fortification Level (µg test item /L) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Water | 1* | 141, 99, 158, 146 | NR | NR | 99-158 |
|  | 2 | 198*, 129, 102, 95 | NR | NR | 95-129 |
|  | 10 | 89, 93, 100, 87 | NR | NR | 87-100 |
|  | 50 | 92, 93, 88, 87 | NR | NR | 87-93 |
|  | Overall | - | 96 | 13 | 87-158129 |

*Fortified samples of 1 μg test item/L were not taken into account for the calculation and considered as an outlier

NR: Not Reported

Linearity

The linearity was determined by single analysis of 9 calibration points in the range of 0.51-50.9 µg/L. The correlation coefficient is 0.9991.

Recovery

Fortified samples were analysed at low, intermediate and high (1, 2, 10 and 50 µg/L) fortification levels. The overall recovery was calculated at 96%.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 13% overall.

Limit of quantification (LOQ)

The LOQ was established at 0.98 µg cyantraniliprole/L. No interfering compounds were found to elute at the same time as cyantraniliprole in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of cyantraniliprole.

Conclusion

The analytical method has been demonstrated to be accurate and reliable for the analysis of cyantraniliprole with a limit of quantification of 0.98 µg cyantraniliprole/L.

(Kuhl R., Wydra V. 2011a)

        - Analytical method used for validation of cyantraniliprole residues in green alga Pseudokirchneriella subcapitata

          - Method validation

| Comments of zRMS: | Water samples from the study were analyzed to determine the concentrations of cyantraniliprole. The analyses were performed using Liquid Chromatography (LC) coupled with UV-Vis detection. The linearity was determined in the range of 0.51-50.92 mg/L. The calibration line equations are not presented. Remark: The LOQ was established at 0.4 mg test item/L corresponding to 0.2 mg cyantraniliprole/L. The analytical calibration must extend to at least the range which is suitable for the determination of recoveries and for assessment of the level of interferences in control samples. For that purpose the concentration range should be covered from 30% of the LOQ to 20% above the highest level. However in this study the following fortification levels: 0.41 (LOQ) and 0.75 mg test item /L were below the calibration range. The overall recovery was calculated at 101% with RSD=7%. General the analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole in water samples. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Kuhl R., Wydra V. (2011b) Cyantraniliprole FS (A17960A) - Toxicity to Pseudokirchneriella subcapitata in an Algal Growth Inhibition Test. Report No: 62991210. Institut für Biologische Analytik, und Consulting IBACON GmbH, Arheilger Weg 17, 64380 Rossdorf, Germany (Syngenta File No. A17960A_10054, VV-397173)

Guidelines:	No guidelines were explicitly followed for this method.

Deviations:		No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Water samples are analysed using Liquid Chromatography (LC) coupled with UV-Vis detection. The limit of quantification (LOQ) is 0.4 mg test item/L corresponding to 0.2 mg cyantraniliprole/L. The analytical method was validated in water as a representative complex aqueous matrix that would demonstrate suitability of the analytical method for analysis of cyantraniliprole.

Detailed information on the analytical method validation can be found on page 25 of the report.

Specificity

LC-UV provides high specificity for the analysis and detection of cyantraniliprole for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (0.4, 0.75, 1, 10 and 100 mg/L) samples exhibit a peak at approximately 5.1 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of cyantraniliprole

| Matrix | Fortification Level (mg/L) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Water | 0.41* | 109, 110, 111, 110 | NR | NR | 109-111 |
|  | 0.75* | 102, 100, 101, 103 | NR | NR | 100-103 |
|  | 1 | 106, 106, 107, 107 | NR | NR | 106-107 |
|  | 10 | 93, 91, 97, 95 | NR | NR | 91-97 |
|  | 100 | 95, 96, 89, 93 | NR | NR | 89-96 |
|  | Overall | - | 101 | 7 | 89-111 |

1 This value is slightly below the calibration range and has to be considered cautiously. The fortification level of 1 mg test item/L was the lowest concentrated fortification level (107 %; RSD 1 %) with a concentration within the calibration range.

*Values slightly below the calibration range

NR: Not Reported

Linearity

The linearity was determined by single analysis of various calibration points in the range of 0.51-50.92 mg/L. The correlation coefficient is 1.0000.

Recovery

Fortified samples were analysed at low, intermediate and high (0.4, 0.75, 1, 10 and 100 mg/L) fortification levels. The overall recovery was calculated at 101%.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 7% overall.

Limit of quantification (LOQ)

The LOQ was established at 0.4 mg test item/L corresponding to 0.2 mg cyantraniliprole/L. No interfering compounds were found to elute at the same time as cyantraniliprole in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of cyantraniliprole.

Conclusion

The analytical method has been demonstrated to be accurate and reliable for the analysis of cyantraniliprole with a limit of quantification of 0.4 mg test item/L corresponding to 0.2 mg cyantraniliprole/L.

(Kuhl R., Wydra V. 2011b)

        - Analytical method used for validation of cyantraniliprole residues in honeybee Apis mellifera

          - Method validation

| Comments of zRMS: | Concentrations of cyantraniliprole were determined in the feeding solutions. For cyantraniliprole the method validation was done with a limited set (one control sample, 3 repetitions each at two fortification levels) of recoveries at the LOQ (0.01 mg/kg) and 10.22 mg/kg levels. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.01 mg/kg (corresponding to 0.27 µg/L in diluted sample extract). Mean recoveries for each fortification level and the overall mean recovery were within the 70 - 110% range with a relative standard deviation ≤ 20%. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole in the feeding solutions. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Ruhland S. (2018) Cyantraniliprole (SYN545377) - Chronic toxicity to the honey bee Apis mellifera L. in a 10 day continuous laboratory feeding study. Report No: 17 48 BAC 0038. BioChem agrar Labor für biologische und chemische Analytik GmbH Kupferstraße 6 04827 Machern OT Gerichshain, Germany (Syngenta File No. SYN545377_12259, VV-468965)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:	No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Feeding solution samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.01 mg/kg (corresponding to 0.27 µg/L in diluted sample extract).

Detailed information on the analytical method validation can be found in appendix 10, page 53 of the report.

Specificity

LC-MS provides high specificity for the analysis and detection of cyantraniliprole for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (0.01, 0.13 and 10.22 mg/kg, respectively) samples exhibit a peak at approximately 3.7 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of cyantraniliprole

| Matrix | Fortification Level (mg/kg) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Feeding solution | 0.01 | 81, 100, 102 | 94 | 12.6 | 81-102 |
|  | 0.13 | 82, 83 | 83 | 0.9 | 82-83 |
|  | 10.22 | 88, 80, 81 | 83 | 5.6 | 80-88 |
|  | Overall | - | NR | NR | 80-102 |

NR: Not Reported

Linearity

The linearity was determined by single analysis of 8 calibration points in the range of 0.0720-4.9968 µg/L. The correlation coefficient is 0.99984625.

Recovery

Fortified samples were analysed at low, intermediate and high (0.01, 0.13 and 10.22 mg/kg) fortification levels. The mean recovery was calculated at 94% for the low level, 83% for the intermediate and high levels.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 12.6% for the low level, 0.9% for the intermediate level and 5.6% for the high level.

Limit of quantification (LOQ)

The LOQ was established at 0.01 mg/kg (corresponding to 0.27 µg/L in diluted sample extract). No interfering compounds were found to elute at the same time as cyantraniliprole in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of cyantraniliprole.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of cyantraniliprole according to SANCO/3029/99 rev.4 with a limit of quantification of 0.01 mg/kg (corresponding to 0.27 µg/L in diluted sample extract).

(Ruland S. 2018)

        - Analytical method used for validation of cyantraniliprole metabolite IN-HGW87 residues in honeybee Apis mellifera

          - Method validation

| Comments of zRMS: | Concentrations of cyantraniliprole metabolite IN HGW87 were determined in the feeding solutions. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.01 mg/kg (corresponding to 0.26 µg/L in diluted sample extract). Mean recoveries for each fortification level and the overall mean recovery were within the 70 - 110% range with a relative standard deviation ≤ 20%. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole metabolite IN-HGW87 in the feeding solutions. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Ruhland S. (2018a) Cyantraniliprole (SYN546190) - Chronic toxicity to the honey bee Apis mellifera L. in a 10 day continuous laboratory feeding study. Report No: 16 10 48 129 B. BioChem agrar Labor für biologische und chemische Analytik GmbH Kupferstraße 6 04827 Machern OT Gerichshain, Germany (Syngenta File No. SYN546190_10003, VV-469913)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:	No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Feeding solution (50% (w/v) aqueous sucrose solution) samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.01 mg/kg (corresponding to 0.26 µg/L in diluted sample extract).

Detailed information on the analytical method validation can be found in appendix 10, page 52 of the report.

Specificity

LC-MS provides high specificity for the analysis and detection of IN-HGW87 for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low and high level (0.010, 0.011, 0.03, 3.41 and 3.502 mg/kg) samples exhibit a peak at approximately 3.4 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of IN-HGW87

| Matrix | Fortification Level (mg/kg) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Feeding solution | 0.010 | 99, 116, 102 | 106 | 8.9 | 99-116 |
|  | 0.011 | 116, 112, 97 | 108 | 9.5 | 97-116 |
|  | 0.03 | 76, 76 | 76 | 0.1 | 76 |
|  | 3.41 | 79, 86, 93 | 86 | 8.2 | 79-93 |
|  | 3.502 | 91, 104, 97 | 97 | 6.7 | 91-104 |
|  | Overall | - | NR | NR | 76-116 |

NR: Not Reported

Linearity

The linearity was determined by single analysis of 7 calibration points in the range of 0.0655-3.4115 µg/L. The correlation coefficient is 0.99015560.

Recovery

Fortified samples were analysed at low and high level (0.010, 0.011, 0.03, 3.41 and 3.502 mg/kg) fortification levels. The mean recovery ranged from 76-108%.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this ranged from 0.1-9.5%.

Limit of quantification (LOQ)

The LOQ was established at 0.01 mg/kg (corresponding to 0.26 µg/L in diluted sample extract).  No interfering compounds were found to elute at the same time as IN-HGW87 in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of IN-HGW87.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of IN-HGW87 according to SANCO/3029/99 rev.4 with a limit of quantification of 0.01 mg/kg (corresponding to 0.26 µg/L in diluted sample extract).

(Ruhland S. 2018a)

        - Analytical method used for validation of cyantraniliprole metabolite IN-J9Z38 residues in honeybee Apis mellifera

          - Method validation

| Comments of zRMS: | Concentrations of cyantraniliprole metabolite IN J9Z38 were determined in the feeding solutions. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.01 mg/kg (corresponding to 0.25 µg/L in diluted sample extract). Mean recoveries for each fortification level and the overall mean recovery were within the 70 - 110% range with a relative standard deviation ≤ 20%. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole metabolite IN J9Z38 in the feeding solutions. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Ruhland S. (2018b) Cyantraniliprole (SYN545871) - Chronic toxicity to the honey bee Apis mellifera L. in a 10 day continuous laboratory feeding study. Report No: 16 10 48 131 B. BioChem agrar Labor für biologische und chemische Analytik GmbH Kupferstraße 6 04827 Machern OT Gerichshain, Germany (Syngenta File No. SYN545871_10012, VV-469865)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:	No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Feeding solution (50% (w/v) aqueous sucrose solution containing 5% v/v acetone, 0.1% w/v xanthan) samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.01 mg/kg (corresponding to 0.25 µg/L in diluted sample extract).

Detailed information on the analytical method validation can be found in appendix 9, page 49 of the report.

Specificity

LC-MS/MS provides high specificity for the analysis and detection of IN-J9Z38 for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (0.01, 0.113 and 802.2 mg/kg) samples exhibit a peak at approximately 4.8 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of IN-J9Z38

| Matrix | Fortification Level (mg/kg) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Feeding solution | 0.01 | 87, 95, 91 | 91 | 4.4 | 87-95 |
|  | 0.113 | 86, 73 | 79 | 11.8 | 73-86 |
|  | 802.2 | 81, 84, 79 | 81 | 2.9 | 79-84 |
|  | Overall | - | NR | NR | 73-95 |

NR: Not Reported

Linearity

The linearity was determined by single analysis of 8 calibration points in the range of 0.076-264.310 µg/L. The correlation coefficient is 0.99981081.

Recovery

Fortified samples were analysed at low, intermediate and high (0.01, 0.113 and 802.2 mg/kg) fortification levels. The mean recovery was calculated at 91% for the low level, 79% for the intermediate level and 81% for the high level.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 4.4% for the low level, 11.8% for the intermediate level and 2.9% for the high level,

Limit of quantification (LOQ)

The LOQ was established at 0.01 mg/kg (corresponding to 0.25 µg/L in diluted sample extract).  No interfering compounds were found to elute at the same time as IN-J9Z38 in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of IN-J9Z38.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of IN-J9Z38 according to SANCO/3029/99 rev.4 with a limit of quantification of 0.01 mg/kg (corresponding to 0.25 µg/L in diluted sample extract).

(Ruhland S. 2018b)

        - Analytical method used for validation of cyantraniliprole residues in larva of honeybee Apis mellifera

          - Method validation

| Comments of zRMS: | Concentrations of cyantraniliprole were determined in final diets. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extracts). Mean recoveries for each fortification level and the overall mean recovery were within the 70 - 110% range with a relative standard deviation ≤ 20%. Blank values did not exceed 30% of the lowest validated concentration. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole in the diet. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Kleebaum K. (2018) Cyantraniliprole (SYN545377) – Repeated Exposure to the Honey Bee (Apis mellifera) Larvae under laboratory Conditions (until Adult Emergence up to Day 22). Report No: 16 10 48 127 B. BioChem agrar Labor für biologische und chemische Analytik GmbH Kupferstraße 6 04827 Machern OT Gerichshain, Germany (Syngenta File No. SYN545377_12260, VV-468966)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:	No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Diet (Royal jelly/ASS containing 18% (w/v) glucose, 18% (w/v) fructose, 4% (w/v) yeast extract) samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extract).

Detailed information on the analytical method validation can be found in appendix 9, page 41 of the report.

Specificity

LC-MS provides high specificity for the analysis and detection of cyantraniliprole for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (0.0016, 0.039 and 0.10 mg/kg respectively) samples exhibit a peak at approximately 3.7 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of cyantraniliprole

| Matrix | Fortification Level (mg/kg) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Diet | 0.0016 | 77, 89, 70 | 79 | 12.1 | 70-89 |
|  | 0.039 | 78, 78 | 78 | 0.1 | 78 |
|  | 0.10 | 85, 86, 80 | 84 | 4.1 | 80-86 |
|  | Overall | - | NR | NR | 70-89 |

NR: Not Reported

Linearity

The linearity was determined by single analysis of 7 calibration points in the range of 0.031- 1.03 µg/L. The correlation coefficient is 0.99960484.

Recovery

Fortified samples were analysed at low, intermediate and high (0.0016, 0.039 and 0.10 mg/kg) fortification levels. The mean recovery was calculated at 79% for the low level, 78% for the intermediate level and 84% for the high level.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 12.1% for the low level, 0.1% for the intermediate level and 4.1% for the high level.

Limit of quantification (LOQ)

The LOQ was established at 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extract).  No interfering compounds were found to elute at the same time as cyantraniliprole in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of cyantraniliprole.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of cyantraniliprole according to SANCO/3029/99 rev.4 with a limit of quantification of 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extract).

(Kleebaum K. 2018)

        - Analytical method used for validation of cyantraniliprole metabolite IN-HGW87 residues in larva of honeybee Apis mellifera

          - Method validation

| Comments of zRMS: | Concentrations of cyantraniliprole metabolite IN HGW87 were determined in final diets. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extracts). Mean recoveries for each fortification level and the overall mean recovery were within the 70 - 110% range with a relative standard deviation ≤ 20%. Blank values did not exceed 30% of the lowest validated concentration. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole metabolite IN HGW87 in the diet. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Kleebaum K. (2018a) Cyantraniliprole (SYN546190) – Repeated Exposure to the Honey Bee (Apis mellifera) Larvae under laboratory Conditions (until Adult Emergence up to Day 22). Report No: 16 10 48 128 B. BioChem agrar Labor für biologische und chemische Analytik GmbH Kupferstraße 6 04827 Machern OT Gerichshain, Germany (Syngenta File No. SYN546190_10004, VV-470531)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:	No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Diet (Royal jelly/ASS containing 18% (w/v) glucose, 18% (w/v) fructose, 4% (w/v) yeast extract) samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extract).

Detailed information on the analytical method validation can be found in appendix 9, page 40 of the report.

Specificity

LC-MS provides high specificity for the analysis and detection of IN-HGW87 for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low, intermediate and high level (0.0016, 0.0104 and 0.071 mg/kg) samples exhibit a peak at approximately 2.3 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of IN-HGW87

| Matrix | Fortification Level (mg/kg) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Diet | 0.0016 | 88, 77, 81 | 82 | 7.2 | 77-88 |
|  | 0.0104 | 90, 96, 88 | 91 | 4.5 | 88-96 |
|  | 0.071 | 79, 80, 81 | 80 | 1.0 | 79-81 |
|  | Overall | - | NR | NR | 77-96 |

NR: Not Reported

Linearity

The linearity was determined by single analysis of 7 calibration points in the range of 0.037-1.179 µg/L. The correlation coefficient is 0.99812635.

Recovery

Fortified samples were analysed at low, intermediate and high level (0.0016, 0.0104 and 0.071 mg/kg) fortification levels. The mean recovery was calculated at 82% for the low level, 91% for the intermediate level and 80% for the high level.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 7.2% for the low level, 4.5% for the intermediate level and 1.0% for the high level.

Limit of quantification (LOQ)

The LOQ was established at 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extract). No interfering compounds were found to elute at the same time as IN-HGW87 in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of IN-HGW87.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of IN-HGW87 according to SANCO/3029/99 rev.4 with a limit of quantification of 0.0016 mg/kg (corresponding to 0.12 µg/L in diluted sample extract).

(Kleebaum K. 2018a)

        - Analytical method used for validation of cyantraniliprole metabolite IN-J9Z38 residues in larva of honeybee Apis mellifera

          - Method validation

| Comments of zRMS: | Concentrations of cyantraniliprole metabolite IN J9Z38 were determined in final diets. Limit of quantification representing the lowest successfully validated fortification level, LOQ = 0.01 mg/kg (corresponding to 0.77 µg/L in diluted sample extracts). Mean recoveries for each fortification level and the overall mean recovery were within the 70 - 110% range with a relative standard deviation ≤ 20%. Blank values did not exceed 30% of the lowest validated concentration. The analytical method is satisfactorily validated in accordance with SANCO/3029/99 rev. 4. for the determination of the concentrations of cyantraniliprole metabolite IN J9Z38 in the diet. The study is acceptable. |
| --- | --- |

| Reference: | KCP 5.1.2.6 |
| --- | --- |

Report:	Kleebaum K. (2018b) Cyantraniliprole (SYN545871) – Repeated Exposure to the Honey Bee (Apis mellifera) Larvae under laboratory Conditions (until Adult Emergence up to Day 22). Report No: 16 10 48 130 B. BioChem agrar Labor für biologische und chemische Analytik GmbH Kupferstraße 6 04827 Machern OT Gerichshain, Germany (Syngenta File No. SYN545871_10013, VV-470102)

Guidelines:	SANCO/3029/99 rev.4 11/07/00: Residues: Guidance for generating and reporting methods of analysis in support of pre-registration data requirements for Annex II (part A; Section 4) and Annex III (PART a; Section 5) of directive 91/414.

Deviations:	No

GLP:			Validation 	Yes

Acceptability:		Yes

Duplication:		No

Principle of the Method

Diet (Royal jelly/ASS containing 18% (w/v) glucose, 18% (w/v) fructose, 4% (w/v) yeast extract) samples are analysed using Liquid Chromatography (LC) coupled with Mass Spectrometry (MS/MS). The limit of quantification (LOQ) is 0.01 mg/kg (corresponding to 0.77 µg/L in diluted sample extract).

Detailed information on the analytical method validation can be found in appendix 9, page 39 of the report.

Specificity

LC-MS provides high specificity for the analysis and detection of IN-J9Z38 for the purpose of ecotoxicity studies i.e. clean, well described test matrix analysing a pre-defined quantity of test item. Chromatograms of low and high level (0.010 and 117.5 mg/kg) samples exhibit a peak at approximately 4.8 minutes with no co-eluting peaks arising from the aqueous matrices, the labware, reagents or solvents.

Recovery data

The recovery and precision data for the method validation of IN-J9Z38

| Matrix | Fortification Level (mg/kg) | Recovery (%) | Mean (%) | RSD (%) | Range (%) |
| --- | --- | --- | --- | --- | --- |
| Diet | 0.010 | 61, 87, 84 | 77 | 18.5 | 61-87 |
|  | 0.010 | 71, 80, 73 | 75 | 6.4 | 71-80 |
|  | 0.010 | 85, 75 | 80 | 8.8 | 75-85 |
|  | 117.5 | 84, 89, 79 | 84 | 5.7 | 79-84 |
|  | Overall | - | NR | NR | 61-87 |

NR: Not Reported

Linearity

The linearity was determined by single analysis of 6 calibration points in the range of 0.209-217.682 µg/L. The correlation coefficient is 0.99913314.

Recovery

Fortified samples were analysed at low and high (0.010 and 117.5 mg/kg) fortification levels. The mean recovery was calculated at 75-80% for the low level and 84% for the high level.

Repeatability

The repeatability of the method was examined by the relative standard deviation (RSD), this was calculated at 6.4-18.5% for the low level and 5.7% for the high level.

Limit of quantification (LOQ)

The LOQ was established at 0.01 mg/kg (corresponding to 0.77 µg/L in diluted sample extract). No interfering compounds were found to elute at the same time as IN-J9Z38 in any of the control samples at levels 30% greater than the defined LOQ.

Matrix extract

No significant matrix effect (suppression or enhancement) was observed during the method validation for the analysis of IN-J9Z38.

Conclusion

The analytical method has been demonstrated to be fit for purpose for the analysis of IN-J9Z38 according to SANCO/3029/99 rev.4 with a limit of quantification of 0.01 mg/kg (corresponding to 0.77 µg/L in diluted sample extract).

(Kleebaum K. 2018b)

      - Description of analytical methods for the determination of residues in support of physical and chemical properties tests (KCP 5.1.2.7)

No new or additional studies have been submitted.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2.1)

No new or additional studies have been submitted.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2.2)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2.3)

        - Analytical method FMC-49234

          - Method validation

| Method validation Comments of zRMS: | Analytical method FMC-49234 has been validated for the for the detection, quantitative analysis, and confirmation of cyantraniliprole (DPX-HGW86) and chlorantraniliprole (DPX-E2Y45) in plasma and urine by HPLC/ESI-MS/MS at a limit of quantitation of 1.0 g/L using HPLC/ESI-MS/MS. At least two ion transitions (primary/quantitative and confirmatory) were monitored during sample analyses. The limit of quantification (LOQ) was established at 1 µg/L in plasma and urine for cyantraniliprole and chlorantraniliprole. Acceptable mean recoveries in the range 70 – 120% and a relative standard deviation (RSD) of less than 20% (SANCO/825/00 rev. 8.1) were found for cyantraniliprole and chlorantraniliprole at both fortification levels (1 µg/L and 10 µg/L) and for both primary and confirmatory transitions in plasma and urine. No residues of cyantraniliprole and chlorantraniliprole greater than 30% of the LOQ were detected in any of the control samples. Analytical method FMC-49234 for the determination of residues of cyantraniliprole and chlorantraniliprole in plasma and urine was successfully validated at a limit of quantification (LOQ) of 1 µg/L according to the SANCO/825/00 rev. 8.1. The study is acceptable. |
| --- | --- |

| Data Point | KCP 5.2.3 |
| --- | --- |
| Report author | Anne M. Pentz & Maria Elena Y. Cabusas |
| Report year | 2018 |
| Report title | Analytical Method for the Determination of Cyantraniliprole (DPX-HGW86) and Chlorantraniliprole (DPX-E2Y45) in Plasma and Urine by HPLC/ESI-MS/MS |
| Report No (Syngenta File Number) | 49234 |
| Guidelines followed in study | SANCO/825/00 rev. 8.1 (2010) EPA OPPTS 860.1340 (1996) |
| Deviations from test guideline | None If yes, explain major deviation |
| Guidance in force at time of submission of supplementary dossier |  |
| Differences between old and current guideline |  |
| Previous evaluation | No, not previously submitted |
| GLP/Officially recognised testing facilities1,2 | Yes |
| Acceptability/Reliability | Yes |

1 See Art.3 of Annex of Regulation No 283/2013 and 284/2013
2 It is the RMS’ responsibility to confirm the accuracy of the applicant’s GLP statement

Principle of the method

Residues of cyantraniliprole and chlorantraniliprole were extracted from plasma or urine sample by two consecutive vigorous shaking in 9:1 acetonitrile:water and centrifugations. The supernatants/extracts were decanted and combined and then partitioned with hexane to remove unwanted matrix components; the hexane fraction was discarded.  The sample extract was filtered through a 0.45-m PTFE disc and an aliquot was taken and diluted with 0.02 M formic acid (aq).  It was then analyzed by reversed-phase HPLC using a SynergiTM Polar-RP (3.0 x 50 mm, 2.5-m dp) analytical column and a mobile phase of 0.1% formic acid (aq) and acetonitrile. Detection of the analytes was by electrospray ionization mass spectrometry/mass spectrometry (ESI-MS/MS). For each analyte, at least two ion transitions (quantitative and confirmatory) were monitored during sample analyses.

During method validation, post-extraction fortified sample(s) was(were) analyzed for each set of analytical samples to determine if matrix effect, either suppression or enhancement, influenced the recovery of cyantraniliprole and chlorantraniliprole. The post-extraction fortified sample in this study was an extract of a control crop sample that was prepared in the same manner as with the other samples but fortified with cyantraniliprole and chlorantraniliprole prior to sample analysis.

The confirmatory method for the HPLC/ESI-MS/MS method was based on acceptable calibration curves and fortification recovery data generated from the confirmatory ion transitions.  Residues in the samples were also confirmed based on the relative ratio of the two MS/MS parent-to-daughter ion transitions (ion ratio) monitored during the validation.

Recovery Findings

Summaries of the results for Cyantraniliprole and Chlorantraniliprole are presented in Tables IIA 4.3-1 and IIA 4.3-4.

Table IIA 4.3-1: Recovery results from validation for Cyantraniliprole in plasma and urine: primary transition m/z 475-286

| Matrix | Fortification Level (µg/L) | Recovery (%) | n | Mean Recovery (%) | RSD (%) | Recovery Range (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Plasma | 1.0 | 102, 104, 114, 103, 110 | 5 | 106 | 4.8 | 102-114 |
|  | 10 | 110, 117, 116, 108, 116 | 5 | 113 | 3.5 | 108-117 |
|  | Overall | - | 10 | 110 | 5.2 | 102-117 |
| Urine | 1.0 | 97, 87, 102, 99, 112 | 5 | 99 | 8.8 | 87-112 |
|  | 10 | 90, 97, 102, 115, 109 | 5 | 103 | 9.7 | 90-115 |
|  | Overall | - | 10 | 101 | 9.0 | 87-115 |

Table IIA 4.3-2: Recovery results from validation for Cyantraniliprole in plasma and urine: primary transition m/z 475-444

| Matrix | Fortification Level (µg/L) | Recovery (%) | n | Mean Recovery (%) | RSD (%) | Recovery Range (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Plasma | 1.0 | 111, 109, 106, 105, 114 | 5 | 109 | 3.3 | 105-114 |
|  | 10 | 123, 117, 116, 116, 112 | 5 | 117 | 3.5 | 112-123 |
|  | Overall | - | 10 | 113 | 4.8 | 105-123 |
| Urine | 1.0 | 117, 108, 119, 100, 119 | 5 | 113 | 7.4 | 100-119 |
|  | 10 | 102, 99, 99, 117, 105 | 5 | 105 | 7.3 | 99-117 |
|  | Overall | - | 10 | 109 | 8.0 | 99-119 |

Table IIA 4.3-3: Recovery results from validation for Chlorantraniliprole in plasma and urine: primary transition m/z 484-453

| Matrix | Fortification Level (µg/L) | Recovery (%) | n | Mean Recovery (%) | RSD (%) | Recovery Range (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Plasma | 1.0 | 96, 94, 116, 91, 94 | 5 | 98 | 10.3 | 91-116 |
|  | 10 | 116, 118, 106, 103, 103 | 5 | 109 | 6.5 | 103-118 |
|  | Overall | - | 10 |  | 9.8 | 91-118 |
| Urine | 1.0 | 101, 100, 100, 88, 104 | 5 | 98 | 6.3 | 88-104 |
|  | 10 | 99, 96, 93, 98, 103 | 5 | 98 | 3.8 | 93-103 |
|  | Overall | - | 10 | 98 | 5.0 | 88-104 |

Table IIA 4.3-4: Recovery results from validation for Chlorantraniliprole in plasma and urine: primary transition m/z 484-286

| Matrix | Fortification Level (µg/L) | Recovery (%) | n | Mean Recovery (%) | RSD (%) | Recovery Range (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Plasma | 1.0 | 93, 107, 118, 116, 98 | 5 | 107 | 10.1 | 93-118 |
|  | 10 | 116, 117, 111, 105, 110 | 5 | 112 | 4.4 | 105-117 |
|  | Overall | - | 10 |  | 7.6 | 93-118 |
| Urine | 1.0 | 99, 89, 108, 102, 106 | 5 | 101 | 7.2 | 89-108 |
|  | 10 | 90, 95, 101, 105, 109 | 5 | 100 | 7.8 | 90-109 |
|  | Overall | - | 10 | 98 | 7.0 | 89-109 |

Accuracy

Fortified samples were analysed in quintuplet at the limit of quantification (LOQ, 1.0 µg/L) and at ten times the LOQ (10 µg/L). Acceptable mean recoveries of between 70% and 110% (except Cyantraniliprole the mean at 10µg/L was 117% and for Chlorantraniliprole the mean at 10 µg/L was 112%)  were found for both transitions on all matrices tested and therefore according to EU guidance (SANCO 3029/99 rev.4 11/7/00) demonstrate the method has satisfactory accuracy.

Precision (Repeatability)

The relative standard deviations (RSDs) of Cyantraniliprole and Chlorantraniliprole recoveries at each fortification level and overall for each animal commodity tested during method validation were <20% and therefore according to the EU guidance (SANCO 3029/99 rev.4 11/7/00) demonstrate the method has satisfactory repeatability.

Precision (Reproducibility)

The method described in report 49234 is not intended for post registration monitoring and control purposes and therefore according to the guidance (see Guidelines section of this summary) an independent laboratory validation to demonstrate reproducibility is not required.

Confirmatory Analysis

LC/MS-MS with two transitions monitored is considered to be a highly specific detection technique and therefore according to EU guidance no further confirmatory technique is required. The method includes two MS/MS transitions for Cyantraniliprole and Chlorantraniliprole, both of which have been validated.

Specificity

LC-MS/MS with two transitions is considered to be a highly specific detection technique and therefore according to EU guidance (SANCO/825/00 rev.8.1, 16/11/2010) no further confirmatory technique is required. The method includes two MS/MS transitions, both of which have been validated. No significant interferences arising from the animal matrices, the labware, reagents or solvents have been observed at the retention times of interest.

Matrix Effects

No significant matrix effects were observed for either Cyantraniliprole or Chlorantraniliprole in the matrices tested during method validation, therefore non-matrix matched linearity standards were used for quantification.

Linearity

The linearity of the LC-MS/MS detector was tested using standard solutions (0.010 ng/ml to 1.0 ng/ml equivalent to 0.25 to 25 µg/L).  Linearity was tested for both MS/MS transitions for both compounds. Standards at six different concentrations were injected and the response plotted against concentration for all calibration points.  Straight lines with correlation coefficients greater than 0.995 were obtained for both compounds.

Limit of Quantification

The limit of quantification for Cyantraniliprole and Chlorantraniliprole residues in animal commodities using the method in report 49234 was established at 1.0 µg/L respectively.  No interfering peaks around the retention time of Cyantraniliprole and Chlorantraniliprole were found in any of the control samples at levels above 30% of the limit of quantification.

Stability of Final Extracts

The final extracts were found to be stable for 48 refrigerated (2 -10oC) or 72 hours when kept frozen (-20oC).

Stability of Standard Solutions

Stock solutions are stable for 6 months when stored in a freezer at a temperature of -20oC. The calibration standards are stable for 72 hours when kept at 4oC and 3 days when kept at -20oC.

Conclusion   
Analytical method described in report 49234 has been shown to be a reliable and accurate procedure for the determination of residues of Cyantraniliprole and Chlorantraniliprole in plasma and urine to a limit of quantification of 1.0 µg/L, using commercially available laboratory equipment and reagents.

Assessment and conclusion by applicant: Analytical method described in report 49234 has been demonstrated to be a reliable and accurate procedure for the determination of Cyantraniliprole or Chlorantraniliprole in plasma and urine to a limit of quantification of 1.0 µg/L, using commercially available laboratory equipment and reagents.

      - Description of Methods for the Analysis of Soil (KCP 5.2.4)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Water (KCP 5.2.5)

No new or additional studies have been submitted.

      - Description of Methods for the Analysis of Air (KCP 5.2.6)

No new or additional studies have been submitted.

      - Other Studies/ Information

No new or additional studies have been submitted.