FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: DTH-GEN 70 WG Product name(s): Danae 70 WG Chemical active substance: dithianon, 700 g/kg

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Pestila Spółka z ograniczoną odpowiedzialnością Submission date: May 2022 Finalisation date: September 2023, January 2024

Version history

| When | What |
| --- | --- |
| 09/2023 | zRMS evaluation |
| 01/2024 | The final Registration Report |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	6

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	6

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	6

5.2.2	Methods for the determination of residues (KCP 5.1.2)	6

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	6

5.3.1	Analysis of the plant protection product (KCP 5.2)	6

5.3.2	Description of analytical methods for the determination of residues of active substance dithianon (KCP 5.2)	7

5.3.2.1	Overview of residue definitions and levels for which compliance is required	7

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	7

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	7

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	7

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	7

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	7

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	7

5.3.2.8	Other studies/ information	7

Appendix 1	Lists of data considered in support of the evaluation	9

Appendix 2	Detailed evaluation of submitted analytical methods	11

A 2.1	Analytical methods for the active substance dithianon	11

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	11

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	11

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for determination of the active substances in the plant protection product DTH-GEN 70 WG. No data gaps.

The analytical method (HPLC-UV/Vis) for determination of dithianon in the formulation Danae 70 WG is validated and meets criteria of specificity, linearity and precision according to the requirements SANCO 3030/99 rev. 5, therefore the method is acceptable.

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Apple | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of active substance dithianon in plant protection product is provided as follows:

| Comments of zRMS: | The analytical method (HPLC-UV/Vis) for determination of dithianon in the formulation Danae 70 WG has been submitted and meets criteria of specificity, linearity and precision according to the requirements SANCO 3030/99 rev 5, therefore the method is acceptable. |
| --- | --- |

| Reference: | 5.1.1 |
| --- | --- |
| Report | DTH-GEN 70 WG. Method validation for determination of the active substance content, Oleska G., 2015, report No. BA-01/15 |
| Guideline(s): | Yes, SANCO/3030/99 rev.4. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The content of active substance in the examined sample was determined by high performance liquid chromatography (HPLC) using reversed phase column, UV/Vis detection (wavelength 230 nm) and external standard method.

Equipment and chromatographic conditions for dithianon analysis

- Apparatus

- Perkin Elmer liquid chromatograph equipped with UV/Vis detector, a termostated column oven, autosampler.

- HPLC column: Luna C18, 250 x 4.6 mm, dimension of grains 5μm

- Analytical balance with readability of 0.01 mg

- Reagents and materials

- Dithianon standard, 98.2 %, (Annex no 2)

- Deionized water (Millipore)

- Acetonitrile for HPLC (POCh)

- Chromatographic conditions

- Column temperature: 35 ºC

- Mobile phase: 50% of CH3CN : 50% of water

- Mobile phase flow rate: v = 1.0 mL/min

- Wavelength: λ = 230nm

- Injected volume: 10 μl

The preparation of standard and sample solutions

Standard solution

About 7 mg of the dithianon standard was weighed with accuracy of 0.01 mg into

10 mL volumetric flask. Than acetonitrile was added to the mark. Three standard

solutions were prepared.

Sample solution

About 10 mg of examined sample was weighed with accuracy of 0.01 mg into the

10 mL volumetric flask. Than acetonitrile was added to the mark. Five samples were

prepared to assess the repeatability.

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of active substance dithianon in plant protection product DTH-GEN 70 WG

|  | dithianon |
| --- | --- |
| Author(s), year | Oleska G., 2015 |
| Principle of method | SANCO/3030/99 rev.4 . HPLC – UV/Vis |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | The linearity of analytical method was assessed using five dithianon standard solutions with concentration range from 0.5018 mg/mL to 0.9005 mg/mL (from 70% to 130% of nominal dithianon content in the sample). Each standard solution was injected three times into the column. y= 17504855.8477x+4118044.9581 Correlation coefficient: R2 = 0.9959 Required: R2 ≥ 0.99 |
| Precision – Repeatability Mean n = 5 (%RSD) | Mean concentration 72.38% RSD = 0.68% Required: RSD ≤ 1.41% Hr = 0.48 < 1 |
| Accuracy n = 5 (% Recovery) | Marginal recovery: Level I: ~ 80% - 100.54 Level II: ~ 90% - 100.42 100.48% (range: 98.45% - 101.97%) Required: 98 - 102 % |
| Interference/ Specificity | The repeatability was expressed as relative standard deviation. Chromatograms of the acetonitrile, standard solution and examined sample were performed and superimpose. There was no inteference. |
| Comment | No comments. Accepted |

Conclusion

The HPLC method, used to quantify dithianon in DTH-GEN 70 WG was fully validated. Method validation included linearity, non-analyte interference, precision, accuracy and specificity. All measured parameters meet the criteria given in SANCO/3030/99 rev.4.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

### Methods for the determination of residues (KCP 5.1.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of active substance dithianon (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of methods for the analysis of soil (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of methods for the analysis of water (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of methods for the analysis of air (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

Not relevant for new registration according art. 34 of Reg. 1107/2009 based on data which protection period has expired.

#### Other studies/ information

Not relevant.

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1 | Oleska G. | 2015 | DTH-GEN 70 WG. Method validation for determination of the active substance content. Report No BA-01/15 Institute of Industrial organic Chemistry GLP Unpublished | N | Pestila* |

* Pestila spółka z ograniczoną odpowiedzialnością

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

The following tables are to be completed by MS

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for the active substance 1

    - Methods used for the generation of pre-authorization data (KCP 5.1)

No new or additional studies have been submitted

When a method used to generate pre-authorization data is identical to a post-registration method, please refer to the post-registration method.

Only studies that have not previously been evaluated within a peer reviewed process at EU level should be reported.

When new methods for the generation of pre-authorization data have been submitted, please describe these methods according to paragraph A 2.1.2.1.1. Sections that are not required for pre-authorization methods, such as ILV, can be deleted if appropriate.

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted

Only studies that have not been previously evaluated within a peer reviewed process at EU level should be reported.

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

Briefly describe the sample preparation: extraction, clean-up, derivatisation, determination (principle, detection mode, ionization technique, mode, if relevant, ion(s), if relevant, calibration type) and in which solvent the standards were prepared.

Results and discussions

Briefly discuss the results. Is an ILV required? Is a confirmatory method required? Method validation data can be summarised in Table A 1 and Table A 2.

Table A 1:	Recovery results from method validation of analyte using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |

Table A 2:	Characteristics for the analytical method used for validation of active substance residues in matrix

|  | Analyte 1 | Analyte 2 |
| --- | --- | --- |
| Specificity | mass spectrum is provided if MS is used blank value < 30 % LOQ) | mass spectrum is provided if MS is used blank value < 30 % LOQ) |
| Calibration (type, number of data points) | individual calibration data presented¦ calibration line equation presented number of data points | individual calibration data presented¦ calibration line equation presented number of data points |
| Calibration range | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) |
| Assessment of matrix effects is presented | yes/no | yes/no |
| Limit of determination/quantification | limit of quantification representing the lowest validated level with sufficient recovery and precision | limit of quantification representing the lowest validated level with sufficient recovery and precision |

Conclusion

Briefly conclude on the acceptability and validation of the method for enforcement of residues. Was the method sufficiently validated? Which analytes are covered by the analytical method? For which commodities or commodity groups was the analytical method validated?

          - Independent laboratory validation

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

The ILV method should be in line with the primary method. A full summary of materials and methods is therefore not required. Briefly describe deviations from the primary method, if any.

Results and discussions

Briefly discuss the results. Method validation data can be summarised in Table A 3 and Table A 4.

Table A 3:	Recovery results from independent laboratory validation of analyte using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |

Table A 4:	Characteristics for the analytical method used for independent laboratory validation of active substance residues in matrix

|  | Analyte 1 | Analyte 2 |
| --- | --- | --- |
| Specificity | mass spectrum is provided if MS is used blank value < 30 % LOQ) | mass spectrum is provided if MS is used blank value < 30 % LOQ) |
| Calibration (type, number of data points) | individual calibration data presented¦ calibration line equation presented number of data points | individual calibration data presented¦ calibration line equation presented number of data points |
| Calibration range | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) |
| Assessment of matrix effects is presented | yes/no | yes/no |
| Limit of determination/quantification | limit of quantification representing the lowest validated level with sufficient recovery and precision | limit of quantification representing the lowest validated level with sufficient recovery and precision |

Conclusion

Briefly conclude on the independent laboratory validation. Is the method acceptable as ILV for the primary method?

          - Confirmatory method (if required)

No confirmatory method is required

If no confirmatory method is required, state reason.

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

Briefly describe the sample preparation: extraction, clean-up, derivatisation, determination (principle, detection mode, ionization technique, mode, if relevant, ion(s), if relevant, calibration type) and in which solvent the standards were prepared.

Results and discussions

Briefly discuss the results. Method validation data can be summarised in Table A 5 and Table A 6.

Table A 5:	Recovery results from confirmatory method validation of analyte using the confirmatory analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |

Table A 6:	Characteristics for the confirmatory method used for validation of active substance residues in matrix

|  | Analyte 1 | Analyte 2 |
| --- | --- | --- |
| Specificity | mass spectrum is provided if MS is used blank value < 30 % LOQ) | mass spectrum is provided if MS is used blank value < 30 % LOQ) |
| Calibration (type, number of data points) | individual calibration data presented¦ calibration line equation presented number of data points | individual calibration data presented¦ calibration line equation presented number of data points |
| Calibration range | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) |
| Assessment of matrix effects is presented | yes/no | yes/no |
| Limit of determination/quantification | limit of quantification representing the lowest validated level with sufficient recovery and precision | limit of quantification representing the lowest validated level with sufficient recovery and precision |

Conclusion

Briefly conclude on the acceptability and validation of the method for enforcement of residues. Was the method sufficiently validated? Which analytes are covered by the analytical method? For which commodities or commodity groups was the analytical method validated?

          - Extraction efficiency

Please comment on the extraction procedures used in this residue analytical method (if applicable).

        - Analytical method 2

If several analytical methods need to be described, section (see A 2.1.2.1.1) should be repeated.

        - Extraction efficiency

When applicable, comment on the extraction efficiency of existing methods for the determination of residues in plant matrices.

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted

Only studies that have not been previously evaluated within a peer reviewed process at EU level should be reported.

        - Analytical method 1

          - Method validation

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

Briefly describe the sample preparation: extraction, clean-up, derivatisation, determination (principle, detection mode, ionization technique, mode, if relevant, ion(s), if relevant, calibration type) and in which solvent the standards were prepared.

Results and discussions

Briefly discuss the results. Method validation data can be summarised in Table A 7 and Table A 8.

Table A 7:	Recovery results from method validation of analyte using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |

Table A 8:	Characteristics for the analytical method used for validation of active substance residues in matrix

|  | Analyte 1 | Analyte 2 |
| --- | --- | --- |
| Specificity | mass spectrum is provided if MS is used blank value < 30 % LOQ) | mass spectrum is provided if MS is used blank value < 30 % LOQ) |
| Calibration (type, number of data points) | individual calibration data presented¦ calibration line equation presented number of data points | individual calibration data presented¦ calibration line equation presented number of data points |
| Calibration range | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) |
| Assessment of matrix effects is presented | yes/no | yes/no |
| Limit of determination/quantification | limit of quantification representing the lowest validated level with sufficient recovery and precision | limit of quantification representing the lowest validated level with sufficient recovery and precision |

Conclusion

Briefly conclude on the acceptability and validation of the method for enforcement of residues. Was the method sufficiently validated? Which analytes are covered by the analytical method? For which commodities or commodity groups was the analytical method validated?

          - Independent laboratory validation

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

The ILV method should be in line with the primary method. A full summary of materials and methods is therefore not required. Briefly describe deviations from the primary method, if any.

Results and discussions

Briefly discuss the results. Method validation data can be summarised in Table A 9 and Table A 10.

Table A 9:	Recovery results from independent laboratory validation of analyte using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of active substance residues in matrix

|  | Analyte 1 | Analyte 2 |
| --- | --- | --- |
| Specificity | mass spectrum is provided if MS is used blank value < 30 % LOQ) | mass spectrum is provided if MS is used blank value < 30 % LOQ) |
| Calibration (type, number of data points) | individual calibration data presented¦ calibration line equation presented number of data points | individual calibration data presented¦ calibration line equation presented number of data points |
| Calibration range | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) |
| Assessment of matrix effects is presented | yes/no | yes/no |
| Limit of determination/quantification | limit of quantification representing the lowest validated level with sufficient recovery and precision | limit of quantification representing the lowest validated level with sufficient recovery and precision |

Conclusion

Briefly conclude on the independent laboratory validation. Is the method acceptable as ILV for the primary method?

          - Confirmatory method (if required)

No confirmatory method is required

If no confirmatory method is required, state reason.

| Comments of zRMS: | Comment on study; acceptable or not; deficiencies, corrections, according to recent guidelines or not, used in evaluation or only as additional information |
| --- | --- |

| Reference: | Data point |
| --- | --- |
| Report | Title, author(s), year, report No, document No, Authority registration No |
| Guideline(s): | Yes/No (If yes, give guidelines; If no, give justification, e.g., “ no guidelines available” or “ methods used comparable to guideline(s) xxx” ) |
| Deviations: | Yes/No (If yes, describe deviations from test guidelines) |
| GLP: | Yes/No (If no, give justification, e.g., state that GLP was not compulsory at the time the study was performed) |
| Acceptability: | Yes/No/Supplementary |

Materials and methods

Briefly describe the sample preparation: extraction, clean-up, derivatisation, determination (principle, detection mode, ionization technique, mode, if relevant, ion(s), if relevant, calibration type) and in which solvent the standards were prepared.

Results and discussions

Briefly discuss the results. Method validation data can be summarised in Table A 11 and Table A 12.

Table A 11:	Recovery results from confirmatory method validation of analyte using the confirmatory analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = x) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |

Table A 12:	Characteristics for the confirmatory method used for validation of active substance residues in matrix

|  | Analyte 1 | Analyte 2 |
| --- | --- | --- |
| Specificity | mass spectrum is provided if MS is used blank value < 30 % LOQ) | mass spectrum is provided if MS is used blank value < 30 % LOQ) |
| Calibration (type, number of data points) | individual calibration data presented¦ calibration line equation presented number of data points | individual calibration data presented¦ calibration line equation presented number of data points |
| Calibration range | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) | Accepted calibration range in concentration units (e.g. in μg/ml or ng/μl) Corresponding calibration range in mass ratio units for the sample (e.g.in mg/kg or μg/L) |
| Assessment of matrix effects is presented | yes/no | yes/no |
| Limit of determination/quantification | limit of quantification representing the lowest validated level with sufficient recovery and precision | limit of quantification representing the lowest validated level with sufficient recovery and precision |

Conclusion

Briefly conclude on the acceptability and validation of the method for enforcement of residues. Was the method sufficiently validated? Which analytes are covered by the analytical method? For which commodities or commodity groups was the analytical method validated?

          - Extraction efficiency

Please comment on the extraction procedures used in this residue analytical method (if applicable).

        - Analytical method 2

If several analytical methods need to be described, section A 2.1.2.2.1 should be repeated.

          - Extraction efficiency

When applicable, comment on the extraction efficiency of existing methods for the determination of residues in animal matrices.

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted

If summaries for new/additional methods for the analysis of soil are required, the relevant sections of paragraph A 2.1.2.2.1 should be repeated.

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted

If summaries for new/additional methods for the analysis of water are required, the relevant sections of paragraph A 2.1.2.2.1 should be repeated.

Please note that an ILV for the analytical method for the determination of residues in drinking water is required according to regulation 1107/2009.

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted

If summaries for new/additional methods for the analysis of air are required, the relevant sections of paragraph A 2.1.2.2.1 should be repeated.

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted

If summaries for new/additional methods for the analysis of body fluids and tissues are required, the relevant sections of paragraph A 2.1.2.2.1 should be repeated.

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted

If summaries for other new/additional methods are required, the relevant sections of paragraph A 2.1.2.2.1 should be repeated.

  - Analytical methods for the active substance 2

If several active substances are included in the plant protection product section A 2.1 should be repeated for each active substance.