FINAL REGISTRATION REPORT Part B Section 8 Environmental Fate Detailed summary of the risk assessment

Product code: DTH-GEN 70 WG Product name(s): Danae 70 WG Chemical active substance: dithianon, 700 g/kg

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Pestila Spółka z ograniczoną odpowiedzialnością Submission date: May 2022, update: August 2023 MS Finalisation date: September 2023, January 2024

Version history

| When | What |
| --- | --- |
| 2023-08-31 | Update with PECgw calculations with the latest versions of the FOCUS-PELMO v6.6.4 and FOCUS - PEARL v5.5.5 |
| 09.2023 | Evaluated by ZRMS |
| 01.2024 | The final Registration Report |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	6

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	8

8.8.1	Justification for new endpoints	8

8.8.2	Dithianon and relevant metabolite- phthalic acid (KCP 9.2.4.1)	8

Considered in support of the evaluation	13

Intended uses

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use-No. (e) | Member state(s) | Crop and/ or situation (crop destination / purpose of crop) | F, Fn, Fpn G, Gn, Gpn or I | Pests or Group of pests controlled (additionally: developmental stages of the pest or pest group) | Application |  |  |  | Application rate |  |  | PHI (days) | Remarks: e.g. g safener/synergist per ha (f) | Conclusion PECgw |
|  |  |  |  |  | Method / Kind | Timing / Growth stage of crop & season | Max. number a) per use b) per crop/ season | Min. interval between applications (days) | kg or L product / ha a) max. rate per appl. b) max. total rate per crop/season | g or kg as/ha a) max. rate per appl. b) max. total rate per crop/season | Water L/ha min / max |  |  |  |
| Zonal uses (field or outdoor uses, certain types of protected crops) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PL | Apple | F | Apple scab (Venturia inaequalis) | Foliar spray | BBCH 53-79 Spring or BBCH 57-79 And BBCH 73-79 | a) 1 b) 6 or a) 1 b) 3 and 3 | Min.5 days Or Min. 7 days And min 5 days | a) 0.5 kg/ha b) 3 kg/ha or a) 0,75 kg/ha and 0,5kg/ha b) 3,75 kg/ha | a) 350 g as/ha b) 2100 g as/ha or a) 525 g as/ha and 350 g as/ha b) 2625 g as/ha | 200-1000 L/ha | 42 days | - | A |
| Intended uses – minor crops (art. 51) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | PL | Pear | F | Pear scab (Venturia inaequalis) | Foliar spray | BBCH 53-79 Spring or BBCH 57-79 And BBCH 73-79 | a) 1 b) 6 or a) 1 b) 3 and 3 | Min.5 days Or Min. 7 days And min 5 days | a) 0.5 kg/ha b) 3 kg/ha or a) 0,75 kg/ha and 0,5kg/ha b) 3,75 kg/ha | a) 350 g as/ha b) 2100 g as/ha or a) 525 g as/ha and 350 g as/ha b) 2625 g as/ha | 200-1000 L/ha | 42 days | - | A |

| Remarks table heading: | (a) e.g. wettable powder (WP), emulsifiable concentrate (EC), granule (GR) (b) Catalogue of pesticide formulation types and international coding system CropLife International Technical Monograph n°2, 6th Edition Revised May 2008 (c) g/kg or g/l |  | (d) Select relevant (e) Use number(s) in accordance with the list of all intended GAPs in Part B, Section 0 should be given in column 1 (f) No authorization possible for uses where the line is highlighted in grey, Use should be crossed out when the notifier no longer supports this use. |
| --- | --- | --- | --- |
| Remarks columns: | 1 Numeration necessary to allow references 2 Use official codes/nomenclatures of EU Member States 3 For crops, the EU and Codex classifications (both) should be used; when relevant, the use situation should be described (e.g. fumigation of a structure) 4 F: professional field use, Fn: non-professional field use, Fpn: professional and non-professional field use, G: professional greenhouse use, Gn: non-professional greenhouse use, Gpn: professional and non-professional greenhouse use, I: indoor application 5 Scientific names and EPPO-Codes of target pests/diseases/ weeds or, when relevant, the common names of the pest groups (e.g. biting and sucking insects, soil born insects, foliar fungi, weeds) and the developmental stages of the pests and pest groups at the moment of application must be named. 6 Method, e.g. high volume spraying, low volume spraying, spreading, dusting, drench Kind, e.g. overall, broadcast, aerial spraying, row, individual plant, between the plants - type of equipment used must be indicated. |  | 7 Growth stage at first and last treatment (BBCH Monograph, Growth Stages of Plants, 1997, Blackwell, ISBN 3-8263-3152-4), including where relevant, information on season at time of application 8 The maximum number of application possible under practical conditions of use must be provided. 9 Minimum interval (in days) between applications of the same product 10 For specific uses other specifications might be possible, e.g.: g/m³ in case of fumigation of empty rooms. See also EPPO-Guideline PP 1/239 Dose expression for plant protection products. 11 The dimension (g, kg) must be clearly specified. (Maximum) dose of a.s. per treatment (usually g, kg or L product / ha). 12 If water volume range depends on application equipments (e.g. ULVA or LVA) it should be mentioned under “application: method/kind”. 13 PHI - minimum pre-harvest interval 14 Remarks may include: Extent of use/economic importance/restrictions |

Table of Contents

8	Fate and behaviour in the environment (KCP 9)	4

8.8	Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)	5

8.8.1	Justification for new endpoints	5

8.8.2	Dithianon and relevant metabolite- phthalic acid (KCP 9.2.4.1)	5

# Fate and behaviour in the environment (KCP 9)

This is the application for registration of a plant protection product under working name DTH-GEN 70 WG according to Article 33 based on Article 34 of Regulation 1107/2009. DTH-GEN 70 WG is a water dispersible granules formulation (WG), containing 700 g/kg of dithianon to be used as a fungicide to protect apple. This is a core dossier in order to allow the approval of product DTH-GEN 70 WG in Poland (zRMS).

The reference product to the product under the code DTH-GEN 70 WG is Delan 700 WG, that has been first registered in Poland according to authorisation Minister of Agricultural and Rural Development No. R-46/2006 of 28.12.2006 which means, that data protection, for the data presented by Delan 700 WG authorisation holder, for purposes of registration, has been expired.

In respect to the above and taking into account Polish requirements for the applications for registration of a plant protection products according to Article 33 based on Article 34 of Regulation 1107/2009 applicant do not provide fate and behaviour in the environment data and apply for using unprotected data of Delan 700 WG.

ZRMS Final Conclusion:

All data referred to points in fate section for Delan 70 WG are in line EFSA conclusion (EFSA Peer Review, EFSA Journal 2010;8 (11) and were accepted by evaluator.

Applicant did not submit this section in the required format. Therefore, the zRMS assessment was carried out in the form of comments for PECs, PECgw, PECsw and a general comment on individual data points.

ZRMS Comments:

The proposed GAP of DTH-GEN 70 WG is covered by GAP used in Plant Protected Product Delan 700 WG and was accepted by zRMS for assessment of fate and behaviour.

GAP for Poland

| Crops in GAP | Covering the uses of pome fruits in the context of a risk envelope approach in a worst case scenario |
| --- | --- |
| FOCUSgw crops | Pome |
| Crop growth stage [BBCH code] | 53-79 |
| Maximum number of applications | 6 |
| Application interval [d] | 5 |
| Application rate [g a.s. ha-1] | 3 x 525 + 3 x 350 |
| Crop Interception [%] | 50 (worst case assumption for trees without leaves) |
| Maximum Soil load [g a.s. ha-1] | 3 x 262.5 + 3 x 175 |
| Maximum total yearly soil load [g a.s. ha-1] | 1312.5 |

ZRMS Comments:

PECs

Calculations for ditianon and its metabolite were carried out with simple program.

For the PECsoil calculations, the worst-case single first-order (SFO) DT50 value from the non-normalised laboratory studies of 37.6 days (20°C and 45% MWHC) was used for dithianon.

The metabolite phthalic acid was  formed from dithianon during soil photolysis with a maximum

observed level of 16% AR at day 15d. The conservative DT50 value of 1  was used to calculation.

No other metabolites occurred in soil in relevant amounts.

EU agreed endpoints (SANCO/10349/2011 final – 11/March/2011 / EFSA Peer Review, EFSA Journal 2010;8 (11): 1904) were used to calculation of PECs.

PECs The maximum PECsoil following six fold application of BAS 216 03 F in the context of EU registration was estimated to 1.368 mg kg-1 for dithianon. PEC21d twa for dithianon was calculated to 1.192 mg kg-1

The maximum, initial PECsoil of phthalic acid was calculated to 0.032 mg kg-1. The predicted concentrations in soil are appropriate to be used for the subsequent risk assessment for soil organisms.

PEC21d twa of phthalic acid was calculated to 0.012 mg kg-1 .

The predicted concentrations in soil are appropriate to be used for the subsequent risk assessment for soil organisms.

ZRMS Comments:

PECgw

The leaching assessment for dithianon and its metabolite – phthalic acid was conducted according to the guidance of the FOCUS groundwater scenarios workgroup [FOCUS (2009): Assessing Potential for Movement of Active Substances and their Metabolites to Groundwater in the EU Final Report of the Groundwater Workgroup of FOCUS, amending FOCUS (2000), EC Document Reference

Sanco/13144/2010, 604 pp; FOCUS (2011): Generic Guidance for Tier 1 FOCUS Ground Water Assessments, version 2.0].

Endpoints used for the PECgw calculations of dithianon and its metabolite were according SANCO/10349/2011 final – 11/March/2011 /  EFSA Peer Review, EFSA Journal 2010;8 (11): 1904.

The models FOCUS-PEARL 4.4.4 and FOCUS-MACRO 4.4.2 were used, and all available scenarios applicable for pome were considered. No model FOCUS-PELMO was used.

However, according harmonization guidance for Poland, posted on the website of the Ministry of Agriculture and Rural Development, necessary is to carry out calculations with the latest versions of the FOCUS-PELMO v6.6.4 and FOCUS - PEARL v5.5.5, models by using the EU agreed input parameters and application dates as suggested by App Date 3.06. Relevant crop interception should be used in line with FOCUS groundwater guidance (2014).

The new calculations of PECgw were performed by Applicant by last version of models.

The 80th percentiles of the predicted annual average leachate concentrations of dithianon and its

metabolite phthalic acid were below 0.001 µg L-1 in all calculated scenarios considering a worst case application pattern that cover the GAP uses in pome and pear fruit.

## Predicted Environmental Concentrations in groundwater (PECgw) (KCP 9.2.4)

### Justification for new endpoints

Not relevant.

### Dithianon and relevant metabolite- phthalic acid (KCP 9.2.4.1)

The calculations for PEC in groundwater for dithianon and its metabolite – phthalic acid were based on the guidance of FOCUS groundwater (2014). The models FOCUS-PEARL 5.5.5 and FOCUS-PELMO 6.6.4 were used, and all available scenarios applicable for pome / stone fruits were considered.

Table 8.8-1:	Input parameters related to application for PECgw calculations

| Use No. | 1,2 |
| --- | --- |
| Crop | Apples* (BBCH 53) |
| Application rate (g as/ha) | 3 x 525 and 3x 350 (worst case) 3 x 47.1 and 3 x 31.4 (metabolite)** |
| Number of applications/interval (d) | 6/5 |
| Relative application date | - |
| Crop interception (%) | 60% |
| Frequency of application | annual |
| Models used for calculation | FOCUS PEARL, v 5.5.5 FOCUS PELMO v6.6.4 |

* Covering the uses of pome and stone fruits in the GAP in the context of a risk envelope approach

** PECgw values of the metabolite phthalic acid were calculated separately from the parent with an application rate corrected by molar mass and maximum occurrence in soil since the metabolite is formed only due to photolysis at the soil surface.

Table 8.8-2:	Application dates used for groundwater risk assessment

| Crop | Scenario | Application dates (absolute) |
| --- | --- | --- |
| Apple | Châteaudun | 06/05, 11/05,16/05, 21/05, 26/05, 31/05 |
|  | Hamburg | 30/05, 04/06, 09/06, 14/06, 19/06, 24/06 |
|  | Jokioinen | 19/05, 24/05, 29/05, 03/06, 08/06, 13/06 |
|  | Kremsmünster | 30/05, 04/06, 09/06, 14/06, 19/06, 24/06 |
|  | Okehampton | 12/05, 17/05, 22/05, 27/05, 01/06, 06/06 |
|  | Piacenza | 06/05, 11/05,16/05, 21/05, 26/05, 31/05 |
|  | Porto | 16/05, 21/05, 26/05, 31/05, 05/06, 11/06 |
|  | Sevilla | 29/04, 04/05, 09/05, 14/05, 19/05, 24/05 |
|  | Thiva | 16/05, 21/05, 26/05, 31/05, 05/06, 11/06 |

Table 8.8-3:	Input parameters related to active substance dithianon and metabolite for PECgw calculations

| Compound | Dithianon | Phthalic acid | Value in accordance with EU endpoint y/n/ Reference* |
| --- | --- | --- | --- |
| Molecular weight (g/mol) | 296.3 | 166.14 | EFSA Journal 2010;8 (11): 1904 |
| Water solubility (g/mol): | 0.3754 | 1000 (default) | EFSA Journal 2010;8 (11): 1904 |
| Saturated vapour pressure (Pa): | <10-10 | 0 | EFSA Journal 2010;8 (11): 1904 |
| DT50 in soil (d) | 10.5 (geomean, laboratory studies, normalisation to 10 kPa or pF2, 20 °C with Q10 of 2.58, n =6) | 1 (Conservative value are shorter than 1 day (n=3)) | EFSA Journal 2010;8 (11): 1904 |
| Kfoc (mL/g)/Kfom | 3179/1844 (geomean, n=5 | 11/6.4 (geomean, n=5) | EFSA Journal 2010;8 (11): 1904 |
| 1/n | 0.9 (aritmethic mean, n=5) | 0.921 (aritmethic mean, n=5) | EFSA Journal 2010;8 (11): 1904 |
| Plant uptake factor | 0 | 0 | default |
| Molar formation fraction |  | 0.56 | - |
| Formation fraction | - | -* | - |
| Maxiumum occurance in soil (%) | - | 16 | EFSA Journal 2010;8 (11): 1904 |

* not used; PECgw calculated with a “pseudo-application” of the metabolite, i.e. with a corrected application rate

Calculations for the metabolite phthalic acid were performed separately with a “pseudo application” of the metabolite since the metabolite was observed only under irradiated conditions of a soil photolysis study. For this purpose, the application rate of the parent was corrected for the molar mass and the maximum occurrence of the metabolite in soil. No dithianon soil metabolites were observed in aerobic laboratory soils above 5% of the total applied radioactivity (TAR). However, in a soil photolysis study the metabolite phthalic acid was observed to form at a maximum of 16% TAR following 15 days of continuous irradiation. As a worst case assumption for loading of phthalic acid to soil in the PECgw simulations, phthalic acid was assumed to instantaneously form at the maximum observed level of 16% after each dithianon application.

Table 8.8-4:	PECgw for dithianon on apples (with FOCUS PEARL 5.5.5/PELMO 6.6.4)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |
| --- | --- | --- |
|  |  | dithianon |
| Apples | Châteaudun | <0.001 |
|  | Hamburg | <0.001 |
|  | Jokioinen | <0.001 |
|  | Kremsmünster | <0.001 |
|  | Okehampton | <0.001 |
|  | Piacenza | <0.001 |
|  | Porto | <0.001 |
|  | Sevilla | <0.001 |
|  | Thiva | <0.001 |
| FOCUS Pearl |  |  |
| Apples | Scenario | dithianon |
|  | Châteaudun | <0.001 |
|  | Hamburg | <0.001 |
|  | Jokioinen | <0.001 |
|  | Kremsmünster | <0.001 |
|  | Okehampton | <0.001 |
|  | Piacenza | <0.001 |
|  | Porto | <0.001 |
|  | Sevilla | <0.001 |
|  | Thiva | <0.001 |

Table 8.8-5:	PECgw for phthalic on apples (with FOCUS PEARL 5.5.5/PELMO 6.6.4)

| Crop | Scenario | 80th Percentile PECgw at 1 m Soil Depth (g/L) |
| --- | --- | --- |
|  |  | phthalic acid |
| Apples | Châteaudun | <0.001 |
|  | Hamburg | <0.001 |
|  | Jokioinen | <0.001 |
|  | Kremsmünster | <0.001 |
|  | Okehampton | <0.001 |
|  | Piacenza | <0.001 |
|  | Porto | <0.001 |
|  | Sevilla | <0.001 |
|  | Thiva | <0.001 |
| FOCUS Pearl |  |  |
| Apples | Scenario | phthalic acid |
|  | Châteaudun | <0.001 |
|  | Hamburg | <0.001 |
|  | Jokioinen | <0.001 |
|  | Kremsmünster | <0.001 |
|  | Okehampton | <0.001 |
|  | Piacenza | <0.001 |
|  | Porto | <0.001 |
|  | Sevilla | <0.001 |
|  | Thiva | <0.001 |

Conclusions:

The 80th percentiles of the predicted annual average leachate concentrations of dithianon and its metabolite phthalic acid were below 0.001 µg L-1 in all calculated scenarios considering a worst case application pattern that covers the GAP uses in pome and stone fruits.

ZRMS Comments:

PECsw

The intended uses in GAP for the formulation Product Delan 700 WG are within those considered acceptable for registration of Danae 70 WG.

The input parameters used in PECsw calculations for Delan 700 WG the same as those utilised in the EFSA conclusion and are therefore considered acceptable for Danae 70 WG.

This risk envelope approach provides a maximum PECsw that covers a range of use of formulation Danae 70 WG in  Poland.

These values are considered appropriate to be used for subsequent risk assessments of aquatic organisms.

No further consideration of PECsw/sed  is required.

- Considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 9.2/01 | Hara-Skrzypiec A. | 2023 | Danae 70 WG- calculation of Predicted Environmental Concentrations of dithianon and its metabolite in ground water using the PEARL 5.5.5 and PELMO 6.6.4 Groundwater Models. Company Report No: EST/16/2023 Source: ESTICON Sp. z o.o., Poland non GLP unpublished | N | Pestila * |