REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: SHA 6500 B Product name: ESPECIALLY Chemical active substance: Esfenvalerate, 50 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: XXXX Submission date: January 2024 Evaluation date: November 2024 MS Finalisation date: March 2025

Version history

| When | What |
| --- | --- |
| November 2024 | Initial RR |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	7

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	9

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	9

5.2.2	Methods for the determination of residues (KCP 5.1.2)	9

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	9

5.3.1	Analysis of the plant protection product (KCP 5.2)	9

5.3.2	Description of analytical methods for the determination of residues of Esfenvalerate (KCP 5.2)	9

5.3.2.1	Overview of residue definitions and levels for which compliance is required	9

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	10

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	13

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	17

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	18

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	19

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	19

5.3.2.8	Other studies/ information	20

Appendix 1	Lists of data considered in support of the evaluation	21

Appendix 2	Detailed evaluation of submitted analytical methods	25

A 2.1	Analytical methods for Esfenvalerate	25

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	25

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	25

# Analytical methods

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are not available for the active substance(s) and relevant impurities in the plant protection product.

Noticed data gaps are:

data gap 1

data gap 2

data gap 3

In the context of the authorisation request sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

The applicant's dRR was not rewritten. In the resulting RR all comments /corrections/ add-ons were placed on the grey background.

However, in the context of 4 ILVs presented by the applicant and not accepted as ILVs by zRMS it is important to note that for ILV procedure reporting and justification of any additions or modifications to the original method is required and crucial for the assessment of the method (SANTE/2020/12830, Rev.2). Thus, to provide such possibilities, the given laboratory should be informed about the method that it will independently validate. Therefore, another validation of the same matrix, from another laboratory, cannot simply be chosen as ILV.

The validation study 21/FSL/09/V, including full validation of the method applied in the residue studies used in the present B7 was provided and presented in Appendix 2 (Other studies).

Thus, although within the present section ILVs for cabbage, wheat and strawberry are missing, the fully validated in high water, acid and dry matrices methods are available in RAR 2013. Therefore, in the context of the authorisation request no data gaps were noticed.

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| High starch content (dry) | Supported |
| High water content | Supported |
| High acid content | Supported |

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in plant protection product is provided as follows:

| Comments of zRMS: | Accepted |
| --- | --- |

| Reference: | KCP 5.1.1 |
| --- | --- |
| Report | Accelerated storage stability test by heating at elevated temperature of Esfenvalerate 50 g/L EW. V.S. Raksha, 2020, Report No. G13895 |
| Guideline(s): | SANCO/3030/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The analytical method for the determination of active ingredient content in test item was validated by establishing specificity, linearity, range, LOD, LOQ, precision and accuracy as detailed below.

Specificity

Preparation of Blank formulation solution: Accurately 0.1042 g of blank formulation (provided by the sponsor) was weighed into a 100mL volumetric flask, dissolved in about 30 mL of acetonitrile by sonicating for 2 min. After equilibrating to room temperature, the solution was made up to the mark with the same and shaken thoroughly. The solution was filtered through 0.45µm syringe filters.

The specificity of the method for active ingredient was studied by injecting individual solutions of working standard solution (DLC-3), test item (PR-1) solution, blank formulation solution and analytical blank (acetonitrile) to the HPLC operated under the conditions of the method being validated. The interferences at the retention time of a.i in blank injection were studied.

Linearity

Preparation of stock solution of Esfenvalerate standard: Accurately weighed 0.00509 g of the Esfenvalerate reference standard (purity 98.5 %) into 10 mL volumetric flask, dissolved in about 5 mL of acetonitrile and the solution was made up to the mark with the same. Shaken well for homogeneity. The concentration of stock solution of Esfenvalerate standard prepared was 501.37 µg/mL.

For detector linearity check, five working standard solutions of different concentrations of the active ingredient were prepared from the stock solution of Esfenvalerate with acetonitrile and injected in triplicate to High Performance Liquid Chromatograph (HPLC)

The sequence of serial dilution is as tabulated.

| Concentration of stock solution (µg/mL) | Aliquot Volume taken (mL) | Final made up (mL) | Concentration of Standard solution prepared (µg/mL) | Solution ID |
| --- | --- | --- | --- | --- |
| 501.37 | 0.2 | 10 | 10.03 | DLC1 |
| 501.37 | 0.5 | 10 | 25.07 | DLC2 |
| 501.37 | 1.0 | 10 | 50.14 | DLC3 |
| 501.37 | 1.5 | 10 | 75.21 | DLC4 |
| 501.37 | 2.0 | 10 | 100.27 | DLC5 |

A graph of instrument response (peak area) versus concentration was plotted (concentration on X-axis and peak area on Y-axis) and linear regression coefficient (r) was established.

Precision

Preparation of working standard solution

Accurately weighed 0.00500 g of Esfenvalerate reference standard into 10 mL volumetric flask, dissolved the contents in about 5 mL of acetonitrile and the solution was made up to the mark with the same. Shaken thoroughly. The concentration of stock solution of Esfenvalerate standard prepared was 500.0 µg/mL. Further 1.0 mL of this stock solution was pipetted out into a 10mL volumetric flask, dissolved and made up to the mark with acetonitrile. Shaken well for homogeneity. This solution was used as working standard solution (50.0 µg/mL) for precision.

Preparation of Test Item solutions

About 0.1 g of the test item, in five replications was taken into separate 100 mL volumetric flasks, dissolved in about 30 mL of acetonitrile and sonicated for 2 minutes. After equilibrating to room temperature, these solutions were made up to the mark with the same and shaken well for homogeneity. Each of these solutions were filtered through 0.45µm syringe filters.

Accuracy

Preparation of Blank formulation solution (Control sample)

Accurately 0.0500 g of blank formulation (provided by the sponsor) was weighed into a 50mL volumetric flask, dissolved the contents in about 30 mL of acetonitrile by sonicating for 2 min. After equilibrating to room temperature, the solution was made up to the mark with the same and shaken thoroughly. The solution was filtered through 0.45µm syringe filters. This solution was also analyzed for Esfenvalerate for comparison.

Preparation of stock solution of Esfenvalerate standard (for working standard solution and fortification)

Accurately 0.0200 g of Esfenvalerate standard was weighed into 20 mL volumetric flask, dissolved the contents in about 10 mL of acetonitrile and the solution was made up to the mark with the same. Shaken thoroughly for homogeneity. The concentration of prepared stock solution is 1000.0 µg/mL.

Preparation of working standard solution

Pipetted out 1.0 mL of Esfenvalerate standard stock solution into a 20 mL volumetric flask, dissolved and the solution was made up to the mark with acetonitrile. Shaken thoroughly. This solution was used as working standard solution for accuracy test.

Fortification

About 0.05 g blank matrix was weighed in triplicate at each of two fortification levels into 50 mL volumetric flasks. To these volumetric flasks, 2.0 mL of stock solution of Esfenvalerate standard was added at T1 (lower fortification level) and 3.2 mL of stock solution of Esfenvalerate standard was added at T2 (higher fortification level). Fortification was done in triplicate at each of two fortification levels. Dissolved the contents in about 20 mL of acetonitrile and sonicated for 2 minutes. After equilibrating to room temperature, the volume was made up to the mark with the same. Shaken well for homogeneity. These fortified samples were filtered through 0.45µm syringe filters and analyzed for a.i. content using HPLC-DAD.

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of Esfenvalerate in plant protection product Esfenvalerate 5% EW

|  | Esfenvalerate |
| --- | --- |
| Author(s), year | V.S. Raksha, 2020 |
| Principle of method | HPLC-DAD |
| Linearity (linear between mg/L / % range of the declared content) (correlation coefficient, expressed as r) | 5 points 10.03 – 100.27 μg/mL y=3130719.930x-1394491.444 R=1.000 |
| Precision – Repeatability Mean n = 5 (%RSD) | Mean a.i. content c=5.20 % w/w %RSD = 0.58 %RSDR = 3.12 %RSDr = 2.09 Hr = 0.278 ≤ 1 |
| Accuracy n = 6 (% Recovery) | Overall mean recovery: 99.98 ± 0.21% |
| Interference/ Specificity | No interference, the method is specific |
| LOQ | LOQ = 2.688 μg/mL |
| Comment | - |

Conclusion

According to SANCO/3030/99 rev. 5 the method was successfully validated and is suitable to determine content of Esfenvalerate in product ESPECIALLY (SHA 6500 B).

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of relevant impurities in plant protection product is provided as follows:

| Comments of zRMS: | Accepted |
| --- | --- |

| Reference: | KCP 5.1.1-2 |
| --- | --- |
| Report | Esfenvalerate 50 g/L EW Analysis of toluene content (relevant impurity) and relative density of initial preparation and preparation after accelerated storage procedure (CIPAC MT 46.3). E. Nowakowska-Bogdan, 2021, Report No. 35/2021 |
| Guideline(s): | SANCO/3030/99 rev. 5 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Sample solution preparation

Amount of about 11 mg of initial formulation were diluted to 10 mL with acetone. Five independent test item solutions prepared from five individual weights of the test item were analyzed 2 times under the same chromatographic conditions as used for calibration.

Amount of about 11 mg formulation after accelerated storage were diluted to 10 mL with acetone. Two independent test item solutions prepared from two individual weights of the test item were analyzed 2 times under the same chromatographic conditions as used for calibration.

All sample solutions were analyzed by GC/MS under stable chromatographic conditions.

Chromatographic conditions for working and fortification standard solutions:

- Column: HP-5MS; 30m x 0.25μm x 0.25mm;

- Carrier gas: Helium;

- Column flow: 1 mL/min

- Thermal program: formulation: 50°C for 2min. 30°C/min to 280°C isotherm of 20 minutes;

- Thermal program: calibration: 50°C for 2min. 30°C/min to 230°C isotherm of 0 minutes;

- Injection volume: 1 μL;

- Inlet temperature: 280°C;

- Injection mode: split;

- detector MSD SIM mode; mass monitored: m/z 91. 92 and 65

- retention time 2.709– 2.717 minutes

- Mass spectrometric conditions:

- MS system: 7000 Quadrupole MS/MS EI Agilent Technologies

- Source type: Electron Impact (EI);

- Polarity: Positive ion mode

- Transfer line temperature: 280°C

- Source temperature: 280°C

- Quadrupole temperature: 180°C

SPECIFICITY

Confirmation of analyte identification has been done. For this purpose GC-MS measurements with a 3 ions have been registered.

Full scan mass spectrum of ion spectrum and used ions or transitions was reported.

PRECISION (REPEATABILITY)

For sample repeatability evaluation, five independent test item solutions prepared from five individual weights of the test item (initial sample) were analyzed 2 times using the same chromatographic conditions as during the calibration process.

ACCURACY

Accuracy estimation was made on the initial formulation using a Standard Addition Method (Spiking). Therefore. three solutions with concentration of RI (analytical standard) in formulation matrix in analyte range of 90%, 110 % were prepared.

Each solution was injected 7 times using the same chromatographic conditions as during the test item measurements and calibration process.

LINEARITY AND RANGE

For Toluene calibration 5 standard solutions were prepared in the concentration range from 80% - 120% of analyte concentration level detected in sample. Each calibration solution was analysed twice under stable chromatographic conditions.

For LOQ evaluation. the lowest tested concentration (0.062 mg/L). five independent standard solutions) was injected 2 times.

Validation - Results and discussions

Table 5.2-2:	Methods suitable for the determination of the relevant impurities in plant protection product (PPP) ESPECIALLY/SHA 6500 B

|  | Toluene |
| --- | --- |
| Author(s), year | E. Nowakowska-Bogdan |
| Principle of method | GC-MS |
| Linearity (linear between mg/L) (correlation coefficient, expressed as r) | 5 points 0.062 – 0.093 mg/L y=6925.2x+105.3 R = 0.9989 |
| Precision – Repeatability Mean n = 10 (%RSD) | %RSD = 2.41 %RSDR=8.51 %RSDr=5.70 Hr=0.42≤1 |
| Accuracy n = 7 (% Recovery) | 1st level 90%: 108.70% 2nd level 110%: 105.24% |
| Interference/ Specificity | No interferences. The method is specific |
| LOQ | 0.062 mg/L |
| Comment | - |

Conclusion

According to SANCO/3030/99 rev. 5 the method was successfully validated and is suitable for determination of relevant impurity toluene in test item Esfenvalerate 5% EW (SHA 6500 B).

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

Not relevant.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

A CIPAC method No. 481 is available for Esfenvalerate.

### Methods for the determination of residues (KCP 5.1.2)

Please refer to post-registration methods.

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

### Analysis of the plant protection product (KCP 5.2)

Analytical methods for the determination of the active substance and relevant impurities in the plant protection product shall be submitted, unless the applicant shows that these methods already submitted in accordance with the requirements set out in point 5.2.1 can be applied.

### Description of analytical methods for the determination of residues of Esfenvalerate (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-1:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high water content | Fenvalerate (any ratio of constituent isomers (RR, SS, RS & SR) including esfenvalerate) | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Plant, high acid content |  | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Plant, high protein/high starch content (dry commodities) |  | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Plant, high oil content |  | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Plant, difficult matrices (hops, spices, tea) |  | 0.1 mg/kg | Regulation (EU) No. 2015/399 |
| Muscle | Fenvalerate (any ratio of constituent isomers (RR, SS, RS & SR) including esfenvalerate) | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Milk |  | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Eggs |  | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Fat |  | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Liver, kidney | Fenvalerate (any ratio of constituent isomers (RR, SS, RS & SR) including Esfenvalerate) and CPIA (chlorophenyl isovaleric acid), expressed as Fenvalerate | 0.02 mg/kg | Regulation (EU) No. 2015/399 |
| Soil (Ecotoxicology) | Esfenvalerate | 0.05 mg/kg | common limit |
| Drinking water (Human toxicology) | Esfenvalerate | 0.1 µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | Esfenvalerate | 0.001 µg/L | Lowest NOEC for fish (Oncorhynchus mykiss) |
| Air | Esfenvalerate | 3.3 µg/m3 | AOEL sys: 0.011 mg/kg bw/d |
| Tissue (meat or liver) | Esfenvalerate | 0.1 mg/kg | classified as T / T+ |
| Body fluids |  | 0.05 mg/L | classified as T / T+ |

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in plant matrices is given in the following tables.

Table 5.3-2:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: Fenvalerate (any ratio of constituent isomers (RR, SS, RS & SR) including Esfenvalerate) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High water content | Primary | 0.01 mg/kg | GC-ECD | Weeren, R.D. and Pelz, S. (2000), Amendment: Rzepka, S. (2007); Anding, C. (1998) Multi-residue method DFG S19 Report No.: SUM-0005V; Report No.: EAS/98-058 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Mewis, A. (2012) Report No.: SUM-0611V (LLA-0117) RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Mewis, A. (2012) Report No.: SUM-0611V (LLA-0117) RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS/MS | KCP 5.2.1 E. Nowakowska-Bogdan, 2020 Report No. 148/2019 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.2 D. C. Brown, 2012 Report No. S12-04136 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS/MS | KCP 5.2.1 E. Nowakowska-Bogdan, 2020 Report No. 148/2019 |
| High acid content | Primary | 0.01 mg/kg | GC-ECD | Rzepka, S. (2009) Multi-residue method DFG S19 Report No.: LLA-0108 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | ILV | - | - | Since analytical methods for crops with high water and high acid are the same, this ILV is also applicable for high acid crops. (RAR, UK, 2013) |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS | Rzepka, S. (2009) Multi-residue method DFG S19 Report No.: LLA-0108 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS/MS | KCP 5.2.1 E. Nowakowska-Bogdan, 2020 Report No. 148/2019 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.3 M. Rubino, 2018 Report No. 18.625318.0001 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS/MS | KCP 5.2.1 E. Nowakowska-Bogdan, 2020 Report No. 148/2019 |
| High oil content | Primary | 0.01 mg/kg | GC-ECD | Weeren, D.D., Pelz, S., (1998c) Multi-residue method DFG S19 Report No.: SUM-9801V RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Mewis, A. (2012) Report No.: SUM-0611V (LLA-0117) RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Mewis, A. (2012) Report No.: SUM-0611V (LLA-0117) RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS/MS | KCP 5.2.4 M. Rubino, 2018 Report No. 18.625318.0002 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.5 F. Lopez Benet, 2010 Report No. 130-09 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS/MS | KCP 5.2.4 M. Rubino, 2018 Report No. 18.625318.0002 |
| High protein/high starch content (dry) | Primary | 0.01 mg/kg | GC-ECD | Weeren, R.D. and Pelz, S. (1998a, 1998b) Multi-residue method DFG S19 Report No.: SUM-9802V; SUM-9803V RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Mewis, A. (2012) Report No.: SUM-0611V (LLA-0117) RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Mewis, A. (2012) Report No.: SUM-0611V (LLA-0117) RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS/MS | KCP 5.2.6 J. Giner, 2018, Report No. 61017R-WW.DP.S1 Analytical part: A. Augustynek, 2018 Report No. DPL/34/2017 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.7 M. Rubino, 2018 Report No. 18.625318.0003 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS/MS | KCP 5.2.6 J. Giner, 2018, Report No. 61017R-WW.DP.S1 Analytical part: A. Augustynek, 2018 Report No. DPL/34/2017 |

Table 5.3-3:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | - |
| Not required, because: | “Extraction efficiency of the method were not tested, however it appears that the same general extraction procedures used in the metabolism studies were applied in the enforcement method. In the metabolism studies reported recoveries of Fenvalerate ranged between 80-97%.” (RAR Esfenvalerate, 2013) |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in animal matrices is given in the following tables.

Table 5.3-4:	Validated methods for food and feed of animal origin (if appropriate)

| Component of residue definition: Fenvalerate (any ratio of constituent isomers (RR, SS, RS & SR) including Esfenvalerate) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Milk | Primary | 0.01 mg/kg | GC-ECD | Croucher, A. (1998) Report No.: LLA-0084 DAR, addendum EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS | KCP 5.2.8 M. Rubino, 2018 Report No. 18.625318.0005 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.9 J. Kicińska, 2018 Report No. ZBBZ-2016/37/DPL/5 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS | KCP 5.2.8 M. Rubino, 2018 Report No. 18.625318.0005 |
| Eggs | Primary | 0.01 mg/kg | GC-ECD | Croucher, A. (1998) Report No.: LLA-0083 DAR, addendum EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS | KCP 5.2.10 M. Rubino, 2018 Report No. 18.625318.0006 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.11 J. Kicińska, 2018 Report No. ZBBZ-2016/37/DPL/4 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS | KCP 5.2.10 M. Rubino, 2018 Report No. 18.625318.0006 |
| Muscle | Primary | 0.01 mg/kg | GC-ECD | Croucher, A. (1998) Report No.: LLA-0084; LLA-0083 DAR, addendum EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS | KCP 5.2.12 M. Rubino, 2018 Report No. 18.625318.0007 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.13 J. Kicińska, 2018 Report No. ZBBZ-2016/37/DPL/2 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS | KCP 5.2.12 M. Rubino, 2018 Report No. 18.625318.0007 |
| Fat | Primary | 0.01 mg/kg | GC-ECD | Croucher, A. (1998) Report No.: LLA-0084; LLA-0083 DAR, addendum EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-ECD | Class, T. (2003) Report No.: LLA-0101 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS | KCP 5.2.14 M. Rubino, 2018 Report No. 18.625318.0008 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.15 J. Kicińska, 2018 Report No. ZBBZ-2016/37/DPL/3 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS | KCP 5.2.14 M. Rubino, 2018 Report No. 18.625318.0008 |
| Liver | Primary | 0.01 mg/kg | GC-ECD | Croucher, A. (1998) Report No.: LLA-0084; LLA-0083 DAR, addendum EFSA, 2014 EU agreed |
|  | ILV | 0.01 mg/kg | GC-ECD | Wilde, N., and Class, T (2012a) Report No.: LLA-0119 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS/MS | Wilde, N., and Class, T (2012a) Report No.: LLA-0119 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | GC-MS | KCP 5.2.16 M. Rubino, 2018 Report No. 18.625318.0009 |
|  | ILV | 0.01 mg/kg | GC-MS | KCP 5.2.17 J. Kicińska, 2018 Report No. ZBBZ-2016/37/DPL/1 |
|  | Confirmatory (if required) | 0.01 mg/kg | GC-MS | KCP 5.2.16 M. Rubino, 2018 Report No. 18.625318.0009 |
| Component of residue definition: CPIA expressed as Fenvalerate |  |  |  |  |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Kidney, liver | Primary | 0.01 mg/kg | LC-MS/MS | Wilde, N., and Class, T (2012b) Report No.: LLA-0118 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | ILV | - | - | Not porvided during EU review |
|  | Confirmatory (if required) | 0.01 mg/kg | LC-MS/MS | Wilde, N., and Class, T (2012b) Report No.: LLA-0118 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.01 mg/kg | LC-MS/MS | KCP 5.2.18 M. Rubino, 2018 Report No. 18.625318.0010 |
|  | ILV | 0.01 mg/kg | LC-MS | KCP 5.2.19 G. Suresh, 2019 Report No. BIO-ANM 1227 |
|  | Confirmatory (if required) | 0.01 mg/kg | LC-MS/MS | KCP 5.2.18 M. Rubino, 2018 Report No. 18.625318.0010 |

For any special comments or remarkable points concerning the analytical methods for the determination of residues in animal matrices, please refer to Appendix 2.

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in soil is given in the following tables.

Table 5.3-6:	Validated methods for soil (if appropriate)

| Component of residue definition: Esfenvalerate |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.01 mg/kg | LC-MS/MS | Lindner, M. (2011b) Report No.: LLA-0116 RAR, UK, 2013 EFSA, 2014 EU agreed |
| Confirmatory | 0.01 mg/kg | LC-MS/MS | Lindner, M. (2011b) Report No.: LLA-0116 RAR, UK, 2013 EFSA, 2014 EU agreed |
| Primary | 0.005 mg/kg | LC-MS/MS | KCP 5.2.20 J. Hitchens, 2019 Report No. YV/18/009 |
| Confirmatory | 0.005 mg/kg | LC-MS/MS | KCP 5.2.20 J. Hitchens, 2019 Report No. YV/18/009 |
| Component of residue definition: CONH2-Fenvalerate |  |  |  |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.002 mg/kg | LC-MS/MS | Lindner, M. (2011c) Report No.: LLA-0114 RAR, UK, 2013 EFSA, 2014 EU agreed |
| Confirmatory | 0.002 mg/kg | LC-MS/MS | Lindner, M. (2011c) Report No.: LLA-0114 RAR, UK, 2013 EFSA, 2014 EU agreed |
| Primary | 0.002 mg/kg | LC-MS/MS | KCP 5.2.20 J. Hitchens, 2019 Report No. YV/18/009 |
| Confirmatory | 0.002 mg/kg | LC-MS/MS | KCP 5.2.20 J. Hitchens, 2019 Report No. YV/18/009 |

For any special comments or remarkable points concerning the analytical methods for soil please refer to Appendix 2.

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in surface and drinking water is given in the following tables.

Table 5.3-7:	Validated methods for water (if appropriate)

| Component of residue definition: Esfenvalerate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | 0.001 μg/L | GC-ECD | Provided that a method has been successfully validated for surface water at the LOQ required for drinking water (≤0.1 μg/L), no separate validation in drinking water is required – SANTE/12830/2020, rev. 1 |
|  | Confirmatory | 0.001 μg/L | GC-ECD (column with different selectivity) |  |
|  | Primary | 0.001 μg/L | GC-MS/MS | KCP 5.2.21 D. Longhi, 2019 Report No. BPL-STUDY-18-000013 |
|  | ILV | 0.001 μg/L | GC-MS/MS | KCP 5.2.22 E. Nowakowska-Bogdan, 2020 Report No. 169/2019 |
|  | Confirmatory | 0.001 μg/L | GC-MS/MS | KCP 5.2.21 D. Longhi, 2019 Report No. BPL-STUDY-18-000013 |
| Surface water | Primary | 0.001 μg/L | GC-ECD | Wolf, S.(2003) Report No.: LLA-0102 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Confirmatory | 0.001 μg/L | GC-ECD (column with different selectivity) | Wolf, S.(2003) Report No.: LLA-0102 RAR, UK, 2013 EFSA, 2014 EU agreed |
|  | Primary | 0.001 μg/L | GC-MS/MS | KCP 5.2.21 D. Longhi, 2019 Report No. BPL-STUDY-18-000013 |
|  | Confirmatory | 0.001 μg/L | GC-MS/MS | KCP 5.2.21 D. Longhi, 2019 Report No. BPL-STUDY-18-000013 |

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in air is given in the following tables.

Table 5.3-8:	Validated methods for air (if appropriate)

| Component of residue definition: Esfenvalerate |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.1 μg/m3 | GC-ECD | A. Croucher, 1998 Report No. CLE 333/112-D2140 DAR, Portugal, 1995 EU Agreed |
| Confirmatory | 0.1 μg/m3 | GC-ECD | A. Croucher, 1998 Report No. CLE 333/112-D2140 DAR, Portugal, 1995 EU Agreed |

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of Esfenvalerate in body fluids and tissues is given in the following table.

Table 5.3-9:	Methods for body fluids and tissues (if appropriate)

| Component of residue definition: Esfenvalerate |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary (blood) | 0.005 mg/L | GC-ECD | Bacher, R.(2004) Report No.: LLA-0103 RAR, UK, 2013 EFSA, 2014 EU agreed |
| Confirmatory | 0.005 mg/L | GC-MS/MS | Bacher, R.(2004) Report No.: LLA-0103 RAR, UK, 2013 EFSA, 2014 EU agreed |
| Primary (blood, plasma, serum) | 0.005 mg/L | GC-MS/MS | KCP 5.2.23 E. Signore, 2018 Report No. RAU-021-18 |
| Confirmatory | 0.005 mg/L | GC-MS/MS | KCP 5.2.23 E. Signore, 2018 Report No. RAU-021-18 |

#### Other studies/ information

No additional data provided.

Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1 | V. S. Raksha | 2020 | Accelerated storage stability test by heating at elevated temperature of Esfenvalerate 50 g/L EW. V.S. Raksha, 2020, Report No. G13895 GLP Unpublished | N | XXXX |
| KCP 5.1.1-2 | E. Nowakowska-Bogdan | 2021 | Esfenvalerate 50 g/L EW Analysis of toluene content (relevant impurity) and relative density of initial preparation and preparation after accelerated storage procedure (CIPAC MT 46.3) Report No. 35/2021 GLP Unpublished | N | XXXX |
| KCP 5.2.1 | E. Nowakowska-Bogdan | 2020 | Validation of analytical procedure for the determination of Fenvalerate (sum of constituent isomers including Esfenvalerate) in plant residues (grape and tomato) by Gas Chromatography. E. Nowakowska-Bogdan, 2020, Report No. 148/2019 GLP Unpublished | N | XXXX |
| KCP 5.2.2 | D. C. Brown | 2012 | Esfenvalerate – Determination of residue of Esfenvalerate (expressed as the addition of fenvalerate and esfenvalerate including the different isomers and using GC-MS) in peppers in 2012. D. C. Brown, 2012, Report No. S12-04136 GLP Unpublished | N | XXXX |
| KCP 5.2.3 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of Fenvalerate (CAS: 51630-58-1, sum of constituent isomers including esfenvalerate) in grapefruit by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0001 GLP Unpublished | N | XXXX |
| KCP 5.2.4 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of Fenvalerate (CAS: 51630-58-1, sum of constituent isomers including esfenvalerate) in olives by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0002 GLP Unpublished | N | XXXX |
| KCP 5.2.5 | F. Lopez Benet | 2010 | Residue determination of esfenvalerate in olives. F. Lopez Benet, 2010, Report No. 130-09 GLP Unpublished | N | XXXX |
| KCP 5.2.6 | J. Giner | 2018 | Determination of Residues of Esfenvalerate after Two applications in Wheat of Esfenvalerate 2.5% EC in Four Sites. J. Giner, 2018, Report No. 61017R-WW.FP.S1, Analytical phase: A. Augustynek, Report No. DPL/34/2017 GLP Unpublished | N | XXXX |
| KCP 5.2.7 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of fenvalerate (CAS: 51630-58-1, sum of constituent isomers including Esfenvalerate) in wheat by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0003 GLP Unpublished | N | XXXX |
| KCP 5.2.8 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of Esfenvalerate (CAS: 66230-04-4) in milk by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0005 GLP Unpublished | N | XXXX |
| KCP 5.2.9 | J. Kicińska | 2018 | Independent laboratory validation of a method for the determination of Esfenvalerate in milk by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/5 GLP Unpublished | N | XXXX |
| KCP 5.2.10 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of Esfenvalerate (CAS: 66230-04-3) in egg by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0006 GLP Unpublished | N | XXXX |
| KCP 5.2.11 | J. Kicińska | 2018 | Independent Laboratory Validation of a method for the determination of Esfenvalerate in egg by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/4 GLP Unpublished | N | XXXX |
| KCP 5.2.12 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of esfenvalerate (CAS: 66230-04-4) in meat by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0007 GLP Unpublished | N | XXXX |
| KCP 5.2.13 | J. Kicińska | 2018 | Independent Laboratory Validation of a method for the determination of Esfenvalerate in meat by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/2 GLP Unpublished | N | XXXX |
| KCP 5.2.14 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of Esfenvalerate (CASS: 66230-04-4) in fat by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0008 GLP Unpublished | N | XXXX |
| KCP 5.2.15 | J. Kicińska | 2018 | Independent Laboratory Validation of a method for the determination of Esfenvalerate in fat by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/3 GLP Unpublished | N | XXXX |
| KCP 5.2.16 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of Esfenvalerate (CAS: 66230-04-4) in liver by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0009 GLP Unpublished | N | XXXX |
| KCP 5.2.17 | J. Kicińska | 2018 | Independent Laboratory Validation of a method for the determination of Esfenvalerate in liver by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/1 GLP Unpublished | N | XXXX |
| KCP 5.2.18 | M. Rubino | 2018 | Validation of the analytical procedure for the determination of CPIA (CAS: 2012-74-0), in liver by liquid chromatography. M. Rubino, 2018, Report No. 18.625318.0010 GLP Unpublished | N | XXXX |
| KCP 5.2.19 | G. Sureshi | 2019 | Independent Laboratory validation for the determination of 2-(4-chlorophenyl)-3-methylbutyric acid (CPIA metabolite) residues in liver samples. G. Sureshi, 2019, Report No. BIO-ANM 1227 GLP Unpublished | N | XXXX |
| KCP 5.2.20 | J. Hitchens | 2019 | Terrestrial Soil Dissipation of Esfenvalerate after one Application to Bare Soil in Southern Europe. J. Hitchens. 2019, Report No. YV/18/009 GLP Unpublished | N | XXXX |
| KCP 5.2.21 | D. Longhi | 2019 | Validation of the analytical method to determine Esfenvalerate in Water samples. D. Longhi, 2019, Report No. BPL-STUDY-18-000013 GLP Unpublished | N | XXXX |
| KCP 5.2.22 | E. Nowakowska-Bogdan | 2020 | Validation of the analytical method to determine Esfenvalerate in drinking water samples. E. Nowakowska-Bogdan, 2020, Report No. 169/2019 GLP Unpublished | N | XXXX |
| KCP 5.2.23 | E. Signore | 2018 | Validation of the analytical method to determine Esfenvalerate in body fluids (blood, plasma, serum). E. Signore, 2018, Report No. RAU-021-18 GLP Unpublished | N | XXXX |
| KCP 5.2.24 | Markowicz, A | 2022 | Validation of the method for the determination of the residues of fenvalerate (any ratio of constituent isomers including esfenvalerate) in head cabbage, strawberry and wheat by gas chromatography. Study Code: 21/FSL/09/V, Food Safety Laboratory, The National Institute of Horticultural Research GLP | N | XXXX |

Detailed evaluation of submitted analytical methods

Analytical methods for Esfenvalerate

Methods used for the generation of pre-authorization data (KCP 5.1)

No new or additional studies have been submitted

Methods for post-authorization control and monitoring purposes (KCP 5.2)

Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

Analytical method 1

Method validation

| Comments of zRMS: | The validation is accepted. 5 recovery determinations were performed at the LOQ (0.01 mg/kg) at the 10 x LOQ (0.1 mg/kg) for grape and tomato, respectively. Analysis was performed by extraction and single injection. The mean recovery values at the fortification levels of 0.01 mg/kg and 0.1 mg/kg for grape and tomato for both ion mass transitions were all in the range 70 - 110 %. All precision values at 2 fortification levels for both ion mass transitions were < 20%. Details as follows: |
| --- | --- |

| Reference: | KCP 5.2.1 |
| --- | --- |
| Report | Validation of analytical procedure for the determination of Fenvalerate (sum of constituent isomers including Esfenvalerate) in plant residues (grape and tomato) by Gas Chromatography. E. Nowakowska-Bogdan, 2020, Report No. 148/2019 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The aim of the study was the validation of an analytical method capable of quantitating Fenvalerate (mixture of 4 enantiomers) in raw agricultural commodities (Grapes and Tomatoes). The analyses were carried out using a GC-MS/MS system with a method capable of separating the Fenvalerate diastereoisomers.

An aliquot of the sample solution was injected into the gas chromatograph coupled with tandem mass spectrometry. The MS/MS instrument was operated in the Multiple Monitoring mode (MRM).

Preparation of Sample matrix

Portion of dry ice was added to a homogenizer apparatus (Laboratory mill). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained. Contents of the apparatus were poured into polyethylene bags, and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

10 of homogenized matrix was weighed into a 50 mL PP centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard of Fenvalerate onto the matrix was carried out at this step.

| Fortification level | Concentration (μg/mL) | Volume used (μL) | Volume ISTD (μL) |
| --- | --- | --- | --- |
| LOQ (0.01 mg/kg) | 1 | 100 | 100 |
| 10 x LOQ (0.1 mg/kg) | 10 | 100 | 100 |

For extraction using glass automatic pipettes 9.8 mL of acetonitrile was added. The PP centrifuge tube was closed tightly and shake for 1 min. automatically.

Liquid-liquid partition

Buffer-salt mixture (4g MgSO4, 1 g NaCl, 0.5 g NaCitrate dibasic sesquihydrate, 1 g NaCitrate tribasic dehydrate) was added and the centrifuge tube was closed and shaken by vortex for 1 min. The extract was centrifuged at >3000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from method validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Ion mass transition m/z 167 125 (Quantification) |  |  |  |  |  |
| Grape | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 85 | 9.2 |  |
|  |  | 0.1 | 103 | 4.7 |  |
| Ion mass transition m/z 181 152 (Confirmation) |  |  |  |  |  |
| Grape | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 85 | 6.6 |  |
|  |  | 0.1 | 93 | 6.8 |  |
| Ion mass transition m/z 167 125 (Quantification) |  |  |  |  |  |
| Tomato | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 86 | 13.5 |  |
|  |  | 0.1 | 87 | 3.6 |  |
| Ion mass transition m/z 181 152 (Confirmation) |  |  |  |  |  |
| Tomato | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 95 | 7.9 |  |
|  |  | 0.1 | 87 | 3.4 |  |

Table A 8:	Characteristics for the analytical method used for validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in tomato and grape

|  | Tomato Fenvalerate (sum of constituent isomers including Esfenvalerate) | Grape Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- | --- |
| Specificity | The method is specific | The method is specific |
| Calibration (type, number of data points) | 7 points 0.003 to 0.21 mg/kg First mass transition y= 11.968568x-0.007147 R = 0.99944730 Second mass transition y=8.025838x-0.001865 R = 0.99947680 | 7 points 0.003 to 0.21 mg/kg First mass transition y= 12.195568x-0.010472 R = 0.99978530 Second mass transition y=5.584991x-0.001176 R = 0.99955429 |
| Assessment of matrix effects is presented | Yes | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in grape and tomato.

Independent laboratory validation

| Comments of zRMS: | The validation of the method for the determination of residues of esfenvalerate in peppers has been accepted. It is not an ILV. |
| --- | --- |

| Reference: | KCP 5.2.2 |
| --- | --- |
| Report | Esfenvalerate – Determination of residue of Esfenvalerate (expressed as the addition of fenvalerate and esfenvalerate including the different isomers and using GC-MS) in peppers in 2012. D. C. Brown, 2012, Report No. S12-04136 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The method is based on the multiresidue QUECHERS procedure. A sub-sample of prepared matrix is extracted with acetonitrile (with addition of anhydrous magnesium sulphate and sodium chloride) by shaking and ultrasonication. After centrifugation, an aliquot is transferred to an Eppendorf tube containing further magnesium sulphate. The extract is then shaken and placed in the centrifuge to separate the solid and liquid phases. Toluene is added to an aliquot of the acetonitrile extract which is then evaporated to low volume and adjusted to a final volume 2.0 mL with further toluene. Quantification of esfenvalerate residues is by GC-MS (monitoring 3 ions).

Results and discussions

Table A 9:	Recovery results from independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Peppers | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 90 | 4.3 | First mass transition |
|  |  | 0.1 | 101 | 7.9 |  |
|  |  | 0.01 | 99 | 9.3 | Second mass transition |
|  |  | 0.1 | 100 | 10.9 |  |
|  |  | 0.01 | 83 | 4.7 | Third mass transition |
|  |  | 0.1 | 100 | 7.2 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in Peppers

|  | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- |
| Specificity | Analysis of unfortiffied homogenised control control specimens yielded no residues of esfenvalerate above 30% LOQ by GC/MS The method is specific |
| Calibration (type, number of data points) | 7 points 0.00167 to 0.2 μg/mL First mass transition y=1504508x-2711 R2=0.9997 Second mass transition y=1849975x-4503 R2=0.9997 Third mass transition y=3346479x-10284 R2=0.9996 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.001 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of Esfenvalerate residues in peppers.

Independent laboratory validation

| Comments of zRMS: | The validation has been accepted. It is not an ILV. |
| --- | --- |

| Reference: | KCP 5.2.3 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Fenvalerate (CAS: 51630-58-1, sum of constituent isomers including esfenvalerate) in grapefruit by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0001 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Fenvalerate (CAS: 51630-58-1), sum of constituent isomers including Esfenvalerate) in grapefruit specimens.

Sample extraction

About 5.00 g of grapefruit were exactly weighed into a 50 mL plastic tube, 10 mL of deionized water and with 10 mL of extraction solvent were added together with 50 μL of IRS-TPP and the mixture was vigorously shaken for 1 min. After addition of 4 g MgSO4, 1 g of NaCl, 1 g of trisodium citrate and 0.5 g sodium hydrogencitrate, the samples tubes were shaken at geneogrinder for 2 min and centrifuged. It was the proceed with purification step. About 6 mL of supernatant were transferred into a 15 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged and final extract was directly injected into GC-MS.

Results and discussions

Table A 9:	Recovery results from independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Grapefruit | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 87 | 4 | First mass transition |
|  |  | 0.1 | 91 | 2 |  |
|  |  | 0.01 | 88 | 3 | Second mass transition |
|  |  | 0.1 | 94 | 2 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in Grapefruit

|  | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- |
| Specificity | The method is specific. |
| Calibration (type, number of data points) | 5 points 0.003 to 0.40 mg/kg First mass transition y=3.1945x R2=0.9997 Second mass transition y=6.2582x R2=0.9984 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of Esfenvalerate residues in grapefruit.

Confirmatory method (if required)

No confirmatory method is required

Analytical method 2

Method validation

| Comments of zRMS: | The validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.4 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Fenvalerate (CAS: 51630-58-1, sum of constituent isomers including esfenvalerate) in olives by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0002 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Fenvalerate (CAS: 51630-58-1, sum of constituent isomers including Esfenvalerate) in olives specimens.

Sample extraction.

About 2.00 g of olives were exactly weighed into a 50 mL plastic tube. 10 mL of deionized water and with 10 mL of extraction solvent were added together with 20 μL of IRS-TPP and the mixture was  vigorously shaken for 1 min. After addition of 4 g of MgSO4, 1 g of NaCl, 1 g of trisodium citrate and 0.5 g of sodium hydrogencitrate, the samples tubes were shaken at genogrinder for 2 min and centrifuged. It was the proceed with the purification step, about 6 mL of supernatant were transferred into a 15 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged and final extract was directly injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Olives | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 90 | 4 | First mass transition |
|  |  | 0.1 | 105 | 4 |  |
|  |  | 0.01 | 90 | 2 | Second mass transition |
|  |  | 0.1 | 103 | 2 |  |

Table A 10:	Characteristics for the analytical method used for validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in olives

|  | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution for both transitions 1 and 2. |
| Calibration (type, number of data points) | 5 points 0.003 to 0.40 mg/kg First mass transition y=2.306x R2=0.9991 Second mass transition y=6.8138x R2=0.9993 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in olives.

Independent laboratory validation

| Comments of zRMS: | The validation has been accepted. This is not an ILV. |
| --- | --- |

| Reference: | KCP 5.2.5 |
| --- | --- |
| Report | Residue determination of esfenvalerate in olives. F. Lopez Benet, 2010, Report No. 130-09 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

Esfenvalerate residues, defines as the sum of fenvalerate and esfenvalerate, have been determined in olive samples collected at harvest after the application of the formulate product Esfenvalerato 2.5% EC in field conditions by the test facility.

Sample preparation

The olive samples were defrosted before processing and then, they were thoroughly triturated using the crushing mill M380V.

About 5 g of triturated sample were introduced in a 50 mL Falcon tube and d20 mL of hexane and ca 10 g of sodium sulphate anhydrous were added. The sample was extracted shaking in the rotary agitator during 1 hour at room temperature. The entire extract was filtered by gravity and washed with hexane. The final volume was adjusted to 25 mL with hexane in a volumetric flask.

An aliquot of 5 mL of raw extract was purified by liquid liquid extraction with 10 mL of acetonitrile saturated with hexane shaking during 2 minutes. Liquid liquid extraction was carried out twice more with 7 mL of extractant. Acetonitrile phases were collected in a volumetric flask, adjusting the volume to 25 mL using acetonitrile saturated with hexane. Then, an aliquot of 2.5 mL of this extract was evaporated at 40℃ under a gentle stream of nitrogen to dryness. The residue was diluted with 1 mL of hexane and collected in an autosampler vial. A second vial was stored in the freezer in prevision of need to repeat the analysis.

Results and discussions

Table A 7:	Recovery results from method validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Olives | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 75.8 | 6.0 |  |
|  |  | 0.1 | 76.5 | 6.1 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in olives

|  | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 7 points 0.495 to 49.5 ng/mL y=1357.963x – 273.056 R=0.9997 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.001 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in olives.

Analytical method 3

Method validation

| Comments of zRMS: | The validation has been accepted. This study is not presented in B7. |
| --- | --- |

| Reference: | KCP 5.2.6 |
| --- | --- |
| Report | Determination of Residues of Esfenvalerate after Two applications in Wheat of Esfenvalerate 2.5% EC in Four Sites. J. Giner, 2018, Report No. 61017R-WW.FP.S1, Analytical phase: A. Augustynek, Report No. DPL/34/2017 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The aim of the study was to evaluate the residue levels of fenvalerate (any ratio of constituent isomers (RR, SS, RS and SR) including Esfenvalerate 2.5% EC.

Specimen extraction and determination of residues of fenvalerate were performed according to the multi-residue QuEChERS method. Quantification was performed by use of GC-MS/MS detection.

Sample extraction.

5 g (for grains and green parts of plants) or 2 g (for straw) of the homogenized sample were weighed into a 50 mL centrifuge tube. 10 mL of acetonitrile and 10 mL of deionized water were added together with 50 μL (for grains and green parts of plants) or 20 μL (for straw) of internal standard solution and the mixture was shaken vigorously by hand for one minute. After addition of buffering salts (4 g anhydrous magnesium sulfate, 1 g sodium chloride, 1 g trisodium citrate dehydrate, 0.5 g disodium hydrogencitrate sesquihydrate), the mixture was shaken again intensively for 1 min, then centrifuged at 3000 rpm for 5 min for phase separation. After that, 6 mL of acetonitrile phase was transferred into centrifuge tube containing 150 mg PSA, 900 mg MgSO4 and 150 mg C18 (for grains and straw) or 45 mg ENVI-Carb (for green parts of plants), the mixture was shaken intensively for 1 min and centrifuged at 3000 rpm for 5 min. Afterwards 5 mL of the clean extract was transferred to a 10 mL vial and 50 μL of the 5% formic acid in acetonitrile was added. The final extract was directly employed for GC-MS/MS analysis. Quantification was performed using an internal standard, which was added to the extract after the initial addition of acetonitrile.

Results and discussions

Table A 9:	Recovery results from from method validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat grain | Fenvalerate | 0.01 | 104 | 2.37 |  |
|  |  | 0.1 | 98.2 | 3.37 |  |
|  | Esfenvalerate | 0.01 | 113 | 6.23 |  |
|  |  | 0.1 | 107 | 2.83 |  |
| Wheat green parts of plants | Fenvalerate | 0.01 | 119 | 6.01 |  |
|  |  | 0.1 | 87.2 | 1.94 |  |
|  | Esfenvalerate | 0.01 | 123 | 2.78 |  |
|  |  | 0.1 | 108 | 3.43 |  |
| Wheat straw | Fenvalerate | 0.01 | 75.9 | 2.85 |  |
|  |  | 0.1 | 75.6 | 4.63 |  |
|  | Esfenvalerate | 0.01 | 90.2 | 4.26 |  |
|  |  | 0.1 | 91.6 | 2.50 |  |

Table A 10:	Characteristics for the analytical method used for validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in wheat

|  | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 7 points 0.004 mg/kg to 1.0 mg/kg (grains and green parts of plants) 8 points 0.0025 mg/kg to 2.5 mg/kg (for straw) Grains (isomers SR, RS): y=0.1550589x – 1.145633e-004 R2=0.9993 Grains (isomers SS, RR): y=5.317546e-002x – 8.845741e-005 R2=0.9990 Green parts of plants (isomers SR,RS) y=0.1881298x – 2.011658e-004 R2=0.9984 Green parts of plants (isomers SS,RR) y=0.1030996x-1.974502e-004 R2=0.9986 Straw (isomers SR,RS) y=0.1804656x+2.000034e-004 R2=0.9989 Straw (isomers RR, SS) y=0.682574e-002x+1.605688e-004 R2=0.9983 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in wheat

Independent laboratory validation

| Comments of zRMS: | The validation has been accepted. This is not the ILV. Repeatability evaluation was performed on sample aliquots spiked with Fenvalerate at LOQ (about 0,01 mg/Kg) and 10xLOQ (about 0,1 mg/Kg). 5 replicate analyses were performed for each spiking level. %RSD at each fortified level was calculated for both transitions. |
| --- | --- |

| Reference: | KCP 5.2.7 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of fenvalerate (CAS: 51630-58-1, sum of constituent isomers including Esfenvalerate) in wheat by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0003 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Fenvalerate (CAS: 51630-58-1, sum of constituent isomers including Esfenvalerate) in wheat specimens.

Sample extraction

About 5 g of wheat were exactly weighed into a 50 mL plastic tube. 10  mL of deionized water and with 10 mL of extraction solvent were added together with 50 μL of IRS-TPP and the mixture was vigorously shaken for 1 min. After addition of 4 g of MgSO4, 1 g of NaCl, 1 g of trisodium citrate and 0.5 g of sodium hydrogencitrate, the samples tubes were shaken at genogrinder for 2 min and centrifuged. It was then proceeded with the purification step: about 6 mL of supernatant were transferred into a 15 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged, and final extract was directly injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Wheat | Fenvalerate (sum of constituent isomers including Esfenvalerate) | 0.01 | 88 | 4 | First mass transition |
|  |  | 0.1 | 87 | 2 |  |
|  |  | 0.01 | 89 | 4 | Second mass transition |
|  |  | 0.1 | 87 | 1 |  |
|  |  | 0.01 | 91 | 4 | Third mass transition |
|  |  | 0.1 | 89 | 2 |  |
|  |  | 0.01 | 89 | 4 | Fourth mass transition |
|  |  | 0.1 | 87 | 1 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in wheat

|  | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.40 mg/kg First mass transition y=2.6853x R2=0.9991 Second mass transition y=20.006x R2=0.9991 Third mass transition y=3.7169x R2=0.9991 Fourth mass transition y=11.036x R2=0.9989 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.001 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in wheat.

Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

Analytical method 4

Method validation

| Comments of zRMS: | The method validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.8 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Esfenvalerate (CAS: 66230-04-4) in milk by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0005 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Esfenvalerate in milk specimens.

Sample extraction.

About 10 g of milk were exactly weighed into a 50 mL plastic tube and extracted with 10 mL of extraction solvent. About 4 g of MgSO4, 1 g of NaCl, 1 g of Sodium Citrate and 0.5 g of Disodium citrate sesquihydrate were added together with a ceramic homogenizer and the mixture was vigorously stirred for 2 min with genogrinder. After centrifuging, it was proceeded to purification step. About 6 mL of supernatant were transferred into a 10 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged, then 1 mL of extracted fraction was dried under nitrogen, then resuspended with 1 mL of blank solution and injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Milk | Esfenvalerate | 0.01 | 88 | 3 | First mass transition |
|  |  | 0.1 | 94 | 2 |  |
|  |  | 0.01 | 94 | 7 | Second mass transition |
|  |  | 0.1 | 100 | 2 |  |

Table A 10:	Characteristics for the analytical method used for validation of Esfenvalerate residues in Milk

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.32 mg/kg First mass transition y=2569376x R2=0.9959 Second mass transition y=807738x R2=0.9970 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in milk.

Independent laboratory validation

| Comments of zRMS: | The ILV has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.9 |
| --- | --- |
| Report | Independent laboratory validation of a method for the determination of Esfenvalerate in milk by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/5 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The objective of the study is an independent validation of the method for the determination of Esfenvalerate in Milk described in “Validation of the analytical procedure for the determination of Esfenvalerate (CAS:66230-04-4) in Milk by gas chromatography”, Final Report. 18.625318.0005 and in accordance to the guidance document SANCO/825/00, rev. 8.1 of the European Commission.

Preparation of sample matrix

Portion of dry ice was added to a homogenizer apparatus (robot coupe). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained.. Contents of the apparatus was poured into polyethylene bags and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

10 g of homogenized matrix was weighed into a 50 mL Teflon centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken in a vortex mixer for 10 min.

| Fortification level | Concentration (μg/mL) | Volume used (μL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 0.1 | 1000 |
| 10 x LOQ (0.1 mg/kg) | 1 | 1000 |

Using glass volumetric pipettes 10 mL of acetonitrile was added.

Liquid-Liquid Partition

Buffer salt mixture (4 g of magnesium sulfate anhydrous, 1 g of sodium chloride, 1 g of sodium citrate and 0.5 g of disodium citrate sesquihydrate) was added and the centrifuge tube was closed and shaken vigorously by QuEChERS Hand Motion Shaken for 2 min.

The extract was centrifuged at 4000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Milk | Esfenvalerate | 0.01 | 96 | 3.6 | First mass transition |
|  |  | 0.1 | 89 | 2.6 |  |
|  |  | 0.01 | 98 | 5.0 | Second mass transition |
|  |  | 0.1 | 90 | 2.7 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation for validation of Esfenvalerate residues in milk

|  | Esfenvalerate |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.3 mg/kg First mass transition y=10970447.2111x-18847.272547 R2=0.99848334 Second mass transition y=6367392.161205x-13100.7750828 R2=0.99740823 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.001 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in milk.

Analytical method 5

Method validation

| Comments of zRMS: | The method validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.10 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Esfenvalerate (CAS: 66230-04-3) in egg by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0006 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Esfenvalerate in eggs specimens.

Sample extraction.

About 5 g of eggs were exactly weighed into a 50 mL plastic tube additioned with 5 mL of water and extracted with 10 mL of extraction solvent. About 4 g of MgSO4, 1 g of NaCl, 1 g of Sodium Citrate and 0.5 g of Disodium citrate sesquihydrate were added together with a ceramic homogenizer and the mixture was vigorously stirred for 2 min with genogrinder. After centrifuging, it was proceeded to purification step. About 6 mL of supernatant were transferred into a 10 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged, then 1 mL of extracted fraction was dried under nitrogen, then resuspended with 0.5 mL of blank solution and injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Eggs | Esfenvalerate | 0.01 | 90 | 11 | First mass transition |
|  |  | 0.1 | 95 | 5 |  |
|  |  | 0.01 | 87 | 14 | Second mass transition |
|  |  | 0.1 | 95 | 9 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in Eggs

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.32 mg/kg First mass transition y=642037x R2=0.9964 Second mass transition y=232930x R2=0.9949 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in eggs.

Independent laboratory validation

| Comments of zRMS: | The ILV has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.11 |
| --- | --- |
| Report | Independent Laboratory Validation of a method for the determination of Esfenvalerate in egg by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/4 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The objective of the study is an independent validation of the method for the determination of Esfenvalerate in Egg described in “Validation of the analytical procedure for the determination of Esfenvalerate (CAS:66230-04-4) in Egg by gas chromatography”, Final Report. 18.625318.0006 and in accordance to the guidance document SANCO/825/00, rev. 8.1 of the European Commission.

Preparation of sample matrix

Portion of dry ice was added to a homogenizer apparatus (robot coupe). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained.. Contents of the apparatus was poured into polyethylene bags and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

5 g of homogenized matrix was weighed into a 50 mL Teflon centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken in a vortex mixer for 10 min.

| Fortification level | Concentration (μg/mL) | Volume used (mL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 0.1 | 0.5 |
| 10 x LOQ (0.1 mg/kg) | 1 | 0.5 |

Using glass volumetric pipettes 10 mL of acetonitrile was added.

Liquid-Liquid Partition

Buffer salt mixture (4 g of magnesium sulfate anhydrous, 1 g of sodium chloride, 1 g of sodium citrate and 0.5 g of disodium citrate sesquihydrate) was added and the centrifuge tube was closed and shaken vigorously by QuEChERS Hand Motion Shaken for 2 min.

The extract was centrifuged at 4000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Eggs | Esfenvalerate | 0.01 | 79 | 2.5 | First mass transition |
|  |  | 0.1 | 79 | 3.4 |  |
|  |  | 0.01 | 77 | 3.3 | Second mass transition |
|  |  | 0.1 | 77 | 4.0 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in eggs

|  | Esfenvalerate |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.3 mg/kg First mass transition y=17587607.9143x-14856.841507 R2=0.99961425 Second mass transition y=8994651.013029x-7120.2245381 R2=0.99906597 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in eggs.

Analytical method 6

Method validation

| Comments of zRMS: | The method validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.12 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of esfenvalerate (CAS: 66230-04-4) in meat by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0007 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Esfenvalerate in meat specimens.

Sample extraction.

About 3 g of meat were exactly weighed into a 50 mL plastic tube additioned with 5 mL of water and extracted with 10 mL of extraction solvent. About 4 g of MgSO4, 1 g of NaCl, 1 g of Sodium Citrate and 0.5 g of Disodium citrate sesquihydrate were added together with a ceramic homogenizer and the mixture was vigorously stirred for 2 min with genogrinder. After centrifuging, it was proceeded to purification step. About 6 mL of supernatant were transferred into a 15 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged, then 1 mL of extracted fraction was dried under nitrogen, then resuspended with 0.5 mL of blank solution and injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Meat | Esfenvalerate | 0.01 | 81 | 11 | First mass transition |
|  |  | 0.1 | 95 | 13 |  |
|  |  | 0.01 | 81 | 9 | Second mass transition |
|  |  | 0.1 | 96 | 12 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in meat

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.32 mg/kg First mass transition y=1932520x R2=0.9994 Second mass transition y=626453x R2=0.9990 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in meat.

Independent laboratory validation

| Comments of zRMS: | The method ILV has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.13 |
| --- | --- |
| Report | Independent Laboratory Validation of a method for the determination of Esfenvalerate in meat by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/2 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The objective of the study is an independent validation of the method for the determination of Esfenvalerate in Meat described in “Validation of the analytical procedure for the determination of Esfenvalerate (CAS:66230-04-4) in Meat by gas chromatography”, Final Report. 18.625318.0007 and in accordance to the guidance document SANCO/825/00, rev. 8.1 of the European Commission.

Preparation of sample matrix

Portion of dry ice was added to a homogenizer apparatus (robot coupe). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained.. Contents of the apparatus was poured into polyethylene bags and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

53g of homogenized matrix was weighed into a 50 mL Teflon centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken in a vortex mixer for 10 min.

| Fortification level | Concentration (μg/mL) | Volume used (μL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 0.1 | 0.3 |
| 10 x LOQ (0.1 mg/kg) | 1 | 0.3 |

Using glass volumetric pipettes 10 mL of acetonitrile was added.

Liquid-Liquid Partition

Buffer salt mixture (4 g of magnesium sulfate anhydrous, 1 g of sodium chloride, 1 g of sodium citrate and 0.5 g of disodium citrate sesquihydrate) was added and the centrifuge tube was closed and shaken vigorously by QuEChERS Hand Motion Shaken for 2 min.

The extract was centrifuged at 4000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Meat | Esfenvalerate | 0.01 | 88 | 8.0 | First mass transition |
|  |  | 0.1 | 101 | 1.3 |  |
|  |  | 0.01 | 93 | 9.0 | Second mass transition |
|  |  | 0.1 | 102 | 2.8 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation Esfenvalerate residues in meat

|  | Esfenvalerate |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.3 mg/kg First mass transition y=7542890.4938x+767.461005 R2=0.99574887 Second mass transition y=3200415.160260x+395.8945900 R2=0.99506424 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in meat.

Analytical method 7

Method validation

| Comments of zRMS: | The validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.14 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Esfenvalerate (CASS: 66230-04-4) in fat by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0008 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Esfenvalerate in fat specimens.

Sample extraction.

About 3 g of meat were exactly weighed into a 50 mL plastic tube additioned with 5 mL of water and extracted with 3 mL of extraction solvent. About 4 g of MgSO4, 1 g of NaCl, 1 g of Sodium Citrate and 0.5 g of Disodium citrate sesquihydrate were added together with a ceramic homogenizer and the mixture was vigorously stirred for 2 min with genogrinder. After centrifuging, it was proceeded to purification step. About 6 mL of supernatant were transferred into a 15 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged, then 1 mL of extracted fraction was dried under nitrogen, then resuspended with 0.3 mL of blank solution and injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Fat | Esfenvalerate | 0.01 | 88 | 6 | First mass transition |
|  |  | 0.1 | 96 | 2 |  |
|  |  | 0.01 | 89 | 7 | Second mass transition |
|  |  | 0.1 | 93 | 4 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in fat

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.32 mg/kg First mass transition y=11465428x R2=0.9981 Second mass transition y=486953x R2=0.9994 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in fat.

Independent laboratory validation

| Comments of zRMS: | The independent validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.15 |
| --- | --- |
| Report | Independent Laboratory Validation of a method for the determination of Esfenvalerate in fat by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/3 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The objective of the study is an independent validation of the method for the determination of Esfenvalerate in Fat described in “Validation of the analytical procedure for the determination of Esfenvalerate (CAS:66230-04-4) in Fat by gas chromatography”, Final Report. 18.625318.0008 and in accordance to the guidance document SANCO/825/00, rev. 8.1 of the European Commission.

Preparation of sample matrix

Portion of dry ice was added to a homogenizer apparatus (robot coupe). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained.. Contents of the apparatus was poured into polyethylene bags and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

3g of homogenized matrix was weighed into a 50 mL Teflon centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken in a vortex mixer for 10 min.

| Fortification level | Concentration (μg/mL) | Volume used (μL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 0.1 | 0.3 |
| 10 x LOQ (0.1 mg/kg) | 1 | 0.3 |

Using glass volumetric pipettes 10 mL of acetonitrile was added.

Liquid-Liquid Partition

Buffer salt mixture (4 g of magnesium sulfate anhydrous, 1 g of sodium chloride, 1 g of sodium citrate and 0.5 g of disodium citrate sesquihydrate) was added and the centrifuge tube was closed and shaken vigorously by QuEChERS Hand Motion Shaken for 2 min.

The extract was centrifuged at 4000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Fat | Esfenvalerate | 0.01 | 88 | 4.8 | First mass transition |
|  |  | 0.1 | 89 | 2.1 |  |
|  |  | 0.01 | 88 | 5.4 | Second mass transition |
|  |  | 0.1 | 87 | 1.3 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in fat

|  | Esfenvalerate |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.4 mg/kg First mass transition y=18644317.7684x-43957.662554 R2=0.99389469 Second mass transition y=9559037.909136x-20708.8615811 R2=0.99325012 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in fat.

Analytical method 8

Method validation

| Comments of zRMS: | The method validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.16 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of Esfenvalerate (CAS: 66230-04-4) in liver by gas chromatography. M. Rubino, 2018, Report No. 18.625318.0009 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of Esfenvalerate in liver specimens.

Sample extraction.

About 3 g of meat were exactly weighed into a 50 mL plastic tube additioned with 5 mL of water and extracted with 3 mL of extraction solvent. About 4 g of MgSO4, 1 g of NaCl, 1 g of Sodium Citrate and 0.5 g of Disodium citrate sesquihydrate were added together with a ceramic homogenizer and the mixture was vigorously stirred for 2 min with genogrinder. After centrifuging, it was proceeded to purification step. About 6 mL of supernatant were transferred into a 15 mL plastic tube containing about 1 g of MgSO4, 150 mg of PSA and 150 mg of C18 dispersive. The mixture was stirred for 1 min and centrifuged, then 1 mL of extracted fraction was dried under nitrogen, then resuspended with 0.3 mL of blank solution and injected into GC-MS. Test sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Liver | Esfenvalerate | 0.01 | 92 | 5 | First mass transition |
|  |  | 0.1 | 83 | 4 |  |
|  |  | 0.01 | 81 | 5 | Second mass transition |
|  |  | 0.1 | 80 | 5 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in liver

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.32 mg/kg First mass transition y=2243476x R2=0.9936 Second mass transition y=763412x R2=0.9923 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in liver.

Independent laboratory validation

| Comments of zRMS: | The Independent Laboratory Validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.17 |
| --- | --- |
| Report | Independent Laboratory Validation of a method for the determination of Esfenvalerate in liver by gas chromatography. J. Kicińska, 2018, Report No. ZBBZ-2016/37/DPL/1 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The objective of the study is an independent validation of the method for the determination of Esfenvalerate in liver described in “Validation of the analytical procedure for the determination of Esfenvalerate (CAS:66230-04-4) in liver by gas chromatography”, Final Report. 18.625318.0009 and in accordance to the guidance document SANCO/825/00, rev. 8.1 of the European Commission.

Preparation of sample matrix

Portion of dry ice was added to a homogenizer apparatus (robot coupe). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained.. Contents of the apparatus was poured into polyethylene bags and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

3g of homogenized matrix was weighed into a 50 mL Teflon centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken in a vortex mixer for 10 min.

| Fortification level | Concentration (μg/mL) | Volume used (μL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 0.1 | 0.3 |
| 10 x LOQ (0.1 mg/kg) | 1 | 0.3 |

Using glass volumetric pipettes 5 ml of water and 30 mL of acetonitrile was added.

Liquid-Liquid Partition

Buffer salt mixture (4 g of magnesium sulfate anhydrous, 1 g of sodium chloride, 1 g of sodium citrate and 0.5 g of disodium citrate sesquihydrate) was added and the centrifuge tube was closed and shaken vigorously by QuEChERS Hand Motion Shaken for 2 min.

The extract was centrifuged at 4000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Liver | Esfenvalerate | 0.01 | 96 | 10.7 | First mass transition |
|  |  | 0.1 | 93 | 5.9 |  |
|  |  | 0.01 | 96 | 11.8 | Second mass transition |
|  |  | 0.1 | 93 | 2.1 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in liver

|  | Esfenvalerate |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.3 mg/kg First mass transition y=1933180.368104x-158.270472 R2=0.99744291 Second mass transition y=832610.266787x-542.8208189 R2=0.99948950 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in liver.

Analytical method 9

Method validation

| Comments of zRMS: | The method validation has been accepted. (metabolite CPIA - 2-(4-chlorophenyl)-3-methylbutyric acid) |
| --- | --- |

| Reference: | KCP 5.2.18 |
| --- | --- |
| Report | Validation of the analytical procedure for the determination of CPIA (CAS: 2012-74-0), in liver by liquid chromatography. M. Rubino, 2018, Report No. 18.625318.0010 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Purpose of this short-term analytical study was to validate the in-house method for the determination of residues of CPIA in liver.

Sample extraction.

About 5.0 g of licer were weighed into a 50 mL falcon and 8.5 mL of milliQ water were added in order to hydrate the matrix. Then 10 mL of extraction mixture were added to the sample. After vortexing for about 1 min, about 6 g of magnesium sulphate anhydroud, and 1.5 g of sodium acetate were added to the sample and vortexed again for about 1min. The tube was centrifuged at 4750 rpm for 5 min. Then the tube was centrifuged at 4750 rpm for 5 min and it was proceed to purification of the supernatant. 6 mL of supernatant were transferred into a 10 mL plastic tube, containing about 900 mg of magnesium sulphate anhydrous and 150 mg of C18 resin. Vortexed for about 1 min and centrifuged at 4750 rpm for 5 min. 1 mL of supernatant was transferred in 10 mL tube and dried by nitrogen flux. The dried sample was the resuspended with 0.5 mL of methanol. Vortexed for about 1 m in, then transferred into an HPLC vial and injected. The sample was prepared in duplicate.

Results and discussions

Table A 9:	Recovery results from method validation of CPIA using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Liver | CPIA | 0.01 | 94 | 5 | First mass transition |
|  |  | 0.1 | 87 | 4 |  |
|  |  | 0.01 | 86 | 12 | Second mass transition |
|  |  | 0.1 | 92 | 2 |  |

Table A 10:	Characteristics for the analytical method used for independent laboratory validation of CPIA residues in liver

|  | CPIA |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.32 mg/kg First mass transition y=51282x R2=0.9991 Second mass transition y=8638x R2=0.9987 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of CPIA in liver.

Independent laboratory validation

| Comments of zRMS: | The ILV has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.19 |
| --- | --- |
| Report | Independent Laboratory validation for the determination of 2-(4-chlorophenyl)-3-methylbutyric acid (CPIA metabolite) residues in liver samples. G. Sureshi, 2019, Report No. BIO-ANM 1227 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The objective of this study was to validate an analytical method for the determination of residues of CPIA metabolite in liver (bovine origin).

Preparation of sample matrix

Portion of dry ice was added to a homogenizer apparatus (robot coupe). Subsequent appropriate amount of sample was added to the apparatus in small portions. Sample was blended after each addition until a homogenous mixture was obtained.. Contents of the apparatus was poured into polyethylene bags and stored in a freezer until the last traces of dry ice have sublimed.

Sample extraction

3g of homogenized matrix was weighed into a 50 mL Teflon centrifuge tube. Sample weight was recorded. If necessary fortification of the concurrent recovery sample by aliquoting the fortification standard onto the matrix was carried out at this step. The tube was shaken in a vortex mixer for 10 min.

| Fortification level | Concentration (μg/mL) | Volume used (μL) |
| --- | --- | --- |
| LOQ (0.01 mg/kg) | 0.1 | 0.3 |
| 10 x LOQ (0.1 mg/kg) | 1 | 0.3 |

Using glass volumetric pipettes 5 ml of water and 30 mL of acetonitrile was added.

Liquid-Liquid Partition

Buffer salt mixture (4 g of magnesium sulfate anhydrous, 1 g of sodium chloride, 1 g of sodium citrate and 0.5 g of disodium citrate sesquihydrate) was added and the centrifuge tube was closed and shaken vigorously by QuEChERS Hand Motion Shaken for 2 min.

The extract was centrifuged at 4000 rpm for 5 min.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of CPIA using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Liver | CPIA | 0.01 | 96.24 | 3.16 | First mass transition |
|  |  | 0.1 | 97.02 | 6.15 |  |
|  |  | 0.01 | 96.09 | 6.67 | Second mass transition |
|  |  | 0.1 | 97.28 | 2.69 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of CPIA residues in liver

|  | CPIA |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.003 to 0.3 mg/kg First mass transition y=130812.7926x-221.5110 R2=0.9978 Second mass transition y=2758.8742x-8.5910 R2=0.9969 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.01 mg/kg LOD = 0.003 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of CPIA in liver.

Description of Methods for the Analysis of Soil (KCP 5.2)

Analytical method 10

Method validation

| Comments of zRMS: | The method validation has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.20 |
| --- | --- |
| Report | Terrestrial Soil Dissipation of Esfenvalerate after one Application to Bare Soil in Southern Europe. J. Hitchens. 2019, Report No. YV/18/009 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The purpose of this study was to validate the analytical method to determine the degradation behavior of esfenvalerate under field condition after one application of Esfenvalerate 2.5% EC to bare soil.

Residues of CONH2-fenvalerate were extracted using the QuEChERS extraction  procedure using water and acetonitrile as the extraction solvents. A portion of the extract was transferred to an autosampler vial for analysis of CONH2-fenvalerate by LC-MS/MS monitoring two ion mass transition.

Residues of four fenvalerate isomers and CONH2-fenvalerate were extracted using the QuEChERS extraction procedure using water and acidified acetonitrile (0.1% formic acid in acetonitrile) as the extraction solvents. A portion of the extract was transferred to an autosampler vial for analysis of CONH2-fenvalerate by LC-MS/MS.

A further portion (5mL) of the extract was concentrated under nitrogen blow down to just below 0.5 mL then made to a final volume of 0.5 mL with acidified acetonitrile (0.1% formic acid in acetonitrile). This concentrated extract was transferred to an autosampler vial for analysis of esfenvalerate (fenvalerate SS isomer) and fenvalerate isomers (SR,RS, and RR) by LC-MS/MS monitoring tow ion mass transitions.

Results and discussions

Table A 9:	Recovery results from method validation of CONH2 and Fenvalerate (sum of constituent isomers including Esfenvalerate) using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 6) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Soil | CONH2-fenvalerate Analytical method 1 | 0.002 | 94 | 12.5 | First mass transition |
|  |  | 0.02 | 100 | 2.3 |  |
|  |  | 0.002 | 96 | 8.7 | Second mass transition |
|  |  | 0.02 | 96 | 8.7 |  |
|  | CONH2-fenvalerate Analytical method 2 | 0.002 | 96 | 1.1 | First mass transition |
|  |  | 0.02 | 95 | 1.4 |  |
|  |  | 0.002 | 100 | 2.0 | Second mass transition |
|  |  | 0.02 | 96 | 1.9 |  |
|  | Esfenvalerate | 0.005 | 82 | 13.0 | First mass transition |
|  |  | 0.05 | 89 | 5.9 |  |
|  |  | 0.005 | 76 | 3.7 | Second mass transition |
|  |  | 0.05 | 86 | 4.7 |  |
|  | R,S-Fenvalerate | 0.01 | 88 | 6.5 | First mass transition |
|  |  | 0.1 | 85 | 2.2 |  |
|  |  | 0.01 | 88 | 6.7 | Second mass transition |
|  |  | 0.1 | 76 | 1.9 |  |
|  | R,R-Fenvalerate | 0.01 | 87 | 6.6 | First mass transition |
|  |  | 0.1 | 85 | 5.4 |  |
|  |  | 0.01 | 87 | 6.9 | Second mass transition |
|  |  | 0.1 | 83 | 2.4 |  |
|  | S,R-Fenvalerate | 0.01 | 89 | 11.5 | First mass transition |
|  |  | 0.1 | 85 | 1.9 |  |
|  |  | 0.01 | 97 | 5.1 | Second mass transition |
|  |  | 0.1 | 85 | 1.5 |  |

Table A 10:	Characteristics for the method validation of CONH2 and Fenvalerate (sum of constituent isomers including Esfenvalerate) residues in soil

|  | CONH2-fenvalerate | Fenvalerate (sum of constituent isomers including Esfenvalerate) |
| --- | --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |  |
| Calibration (type, number of data points) | Method 1 First mass transition y=3.57e+004x+1.79e+003 R=0.9993 Second mass transition y=2.15e+005x+2.29e+003 R=0.9993 Method 2 First mass transition y=2.3e+005+2.18e+003 R=0.9994 Second mass transition y=3.82e+004x+1.87e+003 R=0.9987 | Esfenvalerate First mass transition y=2.65e+003x+1.17e+004 R=0.9999 Second mass transition y=1.18e+003x+4.28e+003 R=0.9996 R,S-Fenvalerate First mass transition y=2.01e+003x+4.94e+003 R=0.9997 Second mass transition y=900x+3.07e+003 R=0.9998 R,R,-Fenvalerate First mass transition y=2.27e+003x+1.1e+004 R=0.9995 Second mass transition y=977x+4.23e+003 R=0.9998 S,R-Fenvalerate First mass transition y=2.12e+003x+7.43e+003 R=0.9998 Second mass transition y=951x+4.39e+003 R=0.9997 |
| Assessment of matrix effects is presented | Yes |  |
| Limit of determination/quantification | LOQ = 0.002 mg/L | LOQ = 0.005 mg/L |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of CONH2 and Fenvalerate (sum of constituent isomers including Esfenvalerate) in soil.

Description of Methods for the Analysis of Water (KCP 5.2)

Analytical method 11

Method validation

| Comments of zRMS: | The validation of the method has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.21 |
| --- | --- |
| Report | Validation of the analytical method to determine Esfenvalerate in Water samples. D. Longhi, 2019, Report No. BPL-STUDY-18-000013 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The aim of the study was the validation of an analytical method capable of quantitating Esfenvalerate (SS-Fenvalerate enantiomer) in surface and drinking water. The analyses were carried out using a GC-MS-MS system with a method capable of separating the Fenvalerate diastereoisomers.

Sample extraction.

A volume of 1 L of water sample was taken and put in a separatory funnel. Then, 30 mL of dichloromethane were added and the water extracted vigorously shaking the funnel for 1 minute. The phases were allowed to separate. After that, the bottom organic phase was taken, filtering it with sodium sulphate anhydrous. The upper aqueous phase was then extracted other 4 times in the same manner, combining the organic extracts together. At the end of the extraction phase, the combined organic extract was put in a flask and reduced to about 5-10 mL with a vacuum evaporate system (rotavapor). Then, the extract was transferred in a vial (rinsing the surface with fresh dichloromethane) and reduced to 0.5 mL with a nitrogen stream. The final extract was finally analysed with a GC-MS/MS system.

Results and discussions

Table A 9:	Recovery results from method validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (μg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Surface water | Esfenvalerate | 0.001 | 84.2 | 9.4 | First mass transition |
|  |  | 0.01 | 94.3 | 13.0 |  |
|  |  | 0.001 | 91.1 | 16.1 | Second mass transition |
|  |  | 0.01 | 96.5 | 15.6 |  |
| Drinking water |  | 0.001 | 91.1 | 13.0 | First mass transition |
|  |  | 0.01 | 101 | 7.8 |  |
|  |  | 0.001 | 95.9 | 14.4 | Second mass transition |
|  |  | 0.01 | 99.6 | 8.6 |  |

Table A 10:	Characteristics for the analytical method validation of Esfenvalerate residues in water (drinking and surface)

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.599-29.967 μg/L Surface water First mass transition y=3062.977753x+603.250648 R2=0.99945901 Second mass transition y=913.024829x+270.997256 R2=0.99974027 Drinking water First mass transition y=2984.739062x+605.985257 R2=0.99830793 Second mass transition y=940.657200x+311.719428 R2=0.9979999 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.001 μg/L LOD = 0.0003 μg/L |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in water (drinking and surface).

Independent laboratory validation

| Comments of zRMS: | The validation of the method has been accepted. This is not an ILV. |
| --- | --- |

| Reference: | KCP 5.2.22 |
| --- | --- |
| Report | Validation of the analytical method to determine Esfenvalerate in drinking water samples. E. Nowakowska-Bogdan, 2020, Report No. 169/2019 |
| Guideline(s): | SANCO/825/00 rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | GLP |
| Acceptability: | Yes |

Materials and methods

The aim of the study was the validation of an analytical method capable of quantitating Esfenvalerate (SS-Fenvalerate enantiomer) in surface and drinking water.

Samples extraction

A volume of 1 L of water sample was taken and put in a separatory funnel. Then, 30 mL of dichloromethane were added and the water extracted vigorously shaking the funnel for 1 minute. The phases were allowed to separate. After that, the bottom organic phase was taken, filtering it with sodium sulphate anhydrous. The upper aqueous phase was then extracted other 4 times in the same manner, combining the organic extracts together. At the end of the extraction phase, the combined organic extract was put in a flask and reduced to about 5-10 mL with a vacuum evaporate system (rotavapor). Then, the extract was transferred in a vial (rinsing the surface with fresh dichloromethane) and reduced to 0.5 mL with a nitrogen stream. The final extract was finally analysed with a GC-MS/MS system.

Results and discussions

Table A 7:	Recovery results from the analytical method used for independent laboratory validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (μg/L) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Drinking water | Esfenvalerate | 0.001 | 84.44 | 3.19 | First mass transition |
|  |  | 0.01 | 96.08 | 3.10 |  |
|  |  | 0.001 | 82.73 | 2.70 | Second mass transition |
|  |  | 0.01 | 95.66 | 1.05 |  |

Table A 8:	Characteristics for the analytical method used for independent laboratory validation of Esfenvalerate residues in drinking water

|  | Esfenvalerate |
| --- | --- |
| Specificity | Results obtained proved the no presence of the peaks corresponding to esfenvalerate in the blank samples, being observed in this way the acceptance criterion established in the Protocol (signal of the blank lower than 30% of LOQ signal). The method is specific |
| Calibration (type, number of data points) | 5 points 0.599-29.967 μg/L First mass transition y=333.444264x-197.767605 R2=0.99900397 Second mass transition y=104.415046x-57.398884 R2=0.99907736 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.001 μg/L LOD = 0.0003 μg/L |

Conclusion

According to SANTE/2020/12830, rev.1 method was successfully validated and is suitable for determination of residues of Esfenvalerate in drinking water.

Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted

Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

Analytical method 12

Method validation

| Comments of zRMS: | The validation of the method has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.23 |
| --- | --- |
| Report | Validation of the analytical method to determine Esfenvalerate in body fluids (blood, plasma, serum). E. Signore, 2018, Report No. RAU-021-18 |
| Guideline(s): | SANCO/825/00 – rev. 8.1 SANCO/3029/99 rev. 4 |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

The objective of this Study is to validate the analytical method to determine Esfenvalerate in body fluids of blood, plasma and serum samples of bovine origin.

In order to demonstrate the validity of the analytical method, linearity was evaluated in each analytical sequence; selectivity, accuracy, precision, specificity, limit of quantification and limit of detection were evaluated for each matrix on the following samples:

Blood samples

- 2 untreated samples

- 5 untreated samples fortified at 0.005 mg/Kg (target LOQ)

- 5 untreated sample fortified at 0.05 mg/Kg (10 x LOQ)

Plasma samples

- 2 untreated samples

- 5 untreated samples fortified at 0.005 mg/Kg (target LOQ)

- 5 untreated sample fortified at 0.05 mg/Kg (10 x LOQ)

Serum samples

- 2 untreated samples

- 5 untreated samples fortified at 0.005 mg/Kg (target LOQ)

- 5 untreated sample fortified at 0.05 mg/Kg (10 x LOQ)

Sample preparation

The preparation of Plasma and Serum follows the reccomendation of the procedure of Transfusion hematology area of the Ospedale Maggiore Policlinico di Milano:

- the Plasma sample ES/PL/01 was obtained by centrifuging the blood (containing an anticoagulant) for 10 minutes at 5000 RPM; the obtained supernatant phase was taken and centrifuged again at 5000 RPM for 5 minutes.

- the Serum sample ES/SE/01 was obtained by letting the blood (without the anticoagulant) settle for approx 12 hours, then by centrifuging for 10 minutes at 5000 RPM and taking the obtained supernatant phase.

Plasma and Serum samples ES/PL/01 and ES/SE/01 were stored after the preparation in the Refrigeration Unit of the Test Facility at - 18°C, following SOPs MNG012, last version.

The blood for Blood sample ES/BL/01 is used as it is.

Blood sample ES/BL/01 is stored in the Refrigeration Unit of the Test Facility at + 4°C, following SOPs MNG012, last version.

Extraction procedure

Blood and Plasma matrices

5 g of homogenized sample were weighed in a screw capped falcon and 5 mL of ethyl acetate were added. Then the falcon was vigorously shaken for 3 mintues and vortexed for not less than 5 minutes and taken in the ultrasonic bath for 3 minutes.

After the samples were centrifuged for 5 minutes at 5000 RPM. Successively, the ethyl acetate was transferred in an Eppendorf and centrifuges again at 14000 RPM for 5 minutes.

Finally the centrifuged extract was put in a vial and it ws ready to be analysed.

Serum matrix

5 g of homogenized sample were weighed in a screw capped falcon and 5 mL of ethyl acetate were added. Then the falcon was vigorously shaken for 3 minutes and vortexed for not less than 3 minutes.

3 g of sodium chloride and 3 g of magnesium sulfate anhydrous were added and the samples were vortexed again for 3 minutes.

Afterwards the samples were centrifuged for 5 minutes at 5000 RPM.

Successively, the ethyl acetate pahse was transferred in an Eppendorf and centrifuges again at 14000 RPM for 5 minutes.

Finally the centrifuged extract was put in a vial and it was ready to be analysed.

Results and discussions

Table A 9:	Recovery results from method 	validation of Esfenvalerate using the analytical method

| Matrix | Analyte | Fortification level (mg/kg) (n = 5) | Mean recovery (%) | RSD (%) | Comments |
| --- | --- | --- | --- | --- | --- |
| Blood | Esfenvalerate | 0.005 | 79.69 | 10.22 | First mass transition |
|  |  | 0.05 | 85.06 | 6.65 |  |
|  |  | 0.005 | 76.77 | 9.11 | Second mass transition |
|  |  | 0.05 | 85.43 |  |  |
| Plasma |  | 0.005 | 89.98 | 5.87 | First mass transition |
|  |  | 0.05 | 89.66 | 7.51 |  |
|  |  | 0.005 | 89.19 | 7.75 | Second mass transition |
|  |  | 0.05 | 87.74 |  |  |
| Serum |  | 0.005 | 83.87 | 15.67 | First mass transition |
|  |  | 0.05 | 74.99 | 5.48 |  |
|  |  | 0.005 | 88.15 | 13.44 | Second mass transition |
|  |  | 0.05 | 73.60 |  |  |

Table A 10:	Characteristics for the analytical method validation of Esfenvalerate residues in body fluids

|  | Esfenvalerate |
| --- | --- |
| Specificity | No significant peaks (<30% LOQ) were detected at RT of the target analyte in the Blank and Test Solution compared to the Spiked Test Solution. The method is specific |
| Calibration (type, number of data points) | 5 points 0.0015 mg/kg to 0.10 mg/kg Blood First mass transition y=6306.701708x+4641.655678 R2=0.99672810 Second mass transition y=1613.180103x+1235.194417 R2=0.99708245 Plasma First mass transition y=7301.748121x-6176.933193 R2=0.99573936 Second mass transition y=1876.477129x-985.313433 R2=0.99511311 Serum First mass transition y=9588.982020x-7868.268849 R2=0.99037479 Second mass transition y=1569.076484x-2317.994865 R2=0.99217202 |
| Assessment of matrix effects is presented | Yes |
| Limit of determination/quantification | LOQ = 0.005 mg/kg LOD = 0.0015 mg/kg |

Conclusion

According to SANTE/2020/12830, rev.1 the method was successfully validated and is suitable for determination of residues of Esfenvalerate in body fluids.

Other Studies/ Information

A 2.1.2.7.1	Analytical method validation 21/FSL/09/V (the method applied in the residue studies in the present B7)

| Comments of zRMS: | The validation of the method has been accepted. |
| --- | --- |

| Reference: | KCP 5.2.24 |
| --- | --- |
| Report | Validation of the method for the determination of the residues of fenvalerate (any ratio of constituent isomers including esfenvalerate) in head cabbage, strawberry and wheat by gas chromatography. Markowicz, A; 2022; Study Code: 21/FSL/09/V, Food Safety Laboratory, The National Institute of Horticultural Research |
| Guideline(s): | Regulation (EC) No 1107/2009 of the European Parliament and of the Council of 21 Oct 2009 concerning the placing of plant protection products on the market and repeating Council Directives 79/117/EEC and 91/414/EEC. EC Guidance Document on Pesticide Analytical Methods for Risk Assessment and Post-approval Control and Monitoring Purposes. SANTE/2020/12830, Rev.1, (2021). OECD: Guidance Document on Pesticide Residue Analytical Methods ENV/JM/MONO(2007)17, (2007). |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

The objective of the study was validation of the method for determination of Fenvalerate (any ratio of constituent isomers (RR, SS, RS, SR) including Esfenvalerate) in head cabbage (head) as representative water matrix, strawberry (fruit) as representative acid and water matrix and wheat (plant, grain and straw) as representative starch and/or protein and low water and fat matrix in order to provide a method for residue analysis, and to meet residue regulatory requirements. To achieve the objective appropriate analytical method for determination of Fenvalerate was validated in accordance to the guidance document SANTE/2020/12830, Rev.1 [1] of the European Commission and to meet residue regulatory requirements. The validated limit of quantification for Fenvalerate is 0.01 mg/kg.

The general principles of the analytical procedure were based on the normalized method EN 15662:2018 [2]. In brief, the head cabbage, strawberry (fruit) and wheat (plant, grain and straw) samples without target pesticide residues were extracted with acetonitrile. After addition of a buffer-salt mixture containing magnesium sulphate, sodium chloride and sodium citrate the extract was shaken. Following centrifugation, an aliquot of the upper acetonitrile phase was cleaned by primary secondary amine (PSA) and dehydrated by magnesium sulphate addition. Quantification of Fenvalerate was performed with the help of internal standard triphenyl phosphate (ISTD, TPP).

Selectivity and Confirmation of Residue Identity

Quantification was performed by use of highly selective gas chromatography coupled with tandem mass spectrometry (GC-MS/MS). Two selected ion mass transitions were evaluated in order to demonstrate that the method achieves a high level of selectivity. The retention time of analyte in extracts corresponds to that of the calibration standards with a tolerance of < ± 0.1 min. Confirmation ion ratio for Fenvalerate in all samples were within ± 30 % of the average found for the standard.

No significant interference above 30 % of LOQ was detected in any of the reagent blank or control specimen extracts for head cabbage, strawberry and wheat, so that a highly level of selectivity was demonstrated and an additional confirmatory method is not necessary.

Matrix Effects

Matrix effects on the detection of Fenvalerate in extracts of head cabbage, strawberry and wheat (plant and straw) were higher than 20% and thus considered significant, according to SANTE guidelines. Matrix effects on the detection of Fenvalerate in extracts of wheat grain were lower than 20% and thus considered insignificant, according to SANTE guidelines. Determination of analyte was performed using matrix-matched calibration standards.

Linearity

The correlation between the injected concentration of analyte standard and detector response was demonstrated to be linear by single determination of matrix-matched calibration standards at eight concentration levels ranging from 0.002 µg/mL to 0.5 µg/mL for head cabbage and strawberry, at nine concentration levels ranging from 0.001 µg/mL to 0.5 µg/mL for wheat (plant and grain) and at ten concentration levels ranging from 0.0005 µg/mL to 0.5 µg/mL for wheat (straw). This ranges correspond from 0.002 mg/kg to 0.5 mg/kg for head cabbage and strawberry, from 0.002 mg/kg to 1.0 mg/kg for wheat (plant and grain) and from 0.0025 mg/kg to 2.5 mg/kg for wheat (straw). Thus, covers the range from no more than 30 % of the LOQ and it should be at least + 20 % of the highest analyte concentration level detected in samples. The calibration curves obtained for both ion mass transitions of Fenvalerate were linear with the coefficients of correlation (R) greater than 0.99. Linear regression was performed with 1/x weighting.

Quantification

Quantification was performed by using weighted (1/x) linear regression with the help of the internal standard Triphenyl phosphate (TPP) as described in the section “Linearity”.

System Suitability Test

System suitability test (SST) was performed to verify the performance of GC-MS/MS system and to ensure its adequacy for Fenvalerate determination in head cabbage, strawberry and wheat. %RSD of Fenvalerate peak area for the first and second ion mass transition was found within the acceptable range (≤ 20%) indicating that the system was suitable for the intended analysis.

Stability of Analyte in the Final Dilution

Recoveries of the fortified samples within the acceptable range of 70-110% obtained with calibration solutions and the use of bracketing standards at LOQ level (SST samples) to ensure integrity of the analytical sequence sufficiently demonstrate the stability of analyte in the final dilution.

Accuracy and Precision

Accuracy was determined by fortification of control samples with known amounts of the reference item (Fenvalerate) and subsequent determination of the recoveries when applying the extraction procedure. Precision was determined by repeatability (relative standard deviation – RSD). The mean recovery values at the fortification levels of 0.01 mg/kg and 0.1 mg/kg for both ion mass transitions were all in the range 70 – 110 % and thus comply with the standard acceptance criteria of the guidance document SANTE/2020/12830, Rev.1 [1]. All precision values at the fortification levels of 0.01 mg/kg and 0.1 mg/kg for both ion mass transitions were < 20%. Mean recovery and precision results for both ion mass transitions of Fenvalerate are shown in Table 1.

Table 1: Summary of recovery results

Table 1: Summary of recovery results (cont.)

Limit of Quantification (LOQ) and Limit of Detection (LOD)

The limit of quantification (LOQ) is the lowest validated fortification level and was thus successfully established at 0.01 mg/kg for both ion mass transitions of Fenvalerate in head cabbage, strawberry (fruit) and wheat (plant, grain and seed). The limit of detection (LOD) for Fenvalerate was set at 0.002 mg/kg for head cabbage, strawberry (fruit) and wheat (plant and grain), and at 0.0025 mg/kg for wheat (straw) which is < 30% of the LOQ.

Conclusions

The method was shown to be highly selective, as it includes two parent-daughter ion transitions for Fenvalerate and it yields accurate and repeatable results. The limit of quantification (LOQ) was established at 0.01 mg/kg for analyte, interfering signals in control specimen were negligible, and thus the limit of detection (LOD) is 0.002 mg/kg for head cabbage, strawberry (fruit) and wheat (plant and grain) and 0.0025 mg/kg for wheat (straw).

It is concluded that method fulfils the requirements as defined in EC Guidance document on residue analytical methods (SANTE/2020/12830, Rev.1 [1]) and is, applicable as enforcement and data generation method for determination of Fenvalerate (any ratio of constituent isomers (RR, SS, RS, SR) including Esfenvalerate) in head cabbage, strawberry and wheat.