REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: SHA 148000 A Product name(s): METROPOLITAN Chemical active substance: Metazachlor, 500 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Sharda Cropchem Limited Submission date: October 2022 Evaluation date: July 2023 MS Finalisation date: dd/mm/yyyy

Version history

| When | What |
| --- | --- |
| July 2023 | Version evaluated by zRMS PL |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	Metazachlor	5

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	6

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	6

1.5	Type and code of the plant protection product (KCP 1.5)	6

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	15

4	Section 4: Further information on the plant protection product	16

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	16

Appendix 1	Lists of data considered in support of the evaluation	18

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	20

A 2.1	Metazachlor	20

State whether or not submitted data are sufficient for evaluation. Data gaps and conditions for registration should be listed, if appropriate.

Sufficient data on identity, physical and chemical properties and other information are not available for the plant protection product and the contained technical active substance(s).

Noticed data gaps are:

- A two-year storage stability study at ambient temperature shall be provided for evaluation when available. It can be assessed in post registration.

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Name: 	Sharda Cropchem Ltd.

Address:		Prime Business Park

Dashrathlal Joshi Road

Vile Parle (West)

Mumbai – 400 056

India

Phone number: 	+ 91 22 6678 2800

Fax number: 	+ 91 22 6678 2828/ 2808

Email: 	shardaint@vsnl.com

regn@shardaintl.com

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Metazachlor

| Metazachlor Metazachlor | min. 94.0 g/kg (Regulation (EU) No. 540/2011) min. 987.7 g/kg (Sharda source) |
| --- | --- |

| Toluene | max. 0.1 g/kg (Regulation (EU) No. 540/2011) |
| --- | --- |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Please refer to Registration Report Part A for the relevant country (or) |
| --- | --- |
| Trade name: | METROPOLITAN |
| Company code number: | SHA 148000 A |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Metazachlor | 500 g/L | 475.00 – 525.00 g/L (± 5 %) | 506.23 g/L | 44.03 |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

** 	Based on the density of the formulation = 1.1498 g/L

Table 1.4-2:	Relevant impurities

| Relevant impurity | Maximum content (g/L or g/kg) |
| --- | --- |
| Toluene | 0.025 g/L |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-3:	Information on Metazachlor

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Metazachlor |
| CAS No. | 67129-08-2 |
| EC No. | 266-583-0 |
| CIPAC No. | 411 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Suspension concentrate | [Code: SC] |
| --- | --- |

## Function (KCP 1.6)

Metazachlor 50% SC is intended to be used as a herbicide.

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.  The appearance of the product is that of off white liquid, with  characteristic odour. It is not explosive, has no oxidising properties. The product is not flammable/has a flash point of 93.4 °C. It has a self ignition temperature above 350 °C. In aqueous solution, it has a pH value around 6.14 at 25 °C. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0 °C and 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The shelf life study is on-going and will be provided as soon as available. Its technical characteristics are acceptable for a Suspension concentrate formulation.

The intended concentration of use is 0.375% to 1.0%.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

Neither classification or labelling is relevant for this section.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No risk and safety phrases are relevant for this section.

Compliance with FAO specifications:

The product SHA 148000 A complies with FAO specifications.

Formulation used for tests

The formulation used for tests is the one cited in dRR Part C.

Table 2-4:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | OPPTS 830.6302 OPPTS 830.6303 OPPTS 830.6304 | Metazachlor 50% SC (Batch No. SCL-15602) | Off White liquid with characteristic odor. | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted Odor is not required by the reg 284/2013 |
| Explosive properties (KCP 2.2.1) | EEC A. 14 | Metazachlor 50% SC (Batch No. SCL-15602) | The test item has no explosive properties. | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted |
| Oxidizing properties (KCP 2.2.2) | EEC A. 21 | Metazachlor 50% SC (Batch No. SCL-15602) | The test item has no oxidizing properties. | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Acceptrd |
| Flash point (KCP 2.3.1) | EEC A. 9 | Metazachlor 50% SC (Batch No. SCL-15602) | T = 93.4℃ | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted |
| Flammability (KCP 2.3.2) | - | - | Not flammable. Please refer to KCP 2.3.1 | - | - |  |
| Self-heating (KCP 2.3.3) | EEC A. 15 | Metazachlor 50% SC (Batch No. SCL-15602) | Not igniting up to 350℃ | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted |
| Acidity or alkalinity and pH (KCP 2.4.1) | - | - | Not required since pH is in the range of 4-10. | - | - |  |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | Metazachlor 50% SC (Batch No. SCL-15602) | pH 1% = 6.14 neat = 5.11 | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted |
| Viscosity (KCP 2.5.1) | CIPAC MT 192 | Metazachlor 50% SC (Batch No. SCL-15602) | RPM; Density Dynamic viscosity at 20℃ (cP); Density Dynamic viscosity at 40℃ (cP). 20; 171.1; 130.9. 30; 168.0; 125.0. 50; 157.8; 114.7. 60; 149.3; 105.7. | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted. The lowest calculated kinematic viscosity value at 40℃ 91.9 mm2/s. So, there is no need for considerering a Category 1 aspiration hazard. |
| Surface tension (KCP 2.5.2) | OECD 115 | Metazachlor 50% SC (Batch No. SCL-15602) | 1.15% w/v solution = 59.343 mN/m Neat = 59.349 mN/m | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted the product is surface active |
| Relative density (KCP 2.6.1) | CIPAC MT 3.2 | Metazachlor 50% SC (Batch No. SCL-15602) | 1.1498 g/mL | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted |
| Bulk density (KCP 2.6.2) | - | - | Not relevant for SC formulation. | - | - |  |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.4 OCSPP 830.6302 OCSPP 830.6303 OCSPP 830.6304 CIPAC MT 75.3 CIPAC MT 185 CIPAC MT 148.1 CIPAC MT 184.1 CIPAC MT 160 CIPAC MT 187 SANCO/3030/99 rev. 5 | Metazachlor 50% SC (Batch No. SCL-15602) | Test; Initial; After 14 days at 54℃. Appearance; Off white liquid with characteristic odor; Off white liquid with characteristic odor. Metazachlor content; 499.9 g/L; 499.9 g/L. Relevant impurity toluene content; 0.0250 g/L; 0.0251 g/L. pH 1%; 6.14; 6.10. pH neat; 5.11; 5.09. Wet sieve test (%retained on 0.75 μm sieve); 0.0837; 0.1220. Pourability; R = 0.6589% R’ = 0.0488%; R = 0.6573% R’ = 0.0730%. Suspensibility (in Standard Water D); 1.15% w/v 97.9571% 0.44% w/v 98.0105%; 1.15% w/v 97.0801% 0.44% w/v 96.9840%. Spontaneity of Dispersion %w/w; 97.8819; 97.1597. Particle size distribution; 10 % of sample (µm); 50 % of sample (µm); 90 % of sample (µm); 100 % of sample (µm). 1.191; 4.237; 68.685; 157.296.; 10 % of sample (µm); 50 % of sample (µm); 90 % of sample (µm); 100 % of sample (µm). 1.223; 4.402; 72.625; 157.455.. Stability of packaging; No perforations, darkening, leakages and rustiness; No perforations, darkening, leakages and rustiness. | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted A HDPE container remained intact and resistant after storage. No interaction (deformation, perforation, darkenining, leakage, weight change) of packaging with formulation was observed.All the physicochemical parameters before and after storage are accepted. Analytical method was accepted in the B.5 section |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | - | - | Not relevant. | - | - |  |
| Minimum content after heat stability testing (KCP 2.7.3) | CIPAC MT 46.4 SANCO/3030/99 rev 5 | Metazachlor 50% SC (Batch No. SCL-15602) | Appearance; Off white liquid with characteristic odor; Off white liquid with characteristic odor. Metazachlor content; 499.9 g/L; 499.9 g/L. | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Not required |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 | Metazachlor 50% SC (Batch No. SCL-15602) | Test; Initial; After 7 days at 0℃. Suspensibility (in Standard Water D); 1.15% w/v 97.9571% 0.44% w/v 98.0105%; 1.15% w/v 97.2115% 0.44% w/v 97.1680%. Wet sieve test (%retained on 0.75 μm sieve); 0.0837; 0.1087. The product is stable after 7 days at 0℃ | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted |
| Ambient temperature shelf life (KCP 2.7.5) | - | - | Study on-going. | - | - | On-going This study shall be provided for evaluation when available. |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Not relevant. | - | - |  |
| Wettability (KCP 2.8.1) | - | - | Not relevant for SC formulation. | - | - |  |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.3 | Metazachlor 50% SC (Batch No. SCL-15602) | 1.15% w/v: 0 mL after 1 min. | Y | G. B. Azeema, 2022 Report No. 10874/2022 | Accepted |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184.1 | Metazachlor 50% SC (Batch No. SCL-15602) | 1.15% w/v in Standard Water D 97.9571% 0.44% w/v in Standard Water D 98.0105% | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted It was done using analytical method used for analysing Metazachlor in the PPPP accepted in B5 section). Content of the active substance in suspension dilution was within the method’s linearity |
| Spontaneity of dispersion (KCP 2.8.3.2) | CIPAC MT 160 | Metazachlor 50% SC (Batch No. SCL-15602) | 97.8819 | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted |
| Dispersion stability (KCP 2.8.3.3) | - | - | Not relevant for SC formulation. | - | - |  |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not relevant for SC formulation. | - | - |  |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 187 | Metazachlor 50% SC (Batch No. SCL-15602) | 10 % of sample (µm); 50 % of sample (µm); 90 % of sample (µm); 100 % of sample (µm). 1.191; 4.237; 68.685; 157.296. | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | Metazachlor 50% SC (Batch No. SCL-15602) | 0.0837 %retained on 0.75 μm sieve | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted |
| Dust content (KCP 2.8.5.2.1) | - | - | Not relevant for SC formulation. | - | - |  |
| Particle size of dust (KCP 2.8.5.2.2) | - | - | Not relevant for SC formulation. | - | - |  |
| Attrition (KCP 2.8.5.3) | - | - | Not relevant for SC formulation. | - | - |  |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not relevant for SC formulation. | - | - |  |
| Emulsifiability (KCP 2.8.6.1) | - | - | Not relevant for SC formulation. | - | - |  |
| Emulsion stability (KCP 2.8.6.2) | - | - | Not relevant for SC formulation. | - | - |  |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Not relevant for SC formulation. | - | - |  |
| Flowability (KCP 2.8.7.1) | - | - | Not relevant for SC formulation. | - | - |  |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148.1 | Metazachlor 50% SC (Batch No. SCL-15602) | R = 0.6589% R’ = 0.0488% | Y | G. B. Azeema, 2022 Report No. 10875/2022 | Accepted |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not relevant for SC formulation. | - | - |  |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | Not intended to be tank mixed. | - | - |  |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | Not intended to be tank mixed. | - | - |  |
| Adhesion to seeds (KCP 2.10.1) | - | - | Not relevant | - | - |  |
| Distribution to seed (KCP 2.10.2) | - | - | Not relevant | - | - |  |
| Other/special studies (KCP 2.11) | - | - | Not relevant | - | - |  |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

RMS comment:

Based on the accelerated study done using a HDPE rigid contained, all the proposed below packs are accepted. In case of aqueous formulation like SC all rigid plastic packaging used for plant protection products are allowed.

Table 4.1-1:	Packaging information for 0.250 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (HDPE/PA) |
| Shape/size: | Round bottle / approx. 61 mm diameter x 138.8 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 0.500 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (HDPE/PA) |
| Shape/size: | Round bottle / approx. 69 mm diameter x 199.8 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 1 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (HDPE/PA) |
| Shape/size: | Round bottle / approx. 88.5 mm diameter x 239.5 mm |
| Opening: | 41.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 5 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (HDPE/PA) |
| Shape/size: | jerrycan / approx. 136 mm x 192 mm x 285 mm |
| Opening: | 54.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 10 liter bottle

| Type | Description |
| --- | --- |
| Material: | COEX (HDPE/PA) |
| Shape/size: | jerrycan / approx. 174 mm x 226 mm x 368 mm |
| Opening: | 54.7 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-6: Packaging information for 20 litre jerrycan

| Type | Description |
| --- | --- |
| Material: | COEX (HDPE/PA) |
| Shape/size: | jerrycan / approx. 245.0 mm x 294.0 mm x 400.0 mm |
| Opening: | 55.8 mm inner diameter |
| Closure: | HDPE screw cap |
| Seal: | Induction heat seal |
| Manner of construction | coextrusion |
| UN/ADR | compliant |

- Lists of data considered in support of the evaluation

Tables considered not relevant can be deleted as appropriate.

MS to blacken authors of vertebrate studies in the version made available to third parties/public.

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1 KCP 2.2.2 KCP 2.3.1 KCP 2.3.3 KCP 2.5.1 KCP 2.5.2 KCP 2.6.1 KCP 2.8.2 | G. B. Azeema | 2022 | Physical chemical studies of Metazachlor 50% SC. Report No. 10874/2022 GLP Unpublished | N | Sharda Cropchem Limited |
| KCP 2.4.2 KCP 2.7.1 KCP 2.7.3 KCP 2.7.4 KCP 2.8.3.1 KCP 2.8.3.2 KCP 2.8.5.1.1 KCP 2.8.5.1.2 KCP 2.8.7.2 | G. B. Azeema | 2022 | Accelerated Storage Stability Study of Metazachlor 50% SC. Report No. 10875/2022 GLP Unpublished | N | Sharda Cropchem Limited |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

The following tables are to be completed by MS.

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Additional data on the physical, chemical and technical properties of the active substance

  - Metazachlor

No new data submitted