FINAL REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: CHR/RW/PROH 100 WG Product name(s): Heksal 100 WG, Jamur 100 WG Chemical active substance: Prohexadione calcium, 100 g/kg

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT

Applicant: Innvigo Sp. z o.o. Submission date: December 2021 MS Finalisation date: 22/04/2022

Version history

| When | What |
| --- | --- |
| September 2021 | Dossier sent for evaluation to Merit Mark (PL) |
| December 2021 | zRMS finalised evaluation |
| April 2022 | Final version prepared by zRMS after Commenting period |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	4

1.2.3.1	Prohexadione calcium	4

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	4

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	5

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	5

1.5	Type and code of the plant protection product (KCP 1.5)	5

1.6	Function (KCP 1.6)	5

2	Section 2: Physical, chemical and technical properties of the plant protection product	6

3	Section 3 is presented as a separate document	34

4	Section 4: Further information on the plant protection product	35

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	35

Appendix 1	Lists of data considered in support of the evaluation	57

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	66

Evaluator comments:

The text highlighted in grey was provided by the evaluator.

New and additional informations are highlighted in yellow.

Sufficient data on identity, physical and chemical properties and other information are not available for the plant protection product and the contained technical active substance(s).

Noticed data gaps are:

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name:	Innvigo Sp. z o.o

Address:	Innvigo Sp. z o.o

Al. Jerozoimskie 178 St.

02-486 Warsaw,

Poland

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Prohexadione calcium

According to the SANCO/11023/2011 Rev 2

| Prohexadione calcium | min. 890 g/kg |
| --- | --- |

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Heksal 100 WG/ Jamur 100 WG |
| --- | --- |
| Company code number: | CHR/RW/PROH 100 WG |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-1:	Active substance(s) and variant(s) of the active substance(s)

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content** (%w/w) |
| --- | --- | --- | --- | --- |
| Prohexadione calcium | 100 g/kg | 90 – 110 g/kg | 105.3 | 10.53 % |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

Table 1.4-2:	Relevant impurities

| Relevant impurity | Maximum content (g/L or g/kg) |
| --- | --- |
| Confidential information or data are provided separately (Part C). |  |

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-3:	Information on Prohexadione calcium

| Type | Name/Code Number |
| --- | --- |
| ISO common name | Prohexadione calcium N/A |
| IUPAC name | calcium; 4-(1-oxidopropylidene)-3,5-dioxocyclohexane-1-carboxylate |
| CAS No. | 127277-53-6 |
| EC No. | N/A 603-193-5 |
| CIPAC No. | 567.020 |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

Table 1.4-4:	Information on safeners/ synergists / co-formulant

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Water dispersible granules | [Code: WG] |
| --- | --- |

## Function (KCP 1.6)

Plant growth regulator

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.  The appearance of the product is that of light brown, with a delicate odour. It is not explosive, has no oxidising properties. The product is not flammable. No thermal change of sample above 400 °C was observed. In aqueous solution, it has a pH value for 1% (w/v) suspension is around 6 at the range 20.1 – 23.0 °C. There is no effect of high temperature on the stability of the formulation, since after 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The stability data indicate a shelf life of at least 2 years at ambient temperature when stored in in a tightly closed commercial packaging (HDPE/PA COEX).  Its technical characteristics are acceptable for a WG formulation.

The intended concentration of use is 0.0625 % to 0.25 % 0.00375% to 1.25%.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

Not required. No classification and labelling with respect to physical and chemical properties is needed.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

Not required. No hazard and safety phrases are needed for this section.

Compliance with FAO specifications:

The product CHR/RW/PROH 100 WG complies with FAO specifications.

Formulation used for tests

CHR/RW/PROH 100 WG

Active substances:

Prohexadione calcium 100 g/kg

Production date:

10.05.2019

Batch number: 2

Specification of the product

| No. | Quality parameters | Method | Required |
| --- | --- | --- | --- |
| 1. | Appearance | Visual | Cylindrical light brown granules with delicate odour |
| 2. | Prohexadion calcium content, g/kg | HPLC | 100 ± 10 |
| 3. | pH of 1% (m/v) suspension in destilled water, 20⁰C | CIPAC MT 75.3 | 5,5 – 7,5 |
| 4. | Suspensibility of 1,25% (m/v) suspension in CIPAC D water in 30⁰C after 30 minutes min., % | CIPAC MT 184 | 90 |
| 5. | Water content, max., % | CIPAC MT 30.5 | 2,0 |
| 6. | Wet sieve test 75 µm, max., % | CIPAC MT 185 | 0,1 |
| 7. | Foaming of 1,25% (m/v) suspension in CIPAC D water after 1 minute, max., ml | CIPAC MT 47.3 | 30 |
| 8. | Wettability, max., s | CIPAC MT 53.3 | 20 |

Table 2-5:	Physical, chemical and technical properties of the plant protection product

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | Wisual inspection, nasal inhalation EPA OPPTS 830.6302 EPA OPPTS 830.6303 EPA OPPTS 830.6304 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | After first year of storage: After second year of storage: The colour of the test item was determined in relation to the Munsell Book of Colour. The physical state of test item was performed as a visual assessment. The odour of test item was assessed organoleptically. The study was performed as a visual assessment at 20.0°C and atmospheric pressure of 997.0 hPa, after accelerated storage test: at 20.0°C, 988.4 hPa, and after two years storage at 20.4°C and atmospheric pressure of 990.4 hPa. | Y | KCP 2.1/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.1/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019-part 1 KCP 2.1/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019- part 2 | Accepted Instead of a delicate odour it is more appropriate to describe is light characteristic odour. |
| Explosive properties (KCP 2.2.1) | EEC A.14 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | CHR/RW/PROH 100 WG does not have explosive properties according to the criteria of EEC A.14 method. | Y | KCP 2.2.1 Śliwa, P., 2019, CHR/RW/PROH 100 WG, Determination of explosive properties. BW-05/19 | Accepted |
| Oxidizing properties (KCP 2.2.2) | EEC A.17 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | CH/RW/PROH 100 WG does not have the oxidizing properties in accordance with test A.17 criteria. | Y | KCP 2.2.2 Flasińska, P., 2019, CHR/RW/PROH 100 WG, Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 | Accepted |
| Flash point (KCP 2.3.1) | EEC A.16 N/A | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 N/A | No thermal change of sample above 400 °C is observed. N/A | Y N/A | KCP 2.3.1 Flasińska, P., 2019, CHR/RW/PROH 100 WG, Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 N/A | Not applicable. Determination of the flash point is applicable only for liquid mixtures. |

| Flammability (KCP 2.3.2) | EEC A.10 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | CHR/RW/PROH 100 WG is not highly flammable in accordance with test A.10. criteria. | Y | KCP 2.3.2 Flasińska, P., 2019, CHR/RW/PROH 100 WG, Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 | Accepted |
| --- | --- | --- | --- | --- | --- | --- |
| Self-heating (KCP 2.3.3) | N/A EEC A.16 | N/A CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | N/A No thermal change of sample above 400 °C is observed. The tested material does not have the relative self-ignition temperature in accordance with test A.16. criteria. | N/A Y | N/A KCP 2.3.3 Flasińska, P., 2019, CHR/RW/PROH 100 WG, Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 | N/A EEC A.16 |
| Acidity or alkalinity and pH (KCP 2.4.1) | N/A | N/A | N/A | N/A | N/A | Not applicable CHR/RW/PROH 100 WG is a solid. |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | MT 75.3 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Test item; Measurment; 1% (w/v) suspension. Initial; pH; 6.11. Temperature [°C]; 23.0. Accelerate storage; pH; 6.10. Temperature [°C]; 21.9. After first year of storage; pH; 6.18. Temperature [°C]; 20.1. After second year of storage; pH; 6.18. Temperature [°C]; 20.0 19.9. | Y | KCP 2.4.2/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.4.2/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019- part 1 KCP 2.4.2/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019- part 2 | Accepted |
| Viscosity (KCP 2.5.1) | N/A | N/A | N/A | N/A | N/A | Not applicable CHR/RW/PROH 100 WG is a solid. |
| Surface tension (KCP 2.5.2) | N/A | N/A | N/A | N/A | N/A | Not applicable CHR/RW/PROH 100 WG is a solid. |
| Relative density (KCP 2.6.1) | N/A | N/A | N/A | N/A | N/A | Not applicable CHR/RW/PROH 100 WG is a solid. |
| Bulk density (KCP 2.6.2) | CIPAC MT 186 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Pour density: 0.6133 ± 0.0078 g/mL Tap density: 0.7030 ± 0.0016 g/mL | Y | KCP 2.6.2 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 | Accepted For water-dispersible granules (WG formulation), the preferred method is CIPAC MT 169 (tap density), but the CIPAC MT 186 is the method used for all solid formulations. |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.3 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Colour and physical state: Active substance content: After the end of the accelerated storage the content of prohexadium- calcium was 10.03% (before accelerated storage test was 9.97%). pH: Wettability: Suspensibility: Spontaneity of dispersion: Wet sieve: Residual percentage of test item on the sieve 75 μm was 0.00% after accelerated storage test. Dry sieve test: Determination of tap density was conducted before the sieve analysis according to CIPAC MT 169, tap density of test item was 0.7074 g/mL after accelerated storage test. Dust content: Attrition: The attrition resistance value of test item was 99.4% after accelerated storage test. Flowability: The test item was subjected to the pressure of 25 g/cm2 during storage for 14 days at 54 °C. After storage, the test item was evaluated for flowability by using a sieve with 5 mm holes. The test item is flowable in 100%. | Y | KCP 2.7.1/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.7.1/02 Arévalo, E., 2019, CHR/RW/PROH 100 WG, Determination of dustiness after accelerated storage. BF-33/19-02 | Accepted For accelerated storage the average spontaneity of dispersion is equal to 98% not 97%. |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Minimum content after heat stability testing (KCP 2.7.3) | HPLC-DAD | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | After the end of the accelerated storage the content of prohexadium- calcium was 10.03%. | Y | KCP 2.7.3 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 | Accepted |
| Effect of low temperatures on stability (KCP 2.7.4) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Ambient temperature shelf life (KCP 2.7.5) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | CIPAC MT 75.3 CIPAC MT 184 CIPAC MT 174 CIPAC MT 53.3 CIPAC MT 187 CIPAC MT 185 CIPAC MT 178.2 CIPAC MT 170 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | First year of storage: Second year of storage: | Y | KCP 2.7.6/01 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - Part 1 KCP 2.7.6/02 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Wettability (KCP 2.8.1) | CIPAC MT 53.3 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Test item; Mass of test item- procedure without swirling [g]; Mass of test item- procedure with swirling [g]. Initial; 5.00; 5.03. Accelerated storage; 5.05; 5.02. First year of storage; 5.00; 5.00. After second year of storage; 5.01; 5.00. The wetting time was 1 second without swirling and 0 second with swirling for before and after accelerated storage. The wetting time of test item for first year storage: - without swirling - 14 seconds, - with swirling - 2 seconds. The wetting time of test item for second year of storage: - without swirling - 2 seconds, - with swirling - 1 second. | Y | KCP 2.8.1/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.1/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - Part 1 KCP 2.8.1/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted Complete wetting occurrred within 1 minute therefore, the test with swirling is not required. |
| Persistence of foaming (KCP 2.8.2) | MT 47.3 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The study was conducted according to CIPAC 47.3 for three concentrations 0.00375%, 0.05% and 1.25%. Volume of the foam formed was read after 1 and 12 min. ±10 s. 0.00375%: 0.05%: 1.25%: Maximum: 30 mL after 1 min and 28 mL after 12 min at concentration 0.05%. | Y | KCP 2.8.2/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.2/02 Patrzałek, M.., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/126/2020 | Accepted |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184.1 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The determination was carried out for concentration 0.00375%, 0.05%. 0.2% and 1.25% (w/v) of the test item in Standard Water D at 30°C±2°C. A minimum of 98% shall be in suspension after 30 min in CIPAC Standard Water D at 30 ± 2 °C for 1.25% concentration. After second year of storage: A minimum of 93% shall be in suspension after 30 min in CIPAC Standard Water D at 30 ± 2 °C for for 1.25% concentration. | Y | KCP 2.8.3.1/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.3.1/02 Patrzałek, M.., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/126/2020 KCP 2.8.3.1/03 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - Part 1 KCP 2.8.3.1/04 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted Initial material: for concentration 0.2% for Samle 2 is 98.39% not 98.58%; for concentration 1.25% for suspensibility is 99% not 98%. It is recommended that the test CIPAC MT 184.1 is caried out at 0.1% but applicant performed the test at four different concentrations including the concentration range of 0.1%. |

| Spontaneity of dispersion (KCP 2.8.3.2) | CIPAC MT 174 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Test item; Sample 1 [%]; Sample 2 [%]; Average [%]. Initial; 99.11; 99.40; 99. Accelerated storage; 96.70; 99.55; 978. First year storage; 97.62; 97.05; 97. Second year storage; 92.86; 92.00; 92.43. | Y | KCP 2.8.3.2/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.3.2/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019- Part 1 KCP 2.8.3.2/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| --- | --- | --- | --- | --- | --- | --- |
| Dispersion stability (KCP 2.8.3.3) | N/A | N/A | N/A | N/A | N/A | Not applicable for WG formulations. |
| Degree of dissolution and dilution stability (KCP 2.8.4) | N/A | N/A | N/A | N/A | N/A | Not applicable for WG formulations. |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | CIPAC MT 187 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | After 1st year of storage:; After 2nd year of storage:. | Y | KCP 2.8.5.1.1/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.5.1.1/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019- Part 1 KCP 2.8.5.1.1/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The study was conducted in accordance with CIPAC MT 185. Residual percentage of test item on the sieve 75 μm was 0.00% before, after accelerated storage, after 1st and 2nd year of storage test. | Y | KCP 2.8.5.1.2/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.5.1.2/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/2020- Part 1 KCP 2.8.5.1.2/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Dry sieve test (KCP 2.8.5.1.3) | CIPAC MT 170 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Fraction [μm]; Content [%] Initial Formulation; Content [%] Formulation after accelerated storage; Content [%] Formulation after one year of storage; Content [%] Formulation after two years of storage. 4000; 0.00; 0.00; 0.00; 0.00. 2000; 0.00; 0.00; 0.00; 0.00. 1000; 89.30; 74.89; 80.64; 79.44. 500; 9.27; 24.23; 18.31; 17.96. 250; 0.40; 0.25; 0.41; 0.16. 125; 0.12; 0.16; 0.102; 0.09. 75; 0.05; 0.07; 0.031; 0.84. SUM; 99.14; 99.60; 99.49; 98.49. | Y | KCP 2.8.5.1.3/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.5.1.3/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - Part 1 KCP 2.8.5.1.3/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Dust content (KCP 2.8.5.2.1) | CIPAC MT 171.1 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Initial material: After accelerated storage: After one year of storage: After two years of storage: Initial material: 0.55 mg (Nearly dust free) After accelerated storage: 1.95 mg (Nearly dust free) After one year of storage: 3.10 mg (Nearly dust free) After two years of storage: 1.60 mg (Nearly dust free) | Y | KCP 2.8.5.2.1/01 Arévalo, E., 2019, CHR/RW/PROH 100 WG, Determination of dustiness. BF-33/19-01 KCP 2.8.5.2.1/02 Arévalo, E., 2019, CHR/RW/PROH 100 WG, Determination of dustiness after accelerated storage. BF-33/19-02 KCP 2.8.5.2.1/03 Arévalo, E., 2020, CHR/RW/PROH 100 WG, Determination of dustiness after one year of storage. BF-36/20 KCP 2.8.5.2.1/04 Zieliński, J., 2021, CHR/RW/PROH 100 WG, Determination of dustiness after two years of storage BF-43/21 | Accepted |
| Particle size of dust (KCP 2.8.5.2.2) | CIPAC MT 187 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The particle size value was determined from five independent measurements between 10% and 99.99% of cumulative %. Cum [%]; Initial formulation average [μm]; Formulation after accelarated storage [μm]; Formulation after one year of storage [μm]; Formulation after two years of storage [μm]. 10.00; 0.360; 0.348; 0.614; 0.652. 20.00; 0.535; 0.499; 1.299; 1.281. 30.00; 0.904; 0.948; 1.999; 1.910. 40.00; 1.208; 1.282; 2.706; 2.580. 50.00; 1.534; 1.626; 3.526; 3.376. 60.00; 1.864; 1.970; 4.550; 4.381. 70.00; 2.206; 2.325; 5.861; 5.677. 80.00; 2.617; 2.743; 7.665; 7.552. 90.00; 3.179; 3.337; 10.642; 10.841. 99.99; 7.156; 7.355; 36.166; 40.627. | Y | KCP 2.8.5.2.2/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.5.2.2/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - Part 1 KCP 2.8.5.2.2/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Attrition (KCP 2.8.5.3) | CIPAC MT 178.2 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The test item was initially deprived of dusty fraction, and then it was placed in a glass bottle on a roller and rolled for 142 minutes. The attrition resistance was determined by sieving the granulate through a 125 μm sieve and determining of the mass left on the sieve. The attrition resistance value of test item was 99.9% before accelerated storage test, 99.4% after accelerated storage test, 99.4% after first year of storage and 100% after second year of storage. | Y | KCP 2.8.5.3/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.8.5.3/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 KCP 2.8.5.3/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Hardness and integrity (KCP 2.8.5.4) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Emulsifiability (KCP 2.8.6.1) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Emulsion stability (KCP 2.8.6.2) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Re-emulsifiability (KCP 2.8.6.3) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Flowability (KCP 2.8.7.1) | CIPAC MT 172.1 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The test item was subjected to the pressure of 25 g/cm2 during storage for 14 days at 54 °C. After storage, the test item was evaluated for flowability by using a sieve with 5 mm holes. The test item is flowable in 100%. | Y | KCP 2.8.7.1 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 | Accepted |
| Pourability (KCP 2.8.7.2) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Dustability following accelerated storage (KCP 2.8.7.3) | CIPAC MT 171.1 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | After accelerated storage: After one year of storage: After two years of storage: The average dustiness after accelerated storage is equal to 1.95 mg (0.01%). This result is comparable to the result for two years of storage which is equal 1.60 mg (0.01%). | Y | KCP 2.8.5.2.1/027.3/01 Arévalo, E., 2019, CHR/RW/PROH 100 WG, Determination of dustiness after accelerated storage. BF-33/19-02 KCP 2.8. 5.2.1/02 7.3/02 Arévalo, E., 2020, CHR/RW/PROH 100 WG, Determination of dustiness after one year of storage. BF-36/20 KCP 2.8. 5.2.1/02 7.3/02 Zieliński, J., 2021, CHR/RW/PROH 100 WG, Determination of dustiness after two years of storage BF-43/21 | Accepted |
| Physical compatibility of tank mixes (KCP 2.9.1) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Chemical compatibility of tank mixes (KCP 2.9.2) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Adhesion to seeds (KCP 2.10.1) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Distribution to seed (KCP 2.10.2) | N/A | N/A | N/A | N/A | N/A | Not applicable |
| Stability of packaging and packaging/preparation interactions (KCP 2.11.1) | SPB/38 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | Stability of packaging was determinate by the difference in the weight of the packaging filled with the test item before and after the accelerate storage test in 54 ± 2 °C for 14 days and after one year of storage and after two years of storage. After accelerated storage. Number package; Change in packaging weight [%]; Change in gross weight [%]. 1/1; 0.39; 0.02. After one year of storage. 1/1; 0.49; 0.05. After two years of storage. 1/1; 0.47; 0.03. | Y | KCP 2.11.1/01 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 KCP 2.11.1/02 Patrzałek, M.., 2020, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/2019 - Part 1 KCP 2.11.1/03 Łebek, B., 2021, CHR/RW/PROH 100 WG, Determination of physicochemical properties after shelf-life test. ICB/92/202019 - part 2 | Accepted |
| Effectiveness of cleaning (KCP 2.11.2) | Efficacy Guidline 305 | CHR/RW/PROH 100 WG Active substances: Prohexadione calcium 100 g/kg Production date: 10.05.2019 Batch number: 2 | The study was conducted according to Efficacy Guideline 305. The mixture of test item was prepared at a concentration of 0.05%. | Y | KCP 2.11.2 Patrzałek, M.., 2019, CHR/RW/PROH 100 WG, Determination of physicochemical properties. ICB/91/2019 | Accepted |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

Table 4.1-1 Packaging information

| Type | JAR |
| --- | --- |
| Material: | HDPE |
| size: | 63/64 mm / 91.5 mm |
| Opening: | 46 mm minimum |
| Closure: | screw cap with seal |
| Capacity | 188 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-2:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 64 mm± 2 mm/130 mm ± 3 mm |
| Opening: | 40 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-3:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 72 mm± 2 mm/111,8 mm ± 3 mm |
| Opening: | 38 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-4:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 72±1 mm/111.8±2 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-5:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 69 mm± 2 mm/186.5 mm ± 2 mm |
| Opening: | 45.65± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 564 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-6:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 90,5 mm± 2 mm/151 mm ± 3 mm |
| Opening: | 40,6 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-7:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 77,6 mm± 2 mm/160,6 mm ± 3 mm |
| Opening: | 38 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-8:                    Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 77.6 ±1 mm/160.6±2 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-9:                    Packaging information

The jar is set in an inner box (cardboard box). The inner box is grouped into an outer box

Material: HDPE

Type of construction: jar

Size: approximate diameter/height: 80 mm/138 mm

Capacity: 510 ml overflow

Type of closure: screw-cap with seal

Size of opening: 46 mm minimum

Accessories: one measuring device per each jar

Table 4.1-10:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 145.5mm± 2 mm/78mm ± 2 mm |
| Opening: | 56mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 600 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-11:                  Packaging information

| Type | JAR |
| --- | --- |
| Material: | HDPE |
| size: | 79/80 mm/ 201 mm |
| Opening: | 46 mm minimum |
| Closure: | screw cap with seal |
| Capacity | 800 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-12:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 80 mm± 2 mm/201 mm ± 2 mm |
| Opening: | 64 mm |
| Closure: | screw cap with seal |
| Capacity | 800 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-13:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 88.5 mm± 2 mm/283.5 mm ± 2 mm |
| Opening: | 45.30 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-14:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 88 mm± 4 mm/242 mm ± 6 mm |
| Opening: | 39mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-15:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 238 mm± 2 mm/90mm ± 2 mm |
| Opening: | 39 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-16:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 234 mm± 2 mm/88.5mm ± 2 mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-17:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 84 mm± 2 mm/248.2 mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-18:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 234 mm± 2 mm/88.5mm ± 2 mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | cap with seal |
| Capacity | 1200 ± 50 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-19:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 84 ± 1.5 mm/230.1 ± 3 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-20:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 157,2 mm± 2 mm/101mm ± 2 mm |
| Opening: | 72 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-21:                  Packaging information

| Type | JAR |
| --- | --- |
| Material: | HDPE |
| size: | 108/110 mm/ 266 mm |
| Opening: | 46 mm minimum |
| Closure: | screw cap with seal |
| Capacity | 2000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-22:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 232 mm± 2 mm/195mm± 2 mm/130mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 3000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-23:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 94 ± 1 mm/103 ± 1 mm/272.5 ± 3 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 2000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-24:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE |
| size: | 224,1 mm± 2 mm/122mm ± 2 mm |
| Opening: | 73 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 2000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-25:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 232 mm± 2 mm/195mm± 2 mm/130mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 3000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-26:                  Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 96 ± 3 mm/195 ± 3.5 mm/297.2 ± 4 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 4000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-27:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 305mm± 5 mm/193 mm± 5 mm/142 mm ± 5 mm |
| Opening: | 59.20 mm minimum ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5850 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-28:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 336 mm± 5 mm/195mm± 5 mm/130mm ± 5 mm |
| Opening: | 50 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-29:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE |
| size: | 310,5 mm± 5 mm/195mm± 5 mm/130mm ± 5 mm |
| Opening: | 63 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-30:                  Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 190 mm± 5 mm /140 mm± 5 mm/ 314 mm ± 5 mm |
| Opening: | 54.5 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-31:                  Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 127±2 mm/192±2 mm/285±5 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-32:                  Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE |
| size: | 145±2 mm/190.8±3/294±4 mm |
| Opening: | 38 mm |
| Closure: | screw cap with seal |
| Capacity | 6000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-33:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 50 ± 1 mm/93 ± 1 mm |
| Opening: | 28,4 ± 0,3 mm |
| Closure: | screw cap with seal |
| Capacity | 120 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-34:                  Packaging information

| Packaging information for 120 ml BOTTLE |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/PA COEX |
| size: | 50 ± 1 mm/93 ± 1 mm |
| Opening: | 28,4 ± 0,3 mm |
| Closure: | screw cap with seal |
| Capacity | 120 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-35:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 62.5±1 mm/131.3±1 mm |
| Opening: | 45.65±3 mm |
| Closure: | screw cap with seal |
| Capacity | 323 ± 5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-36:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA |
| size: | 59 ± 1 mm/143 ± 1 mm/ |
| Opening: | 41.7±0.7 mm |
| Closure: | screw cap with seal |
| Capacity | 275 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-37:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA |
| size: | 59 ± 1 mm/143 ± 1 mm/ |
| Opening: | 41.7±0.7 mm |
| Closure: | screw cap with seal |
| Capacity | 275 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-38:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA |
| size: | 69 mm ± 2 mm/186.5 mm ± 2 mm |
| Opening: | 45.65±3 mm |
| Closure: | screw cap with seal |
| Capacity | 574 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-39:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 74± 1 mm/177 ± 1 mm/ |
| Opening: | 41.7±0.7 mm |
| Closure: | screw cap with seal |
| Capacity | 550 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-40:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 74± 1 mm/177 ± 1 mm/ |
| Opening: | 41.7±0.7 mm |
| Closure: | screw cap with seal |
| Capacity | 550 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-41:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA |
| size: | 65 mm/234.8 mm ± 2 mm |
| Opening: | 27.4 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-42:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 88 mm ± 2 mm/238 mm ± 2 mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-43:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA |
| size: | 248.5±3 mm/84±1.5mm |
| Opening: | 50 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-44:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA |
| size: | 248.5±3 mm/84±1.5mm |
| Opening: | 50 mm ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-45:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | PE-PA |
| size: | 234 mm± 2 mm/88.5mm ± 2 mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-46:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 238± 1 mm/88 ± 1 mm/ |
| Opening: | 41.7±0,7 mm |
| Closure: | screw cap with seal |
| Capacity | 1100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-47:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 84± 1.5 mm/248.5 ± 3 mm |
| Opening: | 50 mm ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-48:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 233.5± 1.5 mm/88.5 ± 1 mm/ |
| Opening: | 39 mm ± 2 mm |
| Closure: | screw cap with seal |
| Capacity | 1100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-49:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 84± 1.5 mm/248.5 ± 3 mm |
| Opening: | 50 mm ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-50:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 305mm± 5 mm/193 mm± 5 mm/142 mm ±5 mm |
| Opening: | 63 mm minimum ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 5850 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-51:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 193 ± 3/ 142 ± 5 mm/320 mm± 5 mm |
| Opening: | 63,3 ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 5500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-52:                  Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 195 ± 3/ 130 ± 5 mm/310,5 mm± 5 mm |
| Opening: | 63,3 ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-53:                  Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 313± 5mm/190±3/140±5mm |
| Opening: | 50 mm ± 3mm |
| Closure: | screw cap with seal |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-54:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 305mm/193 mm/142 mm ± 5 mm |
| Opening: | 63 mm minimum ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-55:                  Packaging information

| Type | CONTAINER |
| --- | --- |
| Material: | HDPE/PA COEX |
| size: | 377,7mm/178 mm/239,5 mm ± 5 mm |
| Opening: | 54 mm min ± 5 mm |
| Closure: | screw cap with seal |
| Capacity | 10000 ml±150 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-56: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 63.5±1 mm/126±1 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 318 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-57: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 63.5±1 mm/126±1 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 312 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-58: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 585 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-59: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 580 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-60: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 575 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-61: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 69±1 mm/186±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 570 ± 12.5 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-62: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1150 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-63: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1160 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-64: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1170 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-65: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1185 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-66: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 88.5±1 mm/233.2±1.6 mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 1200 ± 20 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-67: Packaging information

| Type | Cannister |
| --- | --- |
| Material: | HDPE/F |
| size: | 193±2 mm/142±2mm/305±3mm |
| Opening: | 50 mm |
| Closure: | screw cap with seal |
| Capacity | 5880 ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-68: Packaging information

| Type | Cannister |
| --- | --- |
| Material: | HDPE/F |
| size: | 193±2 mm/142±2mm/305±3mm |
| Opening: | 63 mm |
| Closure: | screw cap with seal |
| Capacity | 5880 ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-69: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 54,2 mm ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-70: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 63.4 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-71: Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 67,5 mm ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-72: Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 54,2 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-73: Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 63,4 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-74: Packaging information

| Type | CANNISTER |
| --- | --- |
| Material: | HDPE/F |
| size: | 297,3mm/193 mm/142 mm ± 2 mm |
| Opening: | 67,5 mm min ± 1 mm |
| Closure: | screw cap with seal |
| Capacity | 5950 ml ± 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-75: Packaging information

| Type | Cannister |
| --- | --- |
| Material: | HDPE/F |
| size: | 240±2 mm/179±2mm/375±3mm |
| Opening: | 63 mm |
| Closure: | screw cap with seal |
| Capacity | 10 000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-76:            Packaging information for 500 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| Size: | 69 mm± 2 mm/186.5 mm ± 2 mm |
| Opening: | 45.30 mm ± 2 mm |
| Capacity | 500 ml |
| Closure: | screw cap with cutter |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-77:            Packaging information for 500ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| size: | 65 mm/234.8 mm ± 2 mm |
| Opening: | 27.4 mm |
| Closure: | screw cap with seal |
| Capacity | 500 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-78:            Packaging information for 1000ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| size: | 234±3 mm/88.5±2mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-79:            Packaging information for 1000 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| Size: | 234±3 mm/88.5±2mm |
| Opening: | 42 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-80:            Packaging information for 1000 ml bottle

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| Size: | 234±3 mm/88.5±2mm |
| Opening: | 50 mm ± 3 mm |
| Closure: | screw cap with cutter |
| Capacity | 1000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-81:            Packaging information for 5000 ml container

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| Size: | 165 mm ± 2 mm/195 mm ± 2 mm/228mm± 2 mm |
| Opening: | 48 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 5000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-82:            Packaging information for 10000 ml container

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| Size: | 195 mm ± 2 mm/225mm± 2 mm/306mm± 2 mm |
| Opening: | 48 mm ± 2 mm |
| Closure: | screw cap with cutter |
| Capacity | 10000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-83:            Packaging information for 20000 ml container

| Type | Description |
| --- | --- |
| Material: | HDPE/EvOH |
| Size: | 375 mm ± 2 mm/290mm± 2 mm/245mm± 2 mm |
| Opening: | 85mm ± 2 mm |
| Closure: | Screw cap with seal |
| Capacity | 20000 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-84:	Packaging information

| Type | BOTTLE |
| --- | --- |
| Material: | HDPE/F |
| size: | 50 ± 1 mm/93 ± 1 mm |
| Opening: | 28,4 ± 0,3 mm |
| Closure: | screw cap with seal |
| Capacity | 120 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | compliant |

Table 4.1-85:	Packaging information for HDPE/EvOH 0.1 liter bottle

| Packaging information for 100 ml Bottle |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE / EVOH |
| Body diameter / total height: | 48,50mm +- 1 ,00mm / 95,50mm +- 1,00mm |
| External thread diameter: | 27,3mm +- 0,15mm |
| Closure: | screw cap with seal |
| Capacity | 100 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | N/A |

Table 4.1-86:	Packaging information for HDPE/EvOH 0.25 liter bottle

| Packaging information for 250 ml Bottle |  |
| --- | --- |
| Type | BOTTLE |
| Material: | HDPE/EVOH |
| Body diameter / total height: | 62,50mm +- 0,50mm / 126,50mm +- 1,50mm |
| External thread diameter: | 49,65mm +- 0,35mm |
| Closure: | screw cap with seal |
| Capacity | 250 ml |
| Seal: | Induction seal |
| Manner of construction | extruded |
| UN/ADR | N/A |
| According to guideline from Ministry of Agriculture and Rural Development (Wytyczna w sprawie zasad zatwierdzania opakowań w środkach ochrony roślin) data of stability in the material HDPE are extrapolable to the all materials (HDPE/PA; HDPE/F; HDPE/EvOH). Therefore, no further studies are required for the additional packaging materials. |  |

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.1/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.1/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.2.1 | Śliwa, P. | 2019 | Determination of explosive properties. BW-05/19 Insytut Przemysłu Organicznego, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.2.2 | Flasińska, P. | 2019 | Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 Insytut Przemysłu Organicznego, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.3.12 | Flasińska, P. | 2019 | Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 Insytut Przemysłu Organicznego, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.3.23 | Flasińska, P. | 2019 | Determination of flammability, relative self-ignition temperature and oxidizing properties BC-18/19 Insytut Przemysłu Organicznego, Warsaw, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.2/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.2/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.4.2/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.6.2 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.1/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.1/02 | Arévalo, E. | 2019 | Determination of dustiness after accelerated storage BF-33/19-02 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.3 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.6/01 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.7.6/02 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.1/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.1/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.1/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.2/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.2/02 | Patrzałek, M. | 2021 | Determination of physicochemical properties. ICB/126/2020 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.1/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.1/02 | Patrzałek, M. | 2021 | Determination of physicochemical properties. ICB/126/2020 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.1/03 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.1/04 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.2/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.2/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.3.2/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.1/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.1/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.1/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.2/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.2/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.2/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.3/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.3/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.1.3/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.1/01 | Arévalo, E. | 2019 | Determination of dustiness. BF-33/19-01 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.1/02 | Arévalo, E. | 2019 | Determination of dustiness after accelerated storage BF-33/19-02 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.1/03 | Arévalo, E. | 2020 | Determination of dustiness after one year of storage. BF-36/20 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.1/04 | Zieliński, J. | 2021 | Determination of dustiness after two years of storage. BF-43/21 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.2/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.2/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.2.2/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.3/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.3/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.5.3/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.1 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.3/01 | Arévalo, E. | 2019 | Determination of dustiness after accelerated storage BF-33/19-02 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.3/02 | Arévalo, E. | 2020 | Determination of dustiness after one year of storage. BF-36/20 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.8.7.3/03 | Zieliński, J. | 2021 | Determination of dustiness after two years of storage. BF-43/21 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11.1/01 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11.1/02 | Patrzałek, M. | 2020 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 1 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11.1/03 | Łebek, B. | 2021 | Determination of physicochemical properties after shelf-life test. ICB/92/2019- Part 2 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |
| KCP 2.11.2 | Patrzałek, M. | 2019 | Determination of physicochemical properties ICB/91/2019 ICB Pharma, Jaworzno, Poland GLP Unpublished | N | Chemirol |

- Additional data on the physical, chemical and technical properties of the active substance